3GPP TSG RAN WG1 #110b		R1-2209013

e-Meeting, Oct. 10th – 19th, 2022

Agenda Item:	9.2.3.1

Source:	Fujitsu

Title:	Evaluation on AI/ML for beam management

Document for:	Discussion/Decision

Introduction

In RAN#110 meeting, the EVM has been discussed and agreed [1][2] for the evaluation on AI/ML for beam management. This contribution presents our views on EVM and the evaluation results of spatial-domain DL beam prediction.

EVM on DL beam prediction

For spatial-domain DL beam predication, NW configures subset of RSs for beam measurement. And UE measures L1-RSRPs (Set B) of subset of beam pairs and input them to the AI/ML model. The AI/ML model will predict the L1-RSRPs (Set A) of all beam pairs. Then, the potential one or several best beam pairs are selected among the predicted L1-RSRPs of all beam pairs. The function of AI/ML model is shown as Figure 1.

Beams in transmitter

Beams in receiver

not measured beam

measured beam

AI/ML

Model

RSRPs for sub-set of beam pairs

RSRPs for all beam pairs

Figure 1: the function of AI/ML model for spatial-domain beam prediction

In RAN1#110 meeting, the selection of Set B has been discussed and agreed as follows

Agreement - Study the following options on the selection of Set B of beams (pairs) - Option 1: Set B is fixed across training and inference - FFS on the beams of Set B - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each report/measurement during training and/or inference) - FFS on fixed or variable number of beams (pairs) - FFS on the details - Other options are not precluded. - FFS on the number of beams (pairs) in Set B - Note: This does not preclude the alternative that Set B is different from Set A.

For the selection of Set B, two options have been agreed for further study. For option 1, since the fixed beam pattern will achieve better performance and it’s simpler to achieve the expected gain on AI/ML model, option 1 can be treated as baseline method for selection of Set B. For option 2, the variable Set B will provide configuration flexibility and model generalization, this option can be considered as optional method for selection of Set B.

On the other hand, the details for the selection of Set B for both options are still FFS. For option 1, the beams of fixed Set B can be constructed with an even-spacing sampling rate from beams of Set A. For option 2, the beams of variable Set B can be constructed with different even-spacing sampling rates from beams of Set A.

Proposal 1: For selection of Set B of beams (pairs), it is suggested that the beams of fixed Set B are constructed with an even-spacing sampling rate from beams of Set A.

Proposal 2: For selection of Set B of beams (pairs), it is suggested that the beams of variable Set B are constructed with different even-spacing sampling rates from beams of Set A.

In RAN1#110 meeting, there are a lot of discussions on the KPI of RS overhead reduction as follows and the down selection is expected.

Proposal 2-2-1d: - For RS overhead or RS overhead reduction, further study the following options for down selection: - Option 1: RS OH%=1-NM - where N is the number of beams (pairs) (with reference signal (SSB and/or CSI-RS)) required for measurement (in Set B) - where M is the total number of beams (pairs) to be predicted (in Set A) - Option 2: RS OH%=1-1M1Ntn=1NtNn - Where Nn is the number of beams (pair) (in Set B) required for measurement during time slot n - where M is the total number of beams (pair) to be predicted (in Set A) - Option 3: RS OH%=1-(# of beams in Set B)+# of TopK beams not covered by Set B(# of beams in Set A) - Option 4: RS OH%=1-N+KM - where N is the number of beam pairs (with reference signal (SSB and/or CSI-RS)) required for measurement in Set B - Option 5: Companies report - The RS overhead reduction compared to an exhaustive beam sweep over set A - The RS overhead consisting of the beams being swept in Set B and the Top-K beams for P2 beam sweep after inference (if applicable) - Other options are not precluded - Note: the down selection may be different for BM-Case 1 and BM-Case 2. - [FFS on assumptions of beam sweeping]

When the KPI of RS overhead reduction is discussed, it needs to clarify what baseline beam sweeping method is used to compare with the RS overhead on AI/ML method. From discussion of RAN1#109 meeting, the exhaustive beam sweeping is agreed as the baseline beam sweeping method.

From our understanding, the RS overhead on AI/ML needs to calculate all configured RS resources in the beam sweeping procedure. Considering the beam sweeping with two steps, in the first step, the AI/ML model will predict one or several optimal beams (Top-K predicted beams). The Top-K predicted beams will be configured for beam sweeping in the second step. The RS resources in second step also need to be included when the RS overhead is calculated.

However, the configured RS resources in second step highly depend on what kind of beams is predicted. As discussion in [1], three alternatives for the predicted beams are agreed.

Agreement For the sub use case BM-Case1 and BM-Case2, further study the following alternatives for the predicted beams: - Alt.1: DL Tx beam prediction - Alt.2: DL Rx beam prediction - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam) - Note1: DL Rx beam prediction may or may not have spec impact

The configured RS resources are different for three alternatives of predicted beams in second step if the beam sweeping with two steps is applied.

Proposal 3: For the KPI of RS overhead reduction, it is suggested to consider three alternatives of predicted beams.

- Alt.1: DL Tx beam prediction

- Alt.2: DL Rx beam prediction

- Alt.3: Beam pair prediction

In second step of beam sweeping, when Tx beam is predicted, the predicted Top-K Tx beams will be configured to be measured with each Rx beam. In this case, the additional K*numberof Rx beam RS resources need to be configured. The similar additional K*numberof Tx beam RS resources need to be configured for Rx beam prediction. For beam pair prediction, the additional KRS resources corresponding to Top-K predicted beam pairs will be configured.

Regarding the three alternatives of predicted beam, the KPI of RS overhead reduction is suggested to be calculated as:

|  | Tx beam prediction | Rx beam prediction | Beam pair prediction |
| --- | --- | --- | --- |
| RS overhead reduction | 1- N+K*numberof Rx beamM | 1- N+K*numberof Tx beamM | 1- N+KM |

Proposal 4: Regarding the three alternatives of predicted beam, the KPI of RS overhead reduction is suggested to be calculated as：

|  | Tx beam prediction | Rx beam prediction | Beam pair prediction |
| --- | --- | --- | --- |
| RS overhead reduction | 1- N+K*numberof Rx beamM | 1- N+K*numberof Tx beamM | 1- N+KM |

Evaluation results beam pair prediction

In this section, the performance of the fixed and variable Set B is evaluated for DL spatial-domain beam pairs prediction. In our simulation, gNBs are assumed to be configured with 64 antenna elements which support 32 transmitting beams (4 beams in vertical and 8 beams in horizonal). UEs are configured with 2 panels and total 16 antenna elements which support 8 receiving beams (1 beam in vertical and 4 beams in horizonal for each panel). The details about the simulation parameters are shown in Table 3.

The samples of dataset generated by SLS are about 60k measurement results for total 256 beam pairs. 80% samples are used for model training and 20% samples are used for model testing. The AI/ML model is shown in figure 2.

Pre-processing

AI/ML model (FC)

Measured Set B (fixed or variable)

predicted Set A

Hidden layer (384,512)

Input layer (256)

Output layer (256)

（256,384））

Figure 2 the architecture of AI/ML model for DL spatial-domain beam prediction

To evaluate the performance with variable Set B, the pre-processing is used to adapt to variable Set B. The function of pre-processing can be implemented with non-AI/ML method (e.g., typical interpolation) which extend the variable size of Set B to the fixed size of Set A. In this case, if the variable Set B is applied, the architecture of neural network has no need to be changed (e.g., the numbers of nodes or layers) with help of pre-preprocessing.

Observation 1: The module of pre-processing provides the flexibility to adapt to variable Set B for AI/ML model.

The neural network includes 4 fully connected layers. Two hidden layers have 384 and 512 nodes respectively. The number of nodes for output and input layers depends on the size of Set A. The loss function of the network is MAE which gauges the differences between the predicted L1-RSRPs and the ground truth. The parameters about AI/ML training are shown in Table 4.

To evaluate the performance of AI/ML model for spatial-domain DL beam prediction, the following KPIs are used

- Average L1-RSRP difference of Top-1 predicted beam

- Beam prediction accuracy (%) for Top-1 and/or Top-K beams

- CDF of L1-RSRP difference for Top-1 predicted beam

- RS overhead reduction

  - Exhaustive beam sweeping is used as a baseline

- AI/ML model complexity

Beam pair prediction with fixed Set B

For beam pair prediction with fixed Set B, the Set B is constructed with an even-spacing sampling rate from beams of Set A. Two cases are evaluated as follows.

Case A: sampling rate is 4

16 Tx beams and 4 Rx beams (total 64 beam pairs) are used to predict the L1-RSRPs of all the 256 beam pairs.

Beams in transmitter

Beams in receiver

Not measured beam

Measured beam

Panel 2

Panel 1

Figure 3: beam pattern for DL spatial-domain prediction (Case A)

Case A: sampling rate is 8

8 Tx beams and 4 Rx beams (total 32 beam pairs) are used to predict the L1-RSRPs of all the 256 beam pairs.

Beams in transmitter

Beams in receiver

Not measured beam

Measured beam

Panel 2

Panel 1

Figure 4: beam pattern for DL spatial-domain prediction (Case B)

With above evaluation scenario, the evaluation results are shown as follows.

Table 1: simulation results of beam pair prediction for fixed Set B

|  | Case A (sampling rate = 4) |  |  |  | Case B (sampling rate =8) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | TOP1 | TOP2 | TOP4 | TOP8 | TOP1 | TOP2 | TOP4 | TOP8 |
| Beam prediction accuracy (%) | 68.4 | 88 | 95.9 | 98.4 | 62.3 | 80.7 | 90.7 | 95.1 |
| RS overhead reduction (%) | 75 | 74.2 | 73.4 | 71.9 | 87.5 | 86.7 | 85.9 | 84.3 |
| Average L1-RSRP difference(dB) | 1.61 |  |  |  | 3.11 |  |  |  |

Note: the RS overhead is calculated as proposal 3.

The CDF of L1-RSRP difference for Top-1 predicted beam is shown as following figure

Figure 5 CDF of L1-RSRP difference of beam pair prediction for fixed Set B

Comparing with the exhaustive beam sweeping, total 256 beams pairs (32 Tx beams and 8 Rx beams) are measured while the AI/ML method only measures 64 or 32 beam pairs for two cases respectively. In this case, the RS overhead for beam sweeping is reduced 75% and 87.5% of Top-1 predicted beam for case A and case B.

From the simulation results, it’s observed that the lower sampling rate achieves the better performance.

- The beam prediction accuracy of Top-2 is larger than 80% for both cases (88%@4 sampling rate and 81%@8 sampling rate).

- The average L1-RSRP difference is 1.6dB@4 sampling rate and 3.1dB@8 sampling rate respectively.

- The 70% probability is achieved with less than 2dB@4 sampling rate and 3.5dB@8 sampling rate for the estimated L1-RSRP difference of Top-1 predicted beam.

For the complexity of AI/ML model,

- The AI/ML model has total 427K parameters with single-float data type.

- The computational complexity of AI/ML model is about 426K float point of operations.

Observation 2:

For fixed set B constructed with predefined even-sampling rate (4 or 8) from beams of Set A,

- Comparing with the exhaustive beam sweeping, the RS overhead is reduced 75%@4 sampling rate and 87.5% @8 sampling rate for Top-1 predicted beam.

- The lower sampling rate achieves the better performance.

  - The beam prediction accuracy of Top-2 is larger than 80% for both cases (88%@4 sampling rate and 81%@8 sampling rate).

  - The average L1-RSRP difference is 1.6dB@4 sampling rate and 3.1dB@8 sampling rate respectively.

  - The 70% probability is achieved with less than 2dB@4 sampling rate and 3.5dB@8 sampling rate for the estimated L1-RSRP difference of Top-1 predicted beam.

- For the complexity of AI/ML model,

  - The AI/ML model has total 427 parameters with single-float data type.

  - The computational complexity of AI/ML model is about 426K float point of operations.

Beam pair prediction with variable Set B

To evaluate the performance of AI/ML model with variable Set B, additional dataset for model test is constructed with different sampling rates from beams of Set A. In our simulation, this additional dataset for model test is constructed with 50% samples of data from dataset for model test of case A (sampling rate =4) and case B (sampling rate =8) respectively.

Considering the model generalization, two trained models are considered. we firstly consider the model trained for case A. Secondly, the model is trained by the mixed dataset which is constructed with 50% data samples from the dataset for model training about case A and case B respectively. The evaluation results are shown as follows

Table 2: simulation results for beam pair prediction for variable Set B

| Simulation cases |  | Beam prediction accuracy (%) |  |  |  | Average L1-RSRP diff. (dB) |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | TOP1 | TOP2 | TOP4 | TOP8 |  |
| 1 | Model Training (case A: 100%) Model Test (case A: 50%+ case B: 50%) | 34.9 | 45.2 | 51.2 | 58.2 | 5.74 |
| 2 | Model Training (case A: 50%+ case B: 50%) Model Test (case A: 50%+ case B: 50%) | 64.5 | 84 | 93.5 | 96.9 | 2.4 |
| 3 | Model Training (case A: 50%+ case B: 50%) Model Test (case A: 100%) | 67.2 | 87.5 | 96.3 | 98.8 | 1.64 |
| 4 | Model Training (case A: 50%+ case B: 50%) Model Test (case B: 100%) | 61.9 | 80.5 | 90.6 | 95 | 3.16 |

It is observed that the module of pre-processing provides the flexibility for variable Set B while the number of nodes and layers of neural network has no need to be changed. With the same trained model, the performance is compared with of case A in Table 1(fixed Set B) and case 1 in Table 2 (variable Set B)，the Top-2 beam prediction accuracy with variable Set B is 45.2% which is worse than that with fixed Set B.

To improve the performance (case 1 in Table 2) of AI/ML model with variable Set B, the model is re-trained with mixed dataset as case 2 in Table 2. The simulation results show the performance improves a lot with 84% of Top-2 beam predication accuracy.

With re-trained AI/ML model by mixed dataset for variable Set B, the performance is verified for fixed Set B as case 3 and 4 in Table 2. The simulation results show the performance is almost the same as the model trained by separated dataset for fixed Set B as Table 1 shows.

Observation 3: For variable Set B, the model trained by mixed dataset constructed by samples with different sampling rates from beams of Set A will improve the performance of AI/ML model.

Observation 4: For variable Set B, the performance of the model trained by mixed dataset is almost the same as the model trained by separated dataset for fixed Set B.

- Conclusion

In this contribution, we present our views and the evaluation results of DL beam prediction on AL/ML. For the discussion, we have the following proposals and observation.

Proposal 1: For selection of Set B of beams (pairs), it is suggested that the beams of fixed Set B are constructed with an even-spacing sampling rate from beams of Set A.

Proposal 2: For selection of Set B of beams (pairs), it is suggested that the beams of variable Set B are constructed with different even-spacing sampling rates from beams of Set A.

Proposal 3: For the KPI of RS overhead reduction, it is suggested to consider three alternatives of predicted beams.

- Alt.1: DL Tx beam prediction

- Alt.2: DL Rx beam prediction

- Alt.3: Beam pair prediction

Proposal 4: Regarding the three alternatives of predicted beam, the KPI of RS overhead reduction is suggested to be calculated as：

|  | Tx beam prediction | Rx beam prediction | Beam pair prediction |
| --- | --- | --- | --- |
| RS overhead reduction | 1- N+K*numberof Rx beamM | 1- N+K*numberof Tx beamM | 1- N+KM |

Observation 1: The module of pre-processing provides the flexibility to adapt to variable Set B for AI/ML model.

Observation 2:

For fixed set B constructed with predefined even-sampling rate (4 or 8) from beams of Set A,

- Comparing with the exhaustive beam sweeping, the RS overhead is reduced 75%@4 sampling rate and 87.5% @8 sampling rate for Top-1 predicted beam.

- The lower sampling rate achieves the better performance.

  - The beam prediction accuracy of Top-2 is larger than 80% for both cases (88%@4 sampling rate and 81%@8 sampling rate).

  - The average L1-RSRP difference is 1.6dB@4 sampling rate and 3.1dB@8 sampling rate respectively.

  - The 70% probability is achieved with less than 2dB@4 sampling rate and 3.5dB@8 sampling rate for the estimated L1-RSRP difference of Top-1 predicted beam.

- For the complexity of AI/ML model,

  - The AI/ML model has total 427 parameters with single-float data type.

  - The computational complexity of AI/ML model is about 426K float point of operations.

Observation 3: For variable Set B, the model trained by mixed dataset constructed by samples with different sampling rates from beams of Set A will improve the performance of AI/ML model.

Observation 4: For variable Set B, the performance of the model trained by mixed dataset is almost the same as the model trained by separated dataset for fixed Set B.

- Appendix

Table 3:  parameters of SLS for spatial-domain DL beam prediction

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE Speed | - For spatial-domain beam prediction, 3km/h |
| UE distribution | - 10 UEs per sector/cell for evaluation. - 80% indoor ,20% outdoor as in TR 38.901 |
| Transmission Power | Maximum Power and Maximum EIRP for base station and UE as given by corresponding scenario in 38.802 (Table A.2.1-1 and Table A.2.1-2) |
| BS Antenna Configuration |  One panel: (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), (dV, dH) = (0.5, 0.5) λ Azimuth angle (degree) = [-78.75, -56.25, -33.75, -11.25, 11.25, 33.75, 56.25,78.75] Zenith angle (degree) = [22.5, 67.5,112.5, 157.5] Total 32 beams = 8(H)*4(V), DFT beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE Antenna Configuration | Panel structure: (M, N, P) = (1,4,2)  2 panels (left, right) with (Mg, Ng) = (1, 2) Azimuth angle (degree) = [22.5, 67.5, -67.5, -22.5] Total 8 beams = 4(H)*1(V)*2(panels), DFT beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8, Table A.2.1-10 |
| BS Tx Power | 40 dBm |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |

Table 4:  parameters of AI/ML model training

| Parameter | Value |
| --- | --- |
| Test samples | 12600 |
| Training samples | 50400 |
| Batch-size | 32 |
| Initial learning rate | 1.00E-03 |
| Epoch | 100 |
| Optimizer | Adam |
| Lr adjust schedule | Warm up + cosine annealing |
| Loss function | MAE |

- Reference

# [1] Draft report of 3GPP TSG RAN WG1 #110 v0.2.0, Toulouse, France, 22nd – 26th August 2022

# [2] R1-2208106, Feature lead summary #5 evaluation of AI/ML for beam management, Toulouse, France, 22nd – 26th August 2022