3GPP TSG RAN WG1 #110bis-e	R1-2209940

# Online, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 37.213 | CR | 00xx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Unified changes for Short Control Signaling Exemption and LBT Upgrade |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-10-10 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | To enable SIB1 signaling of uplink contention exemption for short control signaling and RRC signaling to control Channel Access Type upgrade from Type 1 to Type 2 or Type 3 |  |  |  |  |  |  |  |  |
| Summary of change: |  | - SIB1 parameter setting RA-Exempt-r17 to true is allowed if cell-wise msg1/msgA resource is no more than 10ms out of 100ms - RRC Parameter ul-Type2ChannelAccess-r17 controls Channel Access Type upgrade when in gNB COT and for resumption of COT |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | TS37.213 will be incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.4.4, 4.4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR is based on RAN1 common understanding, and it is expected that gNB and UE have been implemented in accordance to the CR. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

====TP A for 37.213===============

### 4.4.5     Exempted transmissions from sensing

In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB/UE may transmit the following transmission(s) on a channel without sensing the channel:

-     Transmission(s) of the discovery burst by the gNB

-     If the higher layer parameter RA-Exempt-r17 is configured, transmission(s) of the first message in a random access procedure by the UE

===End of TP A======================

====TP B for 37.213===============

### 4.4.5	Exempted transmissions from sensing

*** Unchanged part omitted***

When the gNB transmits the above transmissions without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB shall not occupy the corresponding channel more than 10ms over any 100ms interval.

When all the UEs in a cell transmit the above transmissions without sensing on a channel by utilizing the exemption above, the total duration of such transmissions by the all the UEs in a cell shall not occupy the corresponding channel more than 10ms over any 100ms interval.

===End of TP B======================

====TP C for 37.213===============

### 4.4.4	Channel access procedures in an initiated channel occupancy

<**Unchanged Text Omitted**>

If a gNB shares a channel occupancy initiated by a UE using configured grant PUSCH transmission and the UE is configured by cg-COT-SharingList-r17, the gNB may transmit a transmission that follows the configured grant PUSCH transmission by the UE if the following conditions are satisfied:

-	If the gNB determines that the 'COT sharing information' in CG-UCI in slot n indicates a row index that corresponds to a CG-COT-Sharing-r17 that provides channel occupancy sharing information, the gNB can share the UE channel occupancy starting from slot n+O, where O= offset-r17 slots, for a duration of D=duration -r17 slots where duration-r17, and offset-r17 are higher layer parameters provided by CG-COT-Sharing-r17.

4.4.4.1	Channel access procedures in a shared channel occupancy

If a UE determines the duration in time domain and the location in frequency domain of a remaining channel occupancy initiated by the gNB from a DCI format 2_0 as described in clause 11.1.1 of [7], the following is applicable:

-	The UE may switch from Type 1 channel access procedures as described in clause 4.4.1 to Type 2 channel access procedures as described in clause 4.4.2 if the higher layer parameter ul-Type2ChannelAccess-r17 is provided. Otherwise, the UE may switch from Type 1 channel access procedures as described in clause 4.4.1 to Type 3 channel access procedures as described in clause 4.4.3 for its corresponding UL transmissions within the determined duration in time and location in frequency domain of the remaining channel occupancy.

===End of TP C======================

====TP D for 37.213===============

### 4.4.4	Channel access procedures in an initiated channel occupancy

<**Unchanged Text Omitted**>

If a gNB initiates a channel occupancy using the channel access procedures described in clause 4.4.1 on a channel, the gNB may transmit a DL transmission(s) on the channel within the maximum Channel Occupancy Time described in Clause 4.4.1 on the channel after the DL transmission(s) initiating the channel occupancy. The followings are applicable to the DL transmission(s):

-	Regardless of the duration of the gap between the DL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the gNB, the DL transmission(s) occurs following the procedures described in Clause 4.4.3; or

-	if the gap between the DL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the gNB is more than a threshold that is determined by the gNB and is at least 8μs, the DL transmission(s) occurs following the procedures described in Clause 4.4.2.

If a UE initiates a channel occupancy using the channel access procedures described in clause 4.4.1 on a channel, the UE may transmit a UL transmission(s) on the channel within the maximum Channel Occupancy Time described in Clause 4.4.1 on the channel after the UL transmission(s) initiating the channel occupancy. The followings are applicable to the UL transmission(s):

-	If the higher layer parameter ul-Type2ChannelAccess-r17 is provided, if the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the UE is more than 8μs, the UL transmission(s) occurs following the procedures described in Clause 4.4.2.

-	Otherwise, regardless of the duration of the gap between the UL transmission(s) and any previous transmission(s) corresponding to the channel occupancy initiated by the UE, the UL transmission(s) occurs following the procedures described in Clause 4.4.3.

===End of TP D======================