3GPP TSG RAN WG1 #110bis-e			R1-2208329

e-Meeting, October 10th – 19th, 2022

3GPP TSG RAN WG2 Meeting #119-e	 R2-2209207

Online, August, 2022

Title:	LS on resource pool index in DCI Format 3_0

Response to:	-

Release:	Release 17

Work Item:	NR_SL_relay-Core

Source:	RAN2

To:	RAN1

Cc:	-

Contact person:	Xiao XIAO

xiao.xiao@vivo.com

Send any reply LS to:	3GPP Liaisons Coordinator, mailto:3GPPLiaison@etsi.org

Attachments:	-

# 1	Overall description

In Rel-17, RAN2 introduced mode-1 dedicated discovery resource pool configuration for NR SL discovery transmission, i.e. sl-DiscTxPoolScheduling, as now specified in TS 38.331. However, RAN2 found that current DCI Format 3_0, as specified in TS 38.212, cannot schedule any resource in the pool(s) indicated by sl-DiscTxPoolScheduling, since the “Resource pool index” field in DCI format 3_0 is currently unable to refer to any pool configured by sl-DiscTxPoolScheduling.

RAN2 discussed above issue and reached the agreement that the parameter “I” related to the “Resource pool index” field in DCI Format 3_0 in TS 38.212 is the number of resource pools for transmission configured by the higher layer parameter sl-TxPoolScheduling, if configured, and sl-DiscTxPoolScheduling, if configured. Also, according to TS 38.331, the total number of the resource pools configured in sl-TxPoolScheduling and sl-DiscTxPoolScheduling is no larger than 8.

RAN2 would like to request RAN1 to take above RAN2 agreement into account and make necessary specification change.

# 2	Actions

To RAN1

ACTION: 	RAN2 respectfully request RAN1 to take above RAN2 agreement into account, make necessary specification change and provide feedback (if any).

# 3	Dates of next TSG RAN WG2 meetings

TSG RAN WG2 Meeting #119bis-e	10 October – 19 October 2022	e-Meeting

TSG RAN WG2 Meeting #120	14 November – 18 November 2022	Toulouse, France