3GPP TSG RAN WG1 Meeting #110e-bis	R1-2208369

eMeeting, October 10 – 19, 2022

Agenda Item:	9.2.3.2

Source:	Futurewei

Title:	Continued discussion on other aspects of AI/ML for beam management

Document for:	Discussion

# Introduction

In RAN1#110, the following topics for beam management (other than EVM) have been covered during the meeting [1].

- Sub use cases

- Model training/inference alternatives, online vs offline training

- Beam prediction alternatives

- The construction of Set A and Set B of beams

- Input and output of BM-Case1 (spatial) and BM-Case2 (temporal)

- Spec impacts

Companies reached some agreements during the meeting, as summarized below.

- For both BM-Case1 and BM-Case2, model training at either NW side or UE side is supported.

- For both BM-Case1 and BM-Case2, study the following alternatives of beam prediction

  - Alt.1: DL Tx beam prediction

  - Alt.2: DL Rx beam prediction

  - Alt.3: Beam pair prediction (a beam pair consists of a DL Tx beam and a corresponding DL Rx beam)

  - Note1: DL Rx beam prediction may or may not have spec impact

- For MB-Case1, study the following alternatives for Set A and Set B construction

  - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

  - Alt.2: Set B is a subset of Set A

  - Note1: Set A is for DL beam prediction and Set B is for DL beam measurement.

  - Note2: The beam patterns of Set A and Set B can be clarified by the companies.

- For MB-Case2, study the following alternatives for Set A and Set B construction

  - Alt.1: Set A and Set B are different (Set B is NOT a subset of Set A)

  - Alt.2: Set B is a subset of Set A (Set A and Set B are not the same)

  - Alt.3: Set A and Set B are the same

  - Note1: The beam pattern of Set A and Set B can be clarified by the companies.

- For potential necessary specification impact related to data collection, study the following as the starting point.

  - Signalling/configuration/measurement/report for data collection, e.g., signalling aspects related to assistance information (if supported) and reference signals.

  - Content/type of the collected data

  - Other aspect(s) is not precluded

- For specification impacts regarding the model monitoring for BM-Case1 and BM-Case2, investigate the following aspects

  - Performance metric(s)

  - Benchmark/reference for the performance comparison

  - Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals

  - Other aspect(s) is not precluded

The following topics were discussed but no agreement have been reached

- The type of the training supported, i.e., offline vs online (e.g., RL)

- Input: views of companies are quite diverged, especially for the assistance information.

- Output: multiple alternatives discussed but failed to converge.

- Sub use cases other than BM-Case1 and BM-Case2

- Standard impacts related to model inference

Other topics

- The study/evaluation to determine the beam pattern of Set B was moved to the EVM session (Agenda item 9.2.3.1) to avoid duplication.

In this contribution, we further discuss aspects related to AI/ML based beam management other than evaluation methodology/EVM.

# AI/ML based beam management: other aspects to be considered

## Model training/inference related

Online training

During Meeting 110 the support of online training was discussed. Although not as popular as offline training (which was supported by all companies), companies did not rule out the support of online training. The major reason the discussion was tabled was that the definition of online training had not been finalized at the time of discussions. Given that the definition of online training has been accepted as one of the working assumptions, it is now a good time to revisit the proposal of online training.

We believe that, for AI/ML based beam management, online training may have some benefits as it may continuously adapt to changes in the environment, particular with UE mobility (e.g., in BM-Case2). Another popular scenario that could leverage online learning approach is when there is no labelled data available (or too costly to collect in advance) to train the AI/ML model via supervised learning approach. We feel that at the study phase, we do not need to put restriction on what approach(es) companies should use, at least not at this stage. The choice of approach, either online or offline training can be left to the companies.

Given the fundamental differences between the offline and online training like training data collection, interaction with the environment and the associated frequency required for feedback/reward, which involves information exchanges (some of which may be considered as overhead) and many others, the corresponding standards impact may also be different between the two. If a company chooses to use online training, the standards impact of online training should be included as part of the evaluation package. And the proposers are encouraged to justify the benefits of using online training.

Observation 1: Given the dynamic nature in the propagation environment, online training (e.g., reinforcement learning) may be a good alternative for AI/ML-based beam management in addition to offline learning approach like supervised learning. As the definition of online training has been approved, it is time to revisit online training as one of the two alternatives of training.

Proposal 1: For companies that propose to adopt online training, study the standards impacts, as well as the associated signalling overhead with online training for AI/ML-based beam management.

Input of BM-Case1 and BM-Case2

As the FL summarized in the offline discussions, views from companies on the training input diverged quite a bit, especially for the assistance information. The following is a list of all the assistance information received.

- Tx and/or Rx beam shape information (e.g., Tx and/or Rx beam pattern, Tx and/or Rx beam boresight direction (azimuth and elevation), 3dB beamwidth, etc.),

- expected Tx and/or Rx beam for the prediction (e.g., expected Tx and/or Rx angle, Tx and/or Rx beam ID for the prediction),

- UE position information,

- UE direction information,

- Tx beam usage information,

- UE orientation information, etc.

The FL hence proposed to further discuss these issues and encourage the proponents to provide more details/show benefits to convince other companies if assistance information is used for training.

In our view, features selected as input to AI/ML model are considered implementation dependent and proprietary. In our evaluation of AI/ML for beam management in [2], we use only the L1-RSRP as input and we have observed decent performance. As the use of assistance information may include additional overhead, we believe it is reasonable to request that, if assistance information is used for training, the performance gain needs to be justified against the overhead of obtaining and exchanging the assistance information.

Observation 2: Assistance information may come with additional cost like signalling overhead and there is usually a trade-off between performance gain and the associated overhead.

Proposal 2: When assistance information is used as input, study its performance gain vs. the standards impacts and overhead.

Output of BM-Case1 and BM-Case2

The model output of both BM-Case1 and BM-Case2 was also a heavily discussed topic during Meeting 110 and the group did not reach consensus at the end. The most agreeable alternatives are

  - Alt.1 Beam ID(s) and/or the predicted L1-RSRP

  - Alt.2 Beam ID(s) of the N predicted DL Tx and/or Rx beams and other information

  - Alt.3 Beam angle(s) and the predicted L1-RSRP (optional) of the N predicted DL Tx and/or Rx beams

In our view, companies are encouraged to share their output while they should be given some flexibility determining the output of their model. From this aspect, unless there is a standards impact, there is no need to specify the output of the model, as long as their systems know how to interpret the outputs.

Observation 3: Model outputs are typically used internally so unless there are standards impacts involved, they don’t need to be explicitly specified in the standards.

Proposal 3: Specify model outputs only when standards impact is involved while companies are encouraged to share their model output for AI/ML based beam management.

## Potential standards impact

The general aspect of standards impact

In Meeting 110, most of the proposals on standards impact are at high-level, and they belonged to one of the following aspects:

- AI/ML Model Training (e.g., data collection)

- AI/ML model inference

- AI/ML model life cycle management (e.g., model monitoring)

- UE capability

Among them, consensus have been reached for standards impact related to model training (data collection) and model life cycle management (model monitoring). They are summarized below.

For potential necessary specification impact related to data collection, study the following as the starting point.

- Signalling/configuration/measurement/report for data collection, e.g., signalling aspects related to assistance information (if supported) and reference signals.

- Content/type of the collected data

- Other aspect(s) is not precluded

For specification impacts regarding the model monitoring for BM-Case1 and BM-Case2, investigate the following aspects

- Performance metric(s)

- Benchmark/reference for the performance comparison

- Signalling/configuration/measurement/report for model monitoring, e.g., signalling aspects related to assistance information (if supported), Reference signals

- Other aspect(s) is not precluded

Standards impact related to model inference

For the standard impacts related to model inference, the following was proposed by the FL and was continuously revised for several rounds.

In order to facilitate the AI/ML model inference, study the following aspects as a starting point:

- Enhanced or new configurations/UE reporting/UE measurement, e.g., Enhanced or new beam measurement and/or beam reporting

- Beam indication of the predicted beam(s)

- Enhanced or new signalling for measurement configuration/triggering

- Signalling of assistance information (if supported)

- Other aspect(s) is not precluded

Although this proposal was not approved in Meeting 110 due to lack of online meeting time, it was supported by all companies. We note that if a proposal requires support of assistance information, additional standards impact may be involved; that was the reason we proposed to justify the use of assistance information as in Proposal 2 above.

In general, we support the FL’s proposal above.

Proposal 4: Study the following aspects as a starting point related to model inference on standards impact.

- Enhanced or new configurations/UE reporting/UE measurement, e.g., enhanced or new beam measurement and/or beam reporting

- Beam indication of the predicted beam(s)

- Enhanced or new signalling for measurement configuration/triggering

- Signalling of assistance information (if supported)

- Other aspect(s) is not precluded

For BM-Case1, if the prediction result is Top-N beam pairs, there is a need to map L1-RSRPs to the correct Tx-Rx beam pairs either directly or indirectly for training and inference purpose. Without this mechanism, there could be a mis-mapping between the measurements and the beam pairs. The exchange of such information may have standards impact.

Proposal 5: In AI/ML-based beam management, when model training is at NW side and “Alt.3: Beam pair prediction” is adopted, study the standards impact to enable gNB to map the received L1-RSRP measurements to the corresponding Tx-Rx beam pairs.

Standards impact related to life cycle management

Depending on deployment scenario and UE configuration/capability, multiple AI/ML models may be used to support different scenarios. In this case, model switching/configuration may be required after scenario identification procedure.

Proposal 6: Regarding AI/ML-based beam management, study the standards impact related to AI/ML model selection/configuration (like activation/deactivation) in case multiple trained AI/ML models are deployed.

Standards impact related to UE capabilities reporting

As mentioned in Proposal 6, depending on deployment scenario and UE capability, multiple AI/ML models may be used to support different scenarios. In this case, information like UE capability and/or other attributes like mobility may be used in selecting the AI/ML model.

Proposal 7: Regarding AI/ML-based beam management, study the standards impact, including AI/ML related UE configuration/capability reporting, related to AI/ML model selection/configuration (like activation/deactivation) in case multiple trained AI/ML models are deployed.

Standards impact related to model generalization

Model generalization is a very important aspect of AI/ML-based approaches. It refers to the capability of the model to adapt to previously unseen data, or even sometimes data from different scenarios.  Depending on the source and target scenarios or configurations, their data availability situation and AI/ML tasks expected to be generalized, various techniques can be leveraged, e.g., transfer learning. In some cases, supporting model generalization may require additional information (e.g., data from the new scenario, either labelled or unlabeled) to be collected which may introduce standards impact, then these standards impact needs to be discussed.

Proposal 8: Study Standards impact related to supporting model generalization across scenarios and/or configurations.

# Conclusions

In this contribution, we discussed aspects related to AI/ML-based beam management use case other than EVM; our observations and proposals are as follows.

Online Training

Observation 1: Given the dynamic nature in the propagation environment, online training (e.g., reinforcement learning) may be a good alternative for AI/ML-based beam management in addition to offline learning approach like supervised learning. As the definition of online training has been approved, it is time to revisit online training as one of the two alternatives of training.

Proposal 1: For companies that propose to adopt online training, study the standards impact, as well as the associated signalling overhead with online training for AI/ML-based beam management.

Input of BM-Case1 and BM-Case2

Observation 2: Assistance information may come with additional cost like signalling overhead and there is usually a trade-off between performance gain and the associated overhead.

Proposal 2: When assistance information is used as input, study its performance gain vs. the standards impacts and overhead.

Output of BM-Case1 and BM-Case2

Observation 3: Model outputs are typically used internally so unless there are standards impacts involved, they don’t need to be explicitly specified in the standards.

Proposal 3: Specify model outputs only when standards impact is involved while companies are encouraged to share their model output for AI/ML based beam management.

Standards impact related to model inference

Proposal 4: Study the following aspects as a starting point related to model inference on standards impact.

- Enhanced or new configurations/UE reporting/UE measurement, e.g., enhanced or new beam measurement and/or beam reporting

- Beam indication of the predicted beam(s)

- Enhanced or new signalling for measurement configuration/triggering

- Signalling of assistance information (if supported)

- Other aspect(s) is not precluded

Proposal 5: In AI/ML-based beam management, when model training is at NW side and “Alt.3: Beam pair prediction” is adopted, study the standards impact to enable gNB to map the received L1-RSRP measurements to the corresponding Tx-Rx beam pairs.

Standards impact related to life cycle management

Proposal 6: Regarding AI/ML-based beam management, study the standards impact related to AI/ML model selection/configuration (like activation/deactivation) in case multiple trained AI/ML models are deployed.

Standards impact related to UE capabilities reporting

Proposal 7: Regarding AI/ML-based beam management, study the standards impact, including AI/ML related UE configuration/capability reporting, related to AI/ML model selection/configuration (like activation/deactivation) in case multiple trained AI/ML models are deployed.

Standards impact related to model generalization

Proposal 8: Study Standards impact related to supporting model generalization across scenarios and/or configurations.

# References

R1-2207874, “Summary#4 for other aspects on AI/ML for beam management”, Feature Lead, RAN1#110.

R1-2208368, “Continued discussion on evaluation of AI/ML for beam management”, Futurewei, contribution to meeting 110-bis-e.