3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2208436

e-Meeting, October 10 - 19, 2022

Agenda Item:	9.11.2

Source:	Huawei, HiSilicon

Title:	Discussion on network-verified UE location for NR NTN

Document for:	Discussion and Decision

Introduction

In RAN# 110, a couple of agreements have been achieved on network-verified location for NR NTN [1]:

Agreement

The following 3GPP defined RAT dependent positioning methods shall be considered as starting point for the study on Network verified UE location in case of NGSO based NTN deployment:

- Multi-RTT

- DL/UL-TDOA

Note-1: Other methods (e.g. AoA based) are not precluded

Note-2: RAT independent positioning methods are not under the scope of the study

Agreement

For evaluating positioning performance in NTN, the following metrics apply.

- Horizontal accuracy:

- Horizontal accuracy is the difference between a calculated horizontal position by the network and the actual horizontal position of a UE (for evaluation purposes)

- At least CDFs of horizontal positioning errors are used as a performance metrics in NR positioning evaluations

- At least the following percentiles of positioning error is analyzed 50%, 67%, 80%, 90%, 95%

Agreement

The following parameters are assumed for the evaluation of RAT dependent positioning methods study in NTN:

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km, optional: 1200km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1: 2GHz, S-band (n256). Optional: FR2 |
| BW | To be reported by companies |
| Subcarrier spacing, kHz | 15 for FR1, optional: 120 kHz for FR2 |
| Number of satellite in view | 1 for single satellite case, |
| Orbit inclination | To be reported by companies |
| UE type | Handheld terminal, Optional: VSAT |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration as agreed under AI 9.12.1 |
| Positioning signals (Note 1) | To be reported |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | To be reported |
| RS type of sequence/number of ports | To be reported |
| Number of symbols used per occasion | To be reported |
| number of occasions used per positioning estimate | To be reported |
| Time window for measurement collection | To be reported |
| Interference modelling (ideal muting, or other) | To be reported |
| Reference Signal Transmission Bandwidth | To be reported |
| Reference point for timing measurement | Satellite |
| Description of positioning technique / applied positioning algorithm | To be reported |
| UE speed | 3km/h |
| Maximum timing measurement error | To be reported |
| Performance metrics | Horizontal accuracy (UE 2D position accuracy) |
| Additional notes, if any | Note 1: Time-related measurements can be performed via other downlink and uplink signals than PRS and SRS Note 2: The corresponding link budget should also be reported and the verification procedure should be done within the restriction of minimum elevation angle for service, e.g., 30 degree for LEO |

In this contribution, we provide the evaluations on DL-TDOA based approach, UL-TDoA based approach and Multi-RTT based approach for UE location verification in single-satellite scenario, and provide discussions on network-verified UE location for NR NTN.

UE location verification based on single satellite

In this section, detailed simulation results of horizontal CDF errors are provided under the agreed percentile points for single-satellite based positioning. Simulation assumptions for both DL-TDOA positioning and Multi-RTT positioning are given in Table 7 of Appendix A.1. Parameters related to PRS, SRS, and CSI-RS are configured according to [3].

System level evaluation is used to evaluate the performance of UE location verification, in which UEs are uniformly dropped in an area, where the longitude and latitude span of the area make sure that elevation angles of UEs during the movement of the satellite are within the restriction of minimum elevation angle 30 degree. The distribution of elevation angles of all UEs are shown in Figure 1. It can be observed that the minimum elevation angle among the UEs is 30 degree in this evaluation, therefore, the evaluated area and satellite settings have already covered all the CNR values from the link budget requirement of the agreed evaluation scenario.

Figure 1 Elevation angles of UEs during the movement of satellite.

DL-TDOA and UL-TDOA based UE location verification

In this subsection, time-related measurements in DL-TDOA are performed via PRS under different time intervals, i.e., 2s, 4s, 6s, 8s, and the corresponding parameters are presented in Table 8. Simulation results of the horizontal error obtained via 3 RSTDs (4 measurements) and 4 RSTDs (5 measurements), under different time intervals are shown in Table 1 and Table 2, respectively.

Table 1 Horizontal error of DL-TDOA via PRS with 3 RSTDs (which corresponds to 4 measurement occasions)

|  | Time inverval (s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Horizontal error (km) | 2 | 10.23 | 21.21 | 47.46 | 191.77 | 328.51 |
|  | 4 | 0 | 2.49 | 11.95 | 88.22 | 247.69 |
|  | 6 | 0 | 0 | 1.57 | 16.45 | 122.68 |
|  | 8 | 0 | 0 | 0 | 2.48 | 30.90 |

Table 2 Horizontal error of DL-TDOA via PRS with 4 RSTDs (which corresponds to 5 measurement occasions)

|  | Time inverval (s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Horizontal error (km) | 2 | 4.70 | 11.98 | 32.08 | 161.14 | 319.61 |
|  | 4 | 0 | 0 | 2.48 | 29.32 | 155.86 |
|  | 6 | 0 | 0 | 0 | 2.48 | 31.70 |
|  | 8 | 0 | 0 | 0 | 0 | 4.00 |

For PRS-based DL-TDOA positioning, it is observed that the horizontal error decreases with the increasing of the time interval. As discussed in [4], although a larger time interval causes larger delay for UE location verification, it brings distinguishable measurements such that positioning accuracy can be improved. By comparing Table 1 and Table 2, it is also found that the positioning error decreases with the increasing number of measurements. It should be noticed that for N RSTD measurements, it corresponds to N+1 measurement occasions. Furthermore, with the accuracy requirement of 5-10 km as proposed in [5], DL-TDOA can meet the horizontal position accuracy requirement (< 10km) for 90% of UEs under the time interval of 8s with 3 RSTDs (which corresponds to a latency of 8*(3+1)=32 seconds) or 6s with 4 RSTDs (which corresponds to a latency of 6*(4+1)=30 seconds).

Observation 1: With DL-TDOA positioning, the horizontal error decreases with the increasing of the time interval and the number of measurements.

Observation 2: With DL-TDOA positioning, the positioning accuracy of less than 10km @90% UEs can be achieved under the time interval of 8s with 3 RSTDs (which corresponds to a latency of 32 seconds) or 6s with 4 RSTDs (which corresponds to a latency of 30 seconds).

As for single-satellite based UL-TDOA positioning, it is not directly feasible as TA is autonomously updated by UE via an open loop mechanism, where the value is calculated with GNSS-based UE location and can’t be known by the network, as pointed out in [4]. Even though the pre-compensation of a fixed TA is possible on the UE side to enable the measurements of UL-RTOAs, the remaining TA could exceed the CP length, leading to timing misalignment especially in uplink transmissions.

Observation 3: UL-TDOA positioning is not suitable in verification of UE reported location due to the open-loop TA update on UE, meanwhile even if the pre-compensated TA is fixed on UE in order to enable UL-TDOA based location verification, the remaining TA of UE could exceed the CP length and lead to timing misalignment in uplink transmissions.

Multi-RTT positioning

In this subsection, simulation results of Multi-RTT positioning, based on PRS and SRS, are presented via making use of Rx-Tx measurements to determine RTTs. Specifically, as shown in Figure 2, the UE measures the time difference (Rx-Tx) between the DL reception of PRS and UL transmission of SRS from the same satellite at different time occasions, which corresponds to different locations of the same satellite, while

Figure 2 Visualization of Multi-RTT positioning

PRS

SRS

PRS

PRS

SRS

SRS

the corresponding Tx-Rx time difference at the satellite are also measured by gNB at different time occasions. RTTs are calculated based on UE/gNB Rx-Tx time difference measurements such that the location of UE can be estimated. Parameters of PRS are the same as the previous subsection, while parameters of SRS are provided and can be found in Table 9. Simulation results of the horizontal error obtained via 3 RTTs and 4 RTTs are summarized in Table 3 and Table 4, respectively.

Table 3 Horizontal error of Multi-RTT with 3 RTTs (which corresponds to 3 measurement occasions)

|  | Time inverval (s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Horizontal error (km) | 2 | 0 | 0 | 7.84 | 54.86 | 178.34 |
|  | 4 | 0 | 0 | 0 | 15.69 | 87.68 |
|  | 6 | 0 | 0 | 0 | 5.56 | 47.75 |
|  | 8 | 0 | 0 | 0 | 0 | 27.72 |

Table 4 Horizontal error of Multi-RTT with 4 RTTs (which corresponds to 4 measurement occasions)

|  | Time inverval (s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Horizontal error (km) | 2 | 0 | 0 | 0 | 27.75 | 111.80 |
|  | 4 | 0 | 0 | 0 | 5.56 | 47.70 |
|  | 6 | 0 | 0 | 0 | 0 | 20.00 |
|  | 8 | 0 | 0 | 0 | 0 | 7.84 |

Although Multi-RTT positioning and DL-TDOA positioning follow a similar trend in terms of horizontal error, which gradually decreases with the increasing of time interval, the performance of the Multi-RTT based verification mechanism outperforms the DL-TDOA based solution. The reason is that for two closely located UEs, the difference between their respective DL-RSTDs is usually smaller than that between their respective RTTs such that a large time interval is generally required to bring distinguishable RSTDs to locate an area.

Observation 4: Multi-RTT positioning outperforms DL-TDOA positioning, and its performance improves with increment of time interval between two measurements.

It is also observed that with Multi-RTT based approach, the positioning accuracy of 10 km for 90% of UEs can be achieved under a shorter latency, which is three times of the time interval of 6s to obtain 3 RTT measurements, or four times of the time interval of 4s to obtain 4 RTT measurements

Observation 5: With Multi-RTT positioning, the positioning accuracy of less than 10km @90% UEs can be achieved by 3 RTT measurements with time intervals of 6s (which corresponds to a latency of 18seconds) or 4 RTT measurements with time interval of 4s (which corresponds to a latency of 16seconds).

UE location verification by using CSI-RS

In the previous two subsections, time-related measurements in UE location verification are performed via positioning reference signals, i.e., PRS and SRS, while these measurements can also be performed via existing deployed downlink and uplink signals, e.g. CSI-RS, due to the relaxed requirement of positioning accuracy. In the following, we evaluate the performance of CSI-RS based DL-TDOA positioning, and the corresponding parameters are presented in Table 10. Simulation results of the horizontal error obtained via 3 RSTDs (4 measurements) and 4 RSTDs (5 measurements), under different time intervals are shown in Table 5 and Table 6, respectively.

Table 5 Horizontal error of DL-TDOA via CSI-RS with 3 RSTDs (4 measurements)

|  | Time inverval (s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Horizontal error (km) | 2 | 9.93 | 18.89 | 56.74 | 185.20 | 353.41 |
|  | 4 | 0 | 2.48 | 12.82 | 91.68 | 195.64 |
|  | 6 | 0 | 0 | 0 | 16.62 | 92.66 |
|  | 8 | 0 | 0 | 0 | 1.57 | 13.60 |

Table 6 Horizontal error of DL-TDOA via CSI-RS with 4 RSTDs (5 measurements)

|  | Time inverval (s) | 50% | 67% | 80% | 90% | 95% |
| --- | --- | --- | --- | --- | --- | --- |
| Horizontal error (km) | 2 | 3.13 | 10.23 | 26.00 | 144.14 | 243.75 |
|  | 4 | 0 | 0 | 2.81 | 40.42 | 135.63 |
|  | 6 | 0 | 0 | 0 | 1.57 | 15.27 |
|  | 8 | 0 | 0 | 0 | 0 | 5.54 |

By comparing Table 5 and Table 6 with Table 1 and Table 2, it is observed that the performance of CSI-RS-based DL-TDOA is slightly better than that of PRS-based DL-TDOA. And a similar performance as Table 3 and Table 4 can also be expected when replacing PRS with CSI-RS in Multi-RTT positioning, in which the details are omitted for brevity. This further validates that the time-related measurements can be performed via existing reference signals, e.g., SSB, CSI-RS, TRS, such that the resource overhead due to UE location verification can be avoided/minimized, as well as better power consumption performance considering the reception time at the UE side is not increased if existing signal is used for measurement.

Observation 6: Reusing the existing reference signal (e.g. CSI-RS) for DL-TDOA and multi-RTT can meet the requirement of UE location verification.

Proposal 1: Support reuse the existing reference signal (e.g. CSI-RS) for DL-TDOA and multi-RTT to minimize the resource overhead and UE power consumption due to UE location verification.

Conclusion

In this contribution, simulation results and discussion about network-verified UE location in single-satellite scenario are provided. The observations and proposals are summarized in the following.

Observation 1: With DL-TDOA positioning, the horizontal error decreases with the increasing of the time interval and the number of measurements.

Observation 2: With DL-TDOA positioning, the positioning accuracy of less than 10km @90% UEs can be achieved under the time interval of 8s with 3 RSTDs (which corresponds to a latency of 32 seconds) or 6s with 4 RSTDs (which corresponds to a latency of 30 seconds).

Observation 3: UL-TDOA positioning is not suitable in verification of UE reported location due to the open-loop TA update on UE, meanwhile even if the pre-compensated TA is fixed on UE in order to enable UL-TDOA based location verification, the remaining TA of UE could exceed the CP length and lead to timing misalignment in uplink transmissions.

Observation 4: Multi-RTT positioning outperforms DL-TDOA positioning, and its performance improves with increment of time interval between two measurements.

Observation 5: With Multi-RTT positioning, the positioning accuracy of less than 10km @90% UEs can be achieved by 3 RTT measurements with time intervals of 6s (which corresponds to a latency of 18seconds) or 4 RTT measurements with time interval of 4s (which corresponds to a latency of 16seconds).

Observation 6: Reusing the existing reference signal (e.g. CSI-RS) for DL-TDOA and multi-RTT can meet the requirement of UE location verification.

Proposal 1: Support reuse the existing reference signal (e.g. CSI-RS) for DL-TDOA and multi-RTT to minimize the resource overhead and UE power consumption due to UE location verification.

# References

R1-2207631, FL Summary #4: Network verified UE location for NR NTN.

TR 38.857, Study on NR Positioning Enhancements.

TS 38.211, NR; Physical channels and modulation.

R1-2205859, Discussion on network-verified UE location for NR NTN.

TS 38.882, Study on requirements and use cases for network verified UE location for Non-Terrestrial-Networks (NTN) in NR.

RP-221444, Discussion on network-verified UE location for NR NTN.

- Appendix A: Evaluation Results

- A.1 Common parameters

Table 7 Common simulation assumption

| Parameter | Description/Value |
| --- | --- |
| Scenarios | Rural, LOS |
| Satellite Orbit | 600km |
| Satellite parameters | Reuse Set-1satellite parameters as in table 6.1.1.1-1/2 of TR38.821 |
| Channel model/ Delay spread | Based on section 6.7.2 of TR 38.811 |
| FR/Carrier frequency | FR1: 2GHz, S-band (n256). |
| BW | DL: 4.5MHz, UL: 2.88 MHz |
| Subcarrier spacing, kHz | 15 for FR1 |
| Number of satellite in view | 1 for single satellite case, |
| Orbit inclination | 90 |
| Satellite location at t=0 | 0°N, 0°E |
| UE type | Handheld terminal |
| UE related parameters | Handheld UE characteristics as in Table 6.1.1.1-3 of TR38.821 with update of polarization, Tx/Rx antenna gain, and antenna type and configuration as agreed under AI 9.12.1 |
| Interference modelling (ideal muting, or other) | Ideal muting |
| Delay between symbol boundary and first path | DL: Uniform in [0,500]ns |
| Network synchronization | Truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1. T1 = 50ns [2]. |
| UE/gNB Tx/Rx calibration error | Truncated Gaussian distribution with zero mean and standard deviation of T1 ns, with truncation of the distribution to the [-T2, T2] range, and with T2=2*T1. gNB Rx/Tx Time error T1 = 1.4ns UE Rx/Tx Time error T1 = 5.6ns |
| Reference point for timing measurement | Satellite |
| UE speed | 3km/h |
| Performance metrics | Horizontal accuracy (UE 2D position accuracy) |

- A.2 Simulations for DL-TDOA positioning

Table 8 PRS simulation assumption

| Parameter | Description/Value |
| --- | --- |
| Positioning signals | PRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | DL: Comb-2 |
| RS type of sequence/number of ports | Gold / 1 |
| Number of symbols used per occasion | 6 |
| Number of occasions used per positioning estimate | 1 |
| Reference Signal Transmission Bandwidth | 1.8 MHz |
| PRS configuration | PRS positions defined in Table 7.4.1.7.3-1 with lstartPRS=3 and koffsetPRS=2 in [3] |

Figure 3 CDF of horizontal error for DL-TDOA via PRS with 3 RSTDs.

Figure 4 CDF of horizontal error for DL-TDOA via PRS with 4 RSTDs.

- A.3 Simulations for Multi-RTT positioning

Table 9 SRS simulation assumption

| Parameter | Description/Value |
| --- | --- |
| Positioning signals (Note 1) | SRS |
| Reference Signal Physical Structure and Resource Allocation (RE pattern) | UL: Comb-2 |
| RS type of sequence/number of ports | ZC / 1 |
| Number of symbols used per occasion | 1 |
| number of occasions used per positioning estimate | 1 |
| Reference Signal Transmission Bandwidth | 2.88 MHz |
| SRS configuration | SRS positions defined in Table 6.4.1.4.3-1 and Table 6.4.1.1.3-2 with CSRS=3, BSRS=0, loffset=0, PF=1, and k0=0 in [3] |

Figure 5 CDF of horizontal error for Multi-RTT with 3 RTTs.

Figure 6 CDF of horizontal error for Multi-RTT with 4 RTTs.

- A.4 Simulations for DL-TDOA positioning based on CSI-RS

Table 10 CSI-RS simulation assumption

| Parameter | Description/Value |
| --- | --- |
| Positioning signals | CSI-RS |
| RS type of sequence/number of ports | Gold / 1 |
| Number of symbols used per occasion | 1 |
| Number of occasions used per positioning estimate | 1 |
| Reference Signal Transmission Bandwidth | 4.5 MHz |
| CSI-RS configuration | CSI-RS positions defined in Table 7.4.1.5.3-1 and Table 7.4.1.5.3-2 with k0=0, l0=9, and ρ=3 in [3] |

Figure 7 CDF of horizontal error for DL-TDOA via CSI-RS with 3 RSTDs.

Figure 8 CDF of horizontal error for DL-DOA via CSI-RS with 4 RSTDs.