3GPP TSG RAN WG1 #110bis-e	R1-2208484

e-Meeting, October 10th – 19th, 2022

Title: 	Discussion of subband non-overlapping full duplex

Source: 	ZTE

Agenda item:	9.3.2

Document for:	Discussion/Decision

Introduction

In RAN#96 meeting, a revised SID on NR duplex evolution has been endorsed. In this contribution, we provide our analysis for subband non-overlapping full duplex for possible solutions, feasibility and impact to legacy operation assuming co-existence in co-channel and adjacent channels.

Overview

Rel-18 subband full duplex SI is the first study item to study full duplex comprehensively. Before digging into the detailed solutions/schemes for subband full duplex, it is beneficial to have some preliminary simulation results to guide and focus the subband full duplex system design. For example, it is necessary to identify which interference is the main challenge. Thus, the subband full duplex simulation should be prioritized.

Proposal 1: Prioritize the subband full duplex simulation and use the simulation results to guide and focus the solution/scheme discussion.

As discussed in our companion contribution R1-2208483, there are at least the following 6 challenges in the conventional TDD operation. Subband full duplex design should take the following challenges of conventional TDD operation into account and try to address the following challenges via full duplex evolution.

- Challenge ①: Ensuring UL throughput + UL coverage simultaneously.

- Challenge ②: Ensuring UL throughput + DL&UL Latency simultaneously.

- Challenge ③: Ensuring UL coverage + DL&UL Latency simultaneously.

- Challenge ④: Ensuring DL throughput + DL&UL Latency simultaneously.

- Challenge ⑤: Ensuring DL throughput + UL throughput simultaneously.

- Challenge ⑥: Ensuring DL throughput + UL coverage simultaneously.

Proposal 2: The solution/scheme of duplex evolution takes the following challenges of conventional TDD operation into account.

- Challenge ①: Ensuring UL throughput + UL coverage simultaneously.

- Challenge ②: Ensuring UL throughput + DL&UL Latency simultaneously.

- Challenge ③: Ensuring UL coverage + DL&UL Latency simultaneously.

- Challenge ④: Ensuring DL throughput + DL&UL Latency simultaneously.

- Challenge ⑤: Ensuring DL throughput + UL throughput simultaneously.

- Challenge ⑥: Ensuring DL throughput + UL coverage simultaneously.

Overall, mainly two aspects need thorough study and analysis for the solution of subband full duplex. One is DL&UL time/frequency resource split and another is interference management/cancellation. In the following sections, we will focus on these two aspects.

Basic framework of SBFD configuration

In Section 3.4, we discussed different schemes for SBFD configuration, i.e., subband-based scheme, BWP-based scheme and CA-based scheme. For simplicity, we use “subband” as example to clarify the detailed mechanism of SBFD in this section. However, these mechanisms can also be applied to BWP-based scheme and CA-based scheme whenever possible.

In RAN1#110 meeting, the following agreement was reached. In the following, we provide detailed analysis for each alternative.

Agreement Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state. - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. UE capability discussion is held in work item phase.

Alt.1: Transparent method with no specification impact

For Alt.1, the UL subband is not configured for the UE. The UE doesn’t get the time/frequency position of the UL subband. Furthermore, UE operates according to the legacy Rel-15/16/17 specification without any new UE behaviors. At least the following two options can realize this kind of transparent method.

- Option.1: Base station configures flexible symbols/slots to the UE via TDD-UL-DL-ConfigCommon and schedules DL/UL on these flexible symbols/slots according to existing rules. Base station ensures that DL transmission and UL transmission are not overlapped in frequency domain within the BWP.

- Option.2: Based station configures UE-specific TDD slot formats for different UEs via TDD-UL-DL-ConfigDedicated and schedules DL/UL based on the UE-specific TDD slot formats and existing rules. Take the following Figure 1 as an example in a cell. Cell-specific slot format configuration is DXXXU. And network configures UE-specific slot format DDDSU for some UEs and configures UE-specific slot format DSUUU for other UEs. In Slot#1, Slot#2 and Slot#3, network schedules DL in the upper half of frequency resource and UL in the lower half of frequency resource respectively for different UEs.

Figure 1: An example of implementation-based solution for subband full duplex

Transparent method with no specification impact can allow gNB/UE to support SBFD via implementation. However, it highly depends on the usage of flexible symbols.

Observation 1: Alt.1 (transparent method with no specification impact for SBFD) has the following two options:

- Option.1: Base station configures flexible symbols/slots to the UE via TDD-UL-DL-ConfigCommon and schedules DL/UL on these flexible symbols/slots according to existing rules.

- Option.2: Based station configures UE-specific TDD slot formats for different UEs via TDD-UL-DL-ConfigDedicated and schedules DL/UL based on UE-specific TDD slot formats according to existing rules.

Alt.2: Transparent method without subband configuration

For Alt.2, UE is not aware of the subband configuration, i.e., UE is not aware of the time/frequency resource configuration for the subband. However, some other spec impacts may be needed for the UE to support SBFD operation. From our perspective, at least the following options can be considered for transparent method without subband configuration.

- Option.1: Allocating some time/frequency resources in DL/flexible symbols for UL transmission

- The UL subband is not configured for the UE. Base station allocates some fixed time/frequency resources (which is an UL subband from the base station's perspective) in the DL symbols (or DL symbols plus flexible symbols) via implementation. For dynamic UL transmission (such as DG PUSCH, dynamic PUCCH and PRACH triggered by PDCCH order, etc.), UE transmits UL transmission in the DL symbols according to gNB’s scheduling and does not detect collisions between the transmission direction and the resource allocation. In addition, semi-static UL transmission (e.g., CG PUSCH, semi-static PUCCH and PRACH, etc.) cannot be supported in the fixed time-frequency resource of DL symbols according to existing rules because UE is not aware of the subband configuration. For this alternative, specification change is required to allow UE to transmit UL transmission in DL symbols, especially more specification change to support semi-static UL transmission.

- Option.2: New TDD slot format for SBFD overrides the legacy one

- The UL subband is not configured for the UE. Legacy UE follows the TDD slot format indicated by the legacy SIB or legacy UE-specific signalling. A new UE-specific TDD slot format is introduced for the SBFD UE. This new UE-specific TDD slot format indication/configuration for SBFD UE can override the legacy TDD slot format indication/configuration. For example, legacy TDD slot format is DDDSU. Base station indicates DSUUU for one SBFD UE, thus SBFD UE can transmit dynamic/semi-static UL transmission in the time/frequency resources (which is a UL subband from the base station's perspective) in its “UL symbol/slot” (which is the DL symbol/slot configured by the legacy signalling) according to gNB’s scheduling/configuring. Another example is, base station indicates DXXXU for SBFD UE and SBFD UE can transmit UL or receive DL on these “X” symbol/slots based on gNB’s scheduling. For this alternative, both semi-static and dynamic UL transmissions can be easily supported based on the new UE-specific TDD slot format.

Alt.2 can allow gNB/UE to support SBFD without configuring SBFD resources. Compared to the UL subband being configured for the UE, these methods have less specification impacts. However, similarly, without indicating the time/frequency resource of subband, UE is not possible to perform RF/digital subband filter for transmissions within the subband. It also potentially impacts other aspects. For example, for PDSCH/PUSCH, potential impacts include non-contiguous FDRA across DL subbands, assignment of fractional RBGs at DL subband boundaries for RA type 0 and PDSCH slot aggregation across SBFD slots and non-SBFD slots . For PUCCH, potential impacts include FDRA and FH due to different available frequency resources in SBFD symbols and non-SBFD symbols. For CSI-RS, potential impacts include CSI report for fractional CSI report subband. Indicating the time/frequency resource of subband can help to increase the spectrum efficiency in terms of resource allocation.

Observation 2: Alt.2 (Transparent method without subband configuration for SBFD) has the following two options:

- Option.1: Allocating some time/frequency resources in DL/flexible symbols for UL transmission

- Base station allocates some fixed time/frequency resources in the DL symbols (or DL symbols plus flexible symbols) via implementation. UE transmits UL transmission in the fixed time/frequency resources of the DL symbols according to gNB’s scheduling.

- Option.2: New TDD slot format for SBFD overrides the legacy one

- Legacy UE follows the TDD slot format indicated by the legacy SIB or legacy UE-specific signalling. A new UE-specific TDD slot format is introduced for the SBFD UE, which can override the legacy TDD slot format indication/configuration. UE transmits UL transmission based on new UE-specific TDD slot format according to gNB’s scheduling/configuring.

Alt.3: Indication of time location of subband

For Alt.3, only the time location of subbands for SBFD operation is known to SBFD aware UEs. From system perspective, the frequency location of subband for SBFD operation is more important aspect. Without the frequency location of subband, UE is not possible to perform RF/digital subband filter for transmissions within the subband. As also discussed in section 3.2, indicating the time/frequency resource of subband can help to increase the spectrum efficiency in terms of resource allocation. Compared with Alt.2, Alt.3 has higher spec impacts but without adding much value via indicating time location of subband.

Observation 3: Compared with Alt.2, Alt.3 has higher spec impacts but without adding much value via indicating time location of subband only.

Alt.4: Non-transparent method

A non-transparent method means that the UL subband is configured for the UE. In this way, the UE knows the time-frequency domain location of the UL subband. Although the non-transparent UL subband has more standardization work, it can bring more benefits, for example, it is beneficial for UE to filter interference in reception. Also, it allows UE to support semi-static UL transmission, UL random access, etc in subband. There are different schemes to configure “subband” for UE, which will be elaborated in this section.

In RAN1#110 meeting, the following agreement was reached. Subband based scheme (i.e., Scheme#1 as shown below) is prioritized. In this section, we provide detailed analysis for each potential scheme.

Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Scheme#1: One/multiple subbands within one BWP

One possible approach is to configure a (or multiple) subband for subband non-overlapping full duplex. The uplink subband can be configured on uplink symbols and/or flexible symbols. Considering that the flexible symbols are not commonly configured in the actual TDD system, if the uplink subband only contains the uplink symbols, it would be difficult to obtain continuous uplink resources in a slot or in a TDD period. Thus, the downlink symbols in the slot (or downlink slots) should be allowed to be configured for an uplink subband to ensure more continuous uplink transmissions in the uplink subband, which is more beneficial to improve uplink coverage and reduce latency. Of course, the size of an uplink subband in the frequency domain can also be flexibly configured to balance DL and UL transmission. As an example, shown in Figure 2, an uplink subband is configured in a slot across both flexible and downlink symbols.

Figure 2: One/multiple subbands within one BWP

UE should consider all the resource in the uplink subband available for uplink transmission even if the uplink subband is overlapped with downlink or flexible symbols. Meanwhile, if the uplink subband is configured for the UE, the UE does not expect the uplink dynamic scheduling and the downlink dynamic scheduling to overlap in the time domain.

In order to better coordinate/manage the interference, it is preferred to have the same uplink subband configuration for all full duplex UEs in the same cell and have the same uplink subband configuration for all the adjacent base stations.

Proposal 3: RAN1 studies the following solution for subband full duplex: configure one/multiple subbands in one BWP.

- All the resource in the uplink subband are available for uplink transmission even if the uplink subband is overlapped with downlink or flexible symbols.

Scheme#2: BWP based SBFD

Starting from NR Rel-15, the framework of BWP is introduced. As the frequency domain range of data transmission, each BWP corresponds to a specific SCS and CP type, and is configured with a set of transmission parameters, such as PDCCH-config, PDSCH-config for DL BWP, PUCCH-config and PUSCH-config for UL BWP, etc. Similarly, BWP can be reused to implement subband full duplex. For example, each subband is defined as one BWP and different frame structures can be configured for different BWPs. In this way, the existing working framework of NR can be used.

Figure 3: BWP based subband full duplex

Take Figure 3 as an example, three BWP pairs are configured, i.e., each BWP pair contains one DL BWP and one UL BWP. BWP1 and BWP3 are the BWP pairs at the two edges of the carrier, and BWP2 is the BWP pair located in the middle. BWP switching delay may be an issue to be considered for crossing operation between different BWP pairs (i.e., across subbands). For that, a UE capability on supporting dual active BWP pair can be defined. For the UE with the dual active BWP pair capability, although it cannot work on both BWP pairs with different traffic directions at the same time, the UE can switch between different active BWP pairs with tolerable or even without delay. Thus, the data transmission/feedback delay can be effectively reduced, which is a major objective of subband full duplex. For example, if UE is activated on BWP1 and BWP2 simultaneously, then network can determine which BWP is used for transmission based on the real time traffic.

Proposal 4: RAN1 studies the following solution for subband full duplex: support dual active BWP pairs, where each BWP pair includes one DL BWP and one UL BWP.

- Different BWP pairs can be configured with different TDD slot configurations.

Scheme#3: Half duplex CA based SBFD

In our understanding, the most important advantages of SBFD (sub-band full duplex) are coverage (especially UL coverage) boosting and latency reduction due to more UL time domain resources and accessibility to UL and DL resources without too much waiting time caused by TDD frame structure. One simple way to support SBFD and achieve its advantages is to regard different ‘subband’ as different cells. And of course, R18 UE will not work in two cells at the same time in the occasion when the two cells have different link direction, i.e. UE will not transmit and receive in two cells respectively at the same time. So we call this method as half duplex CA based SBFD or HDCA-SBFD hereafter.

Figure 4: Half duplex CA based SBFD

As shown in Figure 4, in HDCA-SBFD, gNB can work on cell1 and cell2 simultaneously or cell1 and cell3 simultaneously. Legacy UE can be configured with either one cell or two cells when they share the same D/U configuration. UE capable of HDCA-SBFD can be configured with two cells (cell1+cell2 or cell2+cell3) with different D/U configuration to enjoy the benefits of SBFD.

Half duplex CA (HDCA) is specified in R16, which enables half duplex UE (i.e. UE can only either transmit or receive in a certain time occasion) to be configured with two or more cells with different D/U configurations. Meanwhile, UE behaviors are specified in some cases when directional conflict among different cells occurs as shown in Figure 5.

Figure 5: Legacy conflict handling for half duplex CA

We can see that HDCA and HDCA-SBFD share the same characteristics, i.e.

- UE can only either transmit or receive in a certain occasion;

- two or more cells with different D/U configurations can be configured for UE;

- directional conflict in different cells needs to be tackled.

With the consideration of HDCA already specified in NR R16, HDCA-SBFD has almost been supported by current specification if ‘sub-band’ is regarded as ‘cell’. The only thing we need to do is to lift the restriction of applying directional collision handling mechanism in intra-band case (i.e. removing ‘error case in intra-band’ in Figure 5). Meanwhile we can further discuss if some enhancements to directional collision handling mechanism is needed to better satisfy the requirement of R18 if time allows.

Proposal 5: RAN1 studies half-duplex CA based scheme for sub-band full duplex and taking directional conflict handling mechanism in R16 half-duplex CA as a starting point.

- Further study the necessity of enhancement to directional conflict handling mechanism if time allows in R18.

Summary

Based on the above analysis, transparent method with no specification impact (Alt.1) and transparent method without subband configuration (Alt.2) have some potential drawbacks, which may reduce the SBFD performance, i.e., without indicating the time/frequency resource of subband, UE is not possible to perform RF/digital subband filter for transmissions within the subband. Thus, it is proposed to focus on non-transparent method of SBFD configuration (Alt.4). In addition to Alt.4, Alt.2 can also be studied as second priority considering its minor specification impacts. However, Alt.3 has higher spec impacts but without adding much value via indicating time location of subband only. Thus, from our perspective, we propose to further study Alt.4 and Alt.2. The study of Alt.3 can be postponed or deprioritized.

Proposal 6: Regarding the 4 SBFD operation Alts reached in RAN1#110 meeting, focus the further study in Alt.4 and Alt.2.

- Note: The study of Alt.3 can be postponed or deprioritized.

Details of SBFD framework

In Section 3, we discussed different schemes for SBFD configuration, i.e., subband-based scheme, BWP-based scheme and CA-based scheme. For simplicity, we use “subband” as example to clarify the detailed mechanism of SBFD in this section. However, these mechanisms can also be applied to BWP-based scheme and CA-based scheme whenever applicable.

Time/frequency-domain configuration of subband

In RAN1#110 meeting, the following agreement has been reached. Semi-static subband configuration of the time/frequency-domain location is considered as the baseline.

Agreement For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline.

During the discussion, companies discussed the possibilities of dynamic indication of subband time/frequency location. However, the frequency domain location of the subband should be configured by RRC signaling. From network perspective, the subband bandwidth should not be allowed to be adjusted dynamically because network may need analog filter to mitigate the self-interference. The analog filter is not able to change its filter bandwidth flexibly.

On the other hand, it is worth studying dynamic update of the subband time-domain configuration as long as the subband frequency domain location is fixed. One of the main benefits of allowing dynamic update of the subband time-domain configuration is to adjust to the traffic load. It can also balance sub-band and non sub-band resources to avoid resource shortage or waste. In case of sudden burst of UL traffic, network can indicate more slot/symbols as UL subband. For example, UL subband is configured from slot 2 to slot 4. In case of sudden burst of UL traffic, network can indicate UE to extend the UL subband to slot 1. Two options can be considered for indicating the dynamic change of subband time-domain location:

- Opt.1: Separate indication without scheduling data, e.g., group-common DCI

- Opt.2: DL/UL scheduling command, e.g., DCI scheduling PUSCH, DCI triggering PUCCH. If more UL resources are needed for the PUSCH/PUCCH, the UL subband can be extended if the interference is controllable.

Proposal 8: Regarding the time/frequency-domain configuration of subband,

- The location and bandwidth in frequency domain of subband can NOT be updated dynamically.

- Further study the feasibility and benefits of dynamic update of subband time-domain location.

Compatibility of Legacy UE

In order to better coexist with legacy UE as well as protect some important signals (e.g., SSB) from full duplex, the configuration of sub-band full duplex mentioned above can be limited to a specific time domain range, e.g., defining time domain window for full duplex.

Figure 6: Time domain window for full duplex

As an example shown in Figure 6, the time domain window can be configured via RRC parameters, such as, period, offset and length. More specifically, time domain window with 3 slots length will occur with a periodicity of 10 slots. And the starting point is the second slot within each window, which is determined according to the configuration of offset = 1. For a frame structure configuration with 5 slots periodicity, there will be one time-domain window for every two periods of frame structure. Then, the subband full duplex specific configuration can be applied within each time domain window.

For legacy UEs without supporting subband full duplex related features, network can schedule them outside the window for better compatibility. Similarly, resource for some important signals (e.g., SSB) which have been well designed in legacy procedures should also be considered during the time domain window configuration, for which the minimum impact can be expected.

Proposal 9: RAN1 studies the following solution for subband full duplex:

- Time domain window can be defined for subband full duplex operation for better compatibility with less impact on legacy UE and procedures.

UE collision handling between DL and UL

In the RAN1 #109-e meeting, the UE collision handling between DL and UL was discussed, but no agreement was reached due to the divergence of company views.

From our point of view, if the subband is configured, it will lead to new conflict scenarios because the direction of the subband and the direction of the resource based on the legacy configuration can be different. However, the collision handling is highly associated with the subband configuration, i.e., whether the subband is transparent to the UE or not.

- If the subband is transparent to the UE, then the collision handling between DL and UL should be avoided according to the scheduling of the base station, e.g., the UE always performs corresponding reception or transmission according to the scheduling of the base station, and the UE does not need to consider collision handling. This method is suitable for dynamic transmission. For semi-static transmission, the collision handling between DL and UL still needs further discussion. In this way, the complexity of the UE is reduced, and the complexity of the base station is increased.

- If the subband is non-transparent to the UE, then collision handling between DL and UL should be studied. Here, it is necessary to list various potential conflict scenarios, and formulate corresponding conflict solutions in combination with specific signals/channels and priorities.

Since Rel-16, URLLC has introduced L1 priority indication for DL and UL transmission. In case of collision between DL and UL with different priorities in subband, the L1 priority indication can be reused as much as possible to reduce the specification impact. For example, if both DL transmission and UL transmission are scheduled/configured in the UL subband, the transmission with higher priority will be prioritized. In case of collision between DL and UL with same priority in UL subband, DL transmission is always survived because DL transmission is scheduled in the UL subband only in case of emergency.

Alternatively, UL subband can be configured with one priority. In this way, if the priority of the DL transmission is higher than or equal to the priority of the UL subband, then the DL transmission can be allowed to be scheduled in the UL subband; otherwise, the DL transmission is prohibited in the UL subband.

Proposal 10: Study potential enhancements for collision handling between UL and DL for SBFD aware UE.

Transmission direction

In RAN1#110 meeting, the following agreement was reached. Among all the four options, Option1 is the basic one, which should be supported without any doubt.

Agreement: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

In addition to Option1, Option2 can also be considered for the following two reasons.

- In case of low UL traffic load, DL transmission can be scheduled in the UL subband to increase the spectrum efficiency;

- The victim UE can receive DL in the UL subband and identify the potential aggressor UE.

If Option2 is adopted, then the motivation of introducing Option3 is not strong since allowing DL transmission in UL subband can already serve the same purpose of allowing UL transmission outside the UL subband.

Another issue is whether to allow all the UL subband resource for DL transmission. Theoretically, allowing all the UL subband resource for DL transmission can maximize the spectrum efficiency. However, it will also incur large and sometimes uncontrollable cross-link interferences. To allow this flexibility and mitigate the potential CLI, one way is to configure a subset of subband area and the DL transmission is only allowed in this specific area within the UL subband.

In addition, the previous agreement is mainly from UE perspective. The same analysis can also be applied to gNB. For example, to assist the gNB-gNB CLI measurement, the aggressor gNB may also transmit reference signal in the UL subband. Then the victim gNB can receive the reference in the UL subband and identify the potential aggressor gNBs.

Proposal 11: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, prioritize the study of following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

FFS: Whether DL transmission is allowed in all the UL subband resources or only a certain area.

FFS: gNB transmits DL reference signal in UL subband.

Resource allocation/configuration

In the RAN1 #109-e meeting, the resource allocation in symbols with subbands was discussed, but no agreement was reached due to the divergence of company views.

Based on our understanding, the resource allocation needs to be studied. For example, due to UL subband within DL symbols, DL resource allocation for both physical channels, e.g. PDSCH and PDCCH and physical signal e.g. CSI-RS may be impacted and potential enhancements may be studied. Multi-slot PUSCH, e.g., PUSCH repetition Type A, and PUCCH repetitions may also be impacted and potential enhancements may also be studied.

In RAN1#110 meeting, companies had some preliminary discussion on PUCCH resource configuration. Basically, there are two different options:

- PUCCH resources are separately configured for one UL subband;

- The current PUCCH resource based on UL BWP configuration can be shared with one UL subband. If a PUCCH resource based on UL BWP is located within the resources of the UL subband, then the PUCCH resource can be scheduled for the UL subband.

Similar to PUCCH resource configuration, PUSCH resource configuration should also be studied for UL subbands. For example, PUSCH resources are separately configured for one UL subband, or PUSCH resources configured based on ULBWP are shared with one UL subband.

Proposal 12: RAN1 further studies the resource allocation of relevant physical channels/signals in frequency domain and time domain for SBFD.

- E.g., whether to have the same or separate UL configurations for PUCCH/PUSCH/SRS for UL subband and UL slots.

Initial access

In the RAN1 #109-e meeting, the initial access in UL subband was discussed, but no agreement was reached due to the divergence of company views. In our opinion, if initial access is supported in the UL subband, it is obviously beneficial to improve the initial access capability from at least two aspects, such as shortening the initial access time and providing more initial access opportunities. Some further studies are necessary on this aspect.

Proposal 13: RAN1 further studies the potential enhancements for initial access in the UL subband.

CLI handling

During RAN1#109-e meeting, discussion scope division among different agenda items were discussed [2]. More specifically, the potential schemes for gNB-to-gNB and UE-to-UE CLI handling and gNB self-interference handling that are specific for SBFD will be discussed under AI 9.3.2.

In RAN1#110, the following agreement about CLI handling specific for SBFD was obtained [3]:

Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

In this section, we provide our further analysis on CLI handling schemes that are specific for SBFD.

TA offset determination for avoiding inter-slot interference

In legacy TDD system, UE is provided a TA offset, i.e.,  for a serving cell to reserve enough time for UL/DL switching. Further, it is typically configured with a value larger than zero in current TDD network, e.g., 25600 or 39936.

For operating SBFD shown in Figure 7, if a non-zero TA offset is also configured for UL transmission within the UL subband, inter-symbol and inter-slot interference will be generated as the symbols in DL subband and UL suband are not aligned. One way for avoiding this interference is to set  for all UL transmission no matter it is located with the UL subband or UL slot. However, potential backward compatibility problem exists, that is, legacy UE may not support to set  to zero under the TDD system.

Another way is to define two TA offset values for a SBFD capable UE for UL transmission in UL subband and UL slot, respectively. More specifically, one is zero for UL transmission in UL subband and another value is larger than zero for UL transmission in UL slot. Then, UL transmission of the legacy UE can also be scheduled or configured within UL slot. For SBFD capable UE, it should support two different TA offset values for UL subband and UL slot.

Figure 7: Inter-slot interference for SBFD

Proposal 14: For SBFD, different options of TA offset determination for avoiding inter-slot interference should be studied,

- Option 1: set  for all UL resource;

- Option 2: define two values of  for UL transmission in UL subband and UL slot, respectively.

- for UL subband

- for UL slot

Power control based solution

DL transmissions need to be filtered before transmitting over the air so that power can be restricted within the desired frequency range. Ideally, after filtering, there should be no leakage signal out of the desired frequency range. However, due to the limitation of technology and implementation complexity, there are always leakage signals out of the desired frequency range. For SBFD, the transmission power of DL transmission within DL suband will leak to UL subband, which is decreasing with the increase of frequency distance. The uplink transmissions in the UL subband are subject to different interference levels, depending on the frequency domain isolation between it and the DL subband. As an example shown in Figure 8, the UL transmission located within frequency area 1, area 2 and area 3 will suffer inter-subband interference with highest, medium and lowest level, respectively.

Figure 8: Inter-subband interference with different levels on different frequency areas

On the other hand, the UL transmission in different time domain areas may also suffer different inter-subband interference. As an example shown in Figure 11, PUSCH1 will not be interfered by inter-subband interference as there is no DL transmission in DL subband during the transmission of PUSCH1. While for PUSCH2, it will be interfered by PDSCH2 as they are overlapping in time domain.

Figure 11: Inter-subband interference with different levels on different time domain areas

Observation 4: The uplink transmissions in the UL subband are subject to different interference levels due to the following aspects,

- the frequency domain isolation between it and the DL subband.

- time domain areas where DL transmissions are scheduled or configured in DL subband.

Different interference levels require different power control parameters for improving the performance of UL transmission. One way is to divide resources with different interference levels into multiple areas and make each area mapped with a dedicated power control parameter set. The resources contained in each area can be indicated by DCI. DCI format is similar to UL cancellation, where multiple resource indication fields correspond to multiple resource sets one by one respectively. The power control parameters set can be either open-loop power control parameter (e.g., P0, alpha) or closed-loop power control parameter (e.g., TPC table). The transmission power of an UL transmission will be determined according to the power control parameters set corresponding the area where the UL transmission is located.

Proposal 15: UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the inter-subband interference with different levels.

- The resources contained in each area can be indicated by DCI.

UE-to-UE CLI measurement and reporting

L1-based UE-to-UE CLI measurement and reporting is a potential enhancement direction, which is common for both SBFD and dynamic/flexible TDD. So according to the guideline on discussion scope division, the common part is to be discussed in AI 9.3.3.

Regarding SBFD, considering that inter-subband interference has a non-uniform feature, wideband CLI measurement and reporting may fail to reflect the changes of interference in different frequency resources. Therefore, it is necessary to introduce a finer frequency domain measurement granularity, for example, subband CLI measurement and reporting.

Observation 5: Wideband CLI measurement and reporting may fail to reflect the changes of inter-subband interference in different frequency resources.

For supporting subband CLI measurement and reporting, the current mechanism of subband CQI/PMI measurement and reporting can be considered as a starting point. A further research direction is impact of introduction of the UL subband on existing mechanism, for example, configuration and determination of the measurement subband size, and measurement reporting overheads reduction, etc.

Proposal 16: Further study subband CLI measurement and reporting for UE-to-UE CLI handling under SBFD, e.g., configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc.

Reference signals with different frequency densities

Reference signals may be needed to measure the interference levels between gNBs or between UEs. According to the analysis in Section 5.2, different areas with different frequency isolation from the aggressor lead to different interference levels. To save reference signal overhead, reference signals can be configured with different frequency densities in different areas. For example, as shown in Figure 11, the subband is divided into three areas. The Area1 is closest to the DL in frequency domain, thus the reference signal in the first part can be configured with higher frequency density. The Area3 is far away from the DL in the frequency domain, the interference level may be low in this area. Thus, the reference signal in this area can be configured with lower frequency density

Proposal 17: Different frequency densities can be configured for reference signals transmitted in different areas with different interference levels.

Conclusion

In this contribution, we provide our analysis for subband non-overlapping full duplex for possible solutions with the following proposals and observations.

Overview

Proposal 1: Prioritize the subband full duplex simulation and use the simulation results to guide and focus the solution/scheme discussion.

Proposal 2: The solution/scheme of duplex evolution takes the following challenges of conventional TDD operation into account.

- Challenge ①: Ensuring UL throughput + UL coverage simultaneously.

- Challenge ②: Ensuring UL throughput + DL&UL Latency simultaneously.

- Challenge ③: Ensuring UL coverage + DL&UL Latency simultaneously.

- Challenge ④: Ensuring DL throughput + DL&UL Latency simultaneously.

- Challenge ⑤: Ensuring DL throughput + UL throughput simultaneously.

- Challenge ⑥: Ensuring DL throughput + UL coverage simultaneously.

Basic framework of SBFD configuration

Observation 1: Alt.1 (transparent method with no specification impact for SBFD) has the following two options:

- Option.1: Base station configures flexible symbols/slots to the UE via TDD-UL-DL-ConfigCommon and schedules DL/UL on these flexible symbols/slots according to existing rules.

- Option.2: Based station configures UE-specific TDD slot formats for different UEs via TDD-UL-DL-ConfigDedicated and schedules DL/UL based on UE-specific TDD slot formats according to existing rules.

Observation 2: Alt.2 (Transparent method without subband configuration for SBFD) has the following two options:

- Option.1: Allocating some time/frequency resources in DL/flexible symbols for UL transmission

- Base station allocates some fixed time/frequency resources in the DL symbols (or DL symbols plus flexible symbols) via implementation. UE transmits UL transmission in the fixed time/frequency resources of the DL symbols according to gNB’s scheduling.

- Option.2: New TDD slot format for SBFD overrides the legacy one

- Legacy UE follows the TDD slot format indicated by the legacy SIB or legacy UE-specific signalling. A new UE-specific TDD slot format is introduced for the SBFD UE, which can override the legacy TDD slot format indication/configuration. UE transmits UL transmission based on new UE-specific TDD slot format according to gNB’s scheduling/configuring.

Observation 3: Compared with Alt.2, Alt.3 has higher spec impacts but without adding much value via indicating time location of subband only.

Proposal 3: RAN1 studies the following solution for subband full duplex: configure one/multiple subbands in one BWP.

- All the resource in the uplink subband are available for uplink transmission even if the uplink subband is overlapped with downlink or flexible symbols.

Proposal 4: RAN1 studies the following solution for subband full duplex: support dual active BWP pairs, where each BWP pair includes one DL BWP and one UL BWP.

- Different BWP pairs can be configured with different TDD slot configurations.

Proposal 5: RAN1 studies half-duplex CA based scheme for sub-band full duplex and taking directional conflict handling mechanism in R16 half-duplex CA as a starting point.

- Further study the necessity of enhancement to directional conflict handling mechanism if time allows in R18.

Proposal 6: Regarding the 4 SBFD operation Alts reached in RAN1#110 meeting, focus the further study in Alt.4 and Alt.2.

- Note: The study of Alt.3 can be postponed or deprioritized.

Details of SBFD framework

Proposal 8: Regarding the time/frequency-domain configuration of subband,

- The location and bandwidth in frequency domain of subband can NOT be updated dynamically.

- Further study the feasibility and benefits of dynamic update of subband time-domain location.

Proposal 9: RAN1 studies the following solution for subband full duplex:

- Time domain window can be defined for subband full duplex operation for better compatibility with less impact on legacy UE and procedures.

Proposal 10: Study potential enhancements for collision handling between UL and DL for SBFD aware UE.

Proposal 11: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, prioritize the study of following options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

FFS: Whether DL transmission is allowed in all the UL subband resources or only a certain area.

FFS: gNB transmits DL reference signal in UL subband.

Proposal 12: RAN1 further studies the resource allocation of relevant physical channels/signals in frequency domain and time domain for SBFD.

- E.g., whether to have the same or separate UL configurations for PUCCH/PUSCH/SRS for UL subband and UL slots.

Proposal 13: RAN1 further studies the potential enhancements for initial access in the UL subband.

CLI management and cancellation

Proposal 14: For SBFD, different options of TA offset determination for avoiding inter-slot interference should be studied,

- Option 1: set  for all UL resource;

- Option 2: define two values of  for UL transmission in UL subband and UL slot, respectively.

- for UL subband

- for UL slot

Observation 4: The uplink transmissions in the UL subband are subject to different interference levels due to the following aspects,

- the frequency domain isolation between it and the DL subband.

- time domain areas where DL transmissions are scheduled or configured in DL subband.

Proposal 15: UL subband resources can be divided into multiple areas and each area is mapped with a dedicated power control parameter set for compensating the inter-subband interference with different levels.

- The resources contained in each area can be indicated by DCI.

Proposal 16: Further study subband CLI measurement and reporting for UE-to-UE CLI handling under SBFD, e.g., configuration and determination of the measurement subband size and measurement reporting overheads reduction, etc.

Proposal 17: Different frequency densities can be configured for reference signals transmitted in different areas with different interference levels.

Reference

- RP-221352, “Revised SID: Study on evolution of NR duplex operation”, CMCC (Moderator)

- 3GPP TSG RAN WG1 #109-e, “RAN1 Chair’s Notes”