3GPP TSG RAN WG1 #110bis-e	                                       	 R1-2208545

e-Meeting, October 10th – 19th, 2022

Agenda Item:     9.1.4.2

Source:	Spreadtrum Communications

Title:	Discussion on SRI/TPMI enhancement for enabling 8 TX UL transmission

Document for:	Discussion and decision

Introduction

In this contribution, we will present our opinions on SRI/TPMI enhancement for enabling 8 TX UL transmission, mainly about codebook design, CW number, SRS configuration and SRI/TPMI indication.

Discussion

Codebook design for UL

In RAN1#109 meeting, the schemes of codebook design for UL 8TX have converged to five candidates:

Agreement For 8TX UE codebook-based uplink transmission, down-select one of - Alt1-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs - Alt1-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs - Alt2-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt2-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt3: - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

Then, in the RAN1#110 meeting, the candidates are reduced to two schemes, namely Alt1-b and Alt2-a.

Agreement RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e. - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

The main difference between Alt1-b and Alt2-a is the codebook design for fully-coherent UEs, namely, constructing based NR Rel-15 UL 2TX/4TX codebooks or down-sampling based NR Rel-15 DL Type I codebooks.

Rel-15 UL codebook is designed with reference to several previously existing codebooks, such as LTE DL codebook, LTE UL codebook, NR DL Type I codebook, antenna selection codebook, and etc., which can be summarized as Table 1.

Table 1. Rel-15 UL codebook design

| N-layer transmission | # antenna port | Transform precoding | UE coherence | Based on | Additional operating |
| --- | --- | --- | --- | --- | --- |
| 1 | 1 | / | / | W=1 | / |
| 1 | 2 | / | / | LTE UL 2TX codebook | / |
| 2 | 2 | disabled | / | LTE DL 2TX codebook | Add antenna selection codebook |
| 1 | 4 | enabled | / | LTE UL 4TX codebook | / |
| 1/2/3/4 | 4 | disabled | fully coherent | NR Rel-15 DL Type I 4 TX codebook | Down-sampling |
|  |  |  | partially coherent | LTE UL 4TX codebook | Selection & Modification |
|  |  |  | non-coherent | Antenna selection codebook | Selection |

Constructing the UL 8 TX codebook based on the NR Rel-15 UL 2TX/4TX codebooks is a relatively simple and flexible scheme, and different phase offsets can be attached to the NR Rel-15 UL 2TX/4TX codebooks based on the antenna layout and coherence to ensure the performance of the constructed 8TX codebook, e.g.  W8=W4W4φW4-φW4, where W4 and W8are the UL 4TX codebook and UL 8TX codebook, and φ is the additional phase offset.

Besides, UL 8TX codebook based on the NR Rel-15 UL 2TX/4TX codebooks doesn’t need to define specific antenna layout, while for 8 TX codebook based NR Rel-15 Type I codebooks, in order to apply 2D DFT beams and co-phasing, it maybe necessary to assume that UE is equipped with URA (Uniform Rectangular Array) antenna with cross polarization. However, it is not expected for UE to disclose its antenna layout.

Furthermore, UL 8TX codebook based NR Rel-15 UL 2TX/4TX codebooks is the same in Alt1-b and Alt2-a for partially/non-coherent UEs, and Alt2-a can achieves unified codebook design for all coherent type UEs.

Based on the above analysis, we have the following proposal for the UL codebook design for 8 TX UE codebook based transmission.

Proposal 1: For 8TX UE codebook-based uplink transmission, Alt2-a is preferred.

CW number for UL transmission with rank > 4

In RAN1#110 meeting, the following agreement related to layer number and CW number is achieved:

Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item.

If single CW is used for uplink transmission with rank > 4, new CW to layer mapping should be discussed and decided. To save spec effort, it is necessary to use the unified design for UL transmission and DL transmission, namely, dual CW should be supported for uplink transmission with rank >4 and reuse the rules of DL CW to layer mapping.

Proposal 2: Dual CW should be supported for uplink transmission with rank > 4.

SRS configuration

In RAN1#110 meeting, the following agreement related to SRS configuration is achieved:

Agreement For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources - Alt3: Support both alternatives.

Alt1 has the smallest effect on spec, which only introduce minor change in the number of value, while Alt2 requires two separate configurations for the up to 8 single-port SRS resource which should be equivalent in principle, thus the number of SRS resource should be up to 8 in one SRS resource set for non-codebook UL transmission.

Proposal 3: Alt1 for SRS configuration should be supported.

SRI/TPMI indication for enabling 8 TX UL transmission

In RAN1#110 meeting, the following agreement related to SRS configuration is achieved:

Agreement Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE, - FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded. - Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms

From overhead perspective, single field for the indication for codebook-based and non-codebook-based UL transmission is more efficient while some specification work may be required. For codebook-based transmission, the specification work has a strong correlation with the structure of the codebook design, which is difficult to estimate without UL codebook. For non-codebook-based transmission, two fields which may be corresponding to different SRS resource sets, has no significant benefits, for example, when the SRS to be indicated are all located in the same resource set, a codepoint is needed to indicate there is  no resource in the other resource set, which will increase the indicating overhead to some extent.

On the other hand, the separate fields for the indication generally means physical separation or limitation, such as each field corresponding to up to 4 TX which means the 8 TX is divided into two separate antenna groups or antenna port groups. Further, the assumption of separate fields is also useful for the extension of single filed, especially for the SRI indication for non-codebook-based UL transmission. TPMI indication for codebook-based UL transmission is more complex, using single or separate fields can be decided when the codebook design for UL is stable.

Based on the above analysis, we have the following proposal for SRI/TPMI indication:

Proposal 4: Single filed for SRI for non-codebook-based transmission should be supported.

Proposal 5: Single filed or separate fields for TPMI for codebook-based transmission should be decided after codebook design is stable.

Conclusion

In this contribution, we discussed SRI/TPMI enhancement for enabling 8TX UL transmission. The following observation and proposals are achieved:

Proposal 1: For 8TX UE codebook-based uplink transmission, Alt2-a is preferred.

Proposal 2: Dual CW should be supported for uplink transmission with rank > 4.

Proposal 3: Alt1 for SRS configuration should be supported.

Proposal 4: Single filed for SRI for non-codebook-based transmission should be supported.

Proposal 5: Single filed or separate fields for TPMI for codebook-based transmission should be decided after codebook design is stable.

Reference

- Draft_Minutes_report_RAN1#110-e_v020

- 3GPP TS 38.212 v17.0.0.

- 3GPP TS 38.211 v17.0.0.