3GPP TSG RAN WG1 #110b                                                      R1-2208559

E-Meeting, October 10th – 19th, 2022

Agenda Item:	9.5.2.3

Source:	Spreadtrum Communications

Title:	Discussion on evaluation on LPHAP

Document for:	Discussion and decision

Introduction

The study on LPHAP is approved in [1].

- Study the requirements on LPHAP as developed by SA1 and evaluate whether existing RAN functionality can support these power consumption and positioning requirements. Based on the evaluation, and, if found beneficial, study potential enhancements to help address any limitations [RAN2, RAN1] - Study is limited to a single representative use case (use case 6 as defined TS 22.104). The choice of selected use case can be reviewed at the start of the study. - Study is limited to enhancements to RRC_INACTIVE and/or RRC_IDLE state

In this paper, we share our initial evaluation of power consumption of positioning in RRC_INACTIVE.

Power model for positioning in RRC inactive mode

UL positioning

In RAN1#109-e, the following agreement on the power components and parameter values of UL positioning had been achieved [2].

Agreement - For UL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - UL SRS for positioning transmission with 0.5 ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - Note: The power component and parameter values for UL positioning is also applicable to the UL part of UE-assisted DL+UL positioning method. - Note: Individual company may consider additional power components and different parameter values in bracket in the evaluation. Note: Companies are encouraged to provide the assumption on the timeline between different power consumption events in the evaluation of potential enhancements to reduce the transition times between different power states and to extend the sleeping time as much as possible. Agreement Adopt the following power consumption model for UL SRS for positioning transmission. Power State; Relative power. SRS; 210 (baseline); 700 (optional).

For the UL positioning, the power model for SRS transmission in RRC inactive mode can be shown in Figure 2.1.

Figure 2.1: The power model for UL positioning in RRC inactive mode.

In our evaluation, power components and parameter values of the UE-assisted DL positioning as shown in the following Table.

Table 2.1: Power components and parameter values of the UE-assisted DL positioning

| UE power state | Relative power |
| --- | --- |
| SSB proc. | 50 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| SRS Tx | 210 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |
| Ramp up/down (deep sleep) | 450 |

DL positioning

In RAN1#109-e, the following agreement on the power components and parameter values of DL positioning had been achieved [2].

Agreement - Adopt the following reference configuration and assumption for DL PRS to define the power consumption model for DL PRS measurement: - 1 Number of PFL; - 8 DL PRS resources per slot are measured; - DL PRS instance of smaller than or equal to 1 slot duration; - Adopt the following table as the power consumption model for DL PRS measurement (derived from Table 22 in TR38.840): N: Number of TRPs for DL PRS measurement; Synchronous case (baseline); Asynchronous case (optional). FR1 (baseline); FR2 (optional); FR1; FR2. N=4 (baseline); 120; 195; 140; 255. N=8 (optional); 150; 225; 170; 285.

Agreement - For DL positioning, at least the following power components and parameter values are considered for the baseline evaluation of Rel-17 RRC_INACTIVE positioning: - For the UE-assisted DL positioning, - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - DL PRS measurement with 0.5 ms duration; - CG-SDT with 1ms duration and the periodicity of positioning interval; - RRCRelsease after the CG-SDT can be optionally included with [1] ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - (Optional) RA-SDT (e.g., including CORSET0 + SIB1, PRACH, RAR, Msg 3/4/5) in case of CG-SDT is unavailable; - For the UE-based DL positioning, - SSB proc. with 2 ms duration and the periodicity of I-DRX cycle; - Paging with 2 ms duration, the periodicity of I-DRX cycle, and group paging rate of 10%; - DL PRS measurement with 0.5 ms duration; - (Optional) BWP switching with [1] ms duration; - (Optional) Intra-/inter-frequency RRM measurement in low SINR condition with [1] ms duration; - Note: The power component and parameter values for UE-assisted DL positioning is also applicable to the DL part of UE-assisted DL+UL positioning method. - Note: Individual company may consider additional power components and different parameter values in bracket in the evaluation. - Note: Companies are encouraged to provide the assumption on the timeline between different power consumption events in the evaluation of potential enhancements to reduce the transition times between different power states and to extend the sleeping time as much as possible.

UE-assisted DL positioning

For the UE-assisted DL positioning, the power model for PRS reception and measurement reporting in RRC inactive mode shown in Figure 2.2.1. We assume that UE selects the CG-SDT resource to report the measurement.

Figure 2.2.1: The power model for UE-assisted DL positioning in RRC inactive mode.

In our evaluation, power components and parameter values of the UE-assisted DL positioning as shown in the following table.

Table 2.2.1: Power components and parameter values of the UE-assisted DL positioning

| UE power state | Power unit per slot |
| --- | --- |
| SSB proc. | 50 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| PUSCH | 250 |
| PRS | 120 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |
| Ramp up/down (deep sleep) | 450 |

UE-based DL positioning

For the UE-based DL positioning, the power model for PRS reception in RRC inactive mode shown in Figure 2.2.2.

Figure 2.2.2: The power model for UE-assisted DL positioning in RRC inactive mode.

In our evaluation, power components and parameter values of the UE-based DL positioning as shown in the following table.

Table 2.2.2: Power components and parameter values of the UE-based DL positioning

| UE power state | Power unit per slot |
| --- | --- |
| SSB proc. | 50 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| PRS | 120 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |
| Ramp up/down (deep sleep) | 450 |

Reference device

According TR 38.840, the power model assumption for the reference device considered in the evaluation is listed in Table 2.3-1.

Table 2.3-1: The power model assumption for the reference device

| UE power state | Power unit per slot |
| --- | --- |
| SSB proc. | 50 |
| PDCCH-only | 50 |
| PDCCH+PDSCH | 120 |
| Micro sleep | 45 |
| Light sleep | 20 |
| Deep sleep | 1 |

Evaluation

In RAN1#110e, the following agreement on the relative power unit and the battery life had been achieved [3].

Agreement In the LPHAP evaluation, adopt the following model to convert the relative power unit to the battery life: - Alt. 1: battery life is used as the metric to identify the gap - K is an implementation factor, K = 1 (baseline); K = 0.5, 2, 4 (optional) - Note: The definition of the notations will be captured in the updates of TR. - Note: The voltage is assumed to be the same for the reference device and the LPHAP device. Agreement - In the LPHAP evaluation, adopt the following example parameter values in the conversion model to evaluate the battery life: - For the reference device in the conversion model: C1 (mAh); T1 (hour); X; reference traffic type. 4500; 12; 20%; FTP (model 3). - For the LPHAP device, consider 2 types in the conversion model: LPHAP device; C2 (mAh); T2req (month). Type A (baseline); 800; 6~12. Type B (optional); 4500; 6~12. - Note: As the reference device and LPHAP device characteristics, and therefore the parameter values of the model for determining battery life, is dependent on implementation factors, manufacturer, design options and cost options, it is up to individual company to evaluate the optional K values, and report the corresponding parameter values.

In this section, we share our initial evaluation result of the relative power unit and the battery life of LPHAP device to identify the performance gap.

Reference device

The reference traffic type is FTP model 3. Some other parameter configurations about the reference device are shown in the following table.

Table 3.1-1: Some other parameter configurations about the reference device

| Parameters | Values |
| --- | --- |
| Packet Size | 0.5 Mbytes |
| Mean interval | 200 ms |
| DRX setting | Period: 160ms On duration: 8ms Inactivity timer: 100ms |

In our evaluation, we count the total power unit values for each power states of  the reference device during 1024s. The evaluation results are shown in Table 2.3-3.

Table 3.1-2: The total power unit values for each power states of the reference device during 1024s

| UE power state | Total power unit |
| --- | --- |
| PDCCH-only | 71850040 |
| PDCCH+PDSCH | 1547500 |
| Micro sleep | 10540 |
| Light sleep | 71450 |
| Deep sleep | 3812500 |
| Ramp up/down | 67500 |

According to the total power consumption during 1024s, the relative power unit P1 of the reference device obtained based on the reference traffic type is calculated as follows.

P1 = (71850040+1547500+10540+71450+3812500+67500)/(1024*1000*2)=38 (units/slot)

Evaluation cases and assumptions

Our evaluation cases and assumptions are shown in Table 3.2-1 and Table 3.2-2.

Table 3.2-1: Evaluation cases and assumptions

| Evaluation assumption | [Case 1], [FR1], [UL positioning], [Type A] | [Case 2], [FR1], [UL positioning], [Type B] | [Case 3], [FR1], [UE-assisted DL positioning], [Type A] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep: 1 | Deep sleep: 1 | Deep sleep: 1 |
| DRX cycle | 10.24s | 10.24s | 10.24s |
| Paging reception | Y | Y | Y |
| RS periodicity | 10.24s | 10.24s | 10.24s |
| M-sample | 1 | 1 | 1 |
| RRM measurement | N/A | N/A | N/A |
| BWP switching | N/A | N/A | N/A |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | N/A | N/A | CG-SDT |
| implementation factor K | 1, 2, 4 | 1, 2, 4 | 1, 2, 4 |

Table 3.2-2: Evaluation cases and assumptions

| Evaluation assumption | [Case 4], [FR1], [UE-assisted DL positioning], [Type B] | [Case 5], [FR1], [UE-based DL positioning], [Type A] | [Case 6], [FR1], [UE-based DL positioning], [Type B] |
| --- | --- | --- | --- |
| Sleep state | Deep sleep: 1 | Deep sleep: 1 | Deep sleep: 1 |
| DRX cycle | 10.24s | 10.24s | 10.24s |
| Paging reception | Y | Y | Y |
| RS periodicity | 10.24s | 10.24s | 10.24s |
| M-sample | 1 | 1 | 1 |
| RRM measurement | N/A | N/A | N/A |
| BWP switching | N/A | N/A | N/A |
| Measurement reporting (e.g., RA/CG-SDT, reporting interval) | CG-SDT | N/A | N/A |
| implementation factor K | 1, 2, 4 | 1, 2, 4 | 1, 2, 4 |

Evaluation results

Our evaluation results are shown in Table 3.3-1.

Table 3.3-1: Evaluation results

| Evaluation case description | Slot-averaged relative power unit (P2) | Battery life (in month) | Target requirement are met – Yes/No; If no, provide gaps |  |
| --- | --- | --- | --- | --- |
|  |  |  | 6 months | 12 months |
| Case 1 (K=1) | 1.1 | 0.5 | No (Gap=5.5) | No (Gap=11.5) |
| Case 1 (K=2) | 1.1 | 1 | No (Gap=5) | No (Gap=5) |
| Case 1 (K=4) | 1.1 | 2 | No (Gap=4) | No(Gap=10) |
| Case 2 (K=1) | 1.1 | 2.8 | No(Gap=3.2) | No(Gap=9.2) |
| Case 2 (K=2) | 1.1 | 5.6 | No(Gap=0.4) | No(Gap=6.4) |
| Case 2 (K=4) | 1.1 | 11.2 | Yes | No (Gap=0.8) |
| Case 3 (K=1) | 1.09 | 0.5 | No (Gap=5.5) | No (Gap=11.5) |
| Case 3(K=2) | 1.09 | 1 | No (Gap=5) | No (Gap=5) |
| Case 3 (K=4) | 1.09 | 2 | No (Gap=4) | No(Gap=10) |
| Case 4 (K=1) | 1.09 | 2.8 | No(Gap=3.2) | No(Gap=9.2) |
| Case 4 (K=2) | 1.09 | 5.6 | No(Gap=0.4) | No(Gap=6.4) |
| Case 4 (K=4) | 1.09 | 11.2 | Yes | No (Gap=0.8) |
| Case 5 (K=1) | 1.08 | 0.52 | No (Gap=5.48) | No (Gap=11.48) |
| Case 5 (K=2) | 1.08 | 1.04 | No (Gap=4.96) | No (Gap=10.96) |
| Case 5 (K=4) | 1.08 | 2.08 | No (Gap=3.92) | No (Gap=9.92) |
| Case 6 (K=1) | 1.08 | 2.92 | N (Gap=3.08) | N (Gap=9.08) |
| Case 6 (K=2) | 1.08 | 5.85 | No (Gap=0.15) | No (Gap=6.15) |
| Case 6 (K=4) | 1.08 | 11.7 | Yes | No (Gap=0.3) |

Based on the above evaluation results, we have the following observations.

Observation 1: When implementation factor K is less than 4, the battery life of LPHAP device Type A and Type B cannot meet the requirement of 6 months.

Observation 2: When implementation factor K is equal to 4, the battery life of LPHAP device Type B can meet the requirement of 6 months.

Observation 3: The battery life of LPHAP device Type A and Type B cannot meet the requirement of 12 months with any values of implementation factor K.

Conclusion

In this contribution, we provided initial evaluation on LPHAP. In summary, we have following observation:

Observation 1: When implementation factor K is less than 4, the battery life of LPHAP device Type A and Type B cannot meet the requirement of 6 months.

Observation 2: When implementation factor K is equal to 4, the battery life of LPHAP device Type B can meet the requirement of 6 months.

Observation 3: The battery life of LPHAP device Type A and Type B cannot meet the requirement of 12 months with any values of implementation factor K.

Reference

- RP-213588, “Revised SID on Study on expanded and improved NR positioning”, Intel.

- 3GPP RAN1#109-e Chairman’s Notes, May 9th – 20th, 2022.

- 3GPP RAN1#110 Chairman’s Notes, August 22nd – 26th, 2022.