3GPP TSG RAN WG1 #110bis-e			R1-2208577

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.14.3

Source: 	Spreadtrum Communications

Title: 	Discussion on dynamic switching between DFT-S-OFDM and CP-OFDM

Document for:	Discussion and decision

Introduction

At RAN#96 meeting, a new Rel18 WI [1] on NR coverage enhancement was approved with below objectives:

The objective of this work item is to specify further uplink coverage enhancements for PRACH, power domain and DFT-S-OFDM. - Specify enhancements to support dynamic switching between DFT-S-OFDM and CP-OFDM (RAN1)

In this contribution, we share our views on dynamic switching between DFT-S-OFDM and CP-OFDM.

Discussion

Coverage is one of the key factors that an operator considers when commercializing cellular communication networks. UL performance could be the bottleneck in most of scenarios in real deployment, while there are emerging vertical use cases that have UL heavy traffic.

NR supports two types of uplink transmission waveforms: DFT-S-OFDM and CP-OFDM. DFT-S-OFDM waveform is beneficial for UL coverage limited scenario because of its lower Peak to Average Power Ratio（PAPR）compared with CP-OFDM waveform. The channel quality can be improved by increasing the transmission power of cell-edge UEs and achieve better coverage performance. CP-OFDM can provide higher spectral efficiency and targeted for the UEs with better channel state. The disadvantages of CP-OFDM are high PAPR and require wide linear range of the amplifier.

Since UE will reside at different locations under the cell coverage range at different times, dynamic switching of waveform can help UE to adapt to fast channel condition change. It is an efficient method to make flexible scheduling for UEs in various coverage of a cell. In the coverage limited scenario, the CP-OFDM can be quickly switched to DFT-S-ODFM to improve the coverage performance.

In current specification, the waveform used in uplink is configured via RRC and cannot be dynamically switched. There are four different RRC parameters associated with transform precoding enabled/disabled for different types of PUSCH scheduling: transformPrecoder in pusch-Config, transformPrecoder in configuredGrantConfig, msg3-transformPrecoder in RACH-ConfigCommon, and msgA-TransformPrecoder-r16 in MsgA-PUSCH-Config. The following Table 1 gives a summary of PUSCH types and its waveform determinations.

Table 1: PUSCH types and its waveform determinations

| PUSCH Types | Waveform determination |
| --- | --- |
| PUSCH (including CG-PUSCH re-transmission) scheduled by DCI 0_0 | msg3-transformPrecoder |
| PUSCH transmission (including CG-PUSCH re-transmission) scheduled by other DCI formats | transformPrecoder in pusch-Config otherwise, msg3-transformPrecoder |
| CG-PUSCH | transformPrecoder in configuredGrantConfig, otherwise, msg3-transformPrecoder |
| PUSCH scheduled by RAR UL grant, or DCI 0_0 using TC-RNTI | msg3-transformPrecoder |
| MSG A | msgA-TransformPrecoder-r16 otherwise, msg3-transformPrecoder |

Therefore, whether CP-OFDM or DFT-S-OFDM waveform is used for uplink transmission is determined by above RRC parameters, which cannot achieve flexible and effective uplink scheduling, due to switching between CP-OFDM and DFT-S-OFDM requires RRC reconfiguration. It means that waveform switching is quite slow.

In order to adapt to different channel quality and UE states, a dynamic waveform switching mechanism is needed. Therefore, it is necessary to study dynamic switching between DFT-S-OFDM and CP-OFDM, including the following five aspects:

- Dynamic waveform switching triggering

- Target uplink channel support dynamic waveform switching

- Dynamic waveform switching indication

- Waveform related configurations

- Maximal rank of DFT-S-OFDM

Dynamic waveform switching triggering

First of all, it is important to figure out the switching scenarios for this feature.

It is a reasonable way to trigger dynamic waveform switching by gNB. The gNB dynamically indicates the waveform according to PUSCH transmission conditions, including the channel quality, UL coverage, power usage, etc.

In NR, SRS is used to obtain the uplink channel status information. The UE can report its SRS transmission capability to the gNB and the gNB can configure SRS resource sets for it. The UE transmit SRS to the UE based on the SRS resource settings. The gNB then measures the SRS and obtain the SNR information. Based on the SNR information, the gNB can make scheduling decisions and provide the information about resources allocation and related transmission settings to UE. Thus, gNB can indicate dynamic waveform switching according to SRS measurement. For high SNR UEs, the waveform may be CP-OFDM. For low SNR UEs, the waveform may be DFT-S-OFDM.

In current power headroom report (PHR), the UE can report the difference value between the current maximum transmit power and the estimated uplink transmission power to the network to assist the network in uplink scheduling. The gNB can refer to the PHR reported by the UE for waveform indication.

The current measurement and reporting can provide sufficient information to gNB, and gNB can make a decision whether to switch waveform according to current measurement and reporting information.

Proposal 1. The gNB can trigger waveform switching based on the current measurement and/or reporting information.

Target uplink channel support dynamic waveform switching

Before the RRC connection set up, gNB can only obtain partial information about the UE. The UE can report little information to the gNB. So the gNB cannot do proper and timely waveform switching decision. Besides, for the idle and inactive UE, it is not necessary to switch waveforms. Furthermore, there is little requirement for waveform switching when the demand for UE to transmit PUSCH is small. Only when the UE is in the connected mode, dynamic switching between DFT-S-OFDM and CP-OFDM can be studied.

Proposal 2. PUSCH transmission in RRC-connected can support dynamic switching between DFT-S-OFDM and CP-OFDM.

There are several kinds of PUSCH, including dynamic grant PUSCH (DG-PUSCH)，configured grant (CG) PUSCH，Msg 3 PUSCH and Msg A PUSCH. Thus, the uplink channel that allows dynamic waveform switching should be defined.

DG-PUSCH

For DG-PUSCH, it is natural to use a dynamic waveform switching mechanism. Because of dynamic scheduling, it is very convenient to dynamically indicate the waveform.

However, PUSCH scheduled by fallback DCI 0_0 has high requirement of robustness, and it should be work well during the RRC reconfiguration, which may include the waveform information and would lead to misunderstanding between gNB and UE during the configuration phase if dynamic waveform switching is supported. Therefore, PUSCH scheduled by fallback DCI 0_0 cannot support dynamic waveform switching.

Non-fallback can be compatible with more NR features, which can provide more flexible scheduling. For PUSCH scheduled by other DCI formats, dynamic waveform switching should be supported to achieve flexible and effective uplink scheduling.

Proposal 3. PUSCH scheduled by non-fallback DCI can support dynamic waveform switching.

CG-PUSCH

There are two types of uplink configured grant transmission schemes: type 1 CG-PUSCH and type 2 CG-PUSCH. The UE automatically transmits uplink data on the pre-configured or activated resources. Since CG-PUSCH is semi-statically configured, if the waveform is dynamically switched, all parameters related to the waveform should be changed, such as precoding information, the PTRS-DMRS association information, etc. However, for CG-PUSCH, there is no basis for dynamically switching waveforms.

All parameters involved in the type 1 CG-PUSCH are configured via RRC and UE transmits PUSCH directly on configured resources without DCI activation signalling. Thus, for type 1 CG-PUSCH, all transmission related parameters cannot be dynamically changed, so dynamic waveform switching cannot be performed.

Some parameters of type 2 CG-PUSCH are configured via RRC, and then uplink transmission is activated by DCI with CRC scrambled by CS-RNTI. Once it is activated and the deactivation signalling is not received, the UE will always use the parameters indicated by the DCI for uplink transmission. If the waveform needs to be changed during transmission, other related parameters cannot be changed during the current DCI activation.

Therefore, both type 1 and type 2 CG-PUSCH do not support dynamic waveform switching.

Observation 1. Dynamic waveform switching is not suitable for CG-PUSCH initial transmission.

PUSCH retransmission

For the retransmission of DG-PUSCH and CG-PUSCH, it is dynamically indicated by the 'New Data Indicator' field of the scheduling DCI. In PUSCH retransmission, most parameters are indicated by DCI. Therefore, the waveform can be dynamically indicated by DCI.

The waveforms of initial transmission and retransmission can be dynamically indicated by the DCI scheduling the initial and retransmission respectively. There seems to be no reason to bind the initial and retransmission waveforms. The parameters of initial transmission and retransmission depend on gNB scheduling. For example, the MCS of initial transmission and retransmission can also be different if different waveform is indicated.

Proposal 4. The waveforms of the initial and retransmission of PUSCH can be different and depend on scheduling of gNB.

Proposal 5. For PUSCH retransmission, dynamic waveform switching can be supported.

Msg 3 PUSCH and Msg A PUSCH

The cell-specific random-access parameters are specified by the high-level parameter RACH-ConfigCommon. The waveform of Msg3 PUSCH related to random access should be indicated by higher layer configured parameter msg3-transformPrecoder in RACH-ConfigCommon. From the perspective of parameter configuration, it is not necessary to use dynamic waveform switching for Msg3 PUSCH. Similarly, for Msg A, it is enough to determine the waveform according to the current configuration of high-level parameters msgA-TransformPrecoder-r16.

Proposal 6. Dynamic waveform switching is not supported for Msg 3 PUSCH and Msg A PUSCH.

Based on the above analysis, the following Table 2 summarizes the types of PUSCH and whether it supports dynamic waveform switching or not.

Table 2: Uplink channel whether or not support dynamic waveform switching

| PUSCH Types | Whether or not support dynamic waveform switching |
| --- | --- |
| PUSCH (re)transmission (including CG-PUSCH re-transmission) scheduled by DCI 0_0 | No |
| PUSCH (re)transmission (including CG-PUSCH re-transmission) scheduled by other DCI formats | Yes |
| CG-PUSCH | No |
| PUSCH scheduled by RAR UL grant, or DCI 0_0 using TC-RNTI | No |
| MSG A | No |

Dynamic waveform switching indication

Dynamic waveform switching can be DCI-based or MAC-CE based. DCI mainly includes UE-specific DCI, such as uplink and downlink transmission resource scheduling information DCI0_0/1/2, DCI1_0/1/2. In addition, it also includes group common DCI (GC-DCI), such as uplink power control indication, time slot format indication, etc.

UE-specific DCI

Dynamic indicated waveform by UE-specific DCI is the preferred and more general solution. It is the most straightforward and simplest way to introduce 1 bit to indicate CP-OFDM or DFT-S-OFDM to be used for PUSCH. It is also possible to indicate whether DFT-S-OFDM or CP-OFDM by the time domain resource allocation (TDRA) field in the DCI. In addition to the existing definition of slot offset K0, start and length indicator SLIV and PUSCH resource mapping type, a line of TDRA table also needs to indicate whether the current transmission uses the DFT-S-OFDM or CP-OFDM waveform. Thus, the waveform can be dynamically indicated by adding a column to the TDRA table.

The lengths of some fields in the UE-specific DCI are related to the waveform configuration, mainly including the precoding information and the layer number field, the PTRS-DMRS association field, and the antenna port field. When the UE-specific DCI is used to dynamically indicate the waveform, the DCI size may be misaligned under different waveform configurations. Thus, either align each of the fields associated with the waveform, or align the total DCI for different waveform configurations can be implemented.

Observation 2. It would be easier to reuse the existing DCI format and indicate the waveform only by adding 1 bit.

Proposal 7. The DCI sizes in different waveform configurations need to be aligned.

GC-DCI

For GC-DCI, it can indicate uplink transmission waveform information for a group of UEs. Each UE determines the location of its own indicator information based on the configuration parameters. When multiple configured grant resources are configured in a cell, the uplink transmission waveform information of each CG resource also need to be indicated by GC-DCI. However, waveform switching is mainly determined by UE states, indicating the waveform by GC-DCI will lead to a large signalling overhead. Moreover, it requires a new DCI format and has a greater impact on the specification. Thus, it is difficult to dynamically indicate the waveform by GC-DCI.

Observation 3.  Introducing a new GC-DCI will lead to a large signalling overhead and have a great impact on the specification.

MAC-CE signalling

Uplink transmission waveforms can be dynamically indicated by MAC CE signalling. The MAC CE is more reliable because MAC CE mechanism requires HARQ feedback and it avoids adding signalling overhead to DCI. However, the UE applies the MAC CE command only after the UE transmit HARQ-ACK information and several slots. It means the MAC CE signalling takes a period of time to take effect and the latency is large. The waveform switching can be performed only when the effective time point is reached, which is difficult to meet the requirements of low-latency services. Besides, it does not require much bit overhead in the waveform switching scenario. As a physical layer signalling, DCI can take effect within a few symbol times, which is more conducive to fast switching of waveforms.

Observation 4. As MAC CE signalling has large latency, it may not meet the requirement of fast waveform switching.

Implicit waveform indication

Waveform information can be implicitly indicated by other scheduling information. Implicit indication means that the DCI does not contain waveform indication information, but the waveform switching is triggered by something else in the DCI such as the frequency resource allocation type and multiple value of 2, 3, 5, the layer number, MCS information and whether PUSCH and DMRS are FDM, etc. The UE needs to determine whether or not switch waveform by other scheduling information, which can lead to inconsistency between gNB and UE behavior. Besides, implicit indication is not as flexible as explicit indication. The transmission waveform indicated by the gNB is a more direct and reasonable way.

Observation 5. Explicit UE-specific DCI is more appropriate to dynamic waveform switching.

Waveform related configurations

MCS table

In current specification, the MCS table is strongly correlated with the waveform. The UE shall use the MCS table for PUSCH without transform precoding (CP-OFDM) and use another MCS table for PUSCH with transform precoding (DFT-s-OFDM). When DFT-S-OFDM and CP-OFDM waveforms are dynamically switched, the MCS table should be switched as the waveform is switched. Therefore, mcs-Table and mcs-TableTransformPrecoder can be configured by RRC at the same time. When waveform switching occurs, the corresponding MCS tables are used.

Proposal 8. The MCS table should be alignment with the switched waveform.

Frequency resource allocation

Uplink resource allocation scheme type 0 is supported for PUSCH only when transform precoding is disabled, while resource allocation scheme type 1 is supported for PUSCH for both cases when transform precoding is enabled or disabled. It means that DFT-s-OFDM is restricted to contiguous PRBs in the frequency domain（i.e., type 1）, and the number of PRBs must be a multiple of 2, 3, and 5, while CP-OFDM can allows discontinuous frequency domain resources and any integer number of PRBs in the frequency domain, which is more flexible in resource allocation.

If the configured resource allocation type is type 0, the waveform can only be CP-OFDM and cannot be dynamically switched to DFT-S-OFDM. Thus, the frequency domain resource allocation type should match the waveform. The gNB needs to ensure that type 0 resource allocation is not used for DFT-S-OFDM. There are two resouce allocation configurations, one for CP-OFDM and the other for DFT-S-OFDM. Alternatively, frequency domain resource allocation type also can be dynamically indicated by scheduling DCI.

Proposal 9. The frequency domain resource allocation type should match the waveform.

Maximal rank of DFT-S-OFDM

Regarding for the multi-layer DFT-S-OFDM, there is no use-case to use it because dynamic switching is supported. If it is for spectrum efficiency, we can use CP-OFDM instead because rank >1 already supported by CP-OFDM. It would be redundant to support both multi-layer DFT-S-OFDM and dynamic waveform switching. Besides, multi-layer DFT-s-OFDM has unclear performance gain and large specification impacts. Thus, there is no need to support multi-layer DFT-S-OFDM.

Proposal 10. It remains rank 1 PUSCH only with DFT-s-OFDM.

Conclusion

In this contribution, we have the following observations and proposals:

Observation 1. Dynamic waveform switching is not suitable for CG-PUSCH initial transmission.

Observation 2. It would be easier to reuse the existing DCI format and indicate the waveform only by adding 1 bit.

Observation 3.  Introducing a new GC-DCI will lead to a large signalling overhead and have a great impact on the specification.

Observation 4. As MAC CE signalling has large latency, it may not meet the requirement of fast waveform switching.

Observation 5. Explicit UE-specific DCI is more appropriate to dynamic waveform switching.

Proposal 1. The gNB can trigger waveform switching based on the current measurement and/or reporting information.

Proposal 2. PUSCH transmission in RRC-connected can support dynamic switching between DFT-S-OFDM and CP-OFDM.

Proposal 3. PUSCH scheduled by non-fallback DCI can support dynamic waveform switching.

Proposal 4. The waveforms of the initial and retransmission of PUSCH can be different and depend on scheduling of gNB.

Proposal 5. For PUSCH retransmission, dynamic waveform switching can be supported.

Proposal 6. Dynamic waveform switching is not supported for Msg 3 PUSCH and Msg A PUSCH.

Proposal 7. The DCI sizes in different waveform configurations need to be aligned.

Proposal 8. The MCS table should be alignment with the switched waveform.

Proposal 9. The frequency domain resource allocation type should match the waveform.

Proposal 10. It remains rank 1 PUSCH only with DFT-s-OFDM.

References

RP-221858, New WI: Revised WID on Further NR coverage enhancements，RAN#96, China Telecom

TS 38.214, “NR, Physical layer procedures for data”