3GPP TSG RAN WG1 #110bis-e		R1-2208647

e-Meeting, October 10th – 19th, 2022

Source:	vivo

Title:	Evaluation of sidelink positioning performance

Agenda Item:	9.5.1.1

Document for:	Discussion and Decision

- Introduction

The methodology for sidelink positioning has been discussed in last meeting and major issues have been achieved to conclusions. Major agreements achieved in last meeting are shown in the following table. In this contribution, simulation results are provided based on the evaluation methodology.

Agreement For SL positioning evaluation in IIOT use case, companies should report how to drop anchor UEs and how to select anchor UEs Agreement Adopt the tables in section 3 of R1-2207606 as templates to collect SL positioning simulation results from each company. Agreement In the evaluation, relative positioning or ranging is performed between two UEs within X m, where X value(s) are reported by companies, and companies should also report the minimum distance used in the evaluations for each use case. The assumption used for X will be included in the TR for each set of results. Agreement For SL positioning evaluation purpose, the following assumptions are further adopted - Companies should report whether SL-PRS and other SL signals are FDMed or not FDMed, and whether other SL signals are present - Adopting system level simulations (rather than the link level simulations) as the baseline tool - For SL positioning evaluation in highway scenario or urban grid scenario, the performance metrics can include absolute horizontal accuracy, relative horizontal accuracy, ranging with distance accuracy, and ranging with direction accuracy (optionally). - In highway and urban grid scenarios, companies can further consider other UE types, e.g. pedestrian UE or VRU devices.

- Evaluation results

According to the evaluation model agreed in the last two meetings, simulation results of V2X scenario are provided in this section. Detailed simulation assumptions can be found in Appendix.

2.1 Absolute positioning

In this subsection, evaluation results of absolute positioning in highway scenario and urban scenario are provided based on multiple UEs locations or multiple RSUs locations. The measurement algorithm based on MUSIC is used for the timing estimation between two UEs or RSUs. The number of UEs or RSUs for absolute positioning is 5 and 10. The detailed parameters for each case are provided in Table 1.

Table 1 Assumptions of each case for absolute positioning in Highway and Urban scenario

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- |
| Scenario | Highway | Highway | Highway | Highway | Highway |
| Bandwidth | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| Anchor type | RSU | RSU | RSU | RSU | RSU |
| Anchor number | 5 | 5 | 5 | 5 | 10 |
| RSU deployment for absolute positioning | staggered | staggered | staggered | staggered | staggered |
| Link type | All link | All link | All link | All link | All link |

| Case 6 | Case 7 | Case 8 | Case 9 | Case 10 | Case 11 | Case 12 | Case 13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Highway | Highway | Highway | Highway | Highway |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 10 | 10 | 10 | 5 | 5 | 5 | 5 | 10 |
| staggered | staggered | staggered | symmetric | symmetric | symmetric | symmetric | symmetric |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 14 | Case 15 | Case 16 | Case 17 | Case 18 | Case 19 | Case 20 | Case 21 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 10 | 10 | 10 | 5 | 5 | 5 | 5 | 10 |
| symmetric | symmetric | symmetric | / | / | / | / | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 22 | Case 23 | Case 24 | Case 25 | Case 26 | Case 27 | Case 28 | Case 29 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 10 | 10 | 10 | 5 | 5 | 5 | 5 | 10 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | LOS only | LOS only | LOS only | LOS only | LOS only |

| Case 30 | Case 31 | Case 32 | Case 33 | Case 34 |
| --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 20MHz | 20MHz |
| RSU | RSU | RSU | RSU | RSU |
| 10 | 10 | 10 | 5 | 10 |
| / | / | / | Sync error | Sync error |
| LOS only | LOS only | LOS only | All link | All link |

The absolute positioning accuracy result is provided in Table 2.

Table 2 Accuracy result(m) in V2X scenario with absolute positioning

| Case ID | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Highway 10M 5 staggered RSU | 1.03 | 1.432 | 2.05 | 2.92 | No | No |
| Case 2 Highway 20M 5 staggered RSU | 0.612 | 0.939 | 1.426 | 2.46 | No | No |
| Case 3 Highway 40M 5 staggered RSU | 0.575 | 0.89 | 1.247 | 1.83 | No | No |
| Case 4 Highway 100M 5 staggered RSU | 0.375 | 0.592 | 0.845 | 1.51 | No | No |
| Case 5 Highway 10M 10 staggered RSU | 0.916 | 1.43 | 2.23 | 3.55 | No | No |
| Case 6 Highway 20M 10 staggered RSU | 0.739 | 1.04 | 1.423 | 2.23 | No | No |
| Case 7 Highway 40M 10 staggered RSU | 0.569 | 0.892 | 1.316 | 2.01 | No | No |
| Case 8 Highway 100M 10 staggered RSU | 0.482 | 0.686 | 0.99 | 1.41 | Yes | No |
| Case 9 Highway 10M 5 symmetric RSU | 0.86 | 1.41 | 2.1 | 3.02 | No | No |
| Case 10 Highway 20M 5 symmetric RSU | 0.65 | 1.04 | 1.65 | 2.56 | No | No |
| Case 11 Highway 40M 5 symmetric RSU | 0.48 | 0.84 | 1.34 | 1.78 | No | No |
| Case 12 Highway 100M 5 symmetric RSU | 0.35 | 0.58 | 0.81 | 1.32 | Yes | No |
| Case 13 Highway 10M 10 symmetric RSU | 1.12 | 1.76 | 2.58 | 4.03 | No | No |
| Case 14 Highway 20M 10 symmetric RSU | 0.87 | 1.43 | 2.32 | 3.27 | No | No |
| Case 15 Highway 40M 10 symmetric RSU | 0.77 | 1.12 | 1.63 | 2.58 | No | No |
| Case 16 Highway 100M 10 symmetric RSU | 0.69 | 0.98 | 1.43 | 2.08 | No | No |
| Case 17 Urban 10M 5RSU | 5 | 7.48 | 11.01 | 20.32 | No | No |
| Case 18 Urban 20M 5RSU | 4.11 | 6.05 | 8.64 | 15.81 | No | No |
| Case 19 Urban 40M 5RSU | 3.22 | 4.18 | 5.58 | 8.85 | No | No |
| Case 20 Urban 100M 5RSU | 2.67 | 3.48 | 4.91 | 7.53 | No | No |
| Case 21 Urban 10M 10RSU | 4.31 | 5.58 | 7.32 | 9.93 | No | No |
| Case 22 Urban 20M 10RSU | 3.25 | 4.16 | 5.22 | 7.03 | No | No |
| Case 23 Urban 40M 10RSU | 2.53 | 3.16 | 3.93 | 5.13 | No | No |
| Case 24 Urban 100M 10RSU | 2.17 | 2.68 | 3.31 | 4.18 | No | No |
| Case 25 Urban 10M 5RSU with LOS link only | 4.85 | 7.59 | 10.58 | 20.23 | No | No |
| Case 26 Urban 20M 5RSU with LOS link only | 4.14 | 6.14 | 8.31 | 15.62 | No | No |
| Case 27 Urban 40M 5RSU with LOS link only | 2.96 | 4.03 | 5.62 | 8.63 | No | No |
| Case 28 Urban 100M 5RSU with LOS link only | 2.68 | 3.55 | 4.8 | 7.43 | No | No |
| Case 29 Urban 10M 10RSU with LOS link only | 4.37 | 5.66 | 7.58 | 9.89 | No | No |
| Case 30 Urban 20M 10RSU with LOS link only | 3.23 | 4.09 | 5.18 | 6.99 | No | No |
| Case 31 Urban 40M 10RSU with LOS link only | 2.5 | 3.16 | 3.96 | 5.12 | No | No |
| Case 32 Urban 100M 10RSU with LOS link only | 2.14 | 2.63 | 3.24 | 4.13 | No | No |
| Case 33 Urban 20M 5RSU with sync error | 9.13 | 11.93 | 15.67 | 22.65 | No | No |
| Case 34 Urban 20M 10RSU with sync error | 6.29 | 7.87 | 9.44 | 11.18 | No | No |

According to the evaluation result, following observations can be captured:

Observation 1: For the evaluation result based on V2R absolute positioning in Highway scenario, the positioning accuracy based on staggered RSU outperforms the case with the symmetric RSU.

Observation 2: For the evaluation result based on V2R absolute positioning in Highway scenario

- The requirement of set A cannot be met in most evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 3: For V2R absolute positioning in Urban scenario, the difference of positioning accuracy between the cases with all link and with LOS link only is small.

Observation 4: For the evaluation result based on V2R absolute positioning in Urban scenario

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

2.2 Relative positioning

In this subsection, evaluation results of relative positioning in highway scenario and urban scenario are provided. The measurement algorithm based MUSIC is used for the timing estimation. VUE and RSU are used for relative positioning. The detailed parameters for each case is provided in Table 3.

Table 3 Assumptions of each case for relative positioning in Highway and Urban scenario

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- |
| Scenario | Highway | Highway | Highway | Highway | Highway |
| Bandwidth | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| Anchor type | VUE | VUE | VUE | VUE | VUE |
| Selected values of X(m) | 25 | 25 | 25 | 25 | 50 |
| RSU deployment for absolute positioning | / | / | / | / | / |
| Link type | All link | All link | All link | All link | All link |

| Case 6 | Case 7 | Case 8 | Case 9 | Case 10 | Case 11 | Case 12 | Case 13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Highway | Highway | Highway | Highway | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 50 | 50 | 50 | 100 | 100 | 100 | 100 | 10 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 14 | Case 15 | Case 16 | Case 17 | Case 18 | Case 19 | Case 20 | Case 21 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 10 | 10 | 10 | 25 | 25 | 25 | 25 | 50 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 22 | Case 23 | Case 24 | Case 25 | Case 26 | Case 27 | Case 28 | Case 29 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 50 | 50 | 50 | 10 | 10 | 10 | 10 | 25 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | LOS only | LOS only | LOS only | LOS only | LOS only |

| Case 30 | Case 31 | Case 32 | Case 33 | Case 34 | Case 35 | Case 36 |
| --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 25 | 25 | 25 | 50 | 50 | 50 | 50 |
| / | / | / | / | / | / | / |
| LOS only | LOS only | LOS only | LOS only | LOS only | LOS only | LOS only |

The positioning accuracy result is provided in Table 4.

Table 4 Accuracy result(m) in V2X scenario with relative positioning

| Case ID | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case1 Highway 10M V2V link X=25 | 1.36 | 1.79 | 2.42 | 3.59 | No | No |
| Case 2 Highway 20M V2V link X=25 | 0.84 | 1.31 | 1.99 | 2.81 | No | No |
| Case 3 Highway 40M V2V link X=25 | 0.59 | 0.96 | 1.39 | 2.22 | No | No |
| Case 4 Highway 100M V2V link X=25 | 0.35 | 0.62 | 1.02 | 1.71 | No | No |
| Case 5 Highway 10M V2V link X=50 | 1.69 | 2.58 | 3.81 | 5.76 | No | No |
| Case 6 Highway 20M V2V link X=50 | 1.23 | 2.05 | 3.01 | 4.47 | No | No |
| Case 7 Highway 40M V2V link X=50 | 0.91 | 1.46 | 2.46 | 3.77 | No | No |
| Case 8 Highway 100M V2V link X=50 | 0.54 | 0.96 | 1.57 | 2.65 | No | No |
| Case 9 Highway 10M V2V link X=100 | 2.62 | 4.3 | 7.08 | 11.44 | No | No |
| Case 10 Highway 20M V2V link X=100 | 2.01 | 3.43 | 5.64 | 9.68 | No | No |
| Case 11 Highway 40M V2V link X=100 | 1.57 | 2.88 | 4.75 | 7.87 | No | No |
| Case 12 Highway 100M V2V link X=100 | 0.96 | 1.85 | 3.33 | 5.74 | No | No |
| Case 13 Urban 10M V2V link X=10 | 1.36 | 1.73 | 2.37 | 3.72 | No | No |
| Case 14 Urban 20M V2V link X=10 | 0.8 | 1.07 | 1.67 | 3.27 | No | No |
| Case 15 Urban 40M V2V link X=10 | 0.49 | 0.77 | 1.39 | 3.08 | No | No |
| Case 16 Urban 100M V2V link X=10 | 0.3 | 0.53 | 1.05 | 2.92 | No | No |
| Case 17 Urban 10M V2V link X=25 | 2.35 | 3.95 | 6.36 | 10.52 | No | No |
| Case 18 Urban 20M V2V link X=25 | 1.81 | 3.41 | 5.95 | 9.88 | No | No |
| Case 19 Urban 40M V2V link X=25 | 1.3 | 2.68 | 5.02 | 8.9 | No | No |
| Case 20 Urban 100M V2V link X=25 | 0.86 | 1.99 | 3.76 | 7.89 | No | No |
| Case 21 Urban 10M V2V link X=50 | 5.22 | 9.63 | 16.08 | 25.34 | No | No |
| Case 22 Urban 20M V2V link X=50 | 4.22 | 8.19 | 14.19 | 23.61 | No | No |
| Case 23 Urban 40M V2V link X=50 | 3.26 | 7.03 | 12.2 | 21.96 | No | No |
| Case 24 Urban 100M V2V link X=50 | 2.27 | 5.37 | 10.61 | 20.82 | No | No |
| Case 25 Urban 10M V2V link X=10 with LOS link only | 1.24 | 1.53 | 1.88 | 2.34 | No | No |
| Case 26 Urban 20M V2V link X=10 with LOS link only | 0.72 | 0.9 | 1.14 | 1.52 | No | No |
| Case 27 Urban 40M V2V link X=10 with LOS link only | 0.43 | 0.56 | 0.78 | 1.21 | Yes | No |
| Case 28 Urban 100M V2V link X=10 with LOS link only | 0.25 | 0.37 | 0.53 | 0.83 | Yes | No |
| Case 29 Urban 10M V2V link X=25 with LOS link only | 1.87 | 2.75 | 4.27 | 6.26 | No | No |
| Case 30 Urban 20M V2V link X=25 with LOS link only | 1.32 | 2.2 | 3.57 | 5.45 | No | No |
| Case 31 Urban 40M V2V link X=25 with LOS link only | 0.87 | 1.56 | 2.56 | 4.08 | No | No |
| Case 32 Urban 100M V2V link X=25 with LOS link only | 0.55 | 1.06 | 1.87 | 3.06 | No | No |
| Case 33 Urban 10M V2V link X=50 with LOS link only | 2.88 | 5.23 | 8.44 | 13.46 | No | No |
| Case 34 Urban 20M V2V link X=50 with LOS link only | 2.35 | 4.18 | 6.67 | 10.74 | No | No |
| Case 35 Urban 40M V2V link X=50 with LOS link only | 1.66 | 3.15 | 5.47 | 8.79 | No | No |
| Case 36 Urban 100M V2V link X=50 with LOS link only | 1.04 | 2.06 | 3.67 | 6.29 | No | No |

According to the evaluation result, following observations can be captured:

Observation 5: For the evaluation result based on V2V relative positioning in Highway and Urban scenario,

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 6: For the evaluation result based on V2V relative positioning in Urban scenario with LOS link only.

- The requirement of set A can be met in partial evaluation cases(e.g. Case 27 Urban 40M V2V link X=10 with LOS link only).

- The requirement of set B cannot be met in all evaluation cases.

Observation 7: For V2V relative positioning in Urban scenario, the positioning accuracy of cases with LOS link only outperforms the cases that with all links.

2.3 Ranging

2.3.1 Distance

In this subsection, evaluation results of ranging of distance in highway scenario and urban scenario are provided. The measurement algorithm based on MUSIC is used for the timing estimation. VUE and RSU are used for positioning. The detailed parameters for each case are provided in Table 5.

Table 5 Assumptions of each case for ranging distance in Highway and Urban scenario

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- |
| Scenario | Highway | Highway | Highway | Highway | Highway |
| Bandwidth | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| Anchor type | VUE | VUE | VUE | VUE | VUE |
| Selected values of X(m) | 25 | 25 | 25 | 25 | 50 |
| RSU deployment for absolute positioning | / | / | / | / | / |
| Link type | All link | All link | All link | All link | All link |

| Case 6 | Case 7 | Case 8 | Case 9 | Case 10 | Case 11 | Case 12 | Case 13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Highway | Highway | Highway | Highway | Highway |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | RSU |
| 50 | 50 | 50 | 100 | 100 | 100 | 100 | 25 |
| / | / | / | / | / | / | / | staggered |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 14 | Case 15 | Case 16 | Case 17 | Case 18 | Case 19 | Case 20 | Case 21 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Highway | Highway | Highway | Highway | Highway |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 25 | 25 | 25 | 50 | 50 | 50 | 50 | 100 |
| staggered | staggered | staggered | staggered | staggered | staggered | staggered | staggered |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 22 | Case 23 | Case 24 | Case 25 | Case 26 | Case 27 | Case 28 | Case 29 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Highway | Highway | Highway | Highway | Highway |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 100 | 100 | 100 | 25 | 25 | 25 | 25 | 50 |
| staggered | staggered | staggered | symmetric | symmetric | symmetric | symmetric | symmetric |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 30 | Case 31 | Case 32 | Case 33 | Case 34 | Case 35 | Case 36 | Case 37 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Highway | Highway | Highway | Highway | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | VUE |
| 50 | 50 | 50 | 100 | 100 | 100 | 100 | 10 |
| symmetric | symmetric | symmetric | symmetric | symmetric | symmetric | symmetric | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 38 | Case 39 | Case 40 | Case 41 | Case 42 | Case 43 | Case 44 | Case 45 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 10 | 10 | 10 | 25 | 25 | 25 | 25 | 50 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 46 | Case 47 | Case 48 | Case 49 | Case 50 | Case 51 | Case 52 | Case 53 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 50 | 50 | 50 | 10 | 10 | 10 | 10 | 25 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | LOS only | LOS only | LOS only | LOS only | LOS only |

| Case 54 | Case 55 | Case 56 | Case 57 | Case 58 | Case 59 | Case 60 | Case 61 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | RSU |
| 25 | 25 | 25 | 50 | 50 | 50 | 50 | 10 |
| / | / | / | / | / | / | / | / |
| LOS only | LOS only | LOS only | LOS only | LOS only | LOS only | LOS only | All link |

| Case 62 | Case 63 | Case 64 | Case 65 | Case 66 | Case 67 | Case 68 | Case 69 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 10 | 10 | 10 | 25 | 25 | 25 | 25 | 50 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 70 | Case 71 | Case72 | Case 73 | Case 74 | Case75 | Case76 | Case 77 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 50 | 50 | 50 | 10 | 10 | 10 | 10 | 25 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | LOS only | LOS only | LOS only | LOS only | LOS only |

| Case 78 | Case 79 | Case 80 | Case 81 | Case 82 | Case 83 | Case 84 |
| --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz |
| RSU | RSU | RSU | RSU | RSU | RSU | RSU |
| 25 | 25 | 25 | 50 | 50 | 50 | 50 |
| / | / | / | / | / | / | / |
| LOS only | LOS only | LOS only | LOS only | LOS only | LOS only | LOS only |

The positioning accuracy result is provided in Table 6.

Table 6 Distance accuracy result(m) in V2X scenario with ranging

| Case ID | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Highway 10M V2V link X=25 | 0.94 | 1.09 | 1.2 | 1.312 | Yes | No |
| Case 2 Highway 20M V2V link X=25 | 0.457 | 0.55 | 0.614 | 0.669 | Yes | No |
| Case 3 Highway 40M V2V link X=25 | 0.23 | 0.275 | 0.315 | 0.347 | Yes | Yes |
| Case 4 Highway 100M V2V link X=25 | 0.12 | 0.144 | 0.162 | 0.18 | Yes | Yes |
| Case 5 Highway 10M V2V link X=50 | 0.89 | 1.07 | 1.19 | 1.305 | Yes | No |
| Case 6 Highway 20M V2V link X=50 | 0.455 | 0.546 | 0.617 | 0.674 | Yes | No |
| Case 7 Highway 40M V2V link X=50 | 0.232 | 0.279 | 0.316 | 0.351 | Yes | Yes |
| Case 8 Highway 100M V2V link X=50 | 0.12 | 0.144 | 0.162 | 0.183 | Yes | Yes |
| Case 9 Highway 10M V2V link X=100 | 0.89 | 1.06 | 1.19 | 1.305 | Yes | No |
| Case 10 Highway 20M V2V link X=100 | 0.458 | 0.551 | 0.62 | 0.685 | Yes | No |
| Case 11 Highway 40M V2V link X=100 | 0.234 | 0.281 | 0.319 | 0.359 | Yes | Yes |
| Case 12 Highway 100M V2V link X=100 | 0.12 | 0.144 | 0.166 | 0.19 | Yes | Yes |
| Case 13 Highway 10M V2R link staggered RSU X=25 | 0.79 | 1.03 | 1.167 | 1.31 | Yes | No |
| Case 14 Highway 20M V2R link staggered RSU X=25 | 0.47 | 0.552 | 0.668 | 0.733 | Yes | No |
| Case 15 Highway 40M V2R link staggered RSU X=25 | 0.246 | 0.309 | 0.402 | 0.491 | Yes | Yes |
| Case 16 Highway 100M V2R link staggered RSU X=25 | 0.119 | 0.16 | 0.191 | 0.457 | Yes | Yes |
| Case 17 Highway 10M V2R link staggered RSU X=50 | 0.835 | 1.04 | 1.167 | 1.32 | Yes | No |
| Case 18 Highway 20M V2R link staggered RSU X=50 | 0.434 | 0.545 | 0.631 | 0.733 | Yes | No |
| Case 19 Highway 40M V2R link staggered RSU X=50 | 0.226 | 0.296 | 0.358 | 0.442 | Yes | Yes |
| Case 20 Highway 100M V2R link staggered RSU X=50 | 0.12 | 0.16 | 0.215 | 0.372 | Yes | Yes |
| Case 21 Highway 10M V2R link staggered RSU X=100 | 0.835 | 1.01 | 1.167 | 1.34 | Yes | No |
| Case 22 Highway 20M V2R link staggered RSU X=100 | 0.436 | 0.54 | 0.639 | 0.745 | Yes | No |
| Case 23 Highway 40M V2R link staggered RSU X=100 | 0.226 | 0.3 | 0.377 | 0.492 | Yes | Yes |
| Case 24 Highway 100M V2R link staggered RSU X=100 | 0.12 | 0.179 | 0.245 | 0.421 | Yes | Yes |
| Case 25 Highway 10M V2R link symmetric RSU X=25 | 0.81 | 0.99 | 1.16 | 1.33 | Yes | No |
| Case 26 Highway 20M V2R link symmetric RSU X=25 | 0.5 | 0.58 | 0.66 | 1.02 | Yes | No |
| Case 27 Highway 40M V2R link symmetric RSU X=25 | 0.25 | 0.31 | 0.34 | 0.48 | Yes | Yes |
| Case 28 Highway 100M V2R link symmetric RSU X=25 | 0.13 | 0.16 | 0.21 | 0.28 | Yes | Yes |
| Case 29 Highway 10M V2R link symmetric RSU X=50 | 0.82 | 1.01 | 1.21 | 1.35 | Yes | No |
| Case 30 Highway 20M V2R link symmetric RSU X=50 | 0.44 | 0.53 | 0.64 | 0.78 | Yes | No |
| Case 31 Highway 40M V2R link symmetric RSU X=50 | 0.24 | 0.29 | 0.34 | 0.43 | Yes | Yes |
| Case 32 Highway 100M V2R link symmetric RSU X=50 | 0.13 | 0.17 | 0.22 | 0.31 | Yes | Yes |
| Case 33 Highway 10M V2R link symmetric RSU X=100 | 0.8 | 1.03 | 1.22 | 1.39 | Yes | No |
| Case 34 Highway 20M V2R link symmetric RSU X=100 | 0.42 | 0.55 | 0.66 | 0.8 | Yes | No |
| Case 35 Highway 40M V2R link symmetric RSU X=100 | 0.25 | 0.3 | 0.37 | 0.49 | Yes | Yes |
| Case 36 Highway 100M V2R link symmetric RSU X=100 | 0.13 | 0.17 | 0.24 | 0.38 | Yes | Yes |
| Case 37 Urban 10M V2V link X=25 | 0.947 | 1.16 | 1.33 | 1.74 | No | No |
| Case 38 Urban 20M V2V link X=25 | 0.487 | 0.629 | 0.787 | 1.38 | Yes | No |
| Case 39 Urban 40M V2V link X=25 | 0.255 | 0.346 | 0.481 | 1.12 | Yes | No |
| Case 40 Urban 100M V2V link X=25 | 0.124 | 0.172 | 0.25 | 0.91 | Yes | No |
| Case 41 Urban 10M V2V link X=50 | 0.972 | 1.214 | 1.47 | 2.53 | No | No |
| Case 42 Urban 20M V2V link X=50 | 0.526 | 0.695 | 1.01 | 2.33 | No | No |
| Case 43 Urban 40M V2V link X=50 | 0.279 | 0.413 | 0.723 | 2.12 | No | No |
| Case 44 Urban 100M V2V link X=50 | 0.137 | 0.209 | 0.675 | 1.94 | No | No |
| Case 45 Urban 10M V2V link X=100 | 1.05 | 1.37 | 2.13 | 4.66 | No | No |
| Case 46 Urban 20M V2V link X=100 | 0.605 | 0.916 | 1.96 | 4.25 | No | No |
| Case 47 Urban 40M V2V link X=100 | 0.353 | 0.679 | 1.79 | 3.62 | No | No |
| Case 48 Urban 100M V2V link X=100 | 0.176 | 0.654 | 1.81 | 3.25 | No | No |
| Case 49 Urban 10M V2V link X=25 with LOS link only | 0.92 | 1.11 | 1.23 | 1.35 | Yes | No |
| Case 50 Urban 20M V2V link X=25 with LOS link only | 0.45 | 0.57 | 0.66 | 0.76 | Yes | No |
| Case 51 Urban 40M V2V link X=25 with LOS link only | 0.21 | 0.28 | 0.35 | 0.44 | Yes | Yes |
| Case 52 Urban 100M V2V link X=25 with LOS link only | 0.1 | 0.13 | 0.16 | 0.2 | Yes | Yes |
| Case 53 Urban 10M V2V link X=50 with LOS link only | 0.92 | 1.11 | 1.24 | 1.39 | Yes | No |
| Case 54 Urban 20M V2V link X=50 with LOS link only | 0.45 | 0.57 | 0.66 | 0.78 | Yes | No |
| Case 55 Urban 40M V2V link X=50 with LOS link only | 0.22 | 0.28 | 0.35 | 0.45 | Yes | Yes |
| Case 56 Urban 100M V2V link X=50 with LOS link only | 0.1 | 0.13 | 0.17 | 0.21 | Yes | Yes |
| Case 57 Urban 10M V2V link X=100 with LOS link only | 0.92 | 1.1 | 1.24 | 1.42 | Yes | No |
| Case 58 Urban 20M V2V link X=100 with LOS link only | 0.45 | 0.57 | 0.67 | 0.81 | Yes | No |
| Case 59 Urban 40M V2V link X=100 with LOS link only | 0.22 | 0.29 | 0.36 | 0.46 | Yes | Yes |
| Case 60 Urban 100M V2V link X=100 with LOS link only | 0.1 | 0.14 | 0.17 | 0.21 | Yes | Yes |
| Case 61 Urban 10M V2R link X=25 | 1.01 | 1.44 | 2.51 | 5.02 | No | No |
| Case 62 Urban 20M V2R link X=25 | 0.67 | 1.19 | 2.45 | 4.58 | No | No |
| Case 63 Urban 40M V2R link X=25 | 0.48 | 1.07 | 2.15 | 3.41 | No | No |
| Case 64 Urban 100M V2R link X=25 | 0.42 | 1.18 | 1.88 | 2.76 | No | No |
| Case 65 Urban 10M V2R link X=50 | 1.17 | 1.73 | 2.57 | 5.45 | No | No |
| Case 66 Urban 20M V2R link X=50 | 0.78 | 1.69 | 2.88 | 4.95 | No | No |
| Case 67 Urban 40M V2R link X=50 | 0.68 | 1.49 | 2.57 | 4 | No | No |
| Case 68 Urban 100M V2R link X=50 | 0.61 | 1.46 | 2.31 | 3.28 | No | No |
| Case 69 Urban 10M V2R link X=100 | 1.24 | 2.07 | 3.47 | 6.59 | No | No |
| Case 70 Urban 20M V2R link X=100 | 1.05 | 1.99 | 3.54 | 6.01 | No | No |
| Case 71 Urban 40M V2R link X=100 | 0.99 | 1.99 | 3.24 | 5.01 | No | No |
| Case 72 Urban 100M V2R link X=100 | 1.02 | 1.98 | 2.78 | 4.06 | No | No |
| Case 73 Urban 10M V2R link X=25 with LOS link only | 1.18 | 1.45 | 2.24 | 3.34 | No | No |
| Case 74 Urban 20M V2R link X=25 with LOS link only | 0.44 | 0.86 | 1.75 | 2.56 | No | No |
| Case 75 Urban 40M V2R link X=25 with LOS link only | 0.32 | 0.35 | 1.31 | 2.18 | No | No |
| Case 76 Urban 100M V2R link X=25 with LOS link only | 0.24 | 0.31 | 1.01 | 1.65 | No | No |
| Case 77 Urban 10M V2R link X=50 with LOS link only | 1.45 | 1.93 | 2.71 | 3.82 | No | No |
| Case 78 Urban 20M V2R link X=50 with LOS link only | 0.51 | 0.91 | 1.89 | 3.06 | No | No |
| Case 79 Urban 40M V2R link X=50 with LOS link only | 0.35 | 0.84 | 1.57 | 2.53 | No | No |
| Case 80 Urban 100M V2R link X=50 with LOS link only | 0.13 | 0.67 | 1.28 | 2.01 | No | No |
| Case 81 Urban 10M V2R link X=100 with LOS link only | 1.44 | 2.04 | 3.05 | 5.01 | No | No |
| Case 82 Urban 20M V2R link X=100 with LOS link only | 0.74 | 1.67 | 2.49 | 3.96 | No | No |
| Case 83 Urban 40M V2R link X=100 with LOS link only | 0.35 | 1.34 | 1.93 | 3.34 | No | No |
| Case 84 Urban 100M V2R link X=100 with LOS link only | 0.21 | 1.1 | 1.51 | 2.58 | No | No |

According to the evaluation result, following observations can be captured:

Observation 8: The performance of distance error for ranging distance positioning changes slightly when the value of X increases in the Highway scenario.

Observation 9: The performance of distance error for ranging distance positioning degrades when the value of X increases in the Urban scenario.

Observation 10: The performance of distance error for ranging distance positioning between two VUEs always outperforms that of one VUE and one RSU.

Observation 11:  For the evaluation result based on V2R ranging distance positioning in Highway scenario.

- The requirement of set A can be met in all evaluation cases.

- The requirement of set B can be met in partial evaluation cases(e.g. case 7 the bandwidth is equal to or above 40MHz)

Observation 12: For the evaluation result based on V2V ranging distance positioning in Highway scenario.

- The requirement of set A can be met in all evaluation cases.

- The requirement of set B can be met in partial evaluation cases(e.g. case 7 the bandwidth is equal to or above 40MHz).

Observation 13: For the evaluation result based on V2R ranging distance positioning in Urban scenario with all links.

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 14: For the evaluation result based on V2V ranging distance positioning in Urban scenario with all links.

- The requirement of set A can be met in partial evaluation cases.

- The requirement of set B cannot be met in all evaluation cases

Observation 15: For the evaluation result based on V2R ranging distance positioning in Urban scenario with LOS links.

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 16: For the evaluation result based on V2V ranging distance positioning in Urban scenario with LOS links.

- The requirement of set A can be met in all evaluation cases.

- The requirement of set B can be met in partial evaluation cases(e.g. the bandwidth is equal to or above 40MHz)

    - Angle

In this subsection, evaluation results of relative positioning in highway scenario and urban scenario are provided. The measurement algorithm based on MUSIC is used for the timing estimation. VUE and RSU are used for relative positioning. The detailed parameters for each case are provided in Table 7.

Evaluation results of ranging of angle in urban scenario are provided in Figure 9. And AOA algorithm is used for the simulation.

Table 7 Assumptions of each case for ranging angle in Highway and Urban scenario

| Parameters | Case 1 | Case 2 | Case 3 | Case 4 | Case 5 |
| --- | --- | --- | --- | --- | --- |
| Scenario | Highway | Highway | Highway | Highway | Highway |
| Bandwidth | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| Anchor type | VUE | VUE | VUE | VUE | VUE |
| Selected values of X(m) | 25 | 25 | 25 | 25 | 50 |
| RSU deployment for absolute positioning | / | / | / | / | / |
| Link type | All link | All link | All link | All link | All link |

| Case 6 | Case 7 | Case 8 | Case 9 | Case 10 | Case 11 | Case 12 | Case 13 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Highway | Highway | Highway | Highway | Highway | Highway | Highway | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 50 | 50 | 50 | 100 | 100 | 100 | 100 | 10 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 14 | Case 15 | Case 16 | Case 17 | Case 18 | Case 19 | Case 20 | Case 21 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 10 | 10 | 10 | 25 | 25 | 25 | 25 | 50 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | All link | All link | All link | All link | All link |

| Case 22 | Case 23 | Case 24 | Case 25 | Case 26 | Case 27 | Case 28 | Case 29 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz | 10MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 50 | 50 | 50 | 10 | 10 | 10 | 10 | 25 |
| / | / | / | / | / | / | / | / |
| All link | All link | All link | LOS only | LOS only | LOS only | LOS only | LOS only |

| Case 30 | Case 31 | Case 32 | Case 33 | Case 34 | Case 35 | Case 36 |
| --- | --- | --- | --- | --- | --- | --- |
| Urban | Urban | Urban | Urban | Urban | Urban | Urban |
| 20MHz | 40MHz | 100MHz | 10MHz | 20MHz | 40MHz | 100MHz |
| VUE | VUE | VUE | VUE | VUE | VUE | VUE |
| 25 | 25 | 25 | 50 | 50 | 50 | 50 |
| / | / | / | / | / | / | / |
| LOS only | LOS only | LOS only | LOS only | LOS only | LOS only | LOS only |

The positioning accuracy result is provided in Table 8.

Table 8 Angle accuracy result(degree) in V2X scenario with ranging

| Case ID | 50% | 67% | 80% | 90% | Whether meet the requirement of set A | Whether meet the requirement of set B |
| --- | --- | --- | --- | --- | --- | --- |
| Case 1 Highway 10M V2V link X=25 | 3.79 | 6.2 | 9.18 | 13.03 | Yes | No |
| Case 2 Highway 20M V2V link X=25 | 2.7 | 4.73 | 7.14 | 11.94 | Yes | No |
| Case 3 Highway 40M V2V link X=25 | 2.1 | 3.59 | 6.28 | 8.36 | Yes | No |
| Case 4 Highway 100M V2V link X=25 | 1.3 | 2.43 | 3.93 | 5.83 | Yes | Yes |
| Case 5 Highway 10M V2V link X=50 | 3.49 | 5.48 | 8.04 | 11.64 | Yes | No |
| Case 6 Highway 20M V2V link X=50 | 2.67 | 4.23 | 6.38 | 9.66 | Yes | No |
| Case 7 Highway 40M V2V link X=50 | 2.13 | 3.41 | 5.32 | 7.84 | Yes | Yes |
| Case 8 Highway 100M V2V link X=50 | 1.25 | 2.3 | 3.63 | 5.78 | Yes | Yes |
| Case 9 Highway 10M V2V link X=100 | 3.33 | 5.45 | 8.08 | 11.56 | Yes | No |
| Case 10 Highway 20M V2V link X=100 | 2.66 | 4.39 | 6.67 | 10.08 | Yes | No |
| Case 11 Highway 40M V2V link X=100 | 2.12 | 3.53 | 5.4 | 7.91 | Yes | Yes |
| Case 12 Highway 100M V2V link X=100 | 1.31 | 2.37 | 3.89 | 6.03 | Yes | Yes |
| Case 13 Urban 10M V2V link X=10 | 8 | 12.23 | 18.39 | 27.7 | No | No |
| Case 14 Urban 20M V2V link X=10 | 5.03 | 8.24 | 13.62 | 25.22 | No | No |
| Case 15 Urban 40M V2V link X=10 | 3.29 | 5.65 | 10.88 | 21.11 | No | No |
| Case 16 Urban 100M V2V link X=10 | 2.25 | 4.15 | 8.06 | 20.33 | No | No |
| Case 17 Urban 10M V2V link X=25 | 9.39 | 15 | 23 | 35.06 | No | No |
| Case 18 Urban 20M V2V link X=25 | 7.31 | 13.07 | 20.8 | 33.1 | No | No |
| Case 19 Urban 40M V2V link X=25 | 5.1 | 9.83 | 16.9 | 30.2 | No | No |
| Case 20 Urban 100M V2V link X=25 | 3.47 | 7.21 | 13.73 | 28.51 | No | No |
| Case 21 Urban 10M V2V link X=50 | 11.75 | 19.79 | 29.26 | 43.11 | No | No |
| Case 22 Urban 20M V2V link X=50 | 9.46 | 16.64 | 25.81 | 41.66 | No | No |
| Case 23 Urban 40M V2V link X=50 | 7.26 | 14.08 | 23.15 | 40.13 | No | No |
| Case 24 Urban 100M V2V link X=50 | 5.03 | 10.48 | 20.4 | 39.45 | No | No |
| Case 25 Urban 10M V2V link X=10 with LOS link only | 6.72 | 10.19 | 14.11 | 19.5 | No | No |
| Case 26 Urban 20M V2V link X=10 with LOS link only | 4.09 | 6.3 | 9.04 | 13.55 | Yes | No |
| Case 27 Urban 40M V2V link X=10 with LOS link only | 2.71 | 4.16 | 6.47 | 9.9 | Yes | No |
| Case 28 Urban 100M V2V link X=10 with LOS link only | 1.78 | 2.87 | 4.58 | 7.14 | Yes | Yes |
| Case 29 Urban 10M V2V link X=25 with LOS link only | 7.64 | 11.67 | 17 | 23.8 | No | No |
| Case 30 Urban 20M V2V link X=25 with LOS link only | 5.6 | 9.18 | 13.92 | 19.6 | No | No |
| Case 31 Urban 40M V2V link X=25 with LOS link only | 3.7 | 6.31 | 10.19 | 15.43 | No | No |
| Case 32 Urban 100M V2V link X=25 with LOS link only | 2.46 | 4.42 | 7.28 | 11.7 | Yes | No |
| Case 33 Urban 10M V2V link X=50 with LOS link only | 8.49 | 13.4 | 19.23 | 25.74 | No | No |
| Case 34 Urban 20M V2V link X=50 with LOS link only | 6.2 | 10.42 | 15.35 | 21.58 | No | No |
| Case 35 Urban 40M V2V link X=50 with LOS link only | 4.49 | 7.79 | 12.07 | 17.14 | No | No |
| Case 36 Urban 100M V2V link X=50 with LOS link only | 2.88 | 5.13 | 8.55 | 13.2 | Yes | No |

According to the evaluation result, following observations can be captured:

Observation 17: For V2V link in Highway, majority case can achieve the requirement of set A and some cases with high bandwidth can achieve the requirement of set B.

Observation 18: For V2V link in Urban scenario, all the case with all links cannot achieve the requirement of set A.

Observation 19: For V2V link in Urban scenario, the angle error accuracy of cases with LOS link only outperforms the cases that with all links.

Observation 20: For V2V link in Urban scenario, the angle error accuracy of cases with LOS link only can meet the requirement of set A only when the value of X is small.

  - Summary

Based on the evaluation result of V2X positioning, the following proposals should be captured.

Proposal 1: For absolute positioning and relative positioning in V2X scenario, the accuracy requirement of set B should be dropped in current release.

Proposal 2: For ranging distance in V2X scenario, the accuracy requirement of set B should be dropped in current release.

Proposal 3: For ranging angle in V2X scenario, the accuracy requirement of set B should be dropped in current release.

- Conclusion

In the contribution, we provide our considerations on the evaluation methodology for NR sidelink positioning with the following proposals.

Observation 1: For the evaluation result based on V2R absolute positioning in Highway scenario, the positioning accuracy based on staggered RSU outperforms the case with the symmetric RSU.

Observation 2: For the evaluation result based on V2R absolute positioning in Highway scenario.

- The requirement of set A cannot be met in most evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 3: For V2R absolute positioning in Urban scenario, the difference of positioning accuracy between the cases with all link and with LOS link only is small.

Observation 4: For the evaluation result based on V2R absolute positioning in Urban scenario

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 5: For the evaluation result based on V2V relative positioning in Highway and Urban scenario,

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 6: For the evaluation result based on V2V relative positioning in Urban scenario with LOS link only.

- The requirement of set A can be met in partial evaluation cases(e.g. Case 27 Urban 40M V2V link X=10 with LOS link only).

- The requirement of set B cannot be met in all evaluation cases.

Observation 7: For V2V relative positioning in Urban scenario, the positioning accuracy of cases with LOS link only outperforms the cases that with all links.

Observation 8: The performance of distance error for ranging distance positioning changes slightly when the value of X increases in the Highway scenario.

Observation 9: The performance of distance error for ranging distance positioning degrades when the value of X increases in the Urban scenario.

Observation 10: The performance of distance error for ranging distance positioning between two VUEs always outperforms that of one VUE and one RSU.

Observation 11:  For the evaluation result based on V2R ranging distance positioning in Highway scenario.

- The requirement of set A can be met in all evaluation cases.

- The requirement of set B can be met in partial evaluation cases(e.g. case 7 the bandwidth is equal to or above 40MHz)

Observation 12: For the evaluation result based on V2V ranging distance positioning in Highway scenario.

- The requirement of set A can be met in all evaluation cases.

- The requirement of set B can be met in partial evaluation cases(e.g. case 7 the bandwidth is equal to or above 40MHz).

Observation 13: For the evaluation result based on V2R ranging distance positioning in Urban scenario with all links.

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in all evaluation cases.

Observation 14: For the evaluation result based on V2V ranging distance positioning in Urban scenario with all links.

- The requirement of set A can be met in partial evaluation cases.

- The requirement of set B cannot be met in all evaluation cases

Observation 15: For the evaluation result based on V2R ranging distance positioning in Urban scenario with LoS links.

- The requirement of set A cannot be met in all evaluation cases.

- The requirement of set B cannot be met in partial evaluation cases(e.g. the bandwidth is equal to or above 40MHz)

Observation 16: For the evaluation result based on V2V ranging distance positioning in Urban scenario with LOS links.

- The requirement of set A can be met in all evaluation cases.

- The requirement of set B can be met in partial evaluation cases(e.g. the bandwidth is equal to or above 40MHz).

Observation 17: For V2V link in Highway, majority case can achieve the requirement of set A and some cases with high bandwidth can achieve the requirement of set B.

Observation 18: For V2V link in Urban scenario, all the case with all links cannot achieve the requirement of set A.

Observation 19: For V2V link in Urban scenario, the angle error accuracy of cases with LOS link only outperforms the cases that with all links.

Observation 20: For V2V link in Urban scenario, the angle error accuracy of cases with LOS link only can meet the requirement of set A only when the value of X is small.

Proposal 1: For absolute positioning and relative positioning in V2X scenario, the accuracy requirement of set B should be dropped in current release.

Proposal 2: For ranging distance in V2X scenario, the accuracy requirement of set B should be dropped in current release.

Proposal 3: For ranging angle in V2X scenario, the accuracy requirement of set B should be dropped in current release.

# Appendix

Table 9 Simulation assumption for absolute positioning in V2V/V2R link

| Parameter | V2V/V2R absolute positioning |
| --- | --- |
| Carrier frequency | 6GHz |
| Subcarrier spacing | 30kHz |
| Reference Signal Transmission Bandwidth | 10 MHz /20MHz/40 MHz /100MHz |
| Number of UEs or RSUs | 5 or 10 |
| Number of symbols used per occasion | 1 |
| number of occasions used per positioning estimate | 1 |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution/threshold based |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series |
| Network synchronization assumptions | Perfect sync/Max sync error is 50ns |
| Positioning method | RTT |

Table 10 Simulation assumption for relative positioning in V2V link

| Parameter | V2V relative positioning |
| --- | --- |
| Carrier frequency | 6GHz |
| Subcarrier spacing | 30kHz |
| Reference Signal Transmission Bandwidth | 10 MHz /20MHz/40 MHz /100MHz |
| Number of UEs or RSUs | 5 or 10 |
| Number of symbols used per occasion | 1 |
| number of occasions used per positioning estimate | 1 |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution/threshold based |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series |
| Network synchronization assumptions | Perfect sync |
| Positioning method | AOA |
| Relative distance | {25 50 100}m for highway scenario {10 25 50}m for urban scenario |

Table 11 Simulation assumption for ranging of distance in V2V/V2R link

| Parameter | V2V/V2R distance ranging |
| --- | --- |
| Carrier frequency | 6GHz |
| Subcarrier spacing | 30kHz |
| Reference Signal Transmission Bandwidth | 10 MHz /20MHz/40 MHz /100MHz |
| X value | 25/50/100m |
| Number of symbols used per occasion | 1 |
| number of occasions used per positioning estimate | 1 |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution/threshold based |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series |
| Network synchronization assumptions | Perfect sync |
| Positioning method | RTT |
| Relative distance | {25 50 100}m for highway scenario {10 25 50}m for urban scenario |

Table 12 Simulation assumption for ranging of angle in V2V link

| Parameter | V2V angle ranging |
| --- | --- |
| Carrier frequency | 6GHz |
| Subcarrier spacing | 15kHz |
| Reference Signal Transmission Bandwidth | 20MHz |
| Number of UE | 1 |
| Number of symbols used per occasion | 1 |
| number of occasions used per positioning estimate | 1 |
| interference modelling (ideal muting, or other) | ideal muting |
| Description of Measurement Algorithm (e.g. super resolution, interference cancellation, ….) | super resolution |
| Description of positioning technique / applied positioning algorithm (e.g. Least square, Taylor series, etc) | taylor series |
| Network synchronization assumptions | Perfect sync |
| VUE antenna number | 4 |
| Positioning method | AOA |
| Relative distance | {25 50 100}m for highway scenario {10 25 50}m for urban scenario |