3GPP TSG RAN WG1 #110bis-e		R1-2208876

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.1.4.2

Source:	Google

Title:	On SRI/TPMI Indication for 8Tx Transmission

Document for:	Discussion/Decision

# Introduction

In RAN1 #110, the following agreements on SRI/TPMI indication for 8Tx transmission have been achieved.

Agreement 8TX PUSCH is supported in Rel-18 Agreement For 8TX PUSCH, at least support - Ng=1, 2, 4 Note: The above does not restrict the Ng for the non-coherent case Agreement For evaluation purpose of codebook alternatives when a precoder based on Rel-15 DL Type I is used, following oversampling ratios are assumed - (O1, O2) = (1,1), (2,1), (2,2) - Note: Other values may be used and reported by companies - Note: When deciding the supported O1, O2 combination, the signalling overhead, performance, UE complexity, etc should be considered Agreement RAN1 further studies Alt1b and Alt2a for down-selection of one of the two in RAN1 meeting #110b-e. - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives. Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item. Agreement For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources - Alt3: Support both alternatives. Agreement Study low overhead solutions for SRI and/or transmitter precoder matrix indication for codebook-based, and SRI indication for non-codebook-based UL transmission by an 8TX UE, - FFS using single or separate (exiting or new) fields for the indication, other solutions are not precluded. - Note: Low overhead schemes for study include those using Rel-15 SRI/TPMI indication mechanisms

In this contribution, we provide some discussions on SRI/TPMI indication for 8Tx transmission.

# Discussion

## Antenna coherency

The antenna coherency should consider different coherency levels:

- Antenna coherency within a panel

- Antenna coherency across panels

Since the target UE for this enhancement is not for phone, it can be assumed that coherent transmission within a panel is always supported. But the coherent transmission across panels may not be supported. Thus, only the full coherent and partial-coherent transmission need to be considered, where the partial coherent transmission supports coherent transmission within a panel.

Proposal 1: The enhancement of 8Tx transmission supports both coherent and partial coherent transmission, where the partial coherent transmission assumes coherent transmission within a panel.

## Codebook design

In RAN1 #109, the following alternatives on codebook design have been agreed for further study.

Agreement For 8TX UE codebook-based uplink transmission, down-select one of - Alt1-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully/partially-coherent UEs - Alt1-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of the codebook for partially/non-coherent UEs - Study NR Rel-15 DL Type I codebook as the starting point for design of the codebook for fully-coherent UEs - Alt2-a: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt2-b: - Study NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) in combination with those based on NR Rel-15 DL Type I codebooks as the starting point for design of codebook for fully/partially/non-coherent UEs - Alt3: - Study NR Rel-15 DL Type I codebook as the starting point for design of codebook for fully/partially/non-coherent UEs - Transmission using one or multiple precoders corresponding to one or multiple SRS resources can be studied as part of the above alternatives.

In RAN1 #110, it was agreed to further down-select the codebook design principle from Alt1-b and Alt2-a. The Rel-15 DL Type1 multi-panel codebook support coherent transmission across panels. However, for a UE with multiple panels, usually it is challenging to support coherent transmission. Therefore, the coherent transmission should only be considered for UE with a single panel. The partial coherent transmission can be considered for UEs with multiple panels to support panel selection. Therefore, compared to Alt1-b, Alt2-a is preferred.

Proposal 2: Support to define the 8Tx UL codebook based on NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs (Alt2-a).

## Number of codewords

In RAN1 #110, it was agreed that up to 8 layers can be supported for uplink transmission. Based on the following agreement, the number of codewords for more than 4 layers transmission could be a problem.

Agreement Support up to X layers for codebook and non-codebook UL transmission for 8TX UE where X=4, 8 is determined based on separate UE capability - For uplink transmission with rank<=4, single CW is supported - For uplink transmission with rank>4, whether single or dual CW is used will be decided in RAN1 meeting #110b-e The above applies only with regards to the work scope of this agenda item.

Currently, 2 CW is required for rank>4 downlink transmission. To keep the symmetric design, 2 CW should be required for rank>4 uplink transmission. Otherwise, the UE complexity could get increased as UE needs to implement two different codeword-to-layer mapping schemes. Further the energy for each rank could be quite different, single CW cannot provide good performance as two CWs.

Proposal 3: For uplink transmission with rank>4, two CWs are supported based on the downlink codeword-to-layer mapping scheme.

## Number of PT-RS ports

For uplink 8Tx transmission, for coherent transmission, usually the 8Tx comes from a single panel. Thus, the number of PT-RS ports should be 1. For partial-coherent transmission, as discussed in section 2.2, usually the coherent transmission across uplink panels is challenging from UE implementation point of view. Thus, the partial-coherent transmission should be based on panel selection, where the UE can only transmit the uplink signals from one panel. Then one PT-RS port should be sufficient. Therefore, for 8Tx transmission, a single PT-RS port is sufficient.

Proposal 4: Support up to 1 PT-RS port for 8Tx transmission.

## Number of SRS resource sets

In RAN1 #110, it was agreed to further down-select one of the alternatives on SRS resource set configuration for non-codebook based transmission scheme as follows:

Agreement For SRS configuration for non-codebook UL transmission for an 8TX UE, down-select from - Alt1: A single SRS resource set configured with up to 8 single-port SRS resources - Alt2: Up to two SRS resource sets, each configured with up to 4 single-port SRS resources - Alt3: Support both alternatives.

In Rel-17, to configure 2 SRS resource sets are used as an implicit indication to enable the mTRP operation for PUSCH. Thus, to configure 2 SRS resource sets could unnecessarily complicate the system. Further, simultaneous transmission for 2 SRS resource sets is not allowed, but simultaneous transmission for SRS resources within a resource set for NCB is allowed. Therefore, Alt1 is preferred.

Proposal 5: Support a single SRS resource set for NCB configured with up to 8 single-port SRS resources (Alt1).

# Conclusion

In this contribution, we provided discussion on 8Tx enhancement. Based on the discussion, the following proposals have been achieved.

Proposal 1: The enhancement of 8Tx transmission supports both coherent and partial coherent transmission, where the partial coherent transmission assumes coherent transmission within a panel.

Proposal 2: Support to define the 8Tx UL codebook based on NR Rel-15 UL 2TX/4TX codebooks and/or 8x1 antenna selection vector(s) as the starting point for design of codebook for fully/partially/non-coherent UEs (Alt2-a).

Proposal 3: For uplink transmission with rank>4, two CWs are supported based on the downlink codeword-to-layer mapping scheme.

Proposal 4: Support up to 1 PT-RS port for 8Tx transmission.

Proposal 5: Support a single SRS resource set for NCB configured with up to 8 single-port SRS resources (Alt1).