3GPP TSG-RAN WG1 Meeting #110bis-e 	    R1-2208944

# e-Meeting, October 10th – 19th, 2022

Title:	[Draft] Reply LS on TCI state list for UL TCI

Response to: 	R1-2208354/R4-2214972

Release:	Rel-17

Work Items:	NR_feMIMO-Core

Source:	CATT [to be RAN WG1]

To:	RAN WG4

Cc:	RAN WG2

Contact Person:

Name:

E-mail Address:

Attachments:

1. Overall Description:

RAN1 thanks RAN4 for the LS on TCI state list for UL TCI. Please find RAN1 answers on the questions below.

Question 1: What is the relationship between qd and active UL (or joint) TCI state list? How does network configure the RS resource indexes qd ?

Answer 1: If the UE is configured and activated dl-OrJoint-TCIStateList-r17,  qd is associated with dl-OrJoint-TCIStateList-r17. If the UE is configured and activated TCI-UL-State-r17, qd is included in the TCI-UL-State-r17.

RS resource indexes qd is the index of the reference signal (SSB/CSI-RS) configured in PUSCH-PathlossReferenceRS/ PUCCH-PathlossReferenceRS

Question 2: Is UE expected to maintain the pathloss RSs of all the pathloss reference RS in the active UL (or joint) TCI list?

Answer 2: In RAN1 #104-e meeting, it was agreed that the total number of maintained PL-RSs per CC is no more than 4.

Question 3: What is the UE behavior when number of active UL (or joint) TCI states is larger than 4 if corresponding pathloss RSs need to be maintained?

Answer 3: The UE behaviour is not specified in the current spec if active UL(or joint) TCI states is larger than 4. From RAN1‘s point of view, a predefined rule for UE behaviour can be specified for this case, e.g. UE can delete the pathloss RSs maintained according to order of the activated UL(or joint) TCI states ID or the pathloss RS ID.

2. Actions:

To RAN4 group:

ACTION: 	RAN1 respectfully asks RAN4 to take the above information into account.

3. Date of Next TSG-RAN WG1 Meetings:

TSG-RAN WG1 Meeting #111				Nov. 14 to Nov. 18, 2022	Toulouse, FR

TSG-RAN WG1 Meeting #112				Feb. 27 to Mar. 3, 2023		Athens, GR