3GPP TSG RAN WG1 #110bis-e		    													             R1-2209038

e-Meeting, October 10th – 19th, 2022

Source: 	Intel Corporation

Title:	Discussion on UE features for NR coverage enhancement

Agenda item:	8.16.1

Document for:	Discussion and Decision

# Introduction

At the RAN1#110 meeting, the following agreements were made regarding UE features for NR coverage enhancement [1].

Agreement:

- Apply following proposals in R1-2207702.

  - 2-2-1

  - 2-3-8

  - 2-3-10

  - 2-3-13

  - 2-3-14

  - 2-3-16

  - 2-3-21

  - 2-5-1

  - 2-1-1

  - 2-1-2

  - 2-1-3

  - 2-2-2

  - 2-2-3

  - 2-2-4 with adding “without repetition”

Agreement:

- Apply following proposals in R1-2207937.

  - 2-3-1 with removing “, as defined in TS 38.101-1 & 38.101-2”

  - 2-3-9

  - 2-3-11

  - 2-3-12

  - 2-3-15

Agreement:

- Apply following proposals in R1-2208115.

  - 2-3-17 with option 1

  - 2-4-1 with “4-23 or 25-2”

In the contribution, we discuss UE features for NR coverage enhancement. The updated UE features for NR coverage enhancement after RAN1#109-e meeting was captured in [2] and are listed in the Appendix.

# Discussion on UE features for NR coverage enhancement

## UE features for DMRS bundling for PUSCH and PUCCH

At the RAN1#107b-e meeting, the following agreement was made for DMRS bundling restart during a nominal TDW [3].

Agreement Update the description of the RRC parameters PUSCH-Window-Restart and PUCCH-Window-Restart as follows. - UE bundles PUSCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements - UE bundles PUCCH DM-RS remaining in a nominal time domain window after event(s) triggered by DCI or MAC-CE that violate power consistency and phase continuity requirements Note: Events which are triggered by DCI or MAC CE, but regarded as semi-static events, e.g. frequency hopping, UL beam switching for multi-TRP operation, or other if defined, are excluded.

According to the agreement, it is evident that subject to UE capability, UE restarts DMRS bundling within a nominal TDW only for the dynamic events that violate power consistency and phase continuity. For semi-static events that not triggered by DCI or MAC-CE, UE is mandatory to restart the DMRS bundling within a normal TDW. Hence, the description on FG30-4g needs to be updated.

Further, for DMRS bundling, UE is required to maintain power consistency and phase continuity during an actual TDW. In order to meet the requirement, UE may need to consider or implement different RF components for different bands or in FR1 and FR2, especially when considering that residual absolute frequency offset may have different characteristics in low band and mmWave band.

Based on reply LS [4], RAN4 has not concluded on whether the UE capability signalling to support each of these configurations should be defined per-band, or per-band per band combination. For DMRS bundling, UE is required to maintain power consistency and phase continuity during an actual TDW. In order to meet the requirement, UE may need to consider or implement different RF components for different bands or in FR1 and FR2, especially when considering that residual absolute frequency offset may have different characteristics in low band and mmWave band. Hence, it may be more appropriate to define UE feature groups for DMRS bundling per band.

Based on the discussions above, Table 1 illustrates suggested updates for UE feature groups for DMRS bundling.

Table 1. UE feature groups for DMRS bundling

| Index | Feature group | Components | Consequence if the feature is not supported by the UE |
| --- | --- | --- | --- |
| 30-4g | [Restart DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] | [UE does not support restarting DM-RS bundling after the events triggered by DCI or MAC-CE that violate power consistency and phase continuity] |

Proposal 1

- For UE feature groups of DMRS bundling for PUSCH and PUCCH

  - UE features for DMRS bundling are defined per band.

  - Consider Table 1 for UE feature groups of DMRS bundling.

# Conclusions

In this contribution, we discussed UE features for NR coverage enhancement. Further, we summarize the proposals as follows:

Proposal 1

- For UE feature groups of DMRS bundling for PUSCH and PUCCH

  - UE features for DMRS bundling are defined per band.

  - Consider Table 1 for UE feature groups of DMRS bundling.

# References

- Chairman’s notes, RAN1 #110 Meeting, August 2022

- R1-2207923, “Updated RAN1 UE features list for Rel-17 NR after RAN1 #110 Thursday”, August 2022

- Chairman’s notes, RAN1 #107b e-Meeting, January 2022

- R1-2205715, “Reply LS to RAN1/RAN2 on DMRS bundling”, August 2022

# Appendix:

| Features | Index | Feature group | Components | Prerequisite feature groups | Need for the gNB to know if the feature is supported | Applicable to the capability signalling exchange between UEs (Sidelink WI only)”. | Consequence if the feature is not supported by the UE | Type (the ‘type’ definition from UE features should be based on the granularity of 1) Per UE or 2) Per Band or 3) Per BC or 4) Per FS or 5) Per FSPC) | Need of FDD/TDD differentiation | Need of FR1/FR2 differentiation | Capability interpretation for mixture of FDD/TDD and/or FR1/FR2 | Note | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 30. NR_cov_enh | 30-1 | Increased maximum number of PUSCH Type A repetitions | Maximum value of K (the number of repetitions) = 32 For DG PUSCH, the number of repetitions is indicated in a TDRA list. A row index of the TDRA list is indicated by a DCI. For Type 1 CG PUSCH, the number of repetitions is indicated by repK-r17 For Type 2 CG PUSCH, the number of repetitions is indicated in a TDRA list or by repK-r17. | One of {5-14, 5-16, 11-6} | Yes | N/A | UE does not support more than 16 repetitions PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-2 | PUSCH Type A repetitions based on available slots | Transmission occasions for repetitions for dynamic and configured grant PUSCH are determined on the basis of available slots. | One of {5-14, 5-16, 5-17} | Yes | N/A | UE does not support dynamic or configured grant PUSCH repetitions counted on the basis of available slots. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3 | TB processing over multi-slot PUSCH | Support of TB processing over multi-slot PUSCH for DG and Type 2 CG without repetition in RRC connected mode. |  | Yes | N/A | UE does not support TB processing over multi-slot PUSCH. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-3a | Repetition of TB processing over multi-slot PUSCH | Support repetition of TB processing over multi-slot PUSCH in RRC connected mode. | 30-3 | Yes | N/A | UE does not support repetition of TB processing over multi-slot PUSCH | Per band | N/A | N/A | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4 | The maximum duration for DM-RS bundling | The maximum duration during which UE is able to maintain power consistency and phase continuity to support DM-RS bundling for PUSCH/PUCCH |  | Yes | N/A | UE does not support DM-RS bundling for PUSCH/PUCCH | Per band | N/A | N/A | N/A | Candidate values for the maximum duration for FDD are {4, 8, 16, 32} Candidate values for the maximum duration for TDD are {2, 4, 8, 16} NOTE: DM-RS bundling is only applicable for UL transmissions with pi/2 BPSK, BPSK, and QPSK modulation orders for the corresponding physical channels. | Optional with capability signalling |
| 30. NR_cov_enh | 30-4a | DM-RS bundling for PUSCH repetition type A | Support DM-RS bundling for PUSCH repetition type A over consecutive symbols | 30-4 and one of {5-14, 5-16, 5-17} | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type A | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4b | DM-RS bundling for PUSCH repetition type B | Support DM-RS bundling for PUSCH repetition type B over consecutive symbols | 30-4, 11-5 | Yes | N/A | UE does not Support DM-RS bundling for PUSCH repetition type B | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4c | DM-RS bundling for TB processing over multi-slot PUSCH | Support DM-RS bundling for TB processing over multi-slot PUSCH over consecutive symbols | 30-4, 30-3 | Yes | N/A | UE does not Support DM-RS bundling for TB processing over multi-slot PUSCH | [Per UE] | FFS | [No] | [N/A] | Note: If a UE reports support of FG 30-3a and 30-4c, the UE supports DMRS bundling for the repetitions of TBoMS | Optional with capability signalling |
| 30. NR_cov_enh | 30-4d | DMRS bunding for PUCCH repetitions | Support DM-RS bundling for PUCCH repetitions for PUCCH formats 1/3/4 over consecutive symbols | 30-4, 4-23 | Yes | N/A | UE does not support DMRS bunding for PUCCH repetitions | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4e | Enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | Support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | 30-4a or 30-4b or 30-4c | Yes | N/A | UE does not support enhanced inter-slot frequency hopping with inter-slot bundling for PUSCH | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4f | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | 30-4d | Yes | N/A | UE does not support Enhanced inter-slot frequency hopping for PUCCH repetitions with DMRS bundling | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4g | [Restart DM-RS bundling after the events that violate power consistency and phase continuity] | [Support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | 30-4 | Yes | N/A | [UE does not support restarting DM-RS bundling after the events that violate power consistency and phase continuity] | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-4h | DM-RS bundling for non-back-to-back transmission | Support DM-RS bundling for non-back-to-back transmission for consecutive slots for PUSCH and PUCCH only for corresponding supported back-to-back transmission FGs (30-4a, 30-4b, 30-4c, or 30-4d) | 30-4a, 30-4b, 30-4c, or 30-4d | Yes | N/A | UE does not Support DM-RS bundling for non-back-to-back transmission | [Per UE] | FFS | [No] | [N/A] |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-5 | Slot based dynamic PUCCH repetition indication | Support slot based dynamic PUCCH repetition indication for PUCCH formats 0/1/2/3/4 support slot based dynamic PUCCH repetition for PUCCH formats 0/1/2/3/4 | 4-23 or 25-2 | Yes | N/A | UE does not support Dynamic PUCCH repetition indication | Per UE | No | No | N/A |  | Optional with capability signalling |
| 30. NR_cov_enh | 30-6 | Repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI | Support of repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI |  | Yes | N/A | UE does not support repetition of PUSCH transmission scheduled by RAR UL grant and DCI format 0_0 with CRC scrambled by TC-RNTI. | Per band | N/A | N/A | N/A |  | Optional with capability signalling |