3GPP TSG RAN WG1#110-bis-e	        	R1-2209110

Online, Oct. 10th – 19th, 2022

Agenda Item:					    9.8.1

Source:	Sony

Title:	Considerations on side control information and NCR behavior

Document for:	Discussion and decision

# Introduction

During RAN#97-e meeting, RAN approved Rel-18 WID on NCR in RP-222673 [1][2]. It contains the following objectives:

The objectives of NR NCR WI follow the recommendations defined in TR 38.867 and will focus on scenarios and assumption listed below:

- Network-controlled repeaters are inband RF repeaters used for extension of network coverage on FR1 and FR2 bands based on the NCR model in TR38.867

- For only single hop stationary network-controlled repeaters

- The NCR is transparent to the UE.

- Network-controlled repeater can maintain the gNB-repeater link and repeater-UE link simultaneously

With these considerations, NR NCR supports the following features:

Specify the signaling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2]

- Beamforming

- UL-DL TDD operation

- ON-OFF information

Note: Power control aspect will be checked in RAN#98e.

Specify control plane signaling and procedures [RAN2, RAN1]

- The configuration of signaling for side control information indication

  - NOTE: Down-selection of solutions in section 7.2 of TR 38.867 is needed

Specify the solution of network-controlled repeater management (i.e., the identification and authorization/validation of NCR) [RAN3, RAN2]

- NOTE: Down-selection of solutions in section 8 of TR 38.867 is needed taking into account the feedback of other working groups (i.e., SA3 and SA5). From a security point of view, the feasibility of NCR validation procedure in solution 1 and the feasibility of solution 2 will be decided by SA3.The selected solution shall provide inter-vendor interoperability.

Study the RRM functions to be supported and specify the RRM requirements of NCR-MT if necessary [RAN2, RAN4]

Study and specify the RF and EMC requirements of NCR if necessary [RAN4]

Note: The existing requirements defined in RAN4 can be reused if applicable.

Note: The work in RAN4 for beam related is expected to start on FR2 first.

In this contribution, we focus on the RAN1-related aspects. In particular, we suggest changes to the standard to support the necessary side control information (SCI) for controlling the NCR-Fwd, namely, beamforming, UL/DL TDD configuration and ON/OFF information, and related signaling.

# NCR model and assumptions

A model of an NCR is given in TR 38.867 [3]. For convenience, we reproduce this model in Figure 1 below.

Figure 1. Model of NCR as captured in TR 38.867.

In this contribution, we focus on the case where the NCR-MT and the NCR-Fwd operate in the same frequency range, and at least one of the NCR-MT's carrier(s) is within the set of carriers forwarded by the NCR-Fwd. The discussion of other possible cases, such as when at least some of the NCR-MT's carriers and the set of carriers forwarded by the NCR-Fwd, are at least partially in different frequency ranges or bandwidth parts, can be postponed to future releases.

# NCR model

One can entertain the model shown in Figure 2. What is pictured is the state of the NCR-Fwd. At each time instant t∈{0,…,T-1}, the NCR-Fwd state is completely specified by the quantities Active∈{ON, OFF}, Direction∈{UL, DL}, B-beam ∈{0,…,NTCINCR,MT-1} and A-beam ∈{0,…,NbeamNCR,Fwd-1}, which reflect the side control information (SCI) agreed in the Rel-18 WID [2].

The granularity of the time index t can be slots, OFDM symbols, or whatever time-domain (TD) granularity agreed by 3GPP. For our purpose, we assume that the index t runs over OFDM symbols. The period T represents the time span that NCR-Fwd can "see:" NCR-Fwd time wraps around modulo T. We assume that T is large enough so that new SCI indications do not overlap with old but not-yet-executed SCI indications. The exact value of T is irrelevant to our discussion. In the frequency domain (FD), we assume that the NCR-Fwd has been configured to forward signals within a specific band (or bandwidth part, carrier, and so on), which, once configured, does not change. The quantity NTCINCR,MT represents the number of TCI states of the NCR-MT. The network configures these and, as agreed in earlier discussions [1][2], they are eligible for the NCR-Fwd backhaul link. Last, the quantity NbeamNCR,Fwd represents the number of beams available to the NCR-Fwd access link; this number does not exist in legacy specs and needs to be agreed upon between the network and the NCR, e.g., by signaling or by a pre-defined rule.

Figure 2. Model of NCR-Fwd function unit.

With the above, the network's job is to make sure that the state of the NCR-Fwd is correctly defined at all times. This is accomplished by exchanging SCI signals with the NCR-MT or enforcing pre-defined rules. Valid states are Active=OFF, regardless of Direction, B-beam, and A-beam, in which case the NCR-Fwd does not forward any signal, and Active=ON, in which case the behavior of the NCR-Fwd is dictated by the configured values of Direction, B-beam, and A-beam. If at any time t any of the quantities Active, Direction, B-beam, and A-beam has not been configured, the behavior of the NCR-Fwd is as if Active=OFF.

In the following, we provide our views on the detailed design of the NCR-Fwd state, the associated SCI, and the NCR behavior.

# Discussion

## Beam information

The following description, found in [3], is relevant to the design of the SCI related to beam information for the backhaul link:

Regarding the use of "fixed" beams, we note that by appropriately configuring the TCI states of the NCR-MT, the gNB can always fix the beam of the C-link. As a consequence, the beam of the backhaul link is also fixed. Conversely, the NCR-MT can feature a single beam and still comply with the TCI framework. It follows that the use of fixed beams by the NCR-MT can be seen as transparent to the gNB, and therefore only adaptive beams need to be considered:

- By appropriately configuring the TCI states of the NCR-MT, the gNB can always fix the beam of both the C-link and the backhaul link. Therefore, "fixed beams" are not needed.

- Fixed beams at NCR for both C-link and backhaul link do not need to be considered further. Only adaptive beams should be considered.

There is also the question of whether the NCR-Fwd can determine the beam of the backhaul link from the C-link's TCI states or whether an additional indication from gNB is needed. In Rel-16, multiple TCI states might be relevant for the C-link. For example, in the DL, there might be:

- a TCI state associated with a select SSB occasion;

- a TCI state associated with the latest decoded DCI from the serving cell;

- a TCI state indicated for the latest received PDSCH;

- and so on.

In the uplink, we also have a few possibilities:

- a TCI state indicated for the latest transmitted PUCCH;

- a TCI state indicated for the latest transmitted PUSCH;

- a TCI state indicated for the latest transmitted SRS;

- and so on.

Although these states could be the same one, this is generally not true. Therefore, a way to select from one of the available TCI states of the C-link is needed. To reduce SCI signaling overhead, a pre-defined rule should be used whenever more than one C-link TCI states are eligible for the backhaul link. New signaling may also be acceptable, provided that it is semi-static signaling, i.e., the backhaul link only needs to be configured once during the initial NCR configuration. At least the new signaling should not be required very often, to reduce the signaling overhead.

In our understanding, the situation is different in Rel-17. The unified TCI framework dispenses with redundant TCI states and allows a single TCI state for downlink or uplink transmissions. Since only one TCI state is active for the C-link, this same state can be used for the backhaul link beam. Neither a pre-defined rule nor new signaling is therefore needed. We make the following observations and proposals.

- If the unified TCI framework is adopted for the C-link, the selection of the backhaul link beam is straightforward. No new signaling or pre-defined rules are needed.

- To determine the NCR-Fwd beam for the backhaul link from the TCI states of the C-link, strive after pre-defined rules to keep SCI signaling overhead low. Static new signaling can also be considered.

Regarding beam information to control the behavior of the access link, the following agreements are captured in TR 38.867 [3]:

In our view, both semi-static indications and dynamic indications may be needed. For example, beam information for the NCR-Fwd access link could be semi-statically signaled when it is associated with physical channels occurring periodically or semi-persistently, such as SS/PBCH in the DL and PRACH in the UL. Conversely, beam information related to PDSCH transmissions in the DL and non-preconfigured PUSCH transmissions in the UL may be advantageously signaled to the NCR using dynamic indications. Therefore, both dynamic and semi-static indications can be considered. Regarding the details of each indication and the maximum number of beams configured for the NCR-Fwd access link, these should be discussed during the RAN1#110-bis meeting.

- For signaling beam information of access link for NCR-Fwd, consider both dynamic and semi-static indications.

## Handling of flexible symbols

The following has been captured in TR 38.867 [3]:

As explained earlier, the NCR-Fwd follows the TDD UL/DL configuration of the NCR-MT. There is no ambiguity for OFDM symbols configured as UL or DL symbols; if active, the NCR-Fwd forwards signals in the configured direction, UL or DL. For OFDM symbols configured as flexible symbols, the situation is more complicated. Some flexible symbols may behave as DL or UL symbols according to pre-defined rules. Some may be re-configured at "run-time" as DL or UL OFDM symbols via dynamic TDD UL/DL signaling. Yet some others may remain as flexible symbols. Below, we recap the general rules governing the behavior of flexible symbols, discuss the three options listed in TR 38.867 [3], and provide some recommendations.

### Legacy behavior during flexible symbols

A UE can be configured with a cell-specific UL/DL TDD configuration [6]:

Figure 3. The tdd-UL-DL-ConfigCommon IE.

The above information element (IE) defines a P-periodic time slot structure consisting of dslot⋅Nsymslot+dsym DL OFDM symbols, followed by F flexible symbols, and ending with uslot⋅Nsymslot+usym UL symbols, where

F=dslot+uslot⋅Nsymslot-dsym-usym,

and whereas, S=P⋅2μref is an integer number, and P, dsym, usym, μref are signaled by IE tdd-UL-DL-ConfigCommon. This structure forms a UL/DL TDD pattern, illustrated in Figure 4. Generally, up to two cell-specific UL/DL TDD patterns can be configured. Moreover, the subcarrier spacing (SCS) of the active UL/DL BWP can be different from the reference SCS given by μref. For the sake of simplicity, we nonetheless gloss over these details and assume one UL/DL TDD pattern and subcarrier spacing μref in our discussion.

Figure 4. A UL/DL TDD pattern.

The network can modify the UEs' UL/DL TDD configurations by signaling tdd-UL-DL-ConfigCommon IEs to some UEs or all of them.

Figure 5. The tdd-UL-DL-ConfigDedicated IE.

Using tdd-UL-DL-ConfigDedicated IEs, a gNB may override flexible symbols into DL or UL symbols, i.e., F→D, F→U. Other overrides, i.e., D→U, U→F, and so on, are not allowed. This mechanism can accommodate different UL/DL traffic ratios. For example, some UEs with UL-intensive traffic may be configured so that all flexible symbols become UL symbols. However, for UEs served by an NCR, this only works if the NCR is aware of the UE's actual UL/DL TDD configuration and can relay UL and DL transmissions in flexible symbols. NCR awareness can be accomplished using pre-defined rules or dynamic UL/DL TDD configurations for flexible symbols.

#### Pre-defined rules for flexible symbols

Sec. 11.1 in [4] teaches that:

Hence, by scheduling (by a DCI) a PDSCH or a CSI-RS reception, the network can turn certain flexible symbols at the NCR-MT into DL ones. If active, the NCR-Fwd would follow suit and forward said PDSCH or CSI-RS (or, for that matter, whatever the DL traffic is, since the NCR-Fwd is agnostic to such matters) from the backhaul link and into the access link. The DCI itself, however, needs to be scheduled in DL-configured symbols. Similar behavior is specified for the UL. Figure 6 illustrates this mechanism.

Figure 6. Illustration of F→D and F→U symbol overriding.

#### Dynamic UL/DL TDD configuration of flexible symbols

The network can endow an NCR-MT with dynamic UL/DL TDD configurations using the slot format indicator (SFI) framework, while the UEs behind the NCR are still configured using semi-static UL/DL TDD configurations. Since the NCR-MT is configured to monitor PDCCH for DCI format 2_0, it can now receive DCIs in flexible symbols; see, e.g., [4], Sec. 11.1.1:

### New dynamic side control signaling

Option 3 in the highlight box above opens up the possibility of introducing new dynamic side control signaling of UL/DL forwarding in flexible symbols. In our view, however, this alternative can be given lower priority and only introduced if already standardized solutions cannot satisfactorily handle flexible symbols.

- Side control information can also be considered to handle flexible symbols. However, solutions involving already standardized UE functionality should be favored whenever possible.

We sum up our findings in this section if the following observations and proposals:

- Option 1 is equivalent to setting the NCR-Fwd in the OFF state and is therefore not needed.

- For handling the flexible symbols of UL/DL TDD configurations of NCR, Option 2, i.e., the NCR-Fwd follows the TDD operation of the NCR-MT as determined by SFI indication of scheduling by gNB. Option 3, i.e., new dynamic side control signaling of UL/DL forwarding, can be introduced if it is found that Option 2 cannot satisfactorily handle flexible symbols.

## ON/OFF information

ON/OFF information is recommented in TR 38.867 [3] to steer the behabior of the NCR-Fwd:

We discuss first Option 1. We believe that explicit indication of ON/OFF information with a semi-static or periodic ON/OFF pattern is an effective way to combat interference originating from NCR. One can also consider dynamic ON/OFF state indications, although we are unsure whether those are needed. The reason is that to obtain good statistics, one typically needs to measure interference over a longer time, which points instead in the direction of semi-static or periodic mechanisms. In addition, DRX-like patterns for ON/OFF information can be considered for power saving. Note, however, that ON/OFF patterns and DRX-like patterns serve different purposes. More specifically, an NCR configured to an IDLE or INACTIVE state is expected to neither transmit nor receive any packet; an NCR configured to an OFF state, while not allowed to transmit or forward waveforms, might be expected to perform tasks that only involve receiving, such as listening to side control information on the C-link.

Regarding Option 2, the implicit indication of ON/OFF information via, e.g., beam or UL/DL TDD configuration, can also be discussed.

- For indicating ON/OFF information for controlling the behavior of the NCR-Fwd, RAN1 should consider Option 1, Option 2, or a combination thereof. For example:

  - Explicit semi-static and periodic ON/OFF patterns and explicit dynamic ON/OFF states can be considered to combat interference.

  - Explicit DRX-like patterns can be considered for power saving.

  - Implicit indication of ON/OFF information via, e.g., beam, UL/DL TDD configuration, or PC information can also be considered.

To give an example, Figure 5 shows an ON/OFF or activity pattern superimposed with a DL/UL TDD pattern. In this example, the activity pattern has a transmission periodicity of 8 slots, while a four-slot long UL/DL TDD pattern has been configured. When the activity pattern is applied, the resulting UL/DL TDD pattern becomes 8-slot long, during two of which the NCR-RU is switched OFF—indicated by dashes.

Figure 7. Composition of an ON/OFF pattern, or activity pattern, and a UL/DL TDD pattern.

# Conclusions

We made the following observations and proposals:

- By appropriately configuring the TCI states of the NCR-MT, the gNB can always fix the beam of both the C-link and the backhaul link. Therefore, "fixed beams" are not needed.

- If the unified TCI framework is adopted for the C-link, the selection of the backhaul link beam is straightforward. No new signaling or pre-defined rules are needed.

- Side control information can also be considered to handle flexible symbols. However, solutions involving already standardized UE functionality should be favored whenever possible.

- Option 1 is equivalent to setting the NCR-Fwd in the OFF state and is therefore not needed.

- Fixed beams at NCR for both C-link and backhaul link do not need to be considered further. Only adaptive beams should be considered.

- To determine the NCR-Fwd beam for the backhaul link from the TCI states of the C-link, strive after pre-defined rules to keep SCI signaling overhead low. Static new signaling can also be considered.

- For signaling beam information of access link for NCR-Fwd, consider both dynamic and semi-static indications.

- For handling the flexible symbols of UL/DL TDD configurations of NCR, Option 2, i.e., the NCR-Fwd follows the TDD operation of the NCR-MT as determined by SFI indication of scheduling by gNB. Option 3, i.e., new dynamic side control signaling of UL/DL forwarding, can be introduced if it is found that Option 2 cannot satisfactorily handle flexible symbols.

- For indicating ON/OFF information for controlling the behavior of the NCR-Fwd, RAN1 should consider Option 1, Option 2, or a combination thereof. For example:

  - Explicit semi-static and periodic ON/OFF patterns and explicit dynamic ON/OFF states can be considered to combat interference.

  - Explicit DRX-like patterns can be considered for power saving.

  - Implicit indication of ON/OFF information via, e.g., beam, UL/DL TDD configuration, or PC information can also be considered.

# References

- 3GPP RP-22xxxx, "Draft report of 3GPP TSG RAN #97 eom," Chairman.

- 3GPP RP-222673, "New WID on NR network-controller repeaters," ZTE Corporation.

- 3GPP TR 38.867 v1.0.0, "Study on NR network-controlled repeaters."

- 3GPP TS 38.213 V17.2.0, "Physical layer procedures for control."

- 3GPP TS 38.214 V17.1.0, "Physical layer procedures for data."

- 3GPP TS 38.331 V17.1.0, "Radio resource control (RRC) protocol specification."