3GPP TSG RAN WG1 #110bis-e 	R1-2209244

e-Meeting, October 10th – 19th, 2022

Agenda item:		8.14

Source:	Nokia, Nokia Shanghai Bell

Title:	Maintenance on NB-IoT/eMTC support for Non-Terrestrial Network

Document for:		Discussion and Decision

# Introduction

In RAN1 #109 meeting, most of the discussion on IoT over NTN in RAN1 has been completed with agreements. Implicit signaling on epoch time is remaining for further discussion, which we will discuss and provide our views on these points. Additionally, we also provide our views and proposals for remaining issues, for completion of Rel17.

# Discussion

## Implicit signaling on epoch time

In the previous RAN1 meeting the following was discussed with regards to Epoch time for NTN SIB:

- Otherwise, when Epoch time is not explicitly indicated in SIB, epoch time of assistance information (i.e. Serving satellite ephemeris and Common TA parameters) is implicitly known as the end of the SI window during which the NTN-specific SIB is transmitted.

- Note: The NTN-specific SIB is expected to be updated per SI window

- Note: The UE shall not assume that the NTN-specific SIB is constant across SI windows

This discussion is in conflict with the current release 16 specification of TS 36.331, which in section “5.2.1.2a Scheduling for NB-IoT” states:

The UE is not required to accumulate several SI messages in parallel but may need to accumulate a SI message across multiple SI windows, depending on coverage condition.

Thus, the UE is allowed to accumulate SI messages across SI windows to ensure reliable reception and decoding of SI messages even in challenging propagation conditions. It may be problematic for NTN UEs to prohibit this accumulation feature, because their link budget is effectively reduced.

In the addition to the UE not being able to assume the NTN SIB is constant across SI windows, the network will also not necessarily update the assistance information for every SI window (being in the range 160-1600 ms for NB-IoT). There are multiple reasons for this including the fact that the ephemeris will be provided from the satellite control center and that it may not be delivered with sufficient granularity to warrant such frequent updates. Furthermore, the time domain difference between SI window and granularity of the ephemeris, provided by the satellite control center, means it may not be possible for the eNB to transmit the ephemeris in a SI window that fits the Epoch time and thus the implicit Epoch time will be incorrect if derived based on the SI window.

This means the UE cannot use the start or end of an SI window as an implicit indication of the Epoch time, because the NTN SIB of the SI window may not have been updated since the previous SI window, where the network may consider the Epoch to be started, or there will cause non-aligned understanding on epoch time between UE and network. Thus implicit signaling of the Epoch time does not work as illustrated in Figure 1.

Observation 1: Implicit indication of the Epoch time of assistance information in NTN SIB does not work, because the assistance information may not necessarily be updated every SI window.

Figure 1 Implicit Epoch time indication issue.

Therefore, only explicit signaling of Epoch time shall be specified for IoT NTN. Note, the same line of arguments are valid for NR NTN.

Proposal 1: Only explicit signaling of Epoch time for assistance information shall be specified for IoT NTN.

To ensure this is reflected in relevant specifications RAN1 should send an LS to RAN2 to update the epochTime field in SIB31 such that it becomes a mandatory field.

Proposal 2: RAN1 send LS to RAN2 to update SIB31 description in RRC specification to make the epochTime a mandatory field.

## Extension of legacy UE behavior on segment/dropping

In RAN1 #110 meeting, there were agreement for CR on UL precompensation for 36.213.

Agreement

Adopt the TP to TS 36.213 in Sections 4.2.3 Transmission timing adjustments for eMTC as follows:

- For a BL/CE UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of  time units, where the quantity  is provided by system information, as specified in 3GPP TS 36.331.

Agreement

Adopt the TP to TS 36.213 in Sections 16.1.2 Timing synchronization as follows:

- For a NB-IoT UE communicating over NTN, time and frequency pre-compensation is adjusted per uplink segment with a transmission duration of  time units, where the quantity  is provided by system information, as specified in 3GPP TS 36.331.

Agreement

Final CR is agreed in R1-2208238.

As agreements from RAN1, the segment size may be configured by SIB or by RRC signaling according to RAN1 #106-e and RAN1 #107-e meeting, it will be good to clarify it in 36.213 for UL precompensation related description, i.e. to add RRC signaling together with the system information in the description. It would be like as below for both NB-IoT and eMTC over NTN.

“where the quantity Nsegmentprecompensation is provided by system information or RRC configuration, as specified in 3GPP TS 36.331.”

Proposal 3: RAN1 should add RRC signaling to configure segment size for UL precompensation in 36.213.

The case of segmented transmission of NPRACH is challenging, because the network will not be aware that the UE is transmitting the preamble (for initial access) and will not be aware of the UE capability. The relevant RAN1#107 agreement is as follows:

For NB-IoT, UE pre-compensation per segment of NPRACH is applied from one segment to the next segment by using one or more of the following methods if supported by UE implementation - UE may drop / Insert samples - UE may blank subframe / repetition unit where UE drops a subframe / repetition unit The total transmission time is not changed FFS Details of method(s) to drop / insert samples / blank subframe / repetition unit FFS Specification impact For eMTC, UE pre-compensation per segment of PRACH is applied from one segment to the next segment by drop / insert samples in Guard Period of PRACH preamble. - The total transmission time is not changed - FFS Details of method(s) to drop / insert samples

According to the agreement the NB-IoT UE may drop samples, a subframe or a repetition unit, but as noted above the NW will not be aware of the UE capability and thus the NW cannot configure the type of dropping applied by the UE, but only the segment duration.

One simple approach may be for RAN1 to agree that the NB-IoT UE only applies sample dropping during segmented transmission of NPRACH, but it will be good to discuss the options.

Proposal 4: RAN1 to discuss NB-IoT UE segmented transmission of NPRACH, where network is not aware of the UE capability for dropping during segmented transmission.

The issue does not exist for eMTC where only sample dropping is possible according to the agreement.

Observation 2: eMTC UE may only apply sample dropping/insertion for segmented transmission of PRACH.

## Requirement on when to apply uplink segmentation gap

The RAN1 has agreed to use uplink segmentation to handle timing drift during a repetition period. This is reflected in TS 36.211 as follows:

TS 36.211 section 5.3.4 Mapping to physical resources For BL/CE UEs communicating over NTN, for PUSCH transmission, for frame structure type 1, after a transmission duration of Nsegmentprecompensation time units (which may include subframes that are not BL/CE UL subframes), a transmission gap of Ngapprecompensation time units shall be counted for the PUSCH resource mapping but not used for transmission of the PUSCH, according to the single UE capability ue-CE-NeedSegmentedPrecompensationGaps, as specified in 3GPP TS 36.331 [9]. The quantity Nsegmentprecompensation is provided by higher layers, and the quantity Ngapprecompensation is configured by higher layers based on the UE capability, if signalled.

The current specification indicates the UE after transmission of a segment shall count a transmission gap independently of how the transmission timing changes from the transmitted segment to the next segment. The reason may be that the text is based on the transmission gaps occurring every 256 ms in the terrestrial network specification, which do not consider the fast timing advance change of NTN, but rather allows UE to resynchronize to the downlink.

In our view this was not the intention of the IoT NTN specification. The reason is that the transmission gap is only needed, when the timing advance of the next segment is longer than the previous segment, because this results in the UE not being able to transmit the complete next segment.

Observation 3: The transmission for segmented uplink transmission is only needed when the timing advance of the next segment is longer than the previous segment.

Furthermore, if the UE applies a gap even when it is not needed, i.e. when the timing advance of the next segment is shorter than the previous, it will result in a degradation of the decoding performance because information is not transmitted in the gap.

Observation 4: If the transmission gap is applied, when the timing advance of the next segment is shorter than the previous, the decoding performance will decrease.

Therefore, we propose to update the specification text, such that it is clarified the UE shall only apply the gap when it is needed i.e. when the timing advance of the next segment is longer than the previous segment.

Proposal 5: RAN1 to clarify that segmented uplink transmission gap is only applied when the timing advance of the next segment is longer than the timing advance of the previous, transmitted segment.

We have provided text proposals to reflect the above for eMTC and NB-IoT UEs in TS 36.211 in [1].

# Conclusion

In this contribution, we discussed time and frequency synchronization for NB-IoT/eMTC over NTN, our observations and proposals are presented as following:

Observation 1: Implicit indication of the Epoch time of assistance information in NTN SIB does not work, because the assistance information may not necessarily be updated every SI window.

Observation 2: eMTC UE may only apply sample dropping/insertion for segmented transmission of PRACH.

Observation 3: The transmission for segmented uplink transmission is only needed when the timing advance of the next segment is longer than the previous segment.

Observation 4: If the transmission gap is applied, when the timing advance of the next segment is shorter than the previous, the decoding performance will decrease.

Proposal 1: Only explicit signaling of Epoch time for assistance information shall be specified for IoT NTN.

Proposal 2: RAN1 send LS to RAN2 to update SIB31 description in RRC specification to make the epochTime a mandatory field.

Proposal 3: RAN1 should add RRC signaling to configure segment size for UL precompensation in 36.213.

Proposal 4: RAN1 to discuss NB-IoT UE segmented transmission of NPRACH, where network is not aware of the UE capability for dropping during segmented transmission.

Proposal 5: RAN1 to clarify that segmented uplink transmission gap is only applied when the timing advance of the next segment is longer than the timing advance of the previous, transmitted segment.

# References

- R1-2210183, ”Draft CR on correction of IoT NTN with segment gap in 36.211”, Nokia, Nokia Shanghai Bell