3GPP TSG RAN WG1 #110bis-e			R1-2209284

e-Meeting, October 10th – 19th, 2022

Source: 	     Xiaomi

Title:		     Discussion on subband non-overlapping full duplex

Agenda item:    9.3.2

Document for:  Discussion

Introduction

In RAN1#110 e-meeting, subband non-overlapping full duplex was discussed and the following agreements were achieved:

Agreement: Study the following alternatives with Alt 4 prioritized, for SBFD operation at least for RRC_CONNECTED state. - SBFD operation Alt 1: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors follow existing specifications without introducing new UE behaviors for SBFD operation at gNB side. - SBFD operation Alt 2: - Time and frequency locations of subbands for SBFD operation are not known to UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs - SBFD operation Alt 3: - Only time location of subbands for SBFD operation is known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation - SBFD operation Alt 4: - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs. - UE behaviors for non-SBFD aware UEs follow existing specifications. - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation. UE capability discussion is held in work item phase. Agreement: For indication of subband locations for SBFD operation, study semi-static configuration of subband time and frequency location as baseline. Agreement For semi-static configuration of subband location, consider same subband frequency resources across different SBFD symbols as baseline. Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair Agreement For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, study the following options: - Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol - Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol - Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol - Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol Agreement Study the feasibility and potential benefit of UE-to-UE co-channel CLI measurement and reporting, which can be specific for SBFD, at least includes: - Measurement resource/reporting configuration - Measurement/reporting details (including UE processing delay) - Relevant information exchange (between gNBs) if needed - Usage of measurement at gNB Note: other enhancement(s) for gNB-to-gNB and UE-to-UE CLI handling specific for SBFD are not precluded.

In this contribution, we provide our analyses and views on the subband non-overlapping full duplex.

Discussion

Impacts on legacy UE from subband non-overlapping full duplex

Duplex operation allows simultaneous transmission and reception at gNB side. At UE side, it can either transmit or receive signal on an OFDM symbol.  For a Rel-18 duplex UE, gNB can either configure a UL subband explicitly or schedule uplink transmission in a DL slot without UL subband configuration. Consequently, there would be two kinds of resources in a DL slot, i.e. UL resources and DL resources.  One example is shown in Figure 1. For duplex UE, it only expects uplink scheduling in slot#1, slot#2 and slot#3 on the UL subband. While for legacy UE, there is no way to obtain information related to UL subband. Hence UL subband should be transparent for legacy UE. Accordingly, the following behaviour in Rel-15/16/17 should be maintained for legacy UE.

- UE doesn’t expect to transmit on DL symbols. In the other words, gNB should not schedule a legacy UE to transmit uplink on UL subband on DL symbols if exists.

- UE doesn’t expect to receive on UL symbols. In the other words, gNB should not schedule a legacy UE to receive downlink on DL subband on UL symbols if exists.

- UE can transmit or receive on flexible symbols depending on indication from gNB.

- UE does not expect to receive both dedicated higher layer parameters configuring transmission from the UE on the set of symbols of the slot and dedicated higher layer parameters configuring reception by the UE on the set of symbols of the slot.

Figure 1: Illustration of UL subband in DL slots

Proposal 1: Subband related to duplex operation is transparent for legacy UE and the following legacy behaviour should be kept:

- UE doesn’t expect to transmit on DL symbols.

- UE doesn’t expect to receive on UL symbols.

- UE can transmit or receive on flexible symbols depending on the indication from gNB

- UE does not expect conflict between DL reception and UL transmission on the same flexible OFDM symbol.

Discussion on transparent SBFD operation and non-transparent SBFD operation

For unpaired spectrum, slots within a TDD UL-DL configuration periodicity can be categorized into three types, i.e. DL, UL and flexible. Duplex UE can transmit uplink in a DL slot or receive downlink in a UL slot. While for flexible slot, current specification already allows gNB to schedule a UE with downlink reception and another UE with uplink transmission, in theory. Hence there is no standard impacts for supporting duplex operation at gNB side on flexible symbols. More critical issue is how to identify the UL resources in a DL slot or the DL resources in a UL slot. In RAN1#110 meeting, four alternatives for SBFD operations were agreed with prioritizing the study of alternative 4. Generally speaking, the identified alternatives can be classified into two mechanisms of determining UL subband, i.e. transparent UL subband and non-transparent UL subband.

- Non-transparent SBFD operation: UL subband on DL symbols is indicated explicitly, e.g. via RRC signalling. For SBFD operation Alt.3 and Alt.4, time and/or frequency resources of UL subband are known to UE, i.e., non-transparent SBFD operation is applied.

- Transparent SBFD operation: UL subband on DL symbols is derived from implicit scheme, i.e. UL subband is transparent for UE. For SBFD operation Alt.1 and Alt.2, time and frequency resources of UL subband are not known to UE, i.e.,transparent SBFD operation is applied. The following solutions are some examples:

  - E1) Duplex UE is allowed to transmit uplink in DL slots. Whether it transmits or receives depends on gNB’s scheduling or configuration. In this example, new UE behaviours must be defined.

  - E2) Network configures specific TDD UL-DL configuration for duplex UE, i.e. the DL slots containing UL subband are configured as UL slots for duplex UE. In this example, current UE behaviours are sufficient.

  - E3) Duplex operation is only allowed on flexible symbols. In order to facilitate duplex operation, network needs to configure flexible dominated TDD UL-DL structure. In this example, current UE behaviours are sufficient.

Flexibility on resource allocation

For non-transparent scheme, a UL subband is explicitly configured for a duplex UE in a DL slot. gNB can determine whether to schedule duplex UE to transmit on the UL subband or to receive on the DL resources based on realistic requirement. The UL subband is not necessarily configured within the active UL BWP, which brings more flexibility on UL scheduling.  One example is shown in Figure 2. In case 1, the UL subband is configured starting from the lowest RB in the active DL BWP, which is not contained by the active UL BWP. In case 2, the UL subband is configured within the frequency range occupied by the active UL BWP. On the other hand, the frequency resources available on the DL symbols for UL transmission are confined within the active UL BWP wherein transparent SBFD operation is applied.

Figure 2: Examples of UL subband configuration in DL slots

Observation 1: The frequency resources available on the DL symbols for a SBFD UE are confined within the active UL BWP if transparent SBFD operation, i.e. alt.1 or alt.2, is applied.

Intra-subband CLI mitigation

It is well known that cross link interference is one of the major challenges on implementing subband non-overlapping full duplex. It is much friendly for managing or mitigating CLI in the network if there is an explicit configuration of UL subband. For non-transparent SBFD operation, gNB configures UL subband in DL symbols with explicit signalling. Accordingly, it is easy to achieve aligned UL subband configuration in the network. There is no gNB-gNB intra-subband CLI and UE-UE intra-subband CLI as gNB can configure the UL subband in a cooperative manner. The nature of transparent SBFD operation is to determine the UL resources used for UL transmission in DL symbols totally up to individual gNB. In the other word, UL transmission can be any place in frequency domain from gNB perspective. Consequently, intra-subband CLI is inevitable which significantly degrades system performance. In order to mitigate intra-subband CLI, gNB has to exchange related information, e.g. the intended UL scheduling category. However, it is quite difficult or even impossible as the UL scheduling category is dynamic and individual.

Observation 2: It is difficult to mitigate intra-subband CLI if transparent SBFD operation is adopted.

Multiplexing between DL and UL on SBFD slots

There are three types of resource allocation, i.e. RA type 0, RA type 1 and dynamic switching. Considering one of the principles is that legacy UE should not be impacted by SBFD operation, UL subband should be transparent to legacy UE. The DL resource allocation applicable to legacy UE is quite flexible. Therefore, it would be typical that the allocated legacy DL resources collide with UL subband or UL resources indicated on DL symbols. RB-symbol level rate matching patterns can be used for the purpose of multiplexing between legacy DL and UL subband.

For SBFD UE, it is complicated to realize the resources used for UL transmission on DL symbols if UL subband is determined in a transparent way. One example is shown in the following figure. It should be kept in mind that UL resources can be anywhere within an active DL BWP from UE perspective.  In the end, the UL resources occupied by SBFD UE is sporadic on DL symbols.  In Figure 3, assuming there are 4 SBFD UEs which need to transmit uplink data or uplink reference signal across slot#0 to slot#2. As they are indicated by UL grant without any restriction, the time-frequency resources occupied by UL transmission in each DL slot are diverse. Hence, it is difficult if not impossible to indicate the UL subband for rate matching via the current rate matching pattern. It should be noticed that it is not only for SBFD UE, but also for legacy UE.

Figure 3: Example of multiplexing between UL and DL on SBFD slots

In order to facilitate multiplexing between UL transmission and DL reception on SBFD slots, UL subband can be explicitly configured. In this way, any UL transmission is confined into a configured UL subband. gNB only needs to configure a RMR aligned with the UL subband.

Observation 3: Transparent SBFD operation, i.e. alt.1 or alt.2, complicates multiplexing between UL and DL on the SBFD slot.

Valid RO determination in SBFD slot

Currently, the time-frequency resources used for RACH is configured by SIB1. Additionally, UE determines which ROs are valid as below in TS38.213:

For unpaired spectrum, - if a UE is not provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last SS/PBCH block reception symbol, where Ngap is provided in Table 8.1-2 and, if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where the UE does not transmit [15, TS 37.213]. - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon , as described in clause 4.1 - If a UE is provided tdd-UL-DL-ConfigurationCommon, a PRACH occasion in a PRACH slot is valid if - it is within UL symbols, or - it does not precede a SS/PBCH block in the PRACH slot and starts at least Ngap symbols after a last downlink symbol and at least Ngap symbols after a last SS/PBCH block symbol, where Ngap is provided in Table 8.1-2, and if channelAccessMode = semistatic is provided, does not overlap with a set of consecutive symbols before the start of a next channel occupancy time where there shall not be any transmissions, as described in [15, TS 37.213] - the candidate SS/PBCH block index of the SS/PBCH block corresponds to the SS/PBCH block index provided by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon, as described in clause 4.1.

In summary, a valid RO cannot exist in DL symbols. It makes it impossible for a non-SBFD UE to transmit preamble on DL symbols. Even for a SBFD UE with transparent UL subband, it is not possible to transmit preamble on DL symbols as it would be an invalid RO according to the above definition. The following figure could be an example.

Figure 4: Example of multiplexing between UL and DL on SBFD slots

For non-transparent SBFD operation, SBFD UE has full knowledge on which resources are reserved for UL transmission. Therefore, it is confident to transmit preamble on the UL subband when random access is triggered, even if the RO is on the DL symbols. However, it needs to be further clarified whether it is possible or necessary to allow UE to transmit preamble on the UL subband.

For transparent SBFD operation, SBFD UE has to rely on the UL grant from gNB to transmit UL on SBFD slot. However, CBRA is event-based. gNB has no idea when and where a UE needs to transmit preamble. Hence it is not possible to introduce more valid RO for SBFD UE if UL subband is transparently determined.

Observation 4: If a RO exists in DL slots configured by tdd-UL-DL-ConfigurationCommon, it cannot be used for preamble transmission even if the RO locates in a transparent UL subband.

Proposal 2: Clarify whether to allow a SBFD UE to transmit preamble within the UL subband in a SBFD slot.

Standard impacts

Non-transparent SBFD operation needs additional RRC signalling. However, the only thing needed to identify a UL subband is the time-frequency resources. Other parameters, e.g. PUCCH-Config, PUSCH-Config, can be fully reused within the UL subband. From this perspective, the standard impacts introduced by non-transparent SBFD operation is trivial. For transparent SBFD operation, we may need to consider solutions to ensure the multiplexing between UL and DL on the SBFD slot.

Table 1: Summary of non-transparent UL subband and transparent UL subband

| Option | Pros | Cons |
| --- | --- | --- |
| Transparent SBFD operation, i.e. alt.1 and alt.2 | - Less standard impacts. - Legacy mechanisms of scheduling can be fully reused. | - Introduce restrictions on the frequency range available for UL scheduling - Not friendly for gNB coordination - Increase the complexity for gNB scheduling - More complicated for multiplexing between DL of legacy UE and UL of duplex UE. - Not friendly to reduce random access latency |
| Non-transparent SBFD operation, i.e. alt.3 and alt.4 | - Has more flexibility on UL subband configuration. - Friendly for interference mitigation. - Friendly for multiplexing between UL and DL in a SBFD slot - Provide more valid RO and reduce random access delay, if preamble transmission is allowed in SBFD slot | - Need to introduce UL subband configuration. |

Proposal 3: Non-transparent SBFD operation is sufficient and there is no need to further study transparent SBFD operation.

Regarding non-transparent SBFD operation, there are two alternatives identified in RAN1#110 meeting:

- SBFD operation Alt 3:

  - Only time location of subbands for SBFD operation is known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time location of subbands for SBFD operation

- SBFD operation Alt 4:

  - Both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

  - UE behaviors for non-SBFD aware UEs follow existing specifications.

  - From RAN1 perspective, new UE behaviors can be introduced for SBFD aware UEs based on the time and frequency locations of subbands for SBFD operation.

SBFD operation Alt 3 only provides time domain information of UL subband. However, the frequency location of UL subband is still unknown for SBFD UE. Therefore, it doesn’t resolve or even only alleviate the issues coming with transparent SBFD operation. Alt 3 shouldn’t be supported.

Proposal 4: For Non-transparent SBFD operation, only Alt 4 is supported, i.e. both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

Regarding to Alt 4, we further discussed how to schedule a SBFD aware UE on SBFD slot. Four options were raised as shown below:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 3: The SBFD aware UE does not expect to be scheduled with DL reception within the UL subband and may be scheduled with UL transmission outside the UL subband in the SBFD symbol

- Option 4: The SBFD aware UE may be scheduled with UL transmission outside the UL subband or DL reception within the UL subband in the SBFD symbol

As analysed in the above, the merits of alt 4, e.g. good interference mitigation, simple UL/DL multiplexing, low latency of random access, come from the deterministic UL subband configuration. The spirit of alt 4 is to confine the uplink transmission within the UL subband. If UL transmission outside the UL subband is allowed, it makes the explicit UL subband meaningless. It is very difficult for gNB to exchange scheduling information for interference mitigation purpose. The intra-subband gNB-to-gNB CLI on the DL subband would significantly degrades the uplink performance. Actually, from the lessons we learned from Rel-16 CLI, the deployment scenario with adjacent channel gNB-to-gNB CLI is not recommended due to the significant performance degradation. For SBFD operation, the gNB-to-gNB CLI is much more severe as neighbouring gNBs are deployed on the same band. On the other hand, it may be not necessary to preclude gNB to schedule DL on the UL subband. Actually, it is the legacy behaviour on DL symbol. The gNB-to-gNB CLI may be well handled by the gNB via exchanging information of whether DL transmission is intended on the UL subband. For example, if the aggressor cell is going to transmit DL on the UL subband, the victim will not schedule UL transmission on the UL subband.

Proposal 5: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, further study the following two options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

Granularity of SBFD operation

In RAN1#109 e-meeting, we discussed SBFD operation with achieving the following agreement:

Agreement At least study SBFD operation within a TDD carrier

In RAN1#110 meeting, we further discussed SBFD operation within a TDD carrier. The following working assumption was achieved:

Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

One potential solution for SBFD operation within a TDD carrier is BWP-based SBFD. The basic idea is to introduce a UE capability of simultaneously supporting multiple active BWP pairs. However, it would be a stringent requirement on UE capability which needs careful and comprehensive study. Secondly, it will introduce BWP switching delay inevitable. The delay comes from switching between the normal DL BWP and the duplex-DL BWP, vice versa. One example is shown as below. The delay components need further study as it is different from DL-DL BWP switching or UL-UL BWP switching. At least it needs more time on transmitter and receiver transition additionally.

Figure 5: Delay of BWP based SBFD

Last but not least, one basic idea behind BWP based SBFD is to configure TDD UL DL configuration per BWP. It is a fundamental change on TDD UL DL configuration as currently it is configured per cell. UE firstly determines the slot type and direction, i.e. DL, UL or flexible, then it determines UL or DL BWP is applied. If TDD UL-DL configuration is configured per BWP, the standard impacts may be uncontrollable.

Proposal 6: Confirm the following working assumption:

Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Based on the aforementioned analyses, we think SBFD operation should be confined within a single configured DL and UL BWP pair. Hence we have the following proposal:

Proposal 7: SBFD operation is within an active BWP pair within a TDD carrier.

Beyond SBFD operation within a TDD carrier, it was also discussed that SBFD operation across carriers, called HDCA-SBFD. The nature of HDCA-SBFD is to regard carrier as subband and allow directional conflicts between carriers within the same band. However, we note that there is nothing about HDCA-SBFD in the SID of Rel-18 duplex enhancement. From this perspective, it is out of scope. On the other hand, we don’t see any bonus compared to single carrier mechanism.  Furthermore, we need to define new UE behaviour for supporting HDCA-SBFD, e.g. currently it is not allowed different directions on carriers within the same band. We may need to study/define the UE behaviour case by case, e.g. Semi-SFI+Semi-SFI, Semi-SFI+Dynamic SFI, Dynamic SFI + Semi-SFI, Dynamic SFI + Dynamic SFI. It will introduce jumbo standard workload.  Furthermore, the carrier used as UL subband basically has an UL dominated TDD-UL-DL configuration. Considering the carrier is also configured to the other UEs, including legacy UE, it will introduce improper TDD configuration for the non-target UE.

Proposal 8: Half duplex CA based SBFD operation is not supported.

Scheduling aspects related to SBFD

For unpaired spectrum, guard period is needed between DL region and UL region. Similarly, guard period is also needed between the last OFDM symbol in DL region to the first OFDM symbol in UL subband. One example is shown in Figure 6, wherein DL-to-UL switching point between DL slot and UL subband is needed in addition to the legacy guard period.  The additional DL-to-UL switching point can be configured explicitly or implicitly, either in DL slots or in UL subband. However, the overhead should also be carefully evaluated as the guard period cannot be used for transmission and reception. Additional DL-to-UL switching point would reduce the spectrum utilization.

Figure 6: Examples of DL to UL switching point between DL and UL subband

Proposal 9: Further study how to configure or determine the guard period between DL region and UL subband.

Assuming a common UL subband is configured in DL slots across the network, the interference in the network can be summarized as below, also illustrated in Figure 7:

- Self-interference from DL RB to UL RB

- Cross link interference from UE to UE

- Cross link interference from UE to gNB

- Cross link interference from gNB to gNB

Self-interference comes from the in-band emission from adjacent DL RB, on which gNB transmits DL signal or channel. Guard band between DL subband and UL subband may be needed to alleviate the self-interference.

Figure 7: Illustration of interference for duplex system

Proposal 10: Study whether and how to define a guard band between DL subband and UL subband.

As analysed aforementioned, the bandwidth of UL subband configured in a DL slot can be different from that of active UL BWP. In this case, the bit length of FDRA in a DCI scheduling PUSCH on the UL subband may not equal to that of a DCI scheduling PUSCH on normal UL slot. Consequently, it may introduce a new payload size in addition to the current three DCI sizes scrambled by C-RNTI.  It will complicate the DCI alignment procedure which is not preferred. In order to avoid increased complexity of handling DCI alignment, the bit length of FDRA information field should be determined by the active UL BWP, no matter the bandwidth of UL subband equals to that of UL BWP or not.

Proposal 11: If a UL subband is configured via RRC signalling, the bit length of FDRA field in a DCI scheduling uplink on the UL subband is determined by the active UL BWP.

Basically, SBFD operation needs to introduce/define/configure a UL subband for UL transmission on DL symbols. There would be different resource types on SBFD slots, i.e. legacy DL resource and UL resources belonging to UL subband. Considering UE can only transmit uplink or receive downlink at the same time, we should consider how to handle conflicts between UL and DL. There are several conflicting cases on SBFD slot which are list below:

- Type 1: Dynamic DL on the DL subband or on the UL subband conflicts with Dynamic UL on the UL subband.

- Type 2: Semi-static DL on the DL subband or on the UL subband conflicts with Dynamic UL on the UL subband.

  - The semi-static DL includes periodic reference signal, semi-persistent reference signal, SPS PDSCH without DL assignment, etc.

- Type 3: Broadcast DL on the DL subband or on the UL subband conflicts with UL on the UL subband.

  - The broadcast DL includes Type0/0A/1/2-PDCCH CSS and SSB

- Type 4: Dynamic DL on the DL subband or on the UL subband conflicts with semi-static UL on the UL subband, including CG PUSCH, periodic uplink reference signal and periodic PUCCH.

- Type 5: Semi-static DL on the DL subband or on the UL subband conflicts with semi-static UL on the UL subband.

For type 1 and type 5 collision, it should be an error case as gNB has full power to avoid such kind of scheduling and configuration. For type 2 and type 4 collision, dynamic scheduling deserves higher priority which is the generic principle since Rel-15. Regarding type 3 collision, UE should drop uplink transmission in the UL subband due to broadcast DL usually provide essential network information.

Proposal 12: The following principles should be considered for handling UL/DL collision on the SBFD slots:

- SBFD UE doesn’t expect collision between dynamic DL and dynamic UL

- SBFD UE doesn’t expect collision between semi-static DL and semi-static UL

- Dynamic DL/UL has higher priority when it conflicts with semi-static UL/DL

- Broadcast DL has higher priority when it conflicts with UL on the UL subband

Others

In RAN#96 plenary meeting, it was concluded that ‘UL symbol as 2nd priority is accepted, no intended suspension of continuation of work in WGs’. From our understanding, the intention of the conclusion is to deprioritize the discussion on whether to support DL subband on UL symbols. When we finish the discussion or the design of UL subband on DL symbols becomes mature, we can discuss whether to support DL subband on UL symbols.

As discussed during preparation phase of Rel-18 NR duplex operation, the motivation of supporting full duplex within a slot is to enhance the uplink transmission, in terms of coverage, latency and capacity. It is also captured in the justification section of SID, which is excerpted as below:

TDD is widely used in commercial NR deployments. In TDD, the time domain resource is split between downlink and uplink. Allocation of a limited time duration for the uplink in TDD would result in reduced coverage, increased latency and reduced capacity. As a possible enhancement on this limitation of the conventional TDD operation, it would be worth studying the feasibility of allowing the simultaneous existence of downlink and uplink, a.k.a. full duplex, or more specifically, subband non-overlapping full duplex at the gNB side within a conventional TDD band. < -----omitted text----> This study aims to identify the feasibility and solutions of duplex evolution in the areas outlined above to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operations in unpaired spectrum. In addition, the regulatory aspects need to be examined for deploying identified duplex enhancements in TDD unpaired spectrum considering potential constraints.

Based on previous discussion and the SID, the common understanding on the type of subband seems to be UL subband, which exists in DL slot and is used for uplink transmission. On the other hand, the motivation of configuring/indicating a DL subband in an uplink slot is unclear.  Considering uplink subband is already on the table, a DL dominated TDD UL-DL configuration is feasible which can certainly guarantee DL performance.  More importantly, configuring/indicating DL subband in an uplink slot has significant impacts on legacy UEs, which is not desired. One example is shown in Figure1 and the interference from UL/DL subband is summarized as below:

- For case a), a UL subband is configured or indicated in a DL slot. For a legacy UE which receives downlink channel or signal, it suffers uplink interference from other UEs in the same cell.

- For case b), a DL subband is configured or indicated in a UL slot. For a legacy UE which transmits uplink channel or signal, gNB suffers downlink interference from cellist own transmitter.

From perspective of legacy UE, the DL interference from same serving cell is much stronger than UL interference from other UE. In the other words, DL subband in a UL slot introduces more significant impacts for legacy UE, which needs carefully study.

Figure 7: Examples of subband type for duplex operation

Proposal 13: For subband non-overlapping full duplex, it cannot be applied to UL symbols.

Conclusion

In this contribution, we provide our views on subband non-overlapping full duplex. We have the following observations:

Observation 1: The frequency resources available on the DL symbols for a SBFD UE are confined within the active UL BWP if the UL subband is transparent.

Observation 2: It is difficult to mitigate intra-subband CLI if transparent UL subband is adopted.

Observation 3: Transparent UL subband complicates multiplexing between UL and DL on the SBFD slot.

Observation 4: If a RO exists in DL slots configured by tdd-UL-DL-ConfigurationCommon, it cannot be used for preamble transmission even if the RO locates in a transparent UL subband.

Based on the aforementioned discussion, we have the following proposals:

Proposal 1: Subband related to duplex operation is transparent for legacy UE and the following legacy behaviour should be kept:

- UE doesn’t expect to transmit on DL symbols.

- UE doesn’t expect to receive on UL symbols.

- UE can transmit or receive on flexible symbols depending on the indication from gNB

- UE does not expect conflict between DL reception and UL transmission on the same flexible OFDM symbol.

Proposal 2: Clarify whether to allow a SBFD UE to transmit preamble within the UL subband in a SBFD slot.

Proposal 3: Non-transparent SBFD operation is sufficient and there is no need to further study transparent SBFD operation.

Proposal 4: For Non-transparent SBFD operation, only Alt 4 is supported, i.e. both time and frequency locations of subbands for SBFD operation are known to SBFD aware UEs.

Proposal 5: For SBFD operation Alt 4, for an SBFD aware UE configured with an UL subband in an SBFD symbol, further study the following two options:

- Option 1: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband or to be scheduled with DL reception within the UL subband in the SBFD symbol

- Option 2: The SBFD aware UE does not expect to be scheduled with UL transmission outside the UL subband and may be scheduled with DL reception within the UL subband in the SBFD symbol

Proposal 6: Confirm the following working assumption:

Working Assumption For SBFD operation within a TDD carrier, study SBFD scheme within a single configured DL and UL BWP pair with aligned center frequencies as baseline. - FFS feasibility and potential benefit of SBFD scheme within a single configured DL and UL BWP pair with unaligned center frequencies - FFS feasibility and potential benefit of SBFD scheme with more than one configured DL and UL BWP pair with aligned/unaligned center frequencies for a DL and UL BWP pair

Proposal 7: SBFD operation is within an active BWP pair within a TDD carrier.

Proposal 8: Half duplex CA based SBFD operation is not supported.

Proposal 9: Further study how to configure or determine the guard period between DL region and UL subband.

Proposal 10: Study whether and how to define a guard band between DL subband and UL subband.

Proposal 11: If UL subband is configured via RRC signalling, the bit length of FDRA field in a DCI scheduling uplink on the UL subband is determined by the active UL BWP.

Proposal 12: The following principles should be considered for handling UL/DL collision on the SBFD slots:

- SBFD UE doesn’t expect collision between dynamic DL and dynamic UL

- SBFD UE doesn’t expect collision between semi-static DL and semi-static UL

- Dynamic DL/UL has higher priority when it conflicts with semi-static UL/DL

- Broadcast DL has higher priority when it conflicts with UL on the UL subband

Proposal 13: For subband non-overlapping full duplex, it cannot be applied to UL symbols.

Reference

  - RP-213591, New SI: Study on evolution of NR duplex operation, CMCC

  - R1-2208122, Summary #5 of subband non-overlapping full duplex, Moderator (CATT)