3GPP TSG RAN WG1 #110bis-e		R1-2209343

e-Meeting, October 10th – 19th, 2022

Agenda item:	9.5.2.2

Source: 	CMCC

Title: 	Discussion on carrier phase positioning

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new SID on study on expanded and improved NR positioning was approved [1]. The motivation to study the GNSS-like NR carrier phase positioning to achieve higher positioning accuracy for both indoor and outdoor scenarios has been justified:

- Improved accuracy, integrity, and power efficiency: - Study solutions for accuracy improvement based on NR carrier phase measurements [RAN1, RAN4] - Reference signals, physical layer measurements, physical layer procedures to enable positioning based on NR carrier phase measurements for both UE-based and UE-assisted positioning [RAN1] - Focus on reuse of existing PRS and SRS, with new reference signals only considered if found necessary

In RAN1#110 meeting, a series of agreements on additional assumptions to evaluate the carrier phase positioning was achieved [2]. In this contribution, we further provide our views on the carrier phase positioning based on the agreements made in the last two RAN1 meetings.

Discussion

Carrier phase positioning technology, as a high-resolution method to improve the positioning accuracy, is widely used in global navigation satellite systems (GNSS). However, the carrier phase positioning has not applied to the cellular network. The NR carrier phase positioning has been proposed back in Rel-16 and Rel-17 as a potential solution to improve the positioning accuracy. However, due to lack of evaluations inputs, the carrier phase positioning is not included in the Rel-16/17 work item phases.

The basic principle of carrier phase positioning is using the carrier phase to estimate the propagation delay of the first arrival path. Suppose that the carrier frequency is 3.5GHz, the wavelength of the carrier is about 0.86cm. Theoretically, the estimation performance using carrier phase is better than 10% of the wavelength, and hence a positioning accuracy of less than 0.08cm can be achieved ideally. On the other hand, when compared to the traditional timing-based positioning, of which the accuracy performance is highly dependent on the bandwidth, the carrier phase positioning may be more tolerant to narrow bandwidth.

However, to achieve the ideal performance of carrier phase positioning, the following challenges needs to be studied and overcame:

- The phase synchronization error degrades the accuracy performance;

- Negative impact of complicated propagation environment (e.g., NLOS, multipath) on the phase error;

- Fast search of the integer ambiguity.

## 2.1 Carrier phase measurement

Based on the discussion in previous RAN1 meetings, it was notified that companies shared different views on how to measure the carrier phase. One, similar to that adopted in GNSS, is to use the phase-locked loop to measure the phase information of received signal. However, considering the TDD system, the discontinuous of carrier phase will cause the cycle jump, and solutions to solve this issue should be further studied. On the other hand, the carrier phase measurement can also be obtained by baseband signal in frequency or time domain.

Suppose that the CIR is estimated based on PRS, which is expressed as

where is the PRS with N as the FFT size,  is the received signal,  is the sampled propagation delay of the l-th path, and  is the carrier phase of the l-th path.

Based on the TOA estimation of the first paths , the channel frequency response of subcarrier  can be approximated as:

The carrier phase measurement can be written as:

Based on ,  the relationship between the carrier phase and the transmission delay is expressed as

where   is the unknown integer ambiguity, and  is the phase noise. It shows that the propagation delay of the direct path is reflected in the carrier phase.

Observation 1: The carrier phase measurement can be obtained in frequency domain or time domain of the base band signal.

Observation 2: By estimating the first path, the carrier phase measurement in time domain can help suppress the impact of multipath.

## 2.2 Differential carrier phase measurements

As mentioned at the beginning of this section, to attain the theoretically cm-level positioning accuracy, the carrier phase synchronization errors should be ideally eliminated. In RAN1#109-e meeting, the following agreement was made to consider the use of PRU in the evaluation of carrier phase positioning:

Agreement The use of PRUs to facilitate NR carrier phase positioning can be evaluated in the SI by RAN1.

The use of PRU, which is similar as the reference stations deployed for carrier phase based RTK positioning, can be adopted to estimate and mitigate the carrier phase synchronization errors  between TRPs.

Proposal 1: For carrier phase positioning, the double differential carrier phase measurement should be considered to eliminate the inter TRP carrier phase synchronization errors.

    - The double differential carrier phase measurement can be obtained by the use of PRU.

## 2.3 Integer ambiguity

For the performance of the carrier phase positioning, one of the key issues that is identified is how to accurately and fast search the integer cycle of the carrier phase. In RAN1#109-e meeting, the following agreement was made to study the impact of integer ambiguity on carrier phase positioning:

Agreement The impact of integer ambiguity on NR carrier phase positioning and potential solutions to resolve the integer ambiguity will be studied in the SI.

On this issue, our preliminarily think that potential solutions to narrow down the search space of the unknown integer ambiguity should be studied. For instance, as discussed in the last RAN1 meeting, the carrier phase measurement obtained from more than one frequency to formulate the “virtual wavelength” and to assist the fast search of the integer ambiguity. On the other hand, some hybrid methods of using both the existing positioning method and carrier phase positioning can also be considered. By using the existing positioning method, the integer ambiguity can be coarsely estimated and therefore narrows down the search space of the carrier phase measurement. This is also related to another open issue of whether carrier phase positioning refers to a standalone positioning technique or a hybrid one.

Proposal 2: At least the following solutions to solve the integer ambiguity are considered feasible:

    - Fast search of integer ambiguity of the carrier phase obtained from more than one frequency;

    - Fast search of integer ambiguity of the carrier phase associated with the legacy positioning techniques.

Conclusions

In this contribution, we provide our views on the carrier phase positioning, and the following observations and proposals are made:

Observation 1: The carrier phase measurement can be obtained in frequency domain or time domain of the base band signal.

Observation 2: By estimating the first path, the carrier phase measurement in time domain can help suppress the impact of multipath.

Proposal 1: For carrier phase positioning, the double differential carrier phase measurement should be considered to eliminate the inter TRP carrier phase synchronization errors.

    - The double differential carrier phase measurement can be obtained by the use of PRU.

Proposal 2: At least the following solutions to solve the integer ambiguity are considered feasible:

    - Fast search of integer ambiguity of the carrier phase obtained from more than one frequency;

    - Fast search of integer ambiguity of the carrier phase associated with the legacy positioning techniques.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e.

- Draft Report of 3GPP TSG RAN WG1 #110 v0.2.0