3GPP TSG RAN WG1 #110-bis-e		R1-2209361

e-Meeting, October 10th – 19th, 2022

Source: 	CMCC

Title:	Discussion on evaluation methodology and applicable scenarios for low-power WUR

Agenda item:	9.13.1

Document for:	Discussion & Decision

- Introduction

As the SID in RP-222145[1], the study should primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables. Other use cases are not precluded, e.g.XR/smart glasses, smart phones.

In this contribution, the evaluation methodology and the applicable scenarios for low-power WUR will be discussed.

- Evaluation methodology for low-power WUR

- 2.1 Power Consumption

As indicated in the snapshot below from [2], the measured results show that the proposed low-power dual-mode receiver achieves a sensitivity of 78.5 dBm and 75 dBm while dissipating an average power of 16.4uW and 22.9uW during Monitoring (MO) mode and Identifier (ID) mode, respectively.

As indicated in the snapshot below from [3], the measured results show that a WuRX that achieves -76dBm sensitivity in the 151.8MHz MURS band and -71dBm sensitivity in the 433MHz ISM band while consuming 7.4nW dc power.

In the examples above, the “core parts” of the low-power WUR includes VCO/Oscillator, IF Amp, Envelop Detector, BB Amplifier, Digital Circuit, Bias, etc.

However, in the real world outside lab, to enable the low-power WUR to work well, besides the “core parts”, quite a lot other “assisted parts” are also needed, including PMIC, Bandgap voltage reference, GPIO, etc. To add all these necessary power consumption up, the total power consumption will be much higher.

Take a typical Bluetooth current consumption for example shown in the table above,

- When in “BLE scan” operating mode, the current consumption is 871.2uW(=152uA*3.6V+180uA*1.8V)

Take BCM43455 Bluetooth 4.1 current consumption for example shown in the table above, with VBAT = 3.6V and VDDIO = 1.8V

- When in “BLE scan” operating mode, the current consumption is 1.18mW(=179uA*3.6V+296uA*1.8V)

- Observation 1: In the real world outside lab, “assisted parts” also need to be considered for low-power WUR. And the total power consumption of low-power WUR might be high up to the order of hundreds of uW or even mW, which is much higher than the power consumption of the “core parts” only.

- Proposal 1: When evaluating the power consumption of low-power WUR, both the power consumption of the “core parts” and the “assisted parts” shall be taken into consideration, including VCO/Oscillator, IF Amp, Envelop Detector, BB Amplifier, Digital Circuit, Bias, PMIC, Bandgap voltage reference, GPIO, etc.

- 2.2 Periodic Services

As indicated in the revised SID[1], “Currently, UEs need to periodically wake up once per DRX cycle, which dominates the power consumption in periods with no signalling or data traffic. If UEs are able to wake up only when they are triggered, e.g., paging, power consumption could be dramatically reduced. This can be achieved by using a wake-up signal to trigger the main radio and a separate receiver which has the ability to monitor wake-up signal with ultra-low power consumption. Main radio works for data transmission and reception, which can be turned off or set to deep sleep unless it is turned on.”

Once the main radio to be turned off or set to deep sleep, the transient period may be increased high up to more than 10ms due to the main radio “warm-up time”, including hardware turning on, synchronization (Oscillator coarse sync & fine Sync) and etc.

- Observation 2: The transient period may be high up to the order of 10ms when the main radio to be turned on from the status of “turned off” or “deep sleep”.

- Proposal 2: The transient period between “main radio turned on” and “main radio turned off/deep sleep” shall be taken into consideration in the evaluation methodology.

- Analysis on the applicable scenarios for low-power WUR

- 3.1 Less applicable areas

As indicated in the table below from [4], low-power receivers have much smaller communication distance corresponding to the worse sensitivity.

Take a single base station for example, compared with the existing Rel-15/Rel-16/Rel-17 receiver, low-power WUR with worse sensitivity has smaller available communication distance. If low-power WUR would enjoy the same communication distance as the existing Rel-15/Rel-16/Rel-17 receiver, increased densities of base stations would be needed which will cost more energy consumption from network side. This goes against the green energy saving target.

Observation 3: With the existing Rel-15/Rel-16/Rel-17 5G NR network deployment, compared with the sensitivity of the main radio, low-power WUR with worse sensitivity has less applicable areas.

- 3.2 Band specific low-power WUR

As indicated in the snapshots below from [5], both Receiver Architecture A and Receiver Architecture B are band specific. If more bands are needed to be supported by main radio, more simultaneously active RF paths may be needed in low-power WUR, which will increase the die area/complexity/cost and decrease the benefits of power saving proportionally.

- Observation 4: To balance among the die area/benefits of power saving/complexity/cost, etc., the less bands supported, the more benefits can be obtained from the low-power WUR solution.

- Proposal 3: Observation 3 and Observation 4 shall be taken into consideration when analyzing the applicable scenarios for low-power WUR.

- Conclusions

In this contribution, the evaluation methodology and applicable scenarios for low power WUR will be discussed.

- Observation 1: In the real world outside lab, “assisted parts” also need to be considered for low-power WUR. And the total power consumption of low-power WUR might be high up to the order of hundreds of uW or even mW, which is much higher than the power consumption of the “core parts” only.

Proposal 1: When evaluating the power consumption of low-power WUR, both the power consumption of the “core parts” and the “assisted parts” shall be taken into consideration, including VCO/Oscillator, IF Amp, Envelop Detector, BB Amplifier, Digital Circuit, Bias, PMIC, Bandgap voltage reference, GPIO, etc.

- Observation 2: The transient period may be high up to the order of 10ms when the main radio to be turned on from the status of “turned off” or “deep sleep”.

Proposal 2: The transient period between “main radio turned on” and “main radio turned off/deep sleep” shall be taken into consideration in the evaluation methodology.

- Observation 3: With the existing Rel-15/Rel-16/Rel-17 5G NR network deployment, compared with the sensitivity of the main radio, low-power WUR with worse sensitivity has less applicable areas.

- Observation 4: To balance among the die area/benefits of power saving/complexity/cost, etc., the less bands supported, the more benefits can be obtained from the low-power WUR solution.

- Proposal 3: Observation 3 and Observation 4 shall be taken into consideration when analyzing the applicable scenarios for low-power WUR.

- References

- RP-222145	Revised SID: Study on low-power Wake-up Signal and Receiver for NR

- An Ultra-Low-Power Energy-Efficient Dual-Mode Wake-Up Receiver; Shahaboddin Moazzeni, Member, IEEE, Mohamad Sawan, Fellow, IEEE, and Glenn E. R. Cowan, Member, IEEE; IEEE TRANSACTIONS ON CIRCUITS AND SYSTEMS—I: REGULAR PAPERS, VOL. 62, NO. 2, FEBRUARY 2015

- A -76dBm 7.4nW Wakeup Radio with Automatic Offset Compensation;Jesse Moody, Pouyan Bassirian, Abhishek Roy, Ningxi Liu, Stephen Pancrazio, N. Scott Barker, Benton H. Calhoun, Steven M. Bowers;ISSCC 2018 / SESSION 28 / WIRELESS CONNECTIVITY

- A Survey of Low-Power Transceivers and Their Applications. Page 6-17, Johannes Blanckenstein, Jirka Klaue, and Holger Karl. IEEE circuits and systems magazine 1531-636X/15©2015IEEE third QUARTER 2015

- A 2GHz 52µW Wake-Up Receiver with -72dBm Sensitivity Using Uncertain-IF Architecture, N. Pletcher, S. Gambini, J. Rabaey;2008 IEEE International Solid-State Circuits Conference