3GPP TSG RAN WG1 Meeting #110bis-e	R1-2209814

e-Meeting, October 10 – 19, 2022

Agenda Item:	9.17

Source:	Huawei, HiSilicon

Title:	Remaining issues on NR PDCCH reception in symbols with LTE CRS REs

Document for:	Discussion and Decision

# Introduction

Following agreements have been achieved at the RAN1#110 meeting [1].

Agreement Reception of NR PDCCH candidates that overlap with LTE CRS REs is supported by Rel18 UEs PDCCH candidates and PDCCH-DMRS RE mapping are based on that of R15 from UE side. Note: depends on UE capability Following options can be used for PDCCH-DMRS channel estimation - legacy CE assumption - RAN1 consider support this, if no RAN4 performance requirements are defined - CE on clean symbol(s) only (this channel estimation option does not apply for 1 symbol CORESET)

In this contribution, we discuss the needed RRC parameter and the corresponding TP on NR PDCCH reception in symbols with LTE CRS REs.

# Discussions

An RRC parameter is needed for configuring the feature of receiving PDCCH on the symbol of CRS to UE, after the corresponding UE capability is reported. Otherwise, if such RRC parameter is not introduced, and the gNB does not support transmitting NR PDCCH in symbols with LTE CRS REs, the UE has to receive NR PDCCH in symbols with LTE CRS REs by default after reporting the capability; as a result, the false alarm probability of PDCCH will increase. So, if a UE reports that it supports the UE capability for NR PDCCH reception in symbols with LTE CRS REs, it should not start the related NR PDCCH reception immediately, but should wait for the corresponding RRC parameter to enable this reception.

# Text proposal

Based on the above discussion, we prepared the following text proposal.

Proposal 1: Adopt the TP for clause 10 of TS 38.213.

10 UE procedure for receiving control information < Unchanged parts are omitted > If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. If a UE is not provided [PDCCH-ReceptionWithCRS], and Iif at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate. If a UE is provided availableRB-SetsPerCell, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets that are indicated as unavailable for receptions by an available RB set indicator field in DCI format 2_0 as described in clause 11.1.1. If the UE does not obtain the available RB set indicator for a symbol, the UE monitors PDCCH candidates on all RB sets in the symbol. < Unchanged parts are omitted >

# Conclusions

According to the discussion, following proposals are provided:

Proposal 1: Adopt the following TP for clause 10 of TS 38.213.

10 UE procedure for receiving control information < Unchanged parts are omitted > If a UE monitors the PDCCH candidate for a Type0-PDCCH CSS set on the serving cell according to the procedure described in clause 13, the UE may assume that no SS/PBCH block is transmitted in REs used for monitoring the PDCCH candidate on the serving cell. If a UE is not provided [PDCCH-ReceptionWithCRS], and Iif at least one RE of a PDCCH candidate for a UE on the serving cell overlaps with at least one RE of lte-CRS-ToMatchAround, or of LTE-CRS-PatternList, the UE is not required to monitor the PDCCH candidate. If a UE is provided availableRB-SetsPerCell, the UE is not required to monitor PDCCH candidates that overlap with any RB from RB sets that are indicated as unavailable for receptions by an available RB set indicator field in DCI format 2_0 as described in clause 11.1.1. If the UE does not obtain the available RB set indicator for a symbol, the UE monitors PDCCH candidates on all RB sets in the symbol. < Unchanged parts are omitted >

# References

- RAN1 Chair’s Notes, 3GPP TSG RAN WG1 Meeting #110, October 2022.