3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209860

e-meeting, Oct 10-19, 2022

Agenda Item:	9.9.1

Source:	Ericsson

Title:	Multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:	Discussion

# Introduction

In RAN#94, it was agreed to start a Rel 18 WI on multi-carrier enhancements, with the following objectives for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI.

Specify a solution for multi-cell PUSCH/PDSCH scheduling (one PDSCH/PUSCH per cell) with a single DCI [RAN1]

- Identify the maximum number of cells that can be scheduled simultaneously

- Consider both intra-band and inter-band CA operation

- Consider both FR1 and FR2

- The single DCI shall be optimized for 3 or more cells for the multi-cell PUSCH/PDSCH scheduling

In RAN#97, MCE WI down-scoping was discussed, and below is the outcome. Several aspects related to mc-DCI design were down-scoped.

In this paper, we discuss the various aspects to consider in specifying this feature. The agreements on this topic so far are listed in Annex.

# Discussion

In NR releases until Rel-17, a single DCI can schedule PUSCH/PDSCH on one cell and single DCI can schedule multiple PUSCH/PDSCH on single cell. The new Rel-18 WI objective is to specify a feature where a single DCI can be used for multi-cell PUSCH/PDSCH scheduling (one PUSCH/PDSCH per cell).

## High-level design aspects

In current framework, a PDCCH can schedule PDSCH on only one cell using DCI formats 1-0/1-1/1-2. In case of self-carrier scheduling, the PDCCH and PUSCH/PDSCH are received on the same serving cell. For cross-carrier scheduling, a PDCCH is received on a scheduling cell (cell A) and the corresponding PUSCH/PDSCH may be received on a scheduled cell which may be same as the scheduling cell (cell A) or on another cell (cell B).

For single DCI scheduling PUSCH/PDSCH on multiple cells, one PDCCH DCI format on the scheduling cell (e.g. cell A) has to schedule a PUSCH/PDSCH e.g. for the scheduling cell (cell A) and also one other PUSCH/PDSCH for another scheduled cell (cell B). As stated in the WID justification, trade-off between overhead saving and scheduling restriction must be taken into account.

CA is a native feature of NR supported from Rel15 and it is used for aggregating carriers with a wide variety of attributes depending on the specific deployment scenario. This flexibility provided by CA should be retained even when single DCI scheduling PUSCH/PDSCH on multiple cells (or mc-DCI) is used.

### Maximum number of cells scheduled by mc-DCI

Below WA was made in RAN1#110-e. The value of 4 provides the correct balance between DCI overhead reduction and the scheduling restriction. Hence, the working assumption with value 4 can be confirmed.

An FFS was added for maximum number of configurable cells for co-scheduling. In a simple case, the maximum number of configurable cells for co-scheduling can also be 4.

There can be other possibilities, for example, the maximum number of configured cells is 8 (i.e., larger than 4), but only 4 cells can be co-scheduled by a mc-DCI. In such a case, considering full carrier scheduling, two mc-DCIs are needed within a slot to be able to simultaneously schedule all eight carriers. It can become easily complicated if the carrier grouping indication in the DCIs keeps varying assuming at every scheduling instance, there is flexibility to group the eight carriers into up to two DCIs. At this point in the WI phase, it is preferrable to use the simple option of using 4 also for maximum number of configurable cells. Then for case with 8 carriers, two groups, each group with 4 configured cells can be used for mc-DCI scheduling.

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

FFS: The maximum number of configurable cells for co-scheduling

- Maximum number of configurable cells for co-scheduling by a DCI format 1_X/0_X is 4.

### Scheduled/scheduling cells for mc-DCI

Below agreement was made in RAN1#110-e.

Working Assumption

For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell.

- The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously.

  - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells.

- FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats

- FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X

It was agreed in last meeting that mc-DCI can be used for single cell scheduling. The mc-DCI design should allow sufficient scheduling flexibility, considering that not all cells will be always scheduled by the mc-DCI at all instances (e.g., retransmissions depending on HARQ feedback, CSI reporting, etc.). It is also beneficial to also support individual scheduled cells using existing single cell DCI format as it provides the full flexibility/feature support in scheduling (i.e., without compressed DCI fields like in mc-DCI). The specification should support the case where legacy DCI formats are also monitored in all cells.

When UE monitors the DCI format for single DCI scheduling PUSCH/PDSCH on multiple cells, the UE can be configured to also monitor existing DCI format(s) scheduling PUSCH/PDSCH on single cell (i.e., 1-0/1-1/1-2). Thus, it should be possible to schedule as many cells as needed using mc-DCI as well as the possibility to configure the single cell DCI format monitoring for any cell.

- When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, a mc-DCI can schedule PUSCH/PDSCH on all of the cells or a subset of those cells.

- When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, for each of those cells, UE can also be configured to monitor existing single cell DCI format(s) scheduling PUSCH/PDSCH (i.e. 1_1/1_2/0_1/0_2).

### DCI size handling and budgets

Next, we discuss DCI size handling and budgets. Following was agreed in RAN1#109-e.

Agreement

Further study DCI size budget including below options for multi-cell scheduling DCI:

- Option 1: Existing DCI size budget is maintained per scheduled cell.

  - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

- Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell.

  - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

  - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded.

  - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling

  - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell

  - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced.

- Other options/alternatives could be considered.

mc-DCI configuration for uplink and downlink should also be addressed. Typically, the number of aggregated carriers can be different between downlink and uplink. Moreover, it is possible (and perhaps more likely) that the DCI size for downlink and uplink is different. In such a case, it is preferable to keep independent the configuration of mc-DCI for PUSCH and PDSCH as well as to configure the BDs for those separately. For example, if UE is configured with four cells in downlink and only two cells (or even one cell) in the uplink, then a mc-DCI for downlink makes sense, but there may be no need to use mc-DCI for the uplink or perhaps assign separate BDs for mc-DCI for uplink and downlink. Based on above, the below proposals are made.

- Size of mc-DCI (0_X/1_X) is explicitly configured by higher layers.

- Support independent configuration of mc-DCI for PUSCH and PDSCH.

As discussed in last section, it should be possible to schedule as many cells as needed using mc-DCI as well as the possibility to configure the single cell DCI format monitoring for any cell. Therefore, the discussion on size budgets can continue after further progress is made on DCI formats that can be supported along with mc-DCI (0_X/1_X). However, given the limited time, our preference is to allow the DCI size budget to be increased to ensure that for a scheduled cell, there is sufficient flexibility to configure both 0_X/1_X and 0-1/1-1. Size matching can lead to large padding (e.g., only 2 UL cells for 0_X and 4 DL cells for 1_X, etc). At least for PCell, additional budget may be needed since 0-0 and 1-0 also have to be monitored.

- For a scheduled cell configured to be scheduled by mc-DCI, the total number of different DCI sizes with C-RNTI that the UE is configured to monitor is no more than 4 for the cell if it is an SCell and is no more than 5 if it is a PCell.

### BD handling/budgets

Below was agreement for BD/CCE handling.

Agreement

Further study BD/CCE counting for multi-cell scheduling DCI based on below options:

- Alt 1: counted on each co-scheduled cell

- Alt 2: counted only in one scheduled cell

- Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells

- Alt 4: counted as part of the scheduling cell instead of each scheduled cell

- Alt 5: scaled down to each of scheduled cells excluding scheduling cell

- Alt 6: counted on each co-scheduled cell excluding scheduling cell

- Other alternatives could be considered.

Using DCI 0_X/1_X, a single BD/CCE candidate can schedule PDSCH/PUSCH on multiple cells, thereby reducing UE BD complexity. Of course, since a single BD/CCE candidate can schedule multiple cells in a set, the BD/CCE budget can be redistributed to allocate more BDs to other carriers not in the set. However, whether and how to do such redistribution needs strong motivation, especially if it leads to increased UE BDs and spec complexity.

For a scheduled cell, it should be possible to configure a same number of BD/CCEs for monitoring scheduling DCI formats (including 0_X/1_X and legacy DCI formats) as the case of legacy cross-carrier scheduling.

Consider an example. UE is configured with a primary cell with 15 kHz SCS and four SCells with 30 kHz SCS and the UE indicates a pdcch-BlindDetectionCA capability of = 4.

Case 1A : Assume all four SCells are cross-carrier scheduled from the primary cell using legacy mechanism (i.e. Rel-16 cross-carrier scheduling). The primary cell schedules itself using legacy DCI formats.

Case 1B : Assume all four SCells are co-scheduled by mc-DCI and/or legacy DCI formats from the primary cell (i.e. Rel-18 cross-carrier scheduling). The primary cell schedules itself using legacy DCI formats.

For case 1A, according to legacy, for the 15 kHz(μ=0), , and for the 30 kHz (μ=1), , . Thus,

- for the 15 kHz primary cell,

  - the UE can be configured with up to 35 BDs with maximum of 44 non-overlapped CCEs per slot of scheduling cell.

- for the 30 kHz serving cells,

  - the UE can be configured with an aggregate (across all four SCells) of maximum of 115 BDs and maximum of 179 non-overlapped CCEs per slot of scheduling cell,

  - and a per-cell limit of 36 BDs and 56 CCEs per slot of the scheduling cell.

Now consider case 1B. The existing budget limits can still be applicable.  This is shown in below figure.

Figure 1. Legacy CCS limits in case 1A (in LHS), and limits for mc-DCI based CCS in Case 1B (in RHS).

From UE point of view, if it is only configured with mc-DCI formats for Pcell->Scell CCS for case 1B, then the UE can be configured with at most 36 BDs altogether for the four SCells. In that case, the upper limit of 115 BDs aggregate over four SCells should be irrelevant. Duplicate counting of each BD four times (once in each cell) gives 4 * 36 = 144 BDs which exceed 115 BDs and such duplicate counting may be meaningless and should be avoided.

Another example, if UE is configured with mc-DCI formats and legacy DCI formats for Pcell->Scell CCS for case 1B, then if the UE can be configured with at most 20 BDs for mc-DCI, then UE can be configured with a maximum of 16 additional BDs per SCell to satisfy both the aggregate limit (4 * 16 + 20 <=115) and per-cell limit (16 + 20 <=36). Again, duplicate counting of each mc-DCI four times (once in each cell) gives 4 *16 + 4*20 = 144 BDs and this should be avoided.

Overall, the main thing to avoid is to counting the same BD attempt for a mc-DCI, when counting BD/CCE for comparison against the aggregate limits MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ, avoid counting the same BD attempt for a mc-DCI multiple times.

Duplicate counting of a BD/CCE attempt for a mc-DCI format should be avoided when counting BD/CCE across co-scheduled cells for comparison against the aggregate limits MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ.

An example configuration that should be possible is shown in below figure. For mc-DCI the UE can be configured with 20 BDs, while for CCS using legacy DCI, 16 BDs can be configured, from pcell to scell1, and so on. This yields maximum 36 BDs per scheduled SCell, and a total of 16*4+20 = 84 BDs for scheduled cells of 30 kHz SCS.

Figure 2. For Case 1B, configuration of BDs for the different DCI formats. (16 BDs is for legacy DCI CCS from pcell to scell1).

Therefore, in our view, the existing spec limits work for DCI 0_X/1_X also, but update is needed to avoid the duplicate counting issue. Therefore, we propose the following.

- BD/CCE counting for mc-DCI is based on the legacy Rel-15/16/17 BD/CCE counting mechanism with the following update

  - a BD/CCE attempt for a mc-DCI format is counted only once for comparison against aggregate MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ limits in numerology buckets.

### Search Space configuration and definition

For mc-DCI, the search space linking needs to be specified. If a scheduling cell (e.g. c1) schedules a group of cells (e.g. c2,c3,c4), then the search space configuration on the scheduling cell c1 needs to have a linked search space set configuration on at least one of the scheduled cells (one of c2, c3, c4). It is simpler to say that linked search space set is configured on the lowest cell index of the group of cells indicated by the mc-DCI. Below figure shows an example of search space linking, where a single scheduling cell can schedule two DL/UL mc-DCIs, one for Group1 and one for Group2.

Figure 3. Illustration showing linked search space where cell 1 is used to schedule DCI 0_X/1_X for two groups (cells 2 and 3, and cells 4 and 5 respectively).

- Linked search space mechanism is reused for mc-DCI search space configuration.

  - For each group of cells configured to be scheduled by a mc-DCI format, the linked search space on the “scheduled cell” (i.e. containing only BD/CCE info and linked search space ID) is configured on one of the cells in the group.

Search space in which mc-DCI is monitored needs to be specified. From the spec, the search space is determined as follows. For mc-DCI, most parameters are setup via search space configuration (BDs/CCEs/coresets, etc). However, the n_CI parameter needs to be discussed. n_CI is given by the CIF field for legacy CCS. For mc-DCI, n_CI needs to be selected.

L⋅Yp,ns,fμ+ms,nCI⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

Consider the case where a UE is configured to monitor both mc-DCI and legacy DCI for a scheduled cell.

For legacy DCI, n_CI is given by the carrier indicator field configured for CCS from the scheduling cell to the scheduled cell. We do not see need to change that.

For mc-DCI, it is simple if n_CI is explicitly configured by higher layers. Since a group of cells would be configured to be co-scheduled by a mc-DCI, a corresponding n_CI value can also be configured for that group of cells.

- For monitoring DCI 0_X/1_X PDCCH candidates for a set of cells, the n_CI to be used in the search space equation is explicitly configured for the set of cells.

## mc-DCI format design

Below reformulation was agreed in RAN1#110-e meeting.

Agreement

For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below:

- Type-1 field:

  - Type-1A field: A single field indicating common information to all the co-scheduled cells

  - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication

  - Type-1C field: A single field indicating an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells

- Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration.

  - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group.

- Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed.

Furthermore, the following progress was made on DCI 0_X/1_X design.

Agreement

For DCI format 1_X/0_X which can schedule more than one cell,

- Type-1 fields at least include below:

  - Type-1A:

    - Identifier for DCI formats

    - Downlink assignment index

    - TPC for scheduled PUCCH

    - PUCCH resource indicator

    - PDSCH-to-HARQ timing indicator

    - One-shot HARQ-ACK request

- Type-2 fields at least include below:

  - New data indicator per TB

  - Redundancy version per TB

- FFS: Other fields to be included in DCI format 1_X/0_X and which type of the fields belongs to.

- FFS: size for each field

Below we list some aspects that need to be considered for mc-DCI. We note the high-level aspects (as discussed in previous sections) would influence the detailed design of mc-DCI. Nonetheless, some principles for mc-DCI design and simplifications (for certain fields such as UL-related information for PDSCH) are discussed in below.

It needs to be discussed whether mc-DCI format supports the functionality of all the DCI fields specified for existing DCI formats (i.e. 1_1/1_2/0_1/0_2) or whether it supports only a limited subset of DCI fields.

mc-DCI design should be able to maintain scheduling flexibility as much as possible, while compress the DCI information for up to 4 cells. Some fields can be grouped together, e.g. in case of intra-band carriers, while some other fields may still remain separate per-cell.

A field in DCI is needed to indicate the scheduled carriers. If the maximum number of scheduled cells is four, then a simple option for indicating the scheduled carriers is an explicit bitmap, wherein each bit indicates whether the DCI schedules PUSCH/PDSCH for the corresponding cell.

- A carrier selection field (CSF) is introduced for indication of co-scheduled cells by DCI format 0_X/1_X, wherein the field is a bitmap corresponding to the set of configured cells that can be scheduled by the DCI format.

Higher layers configure a group of cells that are scheduled by a single mc-DCI using a scheduling cell. The group of cells may be further divided into one or more subgroups(e.g. a subgroup may represent intra-band carriers and different subgroups may represent inter-band). Note that if there is only one cell per subgroup, then the subgroup specific field becomes same as individual cell-specific field. The DCI fields can then be characterized into:

- Common field

  - DCI field jointly indicates functionalities for all the scheduled cells

    - E.g. carrier selection information

- Sub-group specific field

  - DCI field jointly indicates functionalities for a subset of the scheduled cells, for example the subset may correspond to intra-band carriers

    - E.g. rate-matching indicator, TDRA

- Individual cell-specific field

  - Separate DCI fields are needed for each PUSCH/PDSCH

    - E.g. NDI

For mc-DCI, at least the following fields should be applicable for each subgroup of cells scheduled by mc-DCI

  - TDRA

  - VRB-to-PRB mapping

  - PRB bundling size indicator

  - Rate matching indicator

  - ZP CSI-RS trigger

  - Antenna port(s)

  - Transmission configuration indication

  - DM-RS sequence initialization

  - BWP

FDRA is one of the largest fields in the DCI format and the balance between the flexibility and size should be considered for FDRA in the mc-DCI design. Our preference is to retain the flexibility to indicate different PRB assignments for the cells scheduled by mc-DCI. However, to reduce overhead, larger RBG size can be used for the PRB allocations. The RBG size to use for mc-DCI can be configured individually for the scheduled cells.

For Type 1 FDRA, similar approach as that adopted for DCI format 0_2/1_2 can be used to reduce overhead. Further, 1 there is overhead when encoding the RIV as a binary number. There are NPRB⋅NPRB+12 different RIVs for a carrier with NPRB PRBs. Since this is not a power of 2 in general there is an overhead of up to 1 bit when encoding this as a binary number in the FDRA field. This overhead accumulates when having several RIVs for several carriers. If the RIVs are jointly encoded before being converted into binary, up to N-1 bits can be saved when scheduling N carriers.

- For each cell, support separate configuration of RBG size(s) used for PUSCH/PDSCH scheduling using mc-DCI.

- For frequency domain resource allocation (FDRA) using mc-DCI, support joint coding of individual RIVs of each cell to reduce overhead for FDRA type 1.

RV field can be configurable to reduce overhead.

- For mc-DCI, RV field size is explicitly configurable per cell (0/1/2 bits).

MCS index is five bits per TB. Thus, for four carriers with up to 2 TBs each, there can be 5 *2 * 4 = 40 bits occupied by MCS, which would be too large. Our preference is to allow compressed MCS indication while retaining some flexibility to indicate different MCS indications. Typically, this would allow better flexibility in case of intra-band cases and hence to be supported for case with subgrouping.

- For mc-DCI, support joint coding of MCS indication for co-scheduled cells in a subgroup

  - MCS index for a first cell is five bits (Index 1), and for each of remaining cells in the subgroup, differential MCS index is indicated relative to the Index 1.

  - For each of the remaining cells, up to 3 bits for differential MCS index, and the differential MCS offsets are configured by higher layers.

## HARQ-ACK feedback design for mc-DCI

In the following, we discuss the open issues regarding HARQ-ACK feedback design.

### Timing of HARQ-ACK feedback

With regards to the timing of PUCCH resource based on DCI 1_X, the following proposal was agreed during the last meeting.

Agreement

- When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot , where  is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and  is the last UL slot overlapping with the DL slot  for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot  for sub-slot based PUCCH.

- FFS details of reference PDSCH

In our view, the reference PDSCH should be the last PDSCH scheduled by DCI 1_X. This approach results in more efficient usage of indicated k1 values since the indicated k1 value would be used to determine a timing that it is not possible to determine based on other information. The timing between the first PDSCH and the last PDSCH can be determined from the indicated TDRA table. The timing interval that needs to be clarified, starts from the last PDSCH until the corresponding PUCCH. Moreover, in legacy procedures when a PDSCH is repeated or when multi-slot PDSCHs are scheduled, the timing of PUCCH is based on the last PDSCH.

In our view, this principle should be kept in case of PDSCHs scheduled by DCI 1_X.  Therefore, we propose the following:

- For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH among the set of co-scheduled PDSCHs is the PDSCH with the latest ending symbol.

### HARQ-ACK codebook generation

With regards to the HARQ-ACK codebook, main agreements were made during the last meeting for the design of Type-2 HARQ-ACK codebook. We discuss here the aspects that should be considered for support of Type-1 HARQ-ACK codebook.

In our view, the Type-1 HARQ-ACK codebook can be supported for DCI format 1_X. We have identified one issue that can be easily addressed to reuse the existing procedures for Type-1 HARQ-ACK codebook.

When a single DCI 1_X schedules multiple PDSCHs across cells, the co-scheduled PDSCHs may not be aligned in starting and/or ending time. This may cause problem when Type-1 HARQ-ACK codebook is applied for reporting HARQ-ACK feedback. The HARQ-ACK information corresponding to the co-scheduled PDSCHs is transmitted in a PUCCH in UL slot nU. Using the existing procedures, for a serving cell c, the UE determines a set of MA,c occasions for candidate PDSCH receptions for which the UE can transmit corresponding HARQ-ACK information in a PUCCH in slot nU.

If a co-scheduled PDSCH (or its repetition) ends in a UL slot nE such that nU-nE>maxK1, its corresponding HARQ-ACK information cannot be reported in a Type-1 HARQ-ACK codebook as there is no corresponding occasion in MA,c. Note that the indicated k1 value in the DCI associates to the reference PDSCH among the co-scheduled PDSCHs.

Examples to illustrate when the problem can occur are shown in Figure 4 and 5Figure 5. In both cases, it is assumed that the maximum K1 value is 7 and PDSCH1 is the reference PDSCH. In both cases, PDSCH1 is within UL 7 slots from the PUCCH resource. However, the situation differs for PDSCH2 where a different k1 value is indicated by DCI.

Figure 4: Illustration of scheduled PDSCHs by DCI 1_X and corresponding PUCCH resource for HARQ-ACK transmission. Maximum K1 value of 7 is assumed. The indicated k1 value is 7. PDSCH1 is the reference PDSCH. Both PDSCH1 and PDSCH2 are scheduled within 7 UL slots from PUCCH.

Figure 5: Illustration of scheduled PDSCHs by DCI 1_X and corresponding PUCCH resource for HARQ-ACK transmission. Maximum K1 value of 7 is assumed. The indicated k1 value is 6. PDSCH1 is the reference PDSCH and is scheduled within maximum 7 UL slots from PUCCH, but PDSCH2 falls outside.

In our view, the issue can be easily fixed without imposing any scheduling restrictions and increasing codebook overhead. For example, the occasion corresponding to k1=max(K1) in MA,c can be used instead for the co-scheduled PDSCH on cell c that satisfies the condition nU-nE>maxK1 described above. In case of existing another HARQ-ACK information, bundling can be performed. Therefore, we propose the following:

Type-1 HARQ-ACK codebook is supported when a DCI 1_X schedules PDSCHs across cells.

For Type-1 HARQ-ACK codebook generation corresponding to a set of co-scheduled PDSCHs scheduled by a DCI 1_X across cells, the timing occasion corresponding to Kx  (e.g., Kx=maxK1 ) in MA,c is used for a co-scheduled PDSCH on cell c that ends earlier than maxK1 UL slots from the corresponding PUCCH slot. In case of presence of other HARQ-ACK information corresponding to occasion Kx, bundling of HARQ-ACK information is performed.

### HARQ-ACK multiplexing in PUSCH

Moreover, multiplexing HARQ-ACK on PUSCH can follow the existing procedures where the indicated DAI value in the UL grant represent the codebook size. Also, the same PUSCH prioritization rules for HARQ-ACK multiplexing as legacy can be used irrespective of the PUSCHs are scheduled by a single DCI or DCI 0_X as shown in Figure 6 and 7Figure 7.

Figure 6: Example of multiplexing of a HARQ-ACK codebook corresponding to PDSCHs scheduled by DCI formats 1_X and legacy DCI format in a PUSCH scheduled by legacy DCI. The indicated DAI value in the UL grant is consistent with the size of HARQ-ACK codebook.

Figure 7: Example of multiplexing of a HARQ-ACK codebook corresponding to PDSCHs scheduled by DCI formats 1_X and legacy DCI format in a PUSCH scheduled by DCI format 0_X. The PUSCH for HARQ-ACK multiplexing is selected based on the existing PUSCH prioritization rules. The indicated DAI value in the UL grant is consistent with the size of HARQ-ACK codebook.

Therefore, we propose the following:

The value of the DAI field in a DCI format 0_X is applicable for HARQ-ACK multiplexing in any of the PUSCHs when that PUSCH for HARQ-ACK multiplexing is determined following the existing procedures.

### Other design aspects

It was discussed whether the joint operation of multi-cell DCI and multi-slot DCI should be supported. The views on benefit of the support varies. If it is supported, the joint operation should not unnecessarily increase the complexity. A that regard, HARQ-ACK codebook generation would be complicated in case a codebook includes HARQ-ACK feedback for PDSCHs scheduled by both multi-cell DCI and multi-slot DCI. Therefore, we propose:

It is not expected that the HARQ-ACK bit(s) carried by a PUCCH to include HARQ-ACK information bit(s)for PDSCH(s) scheduled by a multi-cell DCI together with HARQ-ACK bit(s) for PDSCH(s) scheduled by a multi-slot DCI.

Another design aspect that was discussed the issues of UL/SUL scheduling combined with multi-cell PUSCH scheduling and presence of NUL/SUL indicator in DCI format 0_X.  Following the same principle, we believe that we should rely on a design reusing legacy procedures. Hence, NUL/SUL related issues should be treated based on already existing rules.

UL/SUL scheduling combined with multi-cell PUSCH scheduling and presence of UL/SUL indicator in DCI format 0_X should reuse the existing procedures.

# Conclusion

In the previous sections we made the following observations:

Observation 1	For a scheduled cell, it should be possible to configure a same number of BD/CCEs for monitoring scheduling DCI formats (including 0_X/1_X and legacy DCI formats) as the case of legacy cross-carrier scheduling.

Observation 2	Duplicate counting of a BD/CCE attempt for a mc-DCI format should be avoided when counting BD/CCE across co-scheduled cells for comparison against the aggregate limits MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Maximum number of configurable cells for co-scheduling by a DCI format 1_X/0_X is 4.

Proposal 2	When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, a mc-DCI can schedule PUSCH/PDSCH on all of the cells or a subset of those cells.

Proposal 3	When mc-DCI is configured for scheduling PUSCH/PDSCH on multiple cells, for each of those cells, UE can also be configured to monitor existing single cell DCI format(s) scheduling PUSCH/PDSCH (i.e. 1_1/1_2/0_1/0_2).

Proposal 4	Size of mc-DCI (0_X/1_X) is explicitly configured by higher layers.

Proposal 5	Support independent configuration of mc-DCI for PUSCH and PDSCH.

Proposal 6	For a scheduled cell configured to be scheduled by mc-DCI, the total number of different DCI sizes with C-RNTI that the UE is configured to monitor is no more than 4 for the cell if it is an SCell and is no more than 5 if it is a PCell.

Proposal 7	BD/CCE counting for mc-DCI is based on the legacy Rel-15/16/17 BD/CCE counting mechanism with the following update

	a BD/CCE attempt for a mc-DCI format is counted only once for comparison against aggregate MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ limits in numerology buckets.

Proposal 8	Linked search space mechanism is reused for mc-DCI search space configuration.

	For each group of cells configured to be scheduled by a mc-DCI format, the linked search space on the “scheduled cell” (i.e. containing only BD/CCE info and linked search space ID) is configured on one of the cells in the group.

Proposal 9	For monitoring DCI 0_X/1_X PDCCH candidates for a set of cells, the n_CI to be used in the search space equation is explicitly configured for the set of cells.

Proposal 10	A carrier selection field (CSF) is introduced for indication of co-scheduled cells by DCI format 0_X/1_X, wherein the field is a bitmap corresponding to the set of configured cells that can be scheduled by the DCI format.

Proposal 11	For mc-DCI, at least the following fields should be applicable for each subgroup of cells scheduled by mc-DCI

	TDRA

	VRB-to-PRB mapping

	PRB bundling size indicator

	Rate matching indicator

	ZP CSI-RS trigger

	Antenna port(s)

	Transmission configuration indication

	DM-RS sequence initialization

	BWP

Proposal 12	For each cell, support separate configuration of RBG size(s) used for PUSCH/PDSCH scheduling using mc-DCI.

Proposal 13	For frequency domain resource allocation (FDRA) using mc-DCI, support joint coding of individual RIVs of each cell to reduce overhead for FDRA type 1.

Proposal 14	For mc-DCI, RV field size is explicitly configurable per cell (0/1/2 bits).

Proposal 15	For mc-DCI, support joint coding of MCS indication for co-scheduled cells in a subgroup

	MCS index for a first cell is five bits (Index 1), and for each of remaining cells in the subgroup, differential MCS index is indicated relative to the Index 1.

	For each of the remaining cells, up to 3 bits for differential MCS index, and the differential MCS offsets are configured by higher layers.

Proposal 16	For determining the timing of a PUCCH carrying HARQ-ACK information corresponding to a set of co-scheduled PDSCHs by a DCI format 1_X, the reference PDSCH among the set of co-scheduled PDSCHs is the PDSCH with the latest ending symbol.

Proposal 17	Type-1 HARQ-ACK codebook is supported when a DCI 1_X schedules PDSCHs across cells.

Proposal 18	For Type-1 HARQ-ACK codebook generation corresponding to a set of co-scheduled PDSCHs scheduled by a DCI 1_X across cells, the timing occasion corresponding to Kx  (e.g., Kx=maxK1 ) in MA,c is used for a co-scheduled PDSCH on cell c that ends earlier than maxK1 UL slots from the corresponding PUCCH slot. In case of presence of other HARQ-ACK information corresponding to occasion Kx, bundling of HARQ-ACK information is performed.

Proposal 19	The value of the DAI field in a DCI format 0_X is applicable for HARQ-ACK multiplexing in any of the PUSCHs when that PUSCH for HARQ-ACK multiplexing is determined following the existing procedures.

Proposal 20	It is not expected that the HARQ-ACK bit(s) carried by a PUCCH to include HARQ-ACK information bit(s)for PDSCH(s) scheduled by a multi-cell DCI together with HARQ-ACK bit(s) for PDSCH(s) scheduled by a multi-slot DCI.

Proposal 21	UL/SUL scheduling combined with multi-cell PUSCH scheduling and presence of UL/SUL indicator in DCI format 0_X should reuse the existing procedures.

# References

RP-222251, Revised WID on Multi-carrier enhancements , NTT DOCOMO, RAN#97-e, September 2022

R1-1805782,  Reply LS on maximum number of CCs for NR CA, RAN1, RAN1#92-bis, Sanya(CN), April 2018

# Appendix

### RAN1#109-e: Agreements and conclusions

Agreement

Agree the following terminologies ONLY for convenience of discussion:

- DCI format 0_X is used for scheduling multiple PUSCHs on multiple cells with one PUSCH per cell

- DCI format 1_X is used for scheduling multiple PDSCHs on multiple cells with one PDSCH per cell.

The above does not imply introducing new DCI format(s) at this point.

Agreement

- Different TBs are scheduled on different cells by DCI format 0_X.

- Different TBs are scheduled on different cells by DCI format 1_X.

Agreement

Fallback DCI (i.e., DCI formats 0_0 and 1_0) does not support multi-cell scheduling.

Agreement

The DCI for multi-cell scheduling is monitored only in USS set.

Agreement

- PDSCH cannot be scheduled by DCI format 0_X.

- PUSCH cannot be scheduled by DCI format 1_X.

Agreement

- All the co-scheduled cells by a DCI format 1_X and the scheduling cell are included in the same PUCCH group.

- FFS: All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same [cell or PUCCH group].

Agreement

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells including the scheduling cell.

- DCI format 0-X/1-X on a scheduling cell can be used to schedule PUSCHs/PDSCHs on multiple cells not including the scheduling cell.

Agreement

- For a UE, the maximum number of cells scheduled by a DCI format 0_X can be same or different to the maximum number of cells scheduled by a DCI format 1_X.

Working Assumption

- All HARQ-ACK codebook types (Type-1/2/3) are applicable when multi-carrier PDSCH scheduling is configured.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 0_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- One value for the maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is selected from {3, 4, 8}.

- For a UE, the maximum number of co-scheduled cells by a DCI format 1_X can be smaller than or equal to the maximum number supported in Rel-18.

Agreement

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

- DCI format 0_X can be used for single cell PUSCH scheduling.

- DCI format 1_X can be used for single cell PDSCH scheduling.

- FFS: UE monitors one of or both multi-cell scheduling DCI and legacy single cell scheduling DCI for a scheduled cell.

Agreement

- DCI format 0-X/1-X can be transmitted on PCell.

- DCI format 0-X/1-X can be transmitted on a SCell at least when the DCI format 0-X/1-X does not schedule PUSCH/PDSCH on PCell.

- FFS whether a DCI format 0-X/1-X can be transmitted on an SCell if the DCI format 0-X/1-X schedules PUSCH/PDSCH on PCell.

Agreement

Further study DCI size budget including below options for multi-cell scheduling DCI:

- Option 1: Existing DCI size budget is maintained per scheduled cell.

  - Alt 1-1: DCI size budget is maintained via DCI size alignment and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-2: DCI size budget is maintained via configured size for multi-cell scheduling DCI and DCI size budget of DCI format 0_X/1_X is counted for each of the co-scheduled cells.

  - Alt 1-3: DCI size budget is maintained via DCI size alignment and DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

- Option 2: Existing DCI size budget is not necessarily maintained per scheduled cell.

  - Alt 2-1: DCI size budget of multi-cell scheduling DCI is counted only in one scheduled cell.

  - Alt 2-2: DCI size budget of multi-cell scheduling DCI is not counted per serving cell and not considered in the related serving cell specific DCI size alignment procedure, e.g., for K co-scheduled cells, gNB guarantee the total budget of 3*K DCI sizes is not exceeded.

  - Alt 2-3: voiding the “3+1” limit for multi-cell scheduling

  - Alt 2-4: the DCI size budget for DCI size alignment can be separately configured for each cell

  - Alt 2-5: DCI size budget of the scheduling cell can be increased to account for the DCI format for multi-cell scheduling. Accordingly, the DCI size budget of a scheduled cell can be reduced.

- Other options/alternatives could be considered.

Agreement

Further study BD/CCE counting for multi-cell scheduling DCI based on below options:

- Alt 1: counted on each co-scheduled cell

- Alt 2: counted only in one scheduled cell

- Alt 3: scaled down to each of co-scheduled cell according to the number of co-scheduled cells

- Alt 4: counted as part of the scheduling cell instead of each scheduled cell

- Alt 5: scaled down to each of scheduled cells excluding scheduling cell

- Alt 6: counted on each co-scheduled cell excluding scheduling cell

- Other alternatives could be considered.

Agreement

For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered:

- Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells.

  - The table is configured by RRC signaling.

  - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X

  - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling.

- Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not

- Other options are not precluded.

- Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Agreement

For design of multi-cell scheduling DCI, companies are encouraged to consider following types of DCI fields:

- Type-1 field: A single field indicating common information to all the co-scheduled cells or separate information to each of co-scheduled cells via joint indication or an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells, or each sub-group comprising one or more co-scheduled cells where a single field is commonly applied to the co-scheduled cells belonging to a same sub-group

- Type-3 field: Common or separate to each of the co-scheduled cells or to each sub-group.

  - FFS: whether it is dependent on explicit configuration or implicit condition (e.g., intra or inter band CA, FR1 or FR2).

- Other types are not precluded.

### RAN1#110: Agreements and conclusions

Agreement

All the co-scheduled cells by a DCI format 0_X and the scheduling cell are included in the same PUCCH group.

Agreement

Confirm below working assumption reached in RAN1#109e meeting.

- (Working assumption) DCI format 0_X/1_X is a new DCI format for multi-cell scheduling

Working Assumption

For a cell within a set of cells which can be co-scheduled by a DCI format 0_X/1_X, support monitoring the DCI format 0_X/1_X and legacy single cell scheduling DCI format(s) from a same scheduling cell.

- The DCI format 0_X/1_X and the legacy DCI format(s) can be monitored simultaneously.

  - FFS: whether monitoring of the DCI format 0_X/1_X and the legacy DCI format(s) is supported for one, a subset, or all cells within the set of cells.

- FFS: number of different DCI sizes for 0_X/1_X and for legacy DCI formats

- FFS: whether to support a subset or all legacy DCI format(s) to be monitored with DCI 0_X/1_X

Working Assumption

- The maximum number of co-scheduled cells by a DCI format 1_X in Rel-18 is 4.

- The maximum number of co-scheduled cells by a DCI format 0_X in Rel-18 is 4.

- FFS: The maximum number of configurable cells for co-scheduling

Agreement

For discussing field design of DCI format 0_X/1_X which schedules more than one cell, reformulate the types of DCI fields as below:

- Type-1 field:

  - Type-1A field: A single field indicating common information to all the co-scheduled cells

  - Type-1B field: A single field indicating separate information to each of co-scheduled cells via joint indication

  - Type-1C field: A single field indicating an information to only one of co-scheduled cells

- Type-2 field: Separate field for each of the co-scheduled cells

- Type-3 field: Common or separate to each of the co-scheduled cells, or separate to each sub-group, dependent on explicit configuration.

  - Note: One sub-group comprises a subset of co-scheduled cells where a single field is commonly applied to the co-scheduled cell(s) belonging to a same sub-group.

- Note: Handling of any parameters applicable to multi-cell scheduling where corresponding fields are not included in DCI format 0_X/1_X (if any) will be separately discussed.

Agreement

- For DCI format 1_X/0_X which can schedule more than one cell,

- Type-1 fields at least include below:

  - Type-1A:

    - Identifier for DCI formats

    - Downlink assignment index

    - TPC for scheduled PUCCH

    - PUCCH resource indicator

    - PDSCH-to-HARQ timing indicator

    - One-shot HARQ-ACK request

- Type-2 fields at least include below:

  - New data indicator per TB

  - Redundancy version per TB

- FFS: Other fields to be included in DCI format 1_X/0_X and which type of the fields belongs to.

- FFS: size for each field

Agreement

- When UE detects a DCI format 1_X scheduling a set of PDSCHs, the UE provides corresponding HARQ-ACK information in a PUCCH transmission within UL slot , where  is a number of slots and is indicated by the PDSCH-to-HARQ_feedback timing indicator field in the DCI format and  is the last UL slot overlapping with the DL slot  for the reference PDSCH reception for slot-based PUCCH or an UL slot overlapping with the end of the reference PDSCH reception in DL slot  for sub-slot based PUCCH.

- FFS details of reference PDSCH

Agreement

- For Type-2 HARQ-ACK codebook, two sub-codebooks are generated with a first sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling a single cell and a second sub-codebook comprising HARQ-ACK information bits for PDSCH(s) scheduled by DCI(s) with each scheduling more than one cell.

- Separate DAI counting for DCI(s) with each scheduling a single cell and DCI(s) with each scheduling more than one cell.

- FFS whether a DCI scheduling more than one cell is associated with the first sub-codebook or the second sub-codebook when the number of cells with actual PDSCH reception due to collision with semi-static TDD DL/UL configuration is one.

- Type-2 HARQ-ACK codebook is generated by concatenating the first sub-codebook and the second sub-codebook.

- If at least one cell of the set of cells which can be co-scheduled by a DCI format 1_X is configured with maximum 2 codewords per PDSCH without spatial bundling,

  - FFS: the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell;

- Otherwise, the number of HARQ-ACK information bits for each DCI format 1_X that schedules more than one cell is equal to N, where N is the maximum number of cells which can be co-scheduled by a DCI format 1_X in the PUCCH group for the UE.

- HARQ-ACK information bits for co-scheduled PDSCHs by a DCI format 1_X is ordered based on serving cell indices associated with co-scheduled PDSCHs.

- HARQ-ACK bundling across co-scheduled cells is not supported for multi-cell scheduling.

Agreement

UE does not expect to be configured both CBG-based PDSCH/PUSCH transmission and the multi-cell PDSCH/PUSCH scheduling on the same or different cells within a same PUCCH group.

Agreement

- At least cases 1-1 and 1-2 on SCS are supported:

- Case 1-1: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and same SCS is used among all the co-scheduled cells including the scheduling cell.

- Case 1-2: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and same SCS is used among all the co-scheduled cells which may be same or different to the SCS of the scheduling cell.

- Case 1-3: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells including the scheduling cell and different SCS is used among the co-scheduled cells including the scheduling cell.

- Case 1-4: A DCI format 0-X/1-X on a scheduling cell can schedule multiple cells not including the scheduling cell and different SCS is used among the co-scheduled cells.

- FFS: Whether Case 1-3 or 1-4 is additionally supported.