3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2209864

e-Meeting, October 10th – 19th, 2022

Agenda Item:	9.13.3

Source:	Ericsson

Title:	L1 signal design and procedure for low power WUS

Document for:	Discussion

# 1	Introduction

In RAN plenary #97, the study item description (SID) on low-power wake-up signal (WUS)/wake-up receiver (WUR) was revised [1]. This study will not require existing signals to be used as WUS. All WUS solutions identified shall be able to operate in a cell supporting legacy UEs. Solutions should target substantial gains compared to the existing Rel-15/16/17 UE power saving mechanisms. Other aspects such as detection performance, coverage, UE complexity, should be covered by the evaluation.

Detailed objectives of the SI are listed below:

- Identify evaluation methodology (including the use cases) & KPIs [RAN1] - Primarily target low-power WUS/WUR for power-sensitive, small form-factor devices including IoT use cases (such as industrial sensors, controllers) and wearables - Other use cases are not precluded - Study and evaluate low-power wake-up receiver architectures [RAN1, RAN4] - Study and evaluate wake-up signal designs to support wake-up receivers [RAN1, RAN4] - Study and evaluate L1 procedures and higher layer protocol changes needed to support the wake-up signals [RAN2, RAN1] - Study potential UE power saving gains compared to the existing Rel-15/16/17 UE power saving mechanisms, the coverage availability, as well as latency impact of low-power WUR/WUS. System impact, such as network power consumption, coexistence with non-low-power-WUR UEs, network coverage/capacity/resource overhead should be included in the study [RAN1] Note: The need for RAN2 evaluation will be triggered by RAN1 when necessary.

In this contribution, we discuss the signal design and procedures of WUS/WUR according to the SID.

# 2	General design criteria

From a network vendor and ecosystem perspective, it is important that the gNB should be able to transmit the LP-WUS using existing gNB hardware and not require any new emissions or compliance requirements. Otherwise, it is difficult to enable widespread WUS support in existing deployments

It should be possible to generate LP-WUS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

In order to not negatively impact network capacity, it is important the LP-WUS can be multiplexed with other NR transmissions in time and frequency, and that any unused LP-WUS resources can be reused for dynamical scheduling.

It should be possible to multiplex the LP-WUS with other NR transmissions.

It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Regarding coverage, we think it is important from a system perspective that the coverage is not worse than for existing Paging PDCCH. Note that Rel-17 PEI is transmitted using PDCCH and DCI format 2_7.

Target the same coverage for LP-WUS as for Paging PDCCH.

Regarding detection performance, for missed detection probability we propose to use similar target for the joint LP-WUS and paging PDCCH as for PDCCH, i.e. ~10-2. The overall power saving gain depends on the LP-WUR monitoring periodicity and the false detection probability of the LP-WUS. Since PDCCH uses a CRC, the false alarm probability of PDCCH is low, ~10-6 (assuming a list-8 polar decoder). Such a low false alarm probability is likely not needed, and instead for WUS we could assume a false alarm probability for 10-3, an order of magnitude lower than the missed detection probability, or determine the target false alarm probability based on the target coverage and power saving gain.

Target a joint missed detection probability of LP-WUS and paging PDCCH of ~10-2.

Target false alarm probability for LP-WUS can be assumed to be 10-3 or alternatively determined during the evaluations to optimize the target power saving gain (in which case the assumption should be reported).

There are several possible candidates for LP-WUS design. Receiving OFDM based signals similar to NR signals such as SSS/CSI-RS with low-power WUR is one option. Other options include multi-carrier OOK, multi-carrier FSK or multi-carrier PSK. Among these options, multi-carrier OOK is based on envelope detection and is expected to provide lower power consumption while it is expected to be higher for OFDM based receivers.

IEEE 802.11ba [3] uses OFDM-based OOK generation for WUS. Individual OFDM-symbols are either treated as an ON-symbol or as an OFF-symbol. ON-symbols consist of a constant envelope signal, and the transmitter is silent during OFF-symbols. In contrast to the design goals discussed above for LP-WUS, the 802.11ba WUS is not multiplexed with other data transmissions on the same carrier. Similar ON-OFF keying should still be possible to generate and multiplex with other transmissions using selective muting of the LP-WUS subcarriers.

To simplify receiver complexity, Manchester coding is used, where ON and OFF-symbols are transmitted in pairs of either (ON, OFF) or (OFF, ON). This way the receiver can compare the received energy in the first and second half of the pair to make a decision.

For OOK WUS, the goal is to generate a time-domain signal using an OFDM transmitter which is close to an OOK waveform for the WUR.  To ensure a minimum impact on the OFDM transmitter (e.g., gNB) and avoid inter-subcarrier interference, the input to the IFFT (i.e., data in frequency domain) can be optimized such that the output of IFFT can represent a desired time-domain signal. For instance, an OOK signal with a single OOK-symbol per OFDM symbol can be generated by appropriately choosing the input on the REs corresponding to the resources used by the WUS.

Figure 1: OFDM-based generation of LP-WUS multiplexed with other NR transmissions.

The main radio is generally expected to be in sleep during LP-WUR operation. It is not clear whether the LP-WUR will be able to maintain synchronization with the network during long periods where the UE is not paged. Given this, it needs to be investigated if there is need for additional synchronization signals that the LP-WUR can use to stay synchronized or whether existing signals are sufficient.

In IEEE 802.11ba [3], WUR Beacon frames are transmitted periodically by a WUR AP and used by a WUR UE to maintain time synchronization to support the WUR duty cycle operation. In addition, there is a WUR-Sync field transmitted together with every WUS transmission.

If an additional synchronization signal is needed for LP-WUS/WUR operation, the synchronization signal may be transmitted either in conjunction with the LP-WUS or separately from LP-WUS.

Study the need for additional synchronization signals for LP-WUR or whether existing signals are sufficient.

While employing LP-WUR can provide significant power saving for the UE, its performance is also susceptible to false alarm events for which the main radio is mistakenly woken up. Since main radio consumes a considerable amount of energy for ramping up/down, false alarm events can result in additional UE power consumption (or even negative power saving gain). If LP-WUR results in high false alarm rate, employing it in some cases not only may not have any benefit, but it can also drain the UE battery. To cope with this issue, it is important to minimize the false wakeup events.

In general, false wakeups are caused due to false alarms and false paging as defined below:

- False alarm: PHY-layer effect of WUS incorrectly being detected when there is none (single UE effect).

- False paging: HL-effect of WUS to one UE unnecessarily waking up other UEs sharing the same WUS resource (multi-UE effect).

To reduce false wakeups, LP-WUS multiplexing/separation techniques can be employed. For example, enhancements for UE grouping, different LP-WUS time-frequency resources, or using multiple duty-cycle configurations for LP-WUR operation can be considered.

UE sub-grouping is one option to reduce the risk of false paging to other UEs in the same PO. For example, sub-groups can be assigned to the UEs by the NW either randomly or based on UE individual characteristics, such as,  UE type (e.g., RedCap, eMBB), UE mobility patterns, UE traffic patterns, paging probability, etc. In Rel-17 PEI, UEs monitoring the same PO can be divided into one or more subgroups based on CN assignment or UE_ID.

Study solutions for reduction of false wakeup events for efficient LP-WUR operation (e.g., UE subgrouping).

# 3	Conclusion

In this contribution, we made the following proposals:

Proposal 1	It should be possible to generate LP-WUS transmissions using existing gNB hardware and not trigger any new emissions or compliance requirements.

Proposal 2	It should be possible to multiplex the LP-WUS with other NR transmissions.

Proposal 3	It should be possible to reuse any unused LP-WUS time and frequency resources for other transmissions.

Proposal 4	Target the same coverage for LP-WUS as for Paging PDCCH.

Proposal 5	Target a joint missed detection probability of LP-WUS and paging PDCCH of ~10-2.

Proposal 6	Target false alarm probability for LP-WUS can be assumed to be 10-3 or alternatively determined during the evaluations to optimize the target power saving gain (in which case the assumption should be reported).

Proposal 7	Study the need for additional synchronization signals for LP-WUR or whether existing signals are sufficient.

Proposal 8	Study solutions for reduction of false wakeup events for efficient LP-WUR operation (e.g., UE subgrouping).

# References

RP-222644, “Revised SID: Study on low-power Wake-up Signal and Receiver for NR,” 3GPP TSG-RAN#97e, Sept 12 - 16, 2022.

“IEEE Standard for Information Technology--Telecommunications and Information Exchange between Systems--Local and Metropolitan Area Networks-Specific Requirements--Part 11: Wireless LAN Medium Access Control (MAC) and Physical Layer (PHY) Specifications - Amendment 3: Wake-Up Radio Operation”, 2021.