3GPP TSG RAN WG1 #110bis-e			R1-2209893

e-meeting, October 10th – 19th, 2022

Source:	NTT DOCOMO, INC.

Title:	Discussion on multi-panel transmission

Agenda Item:	9.1.4.1

Document for: 	Discussion and Decision

- Introduction

In RAN1#109e meeting, following agreements were made for STxMP [1].

Working assumption Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e. Agreement For single-DCI based STxMP PUSCH SDM scheme, support the layer combinations of {1+1, 1+2, 2+1 and 2+2} - FFS on layer combinations of {1+3} and {3+1} considering the performance gain, system/UE complexity, specification efforts, etc. - FFS: the option of using layer combination of 0+n and n+0 for dynamic switch between single-panel and STxMP (n=1 or 2, 3 or 4). - This applies to SDM with 1 CW at least. Agreement To enhance the DMRS port indication for SDM scheme of STxMP PUSCH transmission in single-DCI based mTRP system, study the following aspects: - Whether the indicated DMRS ports can be in different or same CDM group? - How to determine the port partition among PUSCH layers. - How to map DMRS ports with PUSCH layers from different panels. - Whether to use one DCI field or two DCI fields for DMRS port indication - How to indicate layer combination that is used to partition DMRS port partition among PUSCH layers. - Other aspects are not precluded. Agreement For multi-DCI based STxMP PUSCH+PUSCH transmission, study and evaluate the following aspects: - Two PUSCHs are associated with different TRPs and transmitted from different UE panels. The total number of layers of these two PUSCHs is up to 4. - Study STxMP of PUSCH+PUSCH transmission where it is some combination of DG-PUSCH, CG-PUSCH and msg3/msgA PUSCH. - The overlapping type(s) of fully/partially in time domain and fully/partially/non-overlapping in frequency domain are to be studied and justified for PUSCH+PUSCH. Note: The above study shall take into account the UE implementation and RF considerations. Note: Study the conditions required for STxMP PUSCH+PUSCH. Note: Other aspects are not precluded. Agreement Study the enhancement of SRS resource set configuration and SRI/TPMI indication for single-DCI based STxMP PUSCH scheme: - The configuration of two SRS resource sets, SRS resource set indicator field, two SRI fields and two TPMI fields of Rel-17 mTRP PUSCH TDM repetition is the starting point. - FFS: The configuration of one SRS resource set, one or two SRI fields and one or two TPMI fields - Note: This proposal does not mean that any possible SRI/TPMI enhancement on STxMP would be precluded. In RAN1#110, companies can suggest the detail SRI/TPMI enhancement with reasonable analysis and evaluation result. Agreement Study and evaluate STxMP PUCCH based on the following: - For single-DCI based STxMP PUCCH transmissions, companies to provide the detailed description of the scheme being evaluated along with evaluation results in contribution. - For multi-DCI based STxMP PUCCH transmissions, transmitting two PUCCH resources with independent UCI payload to different TRPs with different UE panels that are fully or partially overlapping in time domain and partially/fully/non-overlapping in frequency domain can be considered. - Note: Companies can reuse the EVM assumptions of Rel-18 STxMP as agreed in RAN1#109-e (other than the parameters that are specific to PUSCH) as well as Rel-17 EVM for PUCCH as agreed in RAN1#102-e (PUCCH format, # of RBs/symbols, UCI payload, and Frequency hopping as shown below). - Baseline scheme can be Rel-15 PUCCH or Rel-17 mTRP PUCCH repetition.

In this contribution, we discuss enhancements on STxMP PUSCH and STxMP PUCCH schemes.

- Discussion on multi-panel PUSCH transmission

2.1 STxMP PUSCH in S-DCI M-TRP

For STxMP PUSCH in S-DCI M-TRP, following working assumption was made in the last meeting.

Working assumption Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18: - SDM scheme - In RAN1#110bis-e, RAN1 will only consider SFN based transmission scheme to support in addition to the above. Decision to support or not to be made in RAN1#110bis-e.

STxMP SDM scheme is beneficial for throughput improvement. For SDM scheme, whether 1CW or 2CWs is transmitted in a PUSCH was discussed. A benefit of transmitting 2CW is that MCS can be indicated per CW, i.e., indicated per panel/TRP. Considering that the channel condition between two panels and two TRPs can be different, the best MCS for two panels/TRPs can be different. System level simulation is performed to evaluate the throughput of single panel Tx, STxMP SDM scheme with 1CW and STxMP SDM scheme with 2CWs. The simulation assumptions are given in Table II in appendix. In the simulation, for single panel Tx, the panel with the best signal strength is selected. For SDM scheme, dynamic switching between single panel Tx and STxMP SDM scheme is used. For SDM scheme with 2CWs, per CW MCS is used. The simulation results are shown in Table I. From the simulation results, it can be observed that in both low RU and high RU cases SDM scheme with 2CWs provides performance gain compared to single panel Tx. SDM scheme with 1CW outperforms single panel Tx in low RU case, while in high RU case, SDM scheme 1CW provides some performance gain compared to single panel Tx for average throughput and has performance loss for 5%-ile UE. SDM scheme with 2CWs outperforms SDM with 1CW in both low RU and high RU cases and especially in high RU case. Thus, SDM scheme should be supported and SDM scheme with 2CWs should be supported.

Table I. Throughput performance of STxMP PUSCH SDM scheme

| RU | (Mbps) | Single panel Tx | SDM scheme (1CW) | SDM scheme (2CW) |
| --- | --- | --- | --- | --- |
| 20% | Avg. UPT | 251.02 | 266.19 6.04% (↑) | 272.07 8.39% (↑) |
|  | 5% UPT | 81.84 | 90.69 10.81% (↑) | 95.33 16.48% (↑) |
| 50% | Avg. UPT | 193.95 | 198.42 2.31% (↑) | 209.62 8.08% (↑) |
|  | 5% UPT | 46.80 | 43.24 -7.60% (↓) | 58.97 26.01% (↑) |

Observation 2.1:

- SDM scheme with 2CWs provides performance gain compared to single panel Tx in both low RU and high RU cases.

- SDM scheme with 1CW outperforms single panel Tx in low RU case. SDM scheme 1CW provides some performance gain compared to single panel Tx for average throughput and has performance loss for 5%-ile UE in high RU case.

- SDM scheme with 2CWs outperforms SDM with 1CW in both low RU and high RU cases and especially in high RU case.

Proposal 2.1:

- Confirm the following working assumption.

  - Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

    - SDM scheme

Proposal 2.2:

  - Support SDM scheme with 2CWs.

To support SDM scheme, for CB PUSCH, per panel/TRP SRI/TPMI indication is needed. For per panel/TRP SRI indication, solutions in Rel-17 M-TRP PUSCH can be reused, i.e., two CB SRS resource sets can be configured and two SRI fields can be indicated. For per panel/TRP TPMI indication, two options can be considered.

- Option1: one TPMI is indicated for layers and antenna ports across two panels.

- Option2: two TPMIs are indicated for layers and antenna ports of two panels respectively.

It is noted in WID that no new codebook is introduced. With existing codebook, option1 can only support the cases where total number of antenna ports across two panels is up to four. And with existing codebook, option1 cannot support the case where one panel is indicated with non-coherent precoder while the other panel is indicated with coherent precoder. Thus, option2 is preferred. To indicate two TPMIs, similar as Rel-17 M-TRP PUSCH, two TPMI fields can be indicated for two panels/TRPs respectively. However, the 2nd TPMI fields in Rel-17 cannot be reused. In Rel-17, two PUSCH repetitions have same rank, hence the 2nd TPMI field indicates from TPMIs with the rank indicated in the 1st TPMI field. In STxMP SDM scheme, the rank of two panels can be different. On the other hand, similar issue as Rel-17 M-TRP PUSCH exists. Candidate rank of the 2nd TPMI field is restricted by the rank indicated by 1st TPMI field, i.e., sum of rank indicated by the two TPMI fields should be no larger than the maxRank of STxMP SDM scheme. Thus, following options are considered for the 2nd TPMI field.

- Option1: Assuming the maxRank of STxMP SDM scheme is X, regardless of the rank indicated by the 1st TPMI field, the 2nd TPMI field applies TPMI table with maxRank=X. And UE expects that the sum of rank indicated by two TPMI fields is no larger than the maxRank of STxMP SDM scheme.

- Option2: Assuming the maxRank of STxMP SDM scheme is X, regardless of the rank indicated by the 1st TPMI field, the 2nd TPMI field applies TPMI table with maxRank=X-1. And UE expects that the sum of rank indicated by two TPMI fields is no larger than the maxRank of STxMP SDM scheme.

- Option3: Assuming the maxRank of STxMP SDM scheme is X and the 1st TPMI field indicates rank of 1st panel/TRP is Y, the 2nd TPMI field applies TPMI table with the maxRank=X-Y.

Compared with option1, option2/3 saves 1-bit DCI overhead in some cases, e.g., when maxRank of STxMP SDM scheme is 2. And compared with option3, option2 is simpler. Thus, option2 is preferred.

Proposal 2.3:

- For STxMP SDM schemes for CB PUSCH, two CB SRS resource sets can be configured and the two SRI fields in Rel-17 can be reused.

Proposal 2.4:

- For STxMP SDM scheme for CB PUSCH, two TPMI fields can be indicated for two panels/TRPs respectively. Assuming the maxRank of STxMP SDM scheme is X, the 2nd TPMI field applies TPMI table with maxRank=X-1.

For STxMP SDM scheme for NCB PUSCH, per panel/TRP SRI indication is needed. For per panel/TRP SRI indication, two NCB SRS resource sets can be configured, and following options can be considered.

- Option1: one SRI(s) is indicated for SRS resources across two panels.

- Option2: two SRI(s) are indicated for SRS resources of two panels respectively.

With existing SRI table, option1 can only support limited cases where total number of antenna ports across two panels is up to four. Thus, option2 is preferred. To indicate two SRI(s), similar as Rel-17 M-TRP PUSCH, two SRI fields can be indicated for two panels/TRPs respectively. However, the 2nd SRI fields in Rel-17 cannot be reused. In Rel-17, two PUSCH repetitions have same rank, hence the 2nd SRI field indicates from SRI(s) with the rank indicated in the 1st SRI field. In STxMP SDM scheme, the rank of two panels can be different. On the other hand, similar issue as Rel-17 M-TRP PUSCH exists. Candidate rank of the 2nd SRI field is restricted by the rank indicated by the1st SRI field, i.e., sum of rank indicated by the two SRI fields should be no larger than the maxRank of STxMP SDM scheme. Thus, following options are considered for the 2nd SRI field.

- Option1: Assuming the maxRank of STxMP SDM scheme is X, regardless of the rank indicated by the 1st SRI field, the 2nd SRI field applies SRI table with maxRank=X. And UE expects that the sum of rank indicated by two SRI fields is no larger than the maxRank of STxMP SDM scheme.

- Option2: Assuming the maxRank of STxMP SDM scheme is X, regardless of rank indicated in the 1st SRI field, the 2nd SRI field applies SRI table with maxRank=X-1. And UE expects that the sum of rank indicated by two SRI fields is no larger than the maxRank of STxMP scheme.

- Option3: Assuming the maxRank of STxMP SDM scheme is X and the 1st SRI field indicates rank=Y, the 2nd SRI field applies SRI table with maxRank=X-Y.

Compared with option1, option2/3 saves 1-bit overhead of DCI in some cases, e.g., when maxRank of STxMP SDM scheme is 2. And compared with option3, option2 is simpler. Thus, option2 is preferred.

Proposal 2.5:

- For STxMP SDM scheme for NCB PUSCH, two NCB SRS resource sets can be configured and two SRI fields can be indicated. Assuming the maxRank of STxMP SDM scheme is X, the 2nd SRI field applies SRI table with maxRank=X-1.

Furthermore, for STxMP SDM scheme, indication of DMRS ports and mapping between DMRS ports and panels/TRPs needs to be studied. For indication of DMRS ports, existing ‘antenna ports” field can be used to indicate DMRS ports across two panels. Enhancement on ‘antenna port’ field is not needed. In addition, whether DMRS ports for two panels can be within same or different CDM group was discussed. For STxMP SDM scheme, interference from the other panel will be tolerable, thus, there is no issue if DMRS ports of two panels/TRPs are within same CDM group. Thus, DMRS port indicated for two panels/TRPs can be either within same or different CDM group. For mapping between DMRS ports and two panels/TRPs, assuming the first X layers of the PUSCH are mapped to the first panel and the remaining layers are mapped to the second panel, a straightforward way is that the first X DMRS ports can be mapped to first panel and the remaining DMRS ports can be mapped to second panel.

Proposal 2.6:

- Existing ‘antenna ports” field is used to indicate DMRS ports across two panels. No enhancement on ‘antenna ports’ field.

- DMRS port indicated for two panels/TRPs can be either within same or different CDM group.

- Assuming the first X layers of the PUSCH are mapped to the first panel and the remaining layers are mapped to the second panel, the first X DMRS ports can be mapped to first panel and the remaining DMRS ports can be mapped to second panel.

To support STxMP PUSCH SDM scheme with 2CW, CW-to-layer mapping needs to be studied. For example, in case of rank=2, CW#0 and #1 may be mapped layer#0 and #1, respectively. In addition, per codeword MCS/NDI/RV indication and UCI multiplexing need to be studied. These issues are similar as for 2CWs in 8Tx UL, hence common design for STxMP and 8Tx can be considered.

Proposal 2.7:

- For STxMP PUSCH SDM scheme with 2CW, following issues need to be further studied.

  - CW-to-layer mapping

  - Per codeword MCS/NDI/RV indication. Common design for STxMP and 8Tx can be considered.

  - UCI multiplexing. Common design for STxMP and 8Tx can be considered.

In addition to SDM scheme, SFN scheme can be beneficial for reliability improvement. In Rel-17, TDM M-TRP PUSCH repetition was supported to improve reliability. Link-level simulation is performed to evaluate the performance of Rel-17 M-TRP TDM repetition and SFN scheme. The simulation assumptions are given in Table I in appendix. In the simulation, for SFN scheme, two repetitions are assumed to use same precoding and same RV. The simulation results are shown in Figure1 and Figure2 for low data rate and moderate data rate respectively. In the figures, “SFN XdB” shows the results assuming each TRP receives signals from both panels and at each TRP the signal from the other panel is assumed to be XdB lower than the signal from its best panel. The results of X=6dB and X=20dB are shown. “SFN w/o” in the figures shows the results assuming each TRP only receives signal from its best panel. From the simulation results, it can be observed that for low code rate, when X=6dB SFN has similar performance as TDM repetition, when X=20dB or when each TRP only receives signal from the best panel SFN is about 0.2dB worse than TDM repetition. For moderate code rate, when X=6dB SFN is about 0.6dB worse than TDM repetition, when X=20dB or when each TRP only receives signal from the best panel SFN is about 0.8dB worse than TDM repetition. On the other hand, SFN scheme has lower latency and less control signaling overhead since same precoding is used than TDM repetition. Thus, STxMP SFN scheme can be considered.

Figure1. BLER performance of STxMP PUSCH SFN scheme at low code rate

Figure2. BLER performance of STxMP PUSCH SFN scheme at moderate code rate

Observation 2.2:

- Fr low code rate, when X=6dB, SFN has similar performance as TDM repetition, when X=20dB or when each TRP only receives signal from the best panel, SFN is about 0.2dB worse than TDM repetition.

- For moderate code rate, when X=6dB, SFN is about 0.6dB worse than TDM repetition, when X=20dB or when each TRP only receives signal from the best panel, SFN is about 0.8dB worse than TDM repetition.

Proposal 2.8:

- For STxMP PUSCH in S-DCI M-TRP, support SFN scheme.

2.2 STxMP PUSCH in M-DCI M-TRP

With M-DCI M-TRP operation, simultaneous multi-panel transmission can be beneficial for throughput improvement of PUSCH.

With STxMP, simultaneous transmission of PUSCHs overlapped in time domain and associated with different panel/TRP can be supported. Furthermore, simultaneous transmission of overlapped DG PUSCH+DG PUSCH, CG PUSCH+CG PUSCH and DG PUSCH+CG PUSCH can be supported. For DG+DG, associated between PUSCH and TRP can be determined based on CORESETPoolIndex of its scheduling DCI, i.e., support two PDCCHs with different CORESETPoolIndex scheduling overlapped PUSCHs. For DG+CG and CG+CG, association between CG PUSCH and TRP can be further discussed, e.g., CORESETPoolIndex can be configured in CG configuration.

Proposal 2.9:

- For STxMP in M-DCI M-TRP, support simultaneous transmission of PUSCHs overlapped in time domain and associated with different panel/TRP.

  - Support simultaneous transmission of overlapped DG PUSCH+DG PUSCH, CG PUSCH+PUSCH and DG PUSCH+CG PUSCH.

  - For DG PUSCH+DG PUSCH, association between PUSCH and TRP can be determined based on CORESETPoolIndex of scheduling DCI, i.e., support two PDCCHs with different CORESETPoolIndex scheduling overlapped PUSCHs

  - For DG PUSCH+CG PUSCH or CG PUSCH+CG PUSCH, association between CG PUSCH and and TRP needs to be further discussed, e.g., CORESETPoolIndex can be configured in CG configuration.

- Discussion on multi-panel PUCCH transmission

3.1 STxMP PUCCH in S-DCI M-TRP

For STxMP PUCCH in S-DCI M-TRP, following schemes can be considered to for reliability improvement.

- PUCCH FDM-A scheme: different frequency domain parts of one PUCCH resource are transmitted by two different UE panels.

- PUCCH FDM-B scheme: two FDM-ed PUCCH transmission occasion of the same UCI with the same PUCCH format are transmitted from two different UE panels

- PUCCH SFN scheme: The same PUCCH/PUCCH-DMRS is transmitted from two different UE panels simultaneously.

In Rel-17, TDM M-TRP PUCCH repetition was supported. Compared to Rel-17 TDM PUCCH repetition, FDM-A/FDM-B/SFN scheme can be beneficial for latency reduction. For FDM-A scheme, different frequency parts of one PUCCH are transmitted by two panels to TRPs respectively, hence a drawback is that if one panel/TRP is blocked, the PUCCH may not be decoded successfully. Considering specification impact, for SFN scheme, per panel/TRP beam indication can be supported with extension of unified TCI framework. For FDM scheme, in addition to per panel/TRP beam indication, frequency domain resource allocation needs to be considered. For FDM-A, the association between frequency parts of a PUCCH resource and two panels/TRPs needs to be studied. For FDM-B, resource allocation of two repetitions needs to be studied. Configuring two sets of frequency domain resources for one PUCCH resource or indicating two PUCCH resources for a PUCCH transmission needs to be considered, which may bring more spec. impact. Thus, for STxMP PUCCH in S-DCI M-TRP, SFN scheme is preferred.

Link-level simulation is performed to evaluate the performance of Rel-17 M-TRP TDM repetition and SFN scheme. The simulation assumptions are given in Table III in appendix. The simulation results are shown in Figure3 and Figure4 for PUCCH format 1 and format 3 respectively. In the figures, “SFN XdB” shows the results assuming each TRP receives signals from both panels and for each TRP the signal from the other panel is assumed to be XdB lower than the signal from its best panel. The results of X=6dB and X=20dB are shown. “SFN w/o” in the figures shows the results assuming each TRP only receives signal from its best panel. From the simulation results, it can be observed that for both PUCCH format 1 and format 3, when X=6dB or X=20dB or when each TRP only receives signal from the best panel, SFN is about 1dB worse than TDM repetition. On the other hand, SFN scheme has lower latency than TDM repetition. Thus, STxMP SFN scheme can be considered

Figure3. BLER performance of STxMP PUCCH SFN scheme for PUCCH format 1

Figure4. BLER performance of STxMP PUCCH SFN scheme for PUCCH format 3

Observation 3.1:

- For both PUCCH format 1 and format 3, when X=6dB or X=20dB or when each TRP only receives signal from the best panel, SFN scheme is about 1dB worse than TDM repetition.

Proposal 3.1:

- For STxMP PUCCH in S-DCI M-TRP, PUCCH SFN scheme can be considered, i.e., the same PUCCH is transmitted from two different UE panels simultaneously.

3.2 STxMP PUCCH in M-DCI M-TRP

For STxMP PUCCH in M-DCI M-TRP, transmitting two PUCCH resources overlapped in time to different TRPs with different UE panels can be considered. In Rel-17, when two PUCCH transmissions overlap, UE resolves the overlapping by multiplexing the UCIs in one PUCCH or dropping one of the PUCCHs following certain rules. With simultaneous multi-panel Tx, allowing simultaneous transmission of two overlapped PUCCHs associated different panel/TRP can be considered. To support simultaneous transmission of two overlapped PUCCHs associated with different panel/TRP, following enhancements on UCI multiplexing procedure can be considered. In case of PUCCH overlapping, overlapping among PUCCH transmissions associated with same panel/TRP can be resolved first with Rel-17 UCI multiplexing/dropping rule. Then, for overlapping between two PUCCH transmissions associated with different panel/TRP, the two PUCCHs can be transmitted simultaneously.

Proposal 3.2:

- For STxMP PUCCH in M-DCI M-TRP, support transmitting two PUCCH resources that are overlapped in time to different TRPs with different UE panels. Following enhancement can be considered for UCI multiplexing procedure.

  - In case of PUCCH overlapping, overlapping among PUCCH transmissions associated with same panel/TRP can be resolved first with Rel-17 UCI multiplexing/dropping rule. Then, for overlapping between two PUCCH transmissions associated with different panel/TRP, the two PUCCHs can be transmitted simultaneously.

- Conclusion

In this contribution, we discussed enhancements on simultaneous multi-panel transmission. Based on the discussion, we made following proposals.

Observation 2.1:

- SDM scheme with 2CWs provides performance gain compared to single panel Tx in both low RU and high RU cases.

- SDM scheme with 1CW outperforms single panel Tx in low RU case. SDM scheme 1CW provides some performance gain compared to single panel Tx for average throughput and has performance loss for 5%-ile UE in high RU case.

- SDM scheme with 2CWs outperforms SDM with 1CW in both low RU and high RU cases and especially in high RU case.

Proposal 2.1:

- Confirm the following working assumption.

  - Support the following scheme for STxMP PUSCH transmission in single-DCI based mTRP system in Rel-18:

    - SDM scheme

Proposal 2.2:

  - Support SDM scheme with 2CWs.

Proposal 2.3:

- For STxMP SDM schemes for CB PUSCH, two CB SRS resource sets can be configured and the two SRI fields in Rel-17 can be reused.

Proposal 2.4:

- For STxMP SDM scheme for CB PUSCH, two TPMI fields can be indicated for two panels/TRPs respectively. Assuming the maxRank of STxMP SDM scheme is X, the 2nd TPMI field applies TPMI table with maxRank=X-1.

Proposal 2.5:

- For STxMP SDM scheme for NCB PUSCH, two NCB SRS resource sets can be configured and two SRI fields can be indicated. Assuming the maxRank of STxMP SDM scheme is X, the 2nd SRI field applies SRI table with maxRank=X-1.

Proposal 2.6:

- Existing ‘antenna ports” field is used to indicate DMRS ports across two panels. No enhancement on ‘antenna ports’ field.

- DMRS port indicated for two panels/TRPs can be either within same or different CDM group.

- Assuming the first X layers of the PUSCH are mapped to the first panel and the remaining layers are mapped to the second panel, the first X DMRS ports can be mapped to first panel and the remaining DMRS ports can be mapped to second panel.

Proposal 2.7:

- For STxMP PUSCH SDM scheme with 2CW, following issues need to be further studied.

  - CW-to-layer mapping

  - Per codeword MCS/NDI/RV indication. Common design for STxMP and 8Tx can be considered.

  - UCI multiplexing. Common design for STxMP and 8Tx can be considered.

Observation 2.2:

- Fr low code rate, when X=6dB, SFN has similar performance as TDM repetition, when X=20dB or when each TRP only receives signal from the best panel, SFN is about 0.2dB worse than TDM repetition.

- For moderate code rate, when X=6dB, SFN is about 0.6dB worse than TDM repetition, when X=20dB or when each TRP only receives signal from the best panel, SFN is about 0.8dB worse than TDM repetition.

Proposal 2.8:

- For STxMP PUSCH in S-DCI M-TRP, support SFN scheme.

Proposal 2.9:

- For STxMP in M-DCI M-TRP, support simultaneous transmission of PUSCHs overlapped in time domain and associated with different panel/TRP.

  - Support simultaneous transmission of overlapped DG PUSCH+DG PUSCH, CG PUSCH+PUSCH and DG PUSCH+CG PUSCH.

  - For DG PUSCH+DG PUSCH, association between PUSCH and TRP can be determined based on CORESETPoolIndex of scheduling DCI, i.e., support two PDCCHs with different CORESETPoolIndex scheduling overlapped PUSCHs

  - For DG PUSCH+CG PUSCH or CG PUSCH+CG PUSCH, association between CG PUSCH and and TRP needs to be further discussed, e.g., CORESETPoolIndex can be configured in CG configuration.

Observation 3.1:

- For both PUCCH format 1 and format 3, when X=6dB or X=20dB or when each TRP only receives signal from the best panel, SFN scheme is about 1dB worse than TDM repetition.

Proposal 3.1:

- For STxMP PUCCH in S-DCI M-TRP, PUCCH SFN scheme can be considered, i.e., the same PUCCH is transmitted from two different UE panels simultaneously.

Proposal 3.2:

- For STxMP PUCCH in M-DCI M-TRP, support transmitting two PUCCH resources that are overlapped in time to different TRPs with different UE panels. Following enhancement can be considered for UCI multiplexing procedure.

  - In case of PUCCH overlapping, overlapping among PUCCH transmissions associated with same panel/TRP can be resolved first with Rel-17 UCI multiplexing/dropping rule. Then, for overlapping between two PUCCH transmissions associated with different panel/TRP, the two PUCCHs can be transmitted simultaneously.

Reference

[1] RAN1 Meeting #110 Chairman’s Notes, August 2022

Appendix

Table I: LLS assumptions for STxMP PUSCH in S-DCI M-TRP

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, |
|  | SCS: 120 kHz, |
| Channel model | CDL-A, DS=20ns |
| BS antenna configuration | 2 TRP and 2 Rx ports per TRP |
| UE antenna configuration | 2 panels and 2 Tx ports on each panel |
| Blockage | No blockage is assumed |
| # of RBs/symbols | 31RBs, 4OS |
| DMRS pattern | DM-RS configuration type 1 for PUSCH |
| Code rates | QPSK CodeRate 120/1024 and 379/1024 |
| Frequency hopping | No frequency hopping |
| Precoding | SVD granularity: 1 RB |
| Redundancy Version | RV =[0, 2] with soft combining for repetition |
| Cross-link interference between 2 panels | At a TRP, signal from the other panel is assumed to be X=0/6/20 dB lower than the signal for its best panel. |
| Receiver assumption | MMSE |

Table II: SLS assumptions for STxMP PUSCH in S-DCI M-TRP

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, BW: 80 MHz, |
| Scenarios | Dense urban (macro-layer only, TR 38.913) @FR2, 200m ISD, 2-tier model with wrap-around (7 sites, 3 sectors/cells per cell), 100% outdoor |
| UE speed | 3 km/hr for all UEs |
| Maximum UE Tx Power | Max TRP of 23 dBm and max EIRP 43 dBm per panel |
| BS receiver Noise Figure | 7 dB |
| BS Antenna Configuration | (M, N, P, Mg, Ng) = (4, 8, 2, 2, 2). (dV, dH) = (0.5, 0.5) λ. (dg,V, dg,H) = (2.0, 4.0) λ 2TxRU per panel, 12 beams |
| BS Antenna radiation pattern | TR 38.802 Table A.2.1-6, Table A.2.1-7 |
| UE antenna configuration | (M, N, P, Mg, Ng) = (2, 4, 2, 1, 2); (dV, dH) = (0.5, 0.5)λ. (dg,V, dg,H) = (0, 0)λ. * Θmg,ng=90°; Ω0,1=Ω0,0+180. The polarization angles are 0 and 90 2TxRU per panel, 8 beams |
| UE Antenna radiation pattern | TR 38.802 Table A.2.1-8 |
| UE dropping | Random |
| UE and panel orientation | Vertical but random in azimuth |
| Traffic Model | FTP model 1 with packet size 0.5Mbytes (other value is not precluded). |
| Control and RS overhead | No control and RS overhead |
| BF/Precoder scheme | SVD |
| UE Antenna height | 1.5 m |
| UL MIMO Mode, rank | UL SU-MIMO Up to rank 4 for STxMP with 2 panels. |
| Per panel power control and other issues that are affected by RF transmission chain architecture | Per panel power control is assumed |
| Cross-link interference between 2 panels | Cross-link interference between 2 panels is assumed |

Table III: LLS assumptions for STxMP PUCCH in S-DCI M-TRP

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz, SCS: 120 kHz, |
| Channel model | CDL-A, DS=20ns |
| BS antenna configuration | 2 TRP and 2 Rx ports per TRP |
| UE antenna configuration | 2 panels and 2 Tx ports on each panel |
| Blockage | No blockage is assumed |
| PUCCH format | Format 1 and 3. |
| # of RBs/symbols | PUCCH Format 1: 4 symbols, 1 RB |
|  | PUCCH Format 3: 4 symbols, 1 RB |
| UCI payload | 2 bits for PUCCH Format 1 22bit for PUCCH format 3 |
| Frequency hopping | No frequency hopping |
| Number of repetitions (when applicable) | 2 for R17 M-TRP TDM repetition |
| Receiver assumption | MMSE |