3GPP TSG RAN WG1 #110bis-e	R1-2209941

Online, October 10 – 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | ChannelAccess-Cpext field in UL RAR grant. |
| --- | --- |
| Source to WG: | Moderator (Qualcomm) |  |  |  |  |  |  |  |  |  |
| Source to TSG: | RAN1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_ext_to_71GHz-Core |  |  |  |  |  | Date: |  |  | 2022-22-08 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ChannelAccess-Cpext field is present in RAR UL grant for FR 2-2, the length is fixed to two bits. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Random access reponse procedure sections 8.2 and 8.2A, also point to a new table in 38.212 to interpre the ChannelAccess-CPext field in the DCI, namely Table 7.3.1.1.1-4B in TS 38.212 for FR2-2 |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - The 38.213 R17 specification will remain incomplete |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2, 8.2A |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  |  | X | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## 8.2	Random access response - Type-1 random access procedure

** Unchanged parts omitted***

The ChannelAccess-CPext field indicates a channel access type and CP extension for operation with shared spectrum channel access [15, TS 37.213] as defined in Table 7.3.1.1.1-4 in [5, TS 38.212] for FR1, or Table 7.3.1.1.1-4B in [5,TS 38.212] for FR2-2 when channelAccessMode2-r17 is provided, or Table 7.3.1.1.1-4A in [5, TS 38.212] if channelAccessMode = "semiStatic" is provided.

** Unchanged parts omitted***

## 8.2A	Random access response - Type-2 random access procedure

** Unchanged parts omitted***

-	for operation with shared spectrum channel access, a channel access type and CP extension [15, TS 37.213] for a PUCCH transmission is indicated by a ChannelAccess-CPext field in the successRAR as defined in Table 7.3.1.1.1-4 in [5, TS 38.212] for FR1, or Table 7.3.1.1.1-4B in [5, TS 38.212] for FR2-2 when channelAccessMode2-r17 is provided,  or Table 7.3.1.1.1-4A in [5, TS 38.212] if channelAccessMode = "semiStatic" is provided

** Unchanged parts omitted***