3GPP TSG RAN WG1 #110bis		     R1-2209966

e-Meeting, October 10th – October 19th, 2022

Agenda item:	8.17

Source: 	Qualcomm Incorporated

Title: 	Remaining Issues of Rel-17 UL Tx switching

Document for:	Discussion/Decision

# Introduction

In RAN1#110 meeting, SRS carrier switching together with UL Tx switching was approved and following CR of TS38.214 was agreed.

< Unchanged parts are omitted > #### 6.2.1.3 UE sounding procedure between component carriers For a carrier of a serving cell c1 with slot formats comprised of DL and UL symbols, not configured for PUSCH/PUCCH transmission, denote c2 as the corresponding carrier of a serving cell whose UL transmissions are temporarily suspended as signalled by higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier. Define the set Sc2={c2, s1(c2)…sN-1(c2)} as the set of carriers of serving cells that each carrier meets one of the following conditions: - si(c2) is in the same band and same TAG as c2; - si(c2) is a carrier of inter-band CA with c2 and si(c2) is indicated through the capability signalling ImpactedBands-SRS-CS-v17 to be affected by the SRS switch from c2 to c1; where 1≤i≤N-1. A UE can be configured with SRS resource(s) on a carrier c1 with slot formats comprised of DL and UL symbols and not configured for PUSCH/PUCCH transmission. For carrier c1, the UE is configured with higher layer parameter srs-SwitchFromServCellIndex and srs-SwitchFromCarrier the switching from carrier c2 which is configured for PUSCH/PUCCH transmission. During SRS transmission on carrier c1 (including any interruption due to uplink or downlink RF retuning time [11, TS 38.133] as defined by higher layer parameters switchingTimeUL and switchingTimeDL of SRS-SwitchingTimeNR), the UE temporarily suspends the uplink transmission on carriers in the set S(c2). < Unchanged parts are omitted >

With these combined feature, there might be more than 3 switches within a short time period. In this paper, we provide views and proposals.

# R17 Back-to-back switching with SRS switching

In Rel-16 UL Tx switching, UE is restricted to support one switch per one slot. However, the switching location could be anywhere inside the slot. For example, if the switch is triggered by SRS transmission, the switching location could be in the middle or even later part of the slot. Therefore, if there is an expected switch on the SRS transmission carrier, there would be two switches in 14 consecutive symbols even if these two switches still belong to two slots.

When we consider SRS carrier switching and if the UL Tx switching is triggered by SRS carrier switching which means there would be 4 switches (2 for SRS and 2 for UL Tx switch) in 14 consecutive symbols. From UE implementation perspective, we prefer avoiding this case as too many symbols are wasted as switch gap and UL throughput would be reduced accordinly.

Proposal: When SRS carrier switching is configured, a maximum of 3 switches (2 for SRS and 1 for UL Tx switching) are supported in 14 consecutive symbols.

UL

DL

DL

Tx Switch

CC1

UL

CC2

DL

DL

CC3

RF tuning

UL

4 switches within 14 consecutive symbols

SRS

RF tuning

SRS

Tx switch

Figure 1: illustrative figure on 4 switches in 14 consecutive symbols

# Conclusions

We shared the views on new switching options, intra band contiguous CA, Categorization of 1Tx and 2Tx, and etc. for inter-band CA, and SUL. Based on the discussion, we have the following proposals:

Proposal: When SRS carrier switching is configured a maximum of 3 switches (2 for SRS and 1 for UL Tx switching) are supported in 14 consecutive symbols.

# Reference

- RAN1 #110 meeting final meeting reports