3GPP TSG RAN WG1 #110bis-e			R1- 2210117

e-Meeting, October 10th – 19th, 2022

Agenda Item:	5

Source:	Ericsson

Title:	Discussion on LS on IUC with non-preferred resource set

Document for:	Discussion, Decision

# 1	Introduction

RAN2 discussed the inter-UE coordination scenario in which UE B receives IUC Scheme 1 non-preferred resource set from UE A, but UE B does not perform sensing in the resource pool associated with the non-preferred resource set, e.g., UE B performs mode 2 random resource selection, etc. Q1: Is the scenario described above a valid scenario or not? Q2: If the answer to Q1 is yes, does resource exclusion based on non-preferred resource set needs to be performed by UE B or not? Q3: If the answer to Q2 is yes, then, in RAN1’s view, which specification (PHY or MAC) should capture the resource exclusion behavior?

RAN1 received a LS from RAN WG2 with questions regarding the UE-B behaviour upon receiving an inter-UE coordination message in Scheme 1 containing non-preferred resources when performing random resource selection to be replied in RAN1#110. However, during the meeting there was no agreement on the reply to be sent to RAN2. The questions from RAN2 are the following:

In our view, this is an important aspect that needs to be addressed from RAN1 perspective in order to have a complete procedure in the specification text.

In order to have a complete procedure from specification point of view, RAN1 has to send a reply to RAN2 regarding the proposed scenario by RAN2.

# 2	Discussion

First of all, we discuss whether the scenario described in the LS from RAN2 is a reasonable assumption. In that regard, we use as argument the following agreement from RAN1#106-e which highlights the UE behaviour for both potential inter-UE coordination scenarios, i.e., a UE receiving a set of preferred resources or a set of non-preferred resources. The agreement is as follows (related text highlighted in yellow):

Agreement In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A: - For preferred resource set, the following two options are supported: - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met - FFS: Details of condition(s) - This option is supported when UE-B performs sensing/resource exclusion - FFS: Other details (if any) - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set - This option is supported at least when UE-B does not support sensing/resource exclusion - FFS: Whether the support is conditional or UE capability - FFS: Other details (if any) - FFS: Other option(s), and other details (if any) - For non-preferred resource set, - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: Details including - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any) - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set - FFS: Other option(s), and other details (if any)

A scenario where a UE performing random resource selection receives an inter-UE coordination message in Scheme 1 containing non-preferred resources is a valid assumption.

Regarding the UE behaviour when receiving a non-preferred resource set, the RAN1 agreements indicate that if the UE has its own sensing results, i.e., the UE is capable of performing sensing, it performs resource exclusion based on a combination of the inter-UE coordination containing the non-preferred set and its own sensing information.

When a UE receives a set of non-preferred resources, the UE uses its own sensing information (if available) in combination with the information from the set of non-preferred resources.

Therefore, the next step is to define the cases where a UE is performing random resource selection and whether the sensing information/resource exclusion is available. Therefore, we define two cases:

- The UE performs random resource selection, and it is not capable of performing sensing/resource exclusion. In this case, the UE performs random resource selection and discards the inter-UE coordination information from UE-A containing the non-preferred resources.

- The UE performs random resource selection, and it is capable of performing sensing/resource exclusion. In this case, the UE performs resource (re-)selection combining its own sensing information and the information contained in the inter-UE coordination message.

For the case when a UE capable of performing sensing/resource exclusion performs random resource selection, upon receiving an inter-UE coordination containing non-preferred resource set, the UE triggers resource (re-)selection based on its own sensing information and the information contained in the inter-UE coordination message.

The last aspect to be discussed is in which layer, i.e., either PHY or MAC layer, the operation of (re-)selecting resources based on the sensing information and the information contained in the inter-UE coordination message is captured. In our view, since the procedures related to the resource selection based on the inter-UE coordination message containing non-preferred resources are already captured in the PHY layer, i.e., in TS 38.214, it is reasonable to include this behaviour also in the PHY specification to have a consistent specification text.

The re-selection of resources by a UE based on its own sensing and the information contained in the inter-UE coordination message for a UE capable of performing sensing/resource exclusion performing random resource selection is included in the PHY specification to have a consistent specification.

# 3	Conclusion

In the previous sections we made the following observations:

Observation 1	In order to have a complete procedure from specification point of view, RAN1 has to send a reply to RAN2 regarding the proposed scenario by RAN2.

Observation 2	A scenario where a UE performing random resource selection receives an inter-UE coordination message in Scheme 1 containing non-preferred resources is a valid assumption.

Observation 3	When a UE receives a set of non-preferred resources, the UE uses its own sensing information (if available) in combination with the information from the set of non-preferred resources.

Based on the discussion in the previous sections we propose the following:

Proposal 1	For the case when a UE capable of performing sensing/resource exclusion performs random resource selection, upon receiving an inter-UE coordination containing non-preferred resource set, the UE triggers resource (re-)selection based on its own sensing information and the information contained in the inter-UE coordination message.

Proposal 2	The re-selection of resources by a UE based on its own sensing and the information contained in the inter-UE coordination message for a UE capable of performing sensing/resource exclusion performing random resource selection is included in the PHY specification to have a consistent specification.