3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210245

e-Meeting, 10th – 19th October 2022

Agenda Item:	8.6

Title:	FL summary #1 on Rel-17 RedCap maintenance

Source:	Moderator (Ericsson)

Document for:	Discussion, Decision

# Introduction

This feature lead (FL) summary (FLS) concerns the Rel-17 work item (WI) for support of reduced capability (RedCap) NR devices [1, 2]. Earlier RAN1 agreements for this WI are summarized in [3], the final FLS from the previous RAN1 meeting can be found in [4], and the 38.213 CR that was agreed in the previous RAN1 meeting can be found in [5].

This document summarizes contributions [6] – [21] submitted to agenda item 8.6 as well as RedCap-related aspects in contribution [22] submitted to another agenda item.

The issues in this document that are expected to be in the focus of the first round of the email discussion are tagged FL1.

FL1 Question 0-1a: Please consider entering contact info below for the points of contact for this email discussion.

| Company | Point(s) of contact | Email address(es) |
| --- | --- | --- |

# Issue #1: QCL properties for NCD-SSB

RAN1#110 agreed the 38.213 CR in [5] which contains an incomplete sentence. The incomplete sentence is a remainder from a longer sentence in Proposal 2.1-1d in the FLS in [4]. The longer sentence in the proposal looked like this:

If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index.

An online (GTW) session during RAN1#110 noted that the above sentence may be superfluous since there already is corresponding text in 38.331:

nonCellDefiningSSB If configured, the RedCap UE operating in this BWP uses this SSB for the purposes for which it would otherwise have used the cell-defining SSB of the serving cell (e.g. obtaining sync, measurements, RLM). Furthermore, other parts of the BWP configuration that refer to an SSB (e.g. the "SSB" configured in the QCL-Info IE; the "ssb-Index" configured in the RadioLinkMonitoringRS; CFRA-SSB-Resource; PRACH-ResourceDedicatedBFR) refer implicitily to this NCD-SSB. The NCD-SSB has the same values for the properties (e.g., ssb-PositionsInBurst, PCI, ssb-periodicity, ssb-PBCH-BlockPower) of the corresponding CD-SSB apart from the values of the properties configured in the NonCellDefiningSSB-r17 IE.

To avoid double specification (in 38.213 and 38.331), it was suggested in the online session that the mentioned longer sentence should not be agreed as part of the 38.213 CR and to potentially come back at later RAN1 and RAN2 meetings with CRs toward both 38.213 and 38.331 to move the QCL related specification text from 38.331 to 38.213.

However, in the end, only part of the mentioned longer sentence was included in the RAN1#110 agreement and in the corresponding final 38.213 CR in [5], which means that there is now an incomplete sentence in 38.213:

If the active DL BWP includes the SS/PBCH blocks provided by NonCellDefiningSSB, these SS/PBCH blocks and the SS/PBCH blocks that the UE used to obtain SIB1 have the same quasi-colocation properties, if they have the same index.

Now, contributions [7, 9, 15, 16, 20] propose to include the missing part of the sentence, whereas contribution [6] proposes to remove the remainder of the sentence and rely on the 38.331 specification text.

FL1 Question 1-1a: Should the QCL-related sentence be included in 38.213? If yes, please comment on whether something needs to be done to avoid double specification in 38.213 and 38.331.

| Company | Y/N | Comments |
| --- | --- | --- |

# Issue #2: Collision between DL transmission and NCD-SSB

RAN1#110 agreed the 38.213 CR in [5] which clarifies the handling of several NCD-SSB collision cases:

- Collision between PUCCH repetition and NCD-SSB in TDD

- Collision between other UL transmission and NCD-SSB in TDD

- Collision between PDCCH and NCD-SSB

Now, new contributions propose to make a similar clarification in 38.213 clause 17.1 for the handling of collision between other DL transmission and NCD-SSB:

- Contribution [17] proposes to add a new paragraph for DL inspired by the existing paragraph for TDD UL:

For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB in unpaired spectrum, collision handling between uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated. For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB, the UE assumptions on the SS/PBCH blocks for reception of a downlink signal or channel are same as described for SS/PBCH blocks for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated.

- Contribution [6] instead proposes to modify the existing paragraph to make it cover DL transmission:

For a RedCap UE indicated presence of SS/PBCH blocks within an active DL BWP by NonCellDefiningSSB in unpaired spectrum, collision handling between downlink or uplink transmissions and the SS/PBCH blocks are same as described for a UE indicated presence of SS/PBCH blocks by ssb-PositionsInBurst in SIB1 or in ServingCellConfigCommon described in all other clauses, unless otherwise stated.

FL1 Question 2-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #3: Collision between UL transmission and NCD-SSB

As mentioned above, RAN1#110 agreed the 38.213 CR in [5] which clarifies these NCD-SSB collision cases:

- Collision between PUCCH repetition and NCD-SSB in TDD

- Collision between other UL transmission and NCD-SSB in TDD

- Collision between PDCCH and NCD-SSB

Now, new contributions propose to make similar clarifications in 38.213 for the handling of collision between other UL transmission and NCD-SSB:

- Contribution [12] proposes to clarify in 38.213 clauses 8.1 and 8.1A that the specification text for RO validation concerns not only CD-SSB but also NCD-SSB.

  - Contribution [11 (section 2.3)] provides some additional discussion on the above draft CR.

- Contribution [21 (section 3)] makes the following proposals related to RO validation in TDD:

  - For operation on a single carrier in unpaired spectrum, a RedCap UE does not expect to transmit PRACH, or PUSCH, or PUCCH, or SRS in a set of symbols of a slot indicated presence of SS/PBCH blocks within the active DL BWP by ssb-PositionsInBurst in SIB1 or by ServingCellConfigCommon, or by NonCellDefiningSSB. The UE does not expect the set of symbols of the slot to be indicated as uplink by tdd-UL-DL-ConfigurationCommon, or tdd-UL-DL-ConfigurationDedicated, when provided to the UE.

  - For operation on a single carrier in unpaired spectrum, if a RedCap UE operates on an active DL BWP configured with NCD-SSB and on an active UL BWP configured with PRACH resources:

    - the valid PRACH occasion(s) will not overlap with SSB symbols of NCD-SSB and CD-SSB

    - the UE does not expect to be configured with a SS/PBCH block by NonCellDefiningSSB, which ends within Ngap symbols of starting symbol of a valid PRACH occasion configured for RedCap UE or succeeds a valid PRACH occasion, configured for RedCap UE, in a slot

    - the valid PRACH occasion(s) and the definition of Ngap for RedCap UE are determined by the rules in Clause 8.1 of TS 38.213

Proposals related to PUSCH repetition in HD-FDD are treated separately under Issues #4 and #5.

FL1 Question 3-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #4: PUSCH repetition type A in HD-FDD

RAN1#110 discussed PUSCH repetition in HD-FDD, which is captured in section 3 in the FLS [4].

Now, new contributions propose to make corrections for PUSCH repetition type A (and TBoMS) in HD-FDD:

- Contribution [19] provides a draft CR for 38.214 clauses 6.1.2.1, 6.1.2.3.1 and 6.1.2.3.3.

  - Contribution [18] provides some additional discussion on the above draft CR.

- The last paragraph in contribution [22] proposes a similar correction for 38.214 clause 6.1.2.3.3.

Proposals related to PUSCH repetition type B in HD-FDD are treated under Issue #5.

FL1 Question 4-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #5: PUSCH repetition type B in HD-FDD

As mentioned above, RAN1#110 discussed PUSCH repetition in HD-FDD, which is captured in section 3 in the FLS [4].

Now, new contributions propose to make corrections for PUSCH repetition type B in HD-FDD:

- Contribution [8] provides a draft CR for 38.214 clause 6.1.2.1.

- Contribution [13] proposes additional potential corrections for 38.214 clause 6.1.2.1.

  - Contribution [11 (section 2.3)] provides some additional discussion on the above draft CR.

Proposals related to PUSCH repetition type A in HD-FDD are treated under Issue #4.

FL1 Question 5-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #6: PUSCH TDRA misalignment

Contributions [11 (section 2.2), 14] propose to clarify in 38.214 clause 6.1.2.1.1 which common search space is used when the UE is addressed with TC-RNTI. Note that similar Rel-15 contributions have been submitted in [23, 24].

FL1 Question 6-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #7: Maximum UL BWP bandwidth

Contribution [6] proposes to clarify in 38.213 clause 17.1 that the separate initial UL BWP for RedCap UEs (if configured) is smaller than or equal to the maximum UL bandwidth that the UE supports.

FL1 Question 7-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #8: Msg1/MsgA retransmission timeline

Contribution [21 (section 2)] proposes to add text about the Msg1/MsgA retransmission timeline for the case when a RedCap UE performs random access on an active DL BWP with SSB in 38.213 clause 17.1, corresponding to the text in clauses 8.2 and 8.2A for non-RedCap UEs.

FL1 Question 8-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #9: SDT operation

Contribution [11 (section 2.1)] proposes that it should be up to the UE implementation whether and how a UE monitors SI change indication during an SDT procedure in a separate initial DL BWP not containing CD-SSB.

FL1 Question 9-1a: Companies are invited to provide comments and suggested priority (Low/Medium/High).

| Company | Priority | Comments |
| --- | --- | --- |

# Issue #10: Specification alignment

Contribution [10] proposes to correct pucch-ResourceCommon-RedCap to pucch-ResourceCommonRedCap in 38.213 clause 17.1. This and any other RRC parameter name correction can be done as part of the next (ordinary or alignment) 38.213 CR.

FL1 Question 10-1a: Companies are invited to provide comments on any potential needs for RRC parameter name correction or specification alignment.

| Company | Comments |
| --- | --- |

# References

| [1] | RP-220966 | Revised WID on support of reduced capability NR devices | Ericsson |
| --- | --- | --- | --- |
| [2] | R1-221163 | Summary of WI on support of reduced capability (RedCap) NR devices | Ericsson |
| [3] | R1-2208274 | RAN1 agreements for Rel-17 NR RedCap | Rapporteur (Ericsson) |
| [4] | R1-2207729 | FL summary #3 for Rel-17 RedCap maintenance | Moderator (Ericsson) |
| [5] | R1-2208247 | 38.213 CR0360 (Rel-17, F) Corrections and clarifications of RedCap UE procedures | Moderator (Ericsson) |
| [6] | R1-2208360 | Corrections and clarifications of RedCap UE procedures | Ericsson |
| [7] | R1-2208537 | Corrections on Support of Reduced Capability NR Devices | Spreadtrum Communications |
| [8] | R1-2208605 | Correction on invalid symbol determination for PUSCH repetition type B for HD-FDD | Vivo, Sharp, Intel, Nokia, Nokia Shanghai Bell |
| [9] | R1-2208941 | Correction on QCL relationship between NCD-SSB and CD-SSB | CATT |
| [10] | R1-2209164 | Alignment between RAN1 and RAN2 specifications | NEC |
| [11] | R1-2209186 | Discussion on RedCap remaining issues | ZTE, Sanechips |
| [12] | R1-2209187 | Correction on NCD-SSB related spec for RedCap in TS38213 | ZTE, Sanechips |
| [13] | R1-2209188 | Correction on NCD-SSB related spec for RedCap in TS38214 | ZTE, Sanechips |
| [14] | R1-2209189 | Correction on TDRA misalignment of PUSCH for RedCap | ZTE, Sanechips |
| [15] | R1-2209222 | Draft CR on NCD-SSB in an active BWP | Lenovo |
| [16] | R1-2209429 | Editorial corrections for RedCap in TS 38.213 | Nokia, Nokia Shanghai Bell |
| [17] | R1-2209431 | Corrections on UE assumptions when configured with NCD-SSB | Nokia, Nokia Shanghai Bell |
| [18] | R1-2209778 | Discussion on available slot determination for PUSCH repetition type A and TBoMS for HD-UE | Sharp |
| [19] | R1-2209779 | Corrections on available slot determination for PUSCH repetition type A and TBoMS for HD-UE | Sharp, Vivo, Nokia, Nokia Shanghai Bell, Intel |
| [20] | R1-2209850 | Correction on NCD-SSB for RedCap UE | Huawei, HiSilicon |
| [21] | R1-2209947 | Remaining issues on procedures of RedCap UE | Qualcomm Incorporated |
| [22] | R1-2209468 | Correction on cancellation of PUSCH repetitions and TBoMS | ZTE |
| [23] | R1-2209184 | Discussion on PUSCH TDRA misalignment issue | ZTE, Sanechips |
| [24] | R1-2209185 | Correction on TDRA misalignment of PUSCH | ZTE, Sanechips |