3GPP TSG-RAN WG1 #110bis-e	  R1-2210371

e-Meeting, October 10 – 19, 2022

Agenda Item:	8.12

Source:	Moderator (Huawei)

Title:	FLS on the issues for NR MBS maintenance

Document for:	Discussion and Decision

# Introduction

[110bis-e-R17-MBS-01] Email discussion to determine maintenance issues to be handled in RAN1#110bis-e by October 12 – Jinhuan (Huawei)

- Additional email discussions will be set up once the maintenance issues for RAN1#110bis-e are determined

- Including discussion on R1-2208581 from agenda item 5

This summary provides the discussion for the remaining issues to be discussed in this e-meeting. FL initial assessment is provided according to:

•	High priority (H): high-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes.

•	Non-essential (N): Not essential issues and the discussion may not be pursued.

•	Editorial (E): editorial issues that will be endorsed for alignment CR.

# Issues for discussion

The HARQ-ACK related issues and the scheduling related issues are summarized in Table 1 and Table 2, respectively, with FL notes and FL initial assessment provided. Companies can provide comments/views for helping sort out the issues to be discussed in this e-meeting.

Note: the issue for discussion on R1-2208581 from agenda item 5 is summarized in Table 2.

Preparation

Table 1: Summary of HARQ-ACK related issues

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 1-1 | PRI for NACK-only mode2 FL Note: sources have different views on whether PRI is interpreted. Need to first clarify whether moreThanOneNackOnlyMode is only applicable to the case when more than one TB is scheduled. | vivo-CR-x08618, Nokia-TP-x08701, CMCC-Dis-x09310, ZTE-CR-x09475, Apple-Dis-x09566, Huawei-CR-x08468, Huawei-Dis-x09822 Samsung-Dis-x09708, Ericsson-Dis-x10173 | H | [QC] We are not clear how to understand ‘more than one TB is scheduled’. Is it from gNB perspective or UE perspective? There is a case that gNB schedules multiple TBs but UE only receives one due to missing DCI(s). Also, the issue 1-1 is related with 1-7, where if case 2/3 are not supported, the decision on PRI would be simpler. Ericsson: OK to discuss and we’re fine to prioritize the case2/3 discussion if that simplifies the process. [MTK]: agree with FL’s comments, and the first clarification is needed. [LGE]: agree with FL’s assessment. FL reply: @QC, it is from UE perspective, but may not be aligned with NW perspective in fact. @Ericsson as well, I doubt discussing case2/3 would be a helpful approach since it is more controversial. They don’t have to be bound together. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue. |
| 1-2 | Codebook type for NACK-only FL Note: Discuss whether type1 CB can be configured if UE is configured with NACK-only. | Samsung -Dis-x09708, Qualcomm-CR-x09955, Huawei-CR-x08467, Huawei-Dis-x09822, Ericsson-Dis-x10173 | H | Ericsson: OK to discuss Lenovo: OK to prioritize this issue. [MTK]: agree with FL’s assessment. [LGE]: agree with FL’s assessment. |
| 1-3 | HARQ-ACK feedback for 4_1 when configured “dci-enabler” FL Note: clarify UE behavior whether generate HARQ-ACK for DCI 4_1 when UE is configured with “dci-enabler”. | vivo-CR-x08617, CATT-CR-x08928, Lenovo-CR-x10156 | H | [CATT]: Agree with FL’s assessment. The field of ‘enabling/disabling HARQ-ACK feedback indication’ is not included in DCI format 4_1. In the current spec, it’s not clear about the UE behavior whether generate HARQ-ACK for PDSCH scheduled by DCI format 4_1 when UE is configured with ‘dci-enabler’. Ericsson: OK to discuss Lenovo: OK to prioritize this issue. [LGE]: agree with FL’s assessment. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue. |
| 1-4 | Timeline for NACK-only FL Note: Tdocs from UE vendors point to a serious implementation issue. | ZTE-Dis-x09470, MediaTek-CR-x09526, Samsung-Dis-x09708, Qualcomm-CR-x09959 Huawei-CR-x08466, Huawei-CR-x09832, | H | Ericsson: OK to discuss MTK: the discussion is needed since it will impact UE’s implementation. |
| 1-5 | PUCCH resources for NACK-only multicast SPS FL Note: FFS from previous meeting and spec change is needed. | Nokia-TP-x08701, ZTE-Dis-x09470, Samsung-TP-x09708, Huawei-Dis-x09822 | H | [vivo]: Agree with FL’s assessment. [ZTE]: We agree with FL initial assessment. Ericsson: OK to discuss [LGE]: agree with FL’s assessment. |
| 1-6 | Type1 CB and “dci-enabler” not configured simultaneously FL Note: This conclusion should be the common understanding but whether spec change is needed can be discussed to collect views. | Qualcomm-CR-x09954, Lenovo-CR-x10158 | H | Ericsson: OK to discuss Lenovo: OK to prioritize this issue. [LGE]: agree with FL’s assessment. |
| 1-7 | NACK-only mode2 for case2 and case3 FL Note: discussed for several meetings. Introducing any RRC parameters should be refrained. FL doubt to reach solution with consensus. The discussion can focus on whether spec clarification change is also needed even if case2 or case 3 is not supported in combination with NACK-only mode2. | Nokia-TP-x08701, NEC-Dis-x09137, CMCC-Dis-x09310, ZTE-Dis-x09470, MediaTek-Dis-x09527, Apple-Dis-x09566, Samsung-TP-x09708, DOCOMO-CR-x09884, Lenovo-Dis-x10159, Ericsson-Dis-x10173 | H | [Nokia] Agree that this [H] priority and would like the FL/group to focus on the options that require minimal spec. changes. [vivo]: This issue has been discussed for several times, but no consensus was achieved. It is suggested not to be pursued. [QC] If case 2 or 3 are not supported, we think the current spec of “A UE that is indicated the second HARQ-ACK reporting mode for only one G-RNTI can be indicated by moreThanOneNackOnlyMode …” is sufficient. [Ericsson]: As explained in our contribution, we strongly prefer to support Case 2/3 with cDAI*. However, since remaining time is very limited and we do not see any emerging consensus on this, as a minimum, we think the current agreement to support Case 1 can be extended to support Case 2 with cDAI. We do not think this would require any changes to RRC signaling. Case 1 would simply be a special case of Case 2. This should not be controversial and would still allow for additional useful functionality compared to unnecessarily restricting the standard to just Case 1. Proposal: For multicast, for addressing how to count and order the HARQ-ACK bits for NACK-only for Alt4, Case 2 with Opt2-1 and Opt 2-1-1 are supported: - Case 2: for the case of all UEs configured with the same set of G-RNTIs - Opt2-1: support Alt4 for this case - Opt2-1-1: based on the sum of C-DAI included in the last scheduling DCI from each G-RNTI for counting the total number of HARQ-ACK bits. The ordering of HARQ-ACK bits is per C-DAI from each G-RNTI and in the ascending order of G-RNTI values. [Lenovo]: we are fine not to support case 2 and case 3. [LGE]: we are fine not to support case 2 and case 3. FL reply: as anticipated, the views are divergent. As mentioned, the discussion can focus on whether spec clarification change is also needed even if case2 or case 3 is not supported in combination with NACK-only mode2. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue and the discussion focuses on whether spec clarification change is also needed even if case2 or case 3 is not supported in combination with NACK-only mode2 |
| 1-8 | Whether DAI should count the PDCCH with “dci-enabler” indicating value 0— if not counted, the spec for Type2 CB needs update for the DAI description. | Langbo-CR-x08995, Lenovo-CR-x10157 | H | Ericsson: OK to discuss Lenovo: OK to prioritize this issue. Langbo：Agree with FL initial assessment [LGE]: agree with FL’s assessment. |
| 1-9 | Type2 CB for multicast SPS—38.213 states UE separately applies the procedures in this clause per G-RNTI or per G-CS-RNTI, but the clause also includes SPS PDSCHs. How to generate Type2 CB for multicast SPS PDSCH is unclear FL Note: FL agrees that it may cause confusion. The discussion in this meeting can focus on whether change is needed. | vivo-CR-x08619 | H | Ericsson: OK to discuss |
| 1-10 | the current description only allows to configure one HARQ-ACK codebook for multicast-- pdsch-HARQ-ACK-Codebook is replaced by the relevant entry in pdsch-HARQ-ACK-CodebookListMulticast-r17 FL Note: aligning with TS38331 should be easily agreeable for this meeting | CMCC-CR-x09312 | H | Ericsson: OK to discuss |
| 1-11 | Change “UE is provided moreThanOneNackOnlyMode” to “UE is not provided moreThanOneNackOnlyMode” FL Note: aligning with TS38331 should be easily agreeable for this meeting | vivo-CR-x08618, Huawei-CR-x08468 | H | [QC] both CRs seem include the modifications more than changing ‘UE is provided moreThanOneNackOnlyMode” to “UE is not provided’. We assume the intention of 1-11 is to simply align the 38331 of moreThanOneNackOnlyMode as suggested by FL. Is it correct understanding? FL reply: @QC, the answer is yes. [Final FL recommendation]: seems no objection to discuss this issue. |
| 1-12 | Multiplexing one unicast SPS and DG multicast-- gNB wouldn’t be able to schedule multicast PDSCHs to transmit HARQ-ACK in the same slot with unicast SPS HARQ-ACK if UE does not support more than one SPS configuration FL Note: FL agrees it is better to be discussed but not sure what solution can be accepted in this meeting. The discussion on focus on solutions and then the CR. | vivo-CR-x08887 | H | Ericsson: OK to discuss |
| 1-13 | PTP retrans for NACK-only when applicable—when NACK-only is converted into ACK/NACK, PTP retrans can be applied. NACK-only taking PTM retrans is too restrictive. | Huawei-CR-x10207 | H | [Nokia] We consider that this discussion can be deprioritized. At this stage, we are unclear of a strong use case for this form of retransmission given the primary motivation for using NACK-only is to have a group-common PUCCH. [vivo]: This issue has been discussed in last meeting. It is suggested not to be pursued. [QC] The issue is not essential but an optimization per our understanding. MTK: Not a critical issue and suggest to deprioritize the discussion [LGE]: The issue is not essential and suggest to deprioritize the discussion FL reply: as anticipated, UE vendor will consider it as optimization but the concern is from network perspective since mandating PTM for NACK-only could be potential risk especially PTP retransmission is applicable. The change is minor but leaves much more flexibility for network scheduling. [Final FL recommendation]: based on the comments above, I would suggest postponing it to the next meeting. |
| 1-14 | Delete the redundant descriptions of “if pdsch-HARQ-ACK-Codebook-Multicast = semiStatic is configured” to 38212. FL Note: avoid unnecessary checking whether other meaning is implied by the redundant description and easily agreeable for this meeting | Lenovo-CR-x10155 | Enhance clarity easily agreeable for progress |  |
| 1-15 | Change “pdsch-HARQ-ACK-Codebook-Multicast” to “pdsch-HARQ-ACK-Codebook”. FL Note: editorial but easily agreeable for this meeting | CMCC-CR-x09312 | E but easily agreeable for progress |  |
| 1-16 | pdsch-HARQ-ACK-retx for multicast- pdsch-HARQ-ACK-retx can include the HARQ-ACK feedback for multicast FL Note: it may be better to be clarified. | CATT-CR-x08925, | Discuss at RAN1#110bis-e only to clarify the issue and whether it is essential (to control the workload) | [CATT] We think this issue is high items and requires essential clarify. It should be clarified whether the parameter pdsch-HARQ-ACK-retx can be applied for both of multicast and unicast. There is no additional spce impact. Note that the HARQ CB of multicast may be multiplexed with HARQ CB of unicast, if pdsch-HARQ-ACK-retx cannot be applied for both of multicast and unicast, and then RAN1 needs specify how to remove HARQ-ACK CB of multicast from the multiplexed CB. [Nokia] In our view, there is no need to specifically work on procedures introduced in Rel-17. This would open doors to many non-essential optimizations. [vivo]: This issue is related with the interaction of different R17 features which is better to have a common principle. Ericsson: OK to discuss MTK: at this stage, no further enhancement for MBS is preferred. FL reply: in general, interaction of different R17 features should be considered as lower priority compared to MBS-only maintenance issues, at least the discussion can focus on whether any spec change is needed even though not considering interaction of different R17 features. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue. |
| 1-17 | spsHARQdeferral for multicast SPS-- whether RRC parameter IE spsHARQdeferral can be used for multicast SPS configuration FL Note: it might be true as claimed that additional spec impact is none or very trivial to have spsHARQdeferral configurable for multicast SPS. | CATT-CR-x08926, | Discuss at RAN1#110bis-e only to clarify the issue and whether it is essential, no discussion on solutions (to control the workload) | [CATT] We think this issue is high items and requires essential clarify. The IE SPS-Config is used to configure SPS transmission of multicast, and the IE spsHARQdeferral has been included in the IE SPS-Config. It should be clarified whether RRC parameter IE spsHARQdeferral can be used for multicast SPS configuration. [Nokia] In our view, there is no need to specifically work on procedures introduced in Rel-17. This would open doors to many non-essential optimizations. [vivo]: This issue is related with the interaction of different R17 features which is better to have a common principle. MTK: at this stage, no further enhancement for MBS is preferred. [LGE] OK to discuss. FL reply: in general, interaction of different R17 features should be considered as lower priority compared to MBS-only maintenance issues, at least the discussion can focus on whether any spec change is needed even though not considering interaction of different R17 features. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue. |
| 1-18 | Type3 for NACK-only mode-- For generating a Type-3 HARQ-ACK codebook with 〖NDI〗_HARQ=1, the UE considers the HARQ-ACK information corresponding to a PDSCH reception has been reported if a PUCCH was not transmitted due to all values of HARQ-ACK information including the one for the PDSCH reception being ‘ACK’ according to the second HARQ-ACK reporting mode. FL Note: the change may be needed for specification completeness. | Langbo-CR-x08996 | Discuss at RAN1#110bis-e only to clarify the issue and whether it is essential (to control the workload) | Ericsson: OK to discuss Langbo： As noted by FL, the specification is incomplete if this change is not included. We think this clarification is essential for the generation of Type-3 HARQ-ACK codebook when NAKC-only reporting mode configured. [LGE] OK to discuss. [Final FL recommendation]: seems no objection to discuss this issue.. |
| 1-19 | - NACK-only mode2 multiplexed with other UCI/PUSCH is supported FL Note: supported but the current spec suffices? - SR and ack/nack based multiplexing with 1bit for unicast and 1 bit for multicast. FL Note: how to change spec? - HARQ-ACK multiplexing for u-cast and m-cast is per priority and then applying the intra-UE multiplexing FL Note: how to change spec? | LGE-Dis-x09449 | Discuss at RAN1#110bis-e only to clarify the issue and whether it is essential (to control the workload) | [vivo]: It is better to clarify the order of UE procedures. In our understanding, UE performs multiplexing HARQ-ACK of unicast and multicast, then performs other intra-UE multiplexing. No spec change is needed. Ericsson: seem no change would be required. In the existing rules, NACK-only mode2 will be transformed to ACK-NACK harq before being multiplexed on PUSCH. [LGE] OK to discuss. We are not sure whether we have common understanding and everything is clear from the current spec. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue.. |
| 1-20 | UE behavior for disabled HARQ for NTN multicast needs to clarified. FL Note: for UE supporting both NTN and multicast. | Qualcomm-CR-x09960 | Discuss at RAN1#110bis-e only to clarify the issue and whether it is essential (to control the workload) | [vivo]: This issue is related with the interaction of different R17 features which is better to have common principle. [QC] we raised this issue in last RAN1 meeting but no time to discuss it. The issue 1-20 should be [H] to clarify the UE/gNB behavior: - whether/which case multicast PDSCH can be scheduled with a HARQ process with disabled feedback or not - If multicast PDSCH is allowed to be scheduled with a HARQ process with disabled feedback, whether to let UE follow the feedback codebook generation based on NTN scheme or multicast scheme. Ericsson: OK to discuss, but we don’t think the harq disabled mode for NTN should affect Multicast. In our understanding, the Harq process is used by multicast in a separate sub-codebook from (unicast) NTN so it should not be a problem. MTK: The issue is related to two new Rel-17 features, and we suggest not cross the discussion at this late stage. FL reply: in general, interaction of different R17 features should be considered as lower priority compared to MBS-only maintenance issues, at least the discussion can focus on whether any spec change is needed even though not considering interaction of different R17 features. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue.. |
| 1-21 | UE will not transmit NACK-only PUCCH when the HARQ-ACK information is ‘ACK’. This is missing for NACK-only mode1?? FL Note: Not sure whether the issue is understood. If it is NACK-only mode1, PUCCH is indicated by PRI, determining overlapping with PUSCH is referring to the legacy procedure. No change is needed. | ZTE-CR-x09476 | Not an essential correction Not pursued | [vivo]: The issue can be discussed together with issue 1-1, since if moreThanOneNackOnlyMode is only applicable to the case when more than one TB is scheduled, the issue is valid. [ZTE]: We think this issue should be discussed. We would like to clarify this issue. For NACK-only feedback, when the NACK-only PUCCH resource overlaps with the other PUCCH or PUSCH, the UE should assume the NACK-only PUCCH is transmitted so as to the UE can perform multiplexing by following the current mechanism even though the UE decodes all the PDSCH correctly. This has been captured in the current spec only for the case of NACK-only mode 2. However, this is still missing for NACK-only mode 1. It should be fixed. We guess it may be easily agreed. Ericsson: agree with the FL assessment. FL reply: It seems ZTE and vivo are talking about different things though honestly I don’t get why ‘moreThanOneNackOnlyMode is only applicable to the case when more than one TB’ matters here. Given two companies view the CR is needed somehow and one company agrees with FL, to be safer, let’s have this issue to be discussed. [Final FL recommendation]: given the situation, I suggest discussing this issue in this meeting. |
| 1-22 | Multiplexing NACK-only and SR FL Note: open issue has been discussed for several times. As clarified, if no consensus for the solution, it means not support the multiplexing and it is up to UE so UE behavior is not needed. FL suggest dropping the discussion | Nokia-TP-x08701, NEC-Dis-x09137, ZTE-Dis-x09470, MediaTek-Dis-x09527, DOCOMO-CR-x09885 | Not an essential correction Not pursued | [Nokia] Can we have a chair’s note to conclude that this is left to UE implementation on which one to drop – i.e., SR or NACK-only? [ZTE]: We think this issue should be discussed because companies may have different understandings for this case if it is not resolved. Ericsson: agree with the FL assessment. MTK: Nokia’s suggestion is fine for us. FL reply: @Nokia, I remembered we earlier clarified verbally that UE behavior is not defined in spec would mean it is up to UE implementation, so no CR is needed. [Final FL recommendation]: given the situation, not pursued for this issue |
| 1-23 | Fix the size of DAI field in DCI format 4_2 to two. -- If different codebook types are configured among UEs in a group, the understanding of the DAI field size will not be aligned. FL Note: UE just follows the configuration and NW will ensure the proper configuration. No change is needed. | DOCOMO-CR-x09882 | Not an essential correction Not pursued | Ericsson: agree with the FL assessment. |
| 1-24 | Specify that ACK/NACK based feedback is provided for the first GC-PDSCH after activation, and NACK-only based feedback is provided for the other SPS GC-PDSCHs. FL Note: FL views no change is needed. The current spec is saying the same thing. | DOCOMO-CR-x09883 | Not an essential correction Not pursued | [vivo]: The current spec only says “The second HARQ-ACK reporting mode is not applicable for the first SPS PDSCH reception after activation of SPS PDSCH receptions for a SPS configuration, or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception”, it does not say anything on how to feedback for the first SPS PDSCH reception after activation of SPS PDSCH receptions or for DCI formats having associated HARQ-ACK information without scheduling a PDSCH reception. It is better to treat the CR as editorial CR. Ericsson: agree with the FL assessment. FL reply:@vivo, to me it is clear, that ‘the second mode is not applicable’ means only the first mode is applicable. [Final FL recommendation]: given the situation, not pursued for this issue |
|  |  |  |  | [ZTE] Seems some issues from our contribution x09470 are not summarized, which are shown below. Hope FL could also collect companies’ views on these issues. Thanks. - The first one is just a clarification issue on PUCCH resource configuration. We are afraid people may have different understanding since the two agreements seem not consistent with each other and the current spec is also not clear. At first, we agree that the gNB can configure at most 32 PUCCH resources for NACK-only. For NACK-only mode 2, 15 PUCCH resources from the PUCCH resource set are used. It is not clear whether they (32 PUCCH resources and 15 PUCCH resources) are in the same PUCCH resource set. If they are in different PUCCH resource sets, how to configure them? FL reply: it is entirely up to network for the configuration, for example, even though we agree 32, network can configure at most two for example for NACK-only. Note there is no CR submitted if I did not miss. - The second one is the HARQ-ACK multiplexing in the PUSCH in the absence of PUCCH resource. We think this issue should be clarified to see whether companies are on the same page. FL reply: I guess you are saying the issue in ZTE-CR-x09476, it is in issue 1-21, if not note there is no CR submitted if I did not miss. [ZTE2] Thanks FL for the reply. Actually, it is not the issue 1-21. This issue was raised by us and discussed in previous RAN1 meeting. There is no conclusion on this issue since it is related to another CR discussion and no agreement on the CR at that time. Now, the CR discussion has been closed with some progress. We think the agreement reached in the CR discussion can be applied to the MBS but not sure whether this is the common understanding in RAN1. Since it is just a clarification question, we think no CR is needed. We guess it may not take much time and a conclusion may be sufficient. Hope this issue can be discussed in this meeting. Thank you. FL reply2: Thanks for the discussion. However, the proposal in the Tdoc does not seem straightforward, I suggest this issue could be postponed to the next meeting. - The third one is the UE behavior in the case of the NACK-only PUCCH overlapping with other PUCCH or PUSCH with different L1 priorities. The DCI format 4_2 can indicate the L1 priority. There is no existing mechanism to follow since NACK-only feedback is not supported for unicast. If it is not discussed, the UE behavior is not clear. FL reply: are you saying issues in section 3.7? if yes, clearly it is optimization to me and note there is no CR submitted for it if I did not miss. [Final FL recommendation]: based on the discussion, FL suggests all these issues are not discussed in this meeting. |

Table 2: Summary of scheduling related issues

| Issue# | Issue | References | FL initial assessment | Company inputs (if any) |
| --- | --- | --- | --- | --- |
| 2-1 | CR on the MBS reception type combinations in TS 38.202/38.213/38.214 FL Note: correct the PDSCH reception restriction/type combinations as well as specs alignment between TS 38.202/38.213/38.214 | CMCC[R1-2209311] ZTE[R1-2209473] MediaTek[R1-2209524] Huawei[R1-2210208][R1-2210209][R1-2210210] | H | [ZTE]: Agree with FL initial assessment. Ericsson: OK to discuss MTK: agree with FL’s assessment |
| 2-2 | CR on the maximum data rate for multiplexing MBS and unicast FL Note: Continue discussion of the clarification of the max data rate/LBRM for unicast and MBS multiplexing | Huawei[R1-2209833] Qualcomm[R1-2209956][R1-2209957][R1-2209958] | H | [ZTE]: Do not agree. The issue has been discussed for several meetings without big progress and it seems to be a non-essential issue. Ericsson: OK to discuss [MTK]: share the similar view with ZTE FL reply: at least not support the larger than one scaling factor, the current spec needs clarification from the submitted CRs. |
| 2-3 | CR on PDCCH monitoring behavior when overlaps with rate matching pattern FL Note: initial discussion in last RAN1 meeting, further discussion is needed in this meeting | Huawei[R1-2208469] MediaTek[R1-2209525] Qualcomm[R1-2209961] | H | [ZTE]: Agree with FL initial assessment. Ericsson: OK to discuss MTK: agree with FL’s assessment |
| 2-4 | TP on FDRA determination of multicast DCI formats FL Note: correct the FDRA bitlength formula, easily to be agreeable | Nokia[R1-2208701] | H | [ZTE]: Agree with FL initial assessment. Ericsson: OK to discuss |
| 2-5 | CR on SS0 availability for scheduling MBS FL Note: align the UE behavior of using SS0 for MBS between TS 38.331 and TS 38.213 | Huawei[R1-2208470] | H | [ZTE]: Agree with FL initial assessment. Ericsson: OK to discuss. the description in 213 seems aligned with the description in section 5.9.1.2 of 38.331: If searchspaceMCCH is set to zero, PDCCH monitoring occasions for MCCH message reception in the MCCH transmission window are the same as PDCCH monitoring occasions for SIB1 where the mapping between PDCCH monitoring occasions and SSBs is specified in TS 38.213[13]. FL reply: @Ericsson, not configured as stated in TS38.213 is not the same meaning of ‘ searchspaceMCCH is set to zero’, it is the point for this CR. |
| 2-6 | CR on multicast rate-matching pattern configuration number FL Note: align the configured multicast rate-matching pattern number between TS 38.331 and TS 38.214 | CMCC[R1-2209313] | H | [ZTE]: Agree with FL initial assessment. Ericsson: OK to discuss |
| 2-7 | CR on CFR configuration and simultaneous configuration of multicast PDSCH on two serving cells FL Note: correct the CFR bandwidth and location configuration | ZTE[R1-2209471] CATT[TP#2 in R1-2208927] | H | [CATT] Agree with FL’s assessment. RAN1 agreed that the UE is not required to be configured PDSCHs on two serving cells. Although the agreement has been captured in the current specification, it doesn’t align with the agreement, [QC] We think the current RAN2 spec has clarified the configuration and RAN1 spec can just delete “ The UE is not required to simultaneously receive PDSCHs on two serving cells.” as spec alignment. [Final FL recommendation]: the above comments will be considered, seems no objection to discuss this issue.. |
| 2-8 | CR on collision handling between SPS and DG for MBS FL Note: define the UE behavior of PDSCH collision between unicast DG and multicast SPS or MBS DG and unicast SPS | CMCC[R1-2209314] | H | [ZTE]: Agree with FL initial assessment. Ericsson: OK to discuss |
| 2-9 | CR on multicast SPS activation validation when UE is only configured one multicast SPS FL Note: To address the issue that SPS index=0 cannot be configured to some UE which only supports one multicast SPS | ASUSTeK[R1-2210075] | H | [ZTE]: Agree with FL initial assessment. Ericsson: OK to discuss |
| 2-10 | CR on definition of G-CS-RNTI for SPS group-common PDSCH retransmission FL Note: FL views this CR is not pursued, the following sentence in TS 38.213 has declared that G-CS-RNTI can be used for SPS GC-PDSCH retransmission. “For the first HARQ-ACK reporting mode and for a transport block that a UE received in a SPS PDSCH, a PDSCH reception providing a retransmission of the transport block can be scheduled either by a unicast DCI format using a CS-RNTI or by a multicast DCI format using a same G-CS-RNTI as the G-CS-RNTI of the initial transmission of the transport block [6, TS 38.214].” | CATT[TP#1 in R1-2208927] | Not an essential correction Not pursued | [CATT] We think the CR on definition of G-CS-RNTI is an essential correction. Since RAN1 has been agreed define G-CS-RNTI for providing activation, release and scheduling retransmission of SPS PDSCH. However, in the current spec, it is described as ‘ The UE can be provided one or more G-CS-RNTI per serving cell for scrambling the CRC of multicast DCI formats providing activation/release for SPS PDSCH receptions.’ , and ‘providing scheduling retransmission for SPS PDSCH receptions’ was not captured. Although G-CS-RNTI used for scheduling retransmission for SPS PDSCH receptions can be inferred from the FL’s explanation, for the sentence of G-CS-RNTI definition, it still miss the description of ‘providing scheduling retransmission for SPS PDSCH reception’. We believe the function of G-CS-RNTI for activation/release/scheduling retransmission should be treated equally in the spec. FL reply: only proponent explained the reason for the CR but also acknowledged it can be inferred. If it is easily agreeable, then can be considered. |
| 2-11 | Alignment CR on RRC parameters correction in TS 38.211 | CMCC[R1-2209315] | E | [ZTE]: Agree with FL initial assessment. |
| 2-12 | Alignment CR on RRC parameters correction in TS 38.213 | CMCC[R1-2209316] ASUSTeK[R1-2210095] | E | [ZTE]: Agree with FL initial assessment. |
| 2-13 | Alignment CR on RRC parameters correction in TS 38.213 | CMCC[R1-2209317] | E | [ZTE]: Agree with FL initial assessment. |
| 2-14 | Alignment CR on RRC parameters correction in TS 38.214 | CMCC[R1-2209318] ASUSTeK[R1-2210096] | E | [ZTE]: Agree with FL initial assessment. |
| 2-15 | CR on terms of G-RNTI used for MTCH FL Note: ok to align the usage of the terms across the entire spec | ZTE[R1-2209472] | E | [ZTE]: Agree with FL initial assessment. |
| 2-16 | per G-RNTI timeDurationForQCL configuration | LGE[R1-2209449] | Discuss at RAN1#110bis-e only to clarify the issue and whether it is essential (to control the workload) | Ericsson: we think it is not critical. The network will have to ensure that TCI states are consistently configured across UEs. [LGE]:We think that the current spec cannot work. How can the network ensure same UE capabilities of timeDurationForQCL across different UEs receiving the same G-RNTI? FL reply: different views, then discussion is needed focusing on whether essential or not. |
| 2-17 | CR on FDM SPS collision handling FL Note: half of companies commented it as non-essential issue in last RAN1 meeting | vivo[R1-2208620] ZTE[R1-2209474 Ericsson [R1-2210173] | Not an essential correction Not pursued | [vivo] We think it should be discussed at least for the case UE is only capable of receiving FDMed unicast and multicast PDSCH, otherwise, the spec is incomplete. [ZTE]: We prefer to discuss this issue and we share the same view with vivo. MTK: agree with FL’s assessment FL reply: FL was assuming no change is needed if not supported FDMed u-cast SPS and m-cast SPS, if companies disagree with this point, then discussion can be conducted focusing on whether change is needed if not supported FDMed |
| 2-18 | MBS SPS configuration on SCell FL Note: The LS was guided to be discussed in UE feature session but was not touched due to limited time in the last meeting. FL assesses LS reply is not necessarily needed even though the issue has not been reached in UE feature discussion session. | vivo-Draf LSR-x08581 | Discuss at RAN1#110bis-e only to clarify whether LS reply is needed though the issue will not be finalized in UE feature session in time (to control the workload) | [ZTE]: Agree with FL initial assessment. |

# Conclusion

As declared by the session chair, the preparation phase discussion concludes the following:

Conclusion of [110bis-e-R17-MBS-01]:

For Rel-17 MBS HARQ maintenance:

- Discuss the following issues: 1-1, 1-2, 1-3, 1-4, 1-5, 1-6, 1-7 (including whether case 2 and case 3 with NACK-only mode2 is supported in Rel-17), 1-8, 1-9, 1-10, 1-11, 1-12, 1-21

- Discuss the following editorial/alignment issues for providing to spec editors: 1-14, 1-15,

- Discuss for clarification of the issue (potentially discuss CR at RAN1#111, or conclude at RAN1#110bis-e that the issue is not essential): 1-16, 1-17, 1-18, 1-19, 1-20

- Postponed: 1-13 to the next meeting

- The corresponding draft CRs are not pursued in Rel-17 maintenance for these issues: 1-22 (a potential conclusion is to be discussed at RAN1#110bis-e), 1-23, 1-24

For Rel-17 MBS scheduling maintenance:

- Discuss the following issues: 2-1, 2-2, 2-3, 2-4, 2-5, 2-6, 2-7 (including whether this is simply a spec alignment issue), 2-8, 2-9

- Discuss the following editorial/alignment issues for providing to spec editors: 2-11, 2-12, 2-13, 2-14, 2-15

- Discuss for clarification of the issue (potentially discuss CR at RAN1#111, or conclude at RAN1#110bis-e that the issue is not essential): 2-10, 2-16, 2-17

- Discuss whether LS reply to R1-2208581 is needed for issue 2-18

# References

R1-2208466	Correction on processing timeline for NACK-only mode2 to TS38.214	Huawei, HiSilicon, CBN

R1-2208467	Correction on codebook type for NACK-only HARQ-ACK feedback to TS38.213	Huawei, HiSilicon, CBN

R1-2208468	Correction on PRI for NACK-only HARQ-ACK feedback to TS38.213	Huawei, HiSilicon, CBN

R1-2208469	Correction on UE behaviors of PDCCH monitoring for configured RM patterns to TS38.213	Huawei, HiSilicon, CBN

R1-2208470	Correction on SS0 availability for scheduling MBS to TS38.213	Huawei, HiSilicon, CBN

R1-2208617	Draft CR on HARQ-ACK feedback for PDSCH scheduled by DCI format 4-1	vivo

R1-2208618	Draft CR on PUCCH determination for UE configured with NACK-only feedback mode	vivo

R1-2208619	Draft CR on type 2 codebook determination with DG PDSCHs and SPS PDSCHs	vivo

R1-2208620	Discussion on SPS PDSCH overlapping handling in FDM case	vivo

R1-2208701	Remaining Issues for RRC_CONNECTED UEs supporting MBS	Nokia, Nokia Shanghai Bell

R1-2208887	Draft CR on HARQ-ACK multiplexing of unicast SPS PDSCHs and multicast DG PDSCHs	vivo

R1-2208923	Discussion on MBS supporting HARQ-ACK codebook retransmission	CATT

R1-2208924	Discussion on MBS supporting  deferring HARQ-ACK for SPS PDSCH	CATT

R1-2208925	Draft CR on MBS supporting HARQ-ACK codebook retransmission	CATT

R1-2208926	Draft CR on MBS supporting  deferring HARQ-ACK for SPS PDSCH	CATT

R1-2208927	Draft CRs for NR Multicast and Broadcast Service	CATT

R1-2208928	Corrections on multicast DCI format to enable/disable HARQ-ACK	CATT

R1-2208929	Discussion on  multicast DCI format to enable/disable HARQ-ACK	CATT

R1-2208995	Correction on Type-2 HARQ-ACK codebook for MBS	Langbo

R1-2208996	Correction on Type-3 HARQ-ACK codebook for MBS	Langbo

R1-2209137	Remaining Issues on NR MBS	NEC

R1-2209310	Remaining issues on HARQ-ACK feedback for multicast	CMCC

R1-2209311	Discussion on specs alignment of PDSCH simultaneous reception for MBS	CMCC

R1-2209312	Draft CR on multicast HARQ-ACK codebook type configuration in DCI formats	CMCC

R1-2209313	Draft CR on multicast rate-matching pattern configuration	CMCC

R1-2209314	Draft CR on SPS and dynamic scheduling PDSCH(s) collision for MBS	CMCC

R1-2209315	Draft CR on RRC parameters correction in TS 38.211	CMCC

R1-2209316	Draft CR on RRC parameters correction in TS 38.212	CMCC

R1-2209317	Draft CR on RRC parameters correction in TS 38.213	CMCC

R1-2209318	Draft CR on RRC parameters correction in TS 38.214	CMCC

R1-2209449	Maintenance on NR Multicast and Broadcast Services	LG Electronics

R1-2209470	Maintenance of broadcast and multicast for MBS	ZTE

R1-2209471	Draft CR on CFR configuration for multicast	ZTE

R1-2209472	Draft CR on terms of G-RNTI used for MTCH	ZTE

R1-2209473	Draft CR on restrictions of simultaneous reception	ZTE

R1-2209474	Draft CR on SPS collision handling	ZTE

R1-2209475	Draft CR on 1 bit NACK-only feedback	ZTE

R1-2209476	Draft CR on determining NACK-only PUCCH in NACK-only mode1	ZTE

R1-2209524	Corrections on the MBS reception type combinations in TS 38.202	MediaTek Inc.

R1-2209525	Corrections on the MBS in TS 38.213	MediaTek Inc.

R1-2209526	Corrections on the MBS in TS 38.214	MediaTek Inc.

R1-2209527	Remaining issues on NR MBS	MediaTek Inc.

R1-2209566	Remaining issues on NR Multicast and Broadcast Services	Apple

R1-2209708	Maintenance on multicast-broadcast services	Samsung

R1-2209822	Remaining issues for Rel-17 MBS	Huawei, HiSilicon, CBN

R1-2209832	Correction on processing timeline for NACK-only mode2 to TS38.213	Huawei, HiSilicon, CBN

R1-2209833	Correction on the max data rate for multiplexing MBS and unicast to TS38.214	Huawei, HiSilicon, CBN

R1-2209882	Draft CR on DAI field in DCI format 4_2	NTT DOCOMO, INC.

R1-2209883	Draft CR on HARQ-ACK feedback for SPS GC-PDSCH	NTT DOCOMO, INC.

R1-2209884	Draft CR on NACK-only based feedback for multicast	NTT DOCOMO, INC.

R1-2209885	Draft CR on multiplexing NACK-only based feedback with SR	NTT DOCOMO, INC.

R1-2209954	Draft CR on DCI-indicated enabling/disabling multicast feedback for Type-1 CB	Qualcomm Incorporated

R1-2209955	Draft CR on Type-2 CB for NACK-only multicast feedback	Qualcomm Incorporated

R1-2209956	Draft CR on max data rate per CC in case of FDMed unicast and MBS PDSCHs	Qualcomm Incorporated

R1-2209957	Scaling factor for FDMed unicast and MBS PDSCHs	Qualcomm Incorporated

R1-2209958	Draft CR on upper bound of TBS LBRM in case of FDMed unicast and MBS PDSCHs	Qualcomm Incorporated

R1-2209959	Draft CR on PDSCH processing time required to select PUCCH for NACK-only mode2 based multicast feedback	Qualcomm Incorporated

R1-2209960	Draft CR on multicast PDSCH with a HARQ process with disabled HARQ-ACK feedback	Qualcomm Incorporated

R1-2209961	Draft CR on PDCCH monitoring when overlapping with rate matching patterns	Qualcomm Incorporated

R1-2210075	Correction on MBS SPS	ASUSTeK

R1-2210095	Correction on configurations of G-RNTI and G-CS-RNTI	ASUSTeK

R1-2210096	Correction on configuration of PDSCH aggregation factor for MBS	ASUSTeK

R1-2210155	Correction on HARQ-ACK codebook types in UL DCI formats for scheduling MBS	Lenovo

R1-2210156	Draft CR on HARQ-ACK feedback for PDSCH scheduled by DCI format 4_1	Lenovo

R1-2210157	Draft CR on DAI update for multicast DCI formats	Lenovo

R1-2210158	Draft CR on simultaneous configuration of Type-1 HARQ-ACK codebook and dci-enabler for multicast service	Lenovo

R1-2210159	Remaining issues on HARQ-ACK feedback for NR MBS	Lenovo

R1-2210173	Maintenance on NR Multicast and Broadcast Services	Ericsson

R1-2210207	Correction on retransmission schemes for MBS HARQ-ACK feedback to TS38.213	Huawei, HiSilicon, CBN

R1-2210208	Correction on the channel combinations for MBS UE handling to TS38.213	Huawei, HiSilicon, CBN

R1-2210209	Correction on the channel combinations for MBS UE handling to TS38.214	Huawei, HiSilicon, CBN

R1-2210210	Correction on the channel combinations for MBS UE handling to TS38.202	Huawei, HiSilicon, CBN