3GPP TSG RAN WG1 #110-bis-e                                                                                                                                       R1-2210483

e-Meeting, October 10th – 19th, 2022

Source:	Moderator (ZTE)

Title:	Summary#3 of discussion on side control information

Agenda Item:	9.8.1

Document for:     	 Discussion and Decision

- Introduction

In this AI, the discussion is expected based on following objectives:

Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information - Note: Power control aspect will be checked in RAN#98e.

Then, according to the companies’ inputs, the views on following essential aspects are summarized as below:

- Side control information:

  - Beam information & Timing indication for access link

  - Beam indication for backhaul link

- Side control information: ON-OFF information

- Side control information: TDD information

- Others

Companies are encouraged to provide the inputs for corresponding topics.

- Topic-1 Beam information

  - Beam information & Timing indication for access link

    - Company view (Round-1)

Regarding the beam indication for access link, following agreement has been made in RAN1#110:

Agreement Beam index is used to indicate an access link beam (Option 1)

To enable the beam indication for access link, based on the inputs, following aspects are highlighted by companies:

- Definition and acquisition of beam characteristic

- Association between beam index and real beam

- Indication of beam index

- Indication of applicable time domain information

      - Definition/acquisition of beam characteristic

For the beam characteristic, the following features are mentioned by companies:

Feature of beam:

- Number of beams supported for access link: [vivo, IDC, ZTE, Intel, CAICT, CMCC, MTK, LGE, CEWiT, NEC, CMCC, ETRI, Lenovo, Ericsson] supports to report the number of supported beam.

    - More specifically, this value can be determined per repeater antenna panel [ETRI], per beam type/width [NEC], per dimension [Ericsson], or for basic coverage [CMCC].

    - In addition, report of number of beams that can operate simultaneously, is also supported by [IDC, ZTE, Intel, CAICT, CMCC, CEWiT]

- Beam types/width: [Huawei, IDC, ZTE, Intel, CAICT, CMCC, NEC] supports to report the beam type.

- Beam arrangement (i.e., defined by the spatial relationship between different beam): [vivo, IDC, ZTE, Intel, CAICT, CMCC, LGE] proposes to report the beam arrangement. Moreover, the beam direction part is also proposed by [vivo, Huawei]

Others, e.g., number of port/panel/element, capability to form the wider beam, are also proposed by single company.

- For the acquisition of beam characteristic:

[IDC, ZTE, CT, Fujitsu, Intel, NEC, CAICT, Xiaomi, CMCC, ETRI, MTK, LGE, Lenovo, CEWiT] highlights that the beam characteristic can be reported as NCR capability. But [Huawei] mentions that OAM-based configuration is simpler than NCR-MT capability reporting

Based on the inputs above, there is clear majority to define the beam characteristic as part of NCR capability. Meanwhile, at least the number of beams supported for access link, Beam types/width and beam arrangement should be supported.

Then, following is provided:

Proposal 1-1-1-A: NCR capability signalling via RRC is supported at least to characterize the beam information of NCR-Fwd for access link

- FFS: Details on beam information

Please noticed that this proposal is for NCR-Fwd and access link. And as common ground, the information changes is definitely needed. Companies are encouraged to share your views and other options if there is concern.

| Companies | Comments and Views |
| --- | --- |
| Nokia | Support in principle but propose to remove “via RRC” as it it not within the scope of RAN1 to determine how these parameters are indicated. |
| Intel | We support NCR capability report by RRC. Per the discussion in RAN 97 meeting, inter-vendor interoperability is important, thus we don’t support the option with only OAM. |
| vivo | We support NCR capability report by RRC. |
| ZTE | It’s definitely in the scope of RAN1 regarding which information are required to define the beam since the corresponding signalling design is based on the assumed information. Meanwhile, we also need to discuss the potential to deliver this message. If no capability is defined, the only way is to assume that all information will be known via implementation. |
| Fujitsu | Support this proposal. |
| Lenovo | Generally fine with it. |
| Samsung | Fine with the proposal. Also suggest to add an FFS for information other than beamforming. FFS: Other information for which NCR capability signaling via RRC |
| CMCC2 | We support the NCR capability report by RRC which benefits to the inter-vender operation. We do not support OAM-only mechanism. |
| NEC | Support. |
| CEWiT | Support capability reporting from NCR to gNB. |
| Apple | Support |
| KDDI | Generally fine with the proposal. And for the “FFS” part, we agree with the proposal from Samsung. |
| Sharp | Support |
| IIT-K | Support this proposal. |
| InterDigital | We support the proposal 1-1-1-A. |
| Qualcomm | Support. |
| Sony | We support this proposal. |
| Philips | We support the proposal. |
| CATT | OK |

Proposal 1-1-1-B: At least the following information is used to characterize the beam of NCR-Fwd for access link:

- Number of beams supported for access link

  - FFS: How to define the detailed value.

- Beam types defined by the beam width (e.g., wider or narrow)

- Beam arrangement (i.e., defined by the spatial relationship between different beam)

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | If different beam types can be reported, then also the number of beams supported for each beam type shall also be reported. On the beam arrangement, we don’t think that the NCR needs to report this. Once the NCR reports number of supported beams and beam types, the network can utilize that information for beam indication. Also, the actual number of beams and beam types might be different (less) than the reported values, so we don’t think it is beneficial to report any beam arrangement. Moreover, discussion on association of beam index and real beam is somewhat related to this and discussed in next section. Here is the suggested update to proposal from our side: Proposal 1-1-1: As NCR capability, the following information should be reported to characterize the beam information of NCR-Fwd for access link: Number of beams supported for access link - FFS: How to define the detailed value (e.g., number of beams for each beam type). - Beam types (i.e., defined by the beam width) - FFS: Number of beam types (e.g., wide beam type and narrow beam type) - Beam arrangement (i.e., defined by the spatial relationship between different beam) |
| CMCC | We fine with current form of the proposal. Our understanding is all those information should be provided in the capability reporting from NCR. If multiple types of beam are supported, the number of each beam type should be also be reported. |
| Huawei, HiSilicon | We don’t support the proposal. We agree that NCR/gNB should have the knowledge of set of access beams and their corresponding beam characteristics used by the NCR-Fwd but whether this is achieved by NCR capability reporting or claimed by each NCR manufacture should be discussed further. In general, NCR is a network node and capability reporting usually does not apply. Moreover, the target coverage area of NCR should be under the control of operator via OAM rather than gNB. Therefore, even if a gNB is provided with the beam information of NCR-Fwd such as number of beams, beam types, beam arrangement, etc., the gNB has no idea which set of access beams should be selected for the access link since the information of target coverage area is not known. Thus, we propose the following: Proposal 1-1-1: As NCR capability, the following beam characteristics information should be reported to characterize the beam information of NCR-Fwd for access link should be provided to the gNB: Number Set of beams supported for access link - FFS: How to define the detailed value. - Beam types (i.e., defined by the beam width) - Beam arrangement (i.e., defined by the spatial relationship between different beam) Beam direction |
| Ericsson | Support. The number of beams should be clarified if it refers to the total number of beams or the number of fundamental (narrowest) beams. |
| LG | Generally fine with us. One step further, we think not only number of supported beams are reported, but also regarding beam information should be reported at the same time. In that aspect, second bullet can be modified as “beam type per access beam”. And for further discussion how to organize those information, following can be added below to the beam arrangement. “FFS: how to organize the information” |
| NTT DOCOMO | Support |
| Nokia | While we are in general supportive of the proposal, it may be more appropriate to request guidance from RAN4 regarding how beam types and beam arrangements can be provide. |
| Intel | We support the proposal in general. Regarding the beam identifier/index commented by companies in GTW, whether beam identifier/index is needed depend on the assumption of what spatial relation information is provided. If some high-level spatial relation is sufficient, e.g., NCR reports the 8 beams with narrow beam and 2 beams with wide beam and every 4 narrow beams are QCL type-D with 1 wide beam, it seems no need of beam identifier/index. gNB can configure beams for NCR, with beam type and the QCL reference beam for each beam index, e.g., beam 0 with wide beam, index 1/2/3/4 with narrow beam which is associated with beam 0. But if detailed spatial relation is reported, e.g., beam direction for each beam, then, beam identifier/index should be reported together with beam type and beam direction. For this case, no need of additional report of total number of beams, because gNB can know the number by counting the reported beam indices. |
| AT&T | We generally support the proposal, along with some of the clerical updates. - Sets of beams supported for the access link - FFS: How to define the detailed value(s), e.g., sets of beams for each beam type. - Beam types - defined by the beam width (e.g., wider beam type or narrow beam type) - Beam arrangement (i.e., defined by the spatial relationship between different beams) |
| vivo | Regarding the beam type, we think report the number of beam type is sufficient |
| Spreadtrum | We are fine with the proposal |
| ZTE | We are fine with this proposal. Regarding the number of beam, as mentioned in the FFS bullet, further discussion on how to define the value can be considered, e.g., per beam type or others. For the beam index, it’s belong to the discussion in section 1.1.1.2. We can conclude whether this value is configured by gNB or reported by NCR or based on pre-defined rule. |
| Fujitsu | We are basically fine with the characteristics listed in the proposal. But it should be noticed that the characteristics listed here are dependent. For example, the beam type can be reported together with the beam number, or beam indexes or beam arrangement. A dedicated field for beam type seems redundancy. We also wonder if the beam types are reported together with beam index, they would be carried by the capability signaling or another signaling dedicated for beam index report. |
| Lenovo | We are generally fine with the proposal as we support RRC reporting of the NCR capability. We also think the number of beams may be impacted by the beam types. Different numbers may be associated with wide beams or narrow beams. Also, we think that the number of beams and their characteristics need to be reported per supported band as the number the characteristics could be different for different frequency bands. Regarding beam arrangement, we also think explicit spatial relationship or implicit way by beam index can be used to determine the beam arrangement. At least we think that signaling in terms of association between narrow and wide beams can be considered (e.g., narrow beams withing a wide beam) for beam refinement. Besides, we also think the number of antenna ports/elements is necessary for setting a suitable transmission rank or to determine the precoder especially when the antenna ports/elements is different from that of gNB or UE. |
| Samsung | This information is essential for gNB to control the NCR operation. Some clarifications are needed. In terms of beam arrangement, not sure the intention is to report the spatial angle of different beams or to report the association of different beam. Also, without reference to a beam index, it is not clear how beam type and especially beam arrangement can be reported. So, we suggest to add beam index to the list as well, while the detailed signaling can be discussed separately. RAN4 list of manufactured declarations for Rel-17 RF repeater includes many more items (beam angle/direction, coverage area, beam power, etc.) whose benefit for NCR can be further discussed. Therefore, we suggest the following modifications: Proposal 1-1-1-B: The gNB is provided/aware of At least the following information which is used to characterize the beam of NCR-Fwd for access link: - Number of beams supported for access link - FFS: How to define the detailed value. - Beam indexes - Beam types defined by the beam width (e.g., wider or narrow) - Beam arrangement (i.e., defined by the spatial relationship between different beam) - FFS: association and signaling mechanism for the information - FFS: information of other beam characteristics |
| CEWiT | Support |
| Apple2 | We are still a bit unclear on the beam arrangement aspect. Also, looking at comments, there seem to be different understanding on this. From our point of view, it needs to be clarified if beam arrangement reporting will include exact reporting in terms of beam index and corresponding relation in terms of relative angle/orientation. Or as also commented by Intel, does it entail some sort of QCL assumption between different beam types. For example, narrow beam indices can be QCLed to a wide beam index Also, for beam types, it can be FFS: how many beam types We think only two types are sufficient i.e., wide and narrow |
| China Telecom | Fine with the proposal. We share the similar view that a FFS may be needed for the bullet of beam types, e.g., the number of beam types, detail mechanism to define the beam types. |
| Sharp | Fine with the proposal. |
| IIT-K | Support in principle |
| InterDigital | We support the Proposal 1-1-1-B. The beam arrangement can be configured via indicating the beam index of the associated wide beam for respective narrow beams. That is the beam index of the wide beam that is associated with a narrow beam can be indicated as part of indication for the beam characteristics of the narrow beam, along with beam index, beam type, and so forth. |
| Qualcomm | Support. |
| Sony | In principle, we support the proposal. |
| Philips | In general, we agree with the proposal but see a need for further study of beam types and arrangements. |
| CATT | Ok with the proposal. We prefer to replace ‘ Beam arrangement’ with ‘beam relationship’ |

      - Association between beam index and real beam

For the association between beam index and real beam, companies [Spreadtrum, ZTE, Samsung, Ericsson] highlights that association between beam index and real beam should be defined.

More specifically,

- [Spreadtrum, ZTE, Samsung] propose to define the association via Gnb’s indication;

- [Samsung, Huawei, NEC] propose to let NCR report this information to Gnb.

- [Ericsson, Samsung] propose to define the rules for the determination of the association

For the detailed association, following options is proposed can be considered:

- Option-1: A unified numbering mechanism can be used to index all beams [Fujitsu, ZTE]

- Option-2: A separate numbering mechanism for different beam types [ZTE]

- Option-3: A hierarchical numbering mechanism for different beam types [ZTE]

However, [MTK] proposes that how NCR-Fwd perform beamforming according to an access link beam index is up to NCR implementation and transparent to Gnb/UE.

Based on the inputs above, from FL’s perspective, although there is different preference to obtain the association between beam index and real beam, at least some basic should be supported to numbering the beam.

Then, followings are proposed:

Proposal 1-1-2: One of following options is supported to define the association between beam index and real beam supported by NCR-Fwd for access link:

- Option-1: All beams are indexed sequentially

- Option-2: Beams are independently indexed per beam types

- Option-3: Beams are hierarchically indexed for different beam types

Note: the sequence of beam can be defined from one dimension, e.g., vertical or horizontal.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | It is not fully clear to us how the above options will help define the association between beam index and real beam. In our view, we need to establish some reference beam direction, to be able to index beam relative to that. In our contribution we discuss the concept of reference coverage region. This could very well be considered as wide beam. The reference could be the backhaul/control link boresight since this can be fixed between the Gnb and NCR as both are static nodes. Then at least the wide beam sequence can be indexed relative to that. Therefore, without this reference direction, it is is meaningless to define sequence of beam index. Moreover, we think that just sequential indexing of wide beam index (relative to reference wide beam) is sufficient, then the sequence of narrow beams within the wide beam can be avoided. Otherwise, the overhead could be quite large. |
| CMCC | Fine with the proposal. Option 2 and 3 are slightly preferred. |
| Huawei, HiSilicon | We would like to have some clarifications. In Option-1, does “all beams” means the sets of beams that would be used by NCR or the set of beams supported by NCR? If it is the latter, we support Option-1. |
| Ericsson | Support Option 2. |
| LG | Support option 1. As given in our contribution, details of beam information, such as beam width, can be reported to gNB therefore makes full flexibility. |
| NTT DOCOMO | Support in general. However, the motivation of the note is not very clear to us. Does it intend to discuss how the real beams are ordered? Among the 3 options, we prefer option1. |
| Nokia | Prefer Option-1. Since there is no expectation that NCR is aware of what channels and signals are being forwarded, there does not seem to be a clear motivation for a hierarchical or structured indexing of beams. What the configuration of beam for the NCR, each beam will need to be uniquely and independently identifiable. |
| Intel | We also have similar question as HW, is “all beams” the sets of beams configured for NCR or reported by NCR ? We should support the mechanism which supports smaller configured beams than reported beams. For example, if NCR reports 2 wide beams and 8 narrow beams while gNB only wants to configure 2 wide beam and 4 narrow beams, how to apply option 1/2/3 respectively? |
| AT&T | Support. We slightly prefer option 2. |
| vivo | We prefer option 1. For option 2/3, we are not sure whether additional information besides beam index (i.e., beam type) will be conveyed by the side control information |
| Spreadtrum | We share the same view with Apple, the essential problem is to establish the association between beam index and beam direction, but the rules of numbering the beams cannot reveal the association between the index and real beam. |
| ZTE | We also prefer to take Option-2 for further progress. For the Option-1, all beams should be the beam supported by NCR, otherwise, even with configuration from gNB, it’s hard to let the NCR know which beams are supported. |
| Fujitsu | We have two questions for clarification. 1. Are the beams in option 1-3 the beams supported by the NCR or the beams the gNB configures for the NCR? 2. Does ‘all beams’ in option 1 include both wide beams and narrow beams? If both are true, we prefer option 1. |
| Lenovo | We are generally fine the proposal. |
| Samsung | In principle, we are supportive of the intention of Option 1 since it is the simplest solution of beam indication. By adopting Option 1, there is no need to further indicate beam type or other information. It is assumed that, the gNB is aware/provided the beam type for each beam index (per FL P1.1.1-B). However, we think concepts such as “dimension”, “vertical”, “horizontal”, and so on are RAN4 domain, and can be specified if needed. From RAN1 perspective, the proposal should be re-formulated as follows (in which, again, we prefer Option 1): Proposal 1-1-2: One of following options is supported to define the association between beam index and real physical beam supported by NCR-Fwd for access link: - Option-1: All beams are indexed sequentially separate beam indexes are used for all beams, regardless of beam type. - Option-2: Beams are independently indexed per beam types Separate beam indexes are used for beams of a same type, but beam indexes can be shared for different beam indexes. - Option-3: Beams are hierarchically indexed for different beam types Note: the sequence of beam can be defined from one dimension, e.g., vertical or horizontal. |
| Xiaomi | We support Option 1 in principle. We share similar concern with HW and Intel, whether “all beams” means beams reported by NCR or beams configured by gNB should be clarified. |
| NEC | Prefer Option-3. And open to the other options. In addition, to associate the beam index with real beam, a mapping relationship should be defined, not only the beam index. |
| CEWiT | Fine with the proposal. Support option 1 |
| China Telecom | We are generally fine with the proposal. We slightly prefer Option 2 and 3. It seems they indicate the similar thing. |
| Sharp | Support the proposals, with preference for Option 1. |
| IIT-K | We prefer Option-1 or Option-2. However, as mentioned by Apple and NEC, mapping between beam index and beam direction needs to be defined. |
| InterDigital | We support the Proposal 1-1-2. In general, we prefer Option 3 as the beam arrangement is explicitly indicated. However, we are ok with other options as long as the beam arrangement is specified via implicit/explicit indications. |
| Qualcomm | Clarifications are needed. There are already so many different interpretations of this proposal by different companies that show the proposal in its current form, and its real objective, is not clear. If the objective is to have “separate” beam indices across all supported beams, or reuse beam indices across beams of different types, then the proposal should be changed following e.g., Samsung’s suggestion. If the objective is to establish an implicit connection between the beam indices and the beam directions – e.g., such that beam i and beam i+1are towards two adjacent beam directions, then this should be clearly mentioned. Also, Option 3 “hierarchical indexing” and the note are not clear either. |
| Sony | We agree with other companies that the proposal is not clear and clarifications are needed, e.g., see comments by Fujitsu and Qualcomm. |
| Philips | We prefer options 2 and 3 on the understanding that beams are mapped to indices based on their properties. |
| CATT | We think it’s better to discuss previous question to get more clear beam structure then we can discuss how to index. BTW, we think there could be more than one options supported |

Proposal 1-1-3: One of following options is supported to determine the beam index of the beam supported by NCR-Fwd for access link:

- Option-1: The beam index is configured by gNB

- Option-2: The beam index is reported by gNB NCR

- Option-3: The beam index is determined by the pre-defined rule.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Not sure what’s option 2. Do you mean beam index is reported by NCR? |
| CMCC | Our understanding is NCR have the knowledge of beam indices, beam types and the association or the relations between wide beam and narrow beam. Then if the NCR can report all the information above, there is no need for gNB to configure any beam indices. And it is not clear how gNB can configure indices to the NCR. The options could be updated as below. - Option-1: The beam index indices is are configured by gNB - Option-2: The beam index is indices are reported by to gNB - Option-3: The beam index is indices are determined by the pre-defined rule. |
| Huawei, HiSilicon | Support and prefer option-1. |
| Ericsson | Support Option 3 with the understanding that this proposal relates to the mapping between the physical properties of a beam and its beam index. |
| LG | Support Option 1. Among all of beams supported by NCR-Fwd, mapping relationship between beam index and access link beam should be configured by gNB to reduce beam indication overhead. That is, if the mapping relation is not configured by gNB, then the access link beam indication should be based on the whole supported beams, which induce redundant signaling overhead. Instead, subset of the supported access link beams of NCR can be selected by gNB for further access link beam indication, which enables to reduce beam indication overhead. By the way, we think for option 2, it seems report “to gNB” rather than “by gNB”. |
| NTT DOCOMO | Support the proposal. We prefer option1. Similar view as LG, we think among the supported beams, gNB can decide whether all or some of the beams are used and configure the index. |
| Nokia | Preference for Option-1 as it seems to provide the most flexibility in the case that NCR may configure more access beams than are actively available at any time. |
| Intel | We support option 1 to enable smaller beams configured for NCR than the beams reported by NCR. Option 2 and option 3 depends on the assumption for NCR reporting. As commented for proposal 1-1-B, NCR may not report a beam index and detailed beam direction, thus option 2/3 may not work. And also if we just reuse vendor declaration as defined for Rel-17 RF repeater, it seems beam identifier can be in any unique title ‘A unique title to identify a beam, e.g. a, b, c or 1, 2, 3. The vendor may declare any number of beams with unique identifiers.’. It is unclear how to use option 2 and option 3. |
| AT&T | Generally, support the proposal and the CMCC clerical change for multiple indices. Support option 1. |
| Vivo | We agree with Ericsson, how to map beam index with the supported beam can be achieved based on defined rule. |
| Spreadtrum | Support the proposal. If the beam index is configured by gNB, how does the NCR know about association between the beam and the direction? Opt2 and Opt3 are preferred. |
| ZTE | Support. The intention of this proposal is to define the mapping relationship between real beam and beam index. For Option-1, it implies that there is already common understanding between gNB and NCR on the detailed information of each beam to support the configured index. For example, if 10 beams are supported by NCR, the NCR should get the information on corresponding beams if only 5 beam are configured by gNB. For Option-2, the NCR can determine the index along with characteristics of beam and report it to gNB together. Then, the corresponding value will be used for following controlling. Option-3 is applicable to cover all cases and can be combined with either Option-1 and -2. |
| Fujitsu | We’d like to check whether our understanding on the relation between Proposal 1-1-2 and Proposal 1-1-3 is in line with the FL’s intention. - In Proposal 1-1-2, the [predefined] rules are discussed on how the beams supported by the NCR are indexed, but who will do it is not said. - In proposal 1-1-3, it is about the beams supported by the NCR are indexed by the gNB (Opt1), by the NCR (Opt2) or by both the gNB and the NCR without additional signaling (Opt3). If our understanding is correct, we slightly prefer Opt2. |
| Lenovo | We are generally fine with the proposal. An editorial correction for option 2 is that the report is to gNB rather than by gNB. Regarding selection of options, our view is that based on the capability reporting on number of beams and beam widths from NCR to gNB and a predefined rule(one option in proposal 1-1-2), the gNB and UE can have common understanding on the association between beam index and a beam. |
| Samsung | For this proposal, we are supportive of option 1. In the end, gNB and NCR should have common understanding on beam index related information. For example, NCR can provide the number of supported number of beams and gNB can provide the configuration of actual beams that will be used for indication. In order to avoid any confusion between operational beams and supported beams, we suggest the following modification: Proposal 1-1-3: One of following options is supported to determine the beam index of the beam supported operated by NCR-Fwd for access link: - Option-1: The beam index is configured by gNB - Option-2: The beam index is reported by gNB NCR - Option-3: The beam index is determined by the pre-defined rule. Note: The beam indexes of the beams supported by NCR-Fwd for access link is reported by the NCR. |
| Xiaomi | We support Option 1, which is feasible for gNB to decide the beam index and avoid useless beam indexes. |
| NEC | Support the proposal in general. Some suggestions shown as follow may be more general: - Option-1: The indicated beam index is based on the beams configured by gNB - Option-2: The indicated beam index is based on the beams information reported by gNB NCR - Option-3: The indicated beam index is determined by the pre-defined rule. Based on the updated version, we prefer option-1. And option-3 is still OK to us for some scenarios. |
| CEWiT | Support the proposal. Prefer option 1, as it provides more flexibility and reduces signaling overhead. |
| Apple2 | Support the proposal and prefer option 1 |
| China Telecom | Support. We slightly prefer option 1. |
| Sharp | Support with preference for Option 1. Also, we support the revision from NEC. The beam index refers to indicated beam index from gNB. As discussed previously, there should be a mapping between the indicated beam and the actual beam on access link. |
| IIT-K | Support the proposal. Prefer option-1. |
| InterDigital | We support Proposal 1-1-3 in general. It seems that options can be combined for the final procedure. For example, the NCR determines and reports (Option-2) the NCR-FWD access link beam characteristics (e.g., number of beams, types, arrangements), based on pre-defined rules (Option-3). The gNB could then determine and indicate the beam indexes based on the received beam indexes (Option -1). |
| Qualcomm | Please modify the proposal to make it clearer and avoid multiple interpretations. We should differentiate between beam reporting (by NCR to gNB) and beam indication (by gNB to NCR). Which one does the proposal address? If it is about the latter (i.e., beam indication from gNB to NCR), please make it clearer – e.g., following Samsung’s suggestion. |
| Sony | We echo Qualcomm’s request for clarity and support Samsung’s modification, but without the note. With the understanding that Samsung’s interpretation is correct, we then support Option 1 and Option for further discussion and down-selection. Option 3 is obscure to us as no example of a “predefined rule” is given. |
| Philips | The proposal is not completely clear to us. Since the NCR has knowledge of the beams, it would seem logical that the NCR reports the beam indices. The indices can be defined according to a pre-defined mapping of beam properties to indices. On that basis, we support a combination of 2 and 3. We are open to further discussion. |
| CATT | In principle fine |

      - Indication of beam index

In RAN1#110, the following agreement is achieved:

Agreement The time domain resource corresponding to an access link beam can be determined with following options: - Option 1: Explicit determination based on the explicitly indicated the time domain resources per beam indication Note-1: Different parameters may be indicated for semi-static or dynamic beam indication Note 2: One or multiple beams can be indicated via single beam indication.

For the indication of beam, companies’ views are summarized below:

- For single beam indication per indications

  - [ZTE, Xiaomi, DCM] proposes that at least the beam indication with one beam index per indication should be supported

- For multiple beams indication per indication

  - [ZTE, Xiaomi, DCM] propose to directly indicate the multiple beam index(es)

  - [vivo, ZTE, CATT, NEC, Panasonic (periodic)] propose to define the beam pattern for multiple beam indication

  - [LGE] propose to define a dedicated beam index to indicate the multiple beam directions.

  - [QC] propose to discuss whether multi-beam NCR operation is supported and able to simultaneously transmit (or receive) signals on two or more beam directions.

Moreover, [Huawei, CT, Fujitsu, LGE] mention that indicated access beam can be a subset of beams of NCR. [vivo] propose to introduce beam restriction to avoid auto-excitation of the NCR, and [LGE] highlights to report the preferred or not-preferred access link beam(s) for each DL-Rx beam of NCR-MT. [Apple] propose to configure the access beam to CNR relative to reference coverage region. Up to 6 bits payload per beam is proposed by [ETRI]. [IDC, Nokia] propose to specify the default beam or behavior, e.g., in the event that a time resource configured for dynamic beam indication does not receive one [Nokia]

In addition, [QC, Ericsson] propose to confirm following assumption:

Working Assumption In access link, a DL beam and a UL beam which are correspondent with each other have the same beam index. - The forwarding direction of an indicated beam in access link can be determined based on its corresponding time domain resource and the UL/DL TDD configuration. - Note: The forwarding behavior (or the forwarding direction) of an indicated beam in access link in flexible symbols is separately discussed in 9.8.1.

Based on the inputs above, from FL’s perspective, the indication of beam index should be able to support indication for either single or multiple beams. The confirmation of WA in RAN1#110 is also reasonable to reduce the complexity for beam indication/management for access link. Regarding the simultaneous multiple-beam operation, if it’s related to the per beam operation, it should be postponed as highlighted in section 4. Otherwise, companies’ views can be further checked.

Then, followings are proposed:

Proposal 1-1-4: The following methods are supported for beam indication:

- Single beam index per indication is supported to indicate one beam.

- Multiple beam index(es) and/or beam pattern per indication is supported to indicate multiple beams

  - FFS: Whether multiples beam are applied in TDMed or simultaneously over same frequency resource.

- Note: The indicated beam can be sub-set of supported beam by NCR-Fwd.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support |
| CMCC | Support. |
| Huawei, HiSilicon | It is not quite clear single beam index per indication means in the proposal. Is it for semi-static transmissions or dynamic transmissions? On the other hand, regardless of semi-static transmissions or dynamic transmissions, multiple beam index(es) per indication has the merit to minimize the signaling overhead. Maybe we could first try to agree on the following Proposal 1-1-4: The following methods are method is supported for beam indication: - Single beam index per indication is supported to indicate one beam. - Multiple beam index(es) and/or beam pattern per indication is supported to indicate multiple beams - FFS: Whether multiples beam multiple beams are applied in TDMed or simultaneously over same frequency resource. - Note: The indicated beam can be sub-set of supported beam by NCR-Fwd. |
| Ericsson | It is unclear to us what the proposal means to with respect to the following: - Signaling, does the proposal relate to semi-static or dynamic signaling? We think different solutions are justified for the two types. - Multiple beams w.r.t. a single or a sequence of time resources? For the FFS, we do not think SDM can be supported. |
| LG | The multi-beam indication in the same time/frequency resource is to support operation with a multiple of beams simultaneously, but the multi-beam indication in different time resources is to save signaling overhead of indication of multiple of beams. In that respect, it is considered that multiple beam indication in the same time / frequency resource and multiple beam indication in different time resources should be discussed separately. Rather than independent signaling for each beam, it would be helpful to reduce overhead if the time resource to which each beam is applied can be informed to a multiple of beams through single signaling. On the other hand, in order to support simultaneous operation with a multiple of beams in the same time/frequency resource, it is not appropriate to indicate a multiple of beam indices in the same time/frequency resource. As discussed in our contribution, it cannot be assumed that the characteristic of the synthesized beam is the sum of the characteristics of the beam before synthesis. Therefore, rather than indicating multi-beam by indication to a multiple of beam directions, it is desirable to indicate based on a dedicated beam index for multiple beam directions. |
| NTT DOCOMO | Support |
| Nokia | Similar concern as Ericsson. We are unclear on how this proposal makes progress on signaling design. |
| Intel | We support single beam or multiple beams per indication. But it is unclear to us, what is difference between multiple beam index(es) and beam pattern? |
| AT&T | Support |
| vivo | Generally fine with the proposal. regarding the FFS, “simultaneously over same frequency resource” means SDM between UEs, we also have interest in the FDMed case, i.e., “simultaneously over same and/or different frequency resource” |
| Spreadtrum | Support |
| ZTE | Support. As discussed in the GTW session, this proposal only focus on how to represent the single or multiple beam in signalling regardless of semi-/dynamic indication. For the difference between multiple beam index and beam pattern, via the latter indication, it assumed that additional configuration of beam pattern is assumed and in each pattern, more beam are included. For single beam indication, no matter via dynamic signalling or semi-static, we can indicate the one beam per time by directly indicate the beam index. To clarify it, following updates can be considered: Proposal 1-1-4: The following methods are supported for beam indication: - Single beam index per indication is supported to indicate one beam. - Multiple beam index(es) and/or beam pattern per indication is supported to indicate multiple beams - FFS: Whether multiples beam are applied in TDMed or simultaneously over same frequency resource. - Note-1: The indicated beam can be sub-set of supported beam by NCR-Fwd. - Note-2: These two methods can be applied for both semi-static or dynamic signaling |
| Fujitsu | It is not clear whether the indication mentioned here is dynamic indication or semi-static indication. Regarding to the first bullet, if it is dynamic indication, we support it. In our understanding, single beam index per indication is for the forwarding of urgent dynamic scheduling of UEs. But, if it is semi-persistent or semi-static signaling, clarifying to support a semi-persistent or semi-static signaling can indicate only one beam is little wired. We cannot see the reasonable scenario. Regarding the second bullet, ‘per indication’ is used. We are not sure if the signaling is semi-persistent, the indication should be counted as one or two? It is not clear. Besides, regarding to the FFS bullet, we think TDM is the most natural way of multiple beams. Not sure why it needs further study. Regarding to the third bullet, the purpose is unclear. If the beams supported by NCR-Fwd is what discussed in Proposal 1-1-1, the indicated beams are definitely a sub-set of the beams supported by the NCR. If not, the meaning of supported beam by NCR-Fwd should be clarified. We’d prefer to discuss the beam indication per dynamic, semi-persistent and semi-static signaling. We think the following cases are necessary: 1. Dynamic indication can indicate single beam and single/multiple time domain resource for forwarding urgent scheduling/information. 2. More than one pattern can be configured by RRC signaling and be valid at same time, e.g., for forwarding SSB, RACH and CSI RS for tracking, as well as other periodic signaling of UEs. Using more than one patterns is an efficient way to indicate the NCR to forward common signaling. For example, the SSB, RACH and CSI RS for tracking may have different periodicities. If the beam for forwarding them is limited to be indicated via one pattern, the indication overhead may be considerable because the indicated periodicity should be the lowest common multiple of the different periodicities. 3. More than one pattern can also be configured by RRC signaling and be (de-)activated by DCI for forwarding CG PUSCH/SPS PDSCH or other semi-persistent signal of UEs. |
| Lenovo | We are generally fine with the proposal. We have a comment on the “same frequency resource”. Our view is that if the bandwidth of NCR-Fwd is large, there may be different beams for different frequency domain resources at least when there are multiple RF chains at NCR. We prefer to delete “over same frequency resource”. |
| Samsung | Support the proposal in general. Suggest to remove the “beam pattern” as it is not clear and seems redundant. For FFS part, we think multiple beams that are applied simultaneously over different frequency resources can also be considered. Hence, we suggest the following revision. Proposal 1-1-4: The following methods are supported for beam indication: - Single beam index per indication is supported to indicate one beam. - Multiple beam index(es) and/or beam pattern per indication is supported to indicate multiple beams - FFS: Whether multiples beam are applied in TDMed or simultaneously over same/different frequency resource. - Note: The indicated beam can be sub-set of supported beam by NCR-Fwd. |
| Xiaomi | Support |
| Panasonic | We share a similar view as Ericsson. |
| CEWiT | Support |
| China Telecom | Support |
| Sharp | Support |
| IIT-K | Generally fine with the proposal. Regarding FFS for multiple beams, we think it could be both over different time resources as well as same time resource. For the same time resource, it could be SDM as well as FDM. Hence prefer to change FFS as follows: - Multiple beam index(es) and/or beam pattern per indication is supported to indicate multiple beams FFS: Whether multiple beams are applied in TDMed and/or at the same time resource. |
| InterDigital | We support Proposal 1-1-4 in general. |
| Qualcomm | OK with the proposal. But isn’t the 1st sub-bullet (single beam) a special case of the 2nd sub-bullet (multiple beams)? Do we really need to keep the 1st sub-bullet, if we have the 2nd one? |
| Sony | To focus the discussion, we prefer the modification suggested by Samsung. In addition, we believe the note is addressed by Proposal 1-1-3 and we do not need to discuss here again. So we propose the further modification: Proposal 1-1-4: The following methods are supported for beam indication: - Single beam index per indication is supported to indicate one beam. - Multiple beam index(es) and/or beam pattern per indication is supported to indicate multiple beams - FFS: Whether multiples beam are applied in TDMed or simultaneously over same/different frequency resource. Note: The indicated beam can be sub-set of supported beam by NCR-Fwd. |
| Philips | Okay with the proposal. |
| CATT | Ok now, maybe we can down-selection in the next step. |

Proposal 1-1-5: Confirm the WA that in access link, a DL beam and a UL beam which are correspondent with each other have the same beam index.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Fine to support |
| CMCC | Support. |
| Huawei, HiSilicon | Support |
| Ericsson | Support |
| LG | Fine with the proposal. |
| NTT DOCOMO | Support |
| Nokia | Support the proposal. |
| Intel | Support to confirm the WA. |
| AT&T | Support. |
| Vivo | OK |
| Spreadtrum | Support |
| ZTE | Support |
| Fujitsu | Support. |
| Lenovo | Fine with the proposal. |
| Samsung | Support. |
| Xiaomi | Support |
| Panasonic | Support |
| NEC | Support |
| CEWiT | Support |
| KDDI | Support |
| Sharp | Support |
| IIT-K | Support |
| InterDigital | We are ok with Proposal 1-1-5. |
| Qualcomm | Support. |
| Sony | Okay. |
| Philips | Support. |
| CATT | OK |

      - Indication of time domain information

Regarding the applicable time indication, following agreement has been made in RAN1#110:

Agreement The time domain resource corresponding to an access link beam can be determined with following options: - Option 1: Explicit determination based on the explicitly indicated the time domain resources per beam indication Note-1: Different parameters may be indicated for semi-static or dynamic beam indication Note 2: One or multiple beams can be indicated via single beam indication.

Then, in this meeting, to indicate the applicable time, following views are highlighted:

- For semi-static beam indication (e.g., periodic)

[Samsung, DCM, ZTE, Intel, LGE, Ericsson] propose that the slot index, starting symbols/slots [Intel, Ericsson], periodicity, duration per beam [ZTE, Intel, Ericsson], slot offset [NEC, MTK], can be considered. Multiple configuration with different value can also be supported.

- For dynamic indication

[Samsung] propose that a TDRA-like time domain resource indication is needed. [ZTE, Panasonic, Apple, ETRI (for symbol level)] propose to use the SLIV to indicate the start and duration for aperiodic beam indication, which is not preferred by [Ericsson]. [DCM, Intel] propose to configure multiple sets of time domain resource, e.g., via RRC configures, and the time domain information including {slot offset, starting symbol, number of consecutive symbols, number of slots} for one or multiple beams are defined. Slot pattern indication filed is preferred by [ETRI] for slot-level granularity.

However, [MTK] propose to indicate the duration for each beam index only. [CEWiT] also prefers to indicate the beam per slot for certain duration. [Ericsson] proposes that the dynamic indication represents a beam index to time resource mapping over a contiguous set of time resources and each time resource is a contiguous set of symbols within a slot (but do not span slot boundaries).

Moreover, [Huawei, Panasonic, Intel, Ericsson] propose to define the priority rule among different indication, e.g., periodic indication with the highest priority [Huawei], but [Intel] propose that dynamic indication overrides the semi-static indication. Indication of priority in the semi-static configuration to allow the repeater to know whether the semi-static configuration or a dynamic indication should be prioritized in case of a conflict is proposed by [Ericsson].

[Nokia, Panasonic, MTK, QC, ZTE] propose to define the minimum time required to apply the beam after the reception of a beam indication by NCR-MT as NCR capability. In addition, the solution including bitmap for non-consecutive applicable time indication is also proposed by single company.

Based on the above inputs, from FL’s perspective, there is convergence on the required parameter for each kind of indication. For the periodic indication, the traditional offset (either in slot or symbol level) is equivalent to the staring position as mentioned by others. For the aperiodic indication, at least the duration is common basis. Regarding the starting position, whether explicit indication is needed or not can be further discussed.

For the priority among different indication, the potential collision should be avoided by gNB’s implementation and if needed, following the principle in existing spec, the priority of AP indication is the highest.

For the application latency of beam, it’s reasonable to define it as NCR capability.

Then, followings are proposed:

Proposal 1-1-6: Following parameters should be supported to define the applicable time resource:

- For periodic time resource, staring positioning, duration per beam(s) and periodicity is needed.

- For aperiodic time resource, at least, duration per beam(s) is needed.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | It is not clear how aperiodic time resource can be indicated without the starting position. Both starting position and duration is needed. As discussed in our contribution, SLIV indication is the most straightforward method for time domain resource indication for access link beam |
| CMCC | We share the similar idea that the starting position should be also included for the aperiodic time resources. And the “duration per beam(s)” should be replaced by “duration of the beam(s)” to avoid the implication of the more specific design which should be in the next level discussion. |
| Huawei, HiSilicon | Since semi-persistent indication has been agreed, and we prefer to keep it in the proposal: Proposal 1-1-6: Following parameters should be supported to define the applicable time resource: - For periodic and semi-persistent time resource, staring positioning, duration per beam(s) and periodicity is needed. - For aperiodic time resource, at least, duration per beam(s) is needed. |
| Ericsson | It is unclear to us if this proposal refers to semi-static or dynamic signaling. We think that should be specified in the agreement since the signaling will quite likely differ. Furthermore, we think it should be made clear if one of multiple such configurations are applied. Finally, we think that we should also agree to prioritization. Hence, we propose the following alternative proposals: Semi-static: The following parameters should be supported to define the applicable time resource for a semi-static configuration: A set of starting times, a duration, a periodicity, a set of beam indices and a priority should be provided. The sets of starting times and beam indices are expected to be of the same size. The NCR may be provided with multiple beam configurations. FFS: range of above parameter values. Dynamic: The dynamic beam indication follows the DCI format 2_0/2_5 framework. |
| LG | Clarification is needed. We also think that time resource should be configured for the beam indication, however we would like to understand why starting position for aperiodic time resource is missing. In the case of periodic time resource, it is natural that the time resource is defined by starting position, duration, periodicity, and in the case of aperiodic time resource, the time resource is defined by the starting position and duration, excluding periodicity. If the starting position is excluded for aperiodic time resource, in order for the NCR to apply an aperiodic beam at a time desired by the gNB, the gNB must perform aperiodic beam indication at a designated time, for example, before a specific slot, which limits gNB’s scheduling flexibility. Therefore, we prefer time resource should be composed of starting position, duration, and periodicity. Last but not least, we think multiple sets of {starting position, duration}, and periodicity can be indicated for the applicable time resource for scheduling flexibility. Details of mapping between time resource sets and beam is discussed in proposal 1-1-9. |
| NTT DOCOMO | Support. Regarding Apple’s comment on “how aperiodic time resource can be indicated without the starting position”, our understanding is it may be predefined or semi-statically configured. |
| Nokia | Support Ericsson’s modified proposal. |
| Intel | We share similar view with Apple that starting position is also needed for aperiodic time resource. Is the proposal intended to leave it open that whether starting position is explicitly indicated or derived by pre-defined rule such as based on application latency? It is unclear to us how it works in case of multiple beams for different time resources, though it may work for single beam case but still quite restricted. We fail to see any benefit. |
| AT&T | Support the proposal with the Ericsson changes. |
| Spreadtrum | We also think starting position is needed for aperiodic time resource. |
| ZTE | We are prefer to indicate the starting time also for dynamic indication. For the periodic indication, it’s open to discuss either one more multiple set of parameters are supported. Proposal 1-1-6: Following parameters should be supported to define the applicable time resource: - For periodic and semi-persistent time resource, staring positioning, duration per beam(s) and periodicity is needed. - For aperiodic time resource, staring time and duration per beam(s) is needed. - Note: This parameter is to define the one set of applicable time resource. |
| Fujitsu | 1. We also think the starting position for aperiodic time resource is necessary. 2. We are not sure whether ‘starting positioning’ in the first has some special meaning compared with a starting position. 3. The ‘applicable time’ in the main bullet is unclear. We are afraid that the ‘applicable time’ may be confused with the application time after an indication for beam or ON/OFF (the time at which the NCR applies the indication). If the applicable time is exactly the time domain resource corresponding to an access link beam, we prefer to change it as what we used in previous agreements. If the application time has any additional meaning, a further clarification is preferable. |
| Lenovo | We prefer also to add “at least” for periodic time resource. The reason is that the periodic time resource may correspond to various periodic RS/channel in legacy release, such as SSB/PRACH/CSI-RS/SRS/SPS PDSCH/CG PUSCH/PDCCH. Different channel/RS may need some specific configurations, e.g. RO for PRACH transmission. |
| Samsung | For periodic part, we think slot index can also be used for the indication of time domain resource. Hence, we suggest a more general wording as follows: Proposal 1-1-6: Following parameters should be supported to define the applicable time resource: - For periodic time resource, staring positioning, duration time domain resource per beam(s) and the corresponding periodicity is needed. - For aperiodic time resource, at least, duration time domain resource per beam(s) is needed. - Note: Time domain resource is a set of symbols or slots. |
| Xiaomi | We think the staring position is necessary for aperiodic time resource. |
| Panasonic | Support Ericsson’s proposal. The indication should carry the priority as well. |
| NEC | Support. |
| CEWiT | For aperiodic indication, starting position is needed either in terms of offset or slot index. |
| China Telecom | We are generally fine with the proposal. Agree with Huawei, HiSilicon. |
| Sharp | Support in principle. |
| IIT-K | We think starting position is required for aperiodic indication. |
| InterDigital | In general, we support Proposal 1-1-6. We are fine with companies’ modified proposals for clarification. |
| Qualcomm | We support the modified proposal by ZTE. |
| Sony | Support ZTE’s modified proposal. |
| Philips | We support Ericsson's proposal. |
| CATT | We think there could be more options to define the periodic solution. Also the proposal seems not clear. |

Proposal 1-1-7: The minimum required time to apply the received beam indication by NCR-Fwd is NCR capability, which is defined as Y symbols after the last symbol of the HARQ-ACK in response to the beam indication received by NCR-MT.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Details of Y can be discussed later as currently it is not agreed if we support HARQ-ACK feedback or not |
| Huawei, HiSilicon | We don’t support the proposal. Based on the above discussion, it seems that the side control information can either be transmitted via PDCCH or PDSCH. It has not been agreed whether HARQ-ACK should be introduced for PDCCH. |
| Ericsson | Do not support. We think the gNB will be unnecessarily complicated if different NCRs have different decoding capabilities. We prefer to have the minimum beam indication application time as a specified value. Additionally, we do not think an agreement HARQ is motivated or even preferable in a repeater considering both the expectation of a robust backhaul link and a desirable low signaling latency. |
| LG | Okay for the capability report for required time. However we do not see further spec support since the network will configure based on the capability report, considering applied time and indication time. |
| NTT DOCOMO | Do not support. We think it is more reasonable that NCR capability of beam switching is defined as the gap between the signaling carrying beam indication and the applicable time. |
| Nokia | In general support, but agree with Apple and Huawei that details of capability should be discussed after further progress is made on signaling details related to beam indication. |
| Intel | We share similar view with Apple and HW we have not agreed to support HARQ-ACK feedback yet (discuss in 9.8.2) but the proposal is based on the assumption of HARQ-ACK feedback. |
| Vivo | If we discuss MT processing capability, we wonder the minimum required time is changed to “maximum required time” or “required time” The capability depends on the container of the SCI, and has nothing to do with HARQ-ACK. Feedback can be discussed separately. |
| Spreadtrum | Since the feedback of NCR has not been agreed, we think it is too early to define the beam application time. |
| ZTE | We prefer to support the feedback for the received SCI. Meanwhile, it’s safer to apply the indicated controlling information once the confirmation is received since all information is delivered over Uu and the reliability should be ensured. |
| Fujitsu | Do not support and share the same view with Ericsson. We also prefer to have the minimum beam indication application time as a specified value to avoid unnecessary complexity at gNB side. |
| Lenovo | We are fine for this minimum required time. However, we are not sure is should be after HARQ-ACK. We think this depends on the signaling carrying the beam indication. If it is by MAC CE, it is fine. If it is by DCI, there is no HARQ-ACK. We prefer to delete the “which…” part for now or keep it FFS. |
| Samsung | In our understanding, there is no discussion on whether beam indication (for example, for DCI based beam indication) is with or without HARQ-ACK information. Hence, we suggest to postpone this discussion until there is a common understanding on the HARQ-ACK of beam indication. |
| Xiaomi | This proposal seems discussing two things. One is NCR capability, the other is the application time of beam indication. We support the former one “ The minimum required time to apply the received beam indication by NCR-Fwd is NCR capability”. When beam indication is applied need further discussion. |
| Panasonic | Do not support. The timing of beam switching is related to the NCR-Fwd capability, while HARQ-ACK transmission is related to NCR-MT. The NCR-Fwd can initiate the beam changes after it receives the beam indication. The Y symbol should be defined as the symbols after receiving the beam indication. |
| NEC | We agree with DOCOMO. |
| CEWiT | Fine with the proposal. In our understanding HARQ feedback is essential in case of NCR to avoid performance degradation due to noise amplification and packet loss. |
| Sharp | Do not support. We share the view of Ericsson that a specific value can be define without further NCR capability signaling. |
| IIT-K | Support the proposal in general. Minimum time required to apply beam is required. And HARQ feedback for SCI is essential. However, setting minimum time to Y symbols after the HARQ feedback restricts operation of NCR-Fwd based on the timing of the HARQ feedback. Hence, it is better to keep it Y symbols after the beam indication is received by NCR-MT. |
| InterDigital | We support the first part of Proposal 1-1-7, that is: minimum required time to apply the received beam indication by NCR-Fwd is NCR capability. |
| Qualcomm | Do not support. We should leave out aspects related to HARQ feedback for now and attempt to get alignment on the first fundamental question that whether the application latency (that will be accurately defined later) is an NCR’s capability or a specified value. |
| Sony | Agree with companies comments that we can discuss first whether or not “The minimum required time to apply the received beam indication by NCR-Fwd is NCR capability.” Whether or not to support HARQ for beam indications can be discussed later. |
| CATT | This could be done by implementation. Not sure why we need to specify such value by RAN1 (ran4 could have test case ) |

Proposal 1-1-8: The NCR is not expected to receive the conflicted beam indication for access link between two indications over same resource.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Does the conflicted beam indication mean that two different beams cannot be indicated for the same resources? In our view, this is too stringent. For example, semi-static access link beam can be configured, but then dynamic indication can possibly update beam on the same resources. In this case, dynamic beam indication is prioritized |
| CMCC | We are general fine with the proposal. Open to further discussion. Some of the RRC configured beams and the corresponding time domain resources are used for the SSBs and other necessary information forwarding, which should not be overridden by the dynamic indications. And on the other side, both RRC configurations and dynamic indications are from gNB. From our understanding gNB should have the best knowledge and decision for the beam indications and configurations. |
| Huawei, HiSilicon | We prefer not to have the agreement now given that this may imply that some of the UL/DL cancellations cannot be supported by the UE served by NCR. |
| Ericsson | Do not support. This will make configurations and indications substantially more complex. For example, SSBs are configured every 20 ms, but a dynamic indication could be based on the TDD UL/DL pattern that is only 10 slots. Without allowing conflicts, indications would need to consider the two cases ‘with SSB’ and ‘without SSB’ separately. We propose to introduce priorities between semi-static and dynamic signaling for this reason. For dynamic signaling, a more recent indication should replace an earlier one. Our proposal is: Introduce priorities among semi-static beam configurations and dynamic beam indications. |
| LG | NCR may not expect conflicts within semi-static indications or within dynamic indications. However, conflict between semi-static indication and dynamic indication seems hard to be avoided. It is necessary to discuss which beam to apply when such conflict occurs, for example, default beam or priority of indication. |
| NTT DOCOMO | Do not support. Similar view with Apple that it is too restrict. We think the conflict can be resolved by defining some rules. It can be further discussed after more details on beam indication signaling are decided. |
| Nokia | Do not support. It seems that further details need to be agreed on the conditions under which multiple indications may be provided for the same time domain resource. |
| Intel | If two indications are both semi-static indications, or both dynamic indications, we’re fine. But if one is semi-static and another is dynamic indication, we share same view with Apple it is too stringent. If gNB does not want to override the semi-static indication, gNB can avoid indication of conflicted dynamic indication. But if gNB wants to override, prioritize dynamic beam indication if confliction happens is reasonable. |
| vivo | It is challenging to achieve this considering co-existence between dynamic indication and semi-static indication, especially considering the dynamic beam indication cannot be pre-planned. |
| Spreadtrum | We are not sure what the “conflict” means. If it means two different beams are indicated on the same resource by semi-static and dynamic indication respectively, we think the solution to handle the conflict needs to be further studied. |
| ZTE | Support. If there is collision, the dynamic should be prioritized. |
| Fujitsu | Do not support. This proposal may cause unnecessary limitation to the scheduling flexibility at the gNB side. We prefer to introduce some simple rules for the potential confliction between indications and the NCR can handle it by itself when any occurs. In this way, the scheduling flexibility for the gNB can be as same as legacy. |
| Lenovo | We don’t support this proposal. We think such kind of restriction will limit the flexibility for scheduling and indication of associated time resource for single or multiple beams. There is no such restriction in legacy release for SSB/PRACH/CSI-RS/SRS/PDCCH/PDSCH/PUCCH configuration. And contradiction resolution scheme is defined in legacy release for beam for different RS/channel, e.g. follow SSB beam when there are both SSB and CSI-RS in overlapped symbols. We think similar mechanism should be adopted. |
| Samsung | Not support. Share similar view with Intel. This imposes unnecessary restrictions on the flexibility of scheduling, for example, the use of dynamic beam indication to override the semi-static beam indication. |
| Xiaomi | We think it is hard to avoid the beam conflict, support to specify the priority between different beam indications. |
| Panasonic | The conflict of beam indications in time domain is inevitable with supporting both semi-static and dynamic indications. Therefore, we see to have some priority of the indication. Among the same priority, we agree NCR is not expected to receive the conflicted beam indication for access link between two indications over same resource. |
| NEC | We don’t support. Pre-defined rules or define priority to address confliction ensures more flexibility on scheduling. |
| CEWiT | Do not support. Priority among semi-static and dynamic beam indication should be defined based on the importance of the signal forwarded. E.g., semi-static configuration given for forwarding important signals (e.g., SSB) should be prioritized over dynamic indication. |
| China Telecom | We share a similar view with Apple. |
| Sharp | Do not support. The conflict situations cannot be fully avoided. |
| IIT-K | We don’t support this proposal. The conflict between semi-static and dynamic indication can arise, and priority needs to be configured between them. |
| InterDigital | We prefer not to have such agreement at this stage of the work. This concept can be discussed later when there is more clarification. |
| Qualcomm | Do not support. Better to allow more flexible signaling (to achieve more efficient and less-overhead signaling), but define rules/priorities to resolve the potential conflicts. |
| Sony | Do not support. It is not clear what “conflicting indications” are, given that multiple beam might be support in the same time and/or frequency resources. |
| Philips | Do not support. Unclear what constitutes a conflict. However, we agree with Apple’s point that a dynamic indication could over-ride a semi-static configuration. |
| CATT | Do not support. There should be conflict resolution defined |

In addition, the detailed solution on how to signalling the beam and corresponding applicable time is elaborated as:

[Samsung, LGE, ZTE, DCM, NEC, Huawei, Fujitsu] highlights that one-one mapping between beam index and time domain resource should be supported. More specifically, the {beam index(es), time domain resource}-pair is supported per indication [Samsung] and for dynamic indication, N pairs of {access link beam index, slot duration} is supported per indication [MTK]. [Fujitsu] propose that for semi-static indication, multiple configurations with each configuration at least includes parameters for periodicity, can be configured by RRC.

Meanwhile, [DCM] also supports to indicate one or more beam mapping to same resource periodically. [NEC] propose to support reuse the MCS, FDRA, and/or TCI state fields in legacy DCI format to dynamically indicate the access beam of NCR-Fwd. [Huawei] highlight that the forwarding occasion, forwarding resource and resource set should be defined as:

- Forwarding occasion is defined as the time that NCR-Fwd forwards signals with a same beam and amplifying gain. - A forwarding resource is defined by a resource index, a beam index, and time domain positions. - A forwarding resource set is defined by a resource set index and a list of forwarding resources.

For the definition of forwarding resource, the information including Resource type, e.g., periodic, semi-persistent, and aperiodic, Time granularity, OFDM position, slot position, and period should be considered.

Based on the above inputs, from FL’s perspective, the one-to-one mapping between beam and applicable time is supported by majority and jointly indication per indication seems to be preferred.

Then, following is proposed:

Proposal 1-1-9: The joint indication of beam and corresponding applicable time in one indication should be supported.

Note-1: The one-to-one mapping between beam and time resource is assumed.

Note-2: FFS on the joint indication, e.g., same or different bit-field(s) in one indication.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | It is not clear to us what joint indication here implies if different bit-fields are used. In our view, first we need to agree on the framework of time resources for beam indication, then we can discuss this proposal later |
| Huawei, HiSilicon | The proposal is not clear. If multiple OFDM symbols are indicated, does it mean “one time resource” or “multiple time resource”? |
| Ericsson | Need clarification. Does this apply to semi-static or dynamic signaling? Further, are there any assumptions on higher layer configurations? |
| LG | Regarding Note-1, we think it is premature since the time resource is not defined yet. For example, when one time resource consists of a multiple of consecutive slot/symbols and the same beam may be applied to several time resources, and in that case one-to-many mapping betweem beam and time resource can be considered. On the other hand, if the time resource can be defined by non-consecutive slot/symbols, one-to-one mapping may be sufficient. |
| NTT DOCOMO | Support the proposal. But for Note-2 we don’t see how and why beam and corresponding applicable time can be indicated by same field. |
| Nokia | In our view, time domain indication of beam state can not be uncoupled from time domain indication of the on-state of the NCR-Fwd. The indicated beam state of the NCR-Fwd should not be ambiguous for any time domain resource in which the NCR-Fwd is configured to be on. Our proposal in this regard is that indication of the time domain application of beam state should be designed jointly with the time domain application of NCR-Fwd on-off state. |
| Intel | We also think some clarification of one time resource is needed, otherwise, it is difficult to understand ‘one-to-one mapping’. And we share same view with Apple. In our understanding, joint indication means single bit field. It is unclear to us what is joint indication by separate bit fields. |
| vivo | If starting time resource is indicated by the SCI, no need to indicate the applicable time explicitly. This proposal can be jointly discussed with proposal 1-1-6 |
| ZTE | Support. The applicable time refers to all time instant, over which, the beam is applied. For the different field, for example, if the indication is done by DCI, these two can be indicated by different fields in DCI. Moreover, for RRC, it refers to the different IE in one RRC configuration. The following updated version is also fine. Proposal 1-1-9: The beam and corresponding applicable time should be indicated with same or different field(s) per indication. |
| Fujitsu | This proposal is unclear to us. Clarifications are needed. 1. What does ‘the one-to-one mapping’ exactly mean in note-1. 2. This proposal is for dynamic signaling? Semi-static/semi-persistent signaling? Or the design of dynamic signaling and semi-static signaling should be common? Besides, we have concern with ‘the application time’ like we commented in Proposal 1-1-6. |
| Lenovo | Fine with the proposal. |
| Samsung | We understand the intention of this proposal but some clarification is needed. The wording ‘joint indication’ should be clarified. For example, one indication includes a beam information and the corresponding applicable time. For the FFS part, we prefer separate bit-field(s) for beam and time domain resource due to simplicity. |
| Xiaomi | We support the main bullet of the proposal. |
| Panasonic | It should be clarified whether it is applicable to only dynamic indication or both dynamic/semi-static indications. |
| CEWiT | One to one mapping may not work always. E.g., in case of periodic signals, a single beam may be configured with a start time and periodicity. In that case one beam is mapped to multiple time resources. |
| Sharp | More clarification is needed on the type of indication, e.g. semi-static or dynamic, and resource definition. |
| IIT-K | Support the main proposal. However, the Note-1 of one-to-one mapping is not clear. Proposal 1-1-6 mentions periodicity for periodic time resource. Does the one-to-one mapping cater this scenario? |
| Qualcomm | The proposal is not clear. |
| Sony | This proposal seems to be related to signaling format. We suggest postponing its discussion until other proposals have been agreed upon. |
| Philips | Unclear what is being proposed here. |
| CATT | In general fine. |

For others,

[ZTE, Fujitsu] mention that for the time-domain resource indication and determination of the access link beam, the reference SCS for time domain resources indication of access link beam indication same as the reference SCS for UL/DL TDD configurations. Others including delicately indicated [Fujitsu] or scaled [ZTE] value can also be considered.

[IDC, ZTE] mentions that granularity of the time domain resource associated to indicate time domain resource should be indicated to NCR.

From FL’s perspective, it’s reasonable to consider these two aspect and companies are encouraged to share the views.

| Companies | Comments and Views |
| --- | --- |
| Huawei, HiSilicon | Support to indicate the time granularity. |
| Ericsson | It is too early to discuss the above in this meeting. |
| LG | To avoid misalignment of application time of side control information between gNB and NCR, they should have same understanding on the time granularity of indication. In that aspect, it will be necessary to configure the reference SCS when the side control information is indicated. |
| NTT DOCOMO | We think the reference SCS of the applicable time domain resources of beam indication needs to be discussed. |
| Intel | We agree that SCS for time domain resource indication should be specified. We can discuss details later. |
| ZTE | Fine with the proposal and also open to discuss it later |
| Fujitsu | Support further consideration on the reference SCS. |
| Lenovo | We agree that the time domain resource granularity should be indicated from gNB to NCR. Whether explicit or implicit indication can be FFS. |
| Samsung | In our view, the SCS of the time domain resource should be (explicitly or implicitly) provided by gNB. |
| NEC | The absolute length of slot/symbol should be aligned among gNB, UE and NCR. |
| CEWiT | Reference SCS and time domain granularity should be discussed |
| IIT-K | Support discussion of reference SCS and time domain granularity. |
| Qualcomm | We agree these two aspects should be discussed. |
| Sony | We can discuss this later on. |

    - Company view (Round-2)

      - Definition/acquisition of beam characteristic

According to the feedback, it seems that NCR capability is still preferred by majority, and in general, only one method is sufficient to exchange the required information. Then, from FL’s perspective, down-selection between OAM or capability via RRC is still needed.

For the detailed information, more companies are in general fine with original Proposal 1-1-1-B.

- Regarding the proposals to add new parameters:

It seems that the information related to the beam direction can also be considered, e.g., to ensure the gNB has knowledge on the potential region can be covered by NCR.  For the reporting of beam index, it’s not essential since it can be either reported by NCR or defined in other ways. The additional FFS seems sufficient.

- Regarding the concerns on how to organize the value, it can be discussed later with more detailed solution based on the agreed method for information delivering.

Then, following proposals are provided for the 2nd round discussion:

Proposal 1-1-1-A: NCR capability signalling via RRC is supported at least to characterize the beam information of NCR-Fwd for access link

- FFS: Details on beam information

Please noticed that this proposal is for NCR-Fwd and access link. And as common ground, the information changes is definitely needed. Companies are encouraged to share your views and other options if there is concern.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support the proposal |
| Intel | Support the proposal. |
| IIT-K | Support the proposal |
| CEWiT | Support |
| LG | Support. As mentioned by other companies in previous round, inter-vendor interoperability is important therefore the capability signaling is needed, not only supported by OAM. |
| NTT Docomo | Support |
| Xiaomi | Support |
| Lenovo | Support. |
| Vivo | Support |
| Fujitsu | Support |
| Huawei, HiSilicon | Our previous comment in the first round are not captured by the FL. We copied it below. @ZTE It is not true that “If no capability is defined, the only way is to assume that all information will be known via implementation.”. We are not against to define beam information of NCR-Fwd for access link, the gNB would need this information to operate NCR. However, we don’t think the capability reporting from NCR to gNB is reasonable as explained below. @ Intel, CMCC, LG Regarding inter-vendor operation, we don’t see an issue if the NCR capability is provided via OAM. There are already RF repeaters from multiple vendors in the field managed via OAM. @All We believe it is better RAN1 can reach a common understanding on how the new network node NCR is operated in a big picture. Q1: what information is needed at gNB to operate NCR? - We believe the set of beams including their beam characteristics that will be used for NCR-Fwd access link is necessary for the gNB. (There are other information as well but let us focus on the beamforming information first) Q2: How does the gNB acquire this information? - Option 1: gNB gets this information directly from OAM - Option 2: gNB gets NCR beamforming capability from reporting, and gNB gets the set of beams for NCR-Fwd access link via OAM - Option 3: gNB gets NCR beamforming capability from reporting, and gNB decides the set of beams for NCR-Fwd access link by itself In our view, Option 1 is straightforward given that NCR as a network node should be managed by the operator via OAM. The NCR are deployed to cover a certain target area. To achieve this, careful network planning is required. Otherwise, it may generate undesirable interference. There are other reasons why the beams used for NCR-Fwd access link should be provided via OAM. With the deployment of NCR, separate physical-layer resources (e.g. SSBs/SIB1/CSI-RS, etc.) allocated to NCR, which may impact the existing UEs served by the gNB. It is not possible for the gNB to provide all configurations for the NCR since it does not have the full information, e.g. target coverage area. Option 3 does not seem to be feasible since the gNB itself cannot determine the set of beams for NCR-Fwd access link based on the NCR capability information. With the capability reporting, gNB only knows what an NCR is capable of but does not know how to make use of it. The target coverage area is not known, it should be provided via OAM. Option 2 could work but the gNB still has to deliver the NCR beamforming capability to OAM so that the set of beams for the NCR-Fwd access link can be decided. Therefore, the NCR capability reporting procedure seems not really needed. Ideally, we can have some discussions on the above three options. If we cannot get a quickly converge, we propose to first agree that gNB can be provided with the set of beams including the information of beam characteristics for the NCR-Fwd to facilitate the rest of the discussions Having said above, we propose the following Proposal 1-1-1-A: NCR capability signalling via RRC is supported at least to characterize The gNB is provided with the beam information of NCR-Fwd for access link FFS: Details on beam information |
| Mediatek | Ok with the proposal |
| CATT | support with the proposal 1-1-1-A |
| Samsung | Support the proposal. |
| Ericsson | Support |
| Philips | Support the proposal |
| InterDigital | Support the proposal. |
| China Telecom | We generally fine with the proposal. |
| Sony | Support. |
| CMCC | We support the proposal. And our understanding is that this does not preclude the OAM based method for gNB and NCR to communicate for the beam related information. As mentioned by other companies, the mechanism based on OAM only is not preferred. Our understanding about the OAM system in the field is that, most macro cell base stations have their own OAM system. And there is no difficulty for the repeater to connect to the OAM system of the same vendor. But for the micro cell, small cell base stations and related the repeaters, the OAM system are not that strong or complete to have a control or communication about the beam information. This is the reason we want to introduce the capability reporting to improve the interoperability. |
| NEC | Support. |

Updated Proposal 1-1-1-B: At least the following information is used to characterize the physical beam(s) supported by NCR-Fwd for access link:

- Number of beams supported for access link

  - FFS: How to define the detailed value (e.g., per beam type)

- Beam types

  - Defined by the beam width (i.e., wide beam type or narrow beam type)

- Beam layout

  - Defined by the spatial relationship between different beams in same or different beam type

- Beam direction

  - Defined by the boresight of beam

- FFS: Other beam characteristics

- FFS: How to deliver this information to gNB by NCR

Companies are encouraged to share your views:

| Companies | Comments and Views |
| --- | --- |
| Apple | Some clarification questions: On beam types, is the intention to define specific beam width? In our view, if two beam types are defined i.e. wide and narrow beam, then beam width range should be sufficient On beam direction, is the intention to define the boresight of each beam? In our view, a reference beam’s boresight is sufficient and then based on the spatial relationship, direction of other beams can be determined |
| Intel | Thanks FL for providing more information for this proposal. We are fine to add beam direction to provide sufficient information for intended coverage. But we don’t agree to leave beam index as FFS. We think it is reasonable to add beam index, and we suggest to make the decision whether to support beam index reported by NCR now rather than leave it to FFS, because this has impact on proposal 1-1-2 or 1-1-3. NCR provides beam index together with beam type, beam layout and beam direction for each beam, something like shown below. By this way, both gNB and NCR can have common understanding on physical beam characteristic for each beam with a unique beam index. It is quite simple and straightforward, and then, we don’t need additional rule to derive the association between beam index and physical beam as in proposal 1-1-2. Beam index 0; Beam type, beam layout and beam direction information for beam index 0. Beam index 1; Beam type, beam layout and beam direction information for beam index 1. …. Beam index N-1; Beam type, beam layout and beam direction information for beam index N-1. |
| IIT-K | Support the proposal |
| CEWiT | Support |
| LG | It seems the main difference between current proposal and the previous version is beam direction, and we would like to understand the necessity of it. Since we think the beam indication based on the measurement of each of beams is sufficient, and physical direction based indication may lead misunderstanding between gNB and NCR in case the direction of NCR's antenna or panel is changed. So we do not think beam direction is needed. |
| NTT Docomo | Support in general. We have a clarification question regarding beam type and beam width. Is the intention to report two or multiple types where each type corresponds to a beam width or a range of beam width and each type corresponds to multiple beams, or is the intention to report beam width of every beam? |
| Xiaomi | It seems there are some conflicts by adding the “beam direction”. For example, if the beam direction and beam width for each beam is provided, the beam layout is not necessary. As the solutions might be various at the beginning, we support to keep all the sub-bullets, but the main bullet is suggested to update as “At least the following information is can be used to characterize the physical beam(s) supported by NCR-Fwd for access link: ” |
| Lenovo | Support. |
| ZTE | Support this proposal. |
| Moderator’s clarification | - Regarding the beam type: @DCM@Apple The first interpretation is correct. The intention is to only define the one or more multiple types based on width instead of reported the exact width per beam. Report the detailed value is not needed and is also hard to define it with finer granularity since it’s related to the different implementation. - Regarding the beam direction @Apple: whether the detailed direction per beam or reference can be further discussed. In general, for the GOB, single reference beam with detailed layout is also available. @LG: The report of direction, e.g., per beam or reference, can provide the information on the serving range of repeater for access link. This information seems beneficial. Otherwise, we may need to assume that specific beamforming is used in 3GPPs - Regarding the beam index @Intel: Whether and how to determine the beam index is also related to method to deliver the beam related information to gNB. The reported via NCR is also one of option in proposal 1-1-3. |
| vivo | OK |
| Fujitsu | Support in principle. Regarding to the boresight of beam, more clarification is necessary. We list following terminology from 38.106 for reference. Beam centre direction: direction equal to the geometric centre of the half-power contour of the beam Beam direction pair: data set consisting of the beam centre direction and the related beam peak direction Beam peak direction: direction where the maximum EIRP is found. |
| Huawei, HiSilicon | As also commented on the reflector, given that the NCR capability reporting is still pending, we suggest to focus on the beamforming information that will be provided to the NCR and characteristics of each beam for NCR-Fwd access link. In our understanding, “number of beams” doesn’t help to define the other related characteristic, e.g. spatial relationships and beam direction. It is better to using “set of beams” which can help to further define characteristics of each beam in the set, such as the direction for each beam and the relationship between different beams. We suggest the following update Updated Proposal 1-1-1-B: At least the following information is used to characterize the physical beam(s) supported by for NCR-Fwd for access link: - Number Set of beams supported for access link - FFS: How to define the detailed value (e.g., per beam type) - Beam types - Defined by the beam width (i.e., wide beam type or narrow beam type) - Beam layout - Defined by the spatial relationship between different beams in same or different beam type - Beam direction - Defined by the boresight of beam - FFS: Other beam characteristics - FFS: How to deliver this information to gNB by NCR |
| Mediatek | We are ok with the number of beams but have concern on the rest of beam-type related parameters. The parameters should be agreed as a whole package; it is premature to directly agree {beam type/width, beam layout, beam direction} as necessary parameters to describe beams and their relationship. Perhaps other than directly put FFS on them (although we prefer to), one way-forward is to list some options for further study and also check if it is feasible from RAN4’s perspective (We think RAN4 work is needed once such parameters are defined) and if there are alternative ways to describe those parameters to save overhead and allow more flexibility for beam design. For example, we are not sure if beam direction is really necessary. If the parameters can describe one wide beam has beam width overlapped with 4 narrow beams, we think it is not necessary to know the physical beam direction. Thus we suggest put FFS on the beam related parameters. |
| CATT | In principle ok but we share the view from Xiaomi. |
| Samsung | We share the similar understanding with Intel. Considering that so many beam features are reported in this proposal, a natural way is to use explicit beam index directly (with the associated beam features). Also, the beam index is unique for each beam feature. If so, there is no need to further discuss proposal 1-1-2. Here is the suggested modification: Updated Proposal 1-1-1-B: At least the following information is used to characterize the physical beam(s) supported by NCR-Fwd for access link: - Beam index - Number of beams supported for access link - FFS: How to define the detailed value (e.g., per beam type) - Beam types - Defined by the beam width (i.e., wide beam type or narrow beam type) - Beam layout - Defined by the spatial relationship between different beams in same or different beam type - Beam direction - Defined by the boresight of beam - FFS: Other beam characteristics - FFS: How to deliver this information to gNB by NCR |
| Ericsson | In our understanding, with the assumptions we are working with, including different beam types, it is reasonable to assume the approximately same coverage area (say 120x30 degrees) for all beam types (narrow, wide, etc) and their respective beam constellations (layout) (say 4x2). In that case, what is needed is the coverage area itself and the beam constellations for the respective beam type. We don’t think all information in the FL’s proposal will be as useful as companies expect. For example, the relative beam locations are what matters in measurements, not the exact beam directions since the gNB needs to manage with the beams that the NCR has. Similarly, the number of beams may not be needed if the beam constellation/layout is provided. However, for the sake of progress, we can agree to the above proposal. |
| Philips | Support the proposal |
| InterDigital | We are not ok with the indication of beam direction as basically it is not required at gNB. Moreover, the NCR may have the capability to change access link beam direction, and this may cause ambiguities or extra overhead in reporting the direction each time it is changed. Indicating the number of the beams, beamwidth, and beams arrangement in the context of relative spatial relationship is what is necessary to be indicated to gNB for further scheduling and respective procedures. |
| Sony | We share the concern of LG and Xiaomi that the need for “beam direction” information has not been motivated. In particular, for the purpose of beam management, which benefit does add beam information on top of the beam layout information? Also, the definition of spatial relationship is not clear. What does it mean for two beams of the same type to have a spatial relationship? Are they adjacent? And if they are of different types, is the narrower one contained within the wider one? In our opinion, this should be spelled out. |
| CMCC | We are fine with the first 3 bullets. For the beam direction, although we have the same understandings that beam direction is useful. But it could be also covered by the beam layout. For the illustration of beam layout, we will not only state that narrow beam 1,2,3 will share a wide beam A. But we should also say there will be wide beams A, B and C which are next to each other and provide the basic coverage in horizontal. Thus, our view is that the beam direction could be covered by the beam layout. And for the more detailed boresight of the beam, we are not sure the absolute values of the boresight direction will provide more information for the beam management. The relative position between the beams could help the beam management for the AC link. |
| NEC | Support in general for both Proposal 1-1-1-B and 1-1-1-B-v2. We think a clarification is needed to point out that whether the wide beam and narrow beam are relative concepts or concepts with absolute width. |

        - Summary of 2nd round discussion

For Proposal 1-1-1-A, the situation is same, companies are encouraged to check the replies from HW.

For Proposal 1-1-1-B, considering the comments from companies, the following updated is proposed:

Updated Proposal 1-1-1-B_v2: At least the following information is used to characterize the physical beam(s) supported by NCR-Fwd for access link:

- Number of beams supported for access link

  - FFS: How to define the detailed value (e.g., per beam type)

  - FFS: Whether the number of beam can be derived by beam layout

- Beam types defined by the beam width (i.e., two types as wide beam and narrow beam type)

- Beam layout defined by the spatial relationship between different beams in same or different beam type

- FFS: Beam direction defined by the boresight of beam

- FFS: How to deliver this information to gNB

      - Association between beam index and real beam

For the definition of association between beam index and physical beam, companies are in general fine with Proposal 1-1-2.

Regarding some clarification question:

- For Option-1: It’s clear that if all beams are indexed sequentially regardless of beam type, different indices are expected per beam, e.g., 0~N-1 if N beams (e.g., narrow or wide beam) are supported.

- For Option-2: The intention is to index the beam per type. For example, 0~M-1 is used for wide beam and 0~L-1 for narrow beam. Further distinguish between beam type can also be considered.

- For Option-3: With assumption of the support of multiple beam types and association, the beam can be indexed hierarchically. For example, the narrow beam (e.g., 0-Y-1 for Y narrow beams belong to same wide beam).

Then, the following updated proposal can be considered:

Updated Proposal 1-1-2: One of following options is supported to define the association between beam index and the physical beam supported by NCR-Fwd for access link:

- Option-1: All beams are indexed sequentially regardless of the beam type for each beam (i.e., different beam index per beam)

- Option-2: Beams are independently indexed per beam type(i.e., all narrow/wide beams belong to the same type are indexed sequentially)

- Option-3: Beams are hierarchically indexed according to the spatial relationship between different beam types (i.e., narrow beams associated with the same wide beam are sequentially indexed, and different wide beams are sequentially indexed)

FFS: the sequence of beam

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | In our view, the beam layout definition in the previous proposal includes this aspect. Nevertheless, if the intention is to describe different options here, then it is fine. In terms of options, our preference would be option-2 or option-3 |
| Intel | Whether we need this proposal highly depends on whether we support beam index report by NCR in proposal 1-1-1-B. If beam index is reported by NCR, we don’t see the necessity for this proposal. We suggest to first identify the necessity for this proposal before discussing which option is preferred. |
| IIT-K | We prefer Option-1 or Option-2 as they are straightforward. |
| CEWiT | Support. Prefer option 1. |
| LG | We are fine with the options itself and our preference is option 1 but have comment on the bracket inside of option 3. It seems unclear to me what the example in bracket of option 3 is, and we haven't discussed the detail of it yet, so we think the detailed relationship could be left as FFS. Or at least change i.e. to e.g. |
| NTT Docomo | Support. But we think there is similar issue as proposal 1-1-3 on the meaning of “beam index”. Is the beam index used in a side control information for beam control? Or is it used for some configuration (e.g. configure a set/subset of beams) before the beam indication in SCI? Or is it used for capability report of NCR in proposal 1-1-1? With different function of “beam index”, different option may be preferred. |
| Xiaomi | Option 1 is preferred among three options. |
| Lenovo | Our first preference is option 2 and second preference is option 3. To us, signaling overhead of option 1 is a bit large. |
| ZTE | Support this proposal. We prefer to take Option-3. |
| Vivo | Prefer option 1, which is straightforward |
| Fujitsu | Support the proposal in principle and prefer Option 1. But we still feel confusing about ‘sequentially’ in option 1. Does it mean that some predefined rule must be defined if option 1 is adopted? If this is the case, we think the predefined rule of the sequence may or may not be necessary. It may depend on how the capability described in Proposal 1-1-1 is reported. If the gNB knows the beam number, beam type and beam layout, the beams can be indexed in any sequence. There would be no ambiguity between the NCR and the gNB. The only case that the sequence needs to be defined is some capability information listed in 1-1-1 are implicitly carried by the index sequence. |
| Huawei, HiSilicon | Isn’t it option 3 a special case of option 1? For Option 2, as the FL mentioned, the indexing is within a beam type and one would need to determine the beam type somehow. Besides, we are also wondering why there is a need to discuss the indexing of all physical beams supported by NCR. It will be sufficient to focus on the indexing of the set of beams configured for NCR-Fwd access link, which minimizes the signaling overhead. Thus, we propose the following: Updated Proposal 1-1-2: One of following options is supported to define the association between beam index and the physical beam supported by NCR-Fwd for access link: - Option-1: All beams are indexed sequentially regardless of the beam type for each beam (i.e., different beam index per beam) - Option-2: Beams are independently indexed per beam type(i.e., all narrow/wide beams belong to the same type are indexed sequentially) - Option-3: Beams are hierarchically indexed according to the spatial relationship between different beam types (i.e., narrow beams associated with the same wide beam are sequentially indexed, and different wide beams are sequentially indexed) - Option-4: the set of beams configured for NCR-Fwd access link are indexed sequentially regardless of the beam type for each beam (i.e., different beam index per beam) FFS: the sequence of beam |
| Mediatek | Not support. This proposal is highly dependent on how RAN1/RAN4 defines beam types and the relation among the beam types. Given that beam types and the relation among the beam types are still unclear, we cannot agree this proposal. |
| Samsung | Prefer option 1. If explicit beam index is conveyed NCR capability, the discussion of this proposal seems unnecessary (i.e., option 1 is automatically adopted). |
| Ericsson | Support, prefer Option 2. As presented in our comment to Updated Proposal 1-1-1-B above, if beam constellations/layouts are supported, it would be beneficial to have a predefined beam order within the constellations. In that case, indexing among the beams within the same constellation would be very simple. |
| Philips | We prefer options 2 or 3. |
| InterDigital | Support the proposal. Our preferences are Options 2 and 3. |
| China Telecom | We generally fine with the proposal. It seems the proposal is similar with the concept of beam layout in previous proposal. We slightly prefer Option 2 and 3. |
| CMCC | We are fine with proposal. If the narrow beam and width beam (different beam types) are supported and there are overlapping between the narrow beams and wide beams, option 2 and 3 could provide more information about the relationship between the multiple beams. Option 2 and 3 are preferred. |
| NEC | Support. We prefer option 3. |

For Proposal 1-1-3, it seems that multiple interpretations are proposed regarding the meaning of beam index. From FL’s perspective, this confusion is mainly due to the potential enhancement, e.g., whether all or sub-set of beams will be used for access link based on the configuration/re-configurtion from Gnb. More specifically, on this aspect, two ways can be considered and covered by Option-1:

- Solution-1: Based on the information on beam characteristic, the Gnb will inform the used beam with corresponding index, i.e., the index will be initially done by Gnb.

- Solution-2: Among the beam with corresponding index determined by either pre-defined rule or reported by NCR, the Gnb will select some of them for further indication. In this case, some companies propose to re-index the beam to reduce the overhead, e.g., N supported beams with corresponding index is known at Gnb side, and Gnb will inform NCR further that only X out of N will be used.

For these two solutions, the further information exchange between Gnb and NCR should be expected, and the details can be further discussion.

More specifically, for Option-1 if index is initially done by Gnb, Option-2 and Option-3, the association principle defined in Updated Proposal 1-1-2 will be applied.

Then, the following updated proposal can be considered:

Updated Proposal 1-1-3: One of following options is supported to determine the beam index used in SCI to control the beam of NCR-Fwd for access link:

- Option-1: The beam indices are configured by Gnb

  - FFS: How to inform the NCR on which beams are selected and/or indexed by Gnb

- Option-2: The beam indices are reported by NCR

  - Note: In this option, the reported beam index of supported beam will be used for beam indication.

- Option-3: The beam indices are determined by the pre-defined rule.

  - Note: In this option, same rule is applied at both Gnb and NCR sides to identify the unique physical beam supported by NCR for each beam index.

Note: Combination between Options is not precluded.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support the proposal and prefer option-1 |
| Intel | We support option 1. In our understanding, at least option 1 and option 2 can work without proposal 1-1-2, if beam index is reported by NCR. For option 3, we’d like to clarify that the pre-defined rule is the rule in proposal 1-1-2? |
| IIT-K | Prefer Option-1 |
| CEWiT | Support Proposal. Prefer option 1 |
| LG | As we discussed in the previous round, we think it is desirable that only subset of supported beams are used for indication of access link beam, considering overhead of indication. Using all of the supported beam for access link for indication seems quite waste of the overhead and redundant to us. Considering that, we support option 1. And in our understanding, option 2 & 3 are not applicable it. Furthermore it is natural for Gnb to configure those beams since the measurement result is known to Gnb, not NCR since it is network controlled repeater, not network informed repeater. |
| NTT Docomo | Support and prefer option1. |
| Xiaomi | Option 1 is preferred among three options. |
| Lenovo | Our understanding is that beam indication signaling overhead will be reduced by option 1, however, there will be additional signaling to indicate which beam indices is selected by the Gnb for indexing. Considering the tradeoff, we slightly prefer option 3. |
| ZTE | Support this proposal. |
| Moderator’s clarification | As highlighted in the summary: From FL’s perspective, in both Option-2 and 3, the principle should be defined and only difference between these two whether to introduce the beam index in the report from NCR. For Option-1, even the beam index is configured by Gnb, the corresponding principle should also be known at NCR side. On top of this common ground for beam index, if there is needs to re-configure or select some set of beam, maybe localized numbering can be considered. So, the following is mentioned in the summary above : “More specifically, for Option-1 if index is initially done by Gnb, Option-2 and Option-3, the association principle defined in Updated Proposal 1-1-2 will be applied.” |
| Vivo | Absolute beam index can be reported by NCR (option 2) or defined based on rule (option 3), but the beam index indicated by SCI can be subset of all the beams, which can be configured by Gnb (option 1). We think at least combination of option 1+option 2, or option 1+ option 3 should be used. |
| Fujitsu | Support the proposal. We think re-index is not necessary to consider now. It is kind of things like the mapping relation between the indication field bits and the beam index. |
| Huawei, HiSilicon | Support and prefer option-1 |
| Mediatek | Fine with the principle of the proposal and prefer option 1 to enable smaller beams configured for NCR than the beams reported by NCR. |
| Ericsson | Support, prefer Option 3. If the beam constellations/layouts follow a predefined rule, the beam indices should also follow a pre-defined rule. |
| Philips | Support the proposal. Preference for options 2 or 3. |
| InterDigital | Support the proposal and we think some combination of options could be used. |
| China Telecom | Support and slightly prefer Option1. |
| Sony | We have a question on Option 3. Can the proponents of Option 3 provide an example of a pre-defined rule to determine the beam indices? |
| CMCC | If the beam layout of the NCR AC beam could be informed to gNB clearly, then both option 1 and option 2 could work. Our preference is option 2 and option 3. If a rule could be defined, then both gNB and NCR could have then same understanding of the beam indices. |
| NEC | We support option 1 and option 3. These two options can be applied for different scenarios. |

        - Summary of 2nd round discussion

Majority is fine with original proposal and regarding the remaining concerns, discussion over GTW is more preferred.

      - Indication of beam index

For the Proposal 1-1-4, majority is fine with proposals and some updates are proposed.  From FL’s perspective, the indication of single beam per indication can be applied for all kind of indication, e.g., dynamic or semi-static. For semi-static indication, for example, the one beam can be indicated to cover the potential SPS behavior of served UE. For the indication of multiple beam, additionally, the beam pattern is proposed as one way.

Moreover, regarding the per frequency resource indication, this is proposed in section-4 and companies are encouraged to check it accordingly.

Updated Proposal 1-1-4: The following methods are supported for beam indication:

- Single beam index per indication is supported to indicate one beam.

  - Note: The single beam indication is supported at least for dynamic indication

- The indication of multiple beam in one indication is supported via one of following options:

  - Option-1: Multiple beam indices

  - Option-2: Beam pattern index, where the each of the beam pattern is defined as a list of beam indices.

  - Note: The multiples beam are applied in TDMed over same frequency resource

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | For option-1, beam pattern may not be suitable as it somehow implies the physical pattern of beams. In our view, the intention is to indicate a sequence of beams via an index. So option-2 could be updates as: - Option-2: Beams pattern sequence index, where the each of the beam pattern sequence is defined as a list of beam indices. |
| Intel | We’d like to further check our understanding for option 1 and option 2. Assume gNB configures 4 beams, beam 0/1/2/3, and if gNB indicates 3 time domain resources by a side control information, Option 1 means, there are 2 bits for beam index for each time domain resource, so totally 6 bits ? 2 bits for 1st time domain resource; 2 bits for 2nd time domain resource; 2 bits for 3rd time domain resource. Option 2 means, gNB configures multiple lists of beams, beams for multiple time domain resources are within one list, and gNB indicates list index ？The number of bits is log2 (N). If yes, we think Apple’s wording is better. Beam list 1; {beam i1, beam i2,… beami4}. Beam list 2; {beam j1, beam j2,… beamj4}. …; …. Beam list N; {beam n1, beam n2 }. |
| IIT-K | Support the proposal. |
| CEWiT | We have following concerns regarding multiples beam are applied in TDMed over same frequency resource in the note other than TDM, multiple beams can also operate simultaneously regarding same frequency resource, NCR-Fwd can operate over multiple subbands and beam indication can be for different subbands as well. We suggest Note: The multiples beam are applied in TDMed or simultaneously over same or different frequency resources |
| LG | We think the indication of single beam per indication can be applied for all kind of indication, e.g., dynamic or semi-static. In that perspective, following modification is suggested: Updated Proposal 1-1-4’: The following methods are supported for beam indication: - Single beam index per indication is supported to indicate one beam. - Note: The single beam indication is supported at least for dynamic indication - The indication of multiple beam in one indication is supported via one of following options: - Option-1: Multiple beam indices - Option-2: Beam pattern index, where the each of the beam pattern is defined as a list of beam indices. Note: The multiples beam are applied in TDMed over same frequency resource |
| NTT Docomo | Support |
| Xiaomi | For the multiple beam indication, we have similar feeling with Apple that beam pattern implies time domain information besides the beam index. We are fine with Apples’ update. |
| Lenovo | We are generally fine with the proposal. Regarding the proposal of Apple, we prefer “beam pattern index” or “beam set index” than “beam sequence index”. Regarding the Note, we think it is a bit restrictive. TDM with frequency hopping can’t be covered. FDMed case is precluded as well. How about we change it to “Note: The multiples beam are applied at least in TDMed way over same frequency resource”? Then we can further discuss the frequency hopping case and FDM/SDM case? |
| ZTE | Support this proposal and SDM or sub-band operation can be discussed later once the basic function is concluded since it’s also related to the RF architecture discussion in RAN4. |
| Moderator’s clarification | @Intel: Yes, for multiple indication, the difference is whether to allow the pre-confgiured pattern or sequence before the indication. |
| Vivo | For multipe beam indication, both option 1 and option 2 can be applied. For CSI-RS beam indication, option 1 can be used, for PDSCH beam indication, option 2 can be used, a pattern is used only when gNB have scheduling plan according to a given pattern. |
| Fujitsu | Support. |
| Huawei, HiSilicon | We think the definition of beam indication is not quite clear. There are periodic, semi-persistent and dynamic signals/channels. The beam indication for different signals/channels can also be different. For dynamic signals/channels, some semi-statically configured “beam patterns” or “beam lists” are needed. From this perspective, we think option 1 under the second subbullet seems like a more generic description while Option 2 contains more signaling design details. |
| Mediatek | Based on Apple’s proposal, we think option 2 can be further modified as follows to avoid any confusions - Option-2: Preconfigured beams sequence index, where the each of the beam sequence is defined as a list of beam indices. |
| Samsung | Support the proposal in general and prefer option 1. In our view, the note for single beam index part is unnecessary. Also, some modifications are suggested: Updated Proposal 1-1-4: The following methods are supported for beam indication: - Single beam index per indication is supported to indicate one beam. - Note: The single beam indication is supported at least for dynamic indication - The indication of multiple beam in one indication is supported via one of following options: - Option-1: Multiple beam indices - Option-2: Beam pattern index, where the each of the beam pattern is defined as a list of beam indices. - Note: The multiples beam are applied in TDMed over same frequency resource - Note: that each indication can be applied to multiple time resources. - FFS: whether to support multiple beams simultaneously in same/different frequency resources. |
| Ericsson | Support. For semi-static configuration of multiple beams, we prefer Option 1, since it is unnecessary to provide beam patterns for semi-static configuration. For dynamic indication, we think beam pattens provided by higher layers will be beneficial and hence we prefer Option 2 for dynamic indication of multiple beams. |
| Philips | We support the proposal with Mediatek's and Apple's modifications. |
| InterDigital | Support the proposal with Apple’s version for Option 2. |
| China Telecom | Support |
| CMCC | We are fine with the 1st bullet. But for the 2nd bullet, option 1 and option 2 are not excluded. Multiple beam indices could be used for e.g. RRC configurations to cover the periodical beam indications. And for the option 2, a list of beam indices may not be 100% correct or complete. As the note say the beams or beam indices would be transmitted in a TDM manner. There would be some time slot between the beams or beams with the indices where no data forwarding is carried out. Then only a list of beam indices may not enough. It could be a beam pattern with specific time slot information or time domain resource information. That is our understanding. |
| NEC | Support both the two proposals (1-1-5 and 1-1-5-v2) in general. A note to include both options can be added to the second bullet for multiple beam indication. The first option is more flexible with larger overhead while the second one is just the opposite. We think both of the two can be supported by considering configuration of different types. |

For Proposal 1-1-5, all companies are fine to this proposal. It can be either endorsed via GTW or email approval. No additional discussion is needed.

        - Summary of 2nd round discussion

For the Updated Proposal 1-1-5, updates to refine the wording based on the comments is listed. For the operation in SDM or sub-band, as highlighted before, it will be discussed later and also required the further check with RAN4.

Then, following is proposed:

Updated Proposal 1-1-4_v2: The following methods are supported for beam indication:

- Single beam index per indication is supported to indicate one beam.

- The indication of multiple beam in one indication is supported via one of following options:

  - Option-1: Multiple beam indices

  - Option-2: Beam sequence index, where the beam sequence is pre-configured and each one is defined as a list of beam indices.

  - Note: The multiples beam are applied in TDMed over same frequency resource

      - Indication of time domain information

For Proposal 1-1-6, based on the inputs, following updated proposal is considered to address the comments from companies:

Updated Proposal 1-1-6: Following parameters should be supported to define the time resource:

- For periodic and semi-persistent time resource, staring time, duration per beam(s) and periodicity is needed.

- For aperiodic time resource, staring time and duration per beam(s) is needed.

- FFS: How to define the duration, e.g., the length of time resource or resource index

Note: This parameter is to define the one set of time resource.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support the proposal |
| Intel | We support the proposal |
| IIT-K | Support the proposal |
| CEWiT | Support |
| LG | Support the proposal. |
| NTT Docomo | Support |
| Xiaomi | Support |
| Lenovo | We are generally fine with the proposal and prefer to update the note as “Note: This parameter is to define the one set of time resource and there can be multiple sets of time resources.” |
| ZTE | Support. If there is need to configure multiple set of information, same parameters in same format is used. |
| vivo | When indicating multiple beams, both the duration per beam and the total time length of the beam indication should be used. We wonder whether this is reflected by duration per beam(s) |
| Fujitsu | Support the proposal |
| Huawei, HiSilicon | Support Not sure about the note, which parameter is referred here? |
| Samsung | Support the proposal in general. Some modification of the FFS part. Updated Proposal 1-1-6: Following parameters should be supported to define the time resource: - For periodic and semi-persistent time resource, staring time, duration per beam(s) and periodicity is needed. - For aperiodic time resource, staring time and duration per beam(s) is needed. - FFS: How to define the duration, e.g., the length of time resource or resource index, a set/list of resource indexes (e.g., a set of slots) - FFS: whether indication of starting time can be implicit (e.g., the first slot after the time to apply the received beam indication) - FFS: the SCS for starting time, duration, and periodicity Note: This parameter is to define the one set of time resource. |
| Ericsson | We would prefer if the proposal said “For semi-static and semi-persistent configurations”, to avoid confusion. There are no semi-static or dynamic time resources but only semi-static configurations or dynamic indications of resources. Further, we do not agree that the parameter starting time and duration need to be supported. The corresponding information, yes, but that can be provided in other ways than a parameter. Our revised proposal is then: The following information should be (explicitly or implicitly) provided to define a time resource of a beam: - For a semi-static or semi-persistent configuration: staring time, duration per beam(s) and periodicity. - For a dynamic indication: staring time and duration per beam(s). - FFS: How to define the duration, e.g., the length of time resource or resource index. |
| Philips | Support. Change 'staring' to 'starting' in the first and second bullets. |
| InterDigital | Support the proposal |
| China Telecom | Support. |
| Sony | Support. |
| CMCC | Fine with the proposal. As mentioned by Philips, some typos need update. Since semi-persistent configurations would be supported, more parameters may be needed, e.g. the duration of the semi-persistent transmission. So the listed parameters may not be complete, an “at least” could be added in the main bullet or somewhere. |
| NEC | Support the updated version 1-1-6-v2. |

For Proposal 1-1-7, it seems that there is common understanding to define the corresponding capability and the feedback related will be treated later. Then, following updated is proposed:

Updated Proposal 1-1-7: The minimum required time to apply the received beam indication by NCR-Fwd is NCR capability.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support the proposal |
| Intel | We support the proposal. One clarification question, if NCR beam information report is unfortunately only supported by OAM, whether all NCR-Fwd relevant capability (e.g., beamforming and on/off) is also only supported by OAM, or we will discuss case by case ? |
| IIT-K | Support the proposal |
| CEWiT | Support |
| LG | Fine for the proposal itself, to report the required time for beam application as capability. |
| NTT Docomo | Support |
| Xiaomi | Support |
| Lenovo | Support. |
| ZTE | Support |
| Moderator’s clarification | @Intel: Case by case is preferred since this information is more relevant to the scheduling. We can discuss it based on majority’s view. |
| vivo | We wonder why maximum value is not reported, do you assume legacy DCI processing time is assumed to bound the maximum required time? |
| Fujitsu | Do not support as we explained in round 1. We don’t see the necessity to support this. |
| Huawei, HiSilicon | We disagree with the comments from intel that beam information report supported by OAM is unfortunate. If you look at the network devices, this is the normal case. Even for IAB node, we only define capabilities for IAB-MT. Then regarding this minimum required time, we prefer not to introduce capability reporting which will complicate the gNB handling of different NCR implementations. Updated Proposal 1-1-7: The minimum required time to apply the received beam indication by NCR-Fwd is NCR capability. Note: The above does not imply capability reporting. |
| Mediatek | Support in principle. A typo: staring -> starting. |
| Samsung | Support the proposal in general. Some wording suggestion as below: Updated Proposal 1-1-7: The minimum required time to apply the received beam indication by NCR-Fwd is subject to NCR capability. |
| Ericsson | Do not support. This should not be a capability but instead a specified value. Having a gNB serving repeaters with different capabilities in this respect is highly undesirable and will only reduce the functionality of the repeater. |
| Philips | Tend to agree with Ericsson. |
| InterDigital | Support the proposal. |
| China Telecom | Support. |
| Sony | Support. |
| CMCC | We are fine to define this as an capability which should be reported to the gNB. We have the same understanding as Ericsson that if we can put some requirements for the process time of NCR to improve the efficiency. But if not all the vendors can fulfil the most high-level requirements, a limited capabilities could also put some requirements to the medium level NCR’s. And the capability reporting could facilitate the scheduling decisions of gNB. |

        - Summary of 2nd round discussion

For the Updated Proposal 1-1-6, updated version is provided considering the inputs:

Updated Proposal 1-1-6_v2: Following parameters should be supported to define the time resource:

- For periodic and semi-persistent configuration of time resource, starting time, duration per beam(s) and periodicity is needed.

- For aperiodic indication of time resource, starting time and duration per beam(s) is needed.

- FFS: the SCS for starting time, duration, and periodicity

- FFS: How to define the duration, e.g., via the length of time resource or resource index

- FFS: whether indication of starting time can be implicit (e.g., the first slot after the time to apply the received beam indication)

For the Updated Proposal 1-1-7, considering the concern from companies, e.g., should not be capability but a specified value, companies are encouraged to further check the impacts. This issue will be handled later once the generic signalling is consolidated.

  - Beam indication for backhaul link

    - Company view (Round-1)

Regarding the beam indication for backhaul link, following agreement has been achieved in RAN1#110:

Agreement In case that adaptive beams are adopted for C-link and backhaul link, the following mechanisms can be considered for the indication and determination of beams of backhaul link: - Option 1: The beam of backhaul link is indicated by a new signaling. - The new signaling is dynamic signaling and/or semi-static signaling (e.g., RRC signaling/ MAC CE) indicating a beam(s) from the set of beams of the C-link - This does not imply that the beam of backhaul link is always indicated by the new signaling - Option 2: The beam of backhaul link is determined by a pre-defined rule. - In slots/symbols with simultaneous DL receptions / UL transmissions in both C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link. Otherwise, the beam of backhaul link follows one of the beams of the C link. - Other predefined rules are not precluded

In this meeting, following views are shared by companies:

- [Huawei, vivo, ZTE, IDC, Intel, AT&T, FirstNet, Sony, NEC, CMCC, MTK, Apple, LGE, Lenovo] support Option-1 to indicate the beam for backhaul link via a new signalling.

More specifically, same beam should be indicated if simultaneous operation is performed between C-link and backhaul link as mentioned by [Huawei]. [IDC] highlights that the legacy procedure for beam management and indication are applied, and using separate beam indication at NCR-MT and NCR-FWD, for C-link and BH-Link, respectively.

Meanwhile, regarding the detailed signalling, as mentioned by [Huawei, vivo], the TCI based signalling via DCI, MAC-CE and RRC are supported to indicate the beam. [ZTE] mentions that the “set of beams of the C-link” refers to the RRC configured or MAC CE indicated TCI set for NCR-MT. [AT&T, FirstNet, Sony] propose that a  semi-static signalling (e.g., MAC CE) can be considered to indicate a beam(s) from the set of beams of the C-link. [NEC] propose to support additional beam indication for backhaul beam of NCR-Fwd, based on the relationship between beams of control link and beams of backhaul link. [CMCC] mentions that the self-interference issue should be considered for the beam determination and indication of BH link.

- [Spreadtrum, Nokia, CAICT, Xiaomi, MTK, Sharp, Samsung, DCM, Ericsson] support Option-2 to implicitly determine the beam by a pre-defined.

More specifically, [Xiaomi] NCR follow the TCI state of the latest received PDCCH, if unified TCI is not configured; NCR follow the TCI state of the latest received unified TCI, if unified TCI is configured. [Samsung] mentions that for DL, NCR-Fwd follows the activated TCI state with lowest ID for PDSCH transmission of NCR-MT and for UL, NCR-Fwd follows the spatial relation for dedicated PUCCH resource with lowest ID of NCR-MT.  [DCM] proposes that in slots/symbols with simultaneous operation in C-link and backhaul link, the beam of backhaul link is the same as the beam of C-link. In slots/symbols with only backhaul link, when Rel-17 unified TCI framework is used for NCR-MT C-link, the indicated unified TCI for C-link can be used for backhaul link. [Ericsson] also highlights that unless the repeater-MT is simultaneously operating, the repeater-Fwd uses a predefined rule to apply the repeater-MT’s last indicated PDSCH TCI state for the backhaul link.

Moreover, [QC] propose to support a new signaling to optionally indicate backhaul link beam. [KDDI, CEWiT] mentions that the beam of backhaul link of the NCR-Fwd used in a slot/symbol is determined according to a new signaling if the indication for the beam of backhaul link received by the new signaling is valid in the slot/symbol. Otherwise, it is determined by a pre-defined rule.

- In addition, [Intel, Samsung, LGE] highlights that clarification on whether the simultaneous UL transmission of C-link and backhaul link, whether to support the adaptive beam for backhaul/C-link, is needed and it can be subject to NCR capability. [Apple] also propose to assume that the application time of the adaptive beam for backhaul link is same as the application time of the beam indicated for access link

Based on the above inputs, from FL’s perspective:

- Regarding the NCR capability: it’s reasonable to introduce the NCR capability on the simultaneous UL transmission as agreed in SI. Also, the implementation based on either adaptive or fixed for backhaul and C-link should be supported.

- Regarding beam indication for backhaul link: majority prefers to introduce the new sginalling for flexibility, and meanwhile, it seems obvious that the default behaviour should be defined since newly introduced signalling is definitely optional feature and also need to cover the case with only fixed beam.

Then, followings are provided:

Proposal 1-2-1: If adaptive beams are adopted for C-link and backhaul link, the new signaling (e.g., MAC CE/DCI) is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link.

- If no valid beam is indicated for backhaul link, the beam of backhaul link follows one of the beams of the C link.

- Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Generally fine with the proposal. But the note can be removed, or it could be FFS, if the beams for backhaul link could be from RRC-configured or activated list of beams for C-link |
| CMCC | Fine with the main bullet. The candidate beams of BH link are configured and based on the candidate beams of C-link. Then the MAC CE and DCI could be used to further indication which beam or beams would be used for the BH link and corresponding AC link. It is not clear to us in which case an invalid beam is indicated by the gNB. The first sub-bullet should be clarified or updated. |
| Huawei, HiSilicon | We have some comments on the first sub-bullet. The beam used by C-link and the backhaul link can be different, e.g. wide beam for C-link and narrow beam for backhaul link. If there is no valid beam indication, this would degrade the NCR-Fwd performance if only wide beam can be used for backhaul link. In addition, the beam used by the backhaul link may also be different, e.g. wide beam for PDCCH and narrow beam for PDSCH/CSI-RS. Given that NCR have no idea what is being forwarded hence it is not possible to have a predefined rule to change the backhaul link beam for different channels. |
| Ericsson | Do not support, for two reasons: First, the motivation most companies have is that the backhaul link, for various reasons, may need to differ from the preferred control link. That may be the case, but that still does not justify separate backhaul link signaling. Presumably, both links should be reasonably good, and the control link could be selected based on the preferred backhaul link and not the other way around. Second, the proposal would require the gNB to make fundamental changes to its operations in places where the repeater should not have an impact. Allowing different backhaul links for different slots would require exception handling in the gNB scheduler and dual TCI tables, one for co-scheduled control and backhaul and one for separated control and backhaul. We think, in practice, this would mean that there would be no co-scheduling of MT and Fwd, which we have agreed. For the above reasons, we prefer the predefined rule for adaptive beam indication on the backhaul link. |
| LG | Generally fine with the proposal. For the sub-bullet, it seems the pre-defined rule is applied when the valid beam is not indicated. Since we have not discussed yet regarding the pre-defined rule, following can be added as an FFS. “FFS: how to determine the beam of backhaul link” |
| NTT DOCOMO | Our first preference is option2. If unified TCI framework is used for NCR-MT beam indication, it is obvious that no new signaling is needed. With unified TCI indication, the beam of NCR-Fwd backhaul can always be updated, even if there is no DL/UL channel/RS Rx/Tx at NCR-MT. If Rel-15/16 beam management framework is used for NCR-MT beam indication, if the rule is NCR-Fwd backhaul follows the beam of a CORESET/PUCCH resource determined by a predefined rule, the beam can always be updated via MAC CE since the TCI state of a CORESET/PUCCH resource can be updated via MAC CE, which also provides some extent of flexibility. |
| Nokia | Do no support. In our view separate indication is redundant in scenarios where C-link and backhaul are being transmitted simultaneously. And in scenarios where the transmissions are not simultaneous, the link is expected to be largely static reducing the need for dynamic indication. In our view it is sufficient that backhaul link beam is either configured or follows behavior based on NCR-MT behavior. |
| Intel | We support new signaling for backhaul beam, but we have concern on 2 sub-bullets. For 1st sub-bullet, how to understand ‘valid’ beam ? If we consider MAC CE or DCI similar to Rel-17 unified TCI, does it mean symbols before application latency for the beam is without valid beam ? For that case, whether NCR should forward or no forwarding is expected ? For 2nd sub-bullet, we also think it can be further discussed whether it is RRC configured list or activated list. Besides, we’re wondering, how to handle the symbol with simultaneous reception for C-link and backhaul link, or the symbol with simultaneous transmission for C-link and backhaul link. Does this proposal imply that gNB ensures same beam is indicated for both links or a rule can be applied, e.g., NCR uses C-link beam for both links? We think this aspect should be discussed. |
| AT&T | Generally, support the main bullet per our discussion tdoc in R1-2209089. For the sub-bullet we think the fallback to the C-link beam would be a good rule, we suggest FFS for the case where If no valid beam is indicated for backhaul link |
| vivo | If there is C-link tx/rx, BH beam is the same as C-link beam. We think this should be applied in addition to the current proposal. |
| Spreadtrum | We prefer to implicitly determine the beam by a pre-defined. Introducing a new beam indication signaling for backhaul link is unnecessary and will increase the overhead. |
| ZTE | We are fine with this proposal, and to avoid the ambiguity, we can further update the proposal as: Proposal 1-2-1: If adaptive beams are adopted for C-link and backhaul link, the new signaling (e.g., MAC CE/DCI) is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link. - If no dedicate beam is indicated for backhaul link via the corresponding signalling, the beam of backhaul link follows one of the beams of the C link. - FFS: how to determine the beam. - Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link. |
| Fujitsu | We prefer option 2 and share similar view to DCM’s. However, if the majority view is to support option 1, the main bullet is fine with us except for the example. We prefer to remove “(e.g., MAC CE/DCI)”. For the sub-bullet, the ‘valid beam’ is unclear to us. What kind of indicated beams can be valid and what kind of them is not? We prefer to remove ‘valid’ from the subbullet. |
| Lenovo | Fine with the proposal. We think there is no need to restrict the backhaul link beams only from a set of beams for C-link. We think, for C-link stability, maybe only wide beams are required/ or more suitable, however for backhaul, both wide as well as narrow beams for enhancing the overall link budgets are needed. |
| Samsung | Do not support the proposal. We prefer not to introduce a new signaling for backhaul link beam indication. As explain by DOCOMO, using a predefine TCI state / spatial relation. For example, for DL, if the predefine rule is to use the lowest ID of activated PDSCH TCI state as backhaul link beam, the legacy MAC-CE can be reused for the update of backhaul link beam. Also, this TCI state can be only used for backhaul link so that independent beam indication of C-link and backhaul link can be achieved. |
| Xiaomi | Don’t support, we prefer a pre-defined rule for backhaul beam indication. It is not necessary to configure two beams for C-link and backhaul link separately and the signaling is unnecessary. |
| NEC | We are fine with this proposal. |
| CEWiT | Support |
| China Telecom | It is not clear under which case the gNB will indicate an invalid beam. |
| KDDI | We are fine with the proposal modified by ZTE. |
| Sharp | We prefer option 2 with a pre-defined rule. It is unnecessary to define a separate signaling only for backhaul link if the same band/carrier is used as the control link. |
| IIT-K | Support the proposal |
| InterDigital | We are fine with Proposal 1-2-1. |
| Qualcomm | We support the updated proposal by ZTE. In response to opposing companies, we would like to note that this new signaling is optional, and their design is already supported by this proposal – because relevant rules (aligned with their proposals) can be defined to be used in case there is no explicit beam indication. |
| Sony | We prefer Option 2. As argued by DOCOMO, we believe new signaling for backhaul link is not needed. |
| Philips | OK with the proposal. |
| CATT | OK with the proposal |

Proposal 1-2-2: The following aspects should be NCR capability:

- Simultaneous UL transmission of C-link and Backhaul link

- Adaptive beam or fixed for C-link/backhaul-link

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | In our view, fixed beam should be default. Adaptive beam should be additional NCR capability. So proposal can be updated as: Proposal 1-2-2: The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and Backhaul link - Adaptive beam or fixed for C-link/backhaul-link - Note: Fixed beam for C-link/backhaul link is default capability |
| CMCC | Fine with the proposal. |
| Huawei, HiSilicon | We don’t understand the reason why fixe beam should be indicated as a NCR capability given that this can leave to implementation. |
| Ericsson | Support |
| LG | Support. |
| NTT DOCOMO | Support |
| Nokia | Support the proposal. |
| Intel | We’re fine with the proposal. |
| AT&T | Support. |
| Vivo | if fixed beam is used, operator will SET the fixed beam manually when deploying the NCR, such capability report seems redundant. |
| Spreadtrum | Support |
| ZTE | Support. We should to define the corresponding capability. Otherwise, the gNB will have no knowledge on whether and how to configure the beam for backhaul. |
| Fujitsu | Support |
| Lenovo | Fine with this proposal. |
| Samsung | Support. |
| Panasonic | The support of simultaneous multiple beams can be added. |
| NEC | Support. |
| CEWiT | Support |
| China Telecom | Support |
| Sharp | Support |
| IIT-K | Support |
| InterDigital | We support Proposal 1-2-2. |
| Qualcomm | Support and prefer Apple’s suggestion. |
| Sony | Support the proposal. |
| Philips | OK with the proposal. |
| CATT | OK |

    - Company view (Round-2)

For the proposal 1-2-1, it seems that majority prefer to define the new signalling for backhaul and only pending issue on the beam determination for some special cases. Considering the difference preference, following proposal with two FFSs are added to address the concerns.

Updated Proposal 1-2-1: If adaptive beams are adopted for C-link and backhaul link, the new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link.

- FFS: How to determine the beam for backhaul link if no dedicate valid beam is indicated；

- FFS: How to determine the beam if simultaneously transmission of C-link and backhaul link is performed

- Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support the proposal |
| Intel | We are fine with the proposal. For 2nd FFS, ‘simultaneously transmission of C-link and backhaul link’ seems like for UL transmission by NCR? But we also should consider DL reception by NCR, right? So, we suggest to say ‘simultaneous transmission or reception’. |
| IIT-K | Support the proposal |
| CEWiT | Support |
| LG | Support the proposal. It seems clearer to us. |
| NTT Docomo | Do not support. we still don’t think new signaling is needed, especially if R17 beam indication framework (unified TCI) is used for NCR-MT, which we have agreed that it is supported. What will be the difference between the new signaling and Rel-17 unified TCI indication signaling for MT? |
| Xiaomi | Don’t support, as we commented in the 1st round, we prefer a pre-defined rule for backhaul beam indication. It is not necessary to configure two beams for C-link and backhaul link separately, and the signaling is unnecessary. |
| Lenovo | We are generally fine with the proposal, however we think the note is a bit restrictive. We prefer the beam for backhaul link can be from the RRC configured list or separately configured. |
| ZTE | Support and flexibility is ensured to adjust the transmission for different links. |
| vivo | Support, Intel’s modification is fine for us |
| Fujitsu | Support the proposal |
| Huawei, HiSilicon | We are fine to support the proposal. |
| KDDI | We support the proposal. |
| CATT | Support the proposal in principle. Want to clarify what ‘if no dedicate valid beam is indicated’ means here |
| Samsung | Do not support. A new signaling for the beam indication is unnecessary. As explained in the previous round, some predefine rule/beam can be used and with similar flexibility as the use of new signaling. Also, since Rel-17 is agreed for NCR-MT, there is no strong need to use new signaling in this case. |
| Ericsson | Do not support, prefer predefined rule. As pointed out previously, we think there is little use for this new signaling: If there is a large difference between the two links, presumably MT and Fwd will both use the better of the two. If the difference is minor, the MT’s coding is sufficiently robust to manage the slightly worse link. Hence, even if the MT’s beam is not optimal for the Fwd, the preferred beam for the Fwd will be sufficiently good for the MT. On the other hand, we think that by allowing frequent backhaul link switching to the Fwd will cause several problems in UE management: - CSI measurements - Power control - Scheduling - TCI management - … In conclusion, this proposal will severely complicate support for repeaters in the gNB, if it is at all possible to combine with the agreement that the repeater should be transparent to the UE. |
| InterDigital | Support the proposal |
| Sony | Our preference is still to not introduce new signaling, i.e., “Option 2: The beam of backhaul link is determined by a pre-defined rule.” The second FFS of Updated Proposal 1-2-1 indicates that C-link and backhaul beams are inter-dependent; pre-defined rules are still needed. |
| CMCC | Support the proposal. The updated sub-bullets are more clear to us. |

For Proposal 1-2-2, the views are aligned among companies with additional editorial changes, i.e., to specify the fixed beam as default behavior. From FL’s perspective, it’s reasonable. Then, following updated version is proposed:

Updated Proposal 1-2-2: The following aspects should be NCR capability:

- Simultaneous UL transmission of C-link and Backhaul link

- Adaptive beam for C-link/backhaul-link

  - Note: Fixed beam for C-link/backhaul link is default capability

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support the proposal |
| Intel | We support the proposal. |
| IIT-K | Support |
| CEWiT | Support |
| LG | Support the proposal. |
| Lenovo | Support. |
| ZTE | Support this proposal |
| vivo | OK |
| Fujitsu | Support |
| Huawei, HiSilicon | We are not sure what “default” capability means. Also, we would like to clarify the above does not imply capability reporting. |
| Samsung | Support in general. Prefer add another note for the first bullet as below. Updated Proposal 1-2-2: The following aspects should be NCR capability: - Simultaneous UL transmission of C-link and Backhaul link - Note: TDM UL is the default capability - Adaptive beam for C-link/backhaul-link - Note: Fixed beam for C-link/backhaul link is default capability |
| Ericsson | Support |
| InterDigital | Support the proposal |
| China Telecom | Support |
| CMCC | Support the proposal. Fine with the note. Our understanding about the default behavior or capability is that if the adaptive beams for C-link/BH link are not supported or reported to the gNB, a fixed beam would be assumed for the C-link and BH link. |
| NEC | Both two proposals are OK. |

      - Summary of 2nd round discussion

For Updated Proposal 1-2-1, although there are concerns from some companies, majority prefer to take the proposal with some editorial changes. Then, following is proposed:

Updated Proposal 1-2-1_v2: If adaptive beams are adopted for C-link and backhaul link, the new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link.

- FFS: How to determine the beam for backhaul link if no dedicate beam for backhaul link is indicated；

- FFS: How to determine the beam if simultaneously transmission and reception of C-link and backhaul link is performed

- Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link.

For Updated Proposal 1-2-2, based on the inputs, it seems that majority is fine with the proposal with one more note. Then, the following proposal will be recommended for email approval for the 1st check point.

Updated Proposal 1-2-2_v2: The following aspects should be NCR capability:

- Simultaneous UL transmission of C-link and backhaul link

- Adaptive beam for C-link/backhaul-link

- Note-1: Fixed beam for C-link/backhaul link is default capability

- Note-2: TDMed UL transmission of C-link and backhaul link is default capability.

- Topic-2 ON-OFF information

  - Company view (Round-1)

For the ON-OFF information, following agreements have been achieved in previous meeting:

Agreement The following options can be considered to indicate the ON-OFF information from Gnb to NCR for controlling the behaviour of NCR-Fwd: - Option 1: Explicit indication with on-off state (e.g., via dynamic or semi-static signalling) or on-off pattern (e.g., periodic/semi-static ON-OFF pattern or new DRX-like pattern for ON-OFF) - Option 2: Implicit indication via the signalling for other information (e.g., beam, DL/UL configuration, or PC information) - Note: This example does not imply that PC information is necessary or not. - Other solutions (e.g., potential combination of explicit and implication solution) can be further discussed. Agreement For indication of NCR-Fwd ON-OFF for efficient interference management and improved energy efficiency, both dynamic and semi-static indication can be considered FFS: RAN1 to consider whether/how to handle the forwarding of broadcast and cell-specific signals/channels. Agreement The NCR-Fwd is always expected to be “OFF” unless otherwise explicitly or implicitly indicated by Gnb. - Note-1: This applies to the case regardless of the RRC state of NCR-MT. - Note-2: Indication (e.g., received when NCR-MT in RRC-connected) or DRX state of NCR-MT to control the ON-OFF behaviour of NCR-Fwd when the NCR-MT is in RRC-idle/inactive is not precluded. The above is not meant to imply any signalling design for NCR-Fwd ON-OFF.

In this meeting, according to the contributions, following views are shared by companies:

- Option-1:

[ZTE, CATT, Intel, AT&T, FirstNet, Sony, CAICT, CMCC, Apple, Sharp, Samsung, KDDI] propose to indicate the ON-OFF information in explicit way since it’s applicable for all frequency ranges, NCR-MT’s state and signalling forwarded by NCR. [Samsung] highlights that the at least the explicit OFF indication can be considered. [DCM] proposes to support the explicit ON-OFF indication in case NCR access link beam indication is not enabled.

More specifically,

- For semi-static ON-OFF indication, [ZTE, Intel, Sony, DCM] propose to indicate the periodic ON-OFF pattern, e.g., a set of time domain resource [Intel], via semi-static signalling. For the periodic pattern, [DCM] mentions following options can be considered.

    - Option1: separate ON/OFF indicator is indicated for each slot in the periodicity.

    - Option2: slot index in the periodicity corresponding to “ON” is indicated.

- For dynamic ON-OFF indication, either one bit to indicate single state or a multiple states, e.g., via bit map [ZTE] or multiple time domain resource in one set [Intel, DCM] can be indicated by DCI. [CMCC] highlights that the time domain resources carrying the broadcast and cell-specific signals could be indicated through the RRC configurations of ON-OFF state by Gnb.

Moreover, regarding the time domain resource, [DCM] mentions that following options can be considered.

  - Option1: To indicate the time domain resource, RRC configures a table with each row including slot offset, starting symbol, number of consecutive symbols, and number of slots, and DCI indicates a row index.

  - Option2: To indicate the time domain resource, RRC configures a table with each row including multiple sets of {slot offset, starting symbol, number of consecutive symbols, number of slots} for multiple beams, and DCI indicates a row index.

In addition, [vivo] highlights that the activation or deactivation signaling to the ON-OFF pattern should be defined and resource type indication (e.g., H, S and NA) in IAB is the baseline for ON-OFF indication. [Apple, Sony] prefers to introduce the semi-static DRX-like operation should be supported for NCR. [CATT] proposes to further specify the mechanism to resolve the potential configuration conflict between semi-static and dynamic ON-OFF indication. [CMCC] prefers to support both symbol-level and slot-level on-off configurations and indications. [Sharp] mentions that a unified solution with dynamic ON/OFF indication can be supported in the flexible symbols. [QC] highlights that to support a hierarchical control framework, based on semi-static configuration of beamforming information, and dynamic/semi-static indication of ON-OFF information, we may need an explicit ON/OFF indication.

[Huawei] also highlights that when an NCR-Fwd is indicated to be “ON” periodically, it cannot be turned OFF for periodic indication, and can be turned OFF by deactivation signaling for semi-persistent indication. Once an NCR is deployed and activated for signal forwarding, long term “OFF” should be avoided.

- Option-2: Implicit indication.

[Huawei, CT, Sony, Xiaomi, Apple, Samsung, LGE, DCM, QC, Ericsson, Nokia] highlights that implicit indication for ON-OFF is supported via beam indication.

More specifically, companies [DCM, Apple] mentions that if a time domain resource is indicated with an NCR access bam, it will be ON. Otherwise, it can be regarded as OFF. But [Ericsson, Samsung, Nokia, QC] propose to extend the beam indication framework and indicate the OFF by using a specific beam index/state in the dynamic beam indication, e.g., beam index 0 indicating OFF, beam index 1 indication ON. In addition, [vivo] proposes some rules to determine the ‘OFF’ or ‘ON’.

- Option-3: Option-1 + Option-2, i.e., combined solution with both explicit and implicit indication

[IDC, Fujitsu, NEC, Lenovo, CEWiT, IIK-T, Apple, Samsung, DCM, Sony] propose to consider the ON-OFF side control information signalling to be a combination of explicit and implicit indications, e.g., explicit signalling may be more efficient in NCR with frequent use-cases, whereas implicit signalling may be more efficient in NCR use-cases with long idle modes of operation. [DCM] mentions that when the beam indication is not enabled, the explicit indication should be used. [Samsung] mentions that implicit on and explicit off is preferred. Meanwhile, explicitly indicated OFF overrides implicitly indicated ON by access link beam indication.

In addition,

- [LGE] propose that NCR-Fwd operation is considered as OFF during RRC-idle/inactive state of NCR-MT. but [KDDI, MTK, ZTE] highlight that NCR can change the state of NCR-Fwd between ON and OFF independently of the transition of RRC state in NCR-MT, e.g., when NCR-MT is in RRC idle/inactive state, NCR-Fwd on/off indication can be transmitted during paging occasion. [ZTE] mentions that when NCR-MT enters RRC IDLE or INACTIVE state, semi-static single state indication in RRC release message can be used to control the ON-OFF behavior of NCR-Fwd. [Intel] mentions that when NCR-MT is in RRC idle/inactive state, NCR-Fwd can perform on/off based on the received on/off information regardless of RRC state of NCR-MT.

- [Nokia, Ericsson] highlight that minimum latency between when an NCR-MT receives an on-off indication and when the corresponding NCR-Fwd state is applied as NCR capability, e.g., 3 µs in FR2 (10 µs in FR1) [Ericsson].

- [Panasonic, Apple] also mention some specific rule to define the NCR-Fwd as OFF, e.g., link/beam failure and expiration of TA timer of NCR-MT or after a continuous no reception on the configured Rx beams (for example, no PRACH reception on configured Rx beams for multiple periods), it could be considered that at least NCR-Fwd receiver at access link can turn itself OFF (assuming same understanding at the Gnb).

Based on the above inputs, from FL’s perspective:

- Regarding the explicit/implicit solution for ON-OFF indication:

It seems that these two options are equally supported by companies. For the implicit solution, there is clear majority to indicate ON-OFF information via beam indication including defining a special case for beam. However, as commented by others, for the beam indication, following is captured in the WID (copied below):

Note: The work in RAN4 for beam related is expected to start on FR2 first.

Then, before concluding the discussion on beam in RAN4, it’s not clear whether the beam will be supported in FR1. Moreover, the complexity of extension the beam indication framework, e.g., introduce some specific beam state, is equivalent to explicit beam indication.

- Regarding the impacts of the RRC state of NCR-MT, considering the possibility of fallback to legacy repeater, e.g., without reception of side control information in long-duration, it’s preferred to decouple the NCR-MT RRC state and ON-OFF behaviour of NCR-Fwd.

- Regarding the minimum latency to turn on/off the NCR based on the reception, it’s proper to define it as a NCR capability separately.

- Then, followings are proposed:

Proposal 2-1: The explicit indication of ON/OFF information is supported for NCR-Fwd.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | We don’t think that this is the right direction for discussion ON/OFF information. In our view, it is clear that beam indication is essential, at least for FR2 and supporting FR2 is considered to have higher priority with NCR. Therefore, implicit indication should rather be the default option. Furthermore, it can be considered, if beam indication is not supported/needed for access link beam (for example in FR1), then explicit indication can be supported. Therefore, we would suggest following modification to the proposal: Proposal 2-1: The explicit implicit indication of ON/OFF information is supported for NCR-Fwd based on access link beam indication - FFS: Explicit indication of ON/OFF information if access link beam indication is not signalled |
| CMCC | Fine with the proposal. It is still controversial to support beam indication for NCR in FR1. And from the practical implementation it is also optional to support beamforming for FR1 repeater. But the on-off indication should be supported in both FR1 and FR2 NCR. |
| Huawei, HiSilicon | We don’t support the proposal. In our understanding, implicit ON and implicit OFF are sufficient for NCR-Fwd. It is not clear in what cases explicit ON is needed, and in what cases explicit OFF is needed. Firstly, time domain resource is always required for indicating the NCR-Fwd, and the ON can be implicitly indicated. So is aperiodic/dynamic forwarding. Other than this, the NCR is expected to be OFF, explicit OFF is not required. Secondly, periodically indicated forwarding (by semi-persistent configuration) can be deactivated by DCI or MAC-CE, thus OFF can be implicitly indicated. Thus, we propose the following: Proposal 2-1: The explicit following indications of ON/OFF information is are supported for NCR-Fwd: - Implicit indication of ON information is supported for NCR-Fwd. - Implicit indication of OFF information is supported for NCR-Fwd. |
| Ericsson | It is unclear to us what explicit ON/OFF actually means. We do not support separate ON/OFF and beam indication. Our proposal is instead to introduce the OFF beam in the beam indication framework and use a beam different from the OFF beam as implicit ON indication. Assuming beams with different beam properties, e.g., beam widths, it is unlikely that the number of beams will be an even power of two. As a result, ON/OFF can be included with zero overhead within the beam indication framework. Regarding potential FR1 problems, both the SI and the WI is justified on and assume beam indication and we don’t think FR1 can/should deviate from that. Finally, we think that a specific agreement about what OFF actually means would be advantageous for future discussions, e.g., If an NCR is configured OFF, it is not expected to forward in the corresponding symbol. |
| LG | Before discussing whether the ON/OFF information is indicated in explicit or implicit manner, aligning the explicit and implicit method should be clarified. Judging by the contributions of other companies, understanding of explicit and implicit indication is quite diverge. For example, only ON or OFF is indicated via specific/explicit indication is considered as explicit indication for some company. And another example is, ON/OFF via beam indication is considered to be explicit indication to some, and implicit to others. Therefore, we think it should be clarified first. On the other hand, if the explicit indication of ON/OFF is the explicit signaling dedicated for ON/OFF signaling, we do not think it is desirable in terms of signaling overhead and future conflict with other indication, e.g., OFF and the access link beam is indicated at the same time resource. Therefore, we think at least time resources that access link beam is indication should be treated to be ON. Considering the ON/OFF indication based on the beam indication, at least OFF for dedicated beam index is desirable. Even if FR1 does not support beam indication, ON-OFF can be implicitly indicated by beam indication for a certain beam index. |
| NTT DOCOMO | Support |
| Nokia | Clarification is needed. As indicated in our contribution, we have a similar view to the framework that Ericsson is proposing. Our preference would be to have indication of on/off information designed jointly with beam indication, where NCR-Fwd ‘off’ operation is indicated as one of the beam states for the NCR-Fwd. |
| Intel | We support the proposal for explicit on/off indication and open for whether only indicate on, or only indicate off or both on and off. As explained by FL, when beam indication is not configured, we anyway need explicit on/off indication. And even when beam indication is configured, there is still some scenarios that a semi-static beam is provided but NCR can be dynamically off to reduce interference or for power saving. According to companies’ comments, it seems even some companies not supporting separate on/off signaling think it is useful to explicit indicate off by a special beam state. Maybe we can first try to agree whether explicit on or off is needed and then discuss whether separate or joint coding for on/off and beamforming. |
| AT&T | Support the proposal per our discussion tdoc in R1-2209089 |
| Vivo | Yes |
| ZTE | Support the proposal. Regarding the clarification on the meaning of “implicit” or “explicit”, companies are encouraged to check the previous agreement. The explicit as Option-1 means that there is dedicate signalling is used to indicate the ON-OFF information only. The implicit way means that this signalling is used to indicate other information and will be re-interpreted by NCR for ON-OFF controlling |
| Fujitsu | Do no support. We prefer to separately discuss the dynamic indication and semi-static indication. For example, the implicit dynamic indication, e.g. reusing the access link beam indication, and explicit semi-static indication are preferred. We see no reason to force dynamic indication and semi-static indication to both implicit or explicit. In our understanding, the NCR at least has two different behaviors in an OFF state. The first one is if the OFF state is the default OFF state like what we agreed in the previous meeting, the NCR doesn’t forward but is ready for forwarding. In this case the access link beam indication can be reused as ON indication. The second behavior is if the OFF state is explicitly indicated and with a certain duration, both the NCR and the Gnb know that the NCR can sleep to a certain level. |
| Lenovo | We are supportive with explicit on/off indication. We also prefer implicit on/off indication by a special beam index, TDD configuration to be supported. |
| Samsung | We think explicit/separate indication of OFF state is necessary/beneficial for interference management. For example, semi-static OFF indication is useful for “nigh-mode” / no-traffic operation where NCR-Fwd forwarding is not expected, without need to make any changes to semi-static beam indication. Dynamic OFF indication is also beneficial to override semi-static beam indication for short-term interference management where semi-static beam may impact UEs/gNBs in same or neighbor cells. It can be further discussed whether explicit/separate ON indication is needed (on top of implicit ON indication, e.g., based on beam indication). However, similar to Ericsson and Nokia, we prefer a unified signaling framework for beam indication and OFF indication, such as beam index “-1” for OFF indication. So, we suggest the following modification: Proposal 2-1: The explicit indication of ON/OFF information is supported for NCR-Fwd. - FFS: unified signaling framework for beam indication and OFF indication FFS: whether to support explicit indication of ON information |
| Panasonic | Do not support. We think the ON/OFF indication can be designed jointly with beam indication. |
| NEC | Support. |
| CEWiT | Support |
| China Telecom | Not support. We think implicit indication is enough. |
| KDDI | We are fine with the proposal. As CMCC pointed out, it is beneficial to support on-off indication in both FR1 and FR2 NCR. |
| Sharp | Support |
| IIT-K | We think a combination of implicit and explicit ON/OFF indication is more appropriate with priority configured for explicit and implicit indications. |
| InterDigital | We support the Proposal 2-1. |
| Qualcomm | We generally support this proposal. However, we also believe in the following two aspects: - We can discuss explicit ON indication and explicit OFF indication separately, as they may address different use-cases. - We should strive to have a smart design like reusing the beam indication framework. As a possible way forward and to progress, we may want to treat OFF indication and ON indication separately. |
| Sony | Support the proposal. |
| Philips | OK with the proposal. |
| CATT | Should add ‘at least’ |

Proposal 2-2: The ON-OFF behavior of NCR-Fwd is controlled by ON-OFF indication regardless of RRC-state of NCR-MT

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Fine to support |
| CMCC | Fine with proposal. |
| Huawei, HiSilicon | Since ON-OFF indication is not agreed yet, it is preferred to revise the proposal as: Proposal 2-2: The ON-OFF behavior of NCR-Fwd is controlled by ON-OFF indication regardless independent of RRC-state of NCR-MT |
| Ericsson | Do not support. It is not clear if this proposal relates to semi-static or dynamic signaling. As long as RAN2 has not decided on RRC state support, RAN1 can postpone this discussion. |
| LG | In our perspective, it is RAN2’s scope do define the behavior of NCR-MT in the RRC idle/inactive state, or even NCR-MT supports RRC idle/inactive state. Therefore it is premature for RAN1 to discuss any proposal regardless of RRC-state of NCR-MT. Even RAN2 decided the behavior of NCR-MT of RRC idle/inactive state, we think enhancement for NCR-MT to receive side control information in RRC idle/inactive state is unnecessary. |
| NTT DOCOMO | Support |
| Nokia | Fine to support. In our view on-off behavior of the NCR-Fwd, should not be connected to the RRC state of the NCR-MT, whether DRx is a supported feature for the NCR-MT or not. |
| Intel | We support the proposal based on the understanding that no enhancement for side control information reception during NCR-MT in RRC idle state, and the semi-static side control information received during NCR-MT in RRC connected state can be applied for NCR-Fwd regardless of NCR-MT RRC state. |
| AT&T | We are ok with the proposal and the RRC state should not matter, but this is probably something that RAN2 should discuss. |
| Spreadtrum | Support. |
| ZTE | We are fine to support this proposal. Regarding whether other RRC state is supported or not, it’s determined by RAN2. Then, the updated proposal is: Proposal 2-2: The ON-OFF behavior of NCR-Fwd is independent of RRC-state of NCR-MT Note-1: The support of RRC-inactive/idle state for NCR-MT will be determined by RAN2. |
| Fujitsu | Now is a little early to conclude this, after all RAN2 doesn’t conclude which RRC states are supported for NCR-MT yet. |
| Lenovo | Fine with the proposal. |
| Samsung | OK with the intention. NCR should operate only based on indications received in the Connected mode, and agree with other companies that support of RRC states for NCR is RAN2 domain. |
| Panasonic | When NCR-MT is RRC_IDLE or INACTIVE, our view is that the communication using side control mechanism between NCR and Gnb is not possible. Therefore, we don’t agree to control NCR-Fwd in NCR-MT RRC_IDLE or INACTIVE. The proposal introduces a sort of new RRC state, which could bring significant complexity in RAN2 discussion. As RRC state handling is RAN2 expertise, this discussion is up to RAN2 can be more reasonable. |
| CEWiT | Support |
| China Telecom | We are generally fine with the proposal. |
| KDDI | We support the proposal modified by ZTE. |
| Sharp | Fine with the proposal. |
| IIT-K | Fine with the propsal |
| InterDigital | We support Proposal 2-2. |
| Qualcomm | We do not support this proposal, and this aspect needs more discussions in RAN2 and RAN1. There are serious questions that should be answered. For example, if NCR-MT goes to idle-mode because of a link failure, do we expect NCR-Fwd to continue its operation as scheduled/indicated before? |
| Sony | In principle, we support this proposal. |
| Philips | Unclear how this would work. We gree with Panasonic. |
| CATT | OK |

Proposal 2-3: The minimum required time to apply the ON/OFF state by NCR-Fwd is NCR capability, which is defined as Y symbols after the last symbol of the HARQ-ACK in response to the ON-OFF indication received by NCR-MT or Y symbols once previous indication is invalid.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | It needs to be discussed and agreed if HARQ-ACK feedback is supported or not. Therefore, the proposal can be updated as: Proposal 2-3: The minimum required time to apply the ON/OFF state by NCR-Fwd is NCR capability, which is defined as Y symbols after the last symbol of the HARQ-ACK in response to the ON-OFF indication received by NCR-MT or Y symbols once previous indication is invalid. FFS: definition of minimum required time |
| Huawei, HiSilicon | Firstly, the required time is up to RAN4. Secondly, side control information can be transmitted via PDCCH and PDSCH. It is not agreed whether there is HARQ-ACK or not for the PDCCH-based side control information. We prefer to defer the discussion on this proposal. |
| Ericsson | See response to Proposal 1-1-7. |
| LG | Similar comment to proposal 1-1-7. Okay for the capability report although we do not see further spec support is necessary. |
| NTT DOCOMO | Do not support. We think it is more reasonable that the NCR capability of ON-OFF switching is defined as gap between the ON-OFF indication and the applicable time. |
| Nokia | Support with similar views to proposal on beam indication time. Further details are needed regarding signaling details for on-off control. |
| Intel | Similar comment to proposal 1-1-7. |
| Vivo | Firstly, the processing time depends on the signaling conveying the SCI, MAC CE/RRC/DCI requires different processing time. Secondly, if we discuss processing capability, we wonder the minimum required time is changed to “maximum required time” |
| ZTE | Support with same argument as above for proposal 1-1-7. |
| Fujitsu | Do not support. The reason is the same with Proposal 1-1-7. |
| Lenovo | We have concern on the “after HARQ-ACK”. We think this is only applicable for MAC CE. |
| Samsung | Can discuss this after HARQ-ACK support has been decided. |
| Panasonic | Similar comments to Proposal 1-1-7. |
| NEC | Similar as the comments on Proposal 1-1-7 |
| CEWiT | Fine with the proposal. In our understanding HARQ feedback is essential in case of NCR to avoid performance degradation due to noise amplification and packet loss. |
| Sharp | The cases for different ON/OFF indication methods should be further clarified, e.g. semi-static indication, MAC CE or DCI indication. |
| IIT-K | Similar comments as proposal 1-1-7 |
| InterDigital | We only support the first part of proposal 2-3, that is: The minimum required time to apply the ON/OFF state by NCR-Fwd is NCR capability. The rest of the proposal needs more clarification. |
| Qualcomm | Do not support. We should leave out aspects related to HARQ feedback for now and attempt to get alignment on the first fundamental question that whether the application latency (that will be accurately defined later) is an NCR’s capability or a specified value. |
| Sony | We suggest to postpone this discussion until other ON/OFF related topics have been agreed upon. |
| CATT | No need to discuss this in ran1 |

  - Company view (Round-2)

For the Proposal 2-1, companies propose to discuss the ON or OFF state separately. Meanwhile, the impacts of beam indication for joint design is also highlighted. From FL’s perspective, it’s potential way to consider it but at least, we need to ensure the proper ON-OFF indication if no beam indication is applicable. Then, the following is proposed:

Updated Proposal 2-1: For the ON/OFF information indication:

- Explicit OFF is supported

- Explicit ON is supported at least if the beam indication is not available

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Do not support this proposal. In our view, we would prefer to first agree on implicit ON/OFF based on beam indication. Also, it is not clear to us if we need explicit OFF. In our view, the default state is OFF, so even if explicit indication is needed, it should be for ON, if beam indication is not available. |
| Intel | We support the proposal, it is a good wayward to go. To answer Apple’s question, for symbols/slots with semi-static beam information, it is also useful to enable ‘OFF’. |
| IIT-K | Support the proposal. |
| CEWiT | Support the proposal. Regarding explicit and implicit indication of ON/OFF, both options should be discussed separately and prioritized later on, just like indication of availability (IA) for soft resources in IAB. |
| LG | It is little bit confusing to us since ON and OFF indication are treated separately, even they are mutually exclusive. Although we understand the different views from companies lead ON and OFF indication separately, following modification can be considered: Updated Proposal 2-1’: For the ON/OFF information indication: - Explicit OFF is supported - For the time resources not indicated by explicit OFF - If the beam indication is available, ON/OFF is determined by beam indication information - If the beam indication is not available, NCR is ON. Explicit ON is supported at least if the beam indication is not available |
| NTT Docomo | Similar comment as Apple. We think explicit OFF may not be needed. Since default state is OFF, we may only need explicit indication of ON |
| Lenovo | Prefer Apple’s way to handle the issue. |
| ZTE | Support this proposal. For the case without beam indication, there is still needs to turn on/off the repeater for energy saving and interference management. |
| vivo | We support the explicit mechanism. We need explicit ON/OFF indication when beam indication feature is not used. Moreover, explicit off is used to dynamically turn off the NCR, e.g., for some semi-static NCR on duration, the NCR can be dynamically turn off. We also think it is redundant to support implicit on-off based on beam indication, when explicit on-off is to be supported. |
| Fujitsu | We support this bullet. In our understanding, explicit OFF is needed. For example, when no UE needs the service of the NCR, the gNB can explicitly indicate the NCR turns OFF (sleep) Regarding to the explicit ON when no beam indication, we think it is not necessary for FR2. For FR1, in our understanding, semi-static ON/OFF is enough. Dynamic ON/OFF is necessary for FR1 if no adaptive beams are supported in AC link. This is because the gNB in FR1 usually do not sweep beams. The gNB in FR1 schedules UEs in a FDM way like LTE eNBs. This is in line with the water-filling principle. Hence, in FR1, if there is no beam indication, explicit OFF is enough. |
| Huawei, HiSilicon | We don’t support the proposal. In our understanding, implicit ON and implicit OFF are sufficient for NCR-Fwd. It is not clear in what cases explicit ON is needed, and in what cases explicit OFF is needed. And we propose the following: Updated Proposal 2-1: For the ON/OFF information indication: - Explicit Implicit OFF is supported if the time domain resource is released or deactivated - Explicit Implicit ON is supported at least if the beam time domain resource indication is not available |
| Mediatek | Support the proposal. |
| KDDI | We are fine with the proposal in principle. But we tend to agree with Apple and docomo, that is, explicit ON is necessary and explicit OFF might be omitted because the default mode of NCR is OFF. |
| CATT | We think explicit OFF is not needed |
| Samsung | We are fine with the first bullet. However, we think it is premature to agree the second bullet. Suggest to add FFS. Updated Proposal 2-1: For the ON/OFF information indication: - Explicit OFF is supported - FFS: Explicit ON is supported at least if the beam indication is not available Also, it is better to clarify the meaning of beam indication is not available. We think the intention is that FR1 does not have beam indication. But it may lead to confusion with the case that beam indication is not yet received (as default behavior). |
| Ericsson | Do not support the proposal. We don’t see the use case with explicit ON without a beam also being indicated. Companies supporting this should provide an explanation for that use case. Explicit OFF may be useful for both semi-static and dynamic configurations. For semi-static, it may be used to cancel an existing semi-static beam (ON) configuration. For dynamic, it may be used to indicate an OFF time within a dynamically indicated time interval. |
| Panasonic | Do not support. We think explicit ON is not required. According to earlier agreement, the beam indication is mapped to time domain resources. So, the NCR should be OFF over time resources without assigned beam(s). Explicit OFF could be used by dynamic indication to override the semi-static indicated beam. |
| InterDigital | Support the proposal. In this way, in case beam indication is not available, explicit ON-OFF signaling is used. Otherwise, for cases with beam indication, the ON signaling can be indicated implicitly. |
| China Telecom | We generally fine with the proposal. |
| Sony | Support the proposal. |
| CMCC | Thanks for FL’s efforts searching for a compromise. For the 1st bullet, We do not support the explicit OFF indication, since the OFF is considered as a default state without further indication. For the 2nd bullet, we understand that if one slot or symbol is indicated with a AC link beam indication, it mean a data forwarding should be carried out. Then a AC beam indication could be considered as an indication of the on state. And on the other side, if no beam is indicated, then the slots or the symbols should be considered as off for the data forwarding. Then there seems introducing a multiple levels behavior. When there is no beam indication, a additional indication of ON is needed. But the intention behind the 2nd bullet is if the AC beam indications are not supported for the NCR (e.g. in FR2), then a literally explicit ON indication is needed. |
| NEC | Support the new version. |

For the Proposal 2-2, the views is similar and companies are fine to define the capability without coupling with the ACK.NACK. Then, following updated is proposed:

Updated Proposal 2-3: The minimum required time to apply the ON/OFF state by NCR-Fwd is NCR capability.

Companies are encouraged to share your views.

| Companies | Comments and Views |
| --- | --- |
| Apple | Support the proposal |
| Intel | We’re fine with the proposal. |
| IIT-K | Support the proposal |
| CEWiT | Support |
| LG | Fine with the proposal but it seems too early to discuss with the required time of ON/OFF since ON/OFF indication itself is not discussed well. |
| NTT Docomo | Support |
| Lenovo | Support. |
| ZTE | Support |
| vivo | We wonder why maximum value is not reported, do you assume legacy DCI processing time is assumed to bound the maximum required time? |
| Fujitsu | Do not support |
| Huawei, HiSilicon | Similar to other proposals, we think the above should not imply there will be multiple values defined by specification and capability reporting. |
| CATT | support |
| Samsung | Support in general. Some wording suggestion is as below: Updated Proposal 2-3: The minimum required time to apply the ON/OFF state by NCR-Fwd is subject to NCR capability. |
| Ericsson | Do not support. See our response to Updated Proposal 1-1-7. Irrespective of our (lack of) support, we think a definition of OFF would be useful for RAN1’s continued work. Our proposal is: If an NCR is configured OFF, it is not expected to forward in the corresponding symbol. |
| Panasonic | Generally, we are fine with the proposal. But is that going to be a separate minimum required time to apply beam? |
| InterDigital | Support the proposal. |
| China Telecom | We generally fine with the proposal. |
| Sony | Okay. |
| CMCC | Support. Application time should be discussed for on-off, beam indication and the indication of the side control information. |
| NEC | Support. |

    - Summary of 2nd round discussion

For the Updated Proposal 2-1, it seems that the main concern is on the necessity of explicit ON indication. Then, all potential solution to indicate the ON state are listed for further down-selection. For the explicit off, potential benefits are foreseen by companies.

Then, following is proposed:

Updated Proposal 2-1_v2: For the ON/OFF information indication:

- Explicit OFF is supported

  - At least the semi-static indication via RRC and/or MAC CE is supported.

- One of following options is supported to indicate the ON state of NCR

  - Alt-1: Explicit indication at least if beam indication is not applicable

  - Alt-2: Implicit indication via the beam indication

  - Alt-3: Implicit indication via the time domain resource indication (i.e., the NCR is assumed to be ON over the indicated time domain resource)

For the Updated Proposal 2-1, considering the concern from companies, e.g., should not be capability but a specified value, companies are encouraged to further check the impacts. This issue will be handled later once the generic signalling is consolidated.

- Topic-3 TDD configuration

  - Company view (Round-1)

Regarding the NCR’s behavior over flexible, following agreement has been achieved in RAN1#110:

Agreement For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), following options are considered for the NCR-Fwd on these symbols - Option 1: The NCR-Fwd is expected to be OFF or not forwarding over these symbols - Option 2: The NCR-Fwd will follow the TDD operation determined by NCR-MT, i.e., determined by NCR-MT based on the received SFI indication or scheduling from Gnb - Note: It means that no new side control signalling is needed. - Option 3: The NCR-Fwd will follow a new dynamic side control signalling of DL/UL forwarding over these symbols to NCR-Fwd

In this meeting, according to the contributions, following views are shared by companies:

- Option-1:

[Huawei, Spreadtrum, CATT, Intel, LGE, Lenovo, KDDI, ZTE, Apple] highlights that Option-1 is preferred, e.g., to save power and reduce the potential interference to the network, and no specific side control signaling is required to support dynamic DL/UL forwarding. [Apple] highlights that the default behavior is NCR-Fwd OFF and no forwarding is expected.

- Option-2:

[Fujitsu, Nokia, AT&T, FirstNet, Sony, Panasonic, ETRI, Sharp, Ericsson, QC] highlights that Option-2 is preferred. And in addition, [QC] proposes that to resolve ambiguity within any remaining “flexible” resources, support an optional dynamic side control signaling of DL/UL forwarding direction over these symbols. [Ericsson] also highlights that the repeater-MT can indicate through capability signaling if it is supporting dynamic TDD UL/DL indication.

- Option-3:

[IDC, Sony (if Option-2 is not sufficient), CMCC, CEWiT, IIT-K, MediaTek, Apple] highlights that Option-3 is preferred allows flexible scheduling and lower latency. [KDDI] mentions that Option-3 can be considered in the case where NCR-MT and NCR-Fwd operate on different frequency bands, which is defined as an NCR capability. [Apple] mentions that for semi-statically configured flexible symbols, dynamic indication of the flexible symbols can be jointly indicated with the time-domain resources for the access link beams

- Option-1 & Option-2:

[vivo, ZTE, DCM, Fujitsu, Ericsson] propose to support the combo solutions, i.e., Option-1 and Option-2. Then, the configured flexible symbol can be set to UL or DL via DCI format 2_0.  And on the flexible symbol which is not dynamically indicated to UL/DL via DCI format 2_0, the NCR-Fwd sets to OFF.

- Option-1 and Option-3:

[Samsung, Apple] propose to support the combo solutions, i.e., Option-1 and Option-3

Based on the above inputs, it’s clear that either Option-2 or Option-3 is not complete solutions since there is still potential flexible symbols without indication. And the dynamic behavior for NCR is also required. From FL’s perspective,

Option-1 is should be considered as default behavior and considering the signalling overhead and majority’s view, the Option-2 can be the way forward with additional capability.

Then, the following is proposed:

Proposal 3-1: For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the NCR-Fwd is expected to be OFF or not forwarding over these symbols if neither SFI indication nor scheduling from Gnb is received by NCR-MT.

- Otherwise, the NCR-Fwd will follow the TDD operation determined by NCR-MT, i.e., determined by NCR-MT based on the received SFI indication or scheduling from Gnb.

- Note: The support the dynamic DL/UL operation is NCR capability.

Companies are encouraged to share your views and if there are concerns, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| Apple | We agree to support the default behavior on semi-statically configured flexible symbols as OFF and not forwarding. Regarding dynamic indication, as discussed in our contribution, we don’t think that just relying on dynamic SFI is sufficient. And it is not clear from the proposal how the scheduling information from Gnb is received by NCR-MT to determine flexible symbol behavior. In our view, this can only be possible if the NCR-MT is required/allowed to decode the scheduling DCIs intended for UE, which is not agreed/supported. Therefore, unless a dedicated dynamic signaling is indicated to NCR about the flexible symbol behavior, there is no way for NCR to know. Also, since the flexible symbols behavior needs to be known only when an access beam is indicated for NCR-Fwd, therefore, we think that such indication can be jointly indicated along with TDRA for access link beam. |
| CMCC | Our understanding is that if there is a need to forward the data over the flexible symbols, Gnb could indicate the corresponding symbols as ON and with the indication of forwarding direction. This belongs to the behavior of the side control signaling. The SFI based mechanisms require the NCR-MT first to interpret the group common signaling and then determine the uplink and downlink based on the indication of SFI. The SFI is sent to the NCR-MT but the real target is for data forwarding to the access UEs. This is not straightforward as direct indicating an on-state and the forwarding direction over the flexible symbols. Additionally, the SFI is an optional capability, which may also not be supported by the NCR-MT. There is no need to require the NCR support an optional UE capability and may increase the cost and complexity of the chipset of NCR-MT. Our preference is to separate the signaling for normal UE and NCR clearly and does not mix them together, which could also increase the complexity of the scheduling of the Gnb. Last but not the least, if two Ues under the NCR are divided into different UE groups and indicated with different behavior, such as downlink and flexible, it should be further clarified which kind of behavior should the NCR follow. |
| Huawei, HiSilicon | Fine with the proposal. |
| Ericsson | Support the intention. In our understanding, SFI is part of dynamic signaling that is, according to the note, based on capability. Hence, it should not be part of the main agreement. Based on this, the proposal would be: For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the NCR-Fwd is expected to be OFF or not forwarding over these symbols. Based on capability, if the MT can determine its TDD operation on flexible symbols based on the received SFI indication or scheduling from Gnb, the NCR-Fwd follows the TDD operation determined by NCR-MT. |
| LG | Fine for the proposal in terms of not introducing new signaling and defining NCR behavior on the time resource where DL/UL indication is absent. For the NCR not supporting dynamic DL/UL operation, it should operate OFF for the time resource indicated as flexible symbol based on the semi-static configuration. And for the dynamic TDD capable NCR, it would be appropriate to perform DL/UL operation according to the conventional SFI indication or scheduling indication, and to operate OFF in the resource finally determined to be flexible. |
| NTT DOCOMO | Support |
|  | In our view, the NCR-Fwd behavior over flexible symbols can be determined by capability. There are then two cases to consider: - The NCR doesn’t support operation over flexible symbols, in which case the NCR-Fwd would be off over flexible symbols, - The NCR does support dynamic TDD in which case NCR-Fwd could follow the NCR-MT. In the case of option 1, transmissions should not be scheduled to be forwarded on flexible resources. In the case of option 2, the NCR-Fwd should not be dependent on SFI indication, but rather only whether there is a channel or signal to forward. |
| Intel | We object the proposal. We support option 1. First of all, we don’t think dynamic TDD is the scenario for NCR. Any optimization to support NCR under Gnb with dynamic TDD is not needed. Option 1 is sufficient. If companies want to support option 2 for more flexibility though no use case, it is very strange to restrict DL/UL forwarding only in symbols with NCR-MT DL or UL scheduling, considering side control information is the main DL for NCR-MT and it is transmitted much less frequent than DL/UL traffic for Ues because a side control information typically applies to multiple slots. And it leads to no UL forwarding in symbols with UL C-link for NCR incapable of simultaneous UL transmission. Furthermore, it complicates NCR behavior and leads to ambiguity on when to forwarding DL considering there is no specified processing time for PDCCH (NCR knows starts of PDSCH only after PDCCH decoding). Therefore, option 2 does not make any sense. Option 3 works better than option 2, but incurs additional signaling overhead. |
| AT&T | Support. The NCR-Fwd should follow the TDD operation determined by NCR-MT, i.e., determined by NCR-MT based on the received SFI indication or scheduling from Gnb. If neither SFI indication nor scheduling from Gnb is received by NCR-MT, the NCR-Fwd is expected to be OFF or not forwarding over these symbols. |
| Vivo | UL/DL of FWD cannot be determined by Gnb’s grant to MT. since FWD forward UE’s UL/DL… Otherwise, the NCR-Fwd will follow the TDD operation determined by NCR-MT, i.e., determined by NCR-MT based on the received SFI indication or scheduling from Gnb. |
| ZTE | We are fine to take this as compromise although the Option-1 is preferred. Meanwhile, it’s clear that Option-1 should be supported regardless of support of Option-2 or 3. |
| Fujitsu | In our understanding, if the NCR supports dynamic TDD, SFI is enough to determine the forwarding direction except for the semi-static TDD config indication. The scheduling of NCR-MT should be independent of the forwarding of NCR-Fwd. Hence, we prefer the following update: Proposal 3-1: For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the NCR-Fwd is expected to be OFF or not forwarding over these symbols if neither SFI indication nor scheduling from Gnb is not received by NCR-MT. - Otherwise, the NCR-Fwd will follow the TDD operation determined by NCR-MT, i.e., determined by NCR-MT based on the received SFI indication or scheduling from Gnb. - Note: The support the dynamic DL/UL operation is NCR capability. |
| Lenovo | We still prefer only semi-static TDD configuration is used to determine the on/off state of NCR. |
| Samsung | Do not support. Usage of flexible symbols by NCR-MT is independent of forwarding operation of NCR-Fwd for Ues on flexible symbols – it is not necessary that both NCR-MT and the Ues are scheduled in the same flexible symbols. The NCR-Fwd can forward in flexible symbols based on Gnb indication, if provided, or stay in OFF state otherwise (i.e., Option 1 + 3). |
| Panasonic | Support. |
| CEWiT | We are fine to define default behavior as NCR-Fwd is OFF when dynamic indication is not provided by Gnb. But, linking the DL/UL state of NCR-Fwd in F symbols with NCR-MT has following limitations (details are mentioned in our contribution R1-2210114) - Mandates Gnb to provide DL/UL configuration to F symbols at NCR-MT even when Gnb is not expecting any transmission/reception from NCR-MT (e.g., TDM between BL and CL) - Creates unnecessary monitoring at NCR-MT (e.g., when F symbols overlap with CORESET and given DL configuration) Does not support the scenario in which where NCR-MT operating in subset of the carriers in which NCR-Fwd is operating and the DL/UL configuration is different across the carriers |
| KDDI | We basically prefer Option 1, but can accept this direction as a compromise. Also we think the modification from Fujitsu is needed because the scheduling of NCR-MT and the forwarding operation of NCR-Fwd should be considered independently. |
| Sharp | Support the proposal. The flexible ymbols can be determined by NCR-MT configuration if the Gnb wants to use these symbols. If not configured, they can be treated as OFF by default. |
| IIT-K | Fine with the default behaviour in case dynamic indication is not provided, but we share views with Samsung and CEWiT that NCR-MT may not be scheduled in the F symbols when NCR-Fwd is scheduled. Hence, Option-3 is needed. |
| InterDigital | We only support the first part of the Proposal 3-1, that is: the NCR-Fwd is expected to be OFF or not forwarding over these symbols if neither SFI indication nor scheduling from Gnb is received by NCR-MT. However, we do not support the second part of the agreement, as there should be a dedicated dynamic signaling for determining the flexible symbols’ TDD direction for the NCR-Fwd. |
| Qualcomm | Do not support. We believe having an optional explicit UL/DL state indication (via side control) strikes a good balance between the signaling overhead, and applicability and futureproof-ness of NCR. If signaling overhead is an issue, the network will not configure this feature of explicit DL/DL state indication. |
| Sony | Support this proposal. |
| CATT | Support this proposal. |

  - Company view (Round-2)

For proposal 3-1, it’s clear that as default behavior, majority agree to take Option-1, i.e., the NCR-Fwd is expected to OFF. Meanwhile, others propose to further discuss the behavior if dynamic DL/UL operation is supported by both NCR-MT and NCR-Fwd. Then, the following is proposed:

Updated Proposal 3-1: For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the NCR-Fwd is expected to be OFF or not forwarding over these symbols

FFS: The behavior over these symbol if dynamic DL/UL operation is supported by both NCR-MT and NCR-Fwd.

Companies are encouraged to share your views and if there are concerns, please directly propose the corresponding updates.

| Companies | Comments and Views |
| --- | --- |
| Apple | We suggest including the main bullet as default behavior and then additionally dynamic behavior can be FFS Updated Proposal 3-1: For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols FFS: The behavior over these symbol if dynamic DL/UL operation is supported by both NCR-MT and NCR-Fwd. |
| Intel | We support the proposal. |
| IIT-K | Support Apple’s updated proposal. Suggest making the main bullet as default behaviour. |
| CEWiT | Support the modification proposed by Apple. |
| LG | Support the proposal and fine with Apple’s modification. |
| NTT Docomo | In our understanding, with the main bullet, dynamic DL/UL indication is not supported because the it says for flexible symbol based on semi-static configuration NCR-Fwd is OFF, then dynamic indication will not be needed for the semi-static flexible symbols. While the FFS intends to further study dynamic operation. Thus, we suggest following update. Updated Proposal 3-1: For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), if DL/UL is not dynamically indicated, the NCR-Fwd is expected to be OFF or not forwarding over these symbols FFS: The behavior over these symbol if dynamic DL/UL operation is supported by both NCR-MT and NCR-Fwd. |
| Lenovo | Agree with Apple’s version. |
| ZTE | We are fine with this proposal and regarding the concept of default, it’s not clear how to reflect it in the spec. Meanwhile, since the dynamic operation is still FFS, it’s better not to highlight the reception of indication dynamically in the main bullet. |
| Fujitsu | Support |
| Huawei, HiSilicon | Support |
| KDDI | Support |
| CATT | Fine with apple modification. |
| Samsung | Agree with the update from Apple. At least to make option 1 as default behavior and further discussion whether option 2 or option 3 is used. |
| Ericsson | Support, including Apple’s version assuming optionally, other behaviors are supported conditioned on capability. Considering Ues have to support flexible symbols, repeaters should not have a more restrictive functionality. |
| Panasonic | We support the Apple’s revised proposal. |
| InterDigital | Support the proposal considering Apple’s version. |
| Sony | Support with Apple’s modification. Our understanding is that “default behavior” is the behavior of the NCR-Fwd before applying any DCI 2_0 indications, if supported. |
| CMCC | Fine with Apple’s updates. If new behavior are to be introduced, the original option 3 with direct indications to NCR as a side control signaling is still our first preference. Based on some group common signaling to UEs and depending on the interpretation of those signallings for normal UEs are not preferred. |
| NEC | Support the new version 3-1-v2. |

    - Summary of 2nd round discussion

Based on the inputs, it seems that majority is fine with the proposal including highlights the default behavior. Then, the following proposal will be recommended for email approval for the 1st check point.

Updated Proposal 3-1_v2: For the flexible symbol based on the semi-static configuration (e.g., TDD-UL-DL-ConfigCommon, TDD-UL-DL-ConfigDedicated), the default behavior of the NCR-Fwd is expected to be OFF or not forwarding over these symbols

- FFS: The behavior over these symbol if dynamic DL/UL operation is supported by both NCR-MT and NCR-Fwd.

- Others [Closed]

In addition, some other aspects are highlighted by companies including:

- Issue-1: Power control information

In RAN1#110, regarding the power control information, no conclusion has been achieved. Moreover, after extensive discussion, following scope is highlighted:

Specify the signalling and behavior of the following side control information for controlling the NCR-Fwd [RAN1, RAN2] - Beamforming - UL-DL TDD operation - ON-OFF information Note: Power control aspect will be checked in RAN#98e.

In this meeting, [Huawei, CMCC, ETRI, Lenovo, CEWiT] highlights that power control needs to be specified as side control information, e.g., via semi-static signalling. [Huawei] also highlights that one or multiple amplifying gains can be indicated by RRC and DCI, and one amplifying gain is indicated for each forwarding occasion.

[Apple, DCM] proposed that power control as side control information can be considered with low priority and before power control is agreed to be included in WID in RAN plenary, RAN1 may not need to discuss power aspect [Ericsson].

From FL’s perspective, it’s better to follow the conclusion in RAN plenary, and no further discussion on Power control is expected before RAN#98.

- Issue-2: Multiple-band operation of NCR-Fwd.

In this meeting, [Spreadtrum] propose to clarify if multi-band operation is supported, whether the side control information is applied to all of the operating bands, or it is only applied to the band(s) which have the same frequency bands as NCR-MT. [CEWiT] mentions that gNB indicates along with SCI in which carrier of NCR-Fwd the SCI is applicable.

More specifically,

- [Spreadtrum, NEC, QC, Ericsson, Intel] propose to support the frequency-selective ON-OFF information indication, e.g., per carrier-group or sub-band;

- [Samsung, Ericsson, LGE] proposed to support the beam indication per carrier or RB-group.

From FL’s perspective, in Rel-17, the multi-band repeater is only supported for FR1 repeater type 1-C with following definition in TS 38.106, and it’s not supported for FR2 repeater type 2-O.

Multi-band repeater: Repeater Type 1-C whose antenna connector is associated with a transmitter and/or receiver that is characterized by the ability to process two or more passband(s) in common active RF components simultaneously, where at least one passband is configured at a different operating band than the other passband(s) and where this different operating band is not a sub-band or superseding-band of another supported operating band

Based on the above definition, since the ‘common active RF components’ is assumed for multiple-band in FR1, it may not be feasible to support the frequency-selective ON-OFF indication.

For the beam indication, as captured in the WID (copied below):

Note: The work in RAN4 for beam related is expected to start on FR2 first.

Before concluding the discussion on beam in FR1 or support the multi-band operation for Type 2-O for FR2 in RAN4, it seems that the frequency-dependent beam indication is also not feasible.

With the consideration above, it’s recommended to postpone the discussion on the additional enhancement on side control information indication with finer granularity until further progress has been made in RAN4.

- Issue-3: Timing relationship

Based on the agreement in SI, [CMCC, Samsung, LGE, CEWiT, vivo] proposed to report the internal delay as NCR capability. In addition, [Samsung] highlights that DL internal delay and UL internal delay is the same.

[vivo] proposed to capture the following NCR behaviors to determine DL timing and UL timing in the specification.

- The DL receiving timing of the NCR-Fwd is aligned with the DL receiving timing of the NCR-MT.

- The DL transmitting timing of the NCR-Fwd is delayed after the DL receiving timing of the NCR-MT (or the NCR-Fwd) by the internal delay.

- The UL receiving timing of the NCR-Fwd is advanced before the UL transmitting timing of the NCR-MT (or the NCR-Fwd) by the internal delay.

- The UL transmitting timing of the NCR-Fwd is aligned with the UL transmitting timing of the NCR-MT.

[Rakuten] proposes to let RAN4 to confirm feasible internal delay value to check necessity of the special handling for timing alignment.

Based on the discussion in SI phase, it seems that others prefer to keep the internal delay as the value claimed by vendor (e.g., follow the Rel-17 RF repeater) instead of part of capability. Meanwhile, since the timing relationship is out of scope of WI for further enhancement, it means that no specification efforts are expected.

Then From FL’s perspective, companies are encouraged to further check the necessity to define the internal delay as part of NCR capability.

- Issue-4: Enabling of the feedback

One company proposed to discuss the feedback mechanism from NCR to gNB with following options:

  - Opt1: NCR sends feedback to gNB based on decoding of SCI

  - Opt2: NCR sends feedback to gNB based on NCR-Fwd operation

  - Opt3: NCR sends periodic feedback about the operating state to the gNB

From FL’s perspective, it’s reasonable to discuss this aspect to ensure the controllable behaviour of NCR. As part of procedure, this topic can be discussed in AI 9.8.2.

- Issue-5: NSA operation

Single company proposes to conclude that NSA operation is supported provided the repeater-MT is capable of NSA operation without additional specification impact from this mode of operation.

More specifically, a subset of the repeater configuration can typically be received on a primary carrier, provided the repeater-MT is DC capable in line with the network configuration, e.g., TDD UL/DL TDD configurations, and remaining configurations, including dynamic indications, can be received on the inband secondary carrier.

From FL’s perspective, it’s reasonable to consider the potential extension of use case for NCR without additional enhancement. Companies are encouraged to share the view on this topic further.

Companies are encouraged to share your views if any

| Companies | Comments and Views |
| --- | --- |
| Ericsson | Issue 2: Subband operation should not be confused with multi-band operation. Subband operation should be discussed since it will severely influence the future work, e.g., in the dynamic beam indication. Without subband operation, efficiency of NCR will be drastically reduced since it would imply the same beam indication is applied to the whole Fwd bandwidth. Issue 4: We don’t think feedback is required but are fine to discuss it. Issue 5: Support, in particular since FR2 networks are to a large extent deployed as NSA. |
| Intel | For Issue 5, we’d like to understand, whether any enhancement for LTE leg is assumed for NSA case, e.g., side control information to be transmitted by LTE gNB and side control information is to be processed by LTE part of NCR-MT? And it is very likely additional RRC and X2 signaling would be required, which has large impact on RAN2. Maybe RAN2 should discuss this rather than RAN1? |
| AT&T | Agree with the FL perspective on Issue 5. |
| Vivo | We think discussion on the “error report” by NCR is necessary, e.g., when self-oscillation occurs, related information is reported to gNB. |
| ZTE | For the Issue-5, in our understanding, the leg from LTE is only to keep the connection/access of NCR-MT. Once it’s connected, all side control information will be delivered over NR-leg. We can also check this issue with RAN2. |
| CEWiT | Issue 2: we share similar view with Ericsson. Suband operation should be discussed. NCR-MT, handling just control signals, should not operate on the entire BW used by NCR-Fwd. Issue 4: In our understanding HARQ feedback is essential for NCR. We are fine to discuss it in 9.8.2 |
| IIT-K | Issue 4: We think feedback is essential. We are fine to discuss in 9.8.2 |
| Qualcomm | Issue 1: we do understand that power control is not strictly in the scope of RAN1. However, we believe RAN1 should further investigate the issue from the “technical” perspectives to identify the its possible necessities and benefits. RAN plenary is not expected to have “technical” discussions, and ideally RAN1 should have an input to the next RP meeting. So, we propose to encourage companies to further check power control aspects. Issue 2: we agree with Ericsson and propose to further discuss this aspect. Issue 4: we agree with FL’s suggestion to further discuss feedback mechanisms. |
| Philips | Issue 4: We would support further discussion of feedback. |
| Moderator’s View | Based on the feedback： For Issue-2: we can firstly check with the feasibility with RAN4 before touching the details; For issue-3: lack of inputs and let’s check it later; For issue-4: It will be handled in 9.8.2. For issue-5: Wait for the inputs in RAN2. |

- Proposals for discussion at GTW sessions

Updated Proposal 1-1-1-B_v2: At least the following information is used to characterize the physical beam(s) supported by NCR-Fwd for access link:

- Number of beams supported for access link

  - FFS: How to define the detailed value (e.g., per beam type)

  - FFS: Whether the number of beam can be derived by beam layout

- Beam types defined by the beam width (i.e., two types as wide beam and narrow beam type)

- Beam layout defined by the spatial relationship between different beams in same or different beam type

- FFS: Beam direction defined by the boresight of beam

- FFS: How to deliver this information to gNB

Updated Proposal 1-1-2: One of following options is supported to define the association between beam index and the physical beam supported by NCR-Fwd for access link:

- Option-1: All beams are indexed sequentially regardless of the beam type for each beam

- Option-2: Beams are independently indexed per beam type(i.e., all narrow/wide beams belong to the same type are indexed sequentially)

- Option-3: Beams are hierarchically indexed according to the spatial relationship between different beam types (i.e., narrow beams associated with the same wide beam are sequentially indexed, and different wide beams are sequentially indexed)

FFS: the sequence of beam

Updated Proposal 1-1-3: One of following options is supported to determine the beam index used in SCI to control the beam of NCR-Fwd for access link:

- Option-1: The beam indices are configured by gNB

  - FFS: How to inform the NCR on which beams are selected and/or indexed by gNB

- Option-2: The beam indices are reported by NCR

  - Note: In this option, the reported beam index of supported beam will be used for beam indication.

- Option-3: The beam indices are determined by the pre-defined rule.

  - Note: In this option, same rule is applied at both gNB and NCR sides to identify the unique physical beam supported by NCR for each beam index.

Note: Combination between options is not precluded.

Updated Proposal 1-1-4_v2: The following methods are supported for beam indication:

- Single beam index per indication is supported to indicate one beam.

- The indication of multiple beam in one indication is supported via one of following options:

  - Option-1: Multiple beam indices

  - Option-2: Beam sequence index, where the beam sequence is pre-configured and each one is defined as a list of beam indices.

  - Note: The multiples beam are applied in TDMed over same frequency resource

Updated Proposal 1-1-6_v2: Following parameters should be supported to define the time resource:

- For periodic and semi-persistent configuration of time resource, starting time, duration per beam(s) and periodicity is needed.

- For aperiodic indication of time resource, starting time and duration per beam(s) is needed.

- FFS: The SCS for starting time, duration, and periodicity

- FFS: How to define the duration, e.g., via the length of time resource or resource index

- FFS: Whether indication of starting time can be implicit (e.g., the first slot after the time to apply the received beam indication)

- Note: These parameters applied to each configuration/indication is to define one set of time resource.

Updated Proposal 1-2-1: If adaptive beams are adopted for C-link and backhaul link, the new signaling is supported to indicate a beam(s) used for backhaul link from the set of beams for C-link.

- FFS: How to determine the beam for backhaul link if no dedicate beam for backhaul link is indicated；

- FFS: How to determine the beam if simultaneously transmission of C-link and backhaul link is performed

- Note: The beam(s) used for backhaul link should be from the RRC-configured list of beams for C-link.

Updated Proposal 2-1_v2: For the ON/OFF information indication:

- Explicit OFF is supported

  - At least the semi-static indication via RRC and/or MAC CE is supported.

- One of following options is supported to indicate the ON state of NCR

  - Alt-1: Explicit indication at least if beam indication is not applicable

  - Alt-2: Implicit indication via the beam indication

  - Alt-3: Implicit indication via the time domain resource indication (i.e., the NCR is assumed to be ON over the indicated time domain resource)

- Conclusion

- Appendix

R1-2208422	Side control information and behaviors for network-controlled repeaters	Huawei, HiSilicon

R1-2208563	Discussion on side control information and NCR behavior	Spreadtrum Communications

R1-2208656	Discussion on side control information and NCR behavior	vivo

R1-2208684	Discussions on side control information for NR network-controlled repeaters	InterDigital, Inc.

R1-2208691	Discussion on side control information for NCR	ZTE

R1-2208778	Discussion on side control information and NCR behavior	China Telecom

R1-2208989	Discussion on side control information and NCR behavior	CATT

R1-2209018	Discussion on side control information and NCR behavior	Fujitsu

R1-2209065	Discussion on Side control information and NCR behaviour	Intel Corporation

R1-2209086	Discussion on side control information and NCR behavior	Nokia, Nokia Shanghai Bell

R1-2209089	Signalling support for NR network-controlled repeaters	AT&T, FirstNet

R1-2209110	Considerations on side control information and NCR behaviour	Sony

R1-2209154	Discussion on side control information to enable NR network-controlled repeaters	NEC

R1-2209238	Discussion on side control information and NCR behavior	CAICT

R1-2209297	Discussion on side control information and NCR behavior	Xiaomi

R1-2209350	Discussion on side control information and NCR behavior	CMCC

R1-2209378	Discussion on side control information and NCR behavior	Panasonic

R1-2209408	Discussion on side control information for network-controlled repeater	ETRI

R1-2209511	Side control information and NCR behavior	MediaTek Inc.

R1-2209593	On side control information and NCR behavior	Apple

R1-2209664	Side control information and configuration for NCR behavior	Sharp

R1-2209744	Discussion on side control information and NCR behavior	Samsung

R1-2209763	Discussion on UL-DL timing alignment in NCR-Fwd	Rakuten Mobile, Inc

R1-2209811	Discussion on side control information and NCR behavior	LG Electronics

R1-2209915	Discussion on Side control information and NCR behavior	NTT DOCOMO, INC.

R1-2209998	On side control information and NCR behavior	Qualcomm Incorporated

R1-2210022	Discussion on side control information and NCR behaviour	Lenovo

R1-2210068	Discussion on side control information and NCR behavior	KDDI Corporation

R1-2210114	Discussion on signaling side control information and NCR behavior	CEWiT, IITK

R1-2210231	Side-control information and behavior in NCR	Ericsson