3GPP TSG-RAN WG1 Meeting #110bis-e	R1-2210573

# e-Meeting, October 10– 19, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev |  | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PUCCH resource determination of multicast HARQ-ACK |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Moderator (Huawei), Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R1 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-10-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The specifications are currently missing the resource determination for a PUCCH with HARQ-ACK for multicast DCI-based PDSCHs and HARQ-ACK for multicast SPS PDSCHs when corresponding HARQ-ACK reporting modes are different (ACK/NACK and NACK-only mode 2). |  |  |  |  |  |  |  |  |
| Summary of change: |  | If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and the first and second HARQ-ACK information are indicated by harq-FeedbackOptionMulticast different HARQ-ACK reporting modes and moreThanOneNackOnlyMode is not provided, the UE determines the PUCCH resource based on: - sps-PUCCH-AN-ListMulticast, or sps-PUCCH-AN-List if sps-PUCCH-AN-ListMulticast is not provided, if the first HARQ-ACK information is according to the first HARQ-ACK reporting mode or - the last multicast DCI format, as described in clause 9.2.3, if the second HARQ-ACK information is according to the first HARQ-ACK reporting mode. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE behaviour is unclear regarding PUCCH resource determination for multiplexing HARQ-ACK for multicast DCI-based PDSCHs and HARQ-ACK for multicast SPS PDSCHs when corresponding HARQ-ACK reporting modes are different (ACK/NACK and NACK-only mode 2). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# 18	Multicast Broadcast Services

< Unchanged parts are omitted >

If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and both the first and second HARQ-ACK information are according to the first HARQ-ACK reporting mode, the UE determines the PUCCH resource based on the last multicast DCI format, as described in clause 9.2.3.

If a UE multiplexes in a PUCCH only first HARQ-ACK information associated with multicast SPS PDSCH receptions and second HARQ-ACK information associated with multicast DCI formats and having same priority value as the first HARQ-ACK information, and the first and second HARQ-ACK information are indicated by harq-FeedbackOptionMulticast different HARQ-ACK reporting modes and moreThanOneNackOnlyMode is not provided, the UE determines the PUCCH resource based on:

-	sps-PUCCH-AN-ListMulticast, or sps-PUCCH-AN-List if sps-PUCCH-AN-ListMulticast is not provided, if the first HARQ-ACK information is according to the first HARQ-ACK reporting mode or

-	the last multicast DCI format, as described in clause 9.2.3, if the second HARQ-ACK information is according to the first HARQ-ACK reporting mode.

< Unchanged parts are omitted >