3GPP TSG RAN WG1 #111	R1-2211082

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.9.1

Source: 	Fujitsu

Title:	Consideration on multi-cell PUSCH/PDSCH scheduling with a single DCI

Document for:		Discussion and decision

- Introduction

In RAN#97-e meeting, the following conclusion was made for multi-cell PUSCH/PDSCH scheduling in Rel-18:

- Deprioritize any optimization for unlicensed spectrum operation for designing the multi-cell PUSCH/PDSCH scheduling in Rel-18 - Enhanced Type-2 HARQ-ACK codebook is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18. - Type-1 HARQ-ACK codebook is supported only for the case where co-scheduled cells by a DCI format 1_X have same SCS/carrier type/duplex mode in Rel-18. Additional restriction(s) can be discussed in RAN1 - Configuring more than one scheduling cell for DCI format 0_X/1_X for each scheduled cell is not supported for the multi-cell PUSCH/PDSCH scheduling in Rel-18. - Following aspects are excluded from multi-cell PDSCH/PUSCH scheduling in Rel-18: - SCell schedules multiple cells including P(S)Cell - Different SCS among co-scheduled cells - Different carrier type (licensed or unlicensed, FR1 or FR2-1 or FR2-2) among co-scheduled cells - Configuration of both multi-cell PDSCH/PUSCH scheduling and multi-TRP for a scheduled cell - Support for any sidelink scheduling - PCell schedules multiple cells by DCI format 0_X/1_X when a sSCell is configured to schedule PCell

Based on the conclusion and the agreements in previous RAN1 meetings, this contribution will discuss how to support multi-cell PUSCH/PDSCH scheduling.

- Discussion

2.1 CSI request

Agreement For DCI format 1_X/0_X, Type-1 fields at least include the following: - Priority indicator - Indicator of co-scheduled cells - beta offset indicator - CSI request - UL-SCH indicator - FFS: ChannelAccess-CPext

According to the agreement above in RAN1#110b-e meeting, DCI format 0_X will include a single CSI request field. Therefore, DCI format 0_X can trigger CSI report.

However, how to support triggering of CSI report by DCI format 0_X is not clear yet. Since this is the final meeting of the WI, a complete solution is necessary. For example, the two options as below are straightforward. However, Option 2 only works when DCI format 0_X supports single-cell scheduling, which will restrict gNB’s implementation. Therefore, Option 1 is preferred.

- Option 1: DCI format 0_X can schedule one or more cells when triggering CSI report. The triggered CSI report is transmitted on the schedule cell with lowest serving cell identifier.

- Option 2: DCI format 0_X can schedule only one cells when triggering CSI report. The triggered CSI report is transmitted on the scheduled cell.

Proposal 1: For CSI request by DCI format 0_X, consider the following two options. Option 1 is preferred for flexibility.

- Option 1: DCI format 0_X can schedule one or more cells when triggering CSI report. The triggered CSI report is transmitted on the schedule cell with lowest serving cell identifier.

- Option 2: DCI format 0_X can schedule only one cells when triggering CSI report. The triggered CSI report is transmitted on the scheduled cell.

2.2 Indication of scheduled cells

In RAN1#109-e meeting, the following agreement was made for indication of scheduled cells.

Agreement For multi-cell scheduling, the co-scheduled cells are indicated by DCI format 0_X/1_X. At least the following options are considered: - Option 1: An indicator in the DCI points to one row of a table defining combinations of scheduled cells. - The table is configured by RRC signaling. - FFS: Separate tables can be configured for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - Option 2: An indicator in the DCI is a bitmap corresponding to a set of configured cells that can be scheduled by the DCI 0_X/1_X - FFS: Separate sets of configured cells for multi-cell PDSCH scheduling and multi-cell PUSCH scheduling. - Option 3: using existing field (e.g., CIF, FDRA) to indicate whether one or more cells are scheduled or not - Other options are not precluded. - Note: It does not preclude other DCI information fields (e.g., BWP) to be jointly indicated by the indicator of the co-scheduled cells.

Compared with Option 2, Option 1 can reduce DCI overhead. Therefore, Option 1 is slightly preferred. As for Option 3, we think it is straightforward to combine it with Option 1. That is, the CIF can be the indicator mentioned in Option 1.

Proposal 2: For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of scheduled cells.

- The indicator is CIF.

- The table is configured by RRC signaling.

2.3 PDCCH candidate set

In the legacy, PDCCH candidate set is determined according to the formula as below.

L⋅Yp,ns,fμ+ms,nCI(L)⋅NCCE,pL⋅Ms,maxL+nCImodNCCE,pL+i

According to the working assumption in RAN1#110b-e meeting, for DCI format 0_X/1_X, the search space is configured on one cell of the set of cells. In our view, it may imply that the number of PDCCH candidates per AL is configured per set of cells, since the number of PDCCH candidates per AL is a part of the search space configuration.

Working Assumption For a set of cells which is configured for multi-cell scheduling, - Existing DCI size budget is maintained on each cell of the set of cells. - DCI size of DCI format 0_X/1_X is counted on one cell among the set of cells. - FFS which cell DCI size of the DCI format 0_X/1_X is counted on. - BD/CCE of DCI format 0_X/1_X is counted on one cell among the set of cells. - FFS which cell BD/CCE of the DCI format 0_X/1_X is counted on. - Search space of DCI format 0_X/1_X is configured on one cell of the set of cells and associated with the search space of the scheduling cell with the same search space ID. - FFS which cell the SS of the DCI format 0_X/1_X is configured on. - FFS: How to address Rel-17 BD/CCE limit for any given cell (operating the feature under Rel-17 BD/CCE limit) - Note: This does not mean a UE is required to support number of BDs/CCEs beyond the Rel-17 limits (i.e., MPDCCHmax,slot,μ, CPDCCHmax,slot,μ, MPDCCHtotal,slot,μ and CPDCCHtotal,slot,μ) for PDCCH candidates for each scheduled cell.

In the meantime, for the value of nCI, the following 2 options were discussed.

- Option 1: the value of nCI is per combination of scheduled cells

  - the value of nCI is equal to the value of the indicator in DCI format 0_X/1_X for indicating scheduled cells.

- Option 2: the value of nCI is configured per set of cells that can be scheduled by the DCI format 0_X/1_X

  - The set of cells includes all cells in the table defining combinations of scheduled cells.

Assuming the number of PDCCH candidates per AL is configured per set of cells according to the working assumption, Option 2 should be supported so that the legacy formula can be reused.

And if the working assumption does not mean that the number of PDCCH candidates per AL is configured per set of cells, it would be fine to support Option 1 for nCI and the number of PDCCH candidates per AL should be per combination of scheduled cell as well to reuse the legacy formula. However, it may complicate discussion on BD/CCE counting.

Considering this is the final meeting of the WI and the TU is very limited, Option 2 is preferred.

Proposal 3: For determining PDCCH candidates for multi-cell scheduling, both the value of nCI and the number of PDCCH candidates per AL are per set of cells that can be scheduled by the DCI format 0_X/1_X. The set of cells includes all cells in the table defining combinations of scheduled cells.

- Conclusion

According to the discussions above, we have the following proposals for multi-cell PUSCH/PDSCH scheduling with a single DCI.

Proposal 1: For CSI request by DCI format 0_X, consider the following two options. Option 1 is preferred for flexibility.

- Option 1: DCI format 0_X can schedule one or more cells when triggering CSI report. The triggered CSI report is transmitted on the schedule cell with lowest serving cell identifier.

- Option 2: DCI format 0_X can schedule only one cells when triggering CSI report. The triggered CSI report is transmitted on the scheduled cell.

Proposal 2: For multi-cell scheduling, the co-scheduled cells are indicated by an indicator in DCI format 0_X/1_X which points to one row of a table defining combinations of scheduled cells.

- The indicator is CIF.

- The table is configured by RRC signaling.

Proposal 3: For determining PDCCH candidates for multi-cell scheduling, both the value of nCI and the number of PDCCH candidates per AL are per set of cells that can be scheduled by the DCI format 0_X/1_X. The set of cells includes all cells in the table defining combinations of scheduled cells.