3GPP TSG RAN WG1 Meeting #111	R1-2211095

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	9.11.3

Source:	MediaTek Inc.

Title:	Disabling of HARQ for IoT NTN

Document for:	Discussion and Decision

- 1 Introduction

In RAN#96e [1], the revised WID on IoT NTN enhancements was endorsed for Release 18. Rel-17 IoT-NTN is considered as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objective for disabling of HARQ feedback.

According to the WID, disabling of HARQ feedback for IoT NTN has been agreed in RAN meetings.

This work considers Rel-17 IoT-NTN as baseline as well as Rel-17 NR-NTN outcome and the further IoT-NTN performance enhancements objectives are listed below: - Disabling of HARQ feedback to mitigate impact of HARQ stalling on UE data rates [RAN1, RAN2] - Study and specify, if needed, improved GNSS operations for a new position fix for UE pre-compensation during long connection times and for reduced power consumption. Simultaneous GNSS and NTN NB-IoT/eMTC operation is not assumed. [RAN1] NOTE: The need for RAN4 Core requirements for this objective will be identified after the conclusion on the need for improvements.

This contribution aims to discuss aspects related to disabling of HARQ feedback for IoT NTN.

- 2 Analysis of DL peak data rates in NB-IoT NTN

In LTE eMTC and NB-IoT specifications, the HARQ process with adaptive retransmissions is asynchronous on DL and UL. In both directions, the receiver tries to decode the transport block (TB) with soft combining of HARQ re-transmissions. In case the transmission is a new TB, the soft buffer should be flushed. In case a retransmission occurred, soft combining of current HARQ re-transmission and previous HARQ transmissions should be performed. The UE knows a scheduled TB is re-transmitted from the new data indicator (NDI) as indicated on DCI.

In satellite systems, larger values of RTD can be typically experienced due to the large channel propagation delay between the UE and the satellite, which depends on the satellite orbit and the elevation angle. HARQ stalling occurs due to the longer satellite RTT between the device and the network, where the UE needs to wait for the reception of a packet or the UL grant to transmit a new packet with HARQ re-transmissions.

In SI phase of IoT NTN, link budget results have been captured and summarized in TR 36.763 [2]. The link budget results would suggest that at very low SNR, repetition levels of 16 or 32 with TBS mapped to 10 RUs will be sufficient in most scenarios.

In cellular NB-IoT, the max TBS is 2536 bits with QPSK modulation as shown in Appendix. Figure 1 illustrates the scheduling of DL packets with 2 HARQ processes. The DCI#1 scheduled NPDSCH #1 can be transmitted during the scheduling delay of the DCI#0 and scheduled NPDSCH #0, and network schedules the NPDSCH #1 right next to NPDSCH #0.

Figure 1: 2 HARQ processes for legacy NB-IoT

Table 1 shows DL maximum data rates of NB-IoT for Cellular, LEO, and GEO with HARQ feedback enabling with Max TBS=2536 bits with QPSK. With repetitions scheduled by the eNB, the impact of the satellite RTD on the NB-IoT data rates in GEO due to HARQ stalling is significant with a 61% reduction of peak data rate observed with 16 repetitions and 10 RUs per TBS compared to cellular (this means TBS is transmitted over 16*10 RUs = 160 ms assuming SCS=15 kHz); for LEO, it is not significant with a 7% reduction of peak data rate observed with 16 repetitions. Without repetitions, the impact of the satellite RTD on the NB-IoT data rates in GEO and LEO is significant. For LEO it is relatively smaller than that in GEO.

Observation 1: Peak data rate reduction with enabling HARQ in NB-IoT NTN scenarios becomes lower with higher repetitions used.

Table 1: peak data rate of enabling HARQ

|  | 2-HARQ Processes TBS=2536 bits |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | DL without repetitions for NPDCCH and NPDSCH | DL with 4 repetitions for NPDCCH and NPDSCH | DL with 8 repetitions for NPDCCH and NPDSCH | DL with 16 repetitions for NPDCCH and NPDSCH | DL with 32 repetitions for NPDCCH and NPDSCH |
| Cellular | 126.8 kbps | 49.2 kbps | 27.1 kbps | 14.3 kbps | 7.3 kbps |
| LEO | 74.6 kbps (~41% reduction) | 38.7 kbps (~21% reduction) | 23.6 kbps (~13% reduction) | 13.2 kbps (~7% reduction) | 7.1 kbps (~3% reduction) |
| GEO | 8.7 kbps (~93% reduction) | 7.8 kbps (~84% reduction) | 6.9 kbps (~74% reduction) | 5.6 kbps (~61% reduction) | 4.1 kbps (~44% reduction) |

- 3 Configure enabling/disabling on HARQ feedback in IoT NTN

Disabling of HARQ feedback was discussed in last few meetings and agreements are made in RAN1 and RAN2 for options of disabling HARQ feedback in Rel-18 IoT NTN.

Agreement (RAN1 109) For IoT NTN, to configure/indicate enabling/disabling on HARQ feedback for downlink transmission, one or more of the following options can be considered: - Option 1: per HARQ process via UE specific RRC signaling - Option 2: per HARQ process via SIB signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly determined by existing configured/indicated parameter(s) (e.g., repetition number, TBS) - Option 5: per HARQ process via MAC CE - Other options or combinations are not excluded Note: Option(s) for eMTC and NBIoT can be separately discussed. Agreement (RAN1 110) For eMTC NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling. - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field). - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above. Agreement (RAN1 110) For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select one or more from the following options: - Option 1: per HARQ process via UE specific RRC signaling - Option 3: explicitly indicated by DCI (e.g., new field or reusing existing field) - Option 4: implicitly indicated by existing configured/indicated/combined parameter(s) in the DCI (e.g., repetition number, TBS) - Option 6: combinations of some options above Agreement (RAN1 110b) For NB-IoT NTN, to configure/indicate enabling/disabling of HARQ feedback for downlink transmission, down select ONE from the following options at RAN1#111: - Option 6a-1: Support RRC signaling configured between Option 1 and Option 3 - Option 6a-4: Support Option 1 by default, and support Option 3 to override default configuration for corresponding transmission Agreements (RAN2 119): - Disabling DL HARQ feedback is supported for NB-IoT and eMTC NTN. FFS on UE capability - For UL HARQ operation, introduce two HARQ modes, i.e., HARQ mode A and HARQ mode B in IoT NTN (both NB-IoT and eMTC NTN), similarly to NR NTN - From RAN2 perspective, at least for eMTC, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signalling. FFS for NB-IoT (and especially for CP solution for NB-IOT). Agreements (RAN2 119b): - For NB-IoT, enabling/disabling HARQ feedback can be configured per DL HARQ process at least via UE specific RRC signaling (e.g. RRCConnectionSetup). This does not preclude other options (e.g. DCI-based). We can also revert this decision if requested by RAN1. - Disabling HARQ feedback is supported for NB-IoT with single HARQ process, and it is up to eNB implementation whether to disable the HARQ feedback - Working Assumption: Blind retransmission can be used in IoT NTN when HARQ feedback is disabled and when HARQ mode B is used (RAN2 assumes there is no spec change for this)

For disabling of HARQ feedback in IoT NTN, enabling/disabling HARQ feedback per HARQ process via UE specific RRC signaling is aligned with RAN2 agreements with low specification impact. As IoT device is more delay tolerant, enabling/disabling HARQ feedback per HARQ process via UE specific RRC signaling is enough to ensure network flexibility in HARQ feedback enabling/disabling scheduling.

Proposal 1: Enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signalling.

There may be the case when the HARQ process is utilized for MAC CE but HARQ feedback is disabled for the HARQ process by UE specific RRC signaling. The network can override RRC disabled configuration per HARQ process per UE by explicit indication via DCI with re-interpreted fields.  The DCI override can be enabled by a higher layer parameter npdsch-HarqFeedbackDisableOverride-Config configured via UE-specific RRC signalling.

Proposal 2: Explicit indication via DCI to override RRC configuration with enabled HARQ feedback for DL transmission per HARQ process per UE is supported if higher layer parameter npdsch-HarqFeedbackDisableOverride-Config is enabled by UE-specific RRC signalling.

The network can enable HARQ feedback for the HARQ process if repetitions are 16 or higher as needed to close link budget. Based on analysis provided in the previous section, which showed that disabling HARQ feedback has marginal impact on peak data rates for larger repetitions. This is the simplest way to override the RRC configuration.

If “HARQ-ACK resource” field ≠ ‘0000’, the network can override RRC disabled configuration with enabled HARQ feedback for the HARQ process. In other words, if “HARQ-ACK resource” field = ‘0000’, then RRC configuration for enable/disable HARQ feedback is used. This seems reasonable way considering that if the HARQ feedback is disabled, then the “HARQ-ACK resource” field becomes redundant since HARQ feedback is not transmitted. This seems a reasonable restriction of the scheduling for transmission of HARQ feedback in case RRC configuration for enable/disable HARQ feedback is override.

The restriction of the scheduling with “HARQ-ACK resource” field = ‘0000’ for transmission of HARQ feedback, when override RRC disabled configuration with enabled HARQ feedback for the HARQ process, could be further mitigated in case HARQ feedback is enabled if repetitions are 16 or higher.

The impact of explicit indication by DCI with reusing existing field to the specifications could be as follows:

- If RRC configuration for HARQ process enable HARQ feedback, then legacy DCI is used for HARQ

- If repetition field indicates 16 repetitions or higher, HARQ feedback for HARQ process is enabled (for any value of “HARQ-ACK resource” field)

- If repetition field indicates fewer than 16 repetitions, and “HARQ-ACK resource” field ≠ ‘0000’, HARQ feedback for HARQ process is enabled

- If repetition field indicates fewer than 16 repetitions, and “HARQ-ACK resource” field = ‘0000’, RRC configuration for HARQ feedback for HARQ process is used

Hence, we make the following proposal:

Proposal 3: Explicit indication via DCI with re-interpreted fields to override RRC configuration to enable HARQ feedback for DL transmission per HARQ process

- If “Repetition number” field is ‘0100’ or above – i.e. 16 repetitions or higher

- If “HARQ-ACK resource” field ≠ ‘0000’.

Note: “HARQ-ACK resource” field = ‘0000’, then RRC configuration for enable/disable HARQ feedback is used

# 4 Conclusion

In this contribution, the following proposals were made

Observation 1: Peak data rate reduction with enabling HARQ in NB-IoT NTN scenarios becomes lower with higher repetitions used.

Proposal 1: Enabling/disabling on HARQ feedback for downlink transmission is configurable per HARQ process via UE specific RRC signalling.

Proposal 2: Explicit indication via DCI to override RRC configuration with enabled HARQ feedback for DL transmission per HARQ process per UE is supported if higher layer parameter npdsch-HarqFeedbackDisableOverride-Config is enabled by UE-specific RRC signalling.

Proposal 3: Explicit indication via DCI with re-interpreted fields to override RRC configuration to enable HARQ feedback for DL transmission per HARQ process

- If “Repetition number” field is ‘0100’ or above – i.e. 16 repetitions or higher

- If “HARQ-ACK resource” field ≠ ‘0000’.

Note: “HARQ-ACK resource” field = ‘0000’, then RRC configuration for enable/disable HARQ feedback is used

# 5 References

- RP-221806, Moderator (MediaTek), Revised WID on IoT NTN enhancements, June 6th – 9th, 2022

- TR 36.763, Release 17

- R1-2205555, Lenovo, Feature lead summary #3 on disabling of HARQ feedback for IoT NTN

- R1-1912125, MediaTek, Delay-tolerant re-transmission mechanisms in NR-NTN, RAN1#99, November 2019

# 6 Appendix

TS 36.213 Table 16.5.1.2-2: Transport block size (TBS) table for NPUSCH.

|  | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 16 | 32 | 56 | 88 | 120 | 152 | 208 | 256 |
| 1 | 24 | 56 | 88 | 144 | 176 | 208 | 256 | 344 |
| 2 | 32 | 72 | 144 | 176 | 208 | 256 | 328 | 424 |
| 3 | 40 | 104 | 176 | 208 | 256 | 328 | 440 | 568 |
| 4 | 56 | 120 | 208 | 256 | 328 | 408 | 552 | 680 |
| 5 | 72 | 144 | 224 | 328 | 424 | 504 | 680 | 872 |
| 6 | 88 | 176 | 256 | 392 | 504 | 600 | 808 | 1000 |
| 7 | 104 | 224 | 328 | 472 | 584 | 712 | 1000 | 1224 |
| 8 | 120 | 256 | 392 | 536 | 680 | 808 | 1096 | 1384 |
| 9 | 136 | 296 | 456 | 616 | 776 | 936 | 1256 | 1544 |
| 10 | 144 | 328 | 504 | 680 | 872 | 1000 | 1384 | 1736 |
| 11 | 176 | 376 | 584 | 776 | 1000 | 1192 | 1608 | 2024 |
| 12 | 208 | 440 | 680 | 1000 | 1128 | 1352 | 1800 | 2280 |
| 13 | 224 | 488 | 744 | 1032 | 1256 | 1544 | 2024 | 2536 |

TS 36.211 Table 16.4.1.3-2: Number of repetitions () for NPDSCH.