3GPP TSG RAN WG1 #111 R1-2211455 Toulouse, France, November 14th – 18th, 2022 CR-Form-v12.0. CHANGE REQUEST. 37.213; CR; rev; -; Current version:; 17.3.0. For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests.. Proposed change affects:; UICC apps; ME; X; Radio Access Network; X; Core Network. Title:; Draft CR on resolving issue for short control signaling. Source to WG:; OPPO. Source to TSG:; R1. Work item code:; NR_ext_to_71GHz-Core; Date:; 2022-11-14. Category:; C; Release:; Rel-17. Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900.; Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16). Reason for change:; Currently it is unclear from the specification text, how to disable this exempted transmission when this is not supported based on regional regulatory requirements. In order to solve this issue, the following CR is proposed.. -. Summary of change:; Remove the feature of SCSt based msg1/msgA transmission.. Consequences if not approved:; unclear specifications. Clauses affected:; 4.4.5. Y; N. Other specs; X; Other core specifications; TS/TR ... CR .... affected:; X; Test specifications; TS/TR ... CR .... (show related CRs); X; O&M Specifications; TS/TR ... CR .... Other comments:. This CR's revision history:; This is the first version for this CR..

- 4.4.5	Exempted transmissions from sensing

In regions where channel sensing is required to access a channel for transmission and short control signalling exemption is allowed by regulation, a gNB may transmit the following transmission(s) on a channel without sensing the channel:

-	Transmission(s) of the discovery burst by the gNB

When the gNB transmits the above transmission(s) without sensing on a channel by utilizing the exemption above, the total duration of such transmission(s) by the gNB shall not occupy the corresponding channel more than 10ms over any 100ms interval.