3GPP TSG RAN WG1 #111	R1-2211486

Toulouse, France, November 14th – 18th, 2022

Source:	OPPO

Title:	Discussion on potential enhancements on dynamic/flexible TDD

Agenda Item:	9.3.3

Document for:	Discussion and Decision

# Introduction

In RAN1 #110-b meeting, potential enhancements on dynamic/flexible TDD were discussed with the following agreement:

Agreement

For gNB-to-gNB co-channel CLI measurement, the potential benefit of uplink resources muting can be studied further.

Note: Proponents of uplink resource muting are encouraged to provide evaluation result for comparison of performance between two cases when uplink resource muting based gNB-gNB CLI handling schemes including both UE transparent and non-UE transparent schemes is applied or not.

Agreement

For gNB-to-gNB co-channel CLI measurement, consider as baseline reusing existing DL channel(s)/signal(s)/measurement_resource(s)

- For example, SSB, NZP/ZP-CSI-RS, DMRS for PDCCH/PDSCH, CSI-IM, RSSI measurement resource, etc.

- FFS: Which type of DL channel(s)/signal(s) can be used for gNB-to-gNB co-channel CLI measurement

- FFS: How resources are used/configured

Agreement

For details of spatial domain coordination method for gNB-to-gNB co-channel CLI handling, at least followings can be studied.

- Recommended/restricted Beams between gNBs

- Beam nulling between gNBs

- Beam pairing between gNBs

- Other schemes are not precluded.

Agreement

For UE-to-UE co-channel CLI handling, study L1/L2 based UE-to-UE CLI measurement and reporting

- Note: Accounting for UE processing/reporting delay – companies to share their assumptions

- Note: Proponents are encouraged to provide the mechanism of L1/L2 based CLI measurement and reporting, and to provide the benefits of L1/L2 based CLI measurement and reporting compared with existing L3 CLI/CSI measurement and report with evaluation result

- Note: Accounting for information exchange delay between gNBs (if applicable)

Agreement

For UE-to-UE co-channel CLI measurement, consider as baseline reusing existing channel(s)/signal(s)/measurement_resource(s)

- For example, SRS resources defined in Rel-16 for SRS-RSRP measurement, CLI-RSSI resources defined in Rel-16 for CLI-RSSI measurement

  - FFS potential enhancements

In this contribution, we share our views on the inter-gNB and inter-UE CLI issues.

# Discussion

# Inter-gNB CLI measurement

NZP-CSI-RS and CLI-RSSI resource are transmitted periodically. They are more suitable for inter-gNB CLI measurement to avoid frequent configuration information exchange over Xn interface. Furthermore, the CLI-RSSI resource configuration is very flexible, so all kinds of CLI measurement requires can be fully satisfied by reasonable configuration. Both short-term and long term inter-gNB CLI measurement can be considered, but we think short-term measurement, e.g. L1-based RSRP/RSSI, is more desirable to reflect the dynamic interference, and L1-based inter-gNB CLI measurement result with timestamp should be exchanged over Xn interface.

Proposal 1: CSI-RS and/or R16 CLI-RSSI resource can be used for inter-gNB CLI measurement.

Proposal 2: To support inter-gNB CLI measurement, L1-based RSRP/RSSI should be considered:

- L1-based inter-gNB CLI measurement result with timestamp should be exchanged over Xn interface.

# Inter-UE CLI measurement

In R16, SRS resources and CLI-RSSI resources are configured to UE by RRC signaling for L3-based SRS-RSRP and CLI-RSSI measurement. In our option, L1-based SRS-RSRP and L1-based CLI-RSSI are very straightforward for inter-UE CLI measurement to reflect short-term interference, and both R16 SRS resources and CLI-RSSI resources can be reused. The L1 CLI measurement results can be reported as a separated CSI report with the extension of R17 CSI reference resource definition to include the SRS resource and CLI-RSSI resource for inter-UE CLI measurement, then R15/16 CSI processing delay should be satisfied.

Proposal 3: RAN1 prioritizes L1-based SRS-RSRP and L1-based CLI-RSSI measurement for inter-UE CLI handling, and R16 configuration of SRS and CLI-RSSI resources can be reused.

Proposal 4: L1 inter-UE CLI measurement results can be reported as a separated CSI report:

- R17 CSI reference resource definition should be extended to include the SRS resource and CLI-RSSI resource for inter-UE CLI measurement;

- R15/16 CSI processing delay should be satisfied.

# Coordinated scheduling

Intended TDD DL-UL configurations exchange over Xn and F1 interfaces has been supported in R16, but the slot configuration list is limited in a 10-ms period. More flexible TDD DL-UL configuration exchange over Xn and F1 interfaces should be studied for R18 dynamic/flexible TDD, such as TDD DL-UL configuration with more than 10-ms period.

Proposal 5: To support coordinated scheduling between gNBs, more flexible TDD DL-UL configuration exchange over Xn/F1 interfaces should be studied, e.g. TDD DL-UL configuration with periodicity longer than 10-ms.

# Conclusions

In this contribution, we discuss inter-gNB and inter-UE CLI issues for dynamic/flexible TDD with the following proposals:

Proposal 1: CSI-RS and/or R16 CLI-RSSI resource can be used for inter-gNB CLI measurement.

Proposal 2: To support inter-gNB CLI measurement, L1-based RSRP/RSSI should be considered:

- L1-based inter-gNB CLI measurement result with timestamp should be exchanged over Xn interface.

Proposal 3: RAN1 prioritizes L1-based SRS-RSRP and L1-based CLI-RSSI measurement for inter-UE CLI handling, and R16 configuration of SRS resources and CLI-RSSI resources can be reused.

Proposal 4: L1 inter-UE CLI measurement results can be reported as a separated CSI report:

- R17 CSI reference resource definition should be extended to include the SRS resource and CLI-RSSI resource for inter-UE CLI measurement;

- R15/16 CSI processing delay should be satisfied.

Proposal 5: To support coordinated scheduling between gNBs, more flexible TDD DL-UL configuration exchange over Xn/F1 interfaces should be studied, e.g. TDD DL-UL configuration with periodicity longer than 10-ms.

# Reference

- R1-2208858, “Discussion on potential enhancements on dynamic/flexible TDD”, OPPO