3GPP TSG RAN WG1 #111			R1-2211686

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.5.2.1

Source: 	CMCC

Title: 	Discussion on integrity of RAT-dependent positioning

Document for:	Discussion and Decision

Introduction

In RAN#94-e meeting, a new SID on study on expanded and improved NR positioning was approved [1]. The motivation to extend the integrity to RAT-dependent positioning is justified:

- Improved accuracy, integrity, and power efficiency: - Study solutions for Integrity for RAT dependent positioning techniques [RAN2, RAN1]: - Identify the error sources, [RAN1, RAN2]. - Study methodologies, procedures, signalling, etc for determination of positioning integrity for both UE-based and UE-assisted positioning [RAN2] - Focus on reuse of concepts and principles being developed for RAT-Independent GNSS positioning integrity, where possible.

In this contribution, we further provide our views on the remaining open issues of error sources and the corresponding models.

Remaining issues on error sources

In the last RAN1 meeting, the following agreements were made to further study error sources [2]:

Agreement - For LMF-based positioning integrity mode, TRP location (e.g., Geographical Coordinates in TS 38.455) is an error source for DL-TDOA, DL-AoD, UL-TDOA, UL-AoA and multi-RTT. - Note: Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 - FFS: Specification impact of TRP location as an error source for LMF-based positioning integrity mode - FFS: Model of the error source (e.g., distribution, mean and/or standard deviation for integrity) Agreement - For UE-based positioning integrity mode, study whether boresight direction of DL PRS (NR-DL-PRS-BeamInfo) and/or beam information (NR-TRP-BeamAntennaInfo) of DL PRS are error sources or not, focusing on the following aspects: - Granularity of boresight direction of DL-PRS and its influence on positioning integrity - Feasibility and complexity of modeling - Feasibility of obtaining quality/statistical parameters of beam information from the gNB - Influence on measurement errors at the UE - Other aspects are not precluded - Note: Definition of “UE-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857 Agreement - Study to determine whether DL PRS RSRP/RSRPP measurement is an error source for DL-AoD, focusing at least on the following aspect - Impact of RSRP/RSRPP measurement on positioning accuracy - FFS: Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) Agreement - Study to determine whether SFN initialization time is an independent error source for the following positioning methods and integrity mode - UL-TDOA with LMF-based positioning integrity mode - UE-assisted DL-TDOA with LMF-based positioning integrity mode - FFS: Model of the error source (e.g., distribution, mean and/or standard deviation for integrity overbounding model, range) - Note: Definition of “LMF-based positioning integrity mode” can be found in Table 9.4.1.1.1 in TR 38.857

On boresight direction and/or beam information

For the UE-based positioning integrity mode, whether boresight direction and/or beam information of DL-PRS for DL-AoD should be considered as error sources was left for FFS. In our views, both errors in boresight direction and beam information of DL-PRS will impact the integrity of the DL-AoD, as they play a similar role as that of RTD information for DL-TDOA. For the UE-based DL AoD positioning, the UE measures the RSRP/RSRPP of different PRS resources with different beam directions and to estimate the angle of departure. The errors of boresight direction and beam information of DL PRS will cause errors in DL-AoD measurement. Hence, we propose that:

Proposal 1: For UE-based positioning integrity mode, support the following error sources in assistance data for DL-AoD:

- boresight direction of DL-PRS

- beam information of DL-PRS

On SFN initialization time

In the last RAN1 meeting, some companies proposed to consider SFN initialization time as an independent error source for TDOA positioning methods, and it was agreed to further determine whether if it is the case. From our perspective, it is not clear to us how can the SFN initialization time be decoupled from the RTD information, which has already agreed as an error source for TDOA positioning. For TDOA positioning method, the RTD information provides the timing misalignment information among different TRPs, and the errors of RTD information will cause timing measurement errors, and in our views, the RTD information should be derived from the SFN initialization time of each TRP, and hence the SFN initialization time as an error is covered by the error of RTD information, and no need to further consider SFN initialization time as an independent error source.

Conclusion 1: For the LMF-based positioning integrity mode, SFN initialization time error is contained in the RTD information error, and not considered as an independent error source for TDOA positioning method.

On LOS/NLOS channel as error sources

Whether multipath/NLOS channel should be identified as an error sources were discussed during the previous RAN1 meetings. First, the impact of multipath channel can be covered by measurement errors. On the other hand, regarding the impact of NLOS scenario, Rel-17 has enhanced the LOS/NLOS indicator to flag the NLOS probability of a measurement, which can be treated as a confidential level of a measurement.

Observation 1: Errors caused by multipath channels can be covered by measurement errors, and errors caused by NLOS scenarios can be reflected by the LOS/NLOS indicator.

Conclusion 2: No need to further identify the error sources resulting from the multipath/NLoS channel.

Conclusions

In this contribution, we provide our views on the integrity of RAT-dependent positioning, and the following proposals are made:

Observation 1: Errors caused by multipath channels can be covered by measurement errors, and errors caused by NLOS scenarios can be reflected by the LOS/NLOS indicator.

Proposal 1: For UE-based positioning integrity mode, consider the following error sources in assistance data for DL-AoD:

- boresight direction of DL-PRS

- beam information of DL-PRS

Conclusion 1: For the LMF-based positioning integrity mode, SFN initialization time error is contained in the RTD information error, and not considered as an independent error source for TDOA positioning method.

Conclusion 2: No need to further identify the error sources resulting from the multipath/NLoS channel.

References

- RP-213588, Revised SID on Study on expanded and improved NR positioning, 3GPP TSG RAN Meeting #94e.

- Chair’s Notes 3GPP TSG RAN WG1 #110bis-e v17

- TR 38.857, Study on NR Positioning Enhancements, v17.0.0.

- Appendix: Error sources for RAT-independent positioning

In Rel-17 discussion on the integrity for A-GNSS positioning, feared events were studied and summarized as the following table [3]:

| Feared Event Category | Feared Event | Examples of positioning integrity assistance information (FFS) |
| --- | --- | --- |
| 1. Feared events in the GNSS Assistance Data | Incorrect computation of the GNSS Assistance Data, e.g. software bug, corrupt or lost data | Validity or quality flags for existing assistance information |
|  | External feared event impacting the GNSS Assistance Data, e.g. satellite, atmospheric or local environment feared events (Category 3) impacting the GNSS reference stations in the GNSS correction provider's network. |  |
| 2. Feared events during positioning data transmission | Data integrity faults | Data corruption check, e.g. CRC |
|  |  | Data Authentication / Signature |
| 3. GNSS feared events | Satellite feared events e.g. bad signal-in-space or bad broadcast navigation data | Satellite health or quality flags |
|  | Atmospheric feared events | Ionospheric indicator |
|  |  | Tropospheric indicator |
|  | Local Environment feared events, e.g. Multipath, Spoofing, Interference | Assistance information: Trustable time reference, Data Authentication / Signature, Regionalized indicator of multipath, interference, jamming, spoofing, etc |
| 4. UE feared events | GNSS receiver measurement error | e.g., GNSS-MeasurementList |
|  | Hardware faults | * |
|  | Software faults | * |
| 5. LMF feared events | Hardware faults | * |
|  | Software faults | * |
| NOTE: The positioning integrity assistance information IEs are FFS as part of the WI. *NOTE: The UE or LMF are responsible for mitigating these feared events locally, outside the scope of the specifications. |  |  |