3GPP TSG-RAN WG1 Meeting #111	R1-2212084

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| DRAFT CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | - | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network | x | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction on UE procedure for reporting HARQ-ACK with PUCCH cell switch |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_enh_IIOT |  |  |  |  |  | Date: |  |  | 2022-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-16, the interpretation of slot n for HARQ-ACK feedback depends on whether UE is configured with “subslotLengthForPUCCH” or not. In Rel-16, there is no ambiguity because a PUCCH group only has Pcell with potential “subslotLengthForPUCCH” configuration. However, In Rel-17, when UE is configured with PUCCH cell switch, and both the Pcell and Scell can be configured with “subslotLengthForPUCCH”, it is unclear in the current Rel-17 spec whether the UE shall follow the “subslotLengthForPUCCH” on PCell or SCell. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Clarify that when UE is configured with PUCCH cell switch: - If the UE is configured with semi-static cell switch, then the UE shall follow the “subslotLengthForPUCCH” configuration on the PCell to determine the reference subslot for K1 - If the UE is configured with dynamic cell switch, then the UE shall follow the “subslotLengthForPUCCH” configuration in the target cell to determine the reference subslot for K1. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Misaligned UE and gNB behaviour when the UE is configured with PUCCH cell switch. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: The CR only impacts the feature of PUCCH cell switch. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

### 9.2.3	UE procedure for reporting HARQ-ACK

< Unchanged parts are omitted >

The following apply to the PCell if the UE is provided pucch-sSCellPattern; otherwise, the following apply to the serving cell for the PUCCH transmission. If the UE is provided subslotLengthForPUCCH, n is the last UL slot for PUCCH transmission that overlaps with a PDSCH reception or with a PDCCH reception providing a DCI format having associated HARQ-ACK information without scheduling a PDSCH reception; otherwise, n is the last UL slot for PUCCH transmission that overlaps with the DL slot nD for the PDSCH reception or with the DL slot nD for the PDCCH reception in case of a DCI format that triggers a HARQ-ACK information report and does not schedule a PDSCH reception.

< Unchanged parts are omitted >