3GPP TSG RAN WG1 #111			R1-2212097

Toulouse, France, November 14th – 18th, 2022

Agenda item:	8.14

Source: 	Qualcomm Incorporated

Title: 	Maintenance on NB-IoT/eMTC support for Non-Terrestrial Network

Document for:	Discussion and Decision

# Issue 1: Processing times

## Framework difference between NR and LTE

The timelines for transmission, reception and processing of different channels are critical aspects for the design of UEs. Although both LTE (including eMTC and NBIOT) and NR technologies have clearly defined timelines, the framework used in the two families of standards is quite different:

- In LTE (including eMTC and NBIOT), the timelines are not explicitly defined by the specification. Instead, the timelines can be implicitly derived based on the “logical time” relationships allowed by the specification and the maximum timing advance. For instance, for eMTC FDD, the minimum time between the reception of a PDSCH and the transmission of a PUCCH is obtained as follows:

  - The minimum HARQ-ACK delay is N+4, i.e., if a UE receives a PDSCH in subframe N, the earliest the UE can be scheduled to transmit the HARQ-ACK is in subframe N+4 (3ms gap in logical time)

  - The maximum timing advance is defined in subclause 8.1 of TS 36.211 as 20512Ts=667.7us (corresponding to 100km cell size).

  - Therefore, the UE shall be dimensioned to be able to process a PDSCH and transmit the HARQ-ACK in 2.3323ms

- In NR, the timelines are explicitly defined for the different timing relationships. Although the specifications allow for combinations of {scheduling offset, timing advance} that result in a very small (or non-causal) processing time at the UE, the specifications clearly state for which cases the UE is expected to process the channels and produce a response. For instance, the minimum time between PDSCH and the corresponding HARQ-ACK is explicitly given by Tproc,1 in subclause 5.3 of TS 38.214.

Observation 1: There is a fundamental difference in the framework of defining timelines between eMTC/NB-IOT and NR:

- In eMTC and NB-IoT, the timelines are implicitly defined by the minimum scheduling offset allowed by the specification and the maximum timing advance.

- In NR, the timelines are explicitly defined by e.g. Tproc,x

For NTN IOT, RAN1 has introduced new elements in the timing advance (NTA,adjcommon and NTA,adjUE) that increase the maximum value of TTA. Although the intention of RAN1 was that these larger timing advances would cancel out with the increased scheduling delay (e.g. given by Koffset), RAN1 has not produced any specification text that clearly state for which combinations of {scheduling offset,  timing advance} the UE is required to process the channels. Note that NR NTN does not have this issue, since NR NTN inherits the Rel-15 clearly defined timelines (Tproc,x).

Observation 2: With the introduction of NTA,adjcommon and NTA,adjUE, timelines at the UE (to process a downlink signal and transmit a corresponding uplink signal) can become arbitrarily small or non-causal.

In RAN1#110b-e several companies mentioned that the timelines do not need to be explicitly defined, and their handling can be left to eNB implementation. From a UE design point of view, it is critical to have clearly specified requirements on the minimum time the UE has to process a given channel.

Observation 3: Having timelines clearly defined in the specification is critical for UE design.

## Proposed way forward

One of the main targets of defining eMTC / NB-IoT over NTN is to enable hardware designed for terrestrial networks to operate in satellite networks, hence enabling economies of scale. Therefore, it is natural to define the timeline requirements for NTN IOT to be not more stringent than those of terrestrial networks (otherwise, hardware reuse is not feasible in most cases):

Proposal 1: RAN1 to specify timelines for NTN IOT (eMTC and NB-IoT) based on the following principle:

- For any timing relationship, the minimum “physical time” within which an eMTC / NB-IoT NTN UE is required to process a given downlink physical channel / signal and produce an associated uplink response is equal to the corresponding minimum “physical time” for eMTC / NB-IoT in FDD terrestrial networks.

- NOTE: “physical time” refers to the physical time (in seconds) observed by the UE, i.e., including timing advance and scheduling delays.

In the following table, we present some examples on how to capture this in the specification, where the k* refer to the corresponding timing relationships. Note that, in the final specification text, the appropriate unit conversion has to be properly written down (e.g. replacing k by k⋅30720⋅Ts)

Table 1 Proposed timelines for eMTC and NB-IoT NTN

| Timing relationship | Proposed timeline |
| --- | --- |
| NPDSCH è HARQ-ACK | k0'-1+Koffset-TTA,NTN≥k0',min-1-TTA,terrestrialmax, k'0,min=13 |
| NPDCCH è NPUSCH | k0-1+Koffset-TTA,NTN≥k0min-1-TTA,terrestrialmax , k0min=8 |
| NPDCCH è NPRACH | k2-1+Kcell_offset-TTA,NTN≥k2min-1-TTA,terrestrialmax , k2min=8 |
| PDSCH è HARQ-ACK | k+Koffset-TTA,NTN≥kmin-TTA,terrestrialmax, kmin=4 |
| MPDCCH è PUSCH | k+Koffset-TTA,NTN≥kmin-TTA,terrestrialmax, kmin=4 |
| MPDCCH è NPRACH | k2+Kcell_offset-TTA,NTN≥k2min-TTA,terrestrialmax , k2min=6 |

Proposal 2: RAN1 to endorse Table 1 as the starting point for defining the timelines in RAN1 specifications.

# Issue 2: NTN SIB accumulation

In previous meetings, the issue of NTN SIB accumulation has been discussed (see the summary in R1-2210259). Regarding this matter, we make the following observations:

- Adding new functionality (e.g., explicit signaling) to indicate the possibility of NTN SIB combining is not acceptable at this late stage of Rel-17 maintenance, especially given that it would require new RRC parameters to introduce a non-essential functionality.

- The current RAN2 specifications state that the contents of SIB31 may change at any point in time. This does not force the eNB to change the contents of SIB31 in every window. The eNB is allowed to keep the same content for multiple SI windows (e.g., in the case of explicit epoch time).

- Even for the case where the UE does not know if the SIB contents have changed, combining across multiple SI windows is possible. For instance, the UE may run multiple hypotheses testing across windows (e.g., combine window 1 and window 2, decode 2nd window alone). Alternatively, the UE may detect that the SNR conditions do not allow to decode SIB31 without combining, and may decide to just try combining across windows. These techniques are optimizations that can be done by the UE.

Therefore, we make the following proposed conclusion:

Proposal 3: For the issue of NTN SIB accumulation, capture the following conclusion in the Chairman’s notes:

- Under current specifications, the UE may optionally combine SIB-31 across SI windows by UE implementation (e.g., by hypotheses testing).

# Issue 3: Interference randomization for NB-IoT

After initial Rel-13 specifications, it was discovered that the performance of some NB-IoT channels in interference limited scenarios was very poor due to the lack of a robust scrambling (with losses of more than 6dB in some cases [3]). This led to the introduction of a non-backward compatible change in the NPBCH and SIB scrambling in Rel-13 [1,2] and the introduction of scrambling for NPDSCH and NPDCCH in a backwards compatible way for Rel-14 [3]. For the sake of completeness, we reproduce the simulation results in [3] in Figure 1:

Figure 1 NPDCCH performance in AWGN with random phase between serving and interfering cells, 1Tx standalone (dashed: no interference randomization, solid: interference randomization)

Observation 4: The enhancements introduced in Rel-14 for interference randomization [3] can provide performance improvement of up to ~8dB.

Since the introduction of that feature, interference randomization has been enabled by default for all the cases where there is not a backward compatible issue with Rel-13 UEs, for instance, interference randomization always applies to:

- PUR (Rel-16)

- TDD (Rel-15)

- RA-RNTI in non-anchor (Rel-14)

- P-RNTI in non-anchor (Rel-14)

In our view, RAN1 should continue the same practice, and enable interference randomization in all NB-IoT NTN cells, since there are no backwards compatibility issues and there are substantial gains in the case of interference-limited scenarios (which may be relevant for e.g., the case of multi-beam satellites with small frequency reuse factor).

Therefore, we make the following proposal:

Proposal 4: Interference randomization is enabled by default for all NPDSCH and NPDCCH transmitted from an NB-IoT NTN cell.

- Adopt TP1

====================================== <TP1, 36.211> ==================================

#### 10.2.3.4	Mapping to resource elements

<Unchanged parts are omitted>

For frame structure type 1 operation in a cell not configured with cellAccessRelatedInfo-NTN,

-	for NPDSCH associated with C-RNTI when interferenceRandomisationConfig is used according to [9], or

-	for NPDSCH associated with RA-RNTI, TC-RNTI or P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or

-	for NPDSCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, or

-	for NPDSCH associated with PUR-RNTI/G-RNTI/ SC-RNTI, or

for frame structure type 2, or for frame structure type 1 operation in a cell configured with cellAccessRelatedInfo-NTN,

-	for NPDSCH not carrying the BCCH,

<Unchanged parts are omitted>

#### 10.2.5.5	Mapping to resource elements

<Unchanged parts are omitted>

For frame structure type 1 operation in a cell not configured with cellAccessRelatedInfo-NTN ,

-	for NPDCCH associated with RA-RNTI, TC-RNTI or

-	for P-RNTI and transmitted in an NB-IoT carrier configured by SystemInformationBlockType22-NB, or

-	for NPDCCH associated with C-RNTI in an NB-IoT carrier configured by SystemInformationBlockType22-NB when RadioResourceConfigDedicted-NB is not configured by higher layer, or

-	for NPDCCH associated with PUR-RNTI/G-RNTI/ SC-RNTI, or

-	for NPDCCH associated with C-RNTI or SPS C-RNTI when interferenceRandomisationConfig is used according to [9], or

for frame structure type 1 operation in a cell configured with cellAccessRelatedInfo-NTN, or

for frame structure type 2,

<Unchanged parts are omitted>

======================================== </TP1> ==================================

# Conclusion

On the issue of processing times, we made the following observations and proposals:

Observation 1: There is a fundamental difference in the framework of defining timelines between eMTC/NB-IOT and NR:

- In eMTC and NB-IoT, the timelines are implicitly defined by the minimum scheduling offset allowed by the specification and the maximum timing advance.

- In NR, the timelines are explicitly defined by e.g. Tproc,x

Observation 2: With the introduction of NTA,adjcommon and NTA,adjUE, timelines at the UE (to process a downlink signal and transmit a corresponding uplink signal) can become arbitrarily small or non-causal.

Observation 3: Having timelines clearly defined in the specification is critical for UE physical.

Proposal 1: RAN1 to specify timelines for NTN IOT (eMTC and NB-IoT) based on the following principle:

- For any timing relationship, the minimum “physical time” within which an eMTC / NB-IoT NTN UE is required to process a given downlink physical channel / signal and produce an associated uplink response is equal to the corresponding minimum “physical time” for eMTC / NB-IoT in FDD terrestrial networks.

- NOTE: “physical time” refers to the physical time (in seconds) observed by the UE, i.e., including timing advance and scheduling delays.

Proposal 2: RAN1 to endorse Table 1 as the starting point for defining the timelines in RAN1 specifications.

On the issue of NTN SIB accumulation, we made the following observations and proposals:

Proposal 3: For the issue of NTN SIB accumulation, capture the following conclusion in the Chairman’s notes:

- Under current specifications, the UE may optionally combine SIB-31 across SI windows by UE implementation (e.g. by hypotheses testing).

On the issue of interference randomization for NB-IoT, we made the following proposal:

Observation 4: The enhancements introduced in Rel-14 for interference randomization [3] can provide performance improvement of up to ~6dB.

Proposal 4: Interference randomization is enabled by default for all NPDSCH and NPDCCH transmitted from an NB-IoT NTN cell.

- Adopt TP1

# References

[1] R1-1703087, “Motivation for change to scrambling of NB-IoT MIB/SIB transmission”, Huawei, HiSilicon

[2] R1-1703874,  “On cross-correlation properties of combined Gold sequences applied to NPBCH rotation sequence”, Qualcomm Incorporated

[3] R1-1708756, “Interference randomization for Rel-14 NB-IoT”, Qualcomm Incorporated