3GPPRAN1#111                	R1-2212239

# Electronic Meeting, 14th – 18th Nov., 2022

Agenda item:		9.12.2

Source:	MediaTek Inc.

Title:	UL timing management to reduce handover latency

Document for:		Discussion

1	Introduction

In RAN1 #110-Bis meeting, following agreements regarding Rle-18 mobility enhancement TA management had been achieved:[1]:

Agreement

Support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2 based mobility.

- FFS: whether this can be applied to candidate cell when it is deactivated SCell (if defined in RAN2).

From Oct 19th GTW session

Agreement

On mechanism to acquire TA of the candidate cells, the following solutions can be further studied:

- RACH-based solutions

  - e.g., PDCCH ordered RACH, UE-triggered RACH, higher layer triggered RACH from NW other than L3 HO cmd

- RACH-less solutions

  - e.g., SRS based TA acquisition, Rx timing difference based, RACH-less mechanism as in LTE, UE based TA measurement (including UE based TA measurement with one TAC from serving cell)

Agreement

For TA acquisition of a candidate cell before cell switch command is received, study at least the following alternatives of associating TA/TAG to candidate cell:

- Alt1: Associate TA/TAG and candidate cell implicitly, e.g.,

  - the association between TA/TAG and TCI states can be configured

- Alt2: Associate TA/TAG and candidate cell explicitly, e.g.,

  - the association is provided as a part of candidate cell(s) configuration

  - the association between TA/TAG and SSB(s)/TRS(s) is provided as a part of candidate cell(s) configuration

In Rel-15/16/17, there can be only one single TA per CC, and it is further assumed at the received signals from different TRPs are withing the CP at the UE side. Such assumption limits the deployment scenario and hardly support the need for mobility purpose. In this contribution, we discuss enhancements to support TA acquisition of candidate cell(s) before cell switch command is received in L1/L2-triggered mobility (LTM).

2	Timing advance management to reduce latency

According to the RAN2 conclusions, reducing the HO interruption time is the agreed improvement goal. Considering that the interruption time mainly contributed by the DL/UL synchronization time after the cell switch command, to further reduce the handover interruption time, UE has to measure the DL RS of candidate cell and transmit PRACH toward candidate cell in advance. In the last meeting, companies agreed to support TA acquisition of candidate cell(s) before cell switch command is received in LTM. Based on the baseline time chart provided in RAN2 discussion [Post119-e][036], we provide a modified time chart as shown in below Figure 1.

Figure 1: LTM mobility latency time chart

Based on RAN2 conclusions, we know that ICBM will be considered as one of the cases supported for LTM. As shown in Figure 2 (a), Rel-17 ICBM applies the unified TCI framework, UE switches to a non-serving beam which is indicated in the TCI state. Currently, RAN4 assumes that non-serving cell L1 measurement will only be performed on the known cell, i.e., the cell that L3 measurement has already been done. In the meanwhile, DL reception time difference of SSBs/CSI RS between serving cell and non-serving cell is within one CP duration. It means that the timing difference, i.e. clock error, between cells is relatively small.

However, most LTM cases are as shown in Figure 2 (b), where the DL reception time difference between serving cell and neighbouring cell is larger than CP duration. It means that the TA values toward different candidate cells might be different and UE need to transmit PRACH toward different candidate cells before the cell switch command, such that Network can maintain multiple TAs in advance. In order to transmit PRACH toward different candidate cells, UE need to perform precise time-frequency tracking on the candidate cell and it needs extra DL measurement resource including storage resource, RF resource, and hardware/software resource to perform the DL synchronization. That limits the number of TAs/TAGs that UE can support, so we think that the maximum number of TAs/TAGs among all candidate cells shall be a UE capability.

Proposal 1: For TA management in L1/L2 based mobility, 1 TA/TAG per candidate cell can be acquired

- The number of TAs/TAGs among all candidate cells can be larger than 1

- The maximum number of TAs/TAGs among all candidate cells shall be a UE capability

The next discussion is the mechanism to support multiple TAs/TAGs among all candidate cells. In Rel-15/16/17, only one RACH procedure can be performed at a time. This is because legacy multiple TAs/TAGs management is only applied for CA case or DC case. Under this scenario, UE generally move with very slow speed and there is no urgency for Network to simultaneously acquire TA for different cells. However, for mobility scenario, it is important for Network to acquiring multiple TAs/TAGs as soon as possible; otherwise, the overall mobility latency before the cell switch is completed will be increased a lot and even worse than the legacy handover procedure where only 1 TA/TAG toward target cell need to be maintained. So we suggest to support PRACH transmissions to multiple candidate cells simultaneously, and all those PRACH transmissions can be triggered by the same TCI activation command. To avoid the collision of UL transmission resources, Network can further allocate dedicated RACH resource for each candidate cell

Proposal 2: Support PRACH transmissions to multiple candidate cells simultaneously

Proposal 3: Support PRACH transmissions triggered by a TCI activation command

Proposal 4: Support dedicated RACH resource per candidate cell

(a) Rel-17 ICBM: simultaneously monitor synchronous activated TCI states

(b) Rel-18 L1/L2 mobility: simultaneously monitor asynchronous activated TCI states

Figure 2: Co-existence of Rel-17 ICBM and Rel-18 L1/L2 mobility

In last meeting, more than one companies raise the concern that requiring UE to receive RAR from the neighboring cell will cause huge interruption, it is important for RAN1 to figure out the corresponding solutions. Considering that TA-value is the most important message in RAR and TA-value toward neighbouring cell is unless for UE, we suggest to introduce a signalling for Network to indicate UE whether to receive msg2 after the triggered PRACH transmissions. If indicated, UE expects no msg2 will be transmitted by Network and TA-value of target cell will be given in the cell switch command. If not indicated, UE will attempt to receive msg2 of neighbouring cell through the serving cell to avoid huge interruption (however, only one RACH procedure can be performed at a time in this case).

Proposal 5: NW indicates UE whether to receive msg2 after the triggered PRACH transmissions

- If indicated, UE expects no msg2 will be transmitted by Network and TA-value of target cell will be given in the cell switch command

- If not indicated, Network will transmit msg2 of neighboring cell through the serving cell

- Send LS to inform RAN2/RAN4 above conclusions

The next issue to be studied is how to associate TA/TAG? In last meeting, 2 alternatives of associating TA/TAG to candidate cell are provided. Considering that ICBM has already been agreed and it is intuitive to “associate TAG to TCI-state/spatial relation.” We suggest to apply alternative 1.

Proposal 6: Associate TAG to TCI-state/spatial relation

3	Summary

In this contribution, we propose

Proposal 1: For TA management in L1/L2 based mobility, 1 TA/TAG per candidate cell can be acquired

- The number of TAs/TAGs among all candidate cells can be larger than 1

- The maximum number of TAs/TAGs among all candidate cells shall be a UE capability

Proposal 2: Support PRACH transmissions to multiple candidate cells simultaneously

Proposal 3: Support PRACH transmissions triggered by a TCI activation command

Proposal 4: Support dedicated RACH resource per candidate cell

Proposal 5: NW indicates UE whether to receive msg2 after the triggered PRACH transmissions

- If indicated, UE expects no msg2 will be transmitted by Network and TA-value of target cell will be given in the cell switch command

- If not indicated, Network will transmit msg2 of neighboring cell through the serving cell

- Send LS to inform RAN2/RAN4 above conclusions

Proposal 6: Associate TAG to TCI-state/spatial relation

Reference

- 3GPP TSG-RAN WG1 Meeting #110-Bis Chairman Notes, Oct., 2022.