3GPP TSG-RAN WG1 Meeting #111 	R1-2212702

Toulouse, France, November 14 – 18, 2022

Agenda Item:	9.7.1

Source:	Moderator (Huawei)

Title:	FL summary#1 for NW Energy Savings

Document for:	Discussion and decision

Introduction

This summary contains simulation results submitted to RAN1#111 meeting for the study of network energy savings for NR (referring to a template for collection of companies input, in Template_NW_EnSav_v19_Samsung_CEWiT.xlsx), and provides observations/conclusion for discussion and decision. To ease the TR work, the document uses the same structure as (draft) TR 38.864 (v0.4.0). Note, since the summary is not intended for email discussion, companies are not required to have input for the draft Tables/Texts/Observations.

Companies can search ‘draftTP’ for the relevant draft TPs.

6	Techniques to improve network energy savings

Editor's note: simulation results to be captured under this section.

Editor's note: RAN2 and RAN3 related aspect to be provided by using separate sections like 6.X when applicable.

##### === draftTP0: start ===

Various techniques in time, frequency, spatial and power domains are studied. Companies’ simulation results as well as evaluation assumption details are gathered in [ref.]. In this document, results as well as some notable assumptions and setting are explicitly present in relevant tables. Also, the categorization of techniques in terms of technical domain and results presentation/tabulation are for study/evaluation purpose. This does not preclude to further merge or combine certain techniques.

=== end ===

FL: when checking the simulation results, it appears to be quite challenging to capture all the details reported from companies. FL considers it might be sufficient from study point of view to capture the critical results and notable settings/evaluation assumptions while leave other detailed referable from the excel or companies’ contributions. Therefore, for different schemes, the table may show differently. Company can request to clarify/add details for some results into corresponding tables.

Also, the numbering of techniques, i.e. A-1, A-3, A-4 etc. shall be re-ordered when capturing into TR in consecutive order. For discussion purpose the current TP use the same numbering as those in FL summary of agenda item 9.7.2.

| Company | Comments |
| --- | --- |

6.1	Techniques in time domain

6.1.1	Technique A-1 Adapting transmission/reception of common channels/signals

6.1.1.1	Description of technique

6.1.1.2	Analysis of performance and impacts

##### ===draftTP A-1-1: start ===

The following table captures the simulation results for the schemes that use simplified version of SSB, such as only PSS or only PSS and SSS without PBCH, or PSS and SSS with partial PBCH.

Table 6.1.1-x: BS energy savings by simplified SSB

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/access delay/latency/UE power consumption | Reference configuration | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC [R1-2211692] | SSB and SIB1 repetition period 40ms, for other 20ms occasions, only PSS and SSS are transmitted. | cat.2 | Zero | 15.7% | / | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
|  | SSB and SIB1 repetition period 40ms, for other 20ms occasions, only PSS and SSS are transmitted. | cat.1 | Zero | 28.3% | / |  | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
| vivo [R1-2211018, R1-2212541] | SSB structure adaptation including light SSB (ES scheme: 20ms light-SSB and SIB1, 20ms RACH listening only PSS and SSS for light-SSB) | Cat1 | Zero | 0.9% | 0% | Set 1 | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 20ms UEWUS listening only PSS and SSS for light-SSB) | Cat1 | Zero | 1.2% | 0% | Set 1 | Baseline scheme: 160ms SSB, 20ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 80ms UEWUS listening only PSS and SSS for light-SSB) | Cat1 | Zero | 2.4% | 0% |  | Baseline scheme: 160ms SSB, 80ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 160ms UEWUS listening only PSS and SSS for light-SSB) | Cat1 | Zero | 4.4% | 0% |  | Baseline scheme: 160ms SSB, 160ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 20ms light-SSB and SIB1, 20ms RACH listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.7% | 0% |  | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 20 UEWUS listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.8% | 0% |  | Baseline scheme: 160ms SSB, 20ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 80ms UEWUS listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.8% | 0% |  | Baseline scheme: 160ms SSB, 80ms UEWUS listening |
|  | SSB structure adaptation including light SSB (ES scheme: 160ms light-SSB, 160ms UEWUS listening only PSS and SSS for light-SSB) | Cat2 | Zero | 0.8% | 0% |  | Baseline scheme: 160ms SSB, 160ms UEWUS listening |
| Qualcomm [R1-2212128] | Wake up of gNB triggered by UE wake up signal (WUS) | Category 1 | No Load | 18.7% |  | FR2 Set 3 | "light SSB" combined with UL WUS and on demand SIB 1 |
| CEWiT [R1-2212429] | simplified SSB with repetition period 20ms, only PSS and SSS with partial PBCH are transmitted in simplified SSB | Cat.1 | Zero | 2.4% |  | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |

Based on the simulation results, at empty load, two companies (CMCC, Qualcomm) show that light version of SSB can achieve energy saving by 15.7%~28.3%, depending on the baseline SSB/SIB1 periodicity and possibly combined with WUS triggering on demand SIB1. One company (vivo) show that the gain ranges from 0.8% to 4.4%, which slightly increases as the listening periodicity of WUS from UE becomes larger.

On UPT/access delay/latency, []

On UE power consumption, no negative impact is observed.

=== end ===

FL: Companies are invited to comment on:

- What to be additionally added?

- What results you consider needs clarifications?

- Revisions on observations (the texts right below the table)

Same questions apply to all other tables/texts.

| Company | Comments |
| --- | --- |

##### === draftTP A-1-3: start ===

The following table captures the simulation results for the schemes by which transmission occasion of one or more common signals/channels, which is SIB1 and SSB based on the submitted results, can be skipped.

Table 6.1.1-x: BS energy savings by skipping one or more common signals/channels

| Company | ES scheme | Load scenario | ES gain (%) |  | BS Category/Reference configuration/Baseline configuration/assumption | Other evaluation methodology/assumption details/other performance impact |
| --- | --- | --- | --- | --- | --- | --- |
| OPPO [R1-2211458] | Transmission occasion of SIB1 with 24 RBs for 20 ms periodicity is skipped | low load(RU-10%) | 2.6% | Cat 1 Set 1 SIB1 with 24 RBs for 20 ms periodicity, SSB with 20 RBs for 20 ms periodicity |  | SLS FTP3 (0.5MB as packet size, 200ms as mean inter-arrival time) UPT/Access delay/Latency: almost similar with the baseline |
|  |  | low load(RU-0.2%) | 3.9% |  |  | SLS IM (0.1MB as packet size, 2s as mean inter-arrival time) UPT/Access delay/Latency: almost similar with the baseline |
| Samsung [R1-2212056] | the number of SSB adaptation | Medium load: 42 % RU | 1.9%, 2.8%, 3.3% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 3.1%, 6.1%, 7.6% |  |  |  |
|  |  | Low load: 7.5 % RU | 5.5%, 11.0%, 13.8% |  |  |  |
|  |  | Low load: 2 % RU | 7.2%, 14.3%, 17.9% |  |  |  |
|  |  | Medium load: 42 % RU | 2.0%, 3.0%, 3.5% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 2.8%, 4.2%, 4.9% |  |  |  |
|  |  | Low load: 7.5 % RU | 4.4%, 6.6%, 7.7% |  |  |  |
|  |  | Low load: 2 % RU | 5.3%, 7.9%, 9.2% |  |  |  |
|  |  | Medium load: 42 % RU | 3.3%, 5.0%, 5.8%, 6.3%, 6.5%, 6.6% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 4.3%, 17.6%, 19.5%, 20.5%, 21.0%, 21.3% |  |  |  |
|  |  | Low load: 7.5 % RU | 7.1%, 13.6%, 16.9%, 18.5%, 19.3%, 19.7% |  |  |  |
|  |  | Low load: 2 % RU | 8.3%, 15.9%, 19.7%, 21.6%, 22.6%, 23.0% |  |  |  |
|  |  | Medium load: 42 % RU | 4.0%, 5.9%, 6.9%, 7.4%, 7.7%, 7.8% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 20 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 5.4%, 8.1%, 9.5%, 10.2%, 10.5%, 10.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 8.2%, 12.2%, 14.3%, 15.3%, 15.8%, 16.1% |  |  |  |
|  |  | Low load: 2 % RU | 9.6%, 14.4%, 16.8%, 18.0%, 18.6%, 18.9% |  |  |  |
|  |  | Medium load: 42 % RU | 1.1%, 2.2%, 2.7% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.6%, 3.2%, 4.0% |  |  |  |
|  |  | Low load: 7.5 % RU | 3.1%, 6.1%, 7.7% |  |  |  |
|  |  | Low load: 2 % RU | 4.2%, 8.3%, 10.3% |  |  |  |
|  |  | Medium load: 42 % RU | 1.0%, 1.6%, 1.8% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.5%, 2.2%, 2.5% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.3%, 3.5%, 4.0% |  |  |  |
|  |  | Low load: 2 % RU | 2.8%, 4.2%, 4.9% |  |  |  |
|  |  | Medium load: 42 % RU | 1.9%, 3.6%, 4.4%, 4.8%, 5.0%, 5.2% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 2.6%, 5.0%, 6.1%, 6.7%, 7.0%, 7.2% |  |  |  |
|  |  | Low load: 7.5 % RU | 4.0%, 7.6%, 9.4%, 10.4%, 10.8%, 11.0% |  |  |  |
|  |  | Low load: 2 % RU | 4.8%, 9.2%, 11.4%, 12.5%, 13.1%, 13.3% |  |  |  |
|  |  | Medium load: 42 % RU | 2.1%, 3.1%, 3.6%, 3.9%, 4.0%, 4.1% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 40 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 2.9%, 4.3%, 5.1%, 5.4%, 5.6%, 5.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 4.4%, 6.7%, 7.8%, 8.3%, 8.6%, 8.8% |  |  |  |
|  |  | Low load: 2 % RU | 5.4%, 8.1%, 9.4%, 10.1%, 10.4%, 10.6% |  |  |  |
|  |  | Medium load: 42 % RU | 0.6%, 1.1%, 1.4% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.0%, 2.3%, 3.0% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.5%, 5.9%, 7.6% |  |  |  |
|  |  | Low load: 2 % RU | 4.5%, 10.7%, 13.8% |  |  |  |
|  |  | Medium load: 42 % RU | 0.5%, 0.8%, 0.9% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.7%, 1.1%, 1.3% |  |  |  |
|  |  | Low load: 7.5 % RU | 1.2%, 1.8%, 2.1% |  |  |  |
|  |  | Low load: 2 % RU | 1.4%, 2.1%, 2.5% |  |  |  |
|  |  | Medium load: 42 % RU | 1.0%, 1.8%, 2.3%, 2.5%, 2.6%, 2.6% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.3%, 2.6%, 3.2%, 3.5%, 3.7%, 3.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.1%, 4.1%, 5.0%, 5.5%, 5.8%, 5.9% |  |  |  |
|  |  | Low load: 2 % RU | 2.6%, 5.0%, 6.2%, 6.8%, 7.1%, 7.2% |  |  |  |
|  |  | Medium load: 42 % RU | 1.1%, 1.6%, 1.8%, 2.0%, 2.0%, 2.1% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 80 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 1.5%, 2.2%, 2.6%, 2.8%, 2.9%, 2.9% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.3%, 3.5%, 4.1%, 4.4%, 4.5%, 4.6% |  |  |  |
|  |  | Low load: 2 % RU | 2.9%, 4.3%, 5.0%, 5.4%, 5.5%, 5.6% |  |  |  |
|  |  | Medium load: 42 % RU | 0.3%, 0.6%, 0.7% | Cat 1 Set 1 8 SSBs for FR1 and ssb-periodicity = 160 |  | FTP3 Model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.5%, 1.3%, 1.6% |  |  |  |
|  |  | Low load: 7.5 % RU | 1.5%, 3.5%, 4.5% |  |  |  |
|  |  | Low load: 2 % RU | 3.3%, 7.8%, 10.1% |  |  |  |
|  |  | Medium load: 42 % RU | 0.3%, 0.4%, 0.5% | Cat 2 Set 1 8 SSBs for FR1 and ssb-periodicity = 160 |  | FTP3 model. For each load, reduced the number of SSB transmissions: 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.4%, 0.6%, 0.7% |  |  |  |
|  |  | Low load: 7.5 % RU | 0.6%, 0.9%, 1.0% |  |  |  |
|  |  | Low load: 2 % RU | 0.7%, 1.1%, 1.3% |  |  |  |
|  |  | Medium load: 42 % RU | 0.5%, 0.9%, 1.1%, 1.3%, 1.3%, 1.3% | Cat 1 Set 3 64 SSBs for FR2 and ssb-periodicity = 160 |  | FTP3 model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.7%, 1.3%, 1.6%, 1.8%, 1.9%, 1.9% |  |  |  |
|  |  | Low load: 7.5 % RU | 2.6%, 7.1%, 9.3%, 10.4%, 11.0%, 11.2% |  |  |  |
|  |  | Low load: 2 % RU | 6.0%, 16.0%, 21.0%, 23.5%, 24.8%, 25.4% |  |  |  |
|  |  | Medium load: 42 % RU | 0.5%, 0.8%, 0.9%, 1.0%, 1.0%, 1.1% | Cat 2 Set 3 64 SSBs for FR2 and ssb-periodicity = 160 |  | FTP3 model. For each load, reduced the number of SSB transmissions: 32, 16, 8, 4, 2, 1 |
|  |  | Light load: 24 % RU | 0.8%, 1.1%, 1.3%, 1.4%, 1.5%, 1.5% |  |  |  |
|  |  | Low load: 7.5 % RU | 1.2%, 1.8%, 2.1%, 2.2%, 2.3%, 2.3% |  |  |  |
|  |  | Low load: 2 % RU | 1.5%, 2.2%, 2.6%, 2.8%, 2.8%, 2.9% |  |  |  |

Based on the results, it is observed that

- skipping certain SIB1 transmission occasions under Set 1 reference configuration for BS Category 1 can achieve energy saving gain by 2.6%~3.9% compared to the baseline of 20 ms SSB&SIB1 repetition periodicity at low load;

- adaptation of number of SSB can achieve energy saving gain by 0.3%~25.4% at different scenarios with FTP3 model. The gain generally increases when the traffic load becomes lighter while decreases as the SSB periodicity becomes larger. For a same traffic load and SSB periodicity, the gain increases as the number of SSB can be reduced. For FR2 with larger number of SSB for baseline, there is generally larger gain observed than FR1.

On UPT/access delay/latency,

- skipping SIB1 transmission does not observed negative impact.

On UE power consumption, [].

=== end ===

| Company | Comments |
| --- | --- |

##### === draftTP A-1-5: start ===

The following show the BS energy savings by configuration/adaptation of longer periodicity of common signals and/or uplink random access opportunities.

Table 6.1.1-x: BS energy savings by adapting SSB/SIB1 periodicities

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/Access delay/latency/UE power consumption, etc. | Baseline configuration/assumption/Other notable setting |
| --- | --- | --- | --- | --- | --- | --- |
| CMCC [R1-2211692] | SSB periodicity 20ms, SIB repetition period 40ms. | cat.2 | Zero | 13.7% |  | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |
|  | SSB and SIB1 repetition period 40ms. |  |  | 17.6% |  |  |
|  | SSB periodicity 20ms, SIB repetition period 40ms. | cat.1 | Zero | 25.7% |  |  |
|  | SSB and SIB1 repetition period 40ms. |  |  | 28.7% |  |  |
| vivo [R1-2211018, R1-2212541] | Period adaptation of common signals and channels (ES scheme: 160ms SSB and SIB1, 160ms RACH listening) | Cat1 | Zero | 78.8% | UE power consumption: 0% | Baseline scheme: 20ms SSB and SIB1, 20ms RACH listening |
|  |  | Cat2 |  | 16.6% | UE power consumption: 0% |  |
| NOKIA/NSB [R1-2211097] | SSB/SIB1/RO monitoring period= 160ms | Cat 2 | Zero, Low, Light, Medium | 48.4%, 44.3%, 43.7%, 39.9% | UPT: 0 Mbps, 83 Mbps, 70 Mbps, 55 Mbps | SSB/SIB1/random-access occasion (RO) monitoring periodicity @ 20ms UEs are initially in RRC_idle state |
|  | SSB/SIB1/RO monitoring period= 640ms |  | Zero, Low, Light, Medium | 53.6%, 49.0%, 48.8%, 46.1% | UPT: 0 Mbps, 29 Mbps, 27 Mbps, 25 Mbps |  |
|  | SSB/SIB1/RO monitoring period= 1280ms |  | Zero, Low, Light, Medium | 83.6%, 51.3%, 51.7%, 50.6% | UPT: 0 Mbps, 11.2 Mbps, 11 Mbps, 10.5 Mbps |  |
| Spreadtrum [R1-2211241] | Prolonging the periodicity of SSB/SIB1/paging: 1) SSB burst periodicity is 160ms, and SIB1 repetition periodicity is 160ms. 2) PF periodicity at gNB side is 160ms (T=1280ms, N=8). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 | Zero | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. |
|  |  | Cat 2 |  | Set 1- Set 3: 9.3%, 8.3%, 9.4% |  |  |
|  | Transmission window of SSB/SIB1/paging: 1) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. 2) PF periodicity at gNB side is 20ms (T=1280ms, N=64). 3) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. | Cat 1 |  | Set 1- Set 3: 23.8%, 19.6%, 16.3% |  | 1) The transmission window periodicity is 1280ms, and the transmission window duration is 160ms. 2) SSB burst periodicity is 20ms within the transmission window, and SIB1 repetition periodicity is 20ms within the transmission window. 3) PF periodicity at gNB side is 160ms (T=1280ms, N=8) within the transmission window. 4) gNB can enter light sleep for Cat 1, and can enter both light sleep and micro sleep for Cat 2 (at the tail of the transmission window). |
|  |  | Cat 2 |  | Set 1- Set 3: 51.5%, 47.3%, 20.6% |  |  |
| Intel [R1-2211410] | Increasing the common channel/signal periodicity | Cat 1 | Low | 40.1% | UPT: 819.66 Mbps Avg EE* (baseline): 5.10 Avg EE (ES scheme): 9.17 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity. EE* is defined as cell throughput (in Mbps) / average power consumption (in relative power), and averaged from all BS. |
|  |  |  |  | 45.0% | UPT: 819.66 Mbps Avg EE (baseline): 5.10 Avg. EE (ES scheme): 10.60 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Light | 14.6% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.31 | Baseline: SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 16.8% | UPT: 611.45Mbps Avg EE (baseline): 2.66 Avg. EE (ES scheme): 3.46 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  |  | Medium | 6.2% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.63 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 7.1% | UPT: 457.92Mbps Avg EE (baseline): 1.50 Avg. EE (ES scheme): 1.65 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 640 msec periodicity |
|  |  | Cat2 | Low | 8.2% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 39.23 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 10.9% | UPT: 819.66Mbps Avg EE (baseline): 35.82 Avg. EE (ES scheme): 40.09 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Light | 5.1% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.00 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 5.8% | UPT: 611.45Mbps Avg EE (baseline): 20.75 Avg. EE (ES scheme): 22.19 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
|  |  |  | Medium | 3.0% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.89 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms ES scheme: SSB/SIB1/PRACH: 160 msec periodicity |
|  |  |  |  | 3.4% | UPT: 457.92Mbps Avg EE (baseline): 12.44 Avg. EE (ES scheme): 12.96 | Baseline:SSB/PRACH: 20 msec periodicity; SIB periodicity 40ms. ES scheme: SSB/SIB1/PRACH: 1280 msec periodicity |
| CATT [R1-2211210] | Adaptation of common signals and channels | Cat 1 | Zero load | 10.2%, 72.7%, 84.8% |  | Baseline: 20ms SSB; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Low load | 3.4%, 18.8%, 19.7% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Light load | 1.9%, 5.2%, 5.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
|  |  |  | Medium load | 1.3%, 2.2%, 2.6% |  | Baseline: SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; ES scheme: SSB: 40ms, 80ms, 160ms for each load |
| Fujitsu [R1-2211085] | SSB/SIB1 period= 40ms | Cat2 | Zero, low, light, medium | 17.9%, 13.7%, 11.1%, 8.6% |  | Baseline scheme: 20 ms SSB/SIB1 period |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 26.8%, 20.6%, 16.7%, 12.8% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 31.4%, 24.1%, 19.4%, 15.0% |  |  |
|  | SSB/SIB1 period= 40ms |  | Zero, low, light, medium | 18.3%, 12.6%, 9.4%, 6.9% |  |  |
|  | SSB/SIB1 period= 80ms |  | Zero, low, light, medium | 27.4%, 18.8%, 14.1%, 10.4% |  |  |
|  | SSB/SIB1 period= 160ms |  | Zero, low, light, medium | 32.0%, 22.0%, 16.5%, 12.1% |  |  |
| Ericsson [R1-2212154] | 40ms SSB+SIB1 | Cat1 | Zero | 0.9% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: one SSB. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 48.5% |  |  |
|  | 160ms SSB+SIB1 |  |  | 72.6% |  |  |
|  | 40ms SSB+SIB1 |  |  | -6.2% |  | Baseline scheme: 20ms SSB + 160ms SIB1 ES: Four SSBs. Energy calculation: per symbol energy consumption is modeled. |
|  | 80ms SSB+SIB1 |  |  | 43.8% |  |  |
|  | 160ms SSB+SIB1 |  |  | 70.5% |  |  |
| Qualcomm [R1-2212128] | Adaptation of Common Signals and Channels | Category 1 | No Load | 13.9% | Access delay/latency: 20 ms; UE power consumption: 99 | Note: "SSB period of 40 ms" without any network traffic either in DL or UL. Therefore, there are no statistics for UPT, latency, etc.. |

Based on the results, it can be observed that longer SSB/SIB periodicity can bring BS with significant energy savings in most cases, compared to a selected baseline, for both BS Categories, under all reference configurations. When other configurations/settings are the same, the saving gain generally increase as the periodicity becomes larger, and decrease as the traffic load increases or the number of SSBs increases. Particularly, there are two companies providing results (Nokia, Intel) with SSB periodicity larger than 160ms which is the maximum value that is currently supported, i.e., being 640ms and 1280ms, and observed that together with longer SIB1/RACH/RO monitoring periodicities, then depending on the traffic load, the BS energy saving gain can be 53.6%~7.1% and 83.6%~3.4%, respectively, compared to a baseline with 20ms SSB periodicity.

The scheme does not affect the UPT for empty load case. When traffic occurs and load increases, the UPT also significantly decreases. The latency/access delay increases proportionally as the periodicity of SSB/SIB increases compared to a corresponding baseline.

The UE power consumption is [also negatively impacted].

=== end ===

| Company | Comments |
| --- | --- |

##### === draftTP A-1-7: start ===

The following show the BS energy savings by configuration/adaptation of transmission patterns of common signals, i.e. Paging or SSB based on the submitted results.

Table 6.1.1-x: BS energy savings by adapting Paging/SSB transmission patterns

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/access delay/latency/UE power consumption | Reference configuration | Baseline configuration/assumption | Other evaluation methodology/assumption details/notable settings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Intel [R1-2211410] | Enhanced Paging by increasing the number of consecutive POs within a PF by factor of M while reducing PF density by a factor of M. This keeps the total number of POs same within the DRX cycle. | Cat1 | Zero, Paging load 2% | 21.2% |  | Set 1 | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 1 |
|  |  |  | Zero, Paging load 0.2% | 4.0% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 1 |
|  |  |  | Zero, Paging load 2% | 42.3% |  |  | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 1; |
|  |  |  | Zero, Paging load 0.2% | 6.7% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 1; |
|  |  |  | Zero, Paging load 3.6% | 18.9% |  |  | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
|  |  |  | Zero, Paging load 0.5% | 0.2% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 80 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
|  |  |  | Zero, Paging load 3.6% | 26.4% |  |  | Paging Parameters: N = T/4; Ns = 4; Enh. Paging†: N = T/4; Ns = 4; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
|  |  |  | Zero, Paging load 0.5% | 0.3% |  |  | Paging Parameters: N = T/16; Ns = 2; Enh. Paging†: N = T/16; Ns = 2; M = 4 | No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 4; SSB and SIB1 contained in same slot. 1 SSB per slot along with SIB1 to maximize SSB/SIB1 packing; |
| Qualcomm [R1-2212128] | Adaptation of Common Signals and Channels | Category 1 | No Load | 10.3% | UE power consumption: -4% | FR2 Set 3 |  | "Compact SSB" without any network traffic either in DL or UL. Therefore, there are no statistics for UPT, latency, etc.. |

Based on the results, it is observed that by adapting the transmission patterns of common signals of paging or SSB, the total PO or SSB periodicity is kept and BS energy saving can be achieved by 0.3%~42.3% for Paging adaptation and by 10.3% for SSB adaptation, respectively, both for BS category 1 and at empty load case. The gain generally increases as paging load increases or SSB/SIB/PRACH periodicity increases, but decreases when the number of SSB increases.

On UPT/access delay/latency, []

On UE power consumption, adaptation of SSB has negative impact.

=== end ===

| Company | Comments |
| --- | --- |

##### === draftTP A-1-8: start ===

The following show the BS energy savings by dynamically adapting common signals, i.e. RACH based on the submitted results.

Table 6.1.1-x: BS energy savings by dynamically adapting RACH periodicity/occasions

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/access delay/latency/UE power consumption | Reference configuration | Baseline configuration/assumption/notable settings | Other evaluation methodology/assumption details |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Ericsson [R1-2212154] | PRACH periodicity= 20ms | Cat1 | Zero | 14.4% | Access delay/latency: 10ms increase | Set 1 | Baseline scheme: 20 ms SSB, 40ms SIB1 period, 10ms PRACH periodicity. Per symbol energy consumption is modeled. ES scheme: adapting PRACH periodicity for energy efficiency via dynamic PRACH occasions adaptation. Note separate evaluation performed for different PRACH periodicities (i.e. no switching between these settings). | 1 SSB |
|  | PRACH periodicity= 40ms |  |  | 20.9% | Access delay/latency: 30ms increase |  |  |  |
|  | PRACH periodicity= 80ms |  |  | 22.2% | Access delay/latency: 70ms increase |  |  |  |
|  | PRACH periodicity= 20ms |  |  | 17.3% | Access delay/latency: 10ms increase |  |  | four SSBs |
|  | PRACH periodicity= 40ms |  |  | 23.9% | Access delay/latency: 30ms increase |  |  |  |
|  | PRACH periodicity= 80ms |  |  | 24.9% | Access delay/latency: 70ms increase |  |  |  |

Based on the results, it is observed that dynamic adaptation of RACH occasions can achieve BS energy savings by 14.4%~24.9% for BS Category 1 at empty load case under FR1 TDD compared to 10ms RACH periodicity without adaptation. The gain generally increases as adapted PRACH periodicity increases, and also increases as the number of SSBs increases.

On UPT/access delay/latency, this scheme increases access delay/latency from 10ms to 70ms, same as the increased PRACH periodicity.

On UE power consumption, [].

=== end ===

| Company | Comments |
| --- | --- |

##### === draftTP A-1-9: start ===

The following show the BS energy savings by scheduling of SIB1 by SSB, without PDCCH for SIB1, with repetition period 20ms.

Table 6.1.1-x: BS energy savings by scheduling of SIB1 by SSB

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT Access delay/latency UE power consumption Other KPI(s), if any | Reference configuration | Baseline configuration/assumption | Traffic model | Other evaluation methodology/assumption details - Part 1 (other than power modeling aspects) | Other evaluation methodology/assumption details - Part 2 (power modeling aspects) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CEWiT+B417:R417 [R1-2212429] | Option 11) Scheduling of SIB1 by SSB, without PDCCH for SIB1, with repetition period 20ms | Cat.1 | Zero | 4.8% |  | Set 1 | Baseline: normal SSB/SIB1 transmission, with 20ms repetition period for both. |  | numerical analysis | •SIB1: PDCCH: 3 symbols; PDSCH: 12 OFDM symbols including DMRS. •ղ=1, A=0.4. •Time unit for power model is ms. power consumption is calculated in a 20ms long period |

It is observed that using SSB to schedule SIB1 can obtain 4.8% BS energy savings for Set 1 reference configuration for BS Category 1, compared to SSB/SIB1 periodicity of 20 ms for both.

=== end ===

| Company | Comments |
| --- | --- |

6.1.1.3	Specification impacts

6.1.2	Technique A-3 XX

6.1.2.1	Description of technique

6.1.2.2	Analysis of performance and impacts

##### === draftTP A-3: start ===

The following capture the results for waking up gNB triggered by UE wake up signal (WUS).

Table 6.1.1-x: BS energy savings by UE wake up signal (WUS)

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT | Access delay/latency | UE power consumption | Reference configuration | Baseline configuration/assumption | Traffic model | Other evaluation methodology/assumption details/notable settings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | UE_can_wake_up_gNB | Cat 1 | Low | 49.3% | 0.00% | 0.00% | 0.07% | Set 1 | All 21 cells active | VoIP | SLS; DRX (40, 4, 10); 9 out of 21 cells remian active. BS power consumption value is sum of 21 cells. |
|  |  | Cat 2 |  | 51.9% | 0.00% | 0.00% | 0.07% |  |  |  |  |
| ZTE, Sanechips [R1-2211903] | UE WUS is used to wake up a gNB in an energy saving state without DL transmission including SSB/SIB1 and UL reception including RACH monitoring (i.e., cell off/inactive period), or with sparse SSB/SIB1 transmission and RACH monitoring | 1 | low | 7.4% 19.6% 23.8% | 0.66% 2.59% 5.04% |  |  | Set 1 | no WUS, cell always-on; / WUS period=20ms/80ms/160ms for each load, cell off when no UEs camp on. | FTP3 | UE mobility. slot-level; Pstatic=P3, η(s_f,s_p )=1; time-domain scaling for SSB; time and frequency domain scaling for SIB. |
|  |  |  | light | 4.9% 12.7% 15.5% | 0.11% 0.43% 0.86% |  |  |  |  |  |  |
|  |  | 2 | low | 6.2% 6.4% 6.5% | 0.66% 2.59% 5.04% |  |  |  | no WUS, cell always-on; / WUS period=20ms/80ms/160ms for each load, cell off where no UEs camp on. |  |  |
|  |  |  | light | 4.5% 4.6% 4.7% | 0.11% 0.43% 0.86% |  |  |  |  |  |  |
| vivo [R1-2211018 R1-2212541] | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) | Cat 1 | 0% | 29.7% 66.6% 80.7% |  |  | 0.00% 0.00% 0.00% | Set 1 | legacy BS, where all cells are always in the normal mode. Normal mode: 20ms SSB and SIB1, 20ms RACH listening | NaN NaN NaN | SLS No UE DRX 100% detection reliability |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 27.3% 60.4% 72.8% | 0.8% 15.5% 21.7% | -5.68% -38.73% -39.53% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 20.55% 20.81% 20.49% | 0.8% 4.3% 6.0% | 3.4% 4.5% 8.6% | -9.70% -20.72% -32.51% | -0.98% -1.46% -1.66% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 41.79% 41.17% 41.35% | -2.4% 0.3% 0.1% | 2.7% 6.0% 7.2% | -8.95% -14.55% -20.53% | -1.13% -1.51% -1.97% |  |  |  |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0% | 32.1% 69.6% 83.7% |  |  | 0.00% 0.00% 0.00% |  |  | NaN NaN NaN |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 29.4% 63.3% 75.6% | 0.8% 16.5% 24.2% | -4.17% -38.05% -39.53% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 20.71% 20.51% 20.66% | -0.1% 6.4% 6.6% | 3.9% 7.0% 8.7% | -11.04% -20.31% -29.07% | -1.08% -1.33% -1.65% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 41.74% 41.91% 42.07% | -2.2% -0.7% -0.6% | 1.3% 6.6% 7.5% | -10.32% -16.58% -18.21% | -1.13% -1.63% -1.84% |  |  |  |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) | Cat 2 | 0% | 19.1% 19.4% 19.4% |  |  | 0.00% 0.00% 0.00% |  |  | NaN NaN NaN |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 18.1% 18.3% 18.3% | 0.76% 5.40% 11.79% | -0.58% -8.98% -20.16% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 20.58% 20.28% 20.76% | 0.5% 1.0% -0.4% | 0.69% 1.02% 2.88% | -7.93% -9.93% -17.27% | -0.64% -0.56% -0.99% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: 160ms SSB, 20ms/80ms/160ms UEWUS) |  | 41.46% 41.22% 41.04% | -2.4% -2.1% -1.8% | 0.05% 0.30% 0.45% | -5.94% -10.07% -11.62% | -0.99% -1.10% -0.96% |  |  |  |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0% | 20.3% 20.6% 20.6% |  |  | 0.00% 0.00% 0.00% |  |  | NaN NaN NaN |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 0.002% | 19.2% 19.4% 19.5% | 0.85% 4.17% 10.53% | -2.63% -9.83% -20.86% | 0.00% 0.00% 0.00% |  |  | FTP3, mean packet interval of 10s, packet size of 100bytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 20.55% 20.61% 21.26% | 0.5% 0.3% -1.0% | 0.36% 0.61% 2.21% | -8.30% -10.14% -17.32% | -0.71% -0.73% -1.11% |  |  | FTP3, mean packet interval of 200ms, packet size of 0.5Mbytes |  |
|  | UE WUS to wake up a ES gNB without or with sparse SSB/SIB1 and RACH monitoring (the cells without traffic are switching to ES mode ES mode: no SSB, 20ms/80ms/160ms UEWUS) |  | 42.05% 41.38% 41.74% | -3.1% -2.3% -2.9% | 0.10% 0.21% 0.36% | -7.48% -9.16% -10.22% | -1.26% -0.98% -1.04% |  |  |  |  |
| NOKIA/NSB [R1-2211097] | Wake up of gNB triggered by UE wake up signal (WUS) @ 20ms | Cat 2 | Low | 45.6% | 13,01 Mbps |  |  | Set 1 | SSBs/SIB1s/RO monitoring @ 20ms default periodicity UEs are initially in RRC_idle state | UL - IM | SLS+Post-processing |
|  | Wake up of gNB triggered by UE wake up signal (WUS) @ 160ms |  |  | 51.9% | 6,08 Mbps |  |  |  |  |  |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS) @ 640ms |  |  | 52.5% | 2,15 Mbps |  |  |  |  |  |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS) @ 1280ms |  |  | 66.7% | 1,16 Mbps |  |  |  |  |  |  |
| Samsung [R1-2212056] | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy | Cat 1 | Low | 70.3% 64.0% 57.2% |  | 0.00% 0.00% 0.00% |  | Set 2 | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 90. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @15 ms WUS periodicy |  |  | 76.4% 69.4% 61.8% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 80.1% 73.7% 66.7% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 55. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @15 ms WUS periodicy |  |  | 83.7% 76.5% 68.8% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 92.8% 86.2% 79.0% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 10. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @15 ms WUS periodicy |  |  | 93.0% 85.7% 77.8% |  | 0.00% 0.00% 0.00% |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 45.0% 39.2% 32.8% |  | 29.56 28.9 29.41 |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 90. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 39.1% 32.6% 25.7% |  | 45.37 45.15 45.51 |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 67.1% 60.2% 52.7% |  | 22.44 22.85 22.79 |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 64.7% 58.5% 51.8% |  | 29.56 28.9 29.41 |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 55. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 60.8% 54.1% 46.7% |  | 45.37 45.15 45.51 |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 78.0% 70.9% 63.2% |  | 22.44 22.85 22.79 |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 90.0% 83.4% 76.3% |  | 29.56 28.9 29.41 |  |  | SR periodicity = 10ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE | SLS No UE DRX 100% detection reliability (one shot transmission). slot level, WUS detection power is 10. |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @5 ms WUS periodicy |  |  | 88.9% 81.6% 73.8% |  | 45.37 45.15 45.51 |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |
|  | Wake up of gNB triggered by UE wake up signal (WUS), @10 ms WUS periodicy |  |  | 92.0% 84.7% 76.8% |  | 22.44 22.85 22.79 |  |  | SR periodicity = 15ms | FTP3, mean packet interval of 2s, UL traffic only, 1/5/10 UE |  |

Based on the results, it is observed that, with UE WUS signal triggering a BS of 100% detection assumption, with or without UE C-DRX,

- at empty or low load, 6.2%~93% energy savings can be achieved, depending on the traffic load, WUS periodicity and WUS detection power. The gain generally increases as WUS periodicity increases or gNB detection power decreases;

- when traffic load becomes higher, the gain also significantly decreases. At light load, the relative savings can be reduced to 15% or less, and at medium load, the relative savings can be further reduced to 10% or even there is energy increment.

On UPT/access delay/latency, compared to a baseline where the cell is operating with SSB/SIB/RACH monitoring, the scheme has some UPT loss or increased latency, which is reduced as traffic load increases.

On UE power consumption, small negative impact is observed.

=== end ===

| Company | Comments |
| --- | --- |

6.1.2.3	Specification impacts

6.1.2	Technique A-4 XX

6.1.2.1	Description of technique

6.1.2.2	Analysis of performance and impacts

##### === draftTP A-4: start ===

The following captures the results for adaptation of DTX/DRX.

Table 6.1.1-x: BS energy savings by adaptation of DTX/DRX

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT | Access delay/latency/UE power consumption/Other KPI(s), if any | a) Reference configuration b) Baseline configuration/assumption c) Traffic model d) Other evaluation methodology/assumption details/notable settings |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | DRX_offset_alignment | Cat 1 | Low | 29.8% | 0.91% | Access delay/latency: 0.92% UE power consumption: 2.17% | Set 1 Random DRX offset (granularity = 5 ms) VoIP SLS; DRX (40, 4, 10); DRX offset aligned to 0 |  |  |  |
|  |  | Cat 2 |  | 13.7% | 0.91% |  |  |  |  |  |
| OPPO [R1-2211458] | DRX align | Cat 1 | low load(RU-9.3%) | 4.7% | 361.08Mbps(-15.5%) | Access delay/latency: 78.03ms(+50%) | Set 1 UE-specific DRX, SSB with 20 RBs for 20 ms periodicity |  | FTP (0.5MB as packet size, 200ms as mean inter-arrival time) | SLS, C-DRX config: FTP (160,100,8), DRX align |
|  |  |  | low load(RU-0.15%) | 6.7% | 85.91Mbps(-8.7%) | Access delay/latency: 143.55ms(+3.83%) |  |  | IM (0.1MB as packet size, 2s as mean inter-arrival time) | SLS, C-DRX config: IM (320,80,10), DRX align |
|  | DRX align and dropping SSB outside UE active time |  | low load(RU-9.3%) | 14.4% | 361.08Mbps(-15.5%) | Access delay/latency: 78.03ms(+50%) |  |  | FTP (0.5MB as packet size, 200ms as mean inter-arrival time) | SLS, C-DRX config: FTP (160,100,8), DRX align and dropping SSB outside UE active time |
|  |  |  | low load(RU-0.15%) | 70.1% | 85.91Mbps(-8.7%) | Access delay/latency: 143.55ms(+3.83%) |  |  | IM (0.1MB as packet size, 2s as mean inter-arrival time) | SLS, C-DRX config: IM (320,80,10), DRX align and dropping SSB outside UE active time |
| ZTE, Sanechips [R1-2211903] | DRX alignment | 1 | low | 0.3% 0.9% | 5% 1.30% | unfinished packet ratio=(total number of unfinished packet for baseline-total number of unfinished packet for enhanced)/total number of unfinished packet for baseline: 50%, 54.5% for each BS caterogy | Set 1 UE-specific CDRX/CDRX alignment in a cell FTP3 CDRX pattern for FTP3 |  |  |  |
|  |  | 2 |  | 0.2% 0.4% | 5% 1.30% |  |  |  |  |  |
| Spreadtrum [R1-2211241] | traffic concentration (in a transmission window | Cat 1 | Low | 37.8% 34.9% 30.9% |  |  | a) For each BS Category: Set 1, Set 2, Set 3 b)- 1) There are 5% load (UE specific data) in 40 slots every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) Scaling: Sf≈0.21 in 2 slots every 20ms, and Sf≈0.05 in 38 slots every 20ms c)-1) The load is concentrated in first 10ms. There are 10% load (UE specific data) in the first 20 slots every 20ms, zero load in the last 20 slots every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. 3) Scaling: Sf≈0.26 in 2 slots every 20ms, and Sf≈0.1 in 18 slots every 20ms d)- 1) 160ms duration in total. 2) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. Two SSBs and the corresponding SIB1 share a slot. SSB burst and SIB1 take 40 PRBs. 3) PF periodicity at gNB side is 20ms (T=1280ms, N=64). Paging is transmitted in another slot every PF assuming one PO is effective in each PF. Paging takes 40 PRBs. 4) Scaling: Sa=1, Sp=1, P_static=P3. Numerial evaluation resutls. |  |  |  |
|  |  | Cat 2 |  | 31.1% 27.7% 29.2% |  |  |  |  |  |  |
|  | Offload between cells (the offloaded cell is turned off) | Cat 1 | Low | 57.7% 53.5% 47.9% |  |  | a) For each BS Category: Set 1, Set 2, Set 3 b)-1) Cell #1 and cell #2: There are 5% load (UE specific data) in 40 slots every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) Scaling: Sf≈0.21 in 2 slots every 20ms, and Sf≈0.05 in 38 slots every 20ms c)-1) The load in cell #1 is shifted to cell #2. 1.1) Cell #1: There are zero load. There are only SSB burst and SIB1 in 2 slots every 20ms. gNB can enter light sleep for Cat 1, but can only enter micro sleep for Cat 2. 1.2) Cell #2: There are 10% load (UE specific data) every 20ms. The load is frequency multiplexed with SSB burst and SIB1 in 2 slots every 20ms. 2) Scaling: 2.1) Cell #1: Sf≈0.16; 2.2) Cell #2: Sf≈0.26 in 2 slots every 20ms, and Sf≈0.1 in 38 slots every 20ms d)-1) 160ms duration in total. 2) SSB burst periodicity is 20ms, and SIB1 repetition periodicity is 20ms. Two SSBs and the corresponding SIB1 share a slot. SSB burst and SIB1 take 40 PRBs. 3) PF periodicity at gNB side is 20ms (T=1280ms, N=64). Paging is transmitted in another slot every PF assuming one PO is effective in each PF. Paging takes 40 PRBs. 4) Scaling: Sa=1, Sp=1, P_static=P3. Numerial evaluation results. |  |  |  |
|  |  | Cat 2 |  | 46.5% 44.3% 46.6% |  |  |  |  |  |  |
| Intel [R1-2211410] | Enhanced C-DRX | Cat1 | Light | 2.8% | Baseline: 122.3 Mbps ES: 86.4 Mbps | Avg EE (baseline): 5.20 Avg EE (ES): 4.82 | a)Set1 c) FTP3 d) SLS CSI feedback based on SRS; SIB1 BW: 48 PRB; SSB/PRACH/SIB1: 160 msec periodicity; Number of SSB: 1; Slot-level model For scaling: A = 0.4; η(s_f,s_p )=1 for any sf, sp; | Baseline DRX Parameters: DRX Cycle: 80 msec; ON duration 4ms, Inactivity Timer: 40msec For Enh C-DRX, cycle is 80ms and gNB is active for 20ms. |  |  |
|  |  |  | Medium | 29.7% | Baseline: 93.2 Mbps ES: 29.6 Mbps | Avg EE (baseline): 1.87 Avg EE (ES): 2.33 |  |  |  |  |
|  |  |  | Low | 2.3% | Baseline: 111.2 Mbps ES: 186.5 Mbps | Avg EE (baseline): 8.81 Avg EE (ES): 9.37 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Light | 2.3% | Baseline: 98.1 Mbps ES: 66.6 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 4.66 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Light | 2.6% | Baseline: 98.1 Mbps ES: 164.3 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 5.31 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Medium | 30.9% | Baseline: 75.0 Mbps ES: 28.2 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.54 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Medium | 4.8% | Baseline: 75.0 Mbps ES: 116.6 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.04 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Light | 2.8% | Baseline: 122.3 Mbps ES: 86.4 Mbps | Avg EE (baseline): 5.20 Avg EE (ES): 4.82 |  | Baseline DRX Parameters: DRX Cycle: 80 msec;ON duration 4ms, Inactivity Timer: 40msec For Enh C-DRX, cycle is 80ms and gNB is active for 20ms. |  |  |
|  |  |  | Medium | 29.7% | Baseline: 93.2 Mbps ES: 29.6 Mbps | Avg EE (baseline): 1.87 Avg EE (ES): 2.33 |  |  |  |  |
|  |  |  | Low | 2.3% | Baseline: 111.2 Mbps ES: 186.5 Mbps | Avg EE (baseline): 8.81 Avg EE (ES): 9.37 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Light | 2.3% | Baseline: 98.1 Mbps ES: 66.6 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 4.66 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Light | 2.6% | Baseline: 98.1 Mbps ES: 164.3 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 5.31 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Medium | 30.9% | Baseline: 75.0 Mbps ES: 28.2 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.54 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Medium | 4.8% | Baseline: 75.0 Mbps ES: 116.6 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.04 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Light | 2.8% | Baseline: 122.3 Mbps ES: 86.4 Mbps | Avg EE (baseline): 5.20 Avg EE (ES): 4.82 |  | Baseline DRX Parameters: DRX Cycle: 80 msec;ON duration 4ms, Inactivity Timer: 40msec For Enh C-DRX, cycle is 80ms and gNB is active for 20ms. |  |  |
|  |  |  | Medium | 29.7% | Baseline: 93.2 Mbps ES: 29.6 Mbps | Avg EE (baseline): 1.87 Avg EE (ES): 2.33 |  | Baseline DRX Parameters: DRX Cycle: 80 msec; ON duration 4ms, Inactivity Timer: 40msec For Enh C-DRX, cycle is 80ms and gNB is active for 20ms. |  |  |
|  |  |  | Low | 2.3% | Baseline: 111.2 Mbps ES: 186.5 Mbps | Avg EE (baseline): 8.81 Avg EE (ES): 9.37 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Light | 2.3% | Baseline: 98.1 Mbps ES: 66.6 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 4.66 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Light | 2.6% | Baseline: 98.1 Mbps ES: 164.3 Mbps | Avg EE (baseline): 5.31 Avg EE (ES): 5.31 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
|  |  |  | Medium | 30.9% | Baseline: 75.0 Mbps ES: 28.2 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.54 |  | Baseline DRX Parameters: DRX Cycle: 160 msec;ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 40ms |  |  |
|  |  |  | Medium | 4.8% | Baseline: 75.0 Mbps ES: 116.6 Mbps | Avg EE (baseline):1.97 Avg EE (ES): 2.04 |  | Baseline DRX Parameters: DRX Cycle: 160 msec; ON duration 8ms, Inactivity Timer: 100msec For Enh C-DRX, cycle is 160ms and gNB is active for 80ms |  |  |
| CATT [R1-2211210] | Adaptation of DTX/DRX | Cat 1 | Low load | 71.4% |  |  | Set1 | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup;SSB periodicity: 20ms; CSI-RS/TRS periodicity: 10ms. |
|  |  |  | Light load | 62.6% |  |  |  |  | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes |  |
|  |  |  | Medium load | 47.8% |  |  |  |  | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes |  |

Based on the results, DTX/DRX adaptation achieves BS energy savings gain by 0.2%~71.4% depending on the traffic and UE DRX configurations. At low or light traffic load cases, four results (MediaTek, OPPO, Spreadtrum, CATT) show that the gain can be larger than 14.4% and becomes larger when traffic further increases, while three results (OPPO, ZTE, Intel) show less than 6.7% gain at low and light load cases. Particularly, dropping SSB outside UE active time can further increase the energy savings, and with baseline of UE DRX alignment, the gain can be marginal, less than 1%.

On UPT, one result shows there is marginal negative impact while one result shows it can be up to 15.5%. Also one result shows that the impact on UPT varies: when the UE DRX cycle is 160ms and gNB active time is 80ms, the UPT is increased while in other configurations, there can be large UPT loss.

On access delay/latency, one result shows marginal increment while one result shows the increment can be up to 50%. Also, about 50% unfinished packet ratio is observed from one result.

Additionally, one result shows that at low/light traffic load, the average EE may be negatively impacted while at medium load, the average EE is slightly increased.

=== end ===

| Company | Comments |
| --- | --- |

6.1.2.3	Specification impacts

6.1.2	Technique A-6 XX

6.1.2.1	Description of technique

6.1.2.2	Analysis of performance and impacts

##### === draftTP A-6: start ===

The following capture the results for adaptation of SSB and/or SIB1.

Table 6.1.1-x: BS energy savings by adaptation of SSB and/or SIB1

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT | Access delay/latency/UE power consumption/Other KPI(s), if any | Reference configuration | Baseline configuration/assumption | Traffic model/Other evaluation methodology/assumption details/notable settings |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CMCC [R1-2211692] | SSB/SIB1-less scheme: gNB has 2 co-deployed CCs, both of them are available for UE with single carrier operation to access, but only CC1 has normal SSB and SIB1 with default 20ms transmission period. CC2 only has PSS/SSS for synchronization. | cat.2 | Zero | 31.4% | / | CC1 carries SIB1 of CC2, the power consumption of CC1 increases 1.73% for FDM SIB of both CC. | set 1 | Baseline scheme: gNB has 2 co-deployed CCs, both of them are available for UE with single carrier operation to access, so both CC1 and CC2 has SSB and SIB1 with default 20ms transmission period. As shown in Figure.5 (a). | numerical analysis. •SIB1: -Baseline: for both CC1 and CC2, PDCCH: 2 symbols, 48RB; PDSCH: 12RBs, 12 OFDM symbols including DMRS. -SSB/SIB1-less scheme: no SIB1 on CC2, but CC1 carries SIB1 for CC2, so the TBS will be doubled. The number of PDSCH PRBs is 24 RBs, 12OFDM symbols. PDCCH still occupies 2 OFDM symbols, 48 PRBs. •SSB1 and SSB are transmitted in different slots, e.g. value in Table 13-11 is assumed to be 5ms. •ղ=1, A=0.4. •Time unit for power model is slot. power consumption is calculated in a 40ms long period |
|  |  | cat.1 |  | 56.5% | / | CC1 carries SIB1 of CC2, the power consumption of CC1 increases 1.41% for FDM SIB of both CC. |  |  |  |
| Huawei, HiSilicon [R1-2210858] | SIB-less on ES CC | Cat 2 | 0% load(zero) | 33.6% | N/A | N/A | Set 2 | 4 SIB1 with 20ms period,20RB | FTP3 IM. NO C-DRX; Subband based CSI-feedback in every 5 slots. slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |
|  |  |  | 10% load(low) | 26.2% | N/A | N/A |  |  |  |
|  |  |  | 20% load(light) | 19.0% | N/A | N/A |  |  |  |
|  |  |  | 30% load(medium) | 16.0% | N/A | N/A |  |  |  |
|  | dual SIB on Anchor CC |  | 0% load(zero) | -7.5% | N/A | N/A |  |  |  |
|  |  |  | 10% load(low) | -6.7% | N/A | N/A |  |  |  |
|  |  |  | 20% load(light) | -6.1% | N/A | N/A |  |  |  |
|  |  |  | 30% load(medium) | -5.5% | N/A | N/A |  |  |  |
|  | SIB-less on ES CC |  | 0% load(zero) | 20.2% | N/A | N/A |  | 4 SIB1 with 40ms period,20RB |  |
|  |  |  |  | 11.2% | N/A | N/A |  | 4 SIB1 with 80ms period,20RB |  |
|  |  |  |  | 5.9% | N/A | N/A |  | 4 SIB1 with 160ms period,20RB |  |
|  |  |  | 10% load(low) | 15.7% | N/A | N/A |  | 4 SIB1 with 40ms period,20RB |  |
|  |  |  |  | 9.3% | N/A | N/A |  | 4 SIB1 with 80ms period,20RB |  |
|  |  |  |  | 4.0% | N/A | N/A |  | 4 SIB1 with 160ms period,20RB |  |
| ZTE, Sanechips [R1-2211903] | offloading SIB of the SIB-less cell to another cell. and SIB-less operation is for non-CA case | 1 | zero | 97.9% 95.4% 91.1% |  |  | Set 1 | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell | FTP3 for Set 1. IM for Set 2. For the multiplexing pattern of two SIBs in the anchor cell (when applicable), TDM is considered in the evaluations. |
|  |  |  |  | 19.3% 24.6% 23.5% |  |  |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 14.1% 18.9% 18.1% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | 64.3% 43.6% 28.0% |  |  |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  |  | 15.5% 13.6% 8.8% |  |  |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 11.6% 10.5% 6.8% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  | 2 | zero | 85.8% 83.6% 82.8% |  |  |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  |  | 12.1% 7.0% 3.7% |  |  |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 8.0% 4.6% 2.4% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | 24.5% 13.4% 9.4% |  |  |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  |  | 10.8% 6.2% 3.3% |  |  |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 7.5% 4.3% 2.3% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+40ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  | 1 | zero | 98.4% 96.2% 92.6% |  |  | Set 2 | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  |  | 40.7% 38.4% 37.0% |  |  |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 17.6% 12.4% 12.0% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | 80.3% 59.4% 42.4% |  |  |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  |  | 28.0% 16.0% 11.5% |  |  |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 17.8% 10.8% 7.7% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  | 2 | zero | 87.5% 82.6% 81.4% |  |  |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  |  | 28.2% 9.8% 5.3% |  |  |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 14.1% 4.9% 2.6% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
|  |  |  | low | 42.9% 23.6% 19.1% |  |  |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell |  |
|  |  |  |  | 23.9% 8.0% 4.3% |  |  |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ SIB1-less for non-anchor cell |  |
|  |  |  |  | 14.0% 5.0% 2.0% |  | energy increase for anchor cell with SIB1 transmission for SIB1-less cell |  | SSB+SIB: {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;/ (SSB+SIB1)-less for non-anchor cell, anchor cell with SIB1 transmission for SIB1-less cell |  |
| NOKIA/NSB [R1-2211097] | SSB-less at 20 ms period of RO | CAT2 | Unloaded/low/light/Medium | 27.5% 26.1% 26.0% 22.7% | / 135 Mbps 105 Mbps 74 Mbps |  | SET 1 | Intra-band/collocated cells with non-CA case, consisting of: * Coverage cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring * Capacity cell with 20 ms periodicity of SSB/SIB1 Tx and RO monitoring UEs initially in RRC Idle state. | DL-FTP3. SLS+Post-processing |
|  | SSB-less at 160 ms period of RO |  | Unloaded/low/light/Medium | 51.8% 48.2% 47.3% 43.2% | / 85 Mbps 72 Mbps 56 Mbps |  |  |  |  |
|  | SIB1-less at 20 ms period of RO |  | Unloaded/low/light/Medium | 43.1% 40.5% 40.1% 36.5% | / 132 Mbps 104 Mbps 73 Mbps |  |  |  |  |
|  | SIB1-less at 160 ms period of RO |  | Unloaded/low/light/Medium | 53.8% 51.2% 47.3% 43.2% | / 84 Mbps 72 Mbps 56 Mbps |  |  |  |  |
|  | SSB&SIB1-less at 20 ms period of RO |  | Unloaded/low/light/Medium | 52.7% 50.6% 52.7% 44.4% | / 135 Mbps 105 Mbps 74 Mbps |  |  |  |  |
|  | SSB&SIB1-less at 160 ms period of RO |  | Unloaded/low/light/Medium | 55.1% 53.1% 54.9% 46.8% | / 85 Mbps 72 Mbps 56 Mbps |  |  |  |  |
| CATT [R1-2211210] | Adaptation of SSB/SIB1 | Cat 1 | Low load | 22.0% |  |  | set1 | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms; | FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes. SLS;Cell OFF:Without normal SSB/SIB/CSI-RS transmission within Cell off duration;On demand SSB transmission is trigger by neighbour cell with 300ms transmission duration and 20ms SSB. For the case with DRX, DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup; A=0.4; η(s_f, s_p)=1. |
|  |  |  |  | 43.4% |  |  |  | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms;DTX configuration: gNB starting offset of DTX on locate before UE DRX on duration in order to support UE wakeup; |  |
| Fujitsu [R1-2211085] | SSB&SIB-less | Cat2 | Zero low light medium | 34.5% 27.0% 21.7% 16.7% |  |  | Set 1 | Baseline scheme: 20 ms SSB/SIB1 period | BS goes into mico-sleep on symbolc w/o TX/RX simplified SSB which contains SSS and PSS is transmitted with periodicity of 160 ms No UE DRX. A=0.4, η=1 |
|  |  |  | Zero low light medium | 36.0% 24.7% 18.4% 13.5% |  |  | Set 2 |  |  |
| Ericsson [R1-2212154] | 20ms Discovery signal (4 symbols) + no SIB1 | Cat1 | Zero | 2.6% / 5.9% |  |  | Set 1 | Baseline scheme: 20ms SSB + 160ms SIB1 | one SSB/ four SSBs Energy calculation: per symbol energy consumption is modeled. According to Rel-15 specification, SIB1 can be transmitted with variable transmission repetition periodicity within a 160 ms period, including one SIB1 PDSCH transmission every 160ms or even sparser. |
| Qualcomm [R1-2212128] | on-demand SIB1 | Cat 1 | Empty load | 5.8% / 7.7% / 8.6% |  |  | Set 1 | Baseline: 20ms periodicity for SSB/SIB1/RO, one beam Enhanced: 20%/10%/5% SIB1 Tx rate, C-WUS with 20ms periodicity |  |
|  |  |  |  | 32.1% / 36.6% / 38.8% |  |  |  | Baseline: 20ms periodicity for SSB/SIB1/RO, 8 beams Enhanced: 20%/10%/5% SIB1 Tx rate, C-WUS with 20ms periodicity |  |

One result (Ericsson) shows that with a 4-symbol Discover signal (DRS), and without SIB1 transmission, 2.6% and 5.9% energy savings can be achieved for one SSB and four SSB respectively, at empty load.

One result (CATT) shows on-demand SSB can achieve BS energy savings by 22.0%/43.4% at low load compared to a baseline of 20ms SSB periodicity with or without gNB DTX configuration.

One result (Qualcomm) is provided with on-demand SIB1 at empty load, 5.8%~8.6% BS energy savings can be achieved at SIB1 transmission rate of 20%~5% for one SSB beam, and the gains can increase to 32.1%~38.8% for 8 beams case for a same SIB1 transmission rate range.

For the results other than the above, the observation is to be made together with techniques evaluation for B-1, since company results assume multiple carriers are available at the gNB but not necessarily CA from UE point of view.

=== end ===

| Company | Comments |
| --- | --- |

6.1.2.3	Specification impacts

6.1.z	Impacts on network interfaces

6.1.aa	Cell DTX/DRX

6.1.aa.1	Description of technique

6.1.aa.2	Analysis of performance and impacts

6.1.aa.3	Specification impacts

6.1.aa.4	Higher layer procedures

Cell DTX/DRX is applied to at least UEs in RRC_CONNECTED state. A periodic Cell DTX/DRX (i.e., active and non-active periods) can be configured by gNB via RRC signalling. Below examples on Cell DTX/DRX behaviour during non-active periods are assumed to be possible options, and the UE behaviour/impact will be studied:

- Example 1: gNB is expected to turn off all transmission and reception for data traffic and reference signal during Cell DTX/DRX non-active periods.

- Example 2: gNB is expected to turn off its transmission/reception only for data traffic during Cell DTX/DRX non-active periods (i.e., gNB will still transmit/receive reference signals)

- Example 3: gNB is expected to turn off its dynamic data transmission/reception during Cell DTX/DRX non-active periods (i.e., gNB is expected to still perform transmission/reception in periodic resources, including SPS, CG-PUSCH, SR, RACH, and SRS).

- Example 4: gNB is expected to only transmit reference signals (e.g., CSI-RS for measurement).

The study will focus on UE behavior when at any point in time the cell activates a single DTX/DRX configuration.

Editor's note: FFS if multiple sets of Cell DRX/DTX configuration are allowed.

The Cell DTX mode/configuration can also be indicated to the UE via dynamic L1/L2 signalling. The dynamic L1/L2 signalling at least supports UE dedicated indication. Whether UE group common signalling is also supported will be further studied.

It is beneficial to align UE DRX with Cell DTX and DRX alignment among multiple UEs. The alignment mechanism will be studied.

Editor's note: FFS details of alignment, including UE transmission/reception behavior during DTX.

6.1.aa.5	Assistance information from UE side

Editor's note: will be updated once more agreements are made.

#### 6.1.aa.6	Impacts on network interfaces

Editor's note: will be updated once more agreements are made.

The cell DTX/DRX information is considered necessary to be exchanged and coordinated between neighbour gNBs. The gNB can use the received cell DTX/DTX information to determine its own cell DTX/DRX configuration for network energy saving purpose.

Editor’s note: The details of cell DTX/DRX is finally up to RAN1 and RAN2.

6.2	Techniques in frequency domain

6.2.1	Technique B-1 YY

6.2.1.1	Description of technique

6.2.1.2	Analysis of performance and impacts

##### === draftTP B-1: start ===

The following capture the results by multi-carrier energy savings enhancements.

Table 6.1.1-x: BS energy savings by multi-carrier enhancements

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | KPI | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- |
| Huawei，HiSilicon [R1-2210858] | Inter-band SSB-less on Scell | Cat 2 | 0% load(zero) | 14.4% |  | 4 SSB beams with 20ms period, 20RB |
|  |  |  | 10% load(low) | 9.3% |  |  |
|  |  |  | 20% load(light) | 7.4% |  |  |
|  |  |  | 30% load(medium) | 5.7% |  |  |
| ZTE, Sanechips [R1-2211903] | SSB-less SCell | 1 | zero load | 97.3% |  | SSB20ms for baseline; |
|  |  |  |  | 93.9% |  | SSB80ms for baseline; |
|  |  |  |  | 88.4% |  | SSB160ms for baseline; |
|  |  | 2 |  | 83.8% |  | SSB20ms for baseline; |
|  |  |  |  | 82.4% |  | SSB80ms for baseline; |
|  |  |  |  | 82.1% |  | SSB160ms for baseline; |
|  |  | 1 |  | 97.3% |  | SSB20ms for baseline; |
|  |  |  |  | 93.8% |  | SSB80ms for baseline; |
|  |  |  |  | 88.3% |  | SSB160ms for baseline; |
|  |  | 2 |  | 82.1% |  | SSB20ms for baseline; |
|  |  |  |  | 80.7% |  | SSB80ms for baseline; |
|  |  |  |  | 80.4% |  | SSB160ms for baseline; |
|  |  | 1 | low | 58.4% | UPT:801.79, SSB-less UPT：812.57 | SSB20ms for baseline; |
|  |  |  |  | 35.2% | UPT:804.41, SSB-less UPT：812.57 | SSB80ms for baseline; |
|  |  |  |  | 21.2% | UPT:804.54, SSB-less UPT：812.57 | SSB160ms for baseline; |
|  |  | 2 |  | 15.2% | UPT:801.79, SSB-less UPT：812.57 | SSB20ms for baseline; |
|  |  |  |  | 7.4% | UPT:804.41, SSB-less UPT：812.57 | SSB80ms for baseline; |
|  |  |  |  | 6.1% | UPT:804.54, SSB-less UPT：812.57 | SSB160ms for baseline; |
|  |  | 1 |  | 72.7% | UPT:115.30, SSB-less UPT：119.41 | SSB20ms for baseline; |
|  |  |  |  | 51.7% | UPT:118.78, SSB-less UPT：119.41 | SSB80ms for baseline; |
|  |  |  |  | 34.9% | UPT:119.324565032046, SSB-less UPT：119.41 | SSB160ms for baseline; |
|  |  | 2 |  | 24.9% | UPT:115.30, SSB-less UPT：119.41 | SSB20ms for baseline; |
|  |  |  |  | 16.9% | UPT:118.78, SSB-less UPT：119.41 | SSB80ms for baseline; |
|  |  |  |  | 15.5% | UPT:119.328, SSB-less UPT：119.41 | SSB160ms for baseline; |
|  |  |  |  | 39.4% |  | SSB20ms for baseline; |
|  |  |  |  | 22.4% |  | SSB80ms for baseline; |
|  |  |  |  | 18.7% |  | SSB160ms for baseline; |
| Vivo [R1-2211018 R1-2212541] | Inter-band CA with SSB-less carriers/Scell (ES scheme: CC 1: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening; CC 2: neither transmission nor reception) | Cat 1 | 0% | 14.7% | UE power consumption: 0% | Baseline scheme: CC 1: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening; CC 2: only 20ms SSB |
|  |  | Cat 2 | 0% | 5.1% |  |  |
| Intel [R1-2211410] | inter-band SSB-less Scell | Cat1 | Low | 3.0% | UPT: 1639.3 Mbps;Avg EE (baseline): 6.56; Avg EE (ES): 6.81 | Baseline: CC# 2 (Scell): 160 msec SSB, no SIB1/PRACH, ES: CC# 2 (Scell): no SSB/SIB1/PRACH, |
|  |  | Cat1 | Light | 1.0% | UPT:1222.9 Mbps;Avg EE (baseline): 2.96; Avg EE (ES): 3.00 |  |
|  |  | Cat1 | Medium | 0.3% | UPT: 915.8Mbps;Avg EE (baseline): 1.57; Avg EE (ES): 1.57 |  |
| MTK [R1-2212259] | SCell_w/o_SIB1 | Cat 1 | Light | 2.3% | UPT: 0.00%; Access delay/latency: 0%; UE power consumption: 0% | SCell has SSB and SIB1 |
|  |  | Cat 2 |  | 1.1% |  |  |
|  | SCell_w/o_SSB_SIB1 | Cat 1 |  | 7.9% |  |  |
|  |  | Cat 2 |  | 1.3% |  |  |
| CMCC [R1-2211692] | SCell with simplified SSB: SCell with only PSS/SSS, with 20ms periodicity. PCell with normal SSB, SIB1 and also SIB information for SCell. | cat.2 | Zero | 5.7% | N/A | Baseline: normal SSB on SCell. PCell with normal SSB, SIB1 and also SIB1 information for SCell. |
|  |  | cat.1 |  | 10.5% |  |  |
| Vivo [R1-2211018 R1-2212541] | SSB/SIB-less carrier operation with assistance of anchor carrier (ES scheme: CC 1: 20ms SSB and SIB1(with 72 PRB), 20ms RACH listening; CC 2: only 20ms RACH listening) | Cat 1 | 0% | 14.8% |  | Baseline scheme: CC 1: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening; CC 2: 20ms SSB and SIB1(with 48 PRB), 20ms RACH listening |
|  |  | Cat 2 | 0% | 9.1% |  |  |
| CATT [R1-2211210] | Multi-carrier energy savings enhancements | Cat 1 | Low load | 25.7% |  | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms;CSI-RS/TRS 10ms;Rel-17 Scell activation/deactivation; |
|  |  |  | Light load | 24.1% |  |  |
|  |  |  | Medium load | 15.5% |  |  |
|  |  |  | Low load | 30.3% |  |  |
|  |  |  | Light load | 29.1% |  |  |
|  |  |  | Medium load | 20.3% |  |  |
| Qualcomm [R1-2212128] | Dynamic UE-group Pcell switching | Cat 1 | Medium (39% RU for 1 CC; 22% RU across 2 CCs) | 37.5% | UPT: -14% | Assumption: Number of UEs changes from 25 to 20 Baseline: Keep 2 CCs activated Enhancement: deactivate 1 CC and keep 1CC activated |

Observation includes the results for techniques that are also evaluated under #A-6. The following is observed.

In general, for SSB and/or SIB saved from one carrier of two carriers, BS energy savings can be achieved in all results.

With one of two carriers having simplified SSB and no SIB1, one result shows BS energy saving gain can be achieved by 31.4%~56.5% compared with a baseline of both carriers having SSB and SIB1 periodicity of 20ms; the same source company result also show that with CA configured by which SIB1 is saved from SCell, the gain can be achieved by 5.7%~10.5%.

With SIB-less only from one of two carriers,

- one result shows that 33.6%~16.0% BS energy saving gain is observed which decreases as the traffic load increases, compared with 20ms SIB1 periodicity. Meanwhile, the SIB1 carried on another carrier increase the energy of that carrier by 7.5%~5.5%, resulting a total saving across two carries by 26.1%~10.5%. The gain decreases to 4.0% when the baseline SIB1 periodicity increases to 160ms;

- one result shows BS energy saving gain can be 3.3%~40.7% compared with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell;

- one result shows at different loads, compared to baseline of 20 ms SSB/SIB1 periodicity, that BS energy savings can be achieved by 53.8%~36.5% with RO periodicity of 20ms and 160ms;

- also one result show less than 2.3% BS energy savings when compared with a baseline of SCell having SIB1.

With SSB-less only from one of two carriers, for CA in which case the SIB is already saved from SCell,

- at different loads, compared to a baseline of 20 ms SSB/SIB1 periodicity, one result shows BS energy savings by 27.5%~22.7% and 51.8%~43.2% when the RO periodicity is 20ms and 160ms respectively;

- two results show that BS energy savings can be 5.1%~14.7% at different loads, compared to a baseline of 20ms SSB periodicity;

- one result shows that compared to baseline of SSB periodicities of 20ms~160ms, BS energy savings can be 80.4%~97.3% at empty load, and 6.1%~72.7% at low load;

- one result also shows that when the baseline SCell SSB periodicity is 160ms, only 0.3%~3% BS energy savings can be achieved, and one another shows BS energy savings by less than 7.9% if compared with SCell having SIB1.

With both SSB-less and SIB1-less from one of two carriers, for non-CA

- compared to baseline of 20 ms SSB/SIB1 periodicity on both carriers, one result shows BS energy savings by 55.1%~44.4% with RO periodicity of 20ms~160ms at different loads, and one result shows 9.1%~14.8% energy savings at empty load;

- at different loads, compared to baseline of 20 ms SSB/SIB1 periodicity, one result shows BS energy savings by 36.0%~13.5% when combined with simplified SSB (i.e. PSS and SSS only);

- one result shows that with baseline of SSB+SIB periodicity of {20ms+20ms, 80ms+80ms, 160ms+160ms} for anchor cell and non-anchor cell, BS energy savings can be 9.4%~97.9% if an anchor carrier carries the SSB and SIB1 for another carrier without additional/separate SIB1 depending on the traffic load, and the savings can be 2.0%~18.9% if the SIB1 for another carrier is TDMed transmitted on the anchor carrier.

In most results for SSB and/or SIB saved from one carrier of two carriers, the UPT is not negatively impacted while one result shows slightly increased UPT.

No negative impact observed on UE power consumption for the above schemes.

Additionally, inter-band SSB-less for CA can slightly improve the average EE, reported by one result.

Another scheme with dynamic UE-group PCell switching shows 37.5% BS energy savings at medium load for BS Category 1, at the expense of UPT loss by 14%.

=== end ===

| Company | Comments |
| --- | --- |

6.2.1.3	Specification impacts

6.2.2	Technique B-2 YYY

6.2.2.1	Description of technique

6.2.2.2	Analysis of performance and impacts

##### === draftTP B-2: start ===

The following capture the results for dynamic adaptation of bandwidth part of UE(s) within a carrier.

Table 6.1.1-x: BS energy savings by dynamic BWP adaptation within carrier

| Company | BS Category | Load scenario | ES gain (%) | KPI | Reference configuration | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- |
| Samsung [R1-2212056] | Cat 1 | Baseline traffic: 42.8 % RU Reduced traffic: 28.47 % RU | 38.2% | UPT: 6.05%; Packet latency: 6.44%; Scheduling latency: No increase | Set1 | 100MHz with 55 dBm |
|  | Cat 2 |  | 27.8% |  |  |  |
|  | Cat 1 | Baseline traffic: 7.5 % RU Reduced traffic: 2.75 % RU | 52.2% | UPT: 14.67%; Packet latency: 17.2%; Scheduling latency: No increase |  |  |
|  | Cat 2 |  | 17.6% |  |  |  |
|  | Cat 1 | Baseline traffic: 32.1 % Reduced traffic: 25.7 % | 17.4% | UPT: 28.24%; Packet latency: 39.4%; Scheduling latency: No increase | Set3 | 100MHz with 49 dBm |
|  | Cat 2 |  | 17.8% |  |  |  |

One result observes BS energy savings by 17.4%~52.2% at the expense of UPT loss by 28.4%~14.47%, when traffic is reduced compared to corresponding baseline.

On packet latency/scheduling latency, no negative impact is observed.

=== end ===

| Company | Comments |
| --- | --- |

6.2.2.3	Specification impacts

6.2.3	Technique B-3 YYY

6.2.2.1	Description of technique

6.2.2.2	Analysis of performance and impacts

##### === draftTP B-3: start ===

The following captures the results for dynamic adaptation of bandwidth of active BWP.

Table 6.1.1-x: BS energy savings by dynamic BW adaptation within BWP

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | KPI | Baseline configuration/assumption |
| --- | --- | --- | --- | --- | --- | --- |
| OPPO [R1-2211458] | Dynamic adaptation of bandwidth of active BWP of UEs | Cat 1 | low load(RU-10%) | 1.4% | UPT: 554.74Mbps(-46.8%); Access delay/latency: 9.35ms(+86.3%) | system BW of 100MHz, 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8) |
|  |  |  | low load(RU-0.2%) | 1.3% | UPT: 513.43Mbps(-52%); Access delay/latency: 1.78ms(+48.3%) |  |
| Intel [R1-2211410] | intra-carrier BWP adaptation |  | Low | -20.6% | UPT: Baseline (819.7Mbps), ES (346.8 Mbps); Avg EE (baseline): 5.10; Avg EE (ES): 1.87 | Baseline: Full BW ES: 50% BW |
|  |  |  |  | -75.4% | UPT: Baseline (819.7Mbps), ES (99.4 Mbps); Avg EE (baseline): 5.10; Avg EE (ES): 0.54 | Baseline: Full BW ES: 25% BW |
|  |  |  | Light | -45.9% | UPT: Baseline (611.5Mbps), ES (155.2Mbps); Avg EE (baseline): 2.66; Avg EE (ES): 0.69 | Baseline: Full BW ES: 50% BW |
|  |  |  |  | -61.8% | UPT: Baseline (611.5Mbps), ES (25.7Mbps); Avg EE (baseline): 2.66 Avg EE (ES): 0.26 | Baseline: Full BW ES: 25% BW |
|  |  |  | Medium | -27.6% | UPT: Baseline (457.9Mbps), ES (50.5Mbps); Avg EE (baseline): 1.50 Avg EE (ES): 0.44 | Baseline: Full BW ES: 50% BW |
|  |  |  |  | -13.5% | UPT: Baseline (457.9Mbps), ES (12.3Mbps); Avg EE (baseline): 1.50; Avg EE (ES): 0.44 | Baseline: Full BW ES: 25% BW |

Two results show different observations. One result (OPPO) show small BW energy saving gain by 1.3%/1.4% at the expense of about 50% UPT loss and increased access delay/latency by 48.3%/86.3%. One result (Intel) shows BS energy is increased by 13.5%~75.4%, also together with significantly reduced UPT, and additionally reduced average EE.

=== end ===

| Company | Comments |
| --- | --- |

6.2.1.3	Specification impacts

6.2.z	Impacts on network interfaces

6.3	Techniques in spatial domain

6.3.1	Technique C-1 ZZ

6.3.1.1	Description of technique

6.3.1.2	Analysis of performance and impacts

##### === draftTP C-1: start ===

The following capture the results for dynamic adaptation of spatial elements.

Table 6.1.1-x: BS energy savings by adaptation of spatial elements

| Company | ES scheme | BS Category &Reference configuration | Load scenario | ES gain (%) | UPT/latency/UE power/ Other KPIs | Baseline configuration/assumption | Evaluation methodology/assumption details/traffic model |
| --- | --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | #TxRU_32 | Cat 1, Set 1 | Light | 15.8% | UPT loss:4.54%; latency increase:4.76%; UE power increase:3.48% | BS #TxRU 64 | SLS; DRX (160, 8, 100); FTP3 traffic |
|  |  | Cat 2, Set 1 |  | 15.8% | UPT loss:4.54%; latency increase:4.76%; UE power increase:3.48% |  |  |
|  | #TxRU_16 | Cat 1, Set 1 |  | 19.2% | UPT loss:16.92%; latency increase:20.36%; UE power increase:14.70% |  |  |
|  |  | Cat 2, Set 1 |  | 22.1% | UPT loss:16.92%; latency increase:20.36%; UE power increase:14.70% |  |  |
|  | #TxRU_8 | Cat 1, Set 1 |  | 22.1% | UPT loss:47.48%; latency increase:90.42%; UE power increase:47.94% |  |  |
|  |  | Cat 2, Set 1 |  | 24.9% | UPT loss:47.48%; latency increase:90.42%; UE power increase:47.94% |  |  |
|  | #TxRU_32 | Cat 1, Set 1 | Medium | 25.3% | UPT loss:6.39%; latency increase:6.83%; UE power increase:4.20% |  |  |
|  |  | Cat 2, Set 1 |  | 26.6% | UPT loss:6.39%; latency increase:6.83%; UE power increase:4.20% |  |  |
|  | #TxRU_16 | Cat 1, Set 1 |  | 31.4% | UPT loss:44.78%; latency increase:81.08%; UE power increase:32.85% |  |  |
|  |  | Cat 2, Set 1 |  | 36.7% | UPT loss:44.78%; latency increase:81.08%; UE power increase:32.85% |  |  |
|  | #TxRU_8 | Cat 1, Set 1 |  | 36.0% | UPT loss:87.08%; latency increase:647.07%; UE power increase:79.99% |  |  |
|  |  | Cat 2, Set 1 |  | 45.2% | UPT loss:87.08%; latency increase:647.07%; UE power increase:79.99% |  |  |
|  | #TxRU_32_PDSCH_PowOffset_-3dB | Cat 1, Set 1 | Light | 18.8% | UPT loss:9.06%; latency increase:9.96%; UE power increase:7.62% | BS #TxRU 64; PDSCH power offset 0 dB | SLS; DRX (160, 8, 100); FTP3 traffic model; Single value η (=1) |
|  |  | Cat 2, Set 1 |  | 19.7% | UPT loss:9.06%; latency increase:9.96%; UE power increase:7.62% |  |  |
| OPPO [R1-2211458] | Dynamic adaptation of spatial elements. | Cat 1,Set 1 | low load(RU-10%) | 22.1% | UPT: 550Mbps(-47.2%); latency: 12.41ms(+147%) | system BW of 100MHz, 64T: (M, N, P, Mg, Ng, MP, NP,) = (8, 8, 2, 1, 1, 4, 8) | SLS, 8T: (M, N, P, Mg, Ng, MP, NP,) = (4, 2, 2, 1, 1, 2, 2) is used for evaluation; FTP3 traffic model; A = 0.4 and η=1 |
|  |  |  | low load(RU-0.2%) | 13.7% | UPT: 782.56Mbps(-21.2%); latency:1.79ms(+49.1%) |  | SLS, 8T: (M, N, P, Mg, Ng, MP, NP,) = (4, 2, 2, 1, 1, 2, 2) is used for ES evaluation,; FTP3 IM traffic model; A = 0.4 and η=1 |
| Huawei,HiSilicon [R1-2210858] | Dynamic TRX adaption with Multiple CSIs | Cat 2,Set 1 | 10% load(low) | 7.7% | 0% UPT loss | Dynamic TRX adaption with Single 64T CSI; | NO C-DRX; Subband based CSI-feedback in every 5 slots; FTP3 IM traffic model; A=0.4; η=1, 0.76 |
|  |  |  |  | 4.0% | 5% UPT loss |  |  |
|  |  |  |  | 3.4% | 10% UPT loss |  |  |
|  |  |  | 30% load(medium) | 13.0% | 0% UPT loss |  |  |
|  |  |  |  | 11.3% | 5% UPT loss |  |  |
|  |  |  |  | 9.6% | 10% UPT loss |  |  |
|  |  |  | 10% load(low) | 7.5% | 0% UPT loss |  | C-DRX with (cycle, on-duration, inactivity timer) = (320, 10, 80) ms; Subband based CSI-feedback in every 5 slots; FTP3 IM traffic model; A=0.4; η=1, 0.76 |
|  |  |  | 30% load(medium) | 10.9% | 0% UPT loss |  |  |
|  |  | Cat 2,Set 2 | 10% load(low) | 7.5% | 0% UPT loss |  | NO C-DRX; Subband based CSI-feedback in every 5 slots; FTP3 IM traffic model; A=0.4; η=1, 0.76) |
|  |  |  |  | 13.2% | 5% UPT loss |  |  |
|  |  |  |  | 10.2% | 10% UPT loss |  |  |
|  |  |  | 30% load(medium) | 10.3% | 0% UPT loss |  |  |
|  |  |  |  | 19.2% | 5% UPT loss |  |  |
|  |  |  |  | 14.8% | 10% UPT loss |  |  |
| ZTE,Sanechips [R1-2211903] | TxRU reduction 48TxRU | Cat 2, Set 1 | Low load(RU=8.8%) | 7.8% | 1.5% UPT loss | Baseline: 64TxRU | FTP3: 20K packet size;η=1 |
|  | TxRU reduction 32TxRU |  | Low load(RU=8.8%) | 15.5% | 4.47% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | Low load(RU=8.8%) | 23.5% | 11.06% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | light load(RU=20%) | 10.8% | 1.5% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | light load(RU=20%) | 21.7% | 7.06% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=20%) | 33.7% | 15.31% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | medium load(RU=32%) | 12.5% | 3.34% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | medium load(RU=32%) | 24.6% | 10.44% UPT loss |  |  |
|  | Dynamic TxRUs adaptation via multi-CSI |  | Low load(RU=8.8%) | 27.1% | 0.9% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 28.7% | 1.5% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 31.3% | 7% UPT loss |  |  |
|  |  |  | medium load(RU=32%) | 23.8% | 1.17% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | Low load(RU=10%) | 5.6% | 6.89% UPT loss |  | FTP3: 0.1M packet size,η=1 |
|  | TxRU reduction 32TxRU |  | Low load(RU=10%) | 11.0% | 18.39% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | light load(RU=20%) | 9.1% | 6.32% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | light load(RU=20%) | 18.6% | 14.88% UPT loss |  |  |
|  | TxRU reduction 48TxRU |  | light load(RU=40%) | 11.8% | 8.01% UPT loss |  |  |
|  | TxRU reduction 32TxRU |  | light load(RU=40%) | 25.0% | 20.88% UPT loss |  |  |
|  | Dynamic TxRUs adaptation via multi-CSI |  | Low load(RU=10%) | 7.6% | 3.1% UPT loss |  |  |
|  |  |  | Low load(RU=10%) | 11.1% | 5.04% UPT loss |  |  |
|  |  |  | Low load(RU=10%) | 12.7% | 6.03% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 13.8% | 2.52% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 16.3% | 4.13% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 18.7% | 5.15% UPT loss |  |  |
|  |  |  | light load(RU=20%) | 21.1% | 6.96% UPT loss |  |  |
|  |  |  | light load(RU=40%) | 15.7% | 2.89% UPT loss |  |  |
|  |  |  | light load(RU=40%) | 17.1% | 4.16% UPT loss |  |  |
|  | TxRU reduction 24TxRU | Cat 2, Set 2 | Low load(RU=5%) | 4.8% | 2.03% UPT loss | Baseline: 32TxRU | FTP3: 20K packet size,η=1 |
|  | TxRU reduction 16TxRU |  | Low load(RU=5%) | 9.6% | 5.61% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | Low load(RU=5%) | 14.8% | 12.5% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | light load(RU=11%) | 8.0% | 3.07% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=11%) | 15.9% | 9.75% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | light load(RU=11%) | 25.3% | 19.36% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | light load(RU=20%) | 9.6% | 5.19% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=20%) | 19.7% | 12.87% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | light load(RU=20%) | 32.1% | 23.931% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | Low load(RU=5%) | 7.9% | 0.42% UPT loss |  | FTP3: 4K packet size, η=1 |
|  | TxRU reduction 16TxRU |  | Low load(RU=5%) | 15.8% | 1.72% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | Low load(RU=5%) | 24.3% | 3.54% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | light load(RU=13%) | 11.2% | 0.67% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=13%) | 22.7% | 1.5% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | light load(RU=13%) | 35.0% | 3.84% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | light load(RU=28%) | 14.0% | 1.86% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=28%) | 27.8% | 6.16% UPT loss |  |  |
|  | TxRU reduction 8TxRU |  | light load(RU=28%) | 43.4% | 14.15% UPT loss |  |  |
|  | TxRU reduction 24TxRU |  | light load(RU=48%) | 14.3% | 5.07% UPT loss |  |  |
|  | TxRU reduction 16TxRU |  | light load(RU=48%) | 29.4% | 14.63% UPT loss |  |  |
|  | Dynamic TxRUs adaptation via multi-CSI |  | Low load(RU=5%) | 18.1% | 0.62% UPT loss |  |  |
|  |  |  | light load(RU=13%) | 23.7% | 0.16% UPT loss |  |  |
|  |  |  | light load(RU=28%) | 19.4% | 0.74% UPT loss |  |  |
|  |  |  | light load(RU=48%) | 13.7% | 1.01% UPT loss |  |  |
| Vivo [R1-2211018 , R1-2212541] | Dynamic antenna port adaptation (antenna ports are dynamically adapted (between 64 ports and 8 ports) according to the cell traffic load, in every slot) | Cat 1, Set 1 | 12.38% | 9.4% | UPT loss:0.36%; latency increase:-0.08%; UE power increase:-0.02% | Baseline: antenna ports are always 64 | SLS; No UE DRX; FTP3 traffic model,A=0.4, η=1 |
|  |  |  | 12.57% | 9.4% | UPT loss:1.98%; latency increase:-2.20%; UE power increase:-0.04% |  |  |
|  |  |  | 15.31% | 6.8% | UPT loss:12.26%; latency increase:-14.20%; UE power increase:-1.35% |  |  |
|  |  | Cat 2, Set 1 | 12.52% | 8.1% | UPT loss:2.12%; latency increase:-2.35%; UE power increase:-0.17% |  |  |
|  |  |  | 13.16% | 8.1% | UPT loss:6.48%; latency increase:-8.25%; UE power increase:-0.45% |  |  |
|  |  |  | 16.42% | 6.7% | UPT loss:18.50%; latency increase:-38.22%; UE power increase:-1.96% |  |  |
| NOKIA/NSB [R1-2211097] | Reduced number of TX to 32 | Cat 2, Set 1 | Low | 27.6% | UPT: 163,26 Mbps | Single cell operation as per SET1 (64 TRX). UEs are initially in RRC_CONNECTED state | SLS+Post-processing; FTP3 traffic model; A=0,4; Single value η (=1) |
|  |  |  | Light | 28.5% | UPT: 117,64 Mbps |  |  |
|  |  |  | Medium | 29.5% | UPT: 75,47 Mbps |  |  |
| Intel [R1-2211410] | Antenna port adaptation | Cat 1, Set 1 | Low | 19.1% | UPT Baseline: 819.7Mbps UPT ES: 731.1Mbps; Avg EE (baseline): 5.11 Avg EE (ES): 5.46 | Baseline: 64Tx (fixed) ES: 32Tx (fixed) | SLS No C-DRX used for UEs; CSI feedback based on SRS; FTP3 traffic model; A = 0.4; η(s_f,s_p )=1 for any sf, sp; |
|  |  |  |  | 27.3% | UPT Baseline: 819.7Mbps UPT ES: 585.5Mbps; Avg EE (baseline): 5.11 Avg EE (ES): 4.81 | Baseline: 64Tx (fixed) ES: 16Tx (fixed) |  |
|  |  |  |  | 4.5% | UPT Baseline: 819.7Mbps UPT ES: 801.8Mbps; Avg EE (baseline): 5.11 Avg EE (ES): 5.07 | Baseline: 64Tx (fixed) ES: variable |  |
|  |  |  | Light | 25.7% | UPT Baseline: 611.5Mbps UPT ES: 539.8Mbps; Avg EE (baseline): 2.67 Avg EE (ES): 3.11 | Baseline: 64Tx (fixed) ES: 32Tx (fixed) |  |
|  |  |  |  | 35.7% | UPT Baseline: 611.5Mbps UPT ES: 400.3Mbps; Avg EE (baseline): 2.67 Avg EE (ES): 2.73 | Baseline: 64Tx (fixed) ES: 16Tx (fixed) |  |
|  |  |  |  | 1.9% | UPT Baseline: 611.5Mbps UPT ES: 606.7Mbps; Avg EE (baseline): 2.67 Avg EE (ES): 2.71 | Baseline: 64Tx (fixed) ES: variable |  |
|  |  |  | Medium | 29.6% | UPT Baseline: 457.9Mbps UPT ES: 389.3Mbps; Avg EE (baseline): 1.5 Avg EE (ES): 1.84 | Baseline: 64Tx (fixed) ES: 32Tx (fixed) |  |
|  |  |  |  | 41.8% | UPT Baseline: 457.9Mbps UPT ES: 243.9Mbps; Avg EE (baseline): 1.5 Avg EE (ES): 1.67 | Baseline: 64Tx (fixed) ES: 16Tx (fixed) |  |
|  |  |  |  | 0.0% | UPT Baseline: 457.9Mbps UPT ES: 457.8Mbps; Avg EE (baseline): 1.5 Avg EE (ES): 1.50 | Baseline: 64Tx (fixed) ES: variable |  |
| CATT [R1-2211210] | Dynamic adaptation of spatial elements | Cat 1, Set 1 | Low load | 6.8% | UPT loss:0.32% | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);SSB periodicity 20ms;CSI-RS/TRS 10ms;TxRU= 64. | SLS; (cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms;CSI-RS/TRS 10ms;dynamic spatial antenna adaptation: gNB dynamic adaptation of the number of TxRU from 64TxRU to 32 TxRU; FTP3 traffic model; A=0.4; η(s_f, s_p)=1. |
|  |  |  | Light load | 12.2% | UPT loss:0.62% |  |  |
|  |  |  | Medium load | 18.0% | UPT loss:3.8% |  |  |
|  |  |  | Low load | 6.9% | UPT loss:1.5% | SLS; SSB periodicity 20ms;CSI-RS/TRS 10ms;TxRU= 64. | SLS; No DRX; SSB periodicity 20ms;CSI-RS/TRS 10ms;dynamic spatial antenna adaptation: gNB dynamic adaptation of the number of TxRU from 64TxRU to 32 TxRU; A=0.4; η(s_f, s_p)=1. |
|  |  |  | Light load | 12.3% | UPT loss:1.6% |  |  |
|  |  |  | Medium load | 19.6% | UPT loss:1.8% |  |  |
| Fujitsu [R1-2211085] | Dynamic TxRU adaptation | Cat 2, Set 1 | low | 24.1% | 5.7% average UPT loss | BS #TxRU=64 | CSI feedback period = 20ms, feedback delay = 4ms, immediate antenna adaptation delay gNB dynamically turns out half of the TxRUs if the DL data in the buffer is expected to be transmitted in the next slot; FTP3 traffic model; A=0.4; Single value η (=1) |
|  |  |  | light | 18.6% | 4.1% average UPT loss |  |  |
|  |  |  | medium | 12.0% | 2.3% average UPT loss |  |  |
|  |  | Cat 2, Set 2 | low | 26.5% | 0.6% average UPT loss | BS #TxRU=32 |  |
|  |  |  | light | 20.0% | 0.5% average UPT loss |  |  |
|  |  |  | medium | 12.8% | 0.3% average UPT loss |  |  |
| Ericsson [R1-2212154] | BS #TxRU 32 | Cat1, Set 1 | Low | 15.0% | UPT loss of 1% for 95-%, UPT loss of 2% for 50-% UPT loss of 8% for 5-% | BS #TxRU 64 | 1 SSB Single value η (=1) FTP3 traffic model |
|  |  |  | Light | 21.2% | UPT loss of 3% for 95-%, UPT loss of 6% for 50-% UPT loss of 12% for 5-% |  |  |
|  |  |  | Medium | 22.4% | UPT loss of 3% for 95-%, UPT loss of 14% for 50-% UPT loss of 22% for 5-% |  |  |
|  | BS #TxRU 16 |  | Low | 21.4% | UPT loss of 3% for 95-%, UPT loss of 5% for 50-% UPT loss of 14% for 5-% |  |  |
|  |  |  | Light | 31.6% | UPT loss of 6% for 95-%, UPT loss of 15% for 50-% UPT loss of 44% for 5-% |  |  |
|  |  |  | Medium | 36.6% | UPT loss of 8% for 95-%, UPT loss of 25% for 50-% UPT loss of 33% for 5-% |  |  |
| Qualcomm [R1-2212128] | #TxRU reduction (64 to 32) | Cat 1, Set 1 | Low | 29.4% | UPT loss at 50%tile: 31%; DL SINR loss at 5% tile: 4.5dB | BS #TxRU 64 | FTP3 traffic model |
|  |  |  | Light | 28.6% | UPT loss at 50%tile: 30%; DL SINR loss at 5% tile: 6.5dB |  |  |
| Samsung [R1-2212056] | #TxRU_32 | Cat 1, Set 1 | Medium | 46.8% | UPT loss: 12.97%; latency increase: 16.69%; Reference traffic: 27.87 % RU Reduced traffic: 32.28 % RU | BS #TxRU 64 | Port adaptation from 64 to 32 TxRU FTP3 traffic model |
|  |  |  | Light | 47.8% | UPT loss: 14.70%; latency increase: 16.84%; Reference traffic: 14.21 % RU Reduced traffic: 16.94 % RU |  |  |
|  |  |  | Low | 49.4% | UPT loss: 19.19%; latency increase: 17.46%; Reference traffic: 3.48 % RU Reduced traffic: 4.05 % RU |  |  |
|  |  |  | Low | 49.4% | UPT loss: 25.12%; latency increase: 16.66%; Reference traffic: 1.29 % RU Reduced traffic: 1.56 % RU |  |  |

Based on the results, it is observed that BS energy savings can be achieved by 3.4%~49.4% at all loads for different sets of reference configurations with FTP3 for FTP3 IM traffic models, with or without UE C-DRX configuration, compared to no dynamic antenna adaption. The gain depends on whether there is multiple CSI report assistance, and the number of antenna ports that can be adapted. More number of elements are reduced, more gain can be generally obtained. One result reports that when dynamic antenna adaptation is variably changed, the gain is reduced to a range of 0~4.5%. One result reports that additional gain can be obtained when this scheme is combined with PDSCH power offset.

When there is no multiple CSI report assistance, the BS energy saving gain comes at the expense of UPT loss which can be up to 87.08%. When there is multiple CSI report assistance, the gain remains in the range of 3.4%~31.3% with UPT loss mitigated/reduced to less than 10% or no UPT loss.

On latency, there is negative impact observed in three results and the increment becomes larger as the number of reduced antenna ports becomes larger. There is also one result showing that the latency can be slightly reduced.

On UE power consumption, one result shows that there is increase and one result shows there can be slight UE power savings.

Additionally, one result shows that the average EE can be generally increased except for the low load case where number of antenna is reduced from 64 to 16, and the case of antenna number variably changing.

=== end ===

| Company | Comments |
| --- | --- |

6.3.1.3	Specification impacts

6.3.2	Technique C-2 ZZZ

6.3.2.1	Description of technique

6.3.2.2	Analysis of performance and impacts

##### === draftTP C-2: start ===

The following capture the results for TRP muting in multi-TRP operation.

Table 6.1.1-x: BS energy savings by TRP muting in multi-TRP operation

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | UPT/latency/UE power consumption/ Other KPIs | Baseline configuration/assumption | Evaluation methodology/assumption details |
| --- | --- | --- | --- | --- | --- | --- | --- |
| NOKIA/NSB [R1-2211097] | Reduced number of TRPs | Cat 2, Set 1 | Low | 38.8% | UPT loss: -14.49% | 2 TRPs are assumed. UEs are initially in RRC_CONNECTED state. | SLS+Post-processing, FTP3 traffic model; A=0,4; Single value η (=1). 70% of the P_Static among TRPs |
|  |  |  | Light | 37.2% | UPT loss: -14.14% |  |  |
|  |  |  | Medium | 36.9% | UPT loss: -7.27% |  |  |
| CATT [R1-2211210] | TRP muting/adaptation in multi-TRP operation | Cat 1, Set 1 | Low load | 28.4% | UE power: 50.6; UE ESG:12.7% | M-TRP configuration: One cell is configured with 2TRPs; Both of TRP are activated. | SLS; (DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms); SSB periodicity 20ms; CSI-RS/TRS 10ms; TRP OFF: 160ms SSB/CSI-RS transmission. When TRP is activated, additional CSI-RS/TRS is transmitted before data scheduling. FTP3 traffic model; A=0.4; η(s_f, s_p)=1. |
|  |  |  | Light load | 28.7% | UE power: 50.6; UE ESG:12.7% |  |  |
|  |  |  | Medium load | 19.7% | UE power: 51.6; UE ESG:12.4% |  |  |
| Qualcomm [R1-2212128] | TRP reduction (2 to 1) | Cat 1, Set 1 | Low | 41.6% | UPT loss at 50%-tile: 16% | 2 TRPs, each with 64 TxRUs | FTP3 traffic model |
|  |  |  | Light | 39.0% | UPT loss at 50%-tile: 22% |  |  |

Based on the results, BS energy savings can be achieved by 19.7%~41.6% for two TRPs configuration at different loads, at the expense of UPT loss.

For the BS energy saving gain around 36.9%~41.6%, UPT loss is observed by 7.27%~22%.

For the BS energy saving gain around 19.7%~28.7%, it is also observed from one result (CATT) that UE power savings can be achieved by about 12%.

=== end ===

| Company | Comments |
| --- | --- |

6.3.2.3	Specification impacts

6.3.z	Impacts on network interfaces

6.4	Techniques in power domain

6.4.1	Technique D-1 WW

6.4.1.1	Description of technique

6.4.1.2	Analysis of performance and impacts

##### === draftTP D-1: start ===

The following capture the results for adaptation of transmission power of signals and channels.

Table 6.1.1-x: BS energy savings by dynamic transmission power adaptation

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | Baseline configuration/assumption | Other KPI |
| --- | --- | --- | --- | --- | --- | --- |
| MTK [R1-2212259] | PDSCH_PowOffset_-3dB | Cat 1 | Light | 8.7% | Baseline：PDSCH power offset 0 dB ES scheme：PDSCH power offset -3/-6/-9 dB reference configuration：Set1 FTP3，DRX (160, 8, 100)，Single value η (=1) | UPT:2.03%,UE power comsumption:1.80%,ltency:2.07% |
|  | PDSCH_PowOffset_-6dB |  |  | 11.1% |  | UPT:5.66%,UE power comsumption:4.47%,latency:6.00% |
|  | PDSCH_PowOffset_-9dB |  |  | 9.0% |  | UPT:10.80%,UE power comsumption:9.17%,latency:12.11% |
|  | PDSCH_PowOffset_-3dB |  | Medium | 13.9% |  | UPT:3.28%,UE power comsumption:2.46%,latency:3.39% |
|  | PDSCH_PowOffset_-6dB |  |  | 18.7% |  | UPT:8.97%,UE power comsumption:6.80%,latency:9.85% |
|  | PDSCH_PowOffset_-9dB |  |  | 17.7% |  | UPT:19.49%,UE power comsumption:14.78%,latency:24.21% |
|  | PDSCH_PowOffset_-3dB | Cat 2 | Light | 8.7% |  | UPT:2.03%,UE power comsumption:1.80%,latency:2.07% |
|  | PDSCH_PowOffset_-6dB |  |  | 11.8% |  | UPT:5.66%,UE power comsumption:4.47%,.latency:6.00% |
|  | PDSCH_PowOffset_-9dB |  |  | 11.2% |  | UPT:10.80%,UE power comsumption:9.17%,latency:12.11% |
|  | PDSCH_PowOffset_-3dB |  | Medium | 14.7% |  | UPT:3.28%,UE power comsumption:2.46%,latency:3.39% |
|  | PDSCH_PowOffset_-6dB |  |  | 20.6% |  | UPT:8.97%,UE power comsumption:6.80%,latency:9.85% |
|  | PDSCH_PowOffset_-9dB |  |  | 21.0% |  | UPT:19.49%,UE power comsumption:14.78%,latency:24.21% |
|  | #TxRU_32_PDSCH_PowOffset_-3dB | Cat 1 | Light | 18.8% | Baseline：PDSCH power offset 0 dB,BS #TxRU 64 ES scheme：PDSCH power offset -3 dB, BS #TxRU 32 reference configuration：Set1 FTP3，DRX (160, 8, 100)，Single value η (=1) | UPT:9.06%,UE power comsumption:7.62%,latency:9.96% |
|  |  | Cat 2 |  | 19.7% |  | UPT:9.06%,UE power comsumption:7.62%,latency:9.96% |
| Huawei，HiSilicon [R1-2210858] | Dynamic Power back-off with Multiple CSIs | Cat 2 | 30% load(medium) | 5.3% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set1 FTP3 IM, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) | UPT: 0% loss |
|  |  |  |  | 7.3% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set1 VoIP, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
|  |  |  |  | 6.9% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set2 VoIP, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
|  |  |  |  | 11.5% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set2 VoIP, 4T for Set 2,NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
|  |  |  |  | 5.3% | baseline: Dynamic Power back-off with Single CSI ES scheme: Dynamic Power back-off with Multiple CSIs reference configuration: Set2 FTP3 IM, 4T for Set 2, NO C-DRX; Subband based CSI-feedback in every 5 slots, slot level with time-domain scaling; A=0.4; η=1, 0.76(s_f*s_p<0.5) |  |
| ZTE,Sanechips [R1-2211903] | PDSCH PSD reduction 53.75dBm | Cat 2 | Low load(RU=10%) | 2.3% | Baseline: 55dBm reference configuration:Set 1: FR1 TDD, FTP3: 20K packet size, slot-level,Pstatic=P3, η=1 | 0.56% UPT loss |
|  |  |  | light load(RU=20%) | 4.4% |  | 1.82% UPT loss |
|  |  |  | Low load(RU=31%) | 6.0% |  | 3.8% UPT loss |
|  | PDSCH PSD reduction 52dBm |  | Low load(RU=10%) | 6.4% |  | 1.26% UPT loss |
|  |  |  | light load(RU=20%) | 10.1% |  | 1.83% UPT loss |
|  |  |  | Low load(RU=31%) | 12.9% |  | 2.38% UPT loss |
|  |  |  | light load(RU=4.7%) | 3.9% | Baseline: 55dBm reference configuration:Set 1: FR1 TDD, FTP3: 0.5M packet size, slot-level,Pstatic=P3, η=1 | 5.74% UPT loss |
|  |  |  | light load(RU=9.6%) | 7.0% |  | 4.32% UPT loss |
|  |  |  | light load(RU=23.5%) | 12.2% |  | 5.48% UPT loss |
|  |  |  | light load(RU=38.4%) | 16.6% |  | 9.81% UPT loss |
|  | PDSCH PSD reduction 47.75dBm |  | Low load(RU=13%) | 5.9% | Baseline: 49dBm reference configuration:Set 2: FR1 FDD, FTP3: 0.1M packet size, slot-level,Pstatic=P3, η=1 | 0.64% UPT loss |
|  |  |  | light load(RU=29%) | 8.6% |  | 0.05% UPT loss |
|  | PDSCH PSD reduction 46dBm |  | Low load(RU=13%) | 11.8% |  | 1.56% UPT loss |
|  |  |  | light load(RU=29%) | 17.0% |  | 0.75% UPT loss |
|  | Dynamic PDSCH PSD adaptation via multi-CSI |  | Low load(RU=10%) | 12.1% | Baseline: 55dBm reference configuration:Set 1: FR1 TDD, FTP3: 20K packet size, slot-level,Pstatic=P3, η=1 | 0.38% UPT loss |
|  |  |  | light load(RU=20%) | 16.6% |  | 0.35% UPT loss |
|  |  |  | Low load(RU=31%) | 23.8% |  | 1.17% UPT loss |
|  |  |  | Low load(RU=31%) | 16.4% |  | 0.27% UPT loss |
| NOKIA/NSB [R1-2211097] | Reduced DL transmit power by 3dB | Cat 2 | Low | 9.8% | Baseline: MaximumTx power of 49 dBm.UEs are initially in RRC_CONNECTED state reference configuration:Set 2, DL-FTP3, A=0,4; Single value η (=1) SLS | UPT:144 Mbps |
|  | Reduced DL transmit power by 6dB |  |  | 12.2% |  | UPT:134 Mbps |
|  | Reduced DL transmit power by 9dB |  |  | 13.4% |  | UPT:119 Mbps |
|  | Reduced DL transmit power by 3dB |  | Light | 15.4% |  | UPT:104 Mbps |
|  | Reduced DL transmit power by 6dB |  |  | 19.7% |  | UPT:97 Mbps |
|  | Reduced DL transmit power by 9dB |  |  | 17.9% |  | UPT:88 Mbps |
|  | Reduced DL transmit power by 3dB |  | Medium | 16.2% |  | UPT:76 Mbps |
|  | Reduced DL transmit power by 6dB |  |  | 23.4% |  | UPT:70 Mbps |
|  | Reduced DL transmit power by 9dB |  |  | 24.7% |  | UPT:63 Mbps |
| DCM [R1-2211994] | PDSCH_PowOffset_-6dB | Cat 1 | Light | 6.6% | Baseline:PDSCH power offset 0 dB reference configuration:Set 1,FTP3, A=0.4, Single value η (=1) | UPT:-1.00% |
|  | PDSCH_PowOffset_-12dB |  |  | 8.3% |  | UPT:-2.60% |
|  | PDSCH_PowOffset_-18dB |  |  | 8.7% |  | UPT:-5.70% |
|  | PDSCH_PowOffset_-6dB |  | Medium | 15.5% |  | UPT:-2.50% |
|  | PDSCH_PowOffset_-12dB |  |  | 19.5% |  | UPT:-2.00% |
|  | PDSCH_PowOffset_-18dB |  |  | 20.4% |  | UPT:-5.80% |
|  | PDSCH_PowOffset_-6dB |  | High | 24.1% |  | UPT:3.10% |
|  | PDSCH_PowOffset_-12dB |  |  | 29.9% |  | UPT:0.70% |
|  | PDSCH_PowOffset_-18dB |  |  | 31.4% |  | UPT:0% |
| Intel [R1-2211410] | Transmit Power Adaptation/ -12dB power | Cat1 | Low | 19.2% | Baseline: Full power SLS; No C-DRX used for UEs; CSI feedback based on SRS; SIB1 BW: 48 PRB; No paging overhead; 1 SSB beam; SSB/PRACH periodicity: 20msec; SIB1 periodicity: 40msec; Slot level model For scaling: A = 0.4; η(s_f,s_p )=1 for any sf, sp; | Baseline: 819.7 Mbps ES: 746 Mbps Avg EE (Baseline): 5.10 Avg EE (ES) : 5.43 |
|  |  |  | Light | 28.2% |  | Baseline: 611.5 Mbps ES: 567.5 Mbps Avg EE (Baseline): 2.66 Avg EE (ES) : 3.25 |
|  |  |  | Medium | 34.3% |  | Baseline: 457.9 Mbps ES: 415.1 Mbps Avg EE (Baseline): 1.5 Avg EE (ES) : 2.03 |
|  | Transmit Power Adaptation/ -6dB power |  | Low | 17.6% |  | Baseline: 819.7 Mbps ES: 798.5 Mbps Avg EE (Baseline): 5.10 Avg EE (ES) : 5.83 |
|  |  |  | Light | 25.4% |  | Baseline: 611.5 Mbps ES: 604.8 Mbps Avg EE (Baseline): 2.66 Avg EE (ES) : 3.41 |
|  |  |  | Medium | 30.0% |  | Baseline: 457.9 Mbps ES: 450.7 Mbps Avg EE (Baseline): 1.5 Avg EE (ES) : 2.06 |
| CATT [R1-2211210] | Adaptation of transmission power of signals and channels | Cat 1 | Low load | 3.9% | Baseline:SSB periodicity 20ms;CSI-RS/TRS 10ms;Transmission power:55dBm; reference configuration:Set 1, FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes; A=0.4; η(s_f, s_p)=1. | UPTloss:1.6% |
|  |  |  | Light load | 7.7% |  | UPTloss:1.8% |
|  |  |  | Medium load | 9.1% |  | UPTloss:2.8% |
|  |  |  | Low load | 4.1% | Baseline: SSB periodicity 20ms;CSI-RS/TRS 10ms;(DRX-cycle, on duration timer, inactivity timer) = (160ms, 8ms, 100ms);Power domain adaptation; reference configuration:Set 1, FTP3, inter-arrival time = 200ms, packet size = 0.5Mbytes; A=0.4; η(s_f, s_p)=1. | UPTloss:1.9% |
|  |  |  | Light load | 7.7% |  | UPTloss:3.9% |
|  |  |  | Medium load | 11.2% |  | UPTloss:3.1% |
| Ericsson [R1-2212154] | Tx power adaptation (reduction up to 12 dB) | Cat1 | Low | 20.9% | Baseline: BS Tx power 55 dBm reference configuration:Set 1,FTP3 1 SSB Single value η (=1) | UPT loss of 1% for 95-% UE, UPT loss of 3% for 50-% UE UPT loss of 22% for 5-% UE |
|  |  |  | Light | 40.5% |  | UPT loss of 1% for 95-% UE, UPT loss of 8% for 50-% UE UPT loss of 36% for 5-% UE |
|  |  |  | Medium | 47.6% |  | UPT loss of 0% for 95-% UE, UPT loss of 9% for 50-% UE UPT loss of 13% for 5-% UE |
|  | Tx power adaptation (reduction up to 6 dB) |  | Low | 17.7% |  | UPT loss of 1% for 95-% UE, UPT loss of 1% for 50-% UE UPT loss of 2% for 5-% UE |
|  |  |  | Light | 33.0% |  | UPT loss of 0% for 95-% UE, UPT loss of 4% for 50-% UE UPT loss of 7% for 5-% UE |
|  |  |  | Medium | 38.5% |  | UPT loss of 2% for 95-% UE, UPT loss of 9% for 50-% UE UPT loss of 7% for 5-% UE |
| Samsung [R1-2212056] | Transmission power adaptation | Cat 1 | Reference traffic: 7.5 % RU Reduced traffic: 4.4 % RU | 51.5% | Baseline: BS Tx power 55 dBm reference configuration:Set 1,FTP3 a static part of power for BS: P_3 A: 0.4 For eta, If two values of η(s_f,s_p ) are used for evaluation,η(s_f,s_p ) = 0.76 if s_f*s_p <0.5; otherwise, η(s_f,s_p )=1. 46.5 and 5.2 relative power per a SSB for Cat 1 and Cat 2 | UPT:10.40%, Packet latency: 24.7% Scheduling latency: No increase |
|  |  | Cat 2 |  | 17.5% |  | UPT:10.40%, Packet latency: 24.7% Scheduling latency: No increase |
| Qualcomm [R1-2212128] | Tx power reduction (55dBm to 52dBm) | Cat 1 | Low | 13.1% | Baseline: BS BS #TxRU 64 with 55dBm Tx power reference configuration:Set 1,FTP3 | UPT lsss at 50%-tile: 10%, DL SINR loss at 5% tile: 4dB |
|  |  |  | Light | 6.6% |  | UPT loss at 50%-tile: 16%, DL SINR loss at 5% tile: 3dB |

Based on the results, dynamic transmission power reduction on PDSCH can achieve BS energy savings gain at all load cases for both Set 1 FR1 TDD and Set 2 FR1 FDD including 4Tx BS antenna configuration, for both BS categories with FTP3 or FTP3 IM model, with or without UE C-DRX configuration. The presented gain varies in the range of 2.3%~51.5%, depending on the exact traffic load, power reduction level (power offset w.r.t. PDSCH) and CSI feedback assistance. The gain generally increases as the traffic load increases except that one result (Qualcomm) show reduced energy savings when the traffic load is from low to light. The gain can also increase when transmission power reduction level becomes higher, but the increment may become slower.

Compared to the baseline of no power reduction, 0.5%~19.49% UPT loss is observed, while with power reduction level less than 9dB, UPT loss is less than 10%. With multi-CSI assistance, the UPT loss can be further mitigated to less than 1% or no loss. One result (Samsung) reports no impact on scheduling latency while one result (MediaTek) reports increased latency by less than 10% in most cases.

On UE power consumption, one result (MediaTek) shows less than 10% increment is observed in most cases.

One result (Intel) shows this scheme can increase the average EE and one result (Qualcomm) shows negative impact by 3 dB on DL SINR.

=== end ===

| Company | Comments |
| --- | --- |

6.4.1.3	Specification impacts

6.4.2	Technique D-2 WWW

6.4.2.1	Description of technique

6.4.2.2	Analysis of performance and impacts

##### === draftTP D-3: start ===

The following capture the results by Channel Aware Tone reservation.

Table 6.1.1-x: BS energy savings by Channel Aware Tone reservation

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | Baseline configuration/assumption | Other KPI | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm [R1-2212129] | PA Backoff reduction of 1-3dB due to PAPR reduction from Channel Aware Tone reservation | Cat 1 |  | 9.5% | Set 3 | UPT:0.00% latency: 0% UE power consumption: 0% | Evaluation showing utilization of PAPR reduction, where the PAPR reduction is used to reduce the backoff PA attribute (Pmax) while maintaining the TX power and signal EVM The Backoff also compensates for the tones used for the TR signal, thus no UPT loss occurs Comparing Channel Aware Tone Reservation to Transparent Tone Reservation |
|  |  |  |  | 2.1% |  |  |  |
|  |  |  |  | 4.4% | Set 1 |  |  |
|  |  |  |  | 2.1% |  |  |  |

It is observed that channel aware tone reservation can achieve PA back-off reduction of 1-3 dB which leads to 2.1%~9.5% BS energy saving gains depending on configurations, compared with transparent tone reservation.

On UPT/latency, no negative impact is observed.

No impact on UE power consumption.

=== end ===

| Company | Comments |
| --- | --- |

6.4.2.3	Specification impacts

6.4.3	Technique D-5 adaptation of transceiver processing algorithm

6.4.3.1	Description of technique

6.4.3.2	Analysis of performance and impacts

##### === draftTP D-5: start ===

The following capture the results for UE post-distortion.

Table 6.1.1-x: BS energy savings by [UE post-distortion]

| Company | ES scheme | BS Category | Load scenario | ES gain (%) | Baseline configuration/assumption | Other KPI | Note |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Qualcomm [R1-2212129] | UE post-distortion | Cat 1 |  | 16.1% | Set 3 | UPT:0.00% latency: 0% | Evaluation showing utilization of PAPR reduction, where the PAPR reduction is used to reduce the backoff PA attribute (Pmax) while maintaining the TX power and signal EVM The Backoff also compensates for the tones used for the TR signal, thus no UPT loss occurs Processing and power consumption of the UE depends on the UE receiver’s design for DPoD. |

It is observed that UE post-distortion can achieve BS energy saving by 16.1% for Set 3 reference configuration.

On UPT or latency, there is no negative impact observed.

The impact on UE power consumption depends on UE receiver’s design for DPoD.

=== end ===

| Company | Comments |
| --- | --- |

6.4.2.3	Specification impacts

6.4.z	Impacts on network interfaces

6.5	Other energy saving aspects and techniques

Editor's note: placeholder.

6.x	Higher layer aspects for network energy savings

Editor's note: This section includes common aspects of higher layers deduced from the above candidate directions.

7	Conclusions

##### === draftTPX: start ===

TBD

=== end ===

| Company | Comments |
| --- | --- |