3GPP TSG-RAN WG1 Meeting #111	     R1-2210889

Toulouse, France, November 14 – 18, 2022

Agenda Item:	9.2.4.1

Source:	Huawei, HiSilicon

Title:	Evaluation on AI/ML for positioning accuracy enhancement

Document for:	Discussion and Decision

Introduction

In the RAN1 #110 meeting [1], it was agreed that both direct AI/ML positioning and AI/ML assisted positioning are to be studied and evaluated by RAN1:

Agreement For AI/ML-based positioning, both approaches below are studied and evaluated by RAN1: - Direct AI/ML positioning - AI/ML assisted positioning

In our companion contribution [2], we have proposed the following two specific techniques belonging to the two types of agreed sub use cases, respectively:

- Direct AI/ML positioning

  - AI/ML-based fingerprint positioning

- AI/ML assisted positioning

  - AI/ML-based LOS/NLOS Identification

In the remainder of this contribution, the above sub use cases are discussed and their evaluation methodology, KPIs, and also evaluation results are presented.

Evaluation methodology of the agreed use cases

Direct AI/ML positioning

Generic fingerprint positioning mechanism

Traditional positioning algorithms such as TDoA and AoA are based on LOS channels, and are no longer applicable in environments where NLOS paths dominate. In these scenarios, the number of gNBs that have LOS channels with the UE is relatively small. As a result, the precision of the traditional positioning algorithm cannot meet the requirements of high-accuracy positioning applications. At the same time, existing research shows that, based on a large amount of channel data, a mapping relationship between channel features and location coordinates can be established by using an AI/ML method. This method, namely AI/ML-based fingerprint positioning, can achieve reliable accuracy under heavy NLOS conditions, where the positioning accuracy of traditional methods may be > 10m@90%.

Figure 1 gives an overview about the AI/ML-based fingerprint positioning process. It exploits that each UE position can be associated with a unique channel characteristic (i.e. the fingerprint). The AI/ML model can learn this relationship for a given environment and then use it to determine the UE coordinates based on the measured channel characteristics.

Figure 1 AI/ML-based fingerprint positioning process

Baseline for comparison

As agreed in the RAN1 #109-e meeting [3]:

Agreement For AI/ML-based positioning evaluation, the baseline performance to compare against is that of existing Rel-16/Rel-17 positioning methods. - As a starting point, each participating company report the specific existing positioning method (e.g., DL-TDOA, Multi-RTT) used as comparison.

For the baseline for performance evaluations of AI/ML-based fingerprint positioning in NLOS dominated scenarios, we adopt the positioning accuracy achieved by the traditional UL-TDoA positioning method without LOS detection in Rel-17 as proposed in [4].

In the baseline method, the NLOS paths are not removed. In NLOS dominated scenarios, there are hardly at least three LOS paths available at the same time for calculation of the position. This will result in the issue that stronger NLOS paths are mistaken as LOS paths, which leads to poor accuracy. Even if NLOS paths would be identified and removed, then, since the evaluation is under heavy NLOS conditions, there may not be enough number of LOS paths available in most cases. Therefore, under heavy NLOS conditions, the positioning accuracy of the baseline is expected to be significantly low. Based on above discussion, the following proposal is made:

- : For evaluation on AI/ML-based fingerprint positioning, support UL-TDoA positioning as a candidate baseline of the legacy non-AI/ML method.

As shown in Table 1 under both heavy (with clutter parameters of {60%, 6m, 2m}) and moderate (with clutter parameters of {40%, 2m, 2m}) NLOS conditions with 4 receiving antennas, the positioning accuracy error @90% of the baseline UL-TDoA solution in Rel-17 is more than 10 m.

Table 1. Performance of UL-TDoA fingerprint positioning

| BS receiving antennas | Positioning | Positioning Accuracy @90% |
| --- | --- | --- |
| Clutter parameters: 60%, 6m, 2m |  |  |
| 4 | UL-TDoA in Rel-17 without LOS detection | > 10 m |
| Clutter parameters: 40%, 2m, 2m |  |  |
| 4 | UL-TDoA in Rel-17 without LOS detection | > 10 m |

AI/ML-assisted positioning

Generic AI/ML-based LOS/NLOS identification mechanism

By tagging the measurements with LOS/NLOS indicators, the LMF would obtain additional information that can be exploited to improve the positioning accuracy, for example when utilizing the link with a higher LOS probability. NLOS identification has benefits from various aspects as described in [4]:

NLOS detection is an important method to improve the positioning accuracy. By tagging the measurements with LOS/NLOS indicators, the LMF would have the knowledge of LOS/NLOS status of the measurements. By utilizing the LOS/NLOS measurements correctly, for example utilizing the LOS measurements with higher probability, the positioning accuracy can be improved. In addition, NLOS identification has various benefits from the following aspects: - Useful for the reference device - Useful for NLOS dominate scenario - Useful for Computation complexity - Useful for calculating the location uncertainty

LOS/NLOS identification is a typical binary classification problem in the AI/ML field and AI/ML models are well suited for extracting different channel characteristics of the LOS or NLOS paths. The Rel-17 mechanisms that have been established for LOS/NLOS identification can therefore be improved significantly with help of AI/ML-based techniques, and especially as shown in this contribution, for a small number of antennas. Based on above discussion, the following proposal is made:

- : For AI/ML assisted positioning, AI/ML-based LOS/NLOS identification should be evaluated for positioning accuracy enhancements at least under slight/moderate NLOS scenarios in Rel-18.

Figure 2 shows the TDoA positioning process based on AI/ML-based LOS/NLOS identification. The AI/ML-based LOS/NLOS identification is utilized to remove the NLOS paths from the TOA calculation. It uses the channel’s power delay profile (PDP) as the input and calculates a LOS probability. We use a neural network with a convolutional architecture to learn this relationship. This achieves a much better prediction accuracy than traditional methods, especially when the number of antennas is small, as will be observed from our evaluations results in Section 3.2.

Figure 2 Positioning process based on LOS/NLOS identification

The whole processing flow is illustrated in Figure 3 below. After the channel estimation procedure based on the reference signal, the frequency-domain channel is transformed with an IFFT into the time domain. The amplitude of the time-domain signal is then squared to obtain the PDPs which are then normalized on all antennas on the receiver side. The normalized PDPs are used as the input to the AI/ML model in which the LOS probability is inferred. Afterwards the NLOS components are removed and the LOS components are utilized to calculate the coordinates. The TDoA algorithm is then performed at the LMF which requires the identified LOS links from at least three gNBs.

Figure 3 Pre-processing and positioning based on AI/ML-based LOS/NLOS identification in TDoA positioning

Baseline for comparison

The baseline algorithm for performance comparison with AI/ML-based LOS/NLOS identification could be considered to be aligned across companies for comparison. We select the traditional algorithm as proposed in [5]:

- LOS/NLOS identification algorithm Check the energy consistency of the first path across different antenna elements within a polarization. Check the phase consistency of the first path across different antenna elements in both vertical and horizontal direction within a polarization. If both energy and phase consistency meet the energy/phase consistency, it would be identified as a LOS path, otherwise, it would be identified as a NLOS path. Different confidence level of LOS/NLOS label may additionally be reported depending on the degree of the consistency.

Based on above discussion, the following proposal is made:

- : For evaluation of the AI/ML-based LOS/NLOS identification, the achievable positioning accuracy should be compared with the performance of an existing traditional algorithm based on Rel-17 without using AI/ML.

As shown in Table 2, when BS receiving ports is 32, the selected baseline LOS/NLOS identification solution has already achieved a good performance. But when the gNB is configured with a small number of antenna ports (which is reflected by 4 selected receiving ports in the evaluation), the baseline method provides a greatly degraded positioning accuracy error of more than 6m @90%. In contrast, the accuracy of the AI/ML-based LOS/NLOS identification solution with 4 gNB receiving ports can achieve 0.35m. The reason is that for a small number of antennas, the traditional method cannot provide enough resolution to correctly identify with a high probability on whether a path is LOS or NLOS. Therefore, for the widely deployed commercial RF modules with small antenna ports for indoor deployment, using AI/ML-based LOS/NLOS identification solution is meaningful.

Table 2. Performance of Rel-17 LOS/NLOS identification-based positioning

| BS receiving ports | LOS ID method | Positioning Accuracy @90% |
| --- | --- | --- |
| Clutter parameters: 40%, 2m, 2m |  |  |
| 32 | Baseline LOS ID | 0.484m |
| 4 | Baseline LOS ID | 6.447m |

Reporting model complexity

In the RAN1 #110bis-e meeting [6], following agreement was made for reporting the model complexity of assisted AI/ML based positioning.

Agreement For AI/ML assisted positioning, when single-TRP construction is used for the AI/ML model, companies report at least the AI/ML complexity (Model complexity, Computation complexity) for N TRPs, which are used to determine the position of a target UE. Table. Model complexity and computation complexity to support N TRPs for a target UE Model complexity to support N TRPs; Computation complexity to process N TRPs. Single-TRP, same model for N TRPs; PS When the model is at UE-side, where PS is the model complexity for the same model. FFS: if the model is at network-side; N×CS Where CS is the computation complexity of the same model for one TRP.. Single-TRP, N models for N TRPs; When the model is at UE-side, i=1,…NPS,i Where PS,i is the model complexity for the i-th AI/ML model. FFS: if the model is at network-side; i=1,…NCS,i Where CS,i is the computation complexity for the i-th AI/ML model.. Multi-TRP (i.e., one model for N TRPs); PM Where PM is the model complexity for the one model.; CM Where CM is the computation complexity for the one model..

A controversial part of the above agreement was whether it is valid only for a UE-side deployment of the AI-mode or if it also should be applicable for the network side. It led to confusion during last meeting whether the complexity for “Single-TRP, same model for N TRPs” and “Single-TRP, N models for N TRPs” should be accounted for only one entity and or all entities.

For the agreed UE-side deployment, it is clear that the complexity is only counted for one UE, and not for all UEs in the simulation environment. Otherwise, the complexity Ps for “Single-TRP, same model for N TRPs” would have to be scaled by the number of UEs in the simulation environment.

For the NW-side deployed, since the agreement is for assisted positioning, the implementation node will be the gNB. Since the complexity for the UE-side is expressed for one UE, the same should be done for a network side implementation, i.e. the model complexity imposed on one gNB shall be reported. But the text in the table, e.g. “Single-TRP, same model for N TRPs”, seems to suggest that for a NW-side the overall complexity over all models in the simulation environment should be counted, which is not meaningful and would give misleading information about the AI model complexity, when schemes at the UE-side and at the NW-side would be compared. Based on above discussion, the following observation and proposal are made:

- : The agreement for assisted positioning from last meeting on “model complexity” for the UE-side model is achieved for one UE. It is not applicable to the NW-side, since with this agreement the complexity would need to be counted over N models in N TRPs. Thus, no objective information about the complicity of one model could be obtained for a NW-side deployment.

- : The model complexity on the AI/ML assisted positioning shall be counted for one model/entity in the evaluation.

Evaluation Results

Performance evaluations of AI/ML-based fingerprint positioning

Simulation results

In the RAN1 #110bis e-meeting [6], it was agreed that the following table is adopted for reporting the evaluation results by RAN1:

Agreement For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is not evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Test; Train; test; Model complexity; Computation complexity; AI/ML.

And as agreed in the RAN1 #109-e meeting [3], the dataset is generated from the simulation platform according to the defined scenario assumptions in the Appendix A with FR1 settings. The implementation details for our evaluation on AI/ML-based fingerprint positioning are summarized in Table 3.

The model structure for our implementation is Residual-Network (ResNet) architecture. The input to the AI/ML model is the Channel Impulse Responses (CIR), and the output of model inference is the estimated UE coordinates. And the label for training is the UE coordinates. Accordingly, the pre-processing shown in Figure 1 is to obtain the CIR from the measured channel vector/matrix.

And the dataset composition for AI/ML-based fingerprint positioning is also summarized in Table 3. It should be noted that spatial consistency is required for channel modelling in the AI/ML-based fingerprint positioning. With spatial consistency, at the same drop of the simulation, two UEs with close locations will have similar channel characteristics. Spatial consistency also reflects the real channel characteristics in deployment, therefore it should be represented in channel modeling at large scale parameters, small scale parameters and absolute time of arrival for dataset generation. This is important for dataset generation used in AI/ML-based fingerprint positioning under heavy NLOS conditions. The dataset used for evaluation is generated with spatial consistency modeling based on the 2D-filtering method described in Appendix B, and modeled at channel modeling parameters including large scale parameters, small scale parameters and the absolute time of arrival.

As agreed in the RAN1 #110bis e-meeting [6], the dimension NTRP * Nport * Nt of the model input CIR should be reported:

Agreement For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples. - Note: CIR and PDP may have different dimensions. - Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples.

The training dataset consists of 25000 samples, where 1 sample denotes the CIR obtained at each of the 18 gNBs from the transmission by 1 UE, with 4 receiving ports for each gNB. And the number of time domain samples used for the evaluation is 256, so the input dimension is 18*4*256.

Also as shown in Table 3, under both heavy (with clutter parameters of {60%, 6m, 2m}) and moderate (with clutter parameters of {40%, 2m, 2m}) NLOS conditions with 4 receiving antennas, the positioning accuracy error of the AI/ML-based fingerprint positioning solution is around 0.5 m. It is shown in our results that under moderate NLOS conditions the accuracy is now around 0.1 meter worse than for heavy NLOS conditions, which results from the fact that the heavy NLOS provides more diverse channel profiles in the fingerprint pool, which is beneficial for learning.

It can be observed that already with a small receiving antenna number, which is now widely adopted in the commercial RF modules for indoor deployment, the AI/ML scheme already provides remarkable sub-meter level precision.

Table 3. Evaluation results for AI/ML model deployed on network-side, ResNet

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | Test | Model complexity | Computation complexity | AI/ML |
| CIR 18*4*256 | UE coordinates | UE coordinates | {60%, 6m, 2m}, Drop 1 | {60%, 6m, 2m}, Drop 1 | 25000 | 5000 | 34 K | 10M | 0.492 |
|  |  |  | {40%, 2m, 2m}, Drop 1 | {40%, 2m, 2m}, Drop 1 |  |  |  |  | 0.606 |

From the results shown in the table above and the previous discussion, we can make the following observations:

- : From the above evaluation results, it is observed that AI/ML-based fingerprint positioning provides reliable positioning accuracy under both heavy and moderate NLOS conditions.

- : From the above evaluation results, it is observed that AI/ML-based fingerprint positioning provides reliable positioning accuracy under a small number of receiving ports.

According to companies’ contributions from the last meeting, the CIR is a popular AI/ML model input for the fingerprint positioning case. In our view, it would be helpful for aligning and calibrating the AI/ML model implementation as well as the simulation results across companies, if the preferred solutions are similar with each other and for example have the same input. But we are also open to optionally take other inputs or to take pre-processing into consideration.

The details of the CIR to describe the time-domain channel between one gNB and one UE have not been discussed yet but it is obvious that each CIR would be associated with a certain overhead. Furthermore, it has not been evaluated yet, how many CIRs and gNBs actually are required to obtain unique fingerprint information that unambiguously can be mapped to the UE location. This is very different from the legacy approach, where gNBs should be ensured to always have a sufficient number of LOS paths.

We are therefore making the following proposal:

- : For comparison of evaluation on AI/ML-based fingerprint positioning evaluation results, support the channel impulse responses (CIRs) as the model inputs.

Generalization studies

In the RAN1 #110 meeting [1], it was agreed that the following aspects of the model generalization capability of the AI/ML based positioning are considered to be studied and evaluated by RAN1:

Agreement To investigate the model generalization capability, at least the following aspect(s) are considered for the evaluation for AI/ML based positioning: - Different drops - Training dataset from drops {A0, A1,…, AN-1}, test dataset from unseen drop(s) (i.e., different drop(s) than any in {A0, A1,…, AN-1}). Here N>=1. - Clutter parameters, e.g., training dataset from one clutter parameter (e.g., {40%, 2m, 2m}), test dataset from a different clutter parameter (e.g., {60%, 6m, 2m}); - Network synchronization error, e.g., training dataset without network synchronization error, test dataset with network synchronization error; - Other aspects are not excluded. Note: It’s up to participating companies to decide whether to evaluate one aspect at a time, or evaluate multiple aspects at the same time.

Agreement To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: - UE/gNB RX and TX timing error. - The baseline non-AI/ML method may enable the Rel-17 enhancement features (e.g., UE Rx TEG, UE RxTx TEG).

In this section, the dataset composition and the performance results for a series of robustness studies according to the above agreements for AI/ML-based positioning are presented in Table 4.

Table 4. Evaluation results for AI/ML model deployed on network-side, ResNet

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| CIR 18*4*256 | UE coordinates | UE coordinates | {60%, 6m, 2m}, Drop 1 | {60%, 6m, 2m}, Drop 1 | 25000 | 5000 | 34 K | 10M | 0.492 |
|  |  |  | {40%, 2m, 2m}, Drop 1 | {40%, 2m, 2m}, Drop 1 | 25000 | 5000 |  |  | 0.606 |
|  |  |  | {60%, 6m, 2m}, Drop 1 | {60%, 6m, 2m}, Drop 2 | 25000 | 5000 |  |  | >10 |
|  |  |  | {60%, 6m, 2m}, 5 Drops mixed | {60%, 6m, 2m}, Drop 2(outside of the trained Drops) | 25000 (5000/drop) |  |  |  | 8.04 |
|  |  |  | {60%, 6m, 2m}, 5 Drops mixed | {60%, 6m, 2m}, Drop 1 (inside the trained Drops) | 25000 (5000/drop) | 5000 |  |  | 1.28 |
|  |  |  | {60%, 6m, 2m}, Drop 1 & 2 mixed | {60%, 6m, 2m}, Drop 1 (inside the trained Drops) | 25000 (12500/drop) | 5000 |  |  | 0.69 |
|  |  |  | {40%, 2m, 2m}, Drop 1 | {60%, 6m, 2m}, Drop 1 | 25000 | 5000 |  |  | >10 |
|  |  |  | {60%, 6m, 2m} & {40%, 2m, 2m} mixed, Drop 1 | {60%, 6m, 2m}, Drop 1 | 25000 (12500/ paras) | 5000 |  |  | 0.86 |
|  |  |  | {60%, 6m, 2m} & {40%, 2m, 2m} mixed, Drop 1 | {40%, 2m, 2m}, Drop 1 | 25000 (12500/ paras) | 5000 |  |  | 0.88 |
|  |  |  | Without network synchronization error | With network synchronization error @50ns | 25000 | 5000 |  |  | >10 |
|  |  |  | With network synchronization error @50ns | With network synchronization error @50ns | 25000 | 5000 |  |  | 3.02 |
|  |  |  | With network synchronization error @0&30&40&50ns | With network synchronization error @0&30&40&50ns | 25000 (6250/ paras) | 5000 (1250/ paras) |  |  | 2.51 |
|  |  |  | With network synchronization error @0&30&40&50ns | With network synchronization error @50ns | 25000 (6250/ paras) | 5000 (1250/ paras) |  |  | 4.28 |
|  |  |  | Without UE timing error | With UE timing error @10ns | 25000 | 5000 |  |  | 3.12 |
|  |  |  | With UE timing error @10ns | With UE timing error @10ns | 25000 | 5000 |  |  | 0.61 |
|  |  |  | With UE timing error @0&10&20&30ns | With UE timing error @0&10&20&30ns | 25000 (6250/ paras) | 5000 (1250/ paras) |  |  | 0.68 |
|  |  |  | With UE timing error @0&10&20&30ns | With UE timing error @30ns | 25000 (6250/ paras) | 5000 |  |  | 0.89 |

Dimension 1: Different drops.

Dimension 1 is intended for the challenges brought by the loss of spatial consistency when inference happens in a new drop outside the training dataset. From the results shown in the Table 4 above, we can make the following observations:

- : When the inference dataset and the training dataset are from different drops, AI/ML-based fingerprint positioning model provides poor generalization performance. But when the mixed training dataset consists of samples from the same drop as the inference dataset, the generalization performance is improved.

- : Enriching the composition of the mixed training dataset can improve unseen drop’s positioning accuracy.

- : The positioning performance for a seen drop improves when the amount/ratio of data samples from that drop in the mixed training dataset increases.

Dimension 2: Clutter parameters.

Dimension 2 is intended for the challenges brought by the loss of spatial consistency plus the unlearned channel characteristics when inference happens with a different distribution of obstacles than in the training dataset. From the results shown in the Table 4 above, we can make the following observations:

- : When the channel parameters of the inference dataset and the training dataset are different, AI/ML-based fingerprint positioning model provides poor generalization performance.

- : When the mixed training dataset consists of samples with the same channel parameters as the inference dataset, the positioning performance is improved and reaches the sub-meter level.

Dimension 3: Network synchronization error.

Dimension 3 is intended for the challenges brought by the non-ideal assumptions affecting the time-domain channel characteristics when a random synchronization error occurs in channel measurement. From the results shown in the Table 4 above, we can make the following observations:

- : When the model is trained without network synchronization error but inferred with the added network synchronization error randomly distributed with the standard deviation value T_1 = 50ns, AI/ML-based fingerprint positioning model provides poor generalization performance.

- : When the model is both trained and inferred with the added network synchronization error randomly distributed with the standard deviation value T_1 = 50ns, the positioning performance is improved compared with trained without error.

- : When the model is both trained and inferred with the added network synchronization error randomly distributed with mixed T_1 = 0&30&40&50ns, the positioning performance is improved compared with trained without error. The larger synchronization error the network have, the poorer positioning performance they will have.

According to the evaluation results shown in the Table 4 above, the positioning performance of the AI model with CIR input will degrade when the network synchronization error exists. In order to improve the generalization performance of the AI model, we need to find solutions that can address the impact of the network synchronization error.

We therefore make the following proposal:

- : For AI/ML-based positioning, study potential solutions to address the issues of network synchronization error on positioning performance.

Dimension 4: UE timing error.

Dimension 4 is intended for the challenges brought by the non-ideal assumptions affecting the time-domain channel characteristics when a random UE timing error occurs in channel measurement. From the results shown in the Table 4 above, we can make the following observations:

- : When the model is trained without UE timing error but inferred with the added UE timing error randomly distributed with the standard deviation value T_1 = 10ns, the AI/ML-based fingerprint positioning model provides poor generalization performance.

- : When the model is both trained and inferred with the added UE timing error randomly distributed with T_1 = 10ns, the positioning performance is improved compared with trained without error.

- : When the model is both trained and inferred with the added UE timing error randomly distributed with mixed T_1 = 0&10&20&30ns, the positioning performance is improved compared to when trained without error.

As described in TS 38.305, the Tx/Rx timing errors are defined as follows:

- Tx timing error: Result of Tx time delay involved in the transmission of a signal. It is the uncalibrated Tx time delay, or the remaining delay after the TRP/UE internal calibration/compensation of the Tx time delay, involved in the transmission of the DL-PRS/UL SRS signals. The calibration/compensation may also include the calibration/compensation of the relative time delay between different RF chains in the same TRP/UE and may also possibly consider the offset of the Tx antenna phase centre to the physical antenna centre.. - Rx timing error: Result of Rx time delay involved in the reception of a signal before reporting measurements that are obtained from the signal. It is the uncalibrated Rx time delay, or the remaining delay after the UE/TRP internal calibration/compensation of the Rx time delay, involved in the reception of the DL-PRS/UL SRS signals. The calibration/compensation may also include the calibration/compensation of the relative time delay between different RF chains in the same UE/TRP and may also possibly consider the offset of the Rx antenna phase centre to the physical antenna centre.

While for fingerprint positioning, the synchronization error of the transmission link between gNB and UE should also be considered. Therefore, the typical standard deviation value for modelling the UE timing error should also take this into consideration. We are therefore making the following proposal:

- : For evaluation on the AI/ML-based positioning, the synchronization error of the transmission link between gNB and UE should also be considered.

Model updating

In the RAN1 #110bis e-meeting [6], it was agreed that the following table is adopted for reporting the evaluation results of fine-tuning by RAN1:

Agreement For both direct AI/ML positioning and AI/ML assisted positioning, if fine-tuning is evaluated, the template agreed in RAN1#110 is updated to the following for reporting the evaluation results. Table X. Evaluation results for AI/ML model deployed on [UE or network]-side, [short model description] Model input; Model output; Label; Settings (e.g., drops, clutter param, mix); Dataset size; AI/ML complexity; Horizontal pos. accuracy at CDF=90% (m). Train; Fine-tune; Test; Train; Fine-tune; test; Model complexity; Computation complexity; AI/ML.

In addition to mixing the datasets, performance gain brought by fine-tuning is also studied in this paper. And the evaluation results according to the above agreements for AI/ML-based fingerprint positioning are presented in Table 5.

Table 5. Evaluation results for AI/ML model deployed on network-side, ResNet

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  |  | Dataset size |  |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Fine-tune | Test | Train | Fine-tune | test | Model complexity | Computation complexity | AI/ML |
| CIR 18*4*256 | UE coordinates | UE coordinates | {60%, 6m, 2m}, Drop 1 | {60%, 6m, 2m}, Drop 2 | {60%, 6m, 2m}, Drop 2 | 25000 | 1000 | 5000 | 34K | 10M | 3.2 |
|  |  |  | {60%, 6m, 2m}, Drop 1 | {40%, 2m, 2m}, Drop 1 | {40%, 2m, 2m}, Drop 1 | 25000 | 1000 | 5000 |  |  | 3.1 |
|  |  |  | Without network synchronization error | With network synchronization error @50ns | With network synchronization error @50ns | 25000 | 5000 | 5000 |  |  | 8.47 |
|  |  |  | Without UE timing error | With UE timing error@20ns | With UE timing error@20ns | 25000 | 5000 | 5000 |  |  | 1.13 |

As for the evaluated aspect in the generalization studies - Different Drops, the model trained with 25000 samples in Drop 1 with {60%, 6m, 2m} setting is fine-tuned with 1000 samples in Drop 2 with {60%, 6m, 2m} setting. And then the fine-tuned model is inferred in the inference dataset in Drop 2 with {60%, 6m, 2m} setting. From the results shown in the Table 5 above, we can make the following observations:

- : From the model update evaluation results of the above aspect - Different Drops, on top of the dataset with large amount of samples from a different drop, fine-tuned with a relatively small amount of samples from the same drop as the inference dataset will be helpful to improve the generalization performance.

As for the evaluated aspect in the generalization studies - Clutter parameters, the model trained with 25000 samples in Drop 1 with {40%, 2m, 2m} setting is fine-tuned with 1000 samples in Drop 1 with {60%, 6m, 2m} setting. And then the fine-tuned model is inferred in the inference dataset in Drop 1 with {60%, 6m, 2m} setting. From the results shown in the Table 5 above, we can make the following observations:

- : From the model update evaluation results of the above aspect - Clutter parameters, on top of the dataset with large amount of samples from a different clutter setting, fine-tuned with a relatively small amount of samples from the same clutter setting as the inference dataset will be helpful to improve the generalization performance.

As for the evaluated aspect in the generalization studies - Network synchronization error, the model trained with 25000 samples without network synchronization error is fine-tuned with 5000 samples with network synchronization error @50ns. And then the fine-tuned model is inferred in the inference dataset with network synchronization error @50ns. From the results shown in the Table 5 above, we can make the following observations:

- : From the model update evaluation results of the above aspect - Network synchronization error, on top of the dataset with large amount of samples without network synchronization error, fine-tuned with a relatively small amount of samples with the same added network synchronization error randomly distributed as the inference dataset will be helpful to improve the generalization performance but the performance is still poor. Fine-tuning helps less in solving the network synchronization error.

And for the evaluated aspect in the generalization studies - UE timing error, the model trained with 25000 samples without UE timing error is fine-tuned with 5000 samples with UE timing error@20ns. And then the fine-tuned model is inferred in the inference dataset with UE timing error@20ns. From the results shown in the Table 5 above, we can make the following observations:

- : From the model update evaluation results of the above aspect - UE timing error, on top of the dataset with large amount of samples without UE timing error, fine-tuned with a relatively small amount of samples with the same added UE timing error randomly distributed as the inference dataset will be helpful a lot to improve the generalization performance.

According to the evaluation results of the above evaluated scenarios for model updating, the positioning performance of the AI model is improved significantly except for the presence of a network synchronization error. In order to further improve the generalization performance of the AI model, we need to find solutions that can alleviate the impact of this factor.

We therefore make the following proposal:

- : Model updating is supported to improve the performance under the presence of UE timing errors and for the occurrence of the unlearned channel characteristics, including unseen drops and clutter settings.

  - FFS how to improve the performance under the presence of network synchronization errors.

Performance evaluations of AI/ML-based LOS/NLOS identification

Simulation Results

As agreed in the RAN1 #109-e meeting [3], the dataset is generated from the simulation platform for the AI/ML-based LOS/NLOS identification sub use case according to the defined scenario assumptions in the Appendix with FR1 settings and the evaluation is conducted under clutter parameters of {40%, 2m, 2m}. The implementation details for our evaluation on AI/ML-based LOS/NLOS identification are summarized in Table 6.

The model structure for our implementation is CNN. The normalized PDP is selected to be the model input for both training and inference, and the label for training are ideal LOS/NLOS identifications and the output of model inference are LOS/NLOS probabilities. Accordingly, the pre-processing is to obtain the PDP from the measured channel vector/matrix. And the dataset composition for AI/ML-based LOS/NLOS identification is also summarized in Table 6. Note that only a relatively small training dataset size is needed for the AI/ML model applied for this sub use case.

As agreed in the RAN1 #110bis e-meeting [6], the dimension NTRP * Nport * Nt of the model input PDP should be reported:

Agreement For the model input used in evalutions of AI/ML based positioning, if time-domain channel impulse response (CIR) or power delay profile (PDP) is used as model input in the evaluation, companies report the input dimension NTRP * Nport * Nt, where NTRP is the number of TRPs, Nport is the number of transmit/receive antenna port pairs, Nt is the number of time domain samples. - Note: CIR and PDP may have different dimensions. - Note: Companies provide details on their assumption on how PDP is constructed and how (if applicable) it is mapped to Nt samples.

And the training dataset consists of 18000 samples, where 1 sample denotes 1 PDP value of the reference signal transmitted by 1 UE and received by 1 gNB in the deployment with 4 receiving ports by each gNB. And the number of time domain samples used for the evaluation is 256, so the input dimension is 1*4*256.

As shown in Table 6, the accuracy of the AI/ML-based LOS/NLOS identification solution with 4 gNB receiving ports can achieve 0.35m, while the baseline method provides positioning accuracy error of more than 6m @90%. The reason is that for a small number of antennas, the traditional method cannot provide enough resolution to correctly identify with a high probability on whether a path is LOS or NLOS. Therefore, for the widely deployed commercial RF modules with small antenna ports for indoor deployment, using AI/ML-based LOS/NLOS identification solution is meaningful.

Table 6. Evaluation results for AI/ML model deployed on network-side, CNN

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Horizontal pos. accuracy at CDF=90% (m) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| PDP 1*4*256 | LOS probability | Ideal LOS/NLOS identification (LOS probability=0% or 100%) | {40%, 2m, 2m} | {40%, 2m, 2m} | 18000 | 9000 | 582 | 192K | 0.353 |

From the results shown in the Table 6 above, we can make the following observation:

- : From the evaluation results, it is observed that for a small number of gNB antennas, AI/ML-based LOS/NLOS identification could significantly improve the positioning accuracy.

The complexity of the AI/ML model for 4 BS receiving antennas applied is also shown in Table 6. It can be seen that AI/ML model only needs very few parameters and does not require a large number of FLOPs compared to the models used in other use cases, e.g., fingerprint positioning as given in Table 3. However, still the performance improvement compared to the baseline is significant.

- : For AI/ML-based LOS/NLOS identification evaluation, the applied model only needs very small number of parameters and does not require tremendous FLOPs.

    - Generalization studies

In this section, the dataset composition and the performance results of AI/ML-based LOS/NLOS identification for a series of generalization studies according to the agreements referred in Section 3.1.2 are presented. Also in the RAN1 #110bis e-meeting [6], it was agreed that the following aspects of the model generalization capability of the AI/ML based positioning are considered to be studied and evaluated by RAN1:

Agreement To investigate the model generalization capability, the following aspect is also considered for the evaluation of AI/ML based positioning: (e) InF scenarios, e.g., training dataset from one InF scenario (e.g., InF-DH), test dataset from a different InF scenario (e.g., InF-HH)

The generalization studies of AI/ML-based LOS/NLOS identification just focus on these two aspects: Clutter parameters and InF scenarios. That’s because the dataset used for the AI/ML-based LOS/NLOS identification use case doesn’t generated with spatial consistency, so the challenge brought by the different drops exist already in the baseline evaluation results shown in the Table 6. Meanwhile, since the model input of the AI/ML-based LOS/NLOS identification use case is a single-TRP construction, the network synchronization error and the UE timing error no longer affect the model.

Also in the RAN1 #110bis e-meeting [6], it was agreed that an intermediate performance metric of model output should be reported:

Agreement For evaluation of AI/ML assisted positioning, the following intermediate performance metrics are used: - LOS classification accuracy, if the model output includes LOS/NLOS indicator of hard values, where the LOS/NLOS indicator is generated for a link between UE and TRP; - Timing estimation accuracy (expressed in meters), if the model output includes timing estimation (e.g., ToA, RSTD). - Angle estimation accuracy (in degrees), if the model output includes angle estimation (e.g., AoA, AoD). - Companies provide info on how LOS classification accuracy and timing/angle estimation accuracy are estimated, if the ML output is a soft value that represents a probability distribution (e.g., probability of LOS, probability of timing, probability of angle, mean and variance of timing/angle, etc.)

For AI/ML-based LOS/NLOS identification, the identification rate is defined to be the intermediate performance metric to evaluate the model generalization performance:

Identification rate Pr = PrLOS→LOS*RLOS+PrNLOS→NLOS*RNLOS

Where RLOS is the LOS percentage, while RNLOS is the NLOS percentage.

In this section, the dataset composition and the performance results for a series of robustness studies according to the generalization capability agreements for AI/ML-based positioning are presented in Table 7

Table 7. Evaluation results for AI/ML model deployed on network-side, CNN

| Model input | Model output | Label | Settings (e.g., drops, clutter param, mix) |  | Dataset size |  | AI/ML complexity |  | Identification rate |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Train | Test | Train | test | Model complexity | Computation complexity | AI/ML |
| PDP 1*4*256 | LOS probability | Ideal LOS/NLOS identification (LOS probability=0% or 100%) | InF-DH {40%, 2m, 2m} | InF-DH {40%, 2m, 2m} | 18000 | 9000 | 582 | 192K | 97.2% |
|  |  |  | InF-DH {40%, 2m, 2m} | InF-DH {60%, 6m, 2m} | 18000 | 9000 |  |  | 98.6% |
|  |  |  | InF-DH {40%, 2m, 2m} | InF-DH {40%, 3m, 5m} | 18000 | 9000 |  |  | 97.7% |
|  |  |  | InF-SH {20%, 2m, 10m} | InF-DH {40%, 2m, 2m} | 18000 | 9000 |  |  | 95.1% |
|  |  |  | InF-DH {40%, 2m, 2m} | InF-SH {20%, 2m, 10m} | 18000 | 9000 |  |  | 78% |
|  |  |  | InF-DH {40%, 2m, 2m}& InF-SH {20%, 2m, 10m} mixed | InF-SH {20%, 2m, 10m} | 18000 | 9000 |  |  | 97.8% |
|  |  |  | InF-DH {40%, 2m, 2m}& InF-SH {20%, 2m, 10m} mixed | InF-DH {40%, 2m, 2m} | 18000 | 9000 |  |  | 97.3% |

Dimension 1: Clutter parameters.

Dimension 1 is intended for the challenges brought by the unlearned channel characteristics when inference happens with a different distribution of obstacles than in the training dataset. From the results shown in the Table 7 above, we can make the following observation:

- : When the channel parameters of the inference dataset and the training dataset are different, AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS Identification rate.

Dimension 2: InF scenarios.

Dimension 2 is intended for the challenges brought by the unlearned channel characteristics when inference happens in an unseen-distributed environment different from the training dataset. From the results shown in the Table 7 above, we can make the following observation:

- : When the model is trained under the InF-DH scenario but inferred under the InF-SH scenario, the performance deteriorates significantly. But when trained under the InF-SH scenario but inferred under the InF-DH scenario, the AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS Identification rate.

- : When the mixed training dataset consists of samples with the same scenarios as the inference dataset, the performance of the intermediate LOS/NLOS Identification rate is improved under both scenarios’ inference.

Conclusion

The contribution mainly discusses evaluations on each potential enhancement for positioning accuracy improvements, based on which the following proposals and observations are made:

Proposal 1: For evaluation on AI/ML-based fingerprint positioning, support UL-TDoA positioning as a candidate baseline of the legacy non-AI/ML method.

Proposal 2: For AI/ML assisted positioning, AI/ML-based LOS/NLOS identification should be evaluated for positioning accuracy enhancements at least under slight/moderate NLOS scenarios in Rel-18.

Proposal 3: For evaluation of the AI/ML-based LOS/NLOS identification, the achievable positioning accuracy should be compared with the performance of an existing traditional algorithm based on Rel-17 without using AI/ML.

Proposal 4: The model complexity on the AI/ML assisted positioning shall be counted for one model/entity in the evaluation.

Proposal 5: For comparison of evaluation on AI/ML-based fingerprint positioning evaluation results, support the channel impulse responses (CIRs) as the model inputs.

Proposal 6: For AI/ML-based positioning, study potential solutions to address the issues of network synchronization error on positioning performance.

Proposal 7: For evaluation on the AI/ML-based positioning, the synchronization error of the transmission link between gNB and UE should also be considered.

Proposal 8: Model updating is supported to improve the performance under the presence of UE timing errors and for the occurrence of the unlearned channel characteristics, including unseen drops and clutter settings.

•	FFS how to improve the performance under the presence of network synchronization errors.

Observation 1: The agreement for assisted positioning from last meeting on “model complexity” for the UE-side model is achieved for one UE. It is not applicable to the NW-side, since with this agreement the complexity would need to be counted over N models in N TRPs. Thus, no objective information about the complicity of one model could be obtained for a NW-side deployment.

Observation 2: From the above evaluation results, it is observed that AI/ML-based fingerprint positioning provides reliable positioning accuracy under both heavy and moderate NLOS conditions.

Observation 3: From the above evaluation results, it is observed that AI/ML-based fingerprint positioning provides reliable positioning accuracy under a small number of receiving ports.

Observation 4: When the inference dataset and the training dataset are from different drops, AI/ML-based fingerprint positioning model provides poor generalization performance. But when the mixed training dataset consists of samples from the same drop as the inference dataset, the generalization performance is improved.

Observation 5: Enriching the composition of the mixed training dataset can improve unseen drop’s positioning accuracy.

Observation 6: The positioning performance for a seen drop improves when the amount/ratio of data samples from that drop in the mixed training dataset increases.

Observation 7: When the channel parameters of the inference dataset and the training dataset are different, AI/ML-based fingerprint positioning model provides poor generalization performance.

Observation 8: When the mixed training dataset consists of samples with the same channel parameters as the inference dataset, the positioning performance is improved and reaches the sub-meter level.

Observation 9: When the model is trained without network synchronization error but inferred with the added network synchronization error randomly distributed with the standard deviation value T_1 = 50ns, AI/ML-based fingerprint positioning model provides poor generalization performance.

Observation 10: When the model is both trained and inferred with the added network synchronization error randomly distributed with the standard deviation value T_1 = 50ns, the positioning performance is improved compared with trained without error.

Observation 11: When the model is both trained and inferred with the added network synchronization error randomly distributed with mixed T_1 = 0&30&40&50ns, the positioning performance is improved compared with trained without error. The larger synchronization error the network have, the poorer positioning performance they will have.

Observation 12: When the model is trained without UE timing error but inferred with the added UE timing error randomly distributed with the standard deviation value T_1 = 10ns, the AI/ML-based fingerprint positioning model provides poor generalization performance.

Observation 13: When the model is both trained and inferred with the added UE timing error randomly distributed with T_1 = 10ns, the positioning performance is improved compared with trained without error.

Observation 14: When the model is both trained and inferred with the added UE timing error randomly distributed with mixed T_1 = 0&10&20&30ns, the positioning performance is improved compared to when trained without error.

Observation 15: From the model update evaluation results of the above aspect - Different Drops, on top of the dataset with large amount of samples from a different drop, fine-tuned with a relatively small amount of samples from the same drop as the inference dataset will be helpful to improve the generalization performance.

Observation 16: From the model update evaluation results of the above aspect - Clutter parameters, on top of the dataset with large amount of samples from a different clutter setting, fine-tuned with a relatively small amount of samples from the same clutter setting as the inference dataset will be helpful to improve the generalization performance.

Observation 17: From the model update evaluation results of the above aspect - Network synchronization error, on top of the dataset with large amount of samples without network synchronization error, fine-tuned with a relatively small amount of samples with the same added network synchronization error randomly distributed as the inference dataset will be helpful to improve the generalization performance but the performance is still poor. Fine-tuning helps less in solving the network synchronization error.

Observation 18: From the model update evaluation results of the above aspect - UE timing error, on top of the dataset with large amount of samples without UE timing error, fine-tuned with a relatively small amount of samples with the same added UE timing error randomly distributed as the inference dataset will be helpful a lot to improve the generalization performance.

Observation 19: From the evaluation results, it is observed that for a small number of gNB antennas, AI/ML-based LOS/NLOS identification could significantly improve the positioning accuracy.

Observation 20: For AI/ML-based LOS/NLOS identification evaluation, the applied model only needs very small number of parameters and does not require tremendous FLOPs.

Observation 21: When the channel parameters of the inference dataset and the training dataset are different, AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS Identification rate.

Observation 22: When the model is trained under the InF-DH scenario but inferred under the InF-SH scenario, the performance deteriorates significantly. But when trained under the InF-SH scenario but inferred under the InF-DH scenario, the AI/ML-based LOS/NLOS identification model provides great generalization performance of the intermediate LOS/NLOS Identification rate.

Observation 23: When the mixed training dataset consists of samples with the same scenarios as the inference dataset, the performance of the intermediate LOS/NLOS Identification rate is improved under both scenarios’ inference.

- References

- RAN1 Chairman’s notes, RAN1#110, Toulouse, France, August 22 – 26, 2022.

- R1-2210890, “Discussions on AI/ML for positioning accuracy enhancement”, RAN1#111, Huawei, HiSilicon, Toulouse, France, November 14 – 18, 2022.

- RAN1 Chairman’s notes, RAN1#109-e, May 9 – 20, 2022.

- R1-2106453, “Enhancements to support multi-path and NLOS mitigation”, Huawei, HiSilicon, e-Meeting, August 16 – 27, 2021.

- R1-2007577, “Positioning enhancement in Rel-17”, Huawei, HiSilicon, e-meeting, October 26 – November 13, 2020.

- RAN1 Chairman’s notes, RAN1#110bis-e, e-Meeting, October 10 – 19, 2022.

- Appendix A

Simulation assumptions for the evaluated sub use cases:

|  |  | FR1 Specific Values |  | FR2 Specific Values |
| --- | --- | --- | --- | --- |
| Channel model |  | InF-DH |  | InF-DH |
| Layout | Hall size | InF-DH: (baseline) 120x60 m (optional) 300x150 m |  |  |
|  | BS locations | 18 BSs on a square lattice with spacing D, located D/2 from the walls. - for the small hall (L=120m x W=60m): D=20m - for the big hall (L=300m x W=150m): D=50m |  |  |
|  | Room height | 10m |  |  |
| Total gNB TX power, dBm |  | 24dBm | 24dBm EIRP should not exceed 58 dBm |  |
| gNB antenna configuration |  | (M, N, P, Mg, Ng) = (4, 4, 2, 1, 1), dH=dV=0.5λ – Note 1 Note: Other gNB antenna configurations are not precluded for evaluation | (M, N, P, Mg, Ng) = (4, 8, 2, 1, 1), dH=dV=0.5λ – Note 1 One TXRU per polarization per panel is assumed |  |
| gNB antenna radiation pattern |  | Single sector – Note 1 | 3-sector antenna configuration – Note 1 |  |
| Penetration loss |  | 0dB |  |  |
| Number of floors |  | 1 |  |  |
| UE horizontal drop procedure |  | Uniformly distributed over the horizontal evaluation area for obtaining the CDF values for positioning accuracy, The evaluation area should be selected from - (baseline) the whole hall area, and the CDF values for positioning accuracy is obtained from whole hall area. - (optional) the convex hull of the horizontal BS deployment, and the CDF values for positioning accuracy is obtained from the convex hull. |  |  |
| UE antenna height |  | Baseline: 1.5m (Optional): uniformly distributed within [0.5, X2]m, where X2 = 2m for scenario 1(InF-SH) and X2= for scenario 2 (InF-DH) |  |  |
| UE mobility |  | 3km/h |  |  |
| Min gNB-UE distance (2D), m |  | 0m |  |  |
| gNB antenna height |  | Baseline: 8m (Optional): two fixed heights, either {4, 8} m, or {max(4,), 8}. |  |  |
| Clutter parameters: {density , height ,size } |  | High clutter density: - {40%, 2m, 2m} - {60%, 6m, 2m} |  |  |
| Note 1: According to Table A.2.1-7 in TR 38.802 |  |  |  |  |

- Appendix B

Spatial consistency modeling for the direct AI/ML positioning sub use case:

For modeling the spatial consistency, we adopt 2D-filtering method described as illustrated in Figure 4. Figure 4 2D-filtering spatial consistency modelling according to 3GPP TR 38.901 In 3GPP, a spatial correlation of the small scale fading is introduced in the channel generation. Its procedure is given in TR 38.901, where cluster-specific random variables are simulated spatially consistently for drop-based simulations. To generate spatially consistent random variables at specific coordinates (x, y) using the 2D-filtering method, the following steps are taken: - Step 1: Divide the simulation area into correlated grids of custom length and width (usually set to be smaller than correlation distances); - Step 2: Generate independent and identically distributed random variables for each vertex of one correlated grid; - Step 3: Deliver the generated random variables to the exponential decaying filter in the two dimensional horizontal plane, which is used for spatially consistent LSP (Large Scale Parameters) generation in current 3GPP 3D channel model. Then the random variables at each grid are correlated following the exponential function with respect to correlation distances in the two dimensional horizontal plane. - Step 4: Determine which grid the UE coordinate (x, y) belongs to, and generate the UE’s channel with the random variables of this grid.