3GPP TSG RAN WG1 Meeting #111	 R1-2210907

Toulouse, France, November 14 - 18, 2022

Agenda Item:	9.10.2

Source:	Huawei, HiSilicon

Title:	Discussion on XR-specific capacity enhancements techniques

Document for:	Discussion and Decision

# Introduction

In 3GPP Rel-17 study item “Study on XR Evaluations for NR”, progress has been made on many aspects, e.g. applications, KPIs, evaluation assumptions/methodologies, and performance evaluations. The results have been summarized in TR 38.838 [1]. In Rel-18, a new SID “Study on XR Enhancements for NR” has been approved in RAN#94e with the following areas: XR-awareness in RAN, XR-specific power saving, and XR-specific capacity improvements [2].

In this contribution, we will provide our views for XR-specific capacity enhancements, including CG enhancements to handle variable frame size, SR/BSR enhancements for dynamic scheduling/grant, etc. We also provide our views on measurement gap issue.

# CG enhancements

In RAN1#110bis-e [4], the following agreement on CG enhancements was achieved.

Agreement To study whether/how the enhanced CG candidate techniques are necessary and beneficial for improving XR capacity, focus at least on the following techniques: - Dynamic indication of the unused CG PUSCH occasion(s) or resource(s) by the UE - Increase CG PUSCH transmission occasions in a duration

In the following, we analyze the pros and cons of DG and CG and then focus on discussing the necessity and benefits of CG enhancements.

## CG vs DG scheduling

In RAN1#110-e [5], different opinions on CG enhancements were proposed. Some companies think that CG is suitable for XR UL transmission and they proposed some enhancements to improve capacity, while some other companies proposed that dynamic scheduling can achieve better performance so that there is no need to do any enhancements on CG. In this part, we analyze the capacity performance for UL video traffic with CG and DG scheduling and provide some initial simulation results. The following four cases are considered:

Case1: Legacy DG (first SR, then BSR with small grant, then UL data):

- It is assumed that an SR is triggered upon arrival of a new TB in the UE buffer. A small UL resource is then granted to the UE to transmit BSR and a small amount of UL data. The follow up UL grants are updated based on corresponding BSR.

Con: Large delay caused by SR and BSR report

Generally, SR/BSR procedure may need at least 2 U slots, which may cause delay and decrease UL capacity. If these SR/BSR procedures are saved for frame/data transmission, the UL capacity would be improved. Based on the initial evaluation results in clause 7.3.2.2 of TR 38.838 [1], the UL capacity increases from <1 to ~5 UEs by increasing PDB from 10 ms to 15 ms (adding 2 U slots).

Case2: Legacy DG (first SR, then BSR with large grant, then remaining UL data):

- It is assumed that an SR is triggered upon arrival of a new TB in the UE buffer. A big UL grant is allocated to the UE to transmit both BSR and UL data. The follow up UL grants are updated based on corresponding BSR. The initial big grant can be allocated according to the minimum size of XR frame (e.g. half of the mean frame size) in order to avoid resource waste. This case can reduce the delay caused by BSR procedure.

Con: Delay caused by SR report

According to the initial evaluation results in clause 7.3.2.2 of TR 38.838 [1], the UL capacity is <1 when PDB = 10 ms even though accurate frame size is assumed to be known after SR is sent. Thus, Legacy DG (SR + BSR with big grant) cannot perform well in the tight latency requirement scenario.

Con: Depend on XR awareness objective outcome

Case 2 cannot work well if gNB does not know frame size related information. However, whether the frame size related information would be known by gNB is not clear so far and it highly depends on the XR-awareness objective outcome.

Case3: Pre-scheduling DG (No SR, BSR with large grant, then remaining UL data):

- An initial grant for BSR and UL data is dynamically pre-scheduled without relying on SR. It is assumed that the XR periodicity, frame size range and arrivals related information is known by gNB. The initial grant can be allocated according to the minimum size of XR frame (e.g. half of the mean frame size) in order to avoid resource waste. This case can reduce the delay caused by SR/BSR procedure.

Con: Depend on XR awareness objective outcome

Which XR information would be known by gNB is not clear yet and it highly depends on the XR-awareness objective outcome. Case 3 cannot work well if gNB does not know these XR awareness information, e.g., frame arrival time, frame size, etc.

Con: C-DRX or PDCCH search space set may limit gNB scheduling

Considering C-DRX or PDCCH search space set may be applied for power saving, it is possible that UE cannot receive the pre-scheduling DCI sometimes since UE may fall into sleep state. In this case, the UL capacity equals to that in Case2 or Case1.

Case4: Configured Grant (CG)

- An initial grant for BSR and UL data is pre-configured without relying on SR. Similar with Case3, CG can also reduce the delay caused by SR/BSR procedure. Although CG may also has XR-awareness issues like Case3, the advantage of CG is that CG does not require UE to monitor PDCCH so that CG can perform well even when UE falls into sleep. CG needs to handle variable frame size issue, which is to be discussed in Section 2.2.

Figure 1. UL scheduling comparison (CG vs DG)

Observation 1:  The pros and cons of the above four cases are as shown in Table 1:

Table 1. Pros and Cons of DG and CG

| Scheduling scheme | Pros | Cons |
| --- | --- | --- |
| Case1: Legacy DG (first SR, then BSR with small grant, then UL data) | Not require prior information of XR traffic | Large delay due to SR/BSR report, thus causing capacity loss |
| Case2: Legacy DG (first SR, then BSR with large grant, then remaining UL data) | No delay due to BSR report | Medium delay due to SR report, thus causing capacity loss especially when UL PDB is small |
| Case3: Pre-scheduling DG (No SR, first BSR with large grant, then remaining UL data) | No delay due to SR/BSR report | Too ideal assumptions, may not be feasible in every real deployment. E.g., it’s possible that gNB may not know accurate XR frame arrival/size information, UE may not be available to receive DCI in every slot (e.g., due to C-DRX, search space set, PDCCH skipping, etc.), etc. |
| Case4: Configured Grant (CG) | No delay due to SR/BSR report | Need to handle variable frame size issue |

We simulated the above cases.

- Case 1 (C1: Legacy DG (first SR, then BSR with small grant, then UL data)): We assume that SR can be sent every U slot if SR is triggered upon arrival of a new TB in the UE buffer. A small grant (the gNB allocates resources according to 400bits) can be allocated to the UE to transmit BSR and a small amount of UL data. In this case, the minimum delay introduced by SR/BSR procedure is 5ms. If scheduling delay is further considered, the transmission delay will further increase.

- Case 2 (C2: Legacy DG (first SR, then BSR with large grant, then remaining UL data)): Similar with Case1, SR is also assumed to be sent every U slot if SR is triggered. A big grant (the gNB allocates resources according to half of the mean frame size: 83333 bits) can be allocated to the UE to transmit BSR and some UL data. In this case, the minimum delay introduced by SR procedure is 2.5ms. If scheduling delay is further considered, the transmission delay will further increase.

- Case 3 (C3: Pre-scheduling DG (No SR, first BSR with large grant, then remaining UL data): We assume that BSR can be sent every U slot. XR periodicity, arrivals and frame size related information is known by the gNB. A big grant (according to half of the mean frame size: 83333 bits) can be allocated to the UE to transmit BSR and some UL data. In this case, the delay caused by SR/BSR procedure has been reduced.

- Case 4 (C4: Configured Grant (CG): We assume periodicity of CG resource is matched to XR traffic arrival.

We consider DDDSU configuration. Other simulation assumptions are listed in Appendix A.

Figure 2. Capacity performance comparison between DG and CG

The simulation results are in Figure 2. For the C1, the network can support less than 1 user with PDB=10ms since SR/BSR procedure introduces intolerable delay. Although C2 addresses the delay introduced by BSR, the network still does not support a user due to limited PDB. Comparing with C1 and C2, 1.0 UEs per cell can be supported by C4 since CG can reduce the SR/BSR delay. Configured grant transmission can support more UEs compared with dynamic grant. Note that the current assumptions on SR for C1 and C2 are ideal. If the periodicity of SR is assumed larger, like 5ms, the capacity will decrease further. It can be observed that the network can support 1.7 UEs per cell by C3. However, many ideal assumptions have been made in C3.  If the gNB does not know the actual frame arrival time or UE may not be available to receive pre-scheduling DCI in every slot, the capacity with C3 may decrease.

Observation 2: Configured grant transmission can support more UEs compared with dynamic grant that requires SR/BSR report by reducing the transmission delay.

Observation 3: Pre-scheduling DG, which provides the upper bound of UL capacity in theory, has too ideal assumptions (e.g. gNB knows XR frame arrival/size information and UE are available to receive DCI in every slot), which may not be feasible in every real deployment. RAN1 needs to do comprehensive study which also considers more realistic assumptions.

## CG enhancement to handle variable frame size

Increasing CG PUSCH transmission occasions in a duration has been discussed in the last meeting and different opinions were provided. Most proponents think it is not clear that how multiple CG PUSCH occasions can perform better capacity than DG or Hybrid CG + DG. The comparison between CG and multiple DG schemes has provided in section 2.1. As for Hybrid CG + DG, even though it can save the delay due to SR/BSR procedure, additional dynamic scheduling is needed. Different from the configured resources, UE needs to wait dynamic indication from the gNB. This queueing delay is very common in real application and will increase with the number of UEs in the network, which will decrease XR capacity. When multiple CG PUSCH transmission occasions in a duration is considered, less queueing delay is cost.

Regarding dynamic indication of the unused CG PUSCH occasion(s) by the UE, based on the outcomes in clause 5.1 of TR 38.838 [1], XR frame size of single stream follows truncated Gaussian distribution where the maximum size is 3 times of the minimum size. The parameters of the video packet size distribution are related to video encoding, e.g. error resilience, rate control, etc. As for multi-streams, the average size ratio between one I-frame/slice and one P-frame/slice can be 1.5~3. In this case, the maximum ratio between sizes of two XR frames/slices can be up to 9. Therefore, scheduling based on a fixed radio resource size cannot accommodate variable frame sizes.

Observation 4: The size of XR frames/slices varies in a wide range which does not suite scheduling based on a fixed radio resource size.

As the XR video traffic is periodically generated and has low latency requirement, it is beneficial to adopt configured grant (CG) for the UL XR video transmission without requiring scheduling request (SR) and buffer status report (BSR) procedures.  For XR video, the large frame size may require more than one PUSCHs to be transmitted in each video frame period. According to our evaluation, it generally requires 1~5 PUSCHs per video frame, depending on the channel condition and the video frame size. Therefore, it is necessary to configure multiple PUSCH occasions within a CG period, which is introduced by R16 NR-U copied as follows.

(below is copied from TS 38.214 Clause 6.1.2.3) 6.1.2.3 Resource allocation for uplink transmission with configured grant … A set of allowed periodicities P are defined in [12, TS 38.331]. The higher layer parameter cg-nrofSlots, provides the number of consecutive slots allocated within a configured grant period. The higher layer parameter cg-nrofPUSCH-InSlot provides the number of consecutive PUSCH allocations within a slot, where the first PUSCH allocation follows the higher layer parameter timeDomainAllocation for Type 1 PUSCH transmission or the higher layer configuration according to [10, TS 38.321], and UL grant received on the DCI for Type 2 PUSCH transmissions, and the remaining PUSCH allocations have the same length and PUSCH mapping type, and are appended following the previous allocations without any gaps. The same combination of start symbol and length and PUSCH mapping type repeats over the consecutively allocated slots.

However, the CG resource is semi-statically configured, which cannot adapt to the varying size of XR frames. As illustrated in Figure 3, if 4 PUSCHs in one CG period are configured, there will be resource waste for the video frames that need less than 4 PUSCHs. If 3 PUSCHs in one CG period are configured, if the video frame is large, e.g. 4 PUSCHs are required, then additional dynamic scheduling is needed, resulting in extra delay.

Figure 3. Variable XR frame size and semi-statically configured CG resource

Observation 5: CG resource is semi-statically configured and cannot adapt to the varying size of XR frames,

- If the size of CG resource is larger than the actual frame size, radio resources may be wasted;

- If the size of CG resource is not large enough to transmit the current frame, additional dynamic scheduling is needed, resulting in extra delay and reduced capacity.

Figure 4. Illustration of indicating to gNB the unused PUSCH occasions within one CG period

In order to avoid the extra delay caused by additional dynamic scheduling, the CG resource within one CG period should be configured according to a relatively large size of XR frame. To avoid resource waste, mechanisms to allow re-allocate the unused CG resources within one CG period can be considered. When an UL XR video frame arrives, the UE can know the video frame size and thus the number of PUSCHs needed to transmit the video frame. If there are one or more unused PUSCHs, the UE can indicate the unused CG PUSCHs to the gNB via UCI or MAC CE in the first CG PUSCH occasion, as shown in Figure 4. A new type of UCI or MAC CE may be needed. Similar as CG-UCI, such UCI can be reported on the PUSCH. Then the gNB can re-allocate those unused CG PUSCH resources to other UEs. The detailed indication can be further studied in WI phase.

Proposal 1: Support configuring multiple PUSCH occasions in a CG period, and support dynamically indicating to the gNB the unused PUSCH occasions within a CG period.

# Dynamic scheduling/grant enhancements

In RAN1#110bis-e [4], the following conclusion on SR enhancements was made.

Conclusion No consensus to continue study of multi-bits SR mechanisms for capacity improvement of XR traffic in RAN1 XR SI.

In RAN1#110-e [5], the following agreements on BSR enhancements was achieved.

Agreement Whether/how to enhance BSR to improve capacity performance of XR traffic is within RAN2 scope and is not handled by RAN1. - Note that companies should indicate if and what BSR enhancement is assumed in their RAN1 proposals on CG and DG enhancements. - RAN1 can evaluate BSR enhancement to improve capacity performance

Companies are encouraged to submit BSR evaluation results to RAN2 by FL. However, simulation results may not be discussed in RAN2. On the other hand, based on the above agreement, RAN1 can evaluate BSR enhancement to improve capacity performance. In this section, we provide simulation results to show that it is useful for UE to indicate to the network the data arrival time or the remaining delivery time of UL XR traffic. RAN2 can evaluate different solutions based on RAN1’s simulation results. Thus, it is meaningful for RAN1 to capture this results in TR 38.835 [3].The detailed indication can be further studied in RAN2.

## Enhancements related to SR and/or BSR

Enhancement to enable accurate UL delay-aware scheduling

According to the XR evaluation of Rel-17, some enhanced schedulers, e.g., delay-aware scheduler which takes the remaining delivery time of the XR video frame into account, can achieve large capacity gain. Generally, for delay-aware scheduling, gNB prioritize scheduling users whose packet is about to expire and thus can meet more UE’s PDB requirement and increase capacity. For DL transmission, the buffering time can be aware by the gNB through gNB implementation.

For UL transmission, according to current specification, after UL data arrives, a UE may need to first report SR and then report BSR to the gNB before being scheduled. The gNB can be aware of UL data arrival of a UE only when an SR or BSR is received. Therefore, there exists a time gap between the actual UL data arrival time and the time when an SR or BSR is receive, such that gNB does not have accurate delay information of UL data, resulting in inaccurate delay-aware scheduling and causing capacity loss. For example, for typical TDD configuration, e.g., DDDSUDDDSU as agreed in R17 XR, the maximum delay between data arrival and SR or BSR can be 5 slots. If 15 kHz SCS is used, the delay can be as large as 5 ms. For DDDDD DDSUU configuration, the maximum time gap between data arrival and SR or BSR can be 9 ms.

Figure 5. Example of time gap between data arrival and SR

To cope with the above issues, one straightforward way is to let the gNB knows the exact UL data arrival time, so that it can perform more accurate delay-aware scheduling, i.e., schedule UEs transmission and manage network resources more effectively. We simulated the following cases to show the capacity gain of such enhancement:

- Case 1 (C1: Proportional Fair scheduling): we assume gNB uses PF scheduling, i.e., the scheduling priority of each user is calculated as the ratio of the instantaneous data rate over the historical data rate. The gNB does not consider delay during scheduling.

- Case 2 (C2: Delay-aware, gNB is not aware of the exact data arrival time): we assume gNB is not aware of the exact UL data arrival time, and can only apply SR or BSR reception time for delay-aware scheduling.

- Case 3 (C3: Delay-aware, gNB is aware of the exact data arrival time): we assume gNB is aware of the exact data arrival time, and uses the exact data arrival time for delay-aware scheduling.

In this simulation, we consider DDDSU configuration, and assume SR/BSR is reported every 2 U slots in all 3 Cases. Other simulation assumptions are listed in Appendix A.

The simulation results are in Figure 6. By comparing C1 and C2, it is observed that delay-aware scheduling can increase capacity, but since the gNB cannot know the exact remaining delivery time, the XR capacity gain using delay-aware (C2) can be improved by only 6.7% compared with PF scheduling (C1). If gNB is aware of the exact data arrival time (C3), the capacity can be largely increased. For C3, The XR capacity can be improved by 26.67% compared with PF scheme (C1), and improved by 18.7% compared with C2.

Therefore, similar to reporting buffer status information of UL traffic to the gNB, it is also beneficial to support UE indicating the exact data arrival time or the remaining delivery time of the UL traffic for delay-aware scheduling. The detailed indication can be further studied, e.g., via enhanced SR, BSR, or new MAC CE.

Observation 6: As shown in Table 2, the network capacity of the UL AR video stream can be improved by applying delay aware scheduling. If gNB is aware of the exact data arrival time, the capacity can be largely increased.

Table 2. Capacity of the UL AR video stream with 10Mbps in FR1 Dense Urban MU-MIMO

| Scheduling scheme | Case 1 (Proportional Fair) | Case 2 (Delay-aware, gNB is not aware of the exact data arrival time) | Case 3 (Delay-aware, gNB is aware of the exact data arrival time) |
| --- | --- | --- | --- |
| Average number of supported users per cell | 3.0 | 3.2 | 3.8 |
| Capacity improvement over Case 1 | - | 6.7% | 26.67% |

Proposal 2: To enable accurate UL delay-aware scheduling, support UE indicating to the network the data arrival time or the remaining delivery time of UL XR traffic.

Figure 6. Capacity results of the AR UL video stream with/without knowing the exact data arrival time

# Measurement gap

In RAN1#110bis-e [4], different opinions on MG enhancements were proposed. Some companies proposed enhanced schemes on releasing scheduling restrictions during MG to improve XR capacity, while some other companies showed some concerns on this topic.

Enhancements by releasing scheduling restrictions during MG will affect RRM measurement. However, measurement procedures are defined in TS 38.133, which is handled by RAN4. Whether the impact on RRM measurement can be acceptable needs to be analyzed by RAN4. RAN1 should avoid a situation that spending quite some efforts defining a solution on MG, but may not be acceptable by RAN4 from RRM measurement perspective.

Proposal 3: RAN1 needs to consult RAN4 on the feasibility of transmitting data during measurement gaps before diving into detailed solutions.

# Enhancement of CA HARQ retransmission

In RAN1#110bis-e [4], the following agreement on CA enhancements was achieve.

Agreement - For further study the mechanisms to enable HARQ retransmission of a TB on a different cell than the cell of the initial TB transmission for CA operation on TDD cells, consider at least the following: - Capacity performance evaluation results - Complexity analysis and RAN2 impact

XR traffic poses high requirements for latency and reliability. If DL data transmission fails, retransmission scheduling needs to wait UL slot for UE to feedback NACK, which will lead to large transmission latency. In this case, cross carrier HARQ can bring the benefits of latency reduction since the initial transmission is sent on one particular CC and the associated HARQ retransmissions can be scheduled and sent on another CC. The capacity gain is obvious if re-transmissions can be performed on another complementary TDD band or FDD band. Thus, we support further study the enhancement of CA HARQ retransmission.

Proposal 4: Support further study the enhancements of CA HARQ retransmission.

# Conclusions

In this contribution, we compare the capacity performance of CG and DG and then provide our views on XR-specific capacity enhancements techniques based on XR-specific traffic characteristics and proposed some detailed solutions for XR-specific capacity enhancements, including CG enhancements to handle variable frame size and SR/BSR enhancements for dynamic scheduling/grant. The following proposals and observations are given:

Proposal 1: Support configuring multiple PUSCH occasions in a CG period, and support dynamically indicating to the gNB the unused PUSCH occasions within a CG period.

Proposal 2: To enable accurate UL delay-aware scheduling, support UE indicating to the network the data arrival time or the remaining delivery time of UL XR traffic.

Proposal 3: RAN1 needs to consult RAN4 on the feasibility of transmitting data during measurement gaps before diving into detailed solutions.

Proposal 4: Support further study the enhancements of CA HARQ retransmission.

Observation 1:  The pros and cons of the above four cases are as shown in Table 1:

Table 3. Pros and Cons of DG and CG

| Scheduling scheme | Pros | Cons |
| --- | --- | --- |
| Case1: Legacy DG (first SR, then BSR with small grant, then UL data) | Not require prior information of XR traffic | Large delay due to SR/BSR report, thus causing capacity loss |
| Case2: Legacy DG (first SR, then BSR with large grant, then remaining UL data) | No delay due to BSR report | Medium delay due to SR report, thus causing capacity loss especially when UL PDB is small |
| Case3: Pre-scheduling DG (No SR, first BSR with large grant, then remaining UL data) | No delay due to SR/BSR report | Too ideal assumptions, may not be feasible in every real deployment. E.g., it’s possible that gNB may not know accurate XR frame arrival/size information, UE may not be available to receive DCI in every slot (e.g., due to C-DRX, search space set, PDCCH skipping, etc.), etc. |
| Case4: Configured Grant (CG) | No delay due to SR/BSR report | Need to handle variable frame size issue |

Observation 2: Configured grant transmission can support more UEs compared with dynamic grant that requires SR/BSR report by reducing the transmission delay.

Observation 3: Pre-scheduling DG, which provides the upper bound of UL capacity in theory, has too ideal assumptions (e.g. gNB knows XR frame arrival/size information and UE are available to receive DCI in every slot), which may not be feasible in every real deployment. RAN1 needs to do comprehensive study which also considers more realistic assumptions.

Observation 4: The size of XR frames/slices varies in a wide range which does not suite scheduling based on a fixed radio resource size.

Observation 5: CG resource is semi-statically configured and cannot adapt to the varying size of XR frames,

- If the size of CG resource is larger than the actual frame size, radio resources may be wasted;

- If the size of CG resource is not large enough to transmit the current frame, additional dynamic scheduling is needed, resulting in extra delay and reduced capacity.

Observation 6: As shown in Table 2, the network capacity of the UL AR video stream can be improved by applying delay aware scheduling. If gNB is aware of the exact data arrival time, the capacity can be largely increased.

Table 4. Capacity of the UL AR video stream with 10Mbps in FR1 Dense Urban MU-MIMO

| Scheduling scheme | Case 1 (Proportional Fair) | Case 2 (Delay-aware, gNB is not aware of the exact data arrival time) | Case 3 (Delay-aware, gNB is aware of the exact data arrival time) |
| --- | --- | --- | --- |
| Average number of supported users per cell | 3.0 | 3.2 | 3.8 |
| Capacity improvement over Case 1 | - | 6.7% | 26.67% |

# References

TR 38.838, “Study on XR (Extended Reality) Evaluations for NR,”

RP-213587, “Study on XR Enhancements for NR”, RAN#94e, Dec. 6 - 17, 2021.

R2-2210814, 3GPP TR 38.835 V0.3.0.

Chairman notes, RAN1#110bis-e, October 10 - 19, 2022.

Chairman notes, RAN1#110-e, Aug 22 - 26, 2022.

Chairman notes, RAN1#109-e, May 9 - 20, 2022.

Chairman notes, RAN2#119-e, Aug 17 - 26, 2022.

# Appendix A: System Simulation Parameters

Table A-1. System level simulation assumption for FR1

| Parameter | Value |  |
| --- | --- | --- |
| Scenarios | Scenario-1: Dense Urban | Scenario-2: Urban Macro |
| Layout | 21cells with wraparound |  |
| Channel Model | UMa |  |
| Carrier frequency | 4.0 GHz |  |
| Bandwidth | 100 MHz |  |
| Subcarrier spacing | 30 KHz |  |
| Frame structure | [DDDSU DDDSU] Detailed S slot format is 10D:2F:2U. |  |
| Inter Site Distance | 200m | 500m |
| BS Antenna Height | 25m |  |
| BS Antennas | Option 1: 64 TxRU, (M, N, P, Mg, Ng; Mp, Np) = (8,8,2,1,1;4,8) |  |
| CSI-RS Port | 32 |  |
| UE Antenna Height | General equation: hUT=3(nfl – 1) + 1.5 - nfl for outdoor UEs: 1 - nfl for indoor UEs: nfl ~ uniform(1, Nfl ) where Nfl ~ uniform(4,8) |  |
| UE Antennas | Baseline: 2T/4R, (M, N, P, Mg, Ng; Mp, Np) = (1,2,2,1,1;1,2), (dH, dV) = (0.5, N/A)λ |  |
| BS antenna pattern | 3-sector antenna radiation pattern, 8 dBi |  |
| UE antenna pattern | Omnidirectional, 0 dBi |  |
| TX Power | BS : 44 dBm per 20MHz | BS : 49 dBm per 20MHz |
| UE MAX Power | 23dBm |  |
| Noise Figure | BS:5 dB, UE:9 dB |  |
| Scheduler | SU-MIMO Proportional Fair/MU-MIMO Proportional Fair |  |
| MCS | Up to 256QAM |  |
| UE distribution | 80% indoor, 20% outdoor |  |
| UE speed | 3 km/h |  |
| Channel Estimation | Realistic |  |
| Downtilt | 12 degree |  |
| CSI acquisition | CSI-RS: Period=5ms, Density=1, feedback delay=1ms, 4bit quantization, and modelling the error SRS: Period=5ms |  |
| Transmission scheme | Close loop rank adaptation |  |
| PHY processing delay | UE PDSCH processing Capability #1 |  |
| PDCCH overhead | 2 symbols |  |
| Target BLER | 10% |  |
| Max HARQ transmission | 4 |  |

Note 1: For frame structure, U symbol of S slot is not used for uplink transmission.