3GPP TSG RAN WG1 #111		                                                          R1-2211147

Toulouse, France, November 14th – 18th, 2022

Source:	CATT

Title:	Discussion on MBS supporting  deferring HARQ-ACK for SPS PDSCH

Agenda Item:	8.12

Document for:	Discussion and Decision

- Introduction

In this contribution, the issue on MBS supporting deferring HARQ-ACK for SPS PDSCH was discussed. It is proposed that the procedure of deferring HARQ-ACK codebook is also applied for multicast SPS PDSCH since current specification has already supported it and we just need clarify whether RRC parameter IE spsHARQdeferral can be used for multicast SPS configuration.

- Discussion

In Rel-16, SPS PDSCH transmission can be configured by RRC, and activation/release or re-transmission is trigged by DCI format scrambled by CS-RNTI. When the SPS PDSCH without PDCCH scheduling is transmitted, the k1 value that computes PUCCH slot is following to the value of PDSCH-to-HARQ feedback timing indicator indicated by activation DCI. So it may cause HARQ-ACK of SPS PDSCH dropping if the PUCCH has overlapping with downlink symbols. The action of dropping HARQ-ACK codebook will cause low transmission efficiency since gNB cannot obtain the HARQ-ACK information.

To resolve the dropping HARQ-ACK information issue, the deferring HARQ-ACK for SPS PDSCH was discussed and supported by RAN1 [1]. If spsHARQdeferral is configured in a special SPS configuration by RRC of SPS-config, the HARQ-ACK information can be deferred until UE finds an available PUCCH/PUSCH, the detail procedure can be seen as section 9.2.5.4 [2].

SPS-Config The IE SPS-Config is used to configure downlink semi-persistent transmission. Multiple Downlink SPS configurations may be configured in one BWP of a serving cell. SPS-Config information element SPS-Config ::= SEQUENCE { <uncorrelated; Text is Omitted> pdsch-AggregationFactor-r16 ENUMERATED {n1, n2, n4, n8 } PTIONAL -- Need S ]], [[ sps-HARQ-Deferral-r17 INTEGER (1..32) OPTIONAL, -- Need R n1PUCCH-AN-PUCCHsSCell-r17 PUCCH-ResourceId OPTIONAL, -- Need R periodicityExt-r17 INTEGER (1..40960) OPTIONAL, -- Need R nrofHARQ-Processes-v1710 INTEGER(9..32) PTIONAL, -- Need R harq-ProcID-Offset-v1700 INTEGER (16..31) OPTIONAL -- Need R ]] }

Since MBS also support SPS PDSCH transmission, and the HARQ-ACK information of multicast may be dropped when PUCCH resource has overlapping with downlink symbols. To improve multicast transmission efficiency, it is suggested that spsHARQdeferral can be configured in SPS configuration for multicast. It is noted that there is no any specification work because the multicast and unicast use same RRC parameter SPS-config to configure the SPS PDSCH.

Moreover, the current specification only supports first HARQ-ACK reporting mode (ACK/NACK based HARQ-ACK feedback) for deferring HARQ-ACK for unicast SPS PDSCH. For HARQ-ACK for multicast SPS, to simply complexity of UE/gNB, we also suggest that deferring HARQ-ACK for SPS PDSCH can be supported only for first HARQ-ACK reporting mode. If sps-HARQ-Deferral applies to second HARQ-ACK feedback, it will trigger additional discussions. For example, if UE generates all ‘ACK’ for SPS PDSCH, the UE doesn’t require HARQ CB feedback. How to handle this case for UE is a problem.

Proposal 1：The IE spsHARQdeferral can be configured in SPS configuration for multicast if first HARQ-ACK reporting mode is applied.

Proposal 2：UE is not expected to be configured spsHARQdeferral in SPS configuration for multicast if second HARQ-ACK reporting mode is applied.

In RAN1#110be meeting [4], this issue was discussed during email discussion, most companies believed that deferral is already for multicast SPS in the current specs and no new CR is needed, but there was no conclusion on it. It is suggested RAN1 to discuss the deferral issue of multicast SPS and down select following two options

- Option  1：Conclusion in RAN1 is needed to clarify that deferral procedure of multicast SPS is only applied for  first HARQ-ACK reporting mode (ACK/NACK based HARQ-ACK feedback)

- Option  2：CR on MBS is needed to clarify that deferral procedure of multicast SPS is only applied for first HARQ-ACK reporting mode (ACK/NACK based HARQ-ACK feedback)

Proposal 3：It is proposed that RAN1 to discuss the deferral issue of multicast SPS and down select following two options

- Option  1：Conclusion in RAN1 is needed to clarify that deferral procedure of multicast SPS is only applied for  first HARQ-ACK reporting mode (ACK/NACK based HARQ-ACK feedback)

- Option  2：CR on MBS is needed to clarify that deferral procedure of  multicast SPS is only applied for  first HARQ-ACK reporting mode (ACK/NACK based HARQ-ACK feedback)

The draft CR [3] is attached if option 2 is adopted by RAN1.

- Conclusion

In this document, we discussed the issue that MBS supporting deferring HARQ-ACK for SPS PDSCH was discussed. The proposal is given as following

Proposal 1：spsHARQdeferral can be configured in SPS configuration for multicast if first HARQ-ACK reporting mode is applied.

Proposal 2：UE is not expected to be configured spsHARQdeferral in SPS configuration for multicast if second HARQ-ACK reporting mode is applied.

Proposal 3：It is proposed that RAN1 to discuss the deferral issue of multicast SPS and down select following two options

- Option  1：Conclusion in RAN1 is needed to clarify that deferral procedure of multicast SPS is only applied for  first HARQ-ACK reporting mode (ACK/NACK based HARQ-ACK feedback)

- Option  2：CR is needed to  clarify that deferral procedure of  multicast SPS is only applied for  first HARQ-ACK reporting mode (ACK/NACK based HARQ-ACK feedback)

The draft CR [3] is attached if option 2 is adopted by RAN1.

- References

RP-202854, “Revised WID: Enhanced Industrial Internet of Things (IoT) and ultra-reliable and low latency communication (URLLC) support for NR”, Nokia, Nokia Shanghai Bell

3GPP TS 38.213 V17.3.0:” NR; Physical layer procedures for control”

R1-2211149 “Draft CR on MBS supporting deferring HARQ-ACK for SPS PDSCH”, CATT, RAN1#111, Toulouse, France, November 14th – 18th, 2022

R1-2210374, “FLS#3 on the HARQ-ACK related issues for Rel-17 NR MBS”,HUAWEI, RAN1#110bis-e, October 10th – 19th, 2022.