3GPP TSG RAN WG1 #111			     R1-2211397

Toulouse, France, November 14th – November 18th, 2022

Source:	Intel Corporation

Title:	Evaluation of NR Duplex Evolution

Agenda item:	9.3.1

Document for:	Discussion and Decision

# 1 Introduction

In 3GPP TSG RAN Meeting #94 [1], a new SID related to evolution of NR duplex operation was approved. As part of the objectives of this study item (SI), the following objectives have been identified:

In this study, the followings are assumed: - Duplex enhancement at the gNB side - Half duplex operation at the UE side - No restriction on frequency ranges The detailed objectives are as follows: - Identify applicable and relevant deployment scenarios (RAN1). - Develop evaluation methodology for duplex enhancement (RAN1). - Study the subband non-overlapping full duplex and potential enhancements on dynamic/flexible TDD (RAN1, RAN4). - Identify possible schemes and evaluate their feasibility and performances (RAN1). - Study inter-gNB and inter-UE CLI handling and identify solutions to manage them (RAN1). - Consider intra-subband CLI and inter-subband CLI in case of the subband non-overlapping full duplex. - Study the performance of the identified schemes as well as the impact on legacy operation assuming their co-existence in co-channel and adjacent channels (RAN1). - Study the feasibility of and impact on RF requirements considering adjacent-channel co-existence with the legacy operation (RAN4). - Study the feasibility of and impact on RF requirements considering the self-interference, the inter-subband CLI, and the inter-operator CLI at gNB and the inter-subband CLI and inter-operator CLI at UE (RAN4). - Note: RAN4 should be involved early to provide necessary information to RAN1 as needed and to study the feasibility aspects due to high impact in antenna/RF and algorithm design, which include antenna isolation, TX IM suppression in the RX part, filtering and digital interference suppression. - Summarize the regulatory aspects that have to be considered for deploying the identified duplex enhancements in TDD unpaired spectrum (RAN4).

In this context, in prior RAN1 meetings [2-4] the agreements captured at the end of this document within the Appendix were made.

In this contribution, remaining issues related to the evaluation methodologies are discussed, and some calibration and initial evaluation results are provided. Our views related to other aspects of Rel.18 NR duplex evolution are provided in our companion contributions [5-6].

# Remaining Details for the Deployment Scenarios

During last RAN1 meeting [4], the last details related to the scenario to consider for evaluations of deployment case 3-2 (i.e., 2-layer case with single operator using single carrier, where one layer uses legacy TDD operation while the other layer use SBFD operation with the same SBFD subband configuration) and deployment case 4 (e.g., two operators each using one of two adjacent carriers). In this case one operator uses legacy TDD operation while the other operator uses SBFD operation with the same SBFD subband configuration) have been agreed. However, for both cases it was left for further study whether for the layer 2, which could be composed by either indoor office or indoor factory layout, how may layouts should be dropped in the whole network. Considering simulation complexity, and running time, while still simulating a realistic scenario, it may be preferrable to drop a single indoor office or indoor factor layout for the whole network.

Proposal 1:

- For both SBFD deployment case 3-2 and 4, a single indoor office/factory is dropped for the entire network.

During last RAN1 meeting [4], the details related to the UE clustering distribution have been discussed and the following agreement was reached:

Agreement For UE clustering distribution of Urban Macro and Dense Urban Macro layer, - R =[25] m, Dmacro-to-cluster = 35m+R, Dinter-cluster = 2R m. Note: the UE cluster is totally confined within the macro cell geographical area (i.e. a cluster cannot be partially overlap with adjacent cell area). For calibration purposes, assume clustering with R=25

In the agreement above the exact value to use during evaluations for the radius of a UE clustering (i.e., R) was left for further study, given that it was argued the R= 25 m may not allow to sufficiently emphasize the inter-UE cross-link interference (CLI). In this matter, Figure 1 shows the statistical distribution of the distance among UEs within a cluster composed per prior agreements by 10 UEs as function of the radius of the UE clustering.

Figure 1 – CDF of the inter-UE distance as function of the radius of the UE clustering.

Figure 1 emphasizes that as the radius of the UE clustering is reduced, the diversity in terms of inter-UE distances greatly decreases as we tend to converge toward a layout where the UEs are artificially located at similar distances between each other. While reducing the radius of the UE clustering may certainly allow to create a layout where the inter-UE CLI may be highly emphasized due to the close proximity of all UEs belonging to the same cluster, this may on the other hand create an unrealistic deployment especially for outdoor deployments.

Observation 1:

- By reducing the radius of the cluster, the diversity in terms of distances among UEs within a cluster is reduced up to the point where intra-UE distances are artificially fixed creating an unrealistic deployment.

However, it is important to note that for R = 20 m, ~5% of the UEs may have an inter-UE distance less than 5 meters, while a minimum UE-to-UE distance is fixed to 1 meter, which should sufficiently guarantee that the statistical evaluations would sufficiently capture the inter-UE CLI issue for the 5th percentile.

Proposal 2:

- For UE clustering distribution of urban macro and dense urban macro layer, R = 20 m, Dmacro-to-cluster = 55 m and Dinter- cluster = 40 m.

During the last RAN1 meeting [4], it has been agreed that no penetration loss would be modelled for the indoor office scenario, while for urban macro and dense urban macro a 20% high loss : 80% low loss model would be used:

Agreement Adopt the following gNB-UE O2I building penetration loss model: - Indoor office: penetration loss is not modelled. - Percentage of high loss and low loss building type for Urban Macro / Dense Urban [refer to table 5B of ITU M.2412]: - 80% low-loss model - 20% high-loss model - Note: The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; otherwise the building type is high loss [refer to section 5.3.3 of ITU M.2412]. - FFS for 2-layer Scenario B

However, it was left for further study what penetration loss model should be used for the 2-layer scenario B. In this case, the assumptions for single layer could be reused, and while layer 2 (indoor) no penetration loss is modelled, for layer 1 (urban macro) either of the following two options could be considered:

    - Option 1:

      - 100% Outdoor without car penetration loss: 3km/h

    - Option 2:

      - 20% outdoor in cars: speed with 30km/h, height with 1.5m

      - 80% indoor in houses: speed with 3km/h, height with 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8)

Proposal 3:

- For the 2-layer Scenario B (i.e., HetNet with Urban Macro and Indoor), in the layer 1 either of the following options could be considered:

  - Option 1: 100% Outdoor without car penetration loss: 3km/h

  - Option 2:

    - 20% outdoor in cars: speed with 30km/h, height with 1.5 m;

    - 80% indoor in houses: speed with 3km/h, height with 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8).

For Layer 2, no penetration loss is considered.

# 3 Remaining Details for the Antenna Configurations

In prior RAN1 meetings [2-4], the antenna configurations to be used during the evaluations of both SBFD and dynamic TDD have been agreed. However, one last remaining detail to be sorted out is regarding the specific assumptions in terms of mechanical and electrical tilt. In this matter, the assumptions used during the IMT-2020 self-evaluations could be reused [7]. However, during last meeting [4] companies pointed out that the zenith angles assumed during those evaluations (i.e., θj = {5*pi/8, 7*pi/8}) imply a severe down tilt from the horizon of 22.5 and 67.5 degrees, respectively, which may cause cell edges to not be covered. In order to address this, the same zenith angle used in TR 38.901 [8] for UMa has been proposed. With that said, the assumptions summarized in following table could be considered.

|  | Urban Macro(FR1) | Dense Urban Macro Layer (FR2) |
| --- | --- | --- |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |

Proposal 4:

- For SLS calibration, RAN1 reuses the antenna configurations in terms of mechanical and electric tilt from the IMT-2020 self-evaluations with zenith angle equal to θj = pi*102/180.

# Remaining Details Related to the Performance Metrics

During prior RAN1 meeting [4], it was agreed to conduct an SLS calibration campaign using the following scenarios and metrics:

Agreement RAN1 to conduct a SLS calibration for evaluation of SBFD operation. - The calibration focuses on the following scenarios of SBFD deployment case 1 - FR1: Urban Macro - FFS: Indoor office - FR2: Dense Urban Macro layer - Regarding metrics used for SLS calibration, consider the following: - gNB-UE coupling loss - Inter-gNB coupling loss - Inter-UE coupling loss - Optional: DL SINR for legacy TDD/ DL SINR in DL-only slots for SBFD - Optional: DL SINR in SBFD slots - Optional: UL SINR for legacy TDD/ UL SINR in UL-only slots for SBFD - Optional: UL SINR in SBFD slots - FFS: the detailed definitions of the metrics listed above

With regard to the first FFS related to scenarios to be considered, we believe that a single scenario per frequency band may be sufficient, and even though an indoor office scenario may be more representative than others when looking at CLI, it is preferable to avoid adding further scenarios for calibration.

Proposal 5:

- For SLS calibration, indoor office scenario is not considered.

Regarding coupling loss, it is important to clarify how the statistics are generated. In this matter, similarly to what the FL has proposed during prior RAN1 meeting, the following could be considered.

- For CDF of gNB-UE coupling loss, only the coupling losses between each UE and its serving cell are collected for CDF statistics. Statistics of gNB-UE non-serving links may be reported separately. Further, for gNB-UE links, DL reception is assumed.

- For CDF of gNB-gNB coupling loss:

  - For one SLS drop, generate channels among gNBs, calculate and collect the coupling loss for each gNB pair;

  - The two gNBs in each gNB pair should be from different sites;

  - Run multiple SLS drops and plot the CDF using the collected coupling losses.

- For CDF of UE-UE coupling loss:

  - For one SLS drop, drop UEs in the network and generate channels among UEs, calculate and collect the coupling loss for each UE pair;

  - If the 2D distance between two UEs in a UE pair is larger than 40m (a.k.a, no inter-cluster pairs are considered), the UE pair is not considered for statistic;

  - Run multiple SLS drops and plot the CDF using the collected coupling losses.

Proposal 6:

- For SLS calibration,

  - For CDF of gNB-UE coupling loss, only the coupling losses between each UE and its serving cell are collected for CDF statistics.

  - For CDF of gNB-gNB coupling loss:

    - For one SLS drop, generate channels among gNBs, calculate and collect the coupling loss for each gNB pair;

    - The two gNBs in each gNB pair should be from different sites;

    - Run multiple SLS drops and plot the CDF using the collected coupling losses.

  - For CDF of UE-UE coupling loss:

    - For one SLS drop, drop UEs in the network and generate channels among UEs, calculate and collect the coupling loss for each UE pair;

    - If the 2D distance between two UEs in a UE pair is larger than 40 m (a.k.a, no inter-cluster pairs are considered), the UE pair is not considered for statistic;

    - Run multiple SLS drops and plot the CDF using the collected coupling losses.

# 5 Other Assumptions

In prior RAN1 meeting [2], in order to make sure fair comparison is performed between static TDD and SBFD, the following agreement was made regarding their specific time domain allocations:

Agreement For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives: - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison.

However, while during last RAN1 meeting [4] there was general consensus that Alt.3 should be deprioritized, for this configuration the calculation of the percentage of channel BW devoted to UL in the SBFD Frame #2 seems to be erroneous, since for SBFD the percentage of channel BW devoted to UL is 1/4 of the total BW, which corresponds to 25% and not 20% as per the above agreement. Therefore, the above agreement should be revised as follows:

- Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20% 25%] of the channel bandwidth.

Proposal 7:

- For the agreement related to the time domain allocations for SBFD, the following update should be applied to Alt 3, while this is deprioritized:

  - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

    - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

    - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20% 25%] of the channel bandwidth.

During last RAN1 meeting [4], for the comparison between baseline legacy TDD network and SBFD deployment case 3-2 or SBFD deployment case 4, while companies converged that the same alternatives defined for deployment case 1 and listed in the agreement above could be considered, no agreement was made. Given that Alt.1, Alt.2, and Alt. 4 may cover a range of different SBFD configurations including configurations which may be advantageous in highlighting the SBFD gain for UL (e.g., in Alt.2 at the very least same UL resources may be used in SBFD compared to legacy TDD configuration {DDDSU}) or may allow same UL/DL resource ratio as the TDD counterpart (e.g., Alt. 4), it may be beneficial to support all of the them, and leave up to companies to decide which one to use upon reporting.

Proposal 8:

- For layer 2 in SBFD deployment case 3-2 and for operator #2 for SBFD deployment case 4, the SBFD frame structures in Alt2(XXXXU)/Alt1(DXXXU)/Alt4(XXXXX) agreed for Deployment Case 1 can be reused.

  - Companies to report which SBFD frame structure is used.

# 6 Coverage Enhancement Study and LLS Simulation Assumptions

During prior RAN1 meeting [2], the group has agreed on the performance metrics to consider during both SBFD and flexible/dynamic TDD evaluations, and also agreed that a metric to capture coverage/capacity would be necessary since as described in our companion contribution [6] SBFD operation can help improve coverage/capacity for uplink transmission and reduce latency for both UL and DL transmission, by allocating the resource for PUSCH or PUCCH transmission in the uplink sub-band within the SBFD symbol, which is more critical in case of TDD UL/DL configuration with DL heavy pattern.

From a link-level perspective, simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI phase can be reused to investigate the performance gain for coverage that can be realized by SBFD. More specifically, the following performance metrics can be considered as starting points for coverage analysis for SBFD:

- MCL = Total transmit power – Receiver sensitivity + gNB antenna gain (component 2).

- MIL = Total transmit power – Receiver sensitivity – Tx loss – Rx loss + gNB antenna gain (component 2 + 3 + 4) + UE antenna gain.

- MPL = MIL – Shadow fading margin + BS selection/macro-diversity gain – Penetration margin + Other gains.

Further, for link-level simulations, assumptions for PUSCH and PUCCH as listed in [9] can be considered. To reduce the simulation workload, a subset of simulation assumptions can be selected for coverage analysis for SBFD operation. For instance, target data rate for PUSCH transmission under SBFD operation can be 10 Mbps for DL and 1 Mbps for UL at 4 GHz carrier frequency in FR-1 and  25 Mbps for DL and 5 Mbps for UL at 28 GHz for FR-2. As for the specific simulation assumptions, these are summarized in the Table 1 and Table 2 in the Appendix I, and can be considered as starting points for SBFD performance evaluation for FR1 and FR2, respectively.

Note that for link-level simulation for SBFD coverage analysis, self-interference modelling needs to be considered. With practical self-interference modelling for SBFD operation, performance gains for coverage may also depend on the relative frequency location of DL/UL sub-band and the PUSCH transmission.

Proposal 9:

- In order to investigate the performance gain for coverage realized by SBFD, MPL should be also included in the list of performance metrics.

  - As Rel. 17 NR coverage enhancements studies, RAN1 should focus on link-budget analysis to determine MPL, which already includes MCL/MIL as intermediate steps, with use of LLS evaluations to derive the required SNRs and possibly SLS evaluations to estimate Tx antenna gain correction factor  (up to companies).

  - Simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI can be considered as starting points for evaluation of coverage performance for SBFD operation.

  - Consider Table 1 and Table 2 in the Appendix I for SBFD performance evaluation for FR1 and FR2, respectively.

  - PUSCH is used as a target channel to meet the following targeted data rates:

    - For FR1: DL 10Mbps, UL 1Mbps

    - For FR2: UL 25Mbps, UL: 5Mbps

  - Discuss further on self-interference modelling for link-level simulations and coverage analysis for SBFD.

# 7 Interference Modelling

During last RAN1 meeting [4], a model to capture inter-site gNB-gNB co-channel inter-subband CLI has been introduced both the case when only large scale fading is considered as well as for the case when both large scale fading, and small scale fading are modelled. For this last case, the following was agreed:

Agreement For SLS in RAN1, if both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the inter-site gNB-gNB co-channel inter-subband CLI signal across all Rx chains at UL RB n at victim gNB can be modeled as IInter-Site-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLInWyn is the first part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at UL RB n, caused by power leakage at aggressor gNB, - HCLIn is the NR×NT channel matrix between aggressor gNB and victim gNB at UL RB n, the beamforming of the aggressor gNB and the victim gNB can be taken into account by HCLIn, - yn=y0n,y1n,…,yNT-1nT is the unwanted emission across all Tx chains at UL RB n at aggressor gNB, - NT is the number of Tx chains at aggressor gNB, - ykn~N0,σy,n2, k=0,1,…,NT-1, is modelled as white Gaussian noise, - σy,n2=Ptxper-RB*Nused-DL-RBACLRBS* 1NDLRB is the total leakage power at UL RB n at aggressor gNB, - Ptxper-RB is the DL power transmitted across all Tx chains at one DL RB at aggressor gNB, Ptxper-RB=Ptxmax/NDLRB, - Nused-DL-RB is the number of DL RBs scheduled for DL transmission by aggressor gNB, - NDLRB is the total number of DL RBs in the DL subbands - W is the NT×NT normalized identity matrix with unit norm, WF=1, - FFS whether W can be other values and corresponding conditions - FFS for Iselectivity - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs are used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Send LS to RAN4 to confirm RAN1’s understanding.

For what concerns how to model Iselectivity, which should capture the inter-site gNB-gNB co-channel inter-subband CLI across all RX chains at UL frequency unit n caused by the receiver selectivity at the victim gNB, this could be modelled using ACS as follows and as discussed during last RAN1 meeting [4]:

Iselectivity=m∈Used DL RBsHCLImWmsm(ACSBS*NDLRB)12

where

    - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm,

    - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1,

    - sm is the symbol transmitted at DL RB m at aggressor gNB.

Proposal 10:

- When modelling inter-site gNB-gNB co-channel inter-subband CLI in case for the case when both large scale fading, and small scale fading are considered

Iselectivity=m∈Used DL RBsHCLImWmsm(ACSBS*NDLRB)12

where

    - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm,

    - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1,

    - sm is the symbol transmitted at DL RB m at aggressor gNB.

Among other CLI that an SBFD system may suffer from, RAN1 has also identified the co-site inter-sector co-channel inter-subband CLI. In this case, similar method as gNB self-interference modelling could be used. While in the absence of further RAN4 input, for calibration purposes the interference suppression capability for co-site inter-sector co-channel inter-subband CLI could be assumed to be the same as the RSI value for self-interference, when performing evaluations companies should report the assumed values based on their own assumptions.

Proposal 11:

- For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling.  In the absence of further RAN4 input, at least for calibration purpose, assume the interference suppression capability for co-site inter-sector co-channel inter-subband CLI is the same as the RSI value for self-interference. Companies shall report the parameters assumed and values used in the simulations.

Another CLI that an SBFD system may suffer from is the UE-UE co-channel inter-subband CLI. In this case, given that the in-band emission model defined in TS 38.101-1 and TS 38.101-2 has been already extensively used for purpose to model inter-subband CLI, this could be reused in the context of SFBD as a starting point.

Proposal 12:

- For SLS in RAN1, regarding UE-UE co-channel inter-subband CLI modelling, take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point for TX model.

Finally, when considering an SFBD system the adjacent-channel CLI may need to be modelled as well. In this matter, this component for both inter-site gNB-gNB and UE-UE could be modelled similarly as the  inter-site gNB-gNB co-channel inter-subband CLI, where again ACLR is used to model TX leakages and ACS for receiver impairments.

Proposal 13:

- For inter-site gNB-gNB adjacent-channel CLI modeling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modeling with gNB ACLR for TX leakage and gNB ACS for receiver impairment

Proposal 14:

- For UE-UE adjacent-channel CLI modelling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modeling with UE ACLR for TX and UE ACS for RX.

# 8 Initial Calibration Evaluations

This section provides some calibration evaluations based on the assumptions summarized in Appendix I within Table 1. In particular, this section provides the gNB-UE, gNB-gNB and UE-UE coupling loss for both the Urban Macro scenario for FR1 and the dense urban macro scenario for FR2.

Figure 2 – Coupling loss metrics for calibration for FR1 and FR2-1.

# 9 Initial Evaluations

## Preliminary Coverage Analysis for SBFD Operation

To investigate the coverage performance for SBFD operation, a link-level simulation’s campaign has been conducted for PUSCH repetitions. The simulation assumptions used are those summarized in the Appendix II in Table 2 for FR1 and Table 3 for FR2, respectively. Furthermore, two scenarios, as illustrated in Figure 3, are considered:

- Scenario a): DDDSU for TDD configuration without SBFD operation,

- Scenario b): DNNNU for TDD configuration with SBFD operation, where N indicates as SBFD slot.

Figure 3 - TDD configurations with SBFD for PUSCH repetitions

Figure 4 and Figure 5 illustrate preliminary link-level simulation results for PUSCH repetition with SBFD operation in FR1 and FR2, respectively. From these figures, it can be observed that link-level performance for PUSCH can be improved by increasing the number of repetitions. More specifically, 2~3dB performance gain can be observed by doubling the repetition levels for the PUSCH transmission, depending on UE moving speed and targeted data rate.

Note that in the simulations, leakage from DL sub-band to UL sub-band for SBFD operation are not considered. As mentioned above, after RAN1 will be obtaining self-interference modelling from RAN4, further investigation would be needed to analyze the coverage performance for SBFD operation.

Figure 4 – Link-level simulation results for PUSCH repetition with SBFD operation in FR1

Figure 5 – Link-level simulation results for PUSCH repetition with SBFD operation in FR2

Observation 2:

- Without self-interference modelling, 2~3dB link-level performance gain can be observed by doubling the repetition levels for PUSCH transmission in both FR1 and FR2.

## Preliminary SINR Analysis for Dynamic/Flexible TDD

In order to investigate the CLI impact on system-level perspective when dynamic/flexible TDD is used, some initial simulation campaign has been conducted, which focuses on geometry-only analysis. Four different scenarios are considered:

- Indoor office deployment in FR-1.

- Urban macro deployment in FR-1.

- Indoor office deployment in FR-2.

- Dense urban macro deployment in FR-2.

The simulation assumptions used are those summarized in the Appendix III in Table 4 for indoor office deployment in FR-1, Table 5 for urban macro deployment in FR1, in Table 7 for indoor office deployment in FR-2, Table 8 for dense urban macro deployment in FR2, respectively.

For each deployment, the following figures show both the CDF of the DL and UL SINR, and for each CDF it is also shown the case when a flexible/dynamic TDD is where through which half of the cells operate in UL and the other half in DL.

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 6 - CDF of DL and UL SINR in an indoor deployment for FR-1

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 7 - CDF of DL and UL SINR in an urban macro deployment for FR-1

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 8 - CDF of DL and UL SINR in an indoor deployment for FR-2

- DL SINR Geometry                                                                            b) UL SINR Geometry

Figure 9 - CDF of DL and UL SINR in an dense urban macro deployment for FR-2

Figure 6-9 highlight when dynamic/flexible TDD operation is used, the UL performance degrades substantially, especially in FR-1, due to gNB-to-gNB cross-link interference, while the DL performance may only slightly increase due to the lower TX power used by the UEs. Thus, this motivates considerations of possible enhancements to address gNB-to-gNB CLI as described in our companion document [6].

Observation 3:

  - In absence of any coordination, due to gNB-to-gNB CLI the UL performance may substantially degrade in both FR-1 and FR-2 irrespective of the scenario when dynamic/flexible TDD operations is supported.

# 10 Conclusions

In this contribution, we presented our views the evaluation methodologies to adopt to study the NR duplex evolution, and made the following observations and proposals:

Proposal 1:

- For both SBFD deployment case 3-2 and 4, a single indoor office/factory is dropped for the entire network.

Observation 1:

- By reducing the radius of the cluster, the diversity in terms of distances among UEs within a cluster is reduced up to the point where intra-UE distances are artificially fixed creating an unrealistic deployment.

Proposal 2:

- For UE clustering distribution of urban macro and dense urban macro layer, R = 20 m, Dmacro-to-cluster = 55 m and Dinter- cluster = 40 m.

Proposal 3:

- For the 2-layer Scenario B (i.e., HetNet with Urban Macro and Indoor), in the layer 1 either of the following options could be considered:

  - Option 1: 100% Outdoor without car penetration loss: 3km/h

  - Option 2:

    - 20% outdoor in cars: speed with 30km/h, height with 1.5 m;

    - 80% indoor in houses: speed with 3km/h, height with 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8).

For Layer 2, no penetration loss is considered.

Proposal 4:

- For SLS calibration, RAN1 reuses the antenna configurations in terms of mechanical and electric tilt from the IMT-2020 self-evaluations with zenith angle equal to θj = pi*102/180.

Proposal 5:

- For SLS calibration, Indoor office scenario is not considered.

Proposal 6:

- For SLS calibration,

  - For CDF of gNB-UE coupling loss, only the coupling losses between each UE and its serving cell are collected for CDF statistics.

  - For CDF of gNB-gNB coupling loss:

    - For one SLS drop, generate channels among gNBs, calculate and collect the coupling loss for each gNB pair;

    - The two gNBs in each gNB pair should be from different sites;

    - Run multiple SLS drops and plot the CDF using the collected coupling losses.

  - For CDF of UE-UE coupling loss:

    - For one SLS drop, drop UEs in the network and generate channels among UEs, calculate and collect the coupling loss for each UE pair;

    - If the 2D distance between two UEs in a UE pair is larger than 40 m (a.k.a, no inter-cluster pairs are considered), the UE pair is not considered for statistic;

    - Run multiple SLS drops and plot the CDF using the collected coupling losses.

Proposal 7:

- For the agreement related to the time domain allocations for SBFD, the following update should be applied to Alt 3, while this is deprioritized:

  - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD):

    - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U]

    - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20% 25%] of the channel bandwidth.

Proposal 8:

- For layer 2 in SBFD deployment case 3-2 and for operator #2 for SBFD deployment case 4, the SBFD frame structures in Alt2(XXXXU)/Alt1(DXXXU)/Alt4(XXXXX) agreed for Deployment Case 1 can be reused.

  - Companies to report which SBFD frame structure is used.

Proposal 9:

- In order to investigate the performance gain for coverage realized by SBFD, MPL should be also included in the list of performance metrics.

  - As Rel. 17 NR coverage enhancements studies, RAN1 should focus on link-budget analysis to determine MPL, which already includes MCL/MIL as intermediate steps, with use of LLS evaluations to derive the required SNRs and possibly SLS evaluations to estimate Tx antenna gain correction factor  (up to companies).

  - Simulation assumptions and evaluation methodologies as agreed during Rel-17 NR coverage enhancement SI can be considered as starting points for evaluation of coverage performance for SBFD operation.

  - Consider Table 1 and Table 2 in the Appendix I for SBFD performance evaluation for FR1 and FR2, respectively.

  - PUSCH is used as a target channel to meet the following targeted data rates:

    - For FR1: DL 10Mbps, UL 1Mbps

    - For FR2: UL 25Mbps, UL: 5Mbps

  - Discuss further on self-interference modelling for link-level simulations and coverage analysis for SBFD.

Proposal 10:

- When modelling inter-site gNB-gNB co-channel inter-subband CLI in case for the case when both large scale fading, and small scale fading are considered

Iselectivity=m∈Used DL RBsHCLImWmsm(ACSBS*NDLRB)12

where

    - HCLIm is the NR×NT channel matrix between aggressor gNB and victim gNB at DL RB m, the analog beams of the aggressor gNB and the victim gNB can be taken into account by HCLIm,

    - Wm is the digital precoder at DL RB m at aggressor gNB, WmF=1,

    - sm is the symbol transmitted at DL RB m at aggressor gNB.

Proposal 11:

- For SLS in RAN1, for co-site inter-sector co-channel inter-subband CLI modelling, reuse similar method as gNB self-interference modelling.  In the absence of further RAN4 input, at least for calibration purpose, assume the interference suppression capability for co-site inter-sector co-channel inter-subband CLI is the same as the RSI value for self-interference. Companies shall report the parameters assumed and values used in the simulations.

Proposal 12:

- For SLS in RAN1, regarding UE-UE co-channel inter-subband CLI modelling, take in-band emission (IBE) defined in TS38.101-1 and TS38.101-2 as starting point for TX model.

Proposal 13:

- For inter-site gNB-gNB adjacent-channel CLI modeling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modeling with gNB ACLR for TX leakage and gNB ACS for Receiver impairment

Proposal 14:

- For UE-UE adjacent-channel CLI modelling, reuse similar method as inter-site gNB-gNB co-channel inter-subband CLI modeling with UE ACLR for TX and UE ACS for RX.

Observation 2:

- Without self-interference modelling, 2~3dB link-level performance gain can be observed by doubling the repetition levels for PUSCH transmission in both FR1 and FR2.

Observation 3:

  - In absence of any coordination, due to gNB-to-gNB CLI the UL performance may substantially degrade in both FR-1 and FR-2 irrespective of the scenario when dynamic/flexible TDD operations is supported.

# References

- RP-213591, “New SI: Study on evolution of NR duplex operation”, CMCC, December 2021.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #109-e, May 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 #110, Toulouse France, Aug 2022.

- RAN1 Chairman’s note, 3GPP TSG RAN WG1 e-Meeting #110b-e, October 2022.

- R1-2211398, “ On SBFD in NR systems”, Intel Corporation, November 2022.

- R1-2211399, “ On support of dynamic/flexible TDD in NR systems”, Intel Corporation, November 2022.

- RP-180524, “Summary of calibration results for IMT-2020 self evaluation”, Huawei, March 2018.

- TR 38.901, “Study on channel model for frequencies from 0.5 to 100 GHz”, December 2019.

- TR 38.830, “Study on NR coverage enhancements”, December 2020.

# Appendix I

Table 1  Simulation assumptions for SLS calibration for SBFD evaluation

|  | Urban Macro (FR1) | Dense Urban Macro Layer (FR2-1) |
| --- | --- | --- |
| Carrier frequency | 4 GHz | 30GHz |
| System bandwidth | 100MHz | 100MHz |
| Numerology | 14 OFDM symbol slot SCS = 30kHz | 14 OFDM symbol slot SCS = 120kHz |
| BS transmit power for SBFD | - Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. - 53 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD | - Assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. - 40 dBm for 100MHz is assume for maximum BS transmit power for legacy TDD |
| UE Tx power | 23dBm | 23 dBm. EIRP should not exceed 43 dBm |
| Macro Layout | Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around |  |
| Inter-BS (2D) distance | 500m | 200m |
| Minimum BS-UE (2D) distance | 35m | 35m |
| Minimum UE-UE (2D) distance | 1m | 1m |
| BS antenna height | 25m | 25m |
| UE distribution | UE Clustering: | Clustering |
| UE number per macro TRP (per direction) (M) | 20 | 20 |
| UE cluster number per macro cell (X) | 2 | 2 |
| UE outdoor/indoor proportion | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h | 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h |
| Indoor UE height (m) | 1.5m | 1.5m |
| Outdoor UE height (m) | 1.5m | 1.5m |
| Radius of cluster (R) | [25]m | [25]m |
| Minimum distance between macro TRP to UE cluster center (Dmacro-to-cluster) | [60]m | [60]m |
| Minimum distance between two UE cluster centers (Dinter-cluster) | [50]m | [50]m |
| gNB-UE Channel model | Macro-to-UE: UMa in TR 38.901 gNB-UE O2I penetration loss: 80% low-loss model, 20% high-loss model |  |
| gNB-gNB Channel model (large-scale) | Macro-to-Macro: UMa in TR 38.901 (hUE =25m) LOS probability: If the 2D distance between two Macro gNBs are less than or equal to the ISD, set the LOS probability to 0.75; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. |  |
| gNB-gNB Channel model (small-scale) | Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 |  |
| UE-UE Channel model (large-scale) | Option 1: UE-to-UE: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 | UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802 |
| UE-UE Channel model (small-scale) | Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. | UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. |
| BS antenna array configuration for SBFD | SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,1,1). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ | SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: M,N,P,Mg,Ng= (4,8,2,2,2). - Number of TxRUs: same as legacy TDD - dH,dV = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ |
| BS antenna radiation pattern | reuse Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) | reuse Table 9 in Report ITU-R M.2412 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) |
| UE antenna configuration | - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization | 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° |
| Rx/Tx Reciprocity For coupling loss collection | DL reception is assumed | - |
| UE antenna radiation pattern | Omni-directional with 0 dBi element gain | reuse Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802) |
| BS receiver noise figure | 5dB | 7 dB |
| UE receiver noise figure | 9 dB | 13 dB |
| Open loop power control parameters | P0= -80 dBm, alpha = 0.8 | P0= -86 dBm, alpha = 0.9 |
| Handover margin (dB) | 3 dB | 3 dB |
| UE attachment | Based on RSRP from port 0 | Based on RSRP from port 0. The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. |
| Polarized antenna model | Model-1 in clause 7.3.2 in TR 38.901 | Model-1 in clause 7.3.2 in TR 38.901 |
| Mechanic tilt | 90° in GCS (pointing to horizontal direction) | 90° in GCS (pointing to horizontal direction) |
| Electronic tilt | (According to Zenith angle in "Beam set at TRxP") | (According to Zenith angle in "Beam set at TRxP") |
| Beam set at TRxP (Constraints for the range of selective analog beams per TRxP) | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = 0 Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 | For direction of TRxP analog beam steering (in LCS): Azimuth angle φi = {-5*pi/16, -3*pi/16, -pi/16, pi/16, 3*pi/16, 5*pi/16} Zenith angle θj = pi*102/180 NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |
| Beam set at UE (Constraints for the range of selective analog beams for UE) | - | For direction of UE analog beam steering (in LCS): Azimuth angle φi = {-3*pi/8, -pi/8, pi/8, 3*pi/8}; Zenith angle θj = {pi/4, 3*pi/4}; NOTE: (azimuth, zenith)=(0, pi/2) is the direction perpendicular to the array. Precoder for beam at (φi, θj) is given by equation 1 in Appendix 1 (2D DFT beam) in RP-180524 |

# Appendix II

Table 2. Simulation assumption for PUSCH in FR1

| Parameters | Values |
| --- | --- |
| Physical channel | PUSCH |
| Carrier frequency | Rural: 4 GHz (TDD) – NLOS |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| Bandwidth | 4 GHz – 100MHz |
| Waveform | DFT-s-OFDM |
| Subcarrier spacing for PUCCH | 30kHz for TDD |
| MCS and TBS | QPSK eMBB: Rural, TDD: MCS 3 and TBS 288 bits for 2 DMRS symbols MCS 4 and TBS 304 bits for 4 DMRS symbols |
| Resource allocation | eMBB: Rural: 14 symbols, 4 PRB |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 4Rx for 4GHz |
| Propagation channel | Rural: TDL-C 300ns |
| UE velocity | Rural: 3km/h for indoor, 120km/h for outdoor |
| Timing offset | 0 |
| Frequency offset | 0 |
| DMRS symbols | eMBB: 2 symbols for 3km/h, 4 symbols for 120km/h |
| Frequency hopping | Intra-slot FH |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | MMSE based channel estimation |
| Performance metrics | For eMBB, 10% iBLER |

Table 3. Simulation assumption for PUSCH in FR2

| Parameters | Values |
| --- | --- |
| Physical channel | PUSCH |
| Carrier frequency | 28 GHz |
| Frame structure for TDD | DDDSU (S: 10D:2G:2U) |
| Bandwidth | 100 MHz |
| Waveform | DFT-s-OFDM |
| Subcarrier spacing for PUCCH | 120kHz |
| MCS and TBS | QPSK Urban/Indoor: MCS 6 and TBS 3824 for 2 DMRS symbols MCS 7 and TBS 3752 for 4 DMRS symbols |
| Resource allocation | eMBB: Urban: 14 symbols, 30 PRBs |
| UE antenna configuration | 1Tx |
| gNB antenna configuration | 2Rx |
| Propagation channel | Urban scenario: TDL-A 100ns |
| UE velocity | Urban: 3km/h for indoor, 30km/h for outdoor |
| Timing offset | 0 |
| Frequency offset | 0 |
| DMRS symbols | eMBB: 2 symbols for 3km/h, 4 symbols for 30km/h |
| Frequency hopping | Intra-slot FH |
| Receiver | MMSE-IRC as baseline |
| Channel estimation | MMSE based channel estimation |
| Performance metrics | For eMBB, 10% iBLER |

# Appendix III

Table 4. Simulation assumption for indoor office deployment in FR1

| Parameters | Values |
| --- | --- |
| Layout |  |
| Inter-BS distance | 20 m |
| Minimum BS-to-UE | 0 m |
| Minimum UE-to-UE distance | 3 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 20 MHz |
| BS TX power | 24 dBm |
| UE TX power | 23 dBm |
| BS antenna | (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.5)λ |
| BS antenna height: | 3 m |
| UE antenna | Omni |
| UE antenna height | 1.0 m |
| Antenna gain of UE | 0 dBi |
| Cell selection criteria | Cell selection is based on RSRP |
| BS receiver noise figure | 5 dB |
| UE receiver noise figure | 9 dB |
| Traffic model | FTP traffic model 3 with packet size 0.1 Mbytes |
| UE distribution | 10 UEs per TRP |

Table 5. Simulation assumption for urban macro deployment in FR1

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 500 m |
| Minimum BS-to-UE | 35 m |
| Minimum UE-to-UE distance | 3 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 20 MHz |
| BS Tx power | 49 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | Omni |
| UE antenna height | Same as 3D-Uma in TR 36.873 [4] |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |
| Traffic model | FTP traffic model 3 with packet size 0.1 Mbytes |
| UE distribution | 10 UEs per TRP, 20% outdoor and 80% indoor |

Table 6. Simulation assumption for dense urban deployment in FR1 (Optional)

| Layout | Fixed cluster circle within a macro cell. |
| --- | --- |
| # of micro BSs per macro cell | 3 |
| Inter-BS distance | 200 m |
| Minimum BS-to-UE | 35 m |
| Minimum UE-to-UE distance | 3 m |
| Carrier frequency | 4 GHz |
| Simulation bandwidth | 20 MHz |
| BS Tx power | 44 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 5 dB |
| UE antenna configuration | Omni |
| UE antenna height | Same as 3D-Uma in TR 36.873 [4] |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 9 dB |
| Traffic model | FTP traffic model 3 with packet size 0.1 Mbytes |
| UE distribution | 10 UEs per TRP, 20% outdoor and 80% indoor |

Table 7. Simulation assumption for indoor office deployment in FR2

| Layout |  |
| --- | --- |
| Inter-BS distance | 20 m |
| Minimum BS-to-UE | 0 m |
| Minimum UE-to-UE distance | 3 m |
| Carrier frequency | 30 GHz |
| Simulation bandwidth | 80 MHz |
| BS TX power | 24 dBm |
| UE TX power | 23 dBm |
| BS antenna | (Mg,Ng,M,N,P)=(1,1,4,4,2) (dH,dV)=(0.5,0.5)λ |
| BS antenna height | 3 m |
| BS antenna element gain + connector loss | 5 dBi |
| BS receiver noise figure | 7 dB |
| UE antenna configuration | Omni |
| UE antenna height | 1 m |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 13 dB |
| Traffic model | FTP traffic model 3 with packet size 0.1 Mbytes |
| UE distribution | 10 UEs per TRP |

Table 8. Simulation assumption for dense urban macro deployment in FR2

| Layout | Single layer with 19 hexagonal cell with wrap around |
| --- | --- |
| Inter-BS distance | 200 m |
| Minimum BS-to-UE | 35 m |
| Minimum UE-to-UE distance | 3 m |
| Carrier frequency | 30 GHz |
| Simulation bandwidth | 80 MHz |
| BS Tx power | 40 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 25 m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 7 dB |
| UE antenna configuration | Omni |
| UE antenna height | Same as 3D-Uma in TR 36.873 [4] |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 13 dB |
| Traffic model | FTP traffic model 3 with packet size 0.1 Mbytes |
| UE distribution | 10 UEs per TRP, 20% outdoor and 80% indoor |

Table 9. Simulation assumption for dense urban micro deployment in FR2 (Optional)

| Layout | Fixed cluster circle within a macro cell. |
| --- | --- |
| # of micro BSs per macro cell | 3 |
| Inter-BS distance | 100 m |
| Minimum BS-to-UE | 35 m |
| Minimum UE-to-UE distance | 3 m |
| Carrier frequency | 30 GHz |
| Simulation bandwidth | 80 MHz |
| BS Tx power | 35 dBm |
| UE Tx power | 23 dBm |
| BS antenna configurations | (Mg,Ng,M,N,P)=(1,1,8,8,2) (dH,dV)=(0.5,0.8)λ |
| BS antenna height | 10 m |
| BS antenna element gain + connector loss | 8 dBi |
| BS receiver noise figure | 7 dB |
| UE antenna configuration | Omni |
| UE antenna height | Same as 3D-Uma in TR 36.873 [4] |
| UE antenna gain | 0 dBi |
| UE receiver noise figure | 13 dB |
| Traffic model | FTP traffic model 3 with packet size 0.1 Mbytes |
| UE distribution | 10 UEs per TRP, 20% outdoor and 80% indoor |

# Appendix IV– RAN1 Agreements

This section captures all the agreements made during the past RAN1 meetings [2-3] on evaluation of duplex enhancements in Rel-18:

RAN1 #109e: Agreement For discussion purpose for evaluation, define the following deployment cases for SBFD: - Deployment Case 1 (Non-coexistence case with single SBFD subband configuration): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation with the same SBFD subband configuration. - Deployment Case 2 (Non-coexistence case with multiple SBFD subband configurations): One single operator using one single carrier is considered. All the cells belonging to the operator use SBFD operation, but different cells may use different SBFD subband configurations. - Deployment Case 3 (Co-channel co-existence case): One single operator using one single carrier is considered. Among the cells belonging to the operator, some of them use legacy TDD operation (static TDD operation) while the others use SBFD operation with the same SBFD subband configuration. - Deployment Case 3-1: Only 1-layer is considered - Deployment Case 3-2: 2-layer is considered - Deployment Case 4 (Adjacent-channel co-existence case): Two operators each using one carrier are considered and the two carriers are adjacent carriers. One operator uses legacy TDD operation (static TDD operation) while the other operator uses SBFD operation with the same SBFD subband configuration. Note: This definition has no intention to preclude any potential solutions for SBFD in AI9.3.2 Note: SBFD subband configuration is from gNB perspective. Agreement For SBFD Deployment Case 1, at least consider the following scenarios for evaluation: - For FR1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Urban macro (use Urban macro defined in TR38.802/TR38.901 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban with 1-layer or 2-layer (use Dense Urban defined in TR38.802/TR38.901 as starting point) - FFS: Rural - For FR2-1, - Indoor office (use Indoor office defined in TR38.802/TR38.901 as starting point) - Dense Urban Macro layer (use Dense Urban defined in TR38.802 as starting point) - FFS: UE outdoor/indoor proportion, clustering, etc - Optional: Dense Urban micro (use Dense Urban micro defined in TR38.802/TR38.901 as starting point) - FFS: Whether FR2-2 is considered or not in Rel-18. Note: For optional scenarios, they can be captured in TR and it is up to each company to provide the results. The results can be used to draw conclusion/recommendation depending on the number of companies providing the results. Agreement Regarding gNB self-interference modelling for system level simulation purpose, consider introducing ratio of self-interference (RSI) to represent the overall self-interference suppression capability of gNB by means of spatial isolation, subband frequency isolation, digital interference cancellation and beamform nulling/isolation, etc. RSI also takes into account the impact of Tx/Rx antenna element gain on self-interference. The RSI, denoted as αSIm,n, can be defined as the ratio of the total power transmitted by gNB across all transmit chains on a frequency unit m (e.g., subband/RB/subcarrier m) in a SBFD carrier to the residual self-interference received by the same gNB on a single receiver chain on a different frequency unit n (e.g., another subband/RB/subcarrier n) in the same SBFD carrier. - FFS: Model for link level simulations and relevant questions to ask RAN4 - FFS: details of gNB self-interference modelling using RSI in SLS. As one example based on per-RB-RSI, the gNB self-interference on a single receiver chain at UL RB n can be modelled as - ISIn=m≠nm∈DL subbandISIm,n, wherein, - dBmISIm,n=Ptxm-dBαSIm,n - ISIm,nis the gNB self-interference on a single receiver chain at UL RB n caused by DL transmission on DL RB m. - m is the DL RB index in DL subbands. - Ptxm is gNB’s DL transmission power across all transmit chains at RB m (in dBm). - αSIm,n is the per-RB-RSI. - FFS: consider a statistical clutter model based on statistics of clutter strength and AoA. - The following should be asked to RAN4: - What is the value range of RSI αSI(n,m) for each frequency range, and under what assumptions on the self-interference suppression means the value range of RSI is provided? - RAN1 understands the RSI can be described per subband, per RB, or per subcarrier depending on the granularity of the frequency unit, and it is up to RAN4 to provide the RSI in which granularity. - Whether it is possible for RAN4 to provide RAN1 the respective capabilities of different self-interference suppression means? e.g., is it possible to provide the separate estimates for spatial isolation, subband frequency isolation, beamform nulling/isolation, and digital cancellation, etc., as below? - dB(αSI(m,n))= dB(αSI-spatial)+dB(αSI-frequency (m,n))+dB(αSI-beam)+dB(αSI-digtial) +… - αSI-spatial denotes the spatial isolation. - αSI-frequency (m,n) denotes the suband frequency isolation between the Tx frequency unit m and the Rx frequency unit n. - αSI-beam denotes the beamform nulling or beam isolation. - αSI-digtial denotes the digital cancellation capability. - Whether it is possible to simplify the RSI as frequency flat model, and under which condition(s) the dependency of the RSI on frequency can be ignored? - The feasibility of provided value range of RSI regarding factors such as blocking, AGC, etc. - Does RSI have any dependency with the following factors or any other factors? What are the dependencies? - gNB’s antenna aspects, e.g., the assumed antenna architecture, the number of transmit chains and receive chains, etc. - Frequency aspects, e.g., the frequency distance between the Tx frequency unit m and the Rx frequency unit n, the number of RBs allocated for DL transmission, etc. - Beam aspects, e.g., Tx/Rx beam-pair for FR1/FR2 especially for clutter echo, etc. - Note: RAN1’s consideration on the frequency locations and sizes of SBFD DL subband and SBFD UL subband assumed in SBFD operation can be provided to RAN4. Agreement For discussion of gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation, RAN1 understands at least the following two aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs to the non-allocated RBs in the same carrier. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in the presence of the unwanted signals at the non-allocated RBs. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc) The following questions should be asked to RAN4: - Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE co-channel inter-subband CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following two cases: - inter-site gNB-gNB co-channel inter-subband CLI - co-site inter-sector co-channel inter-subband CLI - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in the same carrier and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver? FFS: Usage of the above model provided by RAN4 in the evaluation Agreement At least the following metrics are considered for SBFD and dynamic/flexible TDD evaluation. - DL/UL UPT or user throughput (CDF or {mean, 5%, 50%, 95%}) using SLS - Latency (CDF or {mean, 5%, 50%, 95%}) using SLS - Resource utilization using SLS - DL/UL received SINR using SLS - Coverage metric - FFS: MPL to achieve a certain bit rate in UL and DL - FFS: definitions of the above metrics - FFS: other metrics Agreement Regarding traffic model for SBFD and dynamic/flexible TDD evaluation, at least FTP3 is considered. Performance evaluation comparison between different duplex modes (e.g., legacy static TDD vs. SBFD) should be performed based on the same amount of input traffic. - FFS: other traffic models, e.g., XR, VoIP - FFS: Packet size, traffic load, ratio of DL/UL traffic - FFS: additionally consider different amount of input traffic at least for adjacent-channel coexistence studies Agreement For discussion for duplex evolution study (all agenda items), consider the following as RAN1’s common understanding: - Co-channel interference: The interference is from the aggressor to the victim in the same carrier. - Co-channel intra-subband interference: The interference is caused by transmission of the aggressor on a set of contiguous RBs in a carrier to reception of the victim on the same set of contiguous RBs in the same carrier. - Co-channel inter-subband interference: The interference is caused by transmission of the aggressor in a first set of contiguous RBs in a carrier to reception of the victim in a second set of contiguous RBs in the same carrier, where the two contiguous RB sets are non-overlapping in frequency. - Adjacent channel interference: The interference is from the aggressor in carrier#1 to the victim in carrier#2, where the carrier#1 and carrier#2 are adjacent carriers. Note 1: ‘Co-channel’ here means ‘co-carrier’. ‘Adjacent-channel’ here means ‘adjacent-carrier’. Agreement For discussion for duplex evolution study (all agenda items), consider the following as the common understanding in RAN1 on the definition of interference types for SBFD operation: - gNB self-interference (SI): Interference caused by DL transmission on a set of DL RBs in a carrier to UL reception on a set of UL RBs in the same carrier at the gNB side, where the two RB sets are non-overlapping in frequency. - gNB-UE co-channel intra-subband interference: This is the same as the legacy DL interference type in legacy TDD network with static TDD UL/DL configuration. - UE-gNB co-channel intra-subband interference: This is the same as the legacy UL interference type in legacy TDD network with static TDD UL/DL configuration. - (inter-cell) inter-site gNB-gNB co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in a different site on the same set of RBs in the same carrier. - (inter-cell) co-site inter-sector co-channel intra-subband CLI: CLI caused by DL transmission of the aggressor gNB on a set of RBs in one carrier to UL reception of the victim gNB in another sector of the same site on the same set of RBs in the same carrier. - (inter-cell) UE-UE co-channel intra-subband CLI: CLI caused by UL transmission of the aggressor UE on a set of RBs in one carrier to DL reception of the victim UE on the same set of RBs in the same carrier. - (inter-cell) inter-site gNB-gNB co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in a different site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (inter-cell) co-site inter-sector co-channel inter-subband CLI: CLI caused by DL transmission of the aggressor gNB on a first set of RBs in a carrier to UL reception of the victim gNB in another sector of the same site on a second set of RBs in the same carrier, where the two RB sets are non-overlapping in frequency. - (intra-cell/inter-cell) UE-UE co-channel inter-subband CLI: CLI caused by UL transmission of the aggressor UE on a first set of RBs in a carrier to DL reception of the victim UE on a second set of RBs in the same cell or neighboring cell in the same carrier, where the two RB sets are non-overlapping in frequency. - gNB-gNB adjacent-channel CLI: CLI caused by DL transmission of the aggressor gNB in a carrier to UL reception of the victim gNB in another adjacent carrier. - This includes adjacent-channel CLI between gNBs in the same and different sectors of the same site, i.e., co-site intra and inter-sector adjacent-channel CLI. - UE-UE adjacent-channel CLI: CLI caused by UL transmission of the aggressor UE in a carrier to DL reception of the victim UE in another adjacent carrier. Note: Some of the interferences may not be used according to the deployment scenarios, e.g, whether the SBFD subband configurations are the same or different across gNBs. Note: This does not imply we need to consider all the above interference types in evaluation for SBFD. Agreement Regarding gNB-gNB and UE-UE adjacent-channel CLI modelling for system level simulation, RAN1 understands at least the following aspects need to be considered: - Aspect 1: The unwanted emissions due to Tx non-linearity at the transmitter of the aggressor from the allocated RBs in one carrier to the non-allocated RBs in the adjacent carrier. - Aspect 2: The receiver selectivity at the victim to receive the desired signal in the allocated RBs in one carrier in the presence of the unwanted signals at the non-allocated RBs in the adjacent carrier. (e.g. receiver blocking at the victim, overload of the receiver dynamic range, etc) The following questions should be asked to RAN4: - Whether it is feasible to consider the above two aspects for gNB-gNB and UE-UE adjacent-channel CLI modelling in system level simulation? Are there any other aspects should also be taken into account? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of gNB-gNB link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor gNB transmits on the DL frequency unit m and the victim gNB receives on the UL frequency unit n, - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 1 (defined above) at the gNB transmitter? - How to model the interference from DL frequency unit m to UL frequency unit n due to Aspect 2 (defined above) at the gNB receiver? - How to model the above interferences for the following cases: - the two gNBs are from the same sector of the same site in adjacent carriers, i.e., co-site co-sector gNB-gNB adjacent-channel CLI - the two gNBs are from different sectors of the same site in adjacent carriers, i.e., co-site inter-sector gNB-gNB adjacent-channel CLI - the two gNBs are from different sites in adjacent carriers, i.e., inter-site gNB-gNB adjacent-channel CLI - Whether it is feasible to define a similar interference ratio as BS-BS ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above? - For example, whether it is feasible to define gNB-gNB-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor gNB on DL frequency unit m to the interference received by the victim gNB on UL frequency unit n? If it is feasible, then what is the value range of the gNB-gNB-adjacent-channel-per-RB/subband interference ratio for each frequency range? - For a specific pair of DL frequency unit m (e.g., subband/RB m) and UL frequency unit n (e.g., subband/RB n) of UE-UE link, where the DL frequency unit m and UL frequency unit n are in adjacent carriers and non-overlapping in frequency, and assuming the aggressor UE transmits on the UL frequency unit n and the victim UE receives on the DL frequency unit m, - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 1 (defined above) at the UE transmitter? - How to model the interference from UL frequency unit n to DL frequency unit m due to Aspect 2 at the UE receiver? - Whether it is feasible to define a similar interference ratio as UE-UE ACIR in TR38.828 but in the subband of the adjacent carrier, with finer granularity (e.g., per subband or per RB), to represent the overall effect of the Aspect 1 and Aspect 2 described above? - For example, whether it is feasible to define UE-UE-adjacent-channel-per-RB/subband interference ratio as the ratio of the power transmitted by the aggressor UE on UL frequency unit n to the interference received by the victim UE on DL frequency unit m? If it is feasible, then what is the value range of the UE-UE-adjacent-channel-per-RB/subband interference ratio for each frequency range? FFS: How to make use of the interference model in RAN1 Agreement For SBFD evaluation, consider the following for SBFD subband configurations: - SBFD Subband configuration#1 with {DUD} pattern, which means one SBFD slot consists of one UL subband at the center of the channel bandwidth and two DL subbands at two sides of the channel bandwidth. - SBFD Subband configuration#2 with {DU} pattern, which means one SBFD slot consists of one UL subband at one side of the channel bandwidth and one DL subband at the other side of the channel bandwidth. - Use the following parameters for description of SBFD subband configuration in evaluation assumptions: - ND: the number of RBs in one DL subband - NU: the number of RBs in one UL subband - NG: the number of RBs in one guard band between one UL subband and one DL subband Agreement For performance evaluation and comparison between baseline legacy TDD operation and SBFD operation under SBFD Deployment Case 1 (Non-coexistence case with single SBFD subband configuration), consider the following alternatives: - Alt 2 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 4 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#3 (XXXXX), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 1 (No SBFD DL subband in the slots/symbols that correspond to UL slots/symbols in legacy TDD): - Legacy TDD: Static TDD UL/DL configuration with {DDDSU}, where S=[12D:2G:0U] - SBFD: Frame structure#1 (DXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. - Alt 3 (strive for the same UL/DL resource ratio between Legacy TDD and SBFD): - Legacy TDD: Static TDD UL/DL configuration with {DDSUU}, where S=[12D:2G:0U] - SBFD: Frame structure#2 (XXXXU), where X denotes a SBFD slot. In time domain, SBFD UL subband spans all the symbols in a SBFD slot. In frequency domain, SBFD UL subband is about [20%] of the channel bandwidth. FFS: whether dynamic TDD can optionally be used for legacy TDD for comparison. Agreement For gNB-gNB co-channel/adjacent-channel channel model and UE-UE co-channel/adjacent-channel channel model in RAN1 SLS, - Large scale fading (e.g., path loss, penetration loss, shadowing) should be modelled, and companies report whether small scale fading (e.g., fast fading including antenna gain) is also modelled in their simulation. - Note: Antenna gain is calculated based on the gNB-gNB or UE-UE LOS direction instead on the multi-path directions if fast fading is not modeled. - FFS: how to model realistic LOS probability for gNB-gNB and UE-UE channel model. - FFS: How to set aligned channel model amongst companies for SLS calibration (if needed). Agreement For gNB-gNB channel model, reuse gNB-to-UE channel model in TR 38.901 with necessary modification - Replacing the UE’s antenna height with gNB’s antenna height, updating the angular spread - FFS: whether/how to update LOS probability. - FFS: Other details and necessary modifications Agreement For SBFD simulation, consider 4GHz for FR1 and 30GHz for FR2-1. Agreement For evaluation of SBFD operation, BS uses separate panels for simultaneous downlink transmission and uplink reception, we can call it separate-Tx/Rx antenna array for description of evaluation assumption. - Companies can report the separation of the Tx panel and Rx panel assumed in their simulation. - Companies can report how the antenna elements are used for transmission or reception in a slot if BS does not perform simultaneous downlink transmission and uplink reception. Agreement For evaluation of legacy TDD operation, BS uses the same antenna array for downlink transmission and uplink reception, we can call it shared-Tx/Rx antenna array for description of evaluation assumption. Agreement For evaluation and comparison between SBFD and legacy TDD, assume the total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. Regarding antenna elements, both of the two options can be used. - Opt 1: The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. - Opt 2: The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. - Companies report which option is assumed in their simulation. Agreement For SBFD Deployment Case 4, at least consider the following scenarios for evaluation from RAN1 perspective: - FR1: Urban Macro - FR2-1: Dense Urban Macro layer - FFS: UE outdoor/indoor proportion, clustering, etc - FFS: the grid shift between two networks, e.g., 0%, 100% - FFS: Indoor hotspot, Dense Urban Micro layer RAN1 #110: Agreement Two types of RU (Resource utilization) are defined for SBFD evaluation. - Type-1 RU: DL/UL Type-1 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of all the RBs per cell including DL, UL and guard bands over observation time. - Type-2 RU (Follow TR 36.814): DL/UL Type-2 RU = Number of RBs per cell used by traffic for the given link direction during observation time / Total number of RBs per cell available for traffic for the given link direction over observation time - Note: In case of MU-MIMO, one RB allocated to N users within a cell is only counted as used once. - Companies are to submit results for both RU definitions - FFS: RU definition for dynamic TDD evaluations Agreement For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline (UE clustering): - 10 users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped throughout the macro geographical area - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - FFS the values of X, Dmacro-to-cluster, Dinter-cluster, R, Y% - Optional: - 10 users per macro TRP, and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - FFS: FR2 details Agreement For Dense Urban with 2-layer for FR1, consider micro cell TRPs are deployed as following - Step 1: Randomly drop [3] micro TRP centers within one macro cell geographical area considering the minimum distance between micro TRP centers (Dinter-micro-center) and the minimum distance between macro TRP and micro TRP center (Dmacro-to-micro-center). - Step 2: Randomly deploy one micro TRP on the area circle around each micro TRP center with the radius of half of Dinter-micro-center - Step 3: Determine the horizontal angle of the micro TRPs with the planer facing to the micro TRP center. - Dinter-micro-center =[57.9 m], Dmacro-to-micro-center = [105 m] Agreement For latency related performance metric for FTP model 3 in SLS, option 1 is baseline, it is up to companies to report the latency with option 2. - Packet latency: defined as the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver. - (baseline) Option 1: Calculate the latency for each packet for each UE, and then generate CDF of latency for all these packets from all the UEs. - Packet-Latency CDF: The CDF of the packet latencies of all the packets from all the UEs. - Mean/5%/50%/95% Packet-Latency: The mean/5%/50%/95% value of Packet-Latency of all the packets from all the UEs. - (optional) Option 2: Calculate the latency for each packet for each UE, and then calculate the average latency for each UE, then generate the CDF for these average latency for each UE - UE-Average-Latency: defined as the average packet latency for a UE - UE-Average-Latency CDF: The CDF of the UE-Average-Latency for all users. - Mean/5%/50%/95% UE-Average-Latency: The mean/5%/50%/95% value of UE-Average-Latency for all users. - Note: HARQ re-transmission should be considered for latency evaluation. - Unfinished/dropped FTP packets are not incorporated in the packet latency calculation. - Unfinished/dropped Packet Rate is defined as the number of the unfinished packets for all users divided by the total number of generated packets for all users - To be reported as part of the system level simulation results Agreement For UPT (user perceived throughput) related performance metrics for FTP model 3 in SLS, adopt the following option. - Option 1: UPT is defined as the size of an FTP packet divided by the time which starts when the packet is received in the transmit buffer and ends when the last bit of the packet is correctly delivered to the receiver [Refer to TR36.814]. - Unfinished FTP packets should be incorporated in the UPT calculation. The number of served bits (possibly zero) of an unfinished FTP packet by the end of the simulation is divided by the served time (simulation end time – file arrival time) [Refer to TR36.889]. - Consider zero bit for dropped FTP packets. - Average-UPT of a user: defined as the average from all UPTs for all FTP packets intended for this user [Refer to TR36.814]. - Tail-UPT of a user: defined as the worst 5% UPT among all FTP packets intended for this user [Refer to TR36.814]. - Median-UPT of a user: defined as the 50% UPT among all FTP packets intended for this user. - Average-UPT CDF: The CDF of the Average-UPTs for all users. - Tail-UPT CDF: The CDF of the Tail-UPTs for all users. - Median-UPT CDF: The CDF of the Median-UPTs for all users. - Mean/5%/50%/95% Average-UPT: The mean/5%/50%/95% value of Average-UPTs for all users. - Mean/5%/50%/95% Tail-UPT: The mean/5%/50%/95% value of Tail-UPTs for all users. - Mean/5%/50%/95% Median-UPT: The mean/5%/50%/95% value of Median-UPTs for all users. Agreement Adopt the following table for traffic model of FTP model 3 for scenarios in deployment case 1 for SBFD. Indoor office (FR1&FR2); Urban Macro (FR1); Dense Urban Macro layer (FR1&FR2); Dense Urban Micro layer (FR2); Dense Urban with 2-layer (FR1). General; UL and DL are simulated simultaneously. Companies to report which option is used. - Option 1: Each UE is either assigned UL traffic or DL traffic. - assume the same number of UEs for UL and DL, FFS the total number of UEs - FFS how to handle the UE clustering case - Option 2: Each UE is assigned both UL traffic and DL traffic.. FTP packet size; Both symmetric and asymmetric packet size for UL and DL can be considered. Companies to report which option is used. - Option 1: Symmetric packet size: - 1Kbyte for DL/UL, 0.1Mbytes for DL/UL, 0.5Mbytes for DL/UL, 2Mbytes for DL/UL - Option 2: Asymmetric packet size: - 4Kbytes for DL and 1Kbyte for UL, 0.5Mbyte for DL and 0.125 Mbytes for UL. UL arrival rate for legacy TDD; - The UL arrival rate is selected to reach a target UL traffic load (RU). - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used; - The UL arrival rate#1 of Macro cell and UL arrival rate#2 of Micro cell are selected to reach target UL traffic load (RU)#1 of Macro cell and target UL traffic load (RU)#2 of Micro cell, respectively - UL Traffic load: low UL RU ([<10%]), medium UL RU ([20%-30%]), and high UL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used. DL arrival rate for legacy TDD; - The DL arrival rate is selected to reach a target DL traffic load (RU). - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used; - The DL arrival rate#1 of Macro cell and DL arrival rate#2 of Micro cell are selected to reach target DL traffic load (RU)#1 of Macro cell and target DL traffic load (RU)#2 of Micro cell, respectively - DL Traffic load: low DL RU ([<10%]), medium DL RU ([20%-30%]), and high DL RU ([~50%]). - Note: Type-2 RU definition (calculated per link direction) is used. Arrival rate for SBFD; The UL and DL FTP packet arrival rate for SBFD are the same as legacy TDD.. Working assumption: Adopt the following table for gNB-gNB channel model and gNB-UE channel model. Dense urban, Urban macro; Indoor office. Large-scale channel parameters; FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m), - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m) FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa in TR 38.901 (hUE =25m) - Macro-to-Micro: UMa in TR 38.901 (hUE =10m) - Micro-to-Micro: UMi-Street canyon in TR 38.901 (hUE =10m); FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m) FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m). Fast fading parameters; FR1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE =25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 FR2-1: - Macro-to-UE: UMa in TR 38.901 - Micro-to-UE: UMi-Street canyon in TR 38.901 - Macro-to-Macro: UMa O2O in TR 38.901 (hUE=25m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0 - Macro-to-Micro: UMa O2O in TR 38.901 - Micro-to-Micro: UMi-Street canyon O2O in TR 38.901 (hUE=10m); ASA and ZSA statistics updated to be the same as ASD and ZSD; ZoD offset = 0; FR1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE=3m), ASA and ZSA statistics updated to be the same as ASD and ZSD FR2-1: - TRP-to-UE: InH-Office in TR 38.901 - TRP-to-TRP: InH-Office in TR 38.901 (hUE =3m), ASA and ZSA statistics updated to be the same as ASD and ZSD. Agreement For evaluation of SBFD and dynamic/flexible TDD, adopt the following evaluation assumptions. FR1; FR2-1. System bandwidth; 100MHz; 100MHz. Numerology; 14 OFDM symbol slot SCS = 30kHz; 14 OFDM symbol slot SCS = 120kHz. UE Tx power; 23dBm; 23 dBm. EIRP should not exceed 43 dBm [refer to TR 38.802 Table A.2.1-1]. Open loop power control parameters; Companies to report power control parameters. For calibration: - P0= -60 dBm, alpha = 0.6 for InH [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] - P0= -86 dBm, alpha = 0.9 for Dense Urban [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip] - P0= -80 dBm, alpha = 0.8 for Urban Macro. BS receiver noise figure; 5dB [refer to TR 38.802 Table A.2.1-1]; 7dB [refer to TR 38.802 Table A.2.1-1]. UE receiver noise figure; 9 dB [refer to TR 38.802 Table A.2.1-1]; 13 dB (baseline), 10 dB (optional) [refer to TR 38.802 Table A.2.1-1]. UE receiver; MMSE-IRC as the baseline receiver. Note: Advanced receiver is not precluded. [refer to TR 38.802 Table A.2.1-1]. Feedback assumption; Realistic [refer to TR 38.802 Table A.2.1-1]. Channel estimation; Companies to report the option used. Option 1: Ideal Option 2: Realistic [refer to TR 38.802 Table A.2.1-1]. UE processing capability; UE processing capability 1 as baseline; UE processing capability 1 as baseline. Handover margin; 3 dB [refer to TR 38.828 Table 5.2.1.4-1]. UE attachment; Based on RSRP from port 0 [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip]; Based on RSRP from port 0. The UE panel with the best receive SNR is chosen. i.e. no combining is done between panels. [refer to TR 37.910, evaluation assumption in B.4.1_eMBB_SE.zip]. Polarized antenna model; Model-1 in clause 7.3.2 in TR 38.901. DL/UL Modulation; Up to 256QAM. Transmission scheme; Companies to report transmission schemes (e.g., SU-MIMO, MU-MIMO, maximum layers for SU-MIMO/MU-MIMO, etc) For calibration, consider SU-MIMO with single layer for both DL and UL. Scheduling; PF. Overhead; Companies to report the overhead assumption. Agreement Update the previous agreement as below: For UE distribution of Urban Macro and Dense Urban Macro layer, - Baseline: (UE clustering at least for FR1) - M users per macro TRP - Step 1: Randomly drop X UE cluster centers within one macro cell geographical area considering the minimum distance between macro TRP to UE cluster center as Dmacro-to-cluster and the minimum distance between two UE cluster centers as Dinter-cluster - Step 2: Y% UEs are randomly and uniformly dropped within the UE clusters with the radius of R, (1-Y%) users randomly and uniformly dropped in the macro geographical area outside the clusters - Note: UEs dropped within the UE cluster(s) are indoor with 3km/h; UEs dropped outside the UE cluster(s) are outdoor in car with 30km/h - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - FFS: Indoor UEs height - Y%=80% - FFS the values of M, X, Dmacro-to-cluster, Dinter-cluster, R - Optional: - 10 users per macro TRP (per direction), and all users are randomly and uniformly dropped within the macro cell - At least for FR1: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) [refer to TR 36.873 Table 6-1] - FFS: FR2 details Agreement For LOS probability of gNB-gNB channel, - For Macro-gNB-to-Macro-gNB case - Option 3: If the 2D distance between two Macro gNBs are less than or equal to the ISD (200m for Dense Urban, and 500m for Urban Macro), set the LOS probability to X; Otherwise, reuse gNB-to-UE LOS probability equation in TR 38.901. - X = 0.75 - For other cases, reuse gNB-to-UE LOS probability equation in TR 38.901. Agreement For Dense Urban Micro layer for FR2-1, - Regarding the layout, only consider the Micro TRPs of Dense Urban 2-layer network. All users communicate with micro TRPs, i.e. macro cell is only used for determining position of micro TRP. - Regarding UE distribution, all users are randomly and uniformly dropped around Micro TRP center with the radius of R (R = [28.9m]). Agreement For UE distribution of Dense Urban with 2-layer, reuse the modeling in TR38.802 as much as possible. - For FTP traffic model 3: 2/3 users randomly and uniformly dropped around micro TRP centers with radius of R (R = [28.9m]), 1/3 users randomly and uniformly dropped throughout the macro geographical area, and 60 users per macro geographical area. - UE outdoor/indoor proportion: 20% outdoor in cars: 30km/h; 80% indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) Agreement For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case. FR1; FR2-1. Urban macro; - Option 1: [53] dBm for 100MHz - Option 2: [49] dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1]; N.A.. Dense Urban Macro layer; - Option 1: [53] dBm for 100MHz - Option 3: [44] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 1: [43] dBm for 200MHz [refer to TR 38.828 Table 5.2.2.4-1]. Dense Urban Micro layer; - Option 3: [40] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 2: [33] dBm for 200MHz. EIRP should not exceed 68 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Indoor hotspot; - Option 2: [24] dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1]; - Option 1: [23] dBm for 200MHz. EIRP should not exceed 58 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Agreement For evaluation of SBFD and dynamic/flexible TDD, use BS antenna radiation pattern as following: - InH: reuse Table 10 in Report ITU-R M.2412 for both FR1&FR2-1 (Table A.2.1-7 in TR 38.802) - Urban Macro/ Dense Urban Macro layer / Dense Urban Micro layer: reuse Table 9 in Report ITU-R M.2412 for both FR1&FR2-1 (same as 3-sector BS antenna radiation model in Table A.2.1-6 in TR 38.802) - Companies can also consider evaluation with other realistic BS antenna radiation pattern Agreement For evaluation of SBFD and dynamic/flexible TDD, use UE antenna radiation pattern as following: - FR1: Omni-directional with 0 dBi element gain - FR2: reuse Table 11 in Report ITU-R M.2412 (same as UE antenna radiation pattern model 1 in Table A.2.1-8 in TR 38.802) Working Assumption Parameters; Indoor office; Urban macro / Dense Urban Macro layer; Dense Urban with 2-layer. Layout; Single layer Indoor floor: (12BSs per 120m x 50m); Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around.; Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: According to previous agreement - Baseline: 3 Micro BSs per Macro BS - Optional: 6, or 9 Micro BSs per Macro BS. Inter-BS (2D) distance; 20m [TR 38.802 Table A.2.1-11]; 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1]; Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 105m Minimum Micro-center-to-micro-center distance: 57.9m. Minimum BS-UE (2D) distance; 0m [TR 38.802 Table A.2.1-11]; 35m [TR 38.802 Table A.2.1-11]; Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11]. Minimum UE-UE (2D) distance; FFS; FFS :3m [TR 38.802 Table A.2.1-11]; FFS: 3m [TR 38.802 Table A.2.1-11]. BS antenna height; 3 m [TR 38.802 Table A.2.1-1]; 25 m [TR 38.802 Table A.2.1-1]; 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1]. Conclusion - For SLS of NR duplex evolution, Rural scenario is not considered in Rel-18. - For NR duplex evolution evaluation, FR2-2 is not considered in Rel-18. Agreement For SBFD evaluation from RAN1 perspective, the evaluation assumptions that are specific for Deployment Case 2 and Case 3-1 can be discussed with low priority. Agreement RAN1 strives to agree on system level simulation parameters for SBFD deployment case 4 by RAN1#110bis-e with specific focus on different power levels and load levels between two operators in adjacent carriers. Agreement For evaluation of SBFD operation, separate-Tx/Rx antenna array can be modelled by two panel groups. - Legacy parameters , and are used for description of each panel group: - M: Number of vertical antenna elements within a panel, on one polarization - N: Number of horizontal antenna elements within a panel, on one polarization - P: Number of polarizations - : Number of panels in a column within a panel group. - : Number of panels in a row within a panel group. - : Antenna panel spacing in horizontal direction within a panel group. - : Antenna panel spacing in vertical direction within a panel group. - Companies are to report the separation of the two panel groups. Introduce new parameters as illustrated in the following figure. - : Panel group spacing in the horizontal direction. Typically, = 0. - : Panel group spacing in the vertical direction. Agreement For evaluation and comparison between SBFD and legacy TDD, the two options for the SBFD antenna configuration agreed in RAN1#109 are further clarified as below: - SBFD antenna configuration option-1 (same as Opt 1 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-2 (same as Opt 2 in RAN1#109 agreement): The total number of antenna elements of the antenna array for SBFD is two times of the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is the same as the total number of TxRUs of the antenna array for legacy TDD. - SBFD antenna configuration option-3 (new): The total number of antenna elements of the antenna array for SBFD is the same as the total number of antenna elements of the antenna array for legacy TDD. The total number of TxRUs of the antenna array for SBFD is half of the total number of TxRUs of the antenna array for legacy TDD. These options are further clarified with examples in the following: - For legacy TDD with shared-Tx/Rx antenna array, assume the antenna configuration is . The total number of TxRUs is , and the total number of antenna elements is . - For SBFD antenna configuration option-1, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is . The total number of TXRUs is (same as legacy TDD), and the total number of antenna elements is (same as legacy TDD). One method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols is illustrated as below. Other methods are not precluded and can be reported by companies. - Method 1: - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Tx chains in TxRU group#2. - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#2. - For SBFD antenna configuration option-2, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is . The total number of TXRUs is (same as legacy TDD), and the total number of antenna elements is (two times of that for legacy TDD). Two methods on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies. - Method 2-1: - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains. - In UL slots, L antenna elements on panel group#2 are connected to K Rx chains. - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains. - Method 2-2: - In DL slots, L antenna elements on panel group#1 are connected to K Tx chains. - In UL slots, L antenna elements on panel group#1 are connected to K Rx chains. - In SBFD slots, L antenna elements on panel group#1 are connected to K Tx chains, and L antenna elements on panel group#2 are connected to K Rx chains. - For SBFD antenna configuration option-3, the separate-Tx/Rx antenna array has two panel groups, and the antenna configuration for each panel group is . The total number of TXRUs is (half of that for legacy TDD), and the total number of antenna elements is (same as legacy TDD). The method on the usage of TXRUs and antenna elements in DL/UL/SBFD slots/symbols are illustrated as below. Other methods are not precluded and can be reported by companies. - Method 3-1: - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains. - In UL slots, L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains. - Method 3-2: - In DL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1. - In UL slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Rx chains in TxRU group#1. - In SBFD slots, L⁄2 antenna elements on panel group#1 are connected to K⁄2 Tx chains in TxRU group#1, and L⁄2 antenna elements on panel group#2 are connected to K⁄2 Rx chains in TxRU group#1. Working Assumption For UE-UE channel model, reuse the UE-UE channel model for flexible duplex evaluation in TR 38.802 for both FR1 and FR2 as baseline, and other models are not precluded. UE-UE channel model Dense urban, Urban macro; Indoor hotspot. Large-scale channel parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843(*), penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-13 in TR38.802 FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901 (hBS =1.5m ~ 22.5m), penetration loss between UEs follows Table A.2.1-12 in TR38.802; FR1: - Option1 : UE-to-UE: A.2.1.2 in TR36.843 (*) - Option 2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m) FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m). Fast fading parameters; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH) for indoor to indoor, and 3D UMi for other cases. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Optioin 2: UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA. FR2-1: - UE-to-UE: UMi-Street canyon in TR 38.901; ASD and ZSD statistics updated to be the same as ASA and ZSA.; FR1: - Option 1: UE-to-UE: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option2: UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA FR2-1: - UE-to-UE: InH-Office in TR 38.901 (hBS =1.5m), ASD and ZSD statistics updated to be the same as ASA and ZSA. (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802.. Agreement For evaluation of adjacent-channel coexistence between two networks for Urban Macro and Dense Urban Macro layer scenarios in RAN1, consider grid shifts between two networks of 0% and 100%. - the topologies shown below can be used for the 0% and 100% grid shift for RAN1 evaluation. Agreement For evaluation of SBFD operation, it is up to companies to report the BS antenna configurations used in their simulations. The BS antenna configurations in the following table can be considered for calibration purpose. Scenarios; FR; Legacy TDD; SBFD. BS antenna configuration for Indoor office; FR1; = (4,4,2,1,1; 4,4) = (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: = (2,4,2,1,1). - Number of TxRUs: same as legacy TDD - = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ. FR2-1; =(16,8,2,1,1; 1,1) = (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: = (8,8,2,1,1). - Number of TxRUs: same as legacy TDD - = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ. BS antenna configuration for Urban Macro/ Dense Urban Macro layer/ Dense Urban Micro layer; FR1; = (8,8,2,1,1;2,8) = (0.5, 0.8)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: = (4,8,2,1,1). - Number of TxRUs: same as legacy TDD - = (0.5, 0.8)λ, +45°/-45° polarization, (da,H,da,V) = (0, 4)λ. FR2-1; = (4,16,2,2,2; 1,1) = (0.5, 0.5)λ, +45°/-45° polarization; - SBFD antenna configuration option-1 (Method 1) - Two panel groups - For each panel group: = (4,8,2,2,2). - Number of TxRUs: same as legacy TDD - = (0.5, 0.5)λ, +45°/-45° polarization, (da,H,da,V) = (0, 30)λ. RAN1 #110: Agreement For evaluation of SBFD Deployment Case 3-2, the following scenario is baseline for FR1: - 2-layer Scenario B - Layer 1: Urban Macro - Layer 2: Indoor office or Indoor factory - Indoor factory is optional (Companies are to report the used layout.) - Regarding the Indoor office layer, reuse the Indoor office (InH) scenario (i.e., open office in Table 7.2-2 in TR38.901) and relevant channel model in TR38.901. - Regarding the Indoor factory layer, reuse the Indoor factory (InF) scenario (i.e., Table 7.2-4 in TR38.901) and relevant channel model in TR38.901. - FFS: consider only one indoor office/factory dropped in the whole network - Layer 1 uses legacy static TDD operation, Layer 2 uses SBFD operation. All the gNBs in Layer 2 use the same SBFD subband configuration. - Other operations are not precluded and can be reported by companies, e.g., Layer 1 uses SBFD operation and Layer 2 uses legacy TDD operation Companies can submit results for other scenarios Agreement For evaluation of dynamic/flexible TDD for the single operator case, consider the following scenarios: - FR1 - 1-layer scenario: Indoor office with dynamic TDD UL/DL assignment - (Optional) 1-layer scenario: Urban Macro with dynamic TDD UL/DL assignment - 2-layer Scenario B - Layer 1: Urban Macro - Layer 2: Indoor office or Indoor factory (companies to report which one is used) - Indoor factory is optional (Companies are to report the used layout.) - Regarding the Indoor office layer, reuse the Indoor office (InH) scenario (i.e., open office in Table 7.2-2 in TR38.901) and relevant channel model in TR38.901. - Regarding the Indoor factory layer, reuse the Indoor factory (InF) scenario (i.e., Table 7.2-4 in TR38.901) and relevant channel model in TR38.901. - FFS: consider only one indoor office/factory dropped in the whole network - Regarding 2-layer scenario, the two layers are deployed in the same carrier - Layer 1 uses legacy static TDD operation with DL dominant static TDD UL/DL configuration - Layer 2 uses one of the following options (companies to report which option is used) - Option 1: All gNBs in layer 2 use legacy static TDD operation with the same UL dominant static TDD UL/DL configuration - Option 2: All gNBs in layer 2 use dynamic TDD UL/DL assignment - Other options are not precluded and can be reported by companies - FR2-1 - 1-layer scenario: Indoor office with dynamic TDD UL/DL assignment - (Optional) 1-layer scenario: Dense Urban Macro layer with dynamic TDD UL/DL assignment - For above scenarios, the following is assumed: - DL dominant static TDD UL/DL configuration: {DDDSU}, where S=[12D:2G:0U] - UL dominant static TDD UL/DL configuration: {DSUUU}, where S=[12D:2G:0U] - dynamic TDD UL/DL assignment: {FFFFF}, companies to report the guard symbols assumed in their simulation - other configurations for dynamic TDD are not precluded and can be reported by companies Companies can submit results for other scenarios Agreement RAN1 assumes frequency isolation value in the overall RSI value ranges provided by RAN4 is based on the assumption of SBFD subband configuration with {DUD=40MHz:20MHz:40MHz} at least for FR1 and all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power. - For SLS of SBFD in RAN1, the RSI is modelled as frequency flat within the UL subband. - Using αSI to denote the overall RSI value provided by RAN4, RAN1 makes the following assumption - ISImax=PtxmaxαSI*NULRBNDLRB - ISImax is the residual self-interference power on the UL subband when all the DL RBs in the DL subbands are allocated with maximum gNB DL Tx Power (in linear scale). - Ptxmax is the maximum gNB DL Tx Power on the two DL subbands (in linear scale). - NDLRB is the total number of DL RBs in the DL subbands. - NULRB is the total number of UL RBs in the UL subband. - Note: αSI is in linear scale - RAN1 further makes a simple assumption that αSI doesn’t change when DL RBs are not fully allocated for DL transmission, and the residual self-interference power on one UL RB when DL RBs are not fully allocated for DL transmission is computed by - ISIper-RB=Ptxper-RBαSI*Nused-DL-RBNDLRB - Ptxper-RB is DL transmission power of gNB per RB, Ptxper-RB=Ptxmax/NDLRB. - Nused-DL-RB is the number of DL RBs allocated for DL transmission. - Send LS to RAN4 to confirm RAN1’s assumptions and the subband configuration assumed for FR1/FR2 - Also ask RAN4 if the above is applicable to other subband configurations Agreement For SLS in RAN1, if only large scale fading is modelled and small scale fading is not modelled for gNB-gNB co-channel channel model, the power of inter-site gNB-gNB co-channel inter-subband CLI experienced by the victim gNB on each receiver chain at one UL RB can be modelled as - IInter-Site-CLIA'→A,per-RB=PtxBS A',per-RB*Nused-DL-RBBS A'*CLBS A'→BS A*1ACLRBS+1ACSBS* 1NDLRB - IInter-Site-CLIA'→A,per-RB is the power of inter-site gNB-gNB co-channel inter-subband CLI from gNB A' to gNB A on each receiver chain at one UL RB (linear value) - PtxBS A',per-RB is DL transmission power of gNB A' across all transmit chains per RB (linear value). PtxBS A',per-RB=Ptxmax/NDLRB. - Nused-DL-RBBS A' is the number of DL RBs allocated for DL transmission by gNB A' - CLBS A'→BS A is the coupling loss between gNB A' and gNB A (linear value), accounting for beamforming at the aggressor gNB and victim gNB. - FFS: the detailed definition of the coupling loss, which can be discussed later - NDLRB is the total number of DL RBs in the DL subbands - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs is used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Note: This model is not applicable for some candidate gNB-gNB CLI handling schemes (for example, spatial digital beam coordination, advanced receivers) - Send LS to RAN4 to confirm RAN1’s understanding Agreement For SLS in RAN1, if both large scale fading and small scale fading are modelled for gNB-gNB co-channel channel model, the inter-site gNB-gNB co-channel inter-subband CLI signal across all Rx chains at UL RB n at victim gNB can be modeled as IInter-Site-CLIn=Ileakagen+Iselectivity, where, - Ileakagen=HCLInWyn is the first part of inter-site gNB-gNB co-channel inter-subband CLI across all Rx chains at UL RB n, caused by power leakage at aggressor gNB, - HCLIn is the NR×NT channel matrix between aggressor gNB and victim gNB at UL RB n, the beamforming of the aggressor gNB and the victim gNB can be taken into account by HCLIn, - yn=y0n,y1n,…,yNT-1nT is the unwanted emission across all Tx chains at UL RB n at aggressor gNB, - NT is the number of Tx chains at aggressor gNB, - ykn~N0,σy,n2, k=0,1,…,NT-1, is modelled as white Gaussian noise, - σy,n2=Ptxper-RB*Nused-DL-RBACLRBS* 1NDLRB is the total leakage power at UL RB n at aggressor gNB, - Ptxper-RB is the DL power transmitted across all Tx chains at one DL RB at aggressor gNB, Ptxper-RB=Ptxmax/NDLRB, - Nused-DL-RB is the number of DL RBs scheduled for DL transmission by aggressor gNB, - NDLRB is the total number of DL RBs in the DL subbands - W is the NT×NT normalized identity matrix with unit norm, WF=1, - FFS whether W can be other values and corresponding conditions - FFS for Iselectivity - Note: ACLRBS and ACSBS are in linear scale. In RAN4 reply LS, gNB ACLR (i.e., ACLRBS) is provided as the candidate for TX leakage, and gNB ACS (i.e., ACSBS) is provided as the candidate for Receiver impairment. - Note: the model is based on the assumption that the same transmission power across different DL RBs are used in SLS. This does not prevent companies to use other DL power allocation schemes in SLS. - Note: This model is not applicable to the RBs in the guardband. - Send LS to RAN4 to confirm RAN1’s understanding. Agreement For SLS of SBFD in RAN1, candidate values for αSI at least can be determined based on the assumption that UL receiver sensitivity degradation due to self-interference is 1dB. - FFS: UL receiver sensitivity degradation due to self-interference is 0.8dB and 0.1dB - The value of αSI can be calculated based on the UL receiver sensitivity degradation, noise floor of UL subband and maximum gNB DL Tx Power as below - αSI=PtxmaxISImax*NDLRBNULRB - For example, for sensitivity degradation of 1dB, ISImax can be computed based on 10*log10ISImax+NN=1dB, where N is the noise floor over the UL subband given by N(dB)=-174 + 10*log10 (20e6) + 5 = -96 dBm, assuming 20MHz UL subband and 5dB noise figure. - Note: the feasibility of the determined αSI values can be discussed separately - Companies shall report what values of the individual components are assumed in order to achieve the alpha_SI value corresponding to 1 dB desense - Other approaches of determining values for αSI are not precluded and can be used and reported by companies. Send LS to RAN4 to confirm RAN1’s understanding. Agreement For SBFD deployment case 3-2, reuse the traffic model assumptions of SBFD deployment case 1 as much as possible. - For comparison, the packet arrival rates are kept the same for each corresponding layer in baseline legacy TDD case (i.e., legacy TDD for both Layer 1 and Layer 2) and SBFD deployment case 3-2 (i.e., legacy TDD for Layer 1 and SBFD for Layer 2) respectively. - The UL traffic load and DL traffic load can be independently selected for each layer. Agreement For SBFD deployment case 4, reuse the traffic model assumptions of SBFD deployment case 1 as much as possible. - For comparison, the packet arrival rates are kept the same for each corresponding operator in baseline legacy TDD case (i.e., legacy TDD for both Operator#1 and Operator#2) and SBFD deployment case 4 (i.e., legacy TDD for Operator#1 and SBFD for Operator#2) respectively. - The UL traffic load and DL traffic load can be independently selected for each operator. Agreement Confirm the working assumption made in RAN1#110 on layout related simulation assumptions with modifications (red text). Parameters; Indoor office; Urban macro / Dense Urban Macro layer; Dense Urban with 2-layer (Optional). Layout; Single layer Indoor floor: (12BSs per 120m x 50m); Single layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around.; Two layer Macro layer: - Baseline: Hexagonal grid with 7 macro sites and 3 sectors per site with wrap around - Optional: Hexagonal grid with 19 macro sites and 3 sectors per site with wrap around. Micro layer: 1/3/6/9 Micro BSs per Macro BS, up to companies report. Inter-BS (2D) distance; 20m [TR 38.802 Table A.2.1-11]; 500m for Urban Macro [TR 38.802 Table A.2.1-11] 200m for Dense Urban Macro layer [TR 38.802 Table A.2.1-1]; Macro-to-macro: 200m Minimum Macro-to-micro-center distance: 42m Minimum Micro-center-to-micro-center distance: 40m. Minimum BS-UE (2D) distance; 0m [TR 38.802 Table A.2.1-11]; 35m [TR 38.802 Table A.2.1-11]; Macro-to-UE: 35m Micro-to-UE: 10m [TR 38.802 Table A.2.1-11]. Minimum UE-UE (2D) distance; 1m; 1m; 1m. BS antenna height; 3 m [TR 38.802 Table A.2.1-1]; 25 m [TR 38.802 Table A.2.1-1]; 25m for macro cells and 10m for micro cells [TR 38.802 Table A.2.1-1]. Agreement For UE clustering distribution of Urban Macro and Dense Urban Macro layer, - M users per macro TRP (per direction) - If each UE is either assigned UL traffic or DL traffic (i.e., option 1 of traffic model), there are 2M users per macro TRP, wherein, M UEs are assigned with UL traffic, and the other M UEs are assigned with DL traffic. - If each UE is assigned both UL traffic and DL traffic (i.e., option 2 of traffic model), there are M users per macro TRP. Agreement For UE clustering distribution of Urban Macro and Dense Urban Macro layer, take Alt-2 as baseline and Alt-3 as optional. M; X; Indoor UE height (m). Alt-2; 20; 2; 1.5. Alt-3; 10; 1; 1.5. Agreement Remove square bracket for the traffic load and update the high traffic load from ~50% to ≥50% (i.e., low (<10%), medium (20%-40%) and high (≥50%)) in previous agreement made in RAN1#110. Agreement Confirm the working assumption for gNB-gNB channel model and gNB-UE channel model made in RAN1#110. Agreement Confirm the working assumption for UE-UE channel model made in RAN1#110. Agreement Adopt the following gNB-UE O2I building penetration loss model: - Indoor office: penetration loss is not modelled. - Percentage of high loss and low loss building type for Urban Macro / Dense Urban [refer to table 5B of ITU M.2412]: - 80% low-loss model - 20% high-loss model - Note: The building type is determined by comparing the random variable with P1, where P1 is the probability of the building type with low loss penetration. If the realization of the random variable is less than P1, the building type is low loss; otherwise the building type is high loss [refer to section 5.3.3 of ITU M.2412]. - FFS for 2-layer Scenario B Agreement Adopt the following table for gNB-gNB channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor). gNB-gNB channel model for 2-layer Scenario B. Large-scale channel parameters; FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). - For Indoor factory layer: InF in TR 38.901 (hUE =3m). - Penetration loss is not modelled. - Macro TRP to Indoor TRP: UMa in TR 38.901 (hUE =3m) - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP. Fast fading parameters; FR1: - Macro TRP to Macro TRP: not needed. - Indoor TRP to Indoor TRP: Only the channel model between Indoor TRPs within the same building is considered. - For Indoor office layer: InH-Office in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD. - For Indoor factory layer: InF in TR 38.901 (hUE =3m). ASA and ZSA statistics updated to be the same as ASD and ZSD - Macro TRP to Indoor TRP: UMa O2I in TR 38.901 - Indoor TRP to Macro TRP: same as Macro TRP to Indoor TRP. Agreement Adopt the following table for UE-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor). UE-UE channel model for 2-layer Scenario B. Large-scale channel parameters; FR1: - Outdoor UE to Outdoor UE: - Option 1: A.2.1.2 in TR36.843 (*) - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m) - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: - For Indoor office layer: InH-Office in TR 38.901 (hBS =1.5m). - For Indoor factory layer: InF in TR 38.901 (hBS =1.5m). - Penetration loss is not modelled. - Outdoor UE to Indoor UE: - Option 1: A.2.1.2 in TR36.843 (*). - Option 2: UMi-Street canyon in TR 38.901 (hBS =1.5m). - Penetration loss between UEs follows Table A.2.1-13 in TR38.802. Fast fading parameters; FR1: - Outdoor UE to Outdoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Indoor UE to Indoor UE: Only the channel model between Indoor UEs within the same building is considered - Option 1: A.2.1.2 in TR36.843 (ITU InH), ASD statistics updated to be the same as ASA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - For Indoor factory layer: InF in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA. - Outdoor UE to Indoor UE: - Option 1: 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: UMi-Street canyon in TR 38.901. ASD and ZSD statistics updated to be the same as ASA and ZSA.. (*): For outdoor to indoor case, and indoor to indoor case, use “Remaining Layout Options” in A.2.1.2 of TR36.843 for pathloss calculation, and “ITU-R IMT UMi” for LOS Probability derivation. For outdoor to indoor case, the penetration loss term “20.0+0.5* din” is excluded in pathloss formula given in A.2.1.2 of TR36.843, and the penetration loss is derived according to Table A.2.1-13 in TR38.802.. Agreement For comparison between legacy TDD and SBFD, companies should report the assumption of BS transmit power on DL slots and SBFD slots in SBFD operation. - For calibration purpose, assume the BS transmit power spectrum density is kept the same for SBFD operation and legacy TDD operation. BS transmit power is proportional to the RBs used for DL transmission. Agreement For SBFD Deployment Case 4, different power levels in adjacent carriers can be simulated and it is up to company to report the power levels. Agreement For dynamic TDD evaluations, the following is assumed. Target dynamic/flexible TDD operation; Baseline operation for comparison; UL/DL arrival rate determination method. 1-layer scenario (FR1/FR2-1); Using dynamic TDD UL/DL assignment based on potential enhancements discussed in AI 9.3.3; using dynamic TDD UL/DL assignment based on Rel-17 specifications; UL/DL arrival rate is selected so that network using legacy static TDD {DDDSU} achieves a certain level of Type-2 RU**(i.e., <10%, 20%-40% and ≥50% for low, medium and high load).. 2-layer Scenario B (FR1)*; Layer 2 using legacy static TDD {DSUUU} based on potential enhancements discussed in AI 9.3.3; Layer 2 using legacy static TDD {DDDSU} based on Rel-17 specifications; UL/DL arrival rate is selected for each layer independently so that each layer using legacy static TDD {DDDSU} achieves a certain level of Type-2 RU**(i.e., <10%, 20%-40% and ≥50% for low, medium and high load).. Layer 2 using dynamic TDD UL/DL assignment based on potential enhancements discussed in AI 9.3.3; Layer 2 using dynamic TDD UL/DL assignment based on Rel-17 specifications. *: For 2-layer Scenario B (FR1), layer 1 using legacy static TDD {DDDSU} for both target and baseline operation **: Type-2 RU definition is the same as that defined for SBFD evaluation. Agreement RAN1 to conduct a SLS calibration for evaluation of SBFD operation. - The calibration focuses on the following scenarios of SBFD deployment case 1 - FR1: Urban Macro - FFS: Indoor office - FR2: Dense Urban Macro layer - Regarding metrics used for SLS calibration, consider the following: - gNB-UE coupling loss - Inter-gNB coupling loss - Inter-UE coupling loss - Optional: DL SINR for legacy TDD/ DL SINR in DL-only slots for SBFD - Optional: DL SINR in SBFD slots - Optional: UL SINR for legacy TDD/ UL SINR in UL-only slots for SBFD - Optional: UL SINR in SBFD slots - FFS: the detailed definitions of the metrics listed above Agreement Adopt the following table for gNB-UE channel model for 2-layer Scenario B (HetNet with Urban Macro and Indoor). gNB-UE channel model for 2-layer Scenario B. Large-scale channel parameters; FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Car penetration loss is modelled - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Penetration loss is not modelled. - Macro TRP to Indoor UE: UMa in TR 38.901 - O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered. - Indoor TRP to Outdoor UE: - Option 1: - A.2.1.2 in TR36.843 - Penetration loss between UEs follows Table A.2.1-13 in TR38.802 - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901 - Both Car penetration (for outdoor UE) and O2I penetration loss are modelled, wherein, O2I penetration loss follows TR 38.901 - For the percentage of high loss and low loss building type, 80% low-loss model and 20% high-loss model is considered.. Fast fading parameters; FR1: - Macro TRP to Outdoor UE: UMa in TR 38.901 - Indoor TRP to Indoor UE: the channel model is considered only when the Indoor TRP and Indoor UE are in the same building - For Indoor office layer: InH-Office in TR 38.901 - For Indoor factory layer: InF in TR 38.901 - Macro TRP to Indoor UE: UMa in TR 38.901 - Indoor TRP to Outdoor UE: - Option 1: - 3D UMi, ASD and ZSD statistics updated to be the same as ASA and ZSA. - Option 2: - For Indoor office layer: InH-Office in TR 38.901 [TR 38.828 Table 5.2.1.1.2-1] - For Indoor factory layer: InF in TR 38.901. Agreement When UE clustering distribution is used, - consider the UEs in the same cluster are in the same building - For Alt-2 (M=20, X=2), consider the UEs in different clusters are in different buildings Agreement Remove the square brackets and update the agreement made in RAN1#110 for BS transmit power for legacy TDD as below. For evaluation of SBFD and dynamic/flexible TDD, the following BS transmit power for legacy TDD are considered. These values are for the single operator case. FR1; FR2-1. Urban macro; - Option 1: 53 dBm for 100MHz - Option 2: 49 dBm for 100MHz [refer to TR 38.828 Table 5.2.1.4-1]; N.A.. Dense Urban Macro layer; - Option 1: 53 dBm for 100MHz - Option 3: 44 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 1: [43] 40 dBm for 200MHz 100MHz [refer to TR 38.828 Table 5.2.2.4-1]. Dense Urban Micro layer; - Option 3: 38 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1]; - Option 1: [33] 30 dBm for 200MHz 100MHz. EIRP should not exceed 68 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Indoor hotspot; - Option 2: 24 dBm for 100MHz [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.1.1.2-1]; - Option 1: [23] 23 dBm for 200MHz 100MHz. EIRP should not exceed 58 dBm. [refer to TR 38.802 Table A.2.1-1 and TR 38.828 Table 5.2.2.4-1]. Agreement The following is assumed for SLS calibration of SBFD regarding the BS transmit power for legacy TDD. FR1. Urban macro; - Option 1: 53 dBm for 100MHz. Agreement For evaluation of SBFD and dynamic/flexible TDD, companies report the UE antenna configurations used in their simulations. The UE antenna configurations in the following can be considered for calibration purpose. - FR1: - 2Tx: (M,N,P,Mg,Ng;Mp,Np) = (1,1,2,1,1;1,1), (dH,dV) = (N/A, N/A)λ, 0°,90° polarization - 4Rx: (M,N,P,Mg,Ng;Mp,Np) = (1,2,2,1,1;1,2), (dH,dV) = (0.5, N/A)λ, 0°,90° polarization - FR2-1: - 4Tx/Rx: (M,N,P,Mg,Ng;Mp,Np) = (2,4,2,1,2;1,1); (dH,dV) = (0.5,0.5)λ,(dg,V,dg,H) = (0, 0)λ, 0°/90° polarization; Θmg,ng=90°; Ω0,1=Ω0,0+180° - Note: introduce (Ωmg,ng, Θmg,ng) for orientation of the panel (mg, ng), 0≤mg<Mg, 0≤ng<Ng, where the orientation of the first panel (Ω0,0, Θ0,0) is the same as UE orientation, Ωmg,ng is the array bearing angle and Θmg,ng is the array downtilt angle defined in [TR 36.873]. Agreement For UE clustering distribution of Urban Macro and Dense Urban Macro layer, - R =[25] m, Dmacro-to-cluster = 35m+R, Dinter-cluster = 2R m. Note: the UE cluster is totally confined within the macro cell geographical area (i.e. a cluster cannot be partially overlap with adjacent cell area). For calibration purposes, assume clustering with R=25 Agreement LS on interference modelling for duplex evolution (draft in R1-2210601) is agreed. Final LS in R1-2210602. Agreement Regarding random and uniform UE distribution in Dense Urban Macro layer scenario and Dense Urban Micro layer scenario for FR2-1, consider the following for UE outdoor/indoor proportion: - Baseline: 100% Outdoor without car penetration loss: 3km/h - Optional: 20% Outdoor in cars: 30km/h, 80% Indoor in houses: 3km/h - Outdoor UEs: 1.5 m; - Indoor UEs: 3(nfl – 1) + 1.5; nfl ~ uniform(1, Nfl) where Nfl ~ uniform(4,8) Agreement For SLS evaluation purposes only, Alt 1/2/4 (SBFD UL subband is about 20% of the channel bandwidth) and SBFD Subband configuration#1 with {DUD} pattern, the following is assumed: - For FR1 - Baseline: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <104, 55, 5> - Optional: 100MHz channel bandwidth and 30kHz SCS (273 PRB): < ND, NU, NG > = <106, 51, 5> - For FR2 - Baseline: 100MHz channel bandwidth and 120kHz SCS (66 PRB) < ND, NU, NG > = <25, 14, 1> - Optional: 200MHz channel bandwidth and 120kHz SCS (132 PRB): < ND, NU, NG > = <47, 32, 3> - Other values of < ND, NU, NG > are not precluded and can be reported by companies. Agreement For SBFD evaluation, companies should report the guard symbols assumed in the SBFD operation. Agreement Regarding Option 2 of UE-UE channel model for Dense urban/Urban macro scenarios, use NLOS when two indoor UEs are in different buildings.