3GPP TSG RAN WG1 #111                                                                                      R1-2211580

Toulouse, France, November 14th – 18th, 2022

Agenda Item:	9.4.1.2

Source:	Lenovo

Title:	Physical layer design framework for sidelink on FR1 unlicensed spectrum

Document for:	Discussion

# Introduction

In RAN1#110bis-e meeting, for Rel-18 NR sidelink transmission on unlicensed spectrum (SL-U), the relevant physical layer design framework has been extensively discussed including some fundamental issues, e.g., structure for PSCCH/PSSCH/PSFCH, PSSCH frequency domain resource allocation, guard band usage, PSFCH enhancement as well as SSB designs. The reached agreements are listed below.

Agreement

For interlace RB-based PSCCH/PSSCH transmission in SL-U:

- Regarding 1 sub-channel equals K interlace(s)

  - At least K=1 and K=2 is supported for 15 kHz SCS

  - At least K=1 is supported for 30 kHz SCS

  - FFS: details related to multiple RB sets

Working assumption:

Support maximum 2 candidate starting symbols within a slot for a PSCCH/PSSCH transmission.

- RAN1 strives to have unified design for PSCCH/PSSCH transmission from 1st or 2nd starting symbol

- The candidate starting symbol(s) are intended for AGC purpose

  - FFS: other potential uses of the candidate starting symbol(s)

- FFS other details, e.g., applicable scenarios (including SCS), position of 2nd starting symbol, TBS determination, PSCCH blind decoding complexity, processing time constraints, etc.

- FFS whether 2 candidate starting symbols is also supported for slots with PSFCH

Agreement

To meet OCB and PSD requirement for S-SSB transmission, down-select between the followings for 15 kHz and 30 kHz SCS:

- Option 1: Using interlaced RB transmission for S-PSS/S-SSS/PSBCH

- Option 3: Repetition of S-PSS/S-SSS/PSBCH in frequency domain

- FFS: whether/how the above options apply to all or subset of channel type of S-PSS/S-SSS/PSBCH

- Note: RAN1 further study the relationship between above options and temporary OCB exemption, and the discussion on temporary OCB exemption can continue even if option 1 or option 3 is supported

FFS: how to handle 60 kHz SCS (if needed, not limited to option 1 or option 3)

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- When more than one RB set is used for transmissions, down-select one of the followings

  - Option A: Support that the used interlace index(s) in different RB sets are always the same

  - Option B: Support that the used interlace index(s) in different RB sets can be different

- FFS details

Agreement

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission:

- Down-select one of the followings

  - Option 1: Support explicitly indicating the used sub-channel index(s) and RB set index(s)

  - Option 2: Support explicitly indicating at least the used sub-channel index(s)

    - At least RB set index(s) is not explicitly indicated

- FFS details

Agreement

For PSCCH and PSSCH in SL-U:

- PSCCH is transmitted within 1 sub-channel

- At least support Option 1 below

  - Option 1: PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH

    - Note: the lowest sub-channel may not be entirely contained in the lowest RB set

- FFS whether/how to handle the case where UEs supporting different bandwidths can use the same resource pool to communicate with each other, e.g., whether/how to additionally support Option 2 below

  - Option 2: PSCCH locates in every RB set of corresponding PSSCH

- Note: the above options do not imply any restriction on the mapping of sub-channels to PRBs.

- FFS other details

Agreement

Regarding usage of PRBs within intra-cell guard band of two adjacent RB sets:

- Such PRBs can be used for PSSCH transmission if and only if a UE can transmit on the respective LBT channels after performing channel access procedure in multi-channel case and the UE uses both of these two RB sets for PSSCH transmission

  - FFS details, e.g., handling of potential unequal sub-channel size, for interlaced RB based transmission, whether the PRB(s) in the intra-cell guard band have the same interlace index(s) as the PRBs for PSSCH transmission in these two RB sets

- Such PRBs are not used for PSCCH transmission

  - FFS: whether or not such PRBs are used for PSFCH/S-SSB transmission

Agreement

At least R16/R17 NR SL S-SSB slots are excluded from SL resource pool.

- Note: whether or not additional candidate S-SSB occasions are excluded from resource pool will be discussed after the details of additional candidate S-SSB occasions are clearer

Agreement

At least there is 1 PSFCH occasion per PSCCH/PSSCH transmission, FFS details

Agreement

To address PSFCH transmission dropping due to LBT failure, the followings are to be studied:

- Alt 1: Support more than 1 PSFCH occasion per PSCCH/PSSCH transmission

- Alt 2: PSFCH resources are dynamically indicated

- Alt 3: Convey SL-HARQ feedback information in PSCCH/PSSCH, e.g., new SCI or new MAC-CE

- Alt 4: drop PSFCH transmission

- Alt 5: Support trigger based HARQ feedback reporting for non-numerical HARQ FB and one shot HARQ FB

- Combination of above alternatives are not precluded

- FFS details of above alternatives

Agreement

Regarding additional candidate S-SSB occasions:

- Their number and time domain locations are (pre-)configured or pre-defined

In this contribution, we further elaborate our views on physical layer design framework for sidelink transmission on FR1 unlicensed spectrum.

# Discussion

## Interlace-based structure

Wireless signal transmission on unlicensed spectrum should meet the requirements of regulation subject to the management of the located country/region. Hence, the design of uplink waveform for NR-U PUSCH/PUCCH should meet the regulation requirements on unlicensed spectrum.

The requirements mainly include two aspects:

- Occupied channel bandwidth (OCB): the bandwidth containing 99% of the power of the signal, shall be between 80% and 100% of declared Nominal Channel Bandwidth;

- Maximum PSD (power spectrum density) with a resolution bandwidth of 1MHz, e.g., 10dBm/MHz.

These two requirements dictate that a signal which occupies a small portion of the channel bandwidth cannot be transmitted at the maximum available power at the UE due to the PSD and OCB constraints.

In Rel-16 NR over unlicensed spectrum (NR-U), interlace-based waveform is adopted as uplink waveform for unlicensed spectrum. Each interlace is equally distributed within the whole carrier bandwidth and spans more than 80% carrier bandwidth. The regulation requirements of OCB can be met if a signal or channel can occupy at least one interlace. Moreover, two adjacent PRBs of one interlace are separated by above 1 MHz distance and power boosting can be adopted for each PRB of one interlace. E.g., for 20MHz bandwidth and 15kHz SCS, all the available 106 PRBs are partitioned into 10 interlaces. Each interlace has 11 or 10 PRBs and is equally distributed within the 20MHz bandwidth. In this way, each interlace spans more than 80% system bandwidth so that the regulation requirements of OCB can be met. Moreover, 10 PRBs of one interlace are equally spaced in frequency so that two adjacent PRBs of one interlace are separated by 1.8MHz distance. Hence, one PRB within one interlace can be transmitted with 10dBm maximum power.

Figure 1: Interlace-based structure for 15kHz SCS

Different to Rel-16 NR-U where the minimum resource allocation unit is a interlace, a concept of sub-channel is defined in existing NR sidelink as the minimum resource allocation unit, where a subchannel comprises of multiple contiguous PRBs. Sidelink transmission/reception happens in a resource pool which can be configured with ‘M’ sub-channels with each subchannel including ‘N’ contiguous PRBs. The possible sub-channel size can be configured within the set of {n10, n12, n15, n20, n25, n50, n75, n100}. A resource pool for sidelink does not span across entire sidelink carrier or LBT sub-band. Furthermore, each sub-channel is a self-contained structure containing PSCCH, PSSCH and PSFCH resources.

- PSCCH is located in the lowest interlace of a sub-channel.  From Table 1, number of subchannels in a resource pool of 20MHz of bandwidth and number of remaining PRBs that may not meet the subchannel size are shown. Since the granularity of PSCCH/PSSCH scheduling is one subchannel size then we cannot ignore smaller subchannel size and RAN1 needs to discuss how to treat these remaining PRBs in an RB Set for PSCCH/PSSCH interlacing.

| Subchannel size (PRBs) | No of sub-channels in a resource pool with 20MHz BW (106PRBs @15KHz SCS) | Remaining PRBs (less than the subchannel size) |
| --- | --- | --- |
| 10 | 10 | 6 |
| 12 | 8 | 10 |
| 15 | 6 | 1 |
| 20 | 5 | 6 |
| 25 | 4 | 6 |
| 50 | 2 | 6 |
| 75 | 1 | 31 |
| 100 | 1 | 6 |

Table 1: Sidelink resource pool structure with 20MHz BW (106 RBs@15KHz SCS)

In addition, there are multiple numerologies defined for NR including 15, 30, 60 kHz for FR1. Rel-16 NR-U has agreed the number of interlaces is determined according to the subcarrier spacing, i.e., 10 interlaces with 10 or 11 PRBs per interlace for 15 kHz subcarrier spacing and 5 interlaces with 10 or 11 PRBs per interlace for 30kHz subcarrier spacing. For 60 kHz subcarrier spacing, it is not specified in Rel-16 NR-U for interlace-based structure.

Table 2: Max transmission bandwidth configuration NRB for FR1 (450 – 7125 MHz)

For the combination of 20MHz bandwidth and 60 kHz subcarrier spacing, candidate PRB-based interlace designs are listed in Table 3, wherein M is the number of interlaces and N is the number of PRBs per interlace. Generally, the combination of 4 interlaces and 6 PRBs per interlace can’t meet the regulation requirement of at least 80% occupancy ratio. The combination of 3 interlaces and 8 PRBs per interlace can nearly meet the occupancy ratio requirement. Instead, the combination of 2 interlaces and 12 PRBs per interlace can obviously meet the requirement. The drawback is the frequency separation is 1.44MHz between two adjacent RBs of one interlace so the power boosting target for 10dBm per PRB can’t be met due to the regulation requirements of PSD requirement of 10dBm/MHz. However, the two interlaces provides some scheduling flexibility on two-UE multiplexing.

Table 3: Candidate interlace structure for 20MHz bandwidth and 60 kHz SCS

| SCS | Candidate design | Occupied channel bandwidth per interlace | Percentage of occupied channel bandwidth per interlace over declared channel bandwidth |
| --- | --- | --- | --- |
| 60 kHz (total 24 PRBs) | M = 4, N = 6 | 15.12MHz (21 PRBs) | 75.6% |
|  | M = 3, N = 8 | 15.84MHz (22 PRBs) | 79.2% |
|  | M = 2, N = 12 | 16.56MHz (23 PRBs) | 82.8% |

Hence, for FR1 unlicensed spectrum, all the subcarrier spacings can be supported, i.e., 10 interlaces for 15kHz subcarrier spacing, 5 interlaces for 30kHz subcarrier spacing and 2 interlaces for 60kHz subcarrier spacing.

The interlacing RB based structure for PSFCH interlace is agreed in the previous meeting for 15kHz and 30kHz SCS, the interlacing structure for PSFCH need not be the same as that of the corresponding PSCCH/PSSCH interlacing. The PSFCH interlacing can contain a greater number of interlaces compared to the PSCCH/PSSCH interlace in a resource pool to provide tradeoffs between the common interlace and dedicated interlace and to optimally use the interlaces to meet the OCB requirement. Considering that, then

L PSFCH interlace can be equal to one PSCCH/PSSCH subchannel and L PSFCH interlace can be equal to one PSCCH/PSSCH interlace. After finalizing the PSFCH interlace design, then RAN1 needs to discuss how to configure common interlace and dedicated interlace for PSFCH and the placement of common interlace within an RB set and the content to be transmitted on the common interlace considering all cast type’s HARQ feedback options.

Since the interlace design is already agreed for PSFCH design, then the interlace based RB transmission for PSFCH configuration contains both common and the dedicated interlace, which is provided by Alt-1.

Based on above analysis, we have below proposals:

Proposal 1: RAN1 should discuss the how to use the remaining PRBs in a RB set

Proposal 2: Support PSFCH interlace design containing L PSFCH interlace equals one PSCCH/PSSCH subchannel, placement of common interlace and the content to be transmitted in the common interlace

Proposal 3: Subcarrier spacing of 60kHz is supported for sidelink transmission in FR1 unlicensed spectrum in addition to 15kHz and 30kHz.

Resource pool configuration is usually up to RAN2 to decide and traditionally multiple resource pool considering different priority levels are supported and the selection of resource pool by MAC is according to the priority of the packet to be transmitted and the channel busy ratio. Although a resource pool configuration containing one or more RB sets were agreed at the previous meetings, a resource pool with subset of an RB set is for further study. In our understanding, multiple resource pools within an RB set is not precluded and hence there is no restriction on (pre-)configuring multiple resource pools within an RB set. RAN1 may further study on how to configure multiple resource pool in an RB set considering interlacing RB structure to meet the OCB requirement.

Proposal 4: Support configuring multiple resource pools within an RB set configured with interlaced RB-based structure

## Multiple candidate starting positions for sidelink

When unlicensed spectrum is used for sidelink transmission, for the purpose of fair coexistence, the channel access procedure is still required before any sidelink transmission. Due to unpredictability of LBT, a Tx UE can’t know when the channel is available for sidelink transmission. If Tx UE prepares PSCCH and associated PSSCH only from symbol 0, then when Tx UE performs a successful LBT and obtains the channel from symbol 7 of one slot, the Tx UE has to wait or transmit some reservation signal and starts transmitting the prepared PSCCH/PSSCH from symbol 0 of the next slot. In this way, one half of the slot is wasted. In order to improve the spectrum utilization efficiency, multiple candidate starting positions are needed for sidelink, the more candidate starting positions, the higher spectrum utilization efficiency.

On the other hand, if multiple candidate starting positions are supported, then multiple PSCCHs/PSSCHs corresponding to multiple candidate starting positions in the slot are prepared by Tx UE before performing LBT, which inevitably leads to high complexity on Tx UE. From a Rx UE’s perspective, it has to blind detect PSCCH/PSSCH from the first candidate starting position in a slot. In that sense, the implementation complexity for the Rx UE is high.

It is noted that support of the multiple candidate starting positions is only necessary for initial slot of a COT initiated by Tx UE. After initial slot, there is only a single candidate starting position, i.e., symbol 0, for the subsequent sidelink transmissions. Hence, two or three candidate starting positions can be supported for sidelink transmission for trade-off.

Since multiple starting positions will introduce different length of symbols for between initial slot transmission and subsequent slot transmissions. The TBS determination should be studied to ensure same TBS between multiple transmissions of a TB. A nominal number of symbols for TBS determination could be used and we should study and specify how to determine the nominal number of symbols for TBS determination.

Proposal 5: Confirm the working assumption to support 2 candidate starting positions within a slot for PSCCH/PSSCH transmission in FR1 unlicensed spectrum, and study how to perform TBS determination for different number of symbols caused by multiple starting positions, e.g., nominal number of symbols for TBS determination.

Furthermore, to ease Rx UE complexity due to extra blind detection on additional starting positions or unnecessary blind detection on non-SL signals, a signal can be considered for Rel-18 SL-U. For example, a Tx UE can transmit the signal in the beginning of a COT after successfully performing Type-1 channel access procedure. This signal can be a sequence with high detection probability and predefined for NR sidelink. From Rx UE’s point of view, the Rx UE does not need to blind detect each starting position for decoding PSCCH. It shall not start decoding PSCCH unless it detects the signal in one starting position. Since UE only needs to perform a sequence detection, the UE complexity is quite low. However, the power saving gain is significant since the UE can skip the PSCCH decoding if the channel is occupied by other non-SL systems, e.g., Wi-Fi, LAA, NR-U.

Proposal 6: Study defining a signal for easing UE blind detection complexity and saving UE battery power.

## Frame structure

When unlicensed spectrum is used for sidelink transmission, needless to say, the channel access procedure, also named LBT, is required before any sidelink transmission. Different to SCI decoding-based sensing mechanism, LBT procedure is performed based on energy detection in each sensing slot. In detail, if the detected energy on one channel in one sensing slot is lower than the energy detection threshold, then the channel is deemed as empty or clear or available in that sensing slot; otherwise, the channel is deemed as occupied or non-available in that sensing slot.

For unlicensed spectrum, by performing channel access Type 1, a UE can obtain a channel occupancy (CO) and occupy the channel till the maximum channel occupancy time (MCOT). According to Table 4.2.1-1 in TS37.213, the MCOT is related to channel access priority class (CAPC) value. Consequently, when a UE performs a successful channel access Type 1, the UE can occupy the channel with maximum 2ms, 4ms, 6ms or 10ms, depending on the corresponding CAPC value which is determined based on the traffic priority. The higher the CAPC value, the longer the MCOT on the unlicensed spectrum. As shown in Table 4.2.1-1 in TS37.213, for CAPC value of 3 or 4, the MCOT can be 10ms, which is equivalent to 10 slots in case of 15kHz subcarrier spacing, 20 slots in case of 30kHz subcarrier spacing, 40 slots in case of 60kHz subcarrier spacing.

To avoid any risk of losing the channel, similar to DL burst and UL burst in TS37.213, a sidelink burst can be defined as a set of transmissions from a UE without any gaps greater than 16us. Multiple consecutive slots can be included in the sidelink burst. Each PSCCH/PSSCH starts from symbol 0 of a slot to avoid complicated designs at both Tx UE side and Rx UE side. One example is shown in Figure 2, where a UE initiates a COT via LBT Cat.4 using CAPC 2, then it can contiguously transmit 4 PSCCH/PSSCHs in the four consecutive slots without sensing. So the sidelink burst can include the four slots from symbol 0 of slot n to symbol 9 of slot n+3.

Figure 2: Burst based transmission for sidelink

When unlicensed spectrum is used for sidelink transmission, the channel access procedure is required before any sidelink transmission. It does make sense that a Tx UE initiated COT can be shared to one or more Rx UEs when the requirements of COT sharing is satisfied. One example is shown in Figure 3. After initiating a COT by performing Type 1 channel access procedure, Tx UE, named UE1 for simplicity, performs contiguous sidelink transmissions without any gap larger than 16us in time domain, and then can share some slots within the COT to Rx UE (e.g., UE2) for transmitting PSCCH and associated PSSCH. There are one or more symbols which may be used as gap between two consecutive PSSCHs. If HARQ-ACK feedback is enabled, then PSFCH transmission occasions should be contained within the COT with at least one symbol as gap before or after each PSFCH transmission. Therefore, the structure of the COT is needed for a Rx UE which intends to share the COT for transmitting PSCCH/PSSCH and/or PSFCH. The COT structure should contain the remaining channel duration in terms of number of slots, CAPC or logical channel priority value as indicated by higher layer, over-riding configuration to over-ride the gap symbol, AGC symbol and PSFCH symbol configured as part of the resource pool to perform PSSCH burst transmission, a dedicated configuration provides the placement location for gap symbol and CP-Ext symbol within the COT needed for COT sharing purpose. The COT structure can also provide the dedicated PSFCH symbol placement in a within a COT. The COT sharing indicator can also be transmitted with the COT structure signaling. The Rx UE determines the reference starting slot depending on the slot where the COT structure signaling is successfully received, which is needed to execute the placement of dedicated symbols, over-riding symbols and to calculate the remaining COT duration for the COT sharing purpose.

Figure 3: UE-to-UE COT sharing for sidelink transmission

Based on above analysis, we have below proposals:

Proposal 7: For burst-based sidelink transmission, from a Tx UE’s point of view, no gap symbol is included in between any two consecutive slots

Proposal 8: Each PSCCH/PSSCH is transmitted from symbol 0 of a slot other than initial slot of a COT.

Proposal 9: COT structure information is needed for sidelink transmissions in FR1 unlicensed spectrum.

## HARQ

In Rel-16/17 sidelink transmission, a UE can be provided, by sl-PSFCH-Period, a number of slots in a resource pool for a period of PSFCH transmission occasion resources. If the number is zero, PSFCH transmissions from the UE in the resource pool are disabled.

A UE expects that a slot t'kSL (0≤k<T'max) has a PSFCH transmission occasion resource if k mod NPSSCHPSFCH=0, where t'kSL is defined in [6, TS 38.214], and T'max is a number of slots that belong to the resource pool within 10240 msec according to [6, TS 38.214], and NPSSCHPSFCH is provided by sl-PSFCH-Period.

If a UE receives a PSSCH in a resource pool and the HARQ feedback enabled/disabled indicator field in an associated SCI format 2-A or a SCI format 2-B has value 1 [5, TS 38.212], the UE provides the HARQ-ACK information in a PSFCH transmission in the resource pool. The UE transmits the PSFCH in a first slot that includes PSFCH resources and is at least a number of slots, provided by sl-MinTimeGapPSFCH, of the resource pool after a last slot of the PSSCH reception.

For Rel-18 sidelink transmission on unlicensed spectrum, by performing channel access Type 1, a UE can obtain a channel occupancy (CO) and occupy the channel till the maximum channel occupancy time (MCOT), e.g., occupy maximum several or tens of slots, depending on the corresponding CAPC value and subcarrier spacing.

If legacy periodic PSFCH configuration is used for unlicensed spectrum, when a UE initiates a COT by performing Type-1 channel access procedure, the COT may be frequently interrupted by the periodic PFSCH occasion. This is especially true since the PSFCH period is relatively small, i.e., 1, 2 or 4 slots. One example is shown in Figure 4 with max 4 slots as one PSFCH periodicity. When the COT has a duration of more than 4 slots, like COT 3, it will be interrupted twice. This interruption brings below drawbacks:

- The Tx UE’s sidelink transmissions can’t be contiguously transmitted in time domain because the Tx UE needs to monitor the channel continuously and perform channel access type 2 before it resumes its sidelink transmission.

- Tx UE is quite likely to lose the channel. When the gap is larger than 16us, then there is risk of losing the channel. When the COT is interrupted by one PSFCH occasion and there is no any PSFCH transmitted during the occasion due to NACK-only based feedback or disabled HARQ-ACK feedback, considering one PSFCH occasion occupies 4 consecutive symbols about 280us, the probability of losing the channel is quite high.

Figure 4: Periodic PSFCH configuration

To address aforementioned drawbacks, PSFCH transmission occasions can be dynamically indicated by SCI. In detail, after initiating a COT by performing channel access type 1, a Tx UE can transmit SCI scheduling a PSSCH and dynamically indicating the corresponding PSFCH occasion. The PSFCH transmission occasion can be determined based on a slot level offset between the slot where the corresponding PSSCH is transmitted and the slot where the corresponding PSFCH occasion is included. For each Rx UE intending to transmit PSFCH, based on the indicated PSFCH transmission occasion, the Rx UE can perform Type 2A/2B/2C LBT in case of the indicated PSFCH transmission occasion inside of the COT or Type 1 LBT in case of the indicated PSFCH transmission occasion outside of the COT.

Due to UE processing delay, some PSFCHs may not be responded in same COT for the corresponding PSSCHs. The Tx UE can suspend the transmission of these PSFCHs and trigger the retransmission in a new COT initiated by the Tx UE.

In this way, the PSFCH transmission occasion can be inside of the COT initiated by Tx UE which ensures a high LBT successful probability due to channel access Type 2A/2B/2C required. The ongoing sidelink transmission from Tx UE will be not interrupted by the PSFCH transmission occasion especially when the PSFCH transmission occasion is located at the end of the COT.

Based on above analysis, we have below proposals:

Proposal 10: Support dynamically indicated PSFCH resources for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 11: Support suspending PSFCH when the PSFCH can’t be transmitted in same COT with corresponding PSSCH and triggering it in a new COT.

For sidelink transmission on unlicensed spectrum, if channel access procedure is successful, a Tx UE transmits a PSSCH to a Rx UE and waits for the reception of PSFCH from the Rx UE. Without such HARQ-ACK feedback, the Tx UE has to retransmit the PSSCH even though the Rx UE has correctly decoded the PSSCH. There are two risks that the Tx UE may not receive the PSFCH from the Rx UE:

- LBT risk: Due to unpredicted LBT outcome at Rx UE side, the sidelink HARQ-ACK feedback may not be transmitted if LBT at Rx UE side is failed.

- Hidden node interference: If LBT at Rx UE side is successful, the Rx UE transmits PSFCH to Tx UE. Due to hidden node interference on the shared unlicensed spectrum, the Tx UE may not correctly decode the PSFCH.

Therefore, when one sidelink HARQ-ACK feedback is to be transmitted on an unlicensed carrier, the sidelink HARQ-ACK feedback can be allowed to have multiple transmission opportunities (TOs) and transmitted in the first transmission opportunity with LBT success. One example is shown in Figure 4, where one PSSCH transmitted in slot 1 can have 4 transmission opportunities in slot 4, 8, 12 and 16 with PSFCH periodicity of 4 slots.

Figure 5: Multiple transmission opportunities for one sidelink HARQ-ACK feedback

Proposal 12: sidelink HARQ-ACK feedback can be allowed to have multiple transmission opportunities in FR1 unlicensed spectrum.

## One shot SL HARQ feedback

In Rel-16, as part of the NR-U feature one shot HARQ feedback report transmission with UE transmitting a snapshot of HARQ-ACK report of all HARQ process using PUCCH or PUSCH when a UE receives DCI format 1_1 contain one bit to request the transmission of one shot HARQ feedback.

The sidelink feedback transmission/reception by a UE could be affected due to LBT failures and there exists NR-U features such as one-shot HARQ feedback, non-numerical HARQ feedback timing indicator to counter this problem. Such feature could be studied and requires adaptation for sidelink unlicensed operation.

However, in sidelink the one shot HARQ feedback reporting might need to be associated to a destination, cast types and moreover there is only a limited number of HARQ process between a source-destination pair of UEs as shown in the below Figure 6. Hence the procedure of transmission of SL one shot HARQ feedback should consider different source-destination ids, cast types such as unicast, groupcast, broadcast, HARQ feedback options and blind retransmissions etc., Also reporting one shot HARQ feedback report from Tx UE to gNB for all SL HARQ process should be studied which could be useful when due to LBT failures there could be delay in sidelink HARQ feedback reporting to gNB.

Figure 6: Showing active HARQ processes between UE1, UE2 and UE3

Proposal 13: Support introducing the one-shot HARQ feedback trigger and reporting mechanism for sidelink unlicensed operation.

## Non-numerical feedback timing indicator

Non-numerical value as feedback timing was introduced which eventually postponed the transmission of the generated HARQ feedback while a subsequent HARQ transmission of numerical value as part of the DL DCI triggers the transmission of the previously generated HARQ feedback for those HARQ processes that was transmitted using non-numerical value indication. The Rel16 sidelink HARQ feedback reporting may get delayed due to LBT failures and channel occupancy limitation. Such delay in the sidelink HARQ feedback reception at the Tx UE may also affect the reporting of SL HARQ feedback to gNB.

Currently the feedback timing for sidelink transmission is (pre)configured in a resource pool and not dynamically signalled and there should be some signalling enhancement needed to dynamically indicate the non-numerical feedback timing value to the Rx UEs or destinations to delay the transmission of SL HARQ feedback reporting. Further a trigger needs to be subsequently transmitted to request the transmission of SL HARQ feedback of those SL HARQ process previously indicated with non-numerical HARQ feedback timing value.  Such mechanism may also beneficial for gNB to delay the HARQ feedback transmission from a Tx UE using the non-numerical HARQ feedback timing value in DCI format 3_0.

Proposal 14: Support delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value for sidelink unlicensed operation.

## Resource pool structure

In Rel-16/17 NR sidelink communication, higher layer signaling configuration, synchronization procedure and physical channel structure are designed and specified based on resource pool. The data transmission/reception and resource allocation for both mode 1 and mode 2 are assigned within a resource pool. So, to consider less standardization workload, resource pool relevant design can be reused for sidelink on FR1 unlicensed spectrum.

Proposal 15: To reuse resource pool relevant design for sidelink on FR1 unlicensed spectrum.

## Physical channel design

In Rel-16/17 NR sidelink communication, potential PSCCH is assigned in each subchannel. When scheduling multiple subchannels, PSCCH is allocated in the lowest subchannel in frequency domain. The size of PSCCH in frequency domain can be {10, 12, 15, 20, 25} RBs. In time domain, the PSCCH occupies 2 to 3 symbols.  For Interlace-based structure, potential PSCCH can be still designed to assign in each interlace and to occupy 2 to 3 symbols.

Proposal 16: To study the location of PSCCH in frequency/time domain for interlace-based structure.

Regarding frequency domain resource indication for interlace RB-based PSSCH transmission. In NR-U, when more than one RB set is used for transmission, the used interlace index(s) and RB set index(s) are indicated and the used interlace index(s) in different  RB sets are the same. This legacy indication mechanism can be reused to SL-U, i.e.,  the used interlace index(s) in different RB sets are always the same and explicitly indicating the used sub-channel index(s) and RB set index(s).

Proposal 17: Support that the used interlace index(s) in different RB sets are always same, and explicitly indicating the used sub-channel index(s) and RB set index(s).

For the location of PSCCH, we support the PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH. For the SL transmission spanning on multiple RB sets, to consider the reliability of PSCCH in legacy NR-SL, the reception UE can decode the transmission successfully based on only one PSCCH. There is no need to further pursue the diversity gain and combination gain in frequency domain for PSCCH. If LBT failure, transmitting UE will drop transmission on each RB set, rather than performing transmission on available RB set(s) on which LBT is success. So there is no obvious gain for PSCCH locating in every RB set of corresponding PSSCH.

Proposal 18: Support PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH.

In Rel-16/17 NR sidelink communication, to consider the difference usage scenario, the number of columns of DMRS can be various, i.e., 2 ,3 or 4 columns in one time slot. The time domain location of DMRS can be reused. The frequency domain location of DMRS should be designed based on interlace-based structure.

Proposal 19: To study 2 ,3 or 4 columns of DMRS in one time slot and frequency domain location of DMRS should be designed based on interlace-based structure.

In Rel-16/17 NR sidelink communication, unicast, groupcast opiton1/2 and broadcast were supported. For the sidelink communication on unlicensed band, each cast type should be supported.

Proposal 20: To support unicast, groupcast option 1/2 and broadcast for unlicensed spectrum。

For unicast and groupcast opiton1/2, the feedback information is transmitted on PSFCH channel. The PSFCH region is assigned the last second and third symbols at one time slot (the last symbol and the fourth symbol from last symbol are GP). This kind of design can be reused for unlicensed spectrum. In addition, each HARQ feedback should be transmitted on one interlace but not on RB on unlicensed band, the PSFCH resource may be insufficient. For example, if a UE performs a groupcast option 2 transmission on 20MHz bandwidth, the 20MHz bandwidth may include 10 interlaces in case of 15KHz SCS, that is only 10 PSFCH resources can be provided to maximum10 number UEs in this group. So besides legacy PSFCH resources, to study increasing the PSFCH resource for unlicensed spectrum.

Proposal 21: Besides legacy PSFCH resources, to study increasing the PSFCH resources for unlicensed spectrum.

## Resource allocation mode 1 and mode 2

### Mode 1

In mode 1, as mentioned in WID objective, Uu operation for mode 1 is limited to licensed spectrum only. In this way, legacy scheduling and feedback mechanism on Uu link for mode 1 can be reused. The difference between legacy resource allocation and resource allocation on unlicensed band is that the scheduled UE needs to perform a LBT operation before its scheduled sidelink transmission. The assistance information for LBT operation at UE side may be signaled from network and/or based on UE determination. The content of this assistance information should be studied. In addition, the LBT operation may be failure and the UE behavior and potential solution should be studied.

Proposal 22: Besides legacy scheduling and feedback mechanism, the assistance information and its content for UE performing LBT operation and the potential solution for LBT failure should be studied.

The sidelink grants are allocated by gNB using DCI format 3_0 and then the Tx UE performs LBT before making any transmission on the indicated sidelink resources. DCI field could indicate the channel access priority value together with the sidelink grant and the behavior of Tx UE as a COT initiator could be different in the following ways:

Tx UE after receiving the sidelink grant could autonomously choose the channel access priority class value to perform Cat 4 LBT and accordingly initiate channel occupancy duration. In this method, the selection of channel access priority class value by the Tx UE could be based on the priority/latency of the data in the buffer which is in accordance with the existing LCP rule. In the second method, if the DCI indicates the channel access priority value, then the Tx UE could perform the Cat 4 LBT and accordingly initiate channel occupancy duration.

Proposal 23: RAN1 should discuss the need for signalling the channel access priority value or maximum channel occupancy duration along with the sidelink grant.

The sidelink grants are allocated by gNB using DCI and then the Tx UE performs LBT before making any transmission on the indicated sidelink resources. When LBT fails on all the indicated resources more sidelink resources are required from gNB. How the Tx UE request more resource from gNB using the physical layer signalling should be discussed.

Proposal 24:  LBT failures on all the indicated resources in the sidelink grant may trigger UE to request more resources from gNB, RAN1 should discuss ways to request more resource from gNB

### Mode 2

In mode 2, UE may perform autonomous resource selection for SL transmission. The LBT failure issue is also existing. When the transmission is transmitted on reserved resource within same COT, e.g., for retransmission of the same TB, LBT type 2 is performed for the retransmission, the LBT failure probability may be reduced. But to consider the long period traffic, i.e., the reserved resource is out of an initial COT, the UE should re-initial a COT, the LBT failure probability may be increased. So, the assistance information of a UE performing LBT operation is also needed to be signaled on sidelink. The assistance information can be used by the UE who performs LBT operation, or it can be used by the UE who performs autonomous resource selection.

Proposal 25: The assistance information of a UE performing LBT operation is needed to be signaled on sidelink.

Proposal 26: Study on how to use the signaled assistance information (including its content) for LBT operation and resource selection.

## Multi-slot scheduling

A sidelink device is required to pass a clear channel assessment (CCA) procedure, such as Cat 4 LBT to initiate a channel occupancy in an unlicensed carrier (shared spectrum). The sidelink transmission are allowed to be performed within the maximum channel occupancy duration defined by the respective channel access priority class which is counted or measured from the first transmission after initiation of the COT. Consecutive transmission within he occupied channel duration without gaps or gap less than 25µs is beneficial so that initiator device does not need to perform Cat 2 LBT to reaffirm the channel. In the current sidelink physical layer design upto three non-consecutive sidelink resources could be allocated or reserved within a boundary of 32 slots using the sidelink grant provided by DCI format 3_0 to the Tx UE and PSCCH transmission to the Rx UE while the same upto three resources could be reserved using mode 2. The current resource scheduling mechanism based on configured grant, dynamic scheduling using mode 1 and candidate resource selection, reservation mechanism using mode 2 requires changes to perform consecutive transmission which could be based on multi-slot scheduling.

Proposal 27: RAN1 could study the possibility of assigning more than 3 consecutive resources with a gap less than 25µs using multi-slot scheduling

## S-SSB design

As agreed in RAN1#110bis-e meeting, the number and time domain locations of additional candidate S-SSB occasions are (pre-)configured or pre-defined. The relative distributions between two kinds of S-SSB occasions can be classified into two cases, as illustrated in Figure 6, where legacy (or R16/R17 NR SL) S-SSB occasions are indexed with NL,i, i∈[0..M1-1] and new additional S-SSB occasions are indexed by NA,j, j∈[0..M2-1]. If all the additional candidate S-SSB occasions locate after all the legacy S-SSB occasions within one S-SSB period, the distribution is called case 1. Otherwise, it is called case 2. That is to say, in case 2, there is at least one additional candidate S-SSB occasion locates prior to a certain legacy S-SSB occasion within one S-SSB period. The relative distributions impact design, such as when to trigger utilization of additional candidate S-SSB occasions. Therefore, both cases of relative distributions should be studied as part of S-SSB in FR1 unlicensed spectrum.

Proposal 28: The S-SSB in FR1 unlicensed spectrum should consider following relative distributions between legacy S-SSB occasions and new additional S-SSB occasions:

- Case 1. all the additional candidate S-SSB occasions locate after all the legacy S-SSB occasions within one S-SSB period.

- Case 2. not all the additional candidate S-SSB occasions locate after all the legacy S-SSB occasions within one S-SSB period.

Figure 6: Relative distributions between two kinds of S-SSB occasions within an S-SSB period

Additional candidate S-SSB are added to compensate for the case that some legacy S-SSB occasions are unavailable due to LBT failure. More candidate occasions typically result in more time-domain resources required. As per the motivation of introducing additional candidate S-SSB occasion(s), once S-SSB is transmitted on the legacy S-SSB occasion(s), there is no need to perform S-SSB transmission on new S-SSB occasion(s) within the same S-SSB period. To reduce the impact on spectrum efficiency, it is beneficial to place new S-SSB occasion(s) within resource pool and determine how to use the new S-SSB occasion(s) based on the S-SSB transmission status on legacy S-SSB occasion(s) within the same S-SSB period.

In addition, following two solutions can also be considered. One is to reduce the length of each new S-SSB occasion. On the other hand, to mitigate the cost of performing channel-access, COT-based transmission can be considered when multiple S-SSBs are needed to be transmitted continuously.

Proposal 29: To support additional candidate S-SSB occasion(s) located in resource pool for SL transmission.

Proposal 30: To reduce the impact of additional candidate S-SSB occasion(s) on spectrum efficiency, following can be studied:

- Place new S-SSB occasion(s) within the resource pool and determine how to use the new S-SSB occasion(s) based on the S-SSB transmission status on legacy S-SSB occasion(s) within the same S-SSB period.

- Reduce the duration of each S-SSB occasion, considering such as 4-symbol or 6-symbol S-SSB.

- Consider the structure in supporting COT-based S-SSB transmissions.

As specified in R16/R17 NR SL, in the frequency domain, the S-SSB spans 11 RBs within the SL BWP. Since one RB consists of 12 subcarriers, the S-SSB bandwidth spans 11 × 12 = 132 subcarriers, illustrated by Figure 7. The SLSS (S-PSS/S-SSS) consist of sequences of 127 bits. The SLSS are modulated with BPSK such that each sequence occupies 127 subcarriers in a symbol, which are from the third subcarrier relative to the start of the S-SSB bandwidth up to the 129th subcarrier.

Figure 7: Slot structure for S-SSB in R16/R17 NR SL

As agreed in the last meeting, for 15 kHz and 30 kHz SCS, the down-select range regarding candidate frequency-domain structures is limited to option 1 and option 3. In addition, how to handle 60 kHz SCS (if needed) is not limited to option 1 or option 3.

Option 1: As a method to meet OCB requirement, interlaced transmission is straightforward to be applied for S-SSBs in FR1 unlicensed spectrum. However, different interlaces may contain different number of RBs even for given SCS and bandwidth. For example, as illustrated in Figure 8, each of the first six interlaces contains 11 RBs, while each of the last four interlaces contains only 10 RBs. Consequently, interlaced mapping of S-SSB (11RBs) may need different numbers of interlaces according to the exact interlace(s) used. For the sake of simplicity, interlaced mapping of S-SSB should be fixed in a range, such as the interlace(s) with the least indices. However, interlaced mapping of S-SSB may provide lower reliability compared to using contiguous RBs.

Figure 8: An exemplary interlace pattern

Option 3: S-SSB using contiguous RBs is beneficial for reliability. In this case, to meet OCB requirement, repetition of S-SSB in frequency is quite straightforward. Figure 9 shows an exemplary structure with repetition of S-SSB in frequency, wherein legacy S-SSB structure as specified in R16/R17 NR can be reused without modification. However, this structure may experience lower spectrum efficiency.

Figure 9: An exemplary structure with repetition of S-SSB in frequency

Option 4: Considering the broader bandwidth available in unlicensed band, new S-SSB structures can be obtained by extension in the frequency domain and reduction in the time domain. Figure 10 shows two exemplary 6-symbol structures. For both structures, the first symbol (or AGC symbol) and the last symbol contain only PSBCH, while the second to fifth symbols contain S-PSS and S-SSS respectively, in addition to PSBCH. Both structures differ from each other depending on the location of SLSS (S-PSS/S-SSS), where PSBCHs can be deployed on one side of SLSS (left part of the figure), alternatively PSBCHs can be deployed on both sides of SLSS (right part of the figure). To meet OCB requirement, repetition of S-SSB components (SLSS/PSBCH) as illustrated in the figure can be used. This new structure is beneficial for obtaining more S-SSB occasions for given time-domain resources. On the other hand, this structure can guarantee the coverage requirement by repeating SLSS (S-PSS/S-SSS) in two consecutive symbols. Consequently, 6-symbol S-SSB structure can achieve a balance between shorter length of S-SSB occasion and desired coverage.

Figure 10: Exemplary 6-symbol structures for S-SSB with broader bandwidth

Proposal 31: For 15 kHz and 30 kHz SCS, the following can be considered for the two candidate frequency structures for S-SSB transmission.

- For option 1, interlaced mapping of S-SSB should be fixed in a range, such as the interlace(s) with the least indices.

- For option 3, either interlaced mapping or using continuous RBs can be considered.

Proposal 32: The option 4 is beneficial for handling 60 kHz SCS, where 6-symbol S-SSB structure can achieve a balance between shorter length of S-SSB occasion and desired coverage.

## Congestion control mechanism

Congestion control mechanism was introduced in Rel-14 LTE-V2X and Rel-16/17 NR sidelink. The motivation of congestion control is that UE can adjust its transmission power and resource based on its own CBR measurement. In detailed, in mode 1, the network can adjust the resource configuration appropriately based on reported CBR from measurement UE. In mode 2, UE can autonomously adjust its own transmission parameters based on congestion control mechanism which is a function of CBR measurement. For SL-U, the resource pool may be not only used by 3GPP device, but the resource also can be obtained by other RAT, e.g., Wi-Fi system, based on channel access result. In this way, congestion control mechanism should be further supported to optimize in high traffic load for 3GPP system, and to study the potential measurement impact caused by the other RAT’s transmission.

Proposal 33: To support congestion control mechanism for SL-U.

Proposal 34: To study the potential measurement impact caused by the other RAT’s transmission.

# Conclusion

In this contribution, we focus on the physical layer framework for sidelink transmission on FR1 unlicensed spectrum and have below observations and proposals:

Proposal 1: RAN1 should discuss the how to use the remaining PRBs in a RB set

Proposal 2: Support PSFCH interlace design containing L PSFCH interlace equals one PSCCH/PSSCH subchannel, placement of common interlace and the content to be transmitted in the common interlace

Proposal 3: Subcarrier spacing of 60kHz is supported for sidelink transmission in FR1 unlicensed spectrum in addition to 15kHz and 30kHz.

Proposal 4: Support configuring multiple resource pools within an RB set configured with interlaced RB-based structure.

Proposal 5: Confirm the working assumption to support 2 candidate starting positions within a slot for PSCCH/PSSCH transmission in FR1 unlicensed spectrum, and study how to perform TBS determination for different number of symbols caused by multiple starting positions, e.g., nominal number of symbols for TBS determination.

Proposal 6: Study defining a signal for easing UE blind detection complexity and saving UE battery power.

Proposal 7: For burst-based sidelink transmission, from a Tx UE’s point of view, no gap symbol is included in between any two consecutive slots.

Proposal 8: Each PSCCH/PSSCH is transmitted from symbol 0 of a slot other than initial slot of a COT.

Proposal 9: COT structure information is needed for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 10: Support dynamically indicated PSFCH resources for sidelink transmissions in FR1 unlicensed spectrum.

Proposal 11: Support suspending PSFCH when the PSFCH can’t be transmitted in same COT with corresponding PSSCH and triggering it in a new COT.

Proposal 12: sidelink HARQ-ACK feedback can be allowed to have multiple transmission opportunities in FR1 unlicensed spectrum.

Proposal 13: Support introducing the one-shot HARQ feedback trigger and reporting mechanism for sidelink unlicensed operation.

Proposal 14: Support delaying the generation and transmission of SL HARQ feedback using non-numerical HARQ feedback timing value for sidelink unlicensed operation.

Proposal 15: To reuse resource pool relevant design for sidelink on FR1 unlicensed spectrum.

Proposal 16: To study the location of PSCCH in frequency/time domain for interlace-based structure.

Proposal 17: Support that the used interlace index(s) in different RB sets are always same, and explicitly indicating the used sub-channel index(s) and RB set index(s).

Proposal 18: Support PSCCH locates in the lowest sub-channel of lowest RB set of corresponding PSSCH.

Proposal 19: To study 2 ,3 or 4 columns of DMRS in one time slot and frequency domain location of DMRS should be designed based on interlace-based structure.

Proposal 20: To support unicast, groupcast option 1/2 and broadcast for unlicensed spectrum.

Proposal 21: Besides legacy PSFCH resources, to study increasing the PSFCH resources for unlicensed spectrum.

Proposal 22: Besides legacy scheduling and feedback mechanism, the assistance information and its content for UE performing LBT operation and the potential solution for LBT failure should be studied.

Proposal 23: RAN1 should discuss the need for signalling the channel access priority value or maximum channel occupancy duration along with the sidelink grant.

Proposal 24:  LBT failures on all the indicated resources in the sidelink grant may trigger UE to request more resources from gNB, RAN1 should discuss ways to request more resource from gNB.

Proposal 25: The assistance information of a UE performing LBT operation is needed to be signaled on sidelink.

Proposal 26: Study on how to use the signaled assistance information (including its content) for LBT operation and resource selection.

Proposal 27: RAN1 could study the possibility of assigning more than 3 consecutive resources with a gap less than 25µs using multi-slot scheduling.

Proposal 28: The S-SSB in FR1 unlicensed spectrum should consider following relative distributions between legacy S-SSB occasions and new additional S-SSB occasions:

- Case 1. all the additional candidate S-SSB occasions locate after all the legacy S-SSB occasions within one S-SSB period.

- Case 2. not all the additional candidate S-SSB occasions locate after all the legacy S-SSB occasions within one S-SSB period.

Proposal 29: To support additional candidate S-SSB occasion(s) located in resource pool for SL transmission.

Proposal 30: To reduce the impact of additional candidate S-SSB occasion(s) on spectrum efficiency, following can be studied:

- Place new S-SSB occasion(s) within the resource pool and determine how to use the new S-SSB occasion(s) based on the S-SSB transmission status on legacy S-SSB occasion(s) within the same S-SSB period.

- Reduce the duration of each S-SSB occasion, considering such as 4-symbol or 6-symbol S-SSB.

- Consider the structure in supporting COT-based S-SSB transmissions.

Proposal 31: For 15 kHz and 30 kHz SCS, the following can be considered for the two candidate frequency structures for S-SSB transmission.

- For option 1, interlaced mapping of S-SSB should be fixed in a range, such as the interlace(s) with the least indices.

- For option 3, either interlaced mapping or using continuous RBs can be considered.

Proposal 32: The option 4 is beneficial for handling 60 kHz SCS, where 6-symbol S-SSB structure can achieve a balance between shorter length of S-SSB occasion and desired coverage.

Proposal 33: To support congestion control mechanism for SL-U.

Proposal 34: To study the potential measurement impact caused by the other RAT’s transmission.

# References

- RP-213678, “New WID on NR Sidelink evolution”, RAN#94-e, Dec. 2021.

- Chairman’s notes on RAN1#110bis-e