3GPP TSG-RAN WG1 #111	R1-2211642

# Toulouse, France, November 14th – 18th, 2022

| CR-Form-v12.0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.213 | CR | xxxx | rev | - | Current version: | 17.3.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR on PUCCH determination for NACK-only mode 1 with one bit |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | ZTE |  |  |  |  |  |  |  |  |  |
| Source to TSG: | - |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_MBS-Core |  |  |  |  |  | Date: |  |  | 2022-11-14 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) Rel-17 (Release 17) |  |
| Reason for change: |  | The higher layer parameter moreThanOneNackOnlyMode should inlcude 1-bit NACK-only feedback. It means that the PUCCH resource determination for 1-bit NACK-only are different when the higher layer parameter moreThanOneNackOnlyMode is provided (i.e., NACK-only mode 2) or not provided (i.e., NACK-only mode 1). In the current specification TS 38.213, 1-bit NACK-only PUCCH resource can be determined according to Table 18-1 for NACK-only mode 2. In addition, it is also agreed that 1-bit NACK-only PUCCH resource is determined based on the PRI indication for NACK-only mode 1. Therefore, PUCCH resource determination for 1-bit NACK-only for mode 1 should be added. Agreement For the separate PUCCH-Config/ PUCCH-ConfigurationList configured to UE for NACK-only based feedback, - 1 PUCCH resource set in each PUCCH-Config. - up to 32 PUCCH resources in PUCCH resource set - Note: the separate PUCCH-Config/PUCCH-ConfigurationList applies to all configured G-RNTIs configured with NACK-only based feedback. Agreement For PRI included in DCI format 4_1/4_2, - if the G-RNTI is configured with NACK-only mode1, PRI indicates the PUCCH for the HARQ-ACK feedback including the case of one TB in scheduled. - If the G-RNTI is configured with NACK-only mode2, - FFS on whether/how to interpret the PRI.. |
| Summary of change: |  | Add the condition of NACK-only mode 1 and the corresponding PUCCH resource determination for 1-bit NACK-only |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The PUCCH resource determination is unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 18 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | Isolated impact analysis: |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | This is the first version for this CR. |  |  |  |  |  |  |  |  |

- Multicast Broadcast Services

<Unchanged parts are omitted>

For the second HARQ-ACK reporting mode, when moreThanOneNackOnlyMode is not provided and a number of HARQ-ACK information bits is one, a UE transmits a PUCCH only when the HARQ-ACK information bit has NACK value. For a PUCCH resource associated with PUCCH format 0, the UE transmits the PUCCH as described in [4, TS 38.211] by obtaining m0 as described for HARQ-ACK information in clause 9.2.3 and by setting mcs=0. For a PUCCH resource associated with PUCCH format 1, the UE transmits the PUCCH as described in [4, TS 38.211] by setting b0=0. The PUCCH resource determination is based on a PUCCH resource indicator field in DCI format 4-1/4-2 from the PUCCH set 0 provided by pucch-ConfigurationMulticast2/pucch-ConfigurationListMulticast2.

< Unchanged parts are omitted >