3GPP TSG RAN WG1 #111	R1-2212423

Toulouse, France, 14 – 18 November 2022

Agenda item:		9.2.3.1

Source:	CEWiT

Title:	Evaluation on AI/ML for beam management

Document for:		Discussion and Decision

# Introduction

In RAN#94 plenary meeting, a new SID on AI/ML for air-interface was approved [1]. One of the objectives of the SID was to evaluate the performance benefits of AI/ML based algorithms for agreed use cases, including CSI feedback enhancements, beam management and positioning accuracy enhancements. For AI/ML based beam management two cases are considered: spatial domain beam prediction and temporal domain beam prediction [2].

Agreement

- BS antenna configuration:

  - antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ

  - Other assumptions are not precluded

- BS Tx power for evaluation:

  - 40dBm (baseline)

  - Other values (e.g. 34 dBm) are not precluded and can be reported by companies

- UE antenna configuration (Clarification of agreement in RAN 1 #110):

  - antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right)

  - Other assumptions are not precluded

Agreement

- For the evaluation of both BM-Case1 and BM-Case2, 32 or 64 downlink Tx beams (maximum number of available beams) at NW side.

  - Other values, e.g., 256, etc, are not precluded and can be reported by companies.

- For the evaluation of both BM-Case1 and BM-Case2, 4 or 8 downlink Rx beams (maximum number of available beams) per UE panel at UE side.

  - Other values, e.g., 16, etc, are not precluded and can be reported by companies

Agreement

- Study the following options on the selection of Set B of beams (pairs)

  - Option 1: Set B is fixed across training and inference

  - Option 2: Set B is variable (e.g., different beams (pairs) patterns in each time instance/report/measurement during training and/or inference), FFS:

    - Opt A: Set B is changed following a set of pre-configured patterns

    - Opt B: Set B is randomly changed among pre-configured patterns

    - Opt C: Set B is randomly changed among Set A beams (pairs)

    - The number of beams(pairs) in Set B can be fixed or variable

    - Note: BM-Case1 and BM-Case2 may be considered for different option.

  - Other options are not precluded.

Following agreements were made in the RAN1#110b-e meeting [4].

Agreement

- For BM Case-1 and BM Case 2, to verify the generalization performance of an AI/ML model over various scenarios/configurations, the set of scenarios/configurations are considered focusing on one or more of the following aspects as a starting point:

  - Scenarios

    - Various deployment scenarios

    - Various outdoor/indoor UE distributions

    - Various UE mobility

  - Configurations

    - Various UE parameters

    - Various gNB settings

    - [Various Set B of beam(pairs)]

  - Other aspects of scenarios/configurations are not precluded

  - The selected scenarios/configurations for generalization verification may consider the AI model inference node (e.g., @UE or @gNB) and use case (e.g., BM-Case1, or BM-Case2)

- Companies to report the selected scenarios/configurations for generalization verification

- Note: other approaches for achieving good generalization performance for AI/ML-based schemes are not precluded.

Agreement

- The options to evaluate beam prediction accuracy (%):

  - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam”

  - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams”

  - Top-1/K (%) (Optional): the percentage of “the Top-1 predicted beam is one of the Top-K genie-aided beams”

  - Where K >1 and values can be reported by companies.

Agreement

- For DL Tx beam prediction, the definition of Top-1 genie-aided Tx beam considers the following options

  - Option A, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B, the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

Agreement

- For DL Tx-Rx beam pair prediction, the definition of Top-1 genie-aided Tx-Rx beam pair considers the following options:

  - Option A: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx and Rx beams

  - Option B: The Tx-Rx beam pair that results in the largest L1-RSRP over all Tx over all Tx beams with specific Rx beam(s)

    - FFS on specific Rx beam(s)

    - Note: specific Rx beams are subset of all Rx beams

In this contribution we present our evaluation results of beam prediction in spatial domain and our views on the evaluation methodologies and KPI’s.

# Evaluation Results

## Evaluation Assumptions

For generating the dataset, we used a system level simulation and in our simulations we followed the simulation parameters that were agreed in the previous RAN1 meetings [2][3][4]. The detailed simulation assumptions are given in the Appendix.

For generating the dataset, we considered a 21-cell layout with 3GPP Uma scenario with 200m inter-site distance with 10 UEs per cell. Using this setup, we performed 1000 random UE drops and thereby generating a total of 210000 samples. Out of the samples generated, 80% of the samples were used for training, 10% for validation and 10% for testing.

The gNB is equipped with a single panel generating a total of 32 beams and the UE uses a total of 8 receive beams, which consist of 4 beams in 2 panels. The beam configuration used for the dataset generation is summarized in the Appendix.

For the selection of beams for Set B, we assume that Set B is fixed across both training and inference. As for the number of beams pairs in Set B, we consider multiple values to analyze the tradeoff between performance and overhead reduction

For the AI/ML model structure, we choose a fully connected sequential model with 3 dense layers and a dropout layer. Each hidden layer has 256 nodes and use ReLU as the activation function. The lost function used was categorical cross entropy and an Adam optimizer was used to minimize the loss. The RSRP values were normalized to have zero mean and unit variance before giving it as input to the AI/ML model The details of the AI/ML model are summarized in Table 1.

| Parameter | Parameter Value |  |
| --- | --- | --- |
| Model description | Sequential model with 3 dense layers of 256 nodes, ReLU, & Dropout |  |
| Loss function | Categorical cross entropy |  |
| Optimization function | ADAM with learning rate of .001 |  |
| Dataset size | 210000 samples, 90% training, 10% validation, 10% testing |  |
| Model inputs | RSRP measurement from Set B beams |  |
| Model output | Probability distribution over beams being best beam |  |
| Batch size | 256 |  |
| Model Complexity | Number of Parameter | 164608 |
|  | Floating point operations | 165 FLOPs |

Table 1: AI/ML model details

## Results

In this contribution we report the following KPI’s:

- Beam prediction accuracy:

  - Top-1 (%): the percentage of “the Top-1 genie-aided beam is Top-1 predicted beam

  - Top-K/1 (%): the percentage of “the Top-1 genie-aided beam is one of the Top-K predicted beams, for K=3, 5, and 10.

  - For the definition of Top-1 genie-aided beam, we select option A, i.e., , the Top-1 genie-aided Tx beam is the Tx beam that results in the largest L1-RSRP over all Tx and Rx beams

- CDF of L1-RSRP difference between the predicted best beam and genie-aided best beam.

- Average L1-RSRP difference between the predicted best beam and genie-aided best beam.

- RS overhead reduction:

  - For the definition of RS overhead reduction, we select the given equation,

RS overhead reduction [%] = 1-NM

  - N is the number of beam/beam pairs in Set B

  - M is the total number of beams/beam pairs to be predicted.

The baseline scheme that we use this exhaustive beam sweeping, where the best beam is selected based on the measurement from all the beams on Set A.

For the evaluation we consider two cases listed below

Case 1: In this case the AI/ML model is used to predict the best beam pair. Set A consists of all the L1-RSRP measurement for all the possible Tx and Rx beam pairs and Set B consists of L1-RSRP measurements from a subset of transmit-receive beam pairs. The size of Set B is fixed a 128 beam pairs measurement which is made by transmitting CSI-RS using 16 different transmit beams. The UE takes measurement using 8 different receive beams.

Case 2: In this case the AI/ML model is used to predict the best transmit beams. For this case we assume that the UE reports the best UE-side receive beam. Set A consist of L1-RSRP using the best UE-side receive beams for all the possible transmit beams, which makes a total of 32 L1-RSRP values as per the evaluation assumption. The size of Set B is fixed to be 16.

The results are summarized in Table 2.

|  | Set sizes |  | Overhead reduction | Average L1-RSRP difference (dB) | Top-1 accuracy | Top-3 accuracy | Top-10 accuracy |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Set A | Set B |  |  |  |  |  |
| Baseline (Exhaustive beam sweeping) | 256 | 256 | 0% | 0% | 100% | 100% | 100% |
| Case 1 | 256 | 128 | 50% | 0.2251 | 68.00% | 90.34% | 98.05% |
| Case 2 | 32 | 16 | 50% | 0.0832 | 83.93% | 99.43% | 99.96% |

Table 2: Evaluation results

Figure 1 RSRP Difference between best beam and best predicted beam.

In Figure 1, the CDF plot for RSRP difference between the best beam and best predicted beam is shown. The mean RSRP difference is <1dB for all the scenarios.

The results show that the spatial domain Tx prediction performs better than spatial domain Tx-Rx beam pair prediction. For example, for joint Tx-Rx beam pair prediction achieves a Top-1 accuracy of 68% with mean RSRP difference of 0.2dB, while Tx beam prediction achieves a Top-1 accuracy of 83% with mean RSRP difference of 0.08dB.

Observation 1: Compared to the spatial domain Tx-Rx beam pair prediction, spatial domain Tx beam prediction achieves a better accuracy with lower mean RSRP difference.

# Conclusion

In this contribution we performed evaluation for AI/ML for spatial domain beam prediction. Based on the evaluation results, we make the following observations.

Observation 1: Compared to the spatial domain Tx-Rx beam pair prediction, spatial domain Tx beam prediction achieves a better accuracy with lower mean RSRP difference.

# References

- RP-213599, New SI: Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, 3GPP TSG RAN Meeting #94e

- Chairman’s Notes, RAN1#109-e meeting

- Chairman’s Notes, RAN1#110, Toulouse, France, 22-26 August 2022

- Chairman’s Notes, RAN1#110bis-e, e-meeting, 10-19 October 2022

- R1-2207068, Evaluation on AI/ML for beam management, CEWiT

- R1-2210107, Evaluation on AI/ML for beam management, CEWiT

# Appendix

| Parameters | Values |
| --- | --- |
| Frequency Range | FR2 @ 30 GHz - SCS: 120 kHz |
| Deployment | 200m ISD, - 2-tier model with wrap-around (7 sites, 3 sectors/cells per site) |
| Channel mode | UMa with distance-dependent LoS probability function defined in Table 7.4.2-1 in TR 38.901. |
| System BW | 80MHz |
| UE distribution | - For spatial domain beam prediction: - Option 2: 100% outdoor - For time domain prediction: 100% outdoor - 10 UE per sector |
| BS Antenna Configuration | antenna setup and port layouts at gNB: (4, 8, 2, 1, 1, 1, 1), (dV, dH) = (0.5, 0.5) λ Azimuth angles (degrees) = [-78.75, -56.25, -33.75, -11.25, 11.25, 33.75, 56.25, 78.75] Zenith angles (degrees) = [85.5, 94.5, 99, 103.5] |
| UE Antenna Configuration | antenna setup and port layouts at UE: (1, 4, 2, 1, 2, 1, 1), 2 panels (left, right) Azimuth angles (degrees) = [ -67.5, -22.5, 22.5, 67.5] |
| BS Tx Power | 40dBm (baseline) |
| Maximum UE Tx Power | 23 dBm |
| BS receiver Noise Figure | 7 dB |
| UE receiver Noise Figure | 10 dB |
| Inter site distance | 200m |
| BS Antenna height | 25m |
| UE Antenna height | 1.5 m |
| Car penetration Loss | 38.901, sec 7.4.3.2: μ = 9 dB, σp = 5 dB |