3GPP TSG RAN WG1 Meeting #111					                     R1-22xxxxx

Toulouse, France, November 14th – 18th, 2022

Agenda item:	8.11

Source:		Moderator (LG Electronics)

Title:	Feature lead summary #1 for AI 8.11: Inter-UE coordination for Mode 2 enhancements and others

Document for:	Discussion and information

- Introduction

In the RAN#110bis-e meeting, the following yellow-marked conclusions were made on the IUC related maintenance issues that WILL or WILL NOT be addressed at this meeting.

Conclusion of [110bis-e-R17-SL-01]: For Rel-17 NR sidelink enhancement maintenance on resource allocation for power saving, based on the issues described in R1-2210333: - Discuss issues 1-6, 1-7, 1-9 - Discuss the following editorial issues for providing to spec editors: issues 1-15, 1-16, 1-17 For Rel-17 NR sidelink enhancement maintenance inter-UE coordination for mode 2 enhancements and other issues, based on the issues described in R1-2210333: - Discuss issues 2-15, 2-4, 2-10, 3-1, 3-3 - Discuss the following editorial issues for providing to spec editors: issues 2-2, 2-7, 2-8, 2-9, 2-17, 2-11, 2-12, 2-13 The following issues in R1-2210333 are postponed and will be treated at RAN1#111: issues 1-2, 1-10, 1-12, 1-13, 2-1. The following issues in R1-2210333 will not be treated at RAN1#111: issues 1-3, 1-4, 1-5, 1-8, 2-3, 2-5, 2-20, 2-21, 2-22.

The maintenance issues for IUC and SL Power Control in the contributions submitted to the RAN1#111 meeting are summarized in the tables below. Note that if an issue in a contribution submitted at the last meeting is resubmitted in this meeting, its issue numbering is the same as R1-2210333 and it is marked in CYAN in the table below. On the other hand, there were several companies that submitted contributions on the IUC related issues that WILL NOT be addressed in this meeting as per the conclusions of the last meeting mentioned above, and those issues are marked in PURPLE. The newly raised issues at this meeting are marked in GREEN.

| Issue# | Description | References |
| --- | --- | --- |
| 2-1 | [Scheme 2] Further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set Conflict information receiver flag to 1 | [1], [9], [11], [16] |
| 2-6 | [Scheme 1] Further clarification on IUC related transmission based on latency bound | [3] |
| 2-14 | [Scheme 1] Modification to UE-B’s behavior of excluding the non-preferred resource set from its candidate single-slot resources | [5], [6] |
| 2-19 | [Scheme 2] Deletion of duplicated part on UE procedure for determining a resource conflict between TS 38.213 (Section 16.3.0) and TS 38.214 (Section 8.1.4B). | [8], [12] |
| 2-16 | [Scheme 1] Modification to Step 6) in Section 8.1.4 of TS 38.214 used when UE-A determines a preferred resources set and the time gap from IUC transmission to the preferred resource is larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL) | [10] |
| 2-15 | [Scheme 1] Further clarification on the use of preferred resource set for resource reselection due to pre-emption/re-evaluation/conflict information | [13] |
| 2-18 | [Scheme 2] Correction/clarification on description of valid PSFCH occasion for Scheme 2 in TS 38.213 | [18], [22] |
| 2-23 | [Scheme 1] Further clarification that UE-A is the destination UE of a TB transmitted by UE-B for the case when IUC information is triggered by an explicit request from UE-B and UE-A determines the set of non-preferred resources for UE-B | [21] |
| 2-20 | [Scheme 1] Further clarification on condition(s) under which Option B can be used for the received preferred resource set | [13] |
| 2-21 | [Scheme 1] Addition of procedure that allows UE-B to distinguish between non-preferred resources generated based on different conditions (i.e., Condition 1-B-1, Condition 1-B-2) | [14] |
| 2-24 | [Scheme 1] Further clarification on cast type for IUC scheme 1 | [7] |
| 2-25 | [Scheme 2] Further clarification on UE procedure for receiving PSFCH with conflict information | [17] |
| 2-26 | [Scheme 2] Correction of misaligned RRC parameter name of indicationUEBScheme2 | [20] |
| 3-2 | Further clarification on which set of SL power control parameters is used by UE | [2], [4], [15], [19] |

- Discussion of selecting essential issues to be addressed at this meeting

The following classification will be used to discuss a list of essential issues that should be addressed at this meeting:

- High priority (H):

  - High-priority item (essential, pending issues, broken spec components) and proposed editorial changes that either enhance the clarity of the specs or correct mistakes

- Non-essential (N):

  - All other purposes such as spec optimization and low priority issues

- Editorial (E):

  - Editorial issues that will be handled as editorial CRs (to be communicated to the editors/chairs)

  - Issue 2-1

The initial opinion of FL on Issue 2-1 is as follows:

- H

  - This is because the conclusion was made at the last RAN1 meeting that Issue 2-1 will be dealt with in this meeting.

[Question #1]: Companies please provide their views on whether Issue 2-1 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| OPPO | Yes, H | Respect the conclusion of last meeting. |
| Qualcomm | OK |  |
| vivo | OK |  |
| Spreadtrum | H |  |
| DCM | OK |  |
| CATT, GOHIGH | Yes, H |  |
| Huawei, HiSilicon | H | Ok to address a missing case that there is only one UE whose occasion has not passed and supports Scheme2. |
| Nokia, NSB | Yes H |  |
| xiaomi | Yes |  |
| Ericsson | Yes |  |

    - Summary of companies’ inputs to Issue 2-1 & FL’s suggestion

All companies that provided inputs supported addressing Issue 2-1 at this meeting. Considering this situation, FL’s suggestion is to have discussions and make conclusions for Issue 2-1 in this meeting.

  - Issue 2-6

The initial opinion of FL on Issue 2-6 is as follows:

- N

  - This proposal is to add a new feature for further optimization, which is not desirable at this late maintenance phase, but I would like to check the views of other companies.

[Question #1]: Companies please provide their views on whether Issue 2-6 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | No | Agree with FL’s assessment. |
| OPPO | No | How to use the parameter sl-LatencyBoundIUC-Report has been captured in 5.22.1.10 of 36.321, the addition is not necessary. |
| Qualcomm | No |  |
| Vivo | H | This CR is not to introduce a new feature. Rather, it is intended to clarify in the spec for the latency bound introduced in RAN2, similar as the relationship between CSI-RS and CSI feedback. Otherwise, without this a mapping relationship, the correspondence between request and IUC feedback is not clear. The UE behaviors would not be clear when the UE-B UE sending multiple IUC requests, nor the UE-A sending multiple IUC messages. Also, for the request and IUC transmission procedure, current spec is not clear. The procedure in 38.321 refer to 38.214 regarding IUC request and IUC transmission procedure, but 38.214 does not describe such procedure |
| Spreadtrum | No | At least UE-A’s behavior should not limited within latency Bound. It is an optimization obviously. |
| DCM | H | In our understanding, this is not a new feature as commented by vivo. We are fine to add some text as CSI report in R16, but simpler text in PHY spec would be better as for CSI report. |
| CATT, GOHIGH | Yes, H | We think this issue should be discussed, and provide a clear conclusion. Even if it cannot be discussed in this meeting, it could be discussed in next meeting. |
| Huawei, HiSilicon | No | Not needed. Latency bound is already captured in higher layer spec 38.321 5.22.1.10 IUC-Information Reporting. (below is copied from 38.321) The MAC entity shall for each pair of the Source Layer-2 ID and the Destination Layer-2 ID corresponding to a PC5-RRC connection which has been established by upper layers: 1> if the SL-IUC Information reporting has been triggered by an SL-IUC Request MAC CE (and/or an SCI) and not cancelled: 2> if the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting is not running: 3> start the sl-IUC-ReportTimer. 2> if the sl-IUC-ReportTimer for the triggered SL-IUC Information reporting expires: 3> cancel the triggered SL-IUC Information reporting. |
| Nokia, NSB | No |  |
| xiaomi | No | We also think a new feature for further optimization is unnecessary. |
| Ericsson | No |  |

    - Summary of companies’ inputs to Issue 2-6 & FL’s suggestion

8 out 11 companies that provided inputs did not support Issue 2-6 being addressed at this meeting. Considering this situation, FL thinks that there is no consensus to have discussions and make conclusions for Issue 2-6 in this meeting.

  - Issue 2-14

The initial opinion of FL on Issue 2-14 is as follows:

- N

  - This proposal is to add a new feature for further optimization, which is not desirable at this late maintenance phase, but I would like to check the views of other companies.

[Question #1]: Companies please provide their views on whether Issue 2-14 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | No | Agree with FL’s assessment. |
| OPPO | No | The current specification is aligned with RAN 1 agreement. |
| Qualcomm | OK | We are ok to discuss the issue |
| vivo | N |  |
| Spreadtrum | No |  |
| DCM | OK | OK to discuss |
| CATT, GOHIGH | Yes, H | From our understanding, this issue is not a new feature and optimization, it is just to complete the design aspects of non-preferred resource set processing. Without this feature, it is not clear for UE-B behavior on how to process the indicated non-preferred resource which is located out of UE-B’s resource selection window. Additionally, this CR just reuses the legacy resource exclusion mechanism to solve this issue. It is not convinced that we do nothing for an obvious issue just because we are in the late maintenance phase. Hence, we think this issue is essential for non-preferred resource set processing. |
| Huawei, HiSilicon | No | Not needed for further optimization which is not captured in the agreement. |
| Nokia, NSB | Yes H | OK to discuss this issue. In our view, this tries to clarify an existing agreement rather than adding a new feature. |
| xiaomi | No | We don’t support this proposal for further optimization. |
| Ericsson | Yes | We are supportive of discussing this issue. |

    - Summary of companies’ inputs to Issue 2-14 & FL’s suggestion

6 out 11 companies that provided inputs did not support Issue 2-14 being addressed at this meeting. Considering this situation, FL thinks that there is no consensus to have discussions and make conclusions for Issue 2-14 in this meeting.

  - Issue 2-19

The initial opinion of FL on Issue 2-19 is as follows:

- E

  - This proposal seems reasonable in terms of enhancing the clarity of the specification, and it can be handled as a category of E.

[Question #1]: Companies please provide their views on whether Issue 2-19 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | Yes, E | Agree with FL’s assessment. |
| OPPO | Yes, E | Agree with FL’s assessment. |
| Qualcomm |  | We do not think the change is necessary but are ok with it if that’s the majority view. |
| vivo | E |  |
| Spreadtrum | E |  |
| DCM | E | If some companies think this editorial correction is unnecessary, they should clarify what is difference between the two spec texts. |
| CATT, GOHIGH | Yes, E |  |
| Huawei, HiSilicon | E | Ok to modify duplicated part for scheme 2 option 1 and option 4. |
| Nokia, NSB | Yes E |  |
| xiaomi | Yes,E | Agree with FL’s assessment. |
| Ericsson | OK |  |

    - Summary of companies’ inputs to Issue 2-19 & FL’s suggestion

All companies that provided inputs supported addressing Issue 2-19 at this meeting. Considering this situation, FL’s suggestion is to have discussions and make conclusions for Issue 2-19 in this meeting.

  - Issue 2-16

The initial opinion of FL on Issue 2-16 is as follows:

- N

  - This proposal is to add a new feature for further optimization, which is not desirable at this late maintenance phase, but I would like to check the views of other companies.

[Question #1]: Companies please provide their views on whether Issue 2-16 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | Yes, H | When the the time gap from IUC transmission to preferred resource is larger than (Tproc,0SL+Tproc,1SL+Tproc,2SL) for availability of the indicated preferred resource (which can be widely supported in IUC), without the proposed change for the parameter q(Q) in Step 6), the resource exclusion procedures in Step 6) cannot work for determining the preferred resource set, since all the hypothetical received SCIs are before the indicated RSW w/o any overlapping. Therefore, this change is not optimization and should be addressed for IUC. |
| OPPO | No | Agree with FL’s assessment. |
| Qualcomm |  | We are ok to discuss the issue. |
| vivo | N |  |
| Spreadtrum | N |  |
| DCM |  | Although the issue seems valid, we are not sure whether this is essential correction |
| CATT, GOHIGH | No |  |
| Huawei, HiSilicon | No | Further optimization not needed. The timing is already up to UE-A implementation. |
| Nokia, NSB | Yes | OK to discuss it (but don’t think it’s H priority, so perhaps defer to next meeting) |
| xiaomi | No | Agree with FL’s assessment. |

    - Summary of companies’ inputs to Issue 2-16 & FL’s suggestion

6 out 10 companies that provided inputs did not support Issue 2-16 being addressed at this meeting. Considering this situation, FL thinks that there is no consensus to have discussions and make conclusions for Issue 2-16 in this meeting.

  - Issue 2-15

The initial opinion of FL on Issue 2-15 is as follows:

- H, but not addressed in this RAN1 meeting

  - Given that the relevant CR has been submitted by the company at this RAN2 meeting, it is expected that RAN2 will discuss this issue. Considering this situation, FL thinks that this issue could not be addressed at this RAN1 meeting. Note that based on the results of the discussion at this RAN2 meeting, we can have further discussion at the next RAN1 meeting about whether it will be addressed in RAN1.

[Question #1]: Companies please provide their views on whether Issue 2-15 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | No | Agree with FL’s assessment. |
| OPPO | No | Fine with us to revisit it next meeting if not addressed by RAN2. |
| Qualcomm | No | We think that RAN2 can address the issue. |
| vivo | N |  |
| Spreadtrum | No | Agree with FL |
| DCM |  | We are OK to wait for RAN2 progress. |
| CATT, GOHIGH | No | Agree with FL |
| Huawei, HiSilicon | No | It is fine for RAN2 to discuss this issue. |
| Nokia, NSB |  | Let us wait for RAN2 outcomes. |
| Xiaomi | No |  |

    - Summary of companies’ inputs to Issue 2-15 & FL’s suggestion

All companies that provided inputs had no concerns about waiting for the outcome of this RAN2 meeting. Considering this situation, FL thinks that the discussion related to Issue 2-15 can be skipped at this meeting.

  - Issue 2-18

The initial opinion of FL on Issue 2-18 is as follows:

- N

  - This doesn't seem critical, but I would like to check the views of other companies.

[Question #1]: Companies please provide their views on whether Issue 2-18 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | No | Agree with FL’s assessment. |
| OPPO |  | To keep it as is, or change it to “not passed” as agreement, either way is fine with us. |
| Qualcomm |  | We are ok to discuss the issue, though we do not think it is high priority. |
| vivo | N |  |
| Spreadtrum |  | We are ok with this modification. |
| DCM |  | For us, either is fine. |
| CATT, GOHIGH | No |  |
| Huawei, HiSilicon | H | No definition given of when a PSFCH occasion is considered “valid”, risking differing interpretations among UEs. |
| Nokia, NSB | Yes | Clarification of “valid” is beneficial |
| xiaomi | No | We think the current specification is clear without updating. |
| Ericsson | Yes, H | We think that this clarification is needed in order to have a proper functionality of the feature. |

    - Summary of companies’ inputs to Issue 2-18 & FL’s suggestion

7 out 11 companies that provided inputs supported addressing Issue 2-18 at this meeting. Considering this situation, FL’s suggestion is to have discussions and make conclusions for Issue 2-18 in this meeting.

  - Issue 2-23

The initial opinion of FL on Issue 2-23 is as follows:

- H

  - This seems to suggest capturing the missing RAN1 agreement into the specification.

[Question #1]: Companies please provide their views on whether Issue 2-23 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | Yes, H | Agree with FL’s assessment. |
| OPPO | No | The agreement says “At least a destination UE of a TB…”, rather than “only a destination UE of a TB …”. As explicit request is transmitted with unicast, UE-A should react as UE-B requested. |
| Qualcomm | No | The proposed change goes beyond the RAN1 agree and limits the non-preferred resource set to (1) request only and (2) when UE-A is a destination of UE-B only. |
| vivo | N | The spec. seems captured in 38.321 chapter 5.22.1.10, if there is anything missing, we suggest send LS to RAN2 to change the spec. |
| Spreadtrum | No | Share the same view with OPPO, i.e., there is no agreement on only intended receiver of UE-B can be UE-A for scheme 1. |
| DCM | N | The rationale seems incorrect, at least cited 321 text is not relevant to this CR |
| CATT, GOHIGH | Yes, H |  |
| Huawei, HiSilicon | H | It is essential to capture the missing RAN1 agreement into the specification, i.e., it’s missing in specification that UE-A needs to be the destination UE of a TB transmitted by UE-B for Scheme 1 “explicit request-based, non-preferred resource” case. |
| Nokia, NSB | No | Rather than capturing a missing agreement this change would impose a restriction that does not follow from the agreement. |
| xiaomi | No | We share the similar view with oppo. |

    - Summary of companies’ inputs to Issue 2-23 & FL’s suggestion

7 out 10 companies that provided inputs did not support Issue 2-23 being addressed at this meeting. Considering this situation, FL thinks that there is no consensus to have discussions and make conclusions for Issue 2-23 in this meeting.

  - Issue 2-24

The initial opinion of FL on Issue 2-24 is as follows:

- H

  - This is to capture the missing RAN1 agreement into the specification. Note that at the last RAN2 meeting, there was a discussion about whether/how to specify this in the RAN2 specification but a conclusion was not made due to e.g., different understanding of whether to capture the RAN1 working assumption in the RAN2 specification. Considering this situation, FL thinks that it should be captured in the RAN1 specification.

[Question #1]: Companies please provide their views on whether Issue 2-24 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | Yes, H | Agree with FL’s assessment. |
| OPPO | Yes, H | The agreement is missing in current specifications. |
| Qualcomm | Yes (H) |  |
| vivo | H | The IUC transmission procedure is not clear in current RAN1 and RAN2 spec., if there is a plan to complete the procedure in RAN1, we suggest to have a new chapter in 38.214 |
| Spreadtrum | H |  |
| DCM | H |  |
| CATT, GOHIGH | Yes, H | Firstly, regarding the support cast type for IUC, from our understanding, it should be captured in RAN2 specification. Secondly, according to current agreement and working assumption correctly, especially for non-preferred resource set, the current CR will somehow cause confusion, since only unicast is used for transmission of non-preferred resource set triggered by explicit request. |
| Huawei, HiSilicon | H | OK to clarify cast types. According to agreements, explicit request and IUC information (both preferred and non-preferred) triggered by the explicit request are only for unicast. For IUC triggered by a condition other than explicit request, preferred is only for unicast, non-preferred is for all cast types. |
| Nokia, NSB | Yes H | OK to discuss it, but there are problems with the proposed change: - The clause is titled “UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission”, so determination of the cast type does not really fit here. - RAN1 assumption has always been that cast type is determined by higher layer => RAN2 should handle this |
| xiaomi | Yes,H |  |

    - Summary of companies’ inputs to Issue 2-24 & FL’s suggestion

All companies that provided inputs supported addressing Issue 2-24 at this meeting. Considering this situation, FL’s suggestion is to have discussions and make conclusions for Issue 2-24 in this meeting.

  - Issue 2-25

The initial opinion of FL on Issue 2-25 is as follows:

- H

  - This seems to suggest capturing the necessary clarification consistent with the RAN1 agreement into the specification.

[Question #1]: Companies please provide their views on whether Issue 2-25 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | Yes, H | Agree with FL’s assessment. |
| OPPO | No | Per 38.212, it is clear that “value of 0 indicates that the UE cannot be a UE to receive conflict information”. |
| Qualcomm | OK (H) |  |
| vivo | E |  |
| DCM | H | We are also OK to consider OPPO’s comment |
| CATT, GOHIGH | Yes, H |  |
| Huawei, HiSilicon | No | Not needed. Definition of sl-IndicationUE-B is already clear as per TS 38.331. Additional description on conditions for a UE to receive PSFCH with conflict information is not needed. sl-IndicationUE-B Indicates whether to enable or disable the usage of 1 LSB of reserved bits of a SCI format 1-A to indicate of whether UE scheduling a conflict TB can be UE-B or not.. |
| Nokia, NSB | Yes H | OK to discuss it, but don’t agree with the proposed change |
| Xiaomi | No | We think this has been captured in 38.212. |

    - Summary of companies’ inputs to Issue 2-25 & FL’s suggestion

6 out 9 companies that provided inputs supported addressing Issue 2-25 at this meeting. Considering this situation, FL’s suggestion is to have discussions and make conclusions for Issue 2-25 in this meeting.

  - Issue 2-26

The initial opinion of FL on Issue 2-26 is as follows:

- E

  - This is an editorial correction for misaligned RRC parameter name, which seems reasonable.

[Question #1]: Companies please provide their views on whether Issue 2-26 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | Yes, E | Agree with FL’s assessment. |
| OPPO | Yes, E |  |
| Qualcomm | OK (E) |  |
| vivo | E |  |
| Spreadtrum | E |  |
| DCM | E |  |
| CATT, GOHIGH | Yes, E |  |
| Huawei, HiSilicon | No | Already captured in chair’s notes in RAN1 110bis-e. TP#5 for Issue #2-12 in section 4.2.5 of R1-2210335 is endorsed for the editorial corrections. |
| Nokia, NSB | Yes E |  |
| Xiaomi | Yes, E |  |

    - Summary of companies’ inputs to Issue 2-26 & FL’s suggestion

As Huawei commented, the agreement below was already made at the last meeting, but it seems that the editor missed reflecting it. So, FL thinks that we do not need to discuss Issue 2-26 in this meeting, and companies can directly comment on this missing part in the CR discussion (led by the editor) after this meeting.

- For alignment TS38.213 CR:

  - TP#5 for Issue #2-12 in section 4.2.5 of R1-2210335 is endorsed for the editorial corrections.

  - Issue 3-2

The initial opinion of FL on Issue 3-2 is as follows:

- H

  - This proposes to capture the necessary clarification to the specification according to the agreement made at the last RAN1 meeting, which seems reasonable.

[Question #1]: Companies please provide their views on whether Issue 3-2 should be addressed at this meeting or not, and what are the technical reasons for it.

| Company | Yes or not (If yes, please indicate either H or E) | Comments (including technical reasons for your view) |
| --- | --- | --- |
| Sharp | Yes, H | Agree with FL’s assessment. |
| OPPO | Yes, H | Missing agreement in specification |
| Qualcomm | Yes (H) |  |
| vivo | H |  |
| Spreadtrum | H |  |
| DCM | H |  |
| CATT, GOHIGH | Yes, H |  |
| Huawei, HiSilicon | H | Based on agreement made in RAN1#110, new Rel-17 parameters SL P0 are introduced. Therefore, power control for S-SSB/PSBCH, PSSCH and PSFCH needs be updated accordingly. |
| Nokia, NSB | Yes H |  |
| Xiaomi | Yes, H |  |

    - Summary of companies’ inputs to Issue 3-2 & FL’s suggestion

All companies that provided inputs supported addressing Issue 3-2 at this meeting. Considering this situation, FL’s suggestion is to have discussions and make conclusions for Issue 3-2 in this meeting.

- Selected essential issues to be addressed in this meeting (i.e., outcome of discussion in Section 2)

  - IUC for mode 2 enhancement

    - [ACTIVE] Issue #2-1: Further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set Conflict information receiver flag to 1

      - Background

Four contributions [1][9][11][16] proposed to have further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set Conflict information receiver flag to 1.

[FUTUREWEI: R1-2210838] Reason for change: The specification on the determination of UE-B does not cover the case when at least one of UEs scheduling conflicting TBs does not support Scheme 2 Summary of change: Add the determination of UE-B when at least one of UEs scheduling conflicting TBs does not support Scheme 2 Consequence if not approved: Ambiguous specifications which may limit the use cases of inter UE coordination Scheme 2 and impact the performance Text proposal for determination of UE-B in TS38.213 Section 16.3.0 --------------------------------- Text proposal begins ----------------------------------------------- ### 16.3.0 UE procedure for transmitting PSFCH with control information ## *** Unchanged text is omitted *** A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 - determines to transmit to the third UE the PSFCH with the conflict information regardless of the values of p1,p2, if the indicationUEB flag in SCI Format 1-A from the second UE is set to 0 and the indicationUEB flag in SCI Format 1-A from the third UE is set to 1, and indicationUEBScheme2 = 'enabled' ## *** Unchanged text is omitted *** --------------------------------- Text proposal ends -----------------------------------------------. [Apple: R1-2211795] Reason for change: The specification on the determination of UE-B does not cover the case where one of UEs scheduling conflicting TBs does not support receiving inter-UE coordination scheme 2. Summary of change: If one UE scheduling conflicting TB supports receiving inter-UE coordination scheme 2, but the other UE scheduling conflicting TB does not support receiving inter-UE coordination scheme 2, then the former UE should be UE-B, regardless the priority values of TBs. Consequence if not approved: Specification is unclear which UE scheduling conflicting TB is UE-B, in the case where one UE supports receiving inter-UE coordination scheme 2 and the other UE does not support receiving inter-UE coordination scheme 2. ========================== Start of Text Proposal ================================= 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, sl-TypeUE-A is not provided, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A is provided, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the conflict information receiver flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 - determines to transmit to the third UE the PSFCH with the conflict information regardless of the values of p1,p2, if the indicationUEB flag in SCI Format 1-A from the second UE is set to 0 and the indicationUEB flag in SCI Format 1-A from the third UE is set to 1, and indicationUEBScheme2 = 'enabled' < Unchanged parts are omitted > ========================== End of Text Proposal ==================================. [DOCOMO: R1-2211960] Reason for change: In the current specification, a UE with higher priority value is set as UE-B when the two UEs scheduling the conflicting TBs support UE-B behavior of scheme 2 (and with further conditions). However, an important case is missed: only either of the two UEs supports UE-B behavior of scheme 2 and the other UE does not support. Rather, this situation is the motivation to introduce ‘Conflict information receiver flag’ field; thus, the correction is an essential feature. That is, when only either of the two UEs indicating overlapping resource(s) supports scheme 2, UE-A sets the capable UE as UE-B. For whole procedure, UE-A should firstly apply the above capability-based UE-B determination and then the priority-based UE-B determination; otherwise, i.e. in the opposite order, none of UE might be set as UE-B. There was an argument that collision indication might be sent to UE transmitting a TB with lower priority, which leads to worse resource efficiency. However, this argument is invalid definitely. Firstly, when the two UEs do not detect each other, which is referred as hidden-node issue and is one of the main situations to be solved in this agenda, there is no impact on resource efficiency and better reliability/latency can be obtained. Secondly, this agenda focuses on better reliability and latency. It is quite strange if we aim to better efficiency at a cost of reliability/latency. Of course, both UEs might perform reselection due to conflict indicator and pre-emption, but this should be OK for better reliability/latency. Thirdly, appropriate (pre-)configuration of collision detection, e.g. SIR-based conflict detection, can handle such a non-preferred situation from resource efficiency perspective. Summary of change: For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and ‘Conflict information receiver flag’ field is set to 1 if sl-IndicationUE-B is (pre)configured to ‘Enabled’, UE-A sets the UE as UE-B. Consequence if not approved: Resource conflict in much less situations can be detected and as the result inter-UE coordination scheme 2 achieves little gain. ### 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, sl-TypeUE-A is not provided, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool sl-TypeUE-A is provided, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - determines that one of the following conditions are met - the conflict information receiver flag in SCI Format 1-A from the second UE is set to 1, the conflict information receiver flag in SCI Format 1-A from the third UE is set to 0, and if sl-IndicationUE-B = 'enabled' - the conflict information receiver flag in SCI Format 1-A from the second UE and the third UE is set to 1, p2<p1, and if indicationUEBScheme2 sl-IndicationUE-B = 'enabled' - p2<p1 if sl-IndicationUE-B = 'disabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 <Unchanged parts omitted>. [ZTE: R1-2212204] Reason for change: For Condition 2-A-1 of IUC scheme 2, it needs further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set indicationUEB flag to 1, and we continue to support following proposal in RAN1#109-e meeting: Proposal in RAN1#109-e: - For Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, - If there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and, if sl-IndicationUE-B is (pre)configured to ‘Enabled’, ‘Conflict information receiver flag’ field is set to 1, that UE is UE-B.. Summary of change: Adding the related description on conditions for UE to be UE-B for Condition 2-A-1 in TS38.213. Consequence if not approved: How to determine UE-B for Condition 2-A-1 of Scheme 2 is not clear. <Unchanged parts are omitted> - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 - determines to transmit to the second UE the PSFCH with the conflict information, if only for the second UE, the PSFCH occasion for resource conflict information is valid and the conflict information receiver flag in SCI Format 1-A is set to 1, if indicationUEBScheme2 = 'enabled' < Unchanged parts are omitted >.

      - FL Proposals for Round 1 (1 question)

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the following Draft proposal 2-1 (I) can be acceptable.

Draft proposal 2-1 (I):

- Reason for change:

- For Condition 2-A-1 of IUC scheme 2, it needs further clarification on conditions for UE to be UE-B when at least one of UEs scheduling conflicting TBs does not set ‘Conflict information receiver flag’ filed to 1.

- Summary of change:

- Adding the related description on conditions for UE to be UE-B for Condition 2-A-1 in TS 38.213, i.e., for Condition 2-A-1 in Scheme 2, when sl-TypeUE-A is enabled or when sl-TypeUE-A is disabled and the destination UE of the conflicting TBs is UE-A, if there is only one UE scheduling the conflicting TB whose PSFCH occasion for resource conflict indication is not yet passed and ‘Conflict information receiver flag’ field is set to 1 if sl-IndicationUE-B is (pre)configured to ‘Enabled’, UE-A sets the UE as UE-B.

- Consequences if not approved:

- How to determine UE-B for Condition 2-A-1 of Scheme 2 is not clear.

--------------------------- Start of text proposal to TS 38.213 v17.3.0 --------------------------- - 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 - determines to transmit to the second UE the PSFCH with the conflict information, if only for the second UE, the PSFCH occasion for resource conflict information is valid and the conflict information receiver flag in SCI Format 1-A is set to 1, if sl-IndicationUE-B = 'enabled' < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.3.0 ---------------------------

| Company | Yes or not | Comments |
| --- | --- | --- |

    - [ACTIVE] Issue #2-19: Deletion of duplicated part on UE procedure for determining a resource conflict between TS 38.213 (Section 16.3.0) and TS 38.214 (Section 8.1.4B).

      - Background

Two contributions [8][12] proposed to delete duplicated part on UE procedure for determining a resource conflict between TS 38.213 (Section 16.3.0) and TS 38.214 (Section 8.1.4B).

[OPPO: R1-2211445] Reason for change: How to determine resource conflict according to Condition 2-A-1 and Condition 2-A-2 are captured in clause 16.3.0 of 38.213, but same contents repeat in Clause 8.1.4A of 38.214. Conflict information is conveyed by PSFCH, it is more suitable to capture the contents in 38.213. Summary of change: Remove Clause 8.1.4B in 38.214. Consequence if not approved: Duplicated contents in 2 different specifications impact the readability and cause confusion. -------------------------------------------- Start of text proposal for TS 38.214 V17.3.0 ------------------------------------------- r1 r1r2 r1RSRP2Thprio2,prio1prio1prio2 r2RSRP1Thprio1,prio2prio1prio2 r1RSRP2-RSRP1 r2RSRP1-RSRP2 RSRP1RSRP2Th(pi,pj) r1r1 ### 8.1.4B UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0SL+ Tproc,1SL+ Tproc,2SL) after the resource of inter-UE coordination information transmission, where Tproc,2SL is equal to (Tproc,0SL+ Tproc,1SL) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2SL is equal to Tproc,0SL when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission. Note: The case when Tproc,2SL is equal to Tproc,0SL is assuming that SCI format 2-C is received. -------------------------------------------- End of text proposal for TS 38.214 V17.3.0 -------------------------------------------. [DOCOMO: R1-2211964] Reason for change: There is a duplicated part on resource conflict detection between 38.213 (section 16.3.0 below) and 38.214 (section 8.1.4B). The latter includes several brackets; that is, this section was not updated based on previous agreements/corrections. Therefore, section 8.1.4B of 38.214 should be removed rather than section 16.3.0 of 38.213. 38.213 Section 16.3.0 … The first UE can be provided conditions by sl-OptionForCondition2-A-1 to determine conflict of reserved resources in a resource pool - if sl-OptionForCondition2-A-1 = '0', the first UE can be provided by, sl-Thres-RSRP-List Thpi,pj, a list of RSRP thresholds for each priority combination pi,pj [6, TS 38.214] - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if the RSRP [6, TS 38.214] of the third UE is above a threshold Thp2,p1 - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if the RSRP of the second UE is above a threshold Thp1,p2 - if sl-OptionForCondition2-A-1 = '1', the first UE can be provided a value deltaRSRPThresh by sl-DeltaRSRP-Thresh - if the first UE is an intended receiver for PSSCH in a reserved resource of the second UE, the first UE determines a resource conflict if RSRP2>RSRP1+Delta_Th, where RSRP1 and RSRP2 are the RSRP measurements from the first UE for the second UE and the third UE, respectively - if the first UE is an intended receiver for PSSCH in a reserved resource of the third UE, the first UE determines a resource conflict if RSRP1>RSRP2+Delta_Th …. Summary of change: The duplicated part in 38.214 is removed. Consequence if not approved: Specification has unnecessary texts. ### 8.1.4B UE procedure for determining a resource conflict Void A UE configured with the higher layer parameter interUECoordinationScheme2 enabling transmission of a resource conflict indication considers that a resource conflict occurs on a first reserved resource r1 indicated by a first received SCI format if at least one of the following conditions is satisfied: - the first reserved resource r1 overlaps with a second reserved resource r2 indicated by a second received SCI format, and - if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 1 enabled], - if the UE is a destination UE of a TB to be transmitted in r1, the RSRP measurement performed for the second received SCI format RSRP2 is higher than Thprio2,prio1 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively. - if the UE is a destination UE of a TB to be transmitted in r2, the RSRP measurement performed for the first received SCI format RSRP1 is higher than Thprio1,prio2 where prio1 and prio2 are the priorities indicated in the first and second received SCI format, respectively. - if [the higher layer parameter for enabling Options 1/4 in Condition 2-A-1] indicates [Option 4 enabled] and the UE supports [Option 4], - if the UE is a destination UE of a TB to be transmitted in r1, RSRP2-RSRP1 is higher than a (pre)configured RSRP threshold. - if the UE is a destination UE of a TB to be transmitted in r2, RSRP1-RSRP2 is higher than a (pre)configured RSRP threshold. - where the RSRP1, RSRP2 measurements are performed according to clause 8.4.2.1 and the parameter Th(pi,pj) is determined [as described in step 3 of clause 8.1.4]. - the first reserved resource r1 occurs in a slot in which the UE does not expect to perform SL reception due to half-duplex operation and the UE is a destination UE of a TB to be transmitted in resource r1..

      - FL Proposals for Round 1 (1 question)

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the following Draft proposal 2-19 (I) can be acceptable.

Draft proposal 2-19 (I):

- Reason for change:

- There is a duplicated part on determining a resource conflict detection between TS 38.213 (i.e., Section 16.3.0) and TS 38.214 (i.e., Section 8.1.4B).

- Summary of change:

- The duplicated part in TS 38.214 is removed.

- Consequences if not approved:

- Specification has unnecessary texts.

--------------------------- Start of text proposal to TS 38.214 v17.3.0 --------------------------- < Unchanged parts are omitted > r1 r1r2 r1RSRP2Thprio2,prio1prio1prio2 r2RSRP1Thprio1,prio2prio1prio2 r1RSRP2-RSRP1 r2RSRP1-RSRP2 RSRP1RSRP2Th(pi,pj) r1r1 ### 8.1.4B UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0SL+ Tproc,1SL+ Tproc,2SL) after the resource of inter-UE coordination information transmission, where Tproc,2SL is equal to (Tproc,0SL+ Tproc,1SL) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2SL is equal to Tproc,0SL when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission. Note: The case when Tproc,2SL is equal to Tproc,0SL is assuming that SCI format 2-C is received. < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.214 v17.3.0 ---------------------------

| Company | Yes or not | Comments |
| --- | --- | --- |

    - [ACTIVE] Issue #2-18: Correction/clarification on description of valid PSFCH occasion for Scheme 2 in TS 38.213

      - Background

Two contributions [18][22] proposed to have correction/clarification on description of valid PSFCH occasion for Scheme 2 in TS 38.213.

[Ericsson: R1-2212217] Reason for change: The current specification text does not include definition of validity for PSFCH occasion for resource conflict information Summary of change: Include a clarification of the valid PSFCH resource by including the timing constraints under which the resources are valid. Consequence if not approved: The specification text is not clear regarding the validity of the PSFCH resources under the agreed timing constraints. 16.3.0 UE procedure for transmitting PSFCH with control information <Unchanged parts omitted> A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid following the timing constraints indicated by PSFCHOccasionScheme2. <Unchanged parts omitted>. [Huawei: R1-2212465] Reason for change: According to RAN1#108-e agreement below, in order to be UE-B, the PSFCH occasions for resource conflict information of each pair of UEs should not be passed. In current TS 38.213, “valid” is used to describe this condition, but there is no meaning given to the word ‘‘valid’’, and interpretations may differ when implementing the specification. Therefore, “not passed” should be captured as in agreement. Agreement: Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not. - Working Assumption: - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines one of the UEs to be UE-B by its implementation. - UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B. Summary of change: Correct description on PSFCH occasion by replacing “valid” with “not passed”. Consequence if not approved: No definition given of when a PSFCH occasion is considered “valid”, risking differing interpretations among UEs. 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are not passed - the indicationUEB flag in SCI Format 1-A from the second UE and the third UE is set to 1, if indicationUEBScheme2 = 'enabled' - determines the first SCI format 1-A and the second SCI format 1-A are not received later than sl-MinTimeGapPSFCH before the PSFCH occasion for conflict information - determines to transmit to the second UE the PSFCH with the conflict information - determines to transmit to either the second UE or the third UE the PSFCH with the conflict information, if p2=p1 < Unchanged parts are omitted >.

      - FL Proposals for Round 1 (1 question)

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the following Draft proposal 2-18 (I) can be acceptable.

Draft proposal 2-18 (I):

- Reason for change:

- The current specification text does not include definition of validity for PSFCH occasion for resource conflict information.

- Summary of change:

- Include a clarification of the valid PSFCH resource by including the timing constraints under which the resources are valid.

- Consequences if not approved:

- The specification text is not clear regarding the validity of the PSFCH resources under the agreed timing constraints.

--------------------------- Start of text proposal to TS 38.213 v17.3.0 --------------------------- < Unchanged parts are omitted > 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a UE for providing the conflict information to in a PSFCH as follows - if, for a resource pool, typeAUEScheme2 is disabled, the first UE has been indicated a first reserved resource and a second reserved resource as resources for PSSCH reception or, if for a resource pool typeAUEScheme2 is enabled, has been indicated at least the first reserved resource or the second reserved resource for PSSCH reception, - detects a first SCI format 1-A that includes a first priority value, p1, and the first reserved resource for PSSCH transmission from a second UE, - detects a second SCI format 1-A that includes a second priority value, p2<p1, and the second reserved resource for PSSCH transmission from a third UE, and - determines that the first and second resources overlap in time and frequency - the PSFCH occasions for resource conflict information of the second UE and the third UE are valid following the timing constraints indicated by sl-PSFCH-Occasion < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.3.0 ---------------------------

| Company | Yes or not | Comments |
| --- | --- | --- |

    - [ACTIVE] Issue #2-24: Further clarification on cast type for IUC scheme 1

      - Background

One contribution [7] proposed to have further clarification on cast type for IUC scheme 1

[LG Electronics: R1-2211262] Reason for change: The followings were made by RAN1, and there was a discussion about whether/how to specify them at the last RAN2 meeting but a conclusion was not made due to e.g., different understanding of capturing the working assumption in the RAN2 specification. From the point of view of RAN1, in Scheme 1, it is clear that only unicast can be used for the transmission of explicit request and condition/explicit request-based preferred resource set, and all cast types can be used for the transmission of condition-based non-preferred resource set. One way to solve this issue is to capture the followings in the RAN1 specification. - Agreement (RAN1#107bis-e meeting): - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request - Working Assumption (RAN1#107bis-e meeting): - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set - FFS: Under which conditions groupcast/broadcast can be supported - Unicast - FFS: Under which conditions unicast can be supported - Conclusion (RAN1#108-e meeting): - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set Summary of change: Add clarification on cast types that can be used for explicit request and condition/explicit request-based resource sets in TS 38.214. Consequence if not approved: The incorrect cast type can be used for Scheme 1, and the support of Scheme 1 is incomplete. <Unchanged parts omitted> ### 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission When this procedure is triggered, the following parameters are provided by the higher layer: - the resource pool from which the preferred or non-preferred resources are to be determined; - the resource selection window [n+T1,n+T2] within which the preferred or non-preferred resources are to be determined; - the resource set type (either preferred or non-preferred resource set); - if the resource set type indicates preferred set, then the higher layer additionally provides the following parameters: - L1 priority, prioTX; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - the resource reservation period, Prsvp_TX, if present. The value of Cresel is determined by the UE according to clause 8.1.5. Only unicast is used for transmission of explicit request and preferred resource set. All cast types are used for transmission of non-preferred resource set. When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows: - The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A. - The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A. When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications: - Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met: - the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined; - the higher layer parameter condition1A2Scheme1Disabled is not set to 'Disabled'. When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s): - resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria: - the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option1Scheme1, where k=pi. - the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option2Scheme1, where k=pi. - resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined. <Unchanged parts omitted>.

      - FL Proposals for Round 1 (1 question)

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the following Draft proposal 2-24 (I) can be acceptable.

Draft proposal 2-24 (I):

- Reason for change:

- The followings were made by RAN1 but not captured in the specification.

- Agreement (RAN1#107bis-e meeting):

  - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

    - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

- Working Assumption (RAN1#107bis-e meeting):

  - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

    - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

      - FFS: Under which conditions groupcast/broadcast can be supported

    - Unicast

      - FFS: Under which conditions unicast can be supported

- Conclusion (RAN1#108-e meeting):

  - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set

- Summary of change:

- Add clarification on cast types that can be used for explicit request and condition/explicit request-based resource sets in TS 38.214.

- Consequences if not approved:

- The incorrect cast type can be used for Scheme 1, and the support of Scheme 1 is incomplete.

--------------------------- Start of text proposal to TS 38.214v17.3.0 --------------------------- < Unchanged parts are omitted > ### 8.1.4A UE procedure for determining a set of preferred or non-preferred resources for another UE's transmission When this procedure is triggered, the following parameters are provided by the higher layer: - the resource pool from which the preferred or non-preferred resources are to be determined; - the resource selection window [n+T1,n+T2] within which the preferred or non-preferred resources are to be determined; - the resource set type (either preferred or non-preferred resource set); - if the resource set type indicates preferred set, then the higher layer additionally provides the following parameters: - L1 priority, prioTX; - the number of sub-channels to be used for the PSSCH/PSCCH transmission in a slot, LsubCH; - the resource reservation period, Prsvp_TX, if present. The value of Cresel is determined by the UE according to clause 8.1.5. Only unicast is used for transmission of explicit request and preferred resource set. All cast types are used for transmission of non-preferred resource set. When this procedure is triggered by another UE's explicit request, the fields in the request are interpreted as follows: - The field 'Resource selection window location' is the concatenation of the starting time location and the ending time location of the resource selection window. The starting and ending time locations of the resource selection window are each encoded in the same way as the reference slot as described in clause 8.1.5A. - The field 'Resource reservation period' is encoded in the same way as the field of the same name in SCI format 1-A. When determining a preferred resource set, the UE applies the procedure described in clause 8.1.4 with the above parameters and the following modifications: - Step 6a) The UE excludes candidate single-slot resource(s) belonging to slot(s) where the UE does not expect to perform SL reception of a TB due to half-duplex operation, if all the following conditions are met: - the UE is a destination UE of the TB for whose transmission the preferred resource set is being determined; - the higher layer parameter condition1A2Scheme1Disabled is not set to 'Disabled'. When determining a non-preferred resource set, the UE considers any resource(s) within the resource selection window, if indicated by a received explicit request, and satisfying at least one of the following conditions as non-preferred resource(s): - resource(s) indicated by a received [SCI format 1-A], satisfying at least one of the following criteria: - the RSRP measurement performed, according to clause 8.4.2.1, for the received [SCI format 1-A], is higher than ThprioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option1Scheme1, where k=pi. - the UE is a destination UE of a TB associated with the received [SCI format 1-A] and the RSRP measurement performed, according to clause 8.4.2.1 for the received [SCI format 1-A], is lower than Th'prioRX where prioRX is the value of the priority field in the received [SCI format 1-A]. The internal parameter Th'(pi) is set to the corresponding value of RSRP threshold indicated by the k-th field in thresholdRSRPCondition1B1Option2Scheme1, where k=pi. - resources(s) in slot(s) in which the UE does not expect to perform SL reception due to half duplex operation, if the UE is a destination UE of a TB for whose transmission the non-preferred resource set is being determined. < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.214v17.3.0 ---------------------------

| Company | Yes or not | Comments |
| --- | --- | --- |

    - [ACTIVE] Issue #2-25: Further clarification on UE procedure for receiving PSFCH with conflict information

      - Background

One contributions [17] proposed to have further clarification on UE procedure for receiving PSFCH with conflict information.

[ZTE: R1-2212205] Reason for change: When indicationUEBScheme2 = 'enabled' in a resource pool, a UE(UE-B) attempts to receive associated PSFCH with conflict information only if conflict information receiver flag in SCI Format 1-A transmitted by the UE is set to 1. Summary of change: Add the related description on conditions for a UE to receive PSFCH with conflict information in a resource pool with indicationUEBScheme2 = 'enabled' in TS38.213. Consequence if not approved: The conditions for a UE receiving PSFCH with conflict information is not clear if indicationUEBScheme2 = 'enabled'. <Unchanged parts are omitted> - 16.3.1 UE procedure for receiving PSFCH with control information < Unchanged parts are omitted > A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by inter-UECoordinationScheme2, and setting the conflict information receiver flag to 1 if indicationUEBScheme2 = 'enabled', attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if slotLevelResourceExclusionScheme2 is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if slotLevelResourceExclusionScheme2 is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A < Unchanged parts are omitted >.

      - FL Proposals for Round 1 (1 question)

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the following Draft proposal 2-25 (I) can be acceptable.

Draft proposal 2-25 (I):

- Reason for change:

- When sl-IndicationUE-B = 'enabled' in a resource pool, a UE (UE-B) attempts to receive associated PSFCH with conflict information only if conflict information receiver flag in SCI Format 1-A transmitted by the UE is set to 1.

- Summary of change:

- Add the related description on conditions for a UE to receive PSFCH with conflict information in a resource pool with sl-IndicationUE-B = 'enabled' in TS 38.213.

- Consequences if not approved:

- The conditions for a UE receiving PSFCH with conflict information is not clear if sl-IndicationUE-B = 'enabled'.

--------------------------- Start of text proposal to TS 38.213 v17.3.0 --------------------------- < Unchanged parts are omitted > - 16.3.1 UE procedure for receiving PSFCH with control information < Unchanged parts are omitted > A UE that transmitted SCI format 1-A, indicating one or more reserved resources in a resource pool enabled by inter-UECoordinationScheme2, and setting the conflict information receiver flag to 1 if sl-IndicationUE-B = 'enabled', attempts to receive associated PSFCH with conflict information in a resource pool with PSFCH resources that the UE determines as described in clause 16.3.0. If the UE determines presence of a resource conflict based on conflict information in a PSFCH reception, the UE reports the resource conflict to higher layers - if slotLevelResourceExclusionScheme2 is not provided, the UE reports resources overlapping with a next in time reserved resource indicated by the SCI format 1-A - if slotLevelResourceExclusionScheme2 is provided, the UE reports resources in a slot of a next in time reserved resource indicated by the SCI format 1-A < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.3.0 ---------------------------

| Company | Yes or not | Comments |
| --- | --- | --- |

  - Others

    - [ACTIVE] Issue #3-2: Further clarification on which set of SL power control parameters is used by UE

      - Background

Four contributions [2][4][15][19] proposed to have further clarification on which set of SL power control parameters is used by UE

[Huawei: R1-2210895] Reason for change: Based on agreement made in RAN1#110, new Rel-17 parameters SL P0 are introduced. Agreement Introduce Rel-17 parameters SL P0 (i.e., dl-P0-PSSCH-PSCCH-r17, sl-P0-PSSCH-PSCCH-r17, dl-P0- PSBCH-r17, dl-P0-PSFCH-r17) for SL open loop power control, where the value range of the new parameters is [-202, 24] - Introduce a new capability for UE supporting SL open loop power control based on these new Rel-17 SL P0 parameters, SL open loop power control based on the new Rel-17 SL P0 parameters should be subject to this UE capability.. Therefore, power control for S-SSB/PSBCH, PSSCH and PSFCH needs be updated accordingly. Summary of change: Power control for S-SSB/PSBCH, PSSCH and PSFCH is updated based on higher layer parameters dl-P0-PSSCH-PSCCH-r17, sl-P0-PSSCH-PSCCH-r17, dl-P0-PSBCH-r17, dl-P0-PSFCH-r17 additionally. Consequence if not approved: Specification does not capture existing RAN1 agreement. UE would implement incorrect power control behaviour and cause large interference on gNB. < Unchanged parts are omitted > ### 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH-r17 if provided when the UE reports p0-OLPC-Sidelink-r17; otherwise, dl-P0-PSBCH if provided; else, PS-SSB(i)=PCMAX - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 < Unchanged parts are omitted > ### 16.2.1 PSSCH A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0- PSSCH-PSCCH -r17 if provided when the UE reports p0-OLPC-Sidelink-r17; otherwise, dl-P0-PSSCH-PSCCH if provided - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH -r17 if provided when the UE reports p0-OLPC-Sidelink-r17; otherwise, sl-P0-PSSCH-PSCCH, if provided - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 - < Unchanged parts are omitted > ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH-r17 if provided when the UE reports p0-OLPC-Sidelink-r17; otherwise, dl-P0-PSFCH < Unchanged parts are omitted >. [vivo: R1-2210979] Reason for change: In RAN1#110, RAN1 reached the following agreement on introducing new parameters for Rel-17 sidelink OLPC and a UE capability to indicate the support of new parameters. Agreement Introduce Rel-17 parameters SL P0 (i.e., dl-P0-PSSCH-PSCCH-r17, sl-P0-PSSCH-PSCCH-r17, dl-P0- PSBCH-r17, dl-P0-PSFCH-r17) for SL open loop power control, where the value range of the new parameters is [-202, 24] - Introduce a new capability for UE supporting SL open loop power control based on these new Rel-17 SL P0 parameters, SL open loop power control based on the new Rel-17 SL P0 parameters should be subject to this UE capability. Summary of change: Update RAN1 specifications to capture the agreement. Consequence if not approved: Ambiguity about OLPC for SL remains, which may result in strong interference from SL to Uu. ### 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH if provided; else, PS-SSB(i)=PCMAX , where the value of dl-P0-PSBCH is provided by dl-P0-PSBCH-r17 if UE indicates the capability of p0-OLPC-Sidelink-r17, or provided by dl-P0-PSBCH-r16, as described in [12, TS 38.331]. - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ ### 16.2.1 PSSCH A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH if provided, where the value of dl-P0-PSSCH-PSCCH is provided by dl-P0-PSSCH-PSCCH-r17 if UE indicates the capability of p0-OLPC-Sidelink-r17, or provided by dl-P0-PSSCH-PSCCH-r16, as described in [12, TS 38.331]. - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH, if provided; where the value of sl-P0-PSSCH-PSCCH is provided by sl-P0-PSSCH-PSCCH-r17 if UE indicates the capability of p0-OLPC-Sidelink-r17, or provided by sl-P0-PSSCH-PSCCH-r16, as described in [12, TS 38.331]. - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 - PLSL= referenceSignalPower – higher layer filtered RSRP, where - referenceSignalPower is obtained from a PSSCH transmit power per RE summed over the antenna ports of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient, and - higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient - MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration The UE splits the power PPSSCH(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power. A UE determines a power PPSSCH2(i) for a PSSCH transmission on a resource pool in the symbols where a corresponding PSCCH is transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as PPSSCH2(i)=10log10MRBPSSCHi-MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm] where MRBPSCCH(i) is a number of resource blocks for the corresponding PSCCH transmission in PSCCH-PSSCH transmission occasion i. The UE splits the power PPSSCH2(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power. ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH; where the value of dl-P0-PSFCH is provided by dl-P0-PSFCH-r17 if UE indicates the capability of p0-OLPC-Sidelink-r17, or provided by dl-P0-PSFCH-r16, as described in [12, TS 38.331]. - αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1].. [Qualcomm: R1-2212089] Reason for change: Implement RAN1 agreement on having a UE capability to indicate which set of power control parameters is used by the UE Agreement Introduce Rel-17 parameters SL P0 (i.e., dl-P0-PSSCH-PSCCH-r17, sl-P0-PSSCH-PSCCH-r17, dl-P0- PSBCH-r17, dl-P0-PSFCH-r17) for SL open loop power control, where the value range of the new parameters is [-202, 24] - Introduce a new capability for UE supporting SL open loop power control based on these new Rel-17 SL P0 parameters, SL open loop power control based on the new Rel-17 SL P0 parameters should be subject to this UE capability. Summary of change: Update specifications such that the UE uses the -r17 power control parameters when it supports the feature, otherwise, the UE uses the -r16 parameters Consequence if not approved: Potential ambiguity about which set of parameters the UE uses. ----------------------------------------------------begin text proposal for 38.213---------------------------------------------------- ### 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH if provided; else, PS-SSB(i)=PCMAX. The value of dl-P0-PSBCH is dl-P0-PSBCH-r17 if supported by the UE; else, dl-P0-PSBCH-r16. - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ ### 16.2.1 PSSCH A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH if provided. The value of dl-P0-PSSCH-PSCCH is dl-P0-PSSCH-PSCCH-r17 if supported by the UE; else, dl-P0-PSSCH-PSCCH-r16. - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH, if provided. The value of sl-P0-PSSCH-PSCCH is sl-P0-PSSCH-PSCCH-r17 if supported by the UE; else, sl-P0-PSSCH-PSCCH-r16. - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 - PLSL= referenceSignalPower – higher layer filtered RSRP, where - referenceSignalPower is obtained from a PSSCH transmit power per RE summed over the antenna ports of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient, and - higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient - MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration The UE splits the power PPSSCH(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power. A UE determines a power PPSSCH2(i) for a PSSCH transmission on a resource pool in the symbols where a corresponding PSCCH is transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as PPSSCH2(i)=10log10MRBPSSCHi-MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm] where MRBPSCCH(i) is a number of resource blocks for the corresponding PSCCH transmission in PSCCH-PSSCH transmission occasion i. The UE splits the power PPSSCH2(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power. ### 16.2.2 PSCCH A UE determines a power PPSCCH(i) for a PSCCH transmission on a resource pool in PSCCH-PSSCH transmission occasion i as PPSCCH(i)=10log10MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm] where - PPSSCH(i) is specified in clause 16.2.1 - MRBPSCCH(i) is a number of resource blocks for the PSCCH transmission in PSCCH-PSSCH transmission occasion i - MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH. The value of dl-P0-PSFCH is dl-P0-PSFCH -r17 if supported by the UE; else, dl-P0-PSFCH-r16. - αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]. ----------------------------------------------------end text proposal for 38.213----------------------------------------------------. [Ericsson: R1-2212220] Reason for change: The current specification text does not include the agreed parameters in Rel-17 for power loop open control SL P0 of PSSCH, PSCCH, PSBCH and PSFCH Summary of change: Introduce Rel-17 parameters SL P0 (i.e., dl-P0-PSSCH-PSCCH-r17, sl-P0-PSSCH-PSCCH-r17, dl-P0- PSBCH-r17, dl-P0-PSFCH-r17) for SL open loop power control Consequence if not approved: The specification text does not follow the agreements reached in Rel-17 ## 16.2 Power control ### 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH-r17 if provided; else, PS-SSB(i)=PCMAX - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ ### 16.2.1 PSSCH A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH-r17 is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH-r17 if provided - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH-r17 is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH-r17, if provided - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 - PLSL= referenceSignalPower – higher layer filtered RSRP, where - referenceSignalPower is obtained from a PSSCH transmit power per RE summed over the antenna ports of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient, and - higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient - MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration <Unchanged parts omitted> ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH-r17 - αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c <Unchanged parts omitted>.

      - FL Proposals for Round 1 (1 question)

[Question #1]: Companies please provide their views (including your suggested/modified wording) on whether the following Draft proposal 3-2 (I) can be acceptable.

Draft proposal 3-2 (I):

- Reason for change:

- Implement RAN1 agreement on having a UE capability to indicate which set of SL power control parameters is used by the UE

- Agreement

  - Introduce Rel-17 parameters SL P0 (i.e., dl-P0-PSSCH-PSCCH-r17, sl-P0-PSSCH-PSCCH-r17, dl-P0- PSBCH-r17, dl-P0-PSFCH-r17) for SL open loop power control, where the value range of the new parameters is [-202, 24]

- Summary of change:

- Update specifications such that the UE uses the -r17 power control parameters when it supports the feature, otherwise, the UE uses the -r16 parameters

- Consequences if not approved:

- Potential ambiguity about which set of parameters the UE uses.

--------------------------- Start of text proposal to TS 38.213 v17.3.0 --------------------------- < Unchanged parts are omitted > ### 16.2.0 S-SS/PSBCH blocks A UE determines a power PS-SSB(i) for an S-SS/PSBCH block transmission occasion in slot i on active SL BWP b of carrier f as PS-SSB(i)=minPCMAX,PO,S-SSB+10log102μ∙MRBS-SSB+αS-SSB⋅PL [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PO,S-SSB is a value of dl-P0-PSBCH if provided; else, PS-SSB(i)=PCMAX. The value of dl-P0-PSBCH is dl-P0-PSBCH-r17 if supported by the UE; else, dl-P0-PSBCH-r16. - αS-SSB is a value of dl-Alpha-PSBCH, if provided; else, αS-SSB=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBS-SSB=11 is a number of resource blocks for a S-SS/PSBCH block transmission with SCS configuration μ ### 16.2.1 PSSCH A UE determines a power PPSSCH(i) for a PSSCH transmission on a resource pool in symbols where a corresponding PSCCH is not transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as: PPSSCH(i)=minPCMAX,PMAX,CBR,minPPSSCH,Di,PPSSCH,SL(i) [dBm] where - PCMAX is defined in [8-1, TS 38.101-1] - PMAX,CBR is determined by a value of sl-MaxTxPower based on a priority level of the PSSCH transmission and a CBR range that includes a CBR measured in slot i-N [6, TS 38.214]; if sl-MaxTxPower is not provided, then PMAX,CBR=PCMAX; - if dl-P0-PSSCH-PSCCH is provided - PPSSCH,D(i)=PO,D+10log102μ⋅MRBPSSCHi+αD⋅PLD [dBm] - else - PPSSCH,D(i)=minPCMAX,PMAX,CBR [dBm] where - PO,D is a value of dl-P0-PSSCH-PSCCH if provided. The value of dl-P0-PSSCH-PSCCH is dl-P0-PSSCH-PSCCH-r17 if supported by the UE; else, dl-P0-PSSCH-PSCCH-r16. - αD is a value of dl-Alpha-PSSCH-PSCCH, if provided; else, αD=1 - PLD=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - MRBPSSCH(i) is a number of resource blocks for the PSSCH transmission occasion i and μ is a SCS configuration - if sl-P0-PSSCH-PSCCH is provided and if a SCI format scheduling the PSSCH transmission includes a cast type indicator field indicating unicast or is SCI format 2-C - PPSSCH,SL(i)=PO,SL+10log102μ⋅MRBPSSCHi+αSL⋅PLSL [dBm] - else - PPSSCH,SL(i)=minPCMAX,PPSSCH,D(i) [dBm] where - PO,SL is a value of sl-P0-PSSCH-PSCCH, if provided. The value of sl-P0-PSSCH-PSCCH is sl-P0-PSSCH-PSCCH-r17 if supported by the UE; else, sl-P0-PSSCH-PSCCH-r16. - αSL is a value of sl-Alpha-PSSCH-PSCCH, if provided; else, αSL=1 - PLSL= referenceSignalPower – higher layer filtered RSRP, where - referenceSignalPower is obtained from a PSSCH transmit power per RE summed over the antenna ports of the UE, higher layer filtered across PSSCH transmission occasions using a filter configuration provided by sl-FilterCoefficient, and - higher layer filtered RSRP is a RSRP, as defined in [7, TS 38.215], that is reported to the UE from a UE receiving the PSCCH-PSSCH transmission and is obtained from a PSSCH DM-RS using a filter configuration provided by sl-FilterCoefficient - MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i and μ is a SCS configuration The UE splits the power PPSSCH(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power. A UE determines a power PPSSCH2(i) for a PSSCH transmission on a resource pool in the symbols where a corresponding PSCCH is transmitted in PSCCH-PSSCH transmission occasion i on active SL BWP b of carrier f as PPSSCH2(i)=10log10MRBPSSCHi-MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm] where MRBPSCCH(i) is a number of resource blocks for the corresponding PSCCH transmission in PSCCH-PSSCH transmission occasion i. The UE splits the power PPSSCH2(i) equally across the antenna ports on which the UE transmits the PSSCH with non-zero power. ### 16.2.2 PSCCH A UE determines a power PPSCCH(i) for a PSCCH transmission on a resource pool in PSCCH-PSSCH transmission occasion i as PPSCCH(i)=10log10MRBPSCCHiMRBPSSCHi+PPSSCH(i) [dBm] where - PPSSCH(i) is specified in clause 16.2.1 - MRBPSCCH(i) is a number of resource blocks for the PSCCH transmission in PSCCH-PSSCH transmission occasion i - MRBPSSCH(i) is a number of resource blocks for PSCCH-PSSCH transmission occasion i ### 16.2.3 PSFCH A UE with Nsch,Tx,PSFCH scheduled PSFCH transmissions for HARQ-ACK information and conflict information, and capable of transmitting a maximum of Nmax,PSFCH PSFCHs, determines a number NTx,PSFCH of simultaneous PSFCH transmissions and a power PPSFCH,k(i) for a PSFCH transmission k, 1≤k≤NTx,PSFCH, on a resource pool in PSFCH transmission occasion i on active SL BWP b of carrier f as - if dl-P0-PSFCH is provided, PPSFCH,one=PO,PSFCH+10log102μ+αPSFCH⋅PL [dBm] where - PO,PSFCH is a value of dl-P0-PSFCH. The value of dl-P0-PSFCH is dl-P0-PSFCH -r17 if supported by the UE; else, dl-P0-PSFCH-r16. - αPSFCH is a value of dl-Alpha-PSFCH, if provided; else, αPFSCH=1 - PL=PLb,f,c(qd) when the active SL BWP is on a serving cell c, as described in clause 7.1.1 except that - the RS resource is the one the UE uses for determining a power of a PUSCH transmission scheduled by a DCI format 0_0 in serving cell c when the UE is configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - the RS resource is the one corresponding to the SS/PBCH block the UE uses to obtain MIB when the UE is not configured to monitor PDCCH for detection of DCI format 0_0 in serving cell c - if Nsch,Tx,PSFCH≤Nmax,PSFCH - if PPSFCH,one+10log10Nsch,Tx,PSFCH≤PCMAX, where PCMAX is determined for Nsch,Tx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nsch,Tx,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - UE autonomously determines NTx,PSFCH PSFCH transmissions first with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, for 1≤i≤ 8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, for i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is defined in [8-1, TS 38.101-1] and is determined for the NTx,PSFCH PSFCH transmissions - else - the UE autonomously selects Nmax,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 - if PPSFCH,one+10log10Nmax,PSFCH≤PCMAX, where PCMAX is determined for the Nmax,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1] - NTx,PSFCH=Nmax,PSFCH and PPSFCH,k(i)=PPSFCH,one [dBm] - else - the UE autonomously selects NTx,PSFCH PSFCH transmissions in ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥max1,i=1KMi where Mi, 1≤i≤8, is a number of PSFCHs with priority value i for PSFCH with HARQ-ACK information and Mi, i>8, is a number of PSFCHs with priority value i-8 for PSFCH with conflict information and K is defined as - the largest value satisfying PPSFCH,one+10log10max1,i=1KMi≤PCMAX where PCMAX is determined according to [8-1, TS 38.101-1] for transmission of all PSFCHs in i=1KMi, if any - zero, otherwise and PPSFCH,k(i)=minPCMAX-10log10(NTx,PSFCH),PPSFCH,one [dBm] where PCMAX is determined for the NTx,PSFCH simultaneous PSFCH transmissions according to [8-1, TS 38.101-1] - else PPSFCH,k(i)=PCMAX-10log10(NTx,PSFCH) [dBm] where the UE autonomously determines NTx,PSFCH PSFCH transmissions with ascending order of corresponding priority field values as described in clause 16.2.4.2 over the PSFCH transmissions with HARQ-ACK information, if any, and then with ascending order of priority value over the PSFCH transmissions with conflict information, if any, such that NTx,PSFCH≥1 and where PCMAX is determined for the NTx,PSFCH PSFCH transmissions according to [8-1, TS 38.101-1]. --------< Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.3.0 ---------------------------

| Company | Yes or not | Comments |
| --- | --- | --- |

- List of RAN1 agreements on IUC for Mode 2 enhancements

## 1) RAN1#103-e meeting

- Conclusion:

  - The schemes of inter-UE coordination in Mode 2 are categorized as being based on the following types of “A set of resources” sent by UE-A to UE-B:

    - UE-A sends to UE-B the set of resources preferred for UE-B’s transmission

        - e.g., based on its sensing result

    - UE-A sends to UE-B the set of resources not preferred for UE-B’s transmission

        - e.g., based on its sensing result and/or expected/potential resource conflict

    - UE-A sends to UE-B the set of resource where the resource conflict is detected

    - FFS: details of resource conflict, e.g., including type of resource conflict

    - FFS: details of sensing operation at UE-A side

    - FFS: which type(s) of resource set information is(are) beneficial/feasible to which cast type(s)

    - Note: these different types may be used in combination with each other

  - From RAN1 perspective, further study on the feasibility/benefit of inter-UE coordination is required

  - Send an LS to RAN plenary

    - Final LS in R1-2009841

- Conclusion:

  - For the schemes of inter-UE coordination identified as feasible/beneficial, at least the following aspects are further discussed.

    - How/when UE-A determines the contents of ”A set of resources”, including consideration of UL scheduling

    - When UE-A sends ”A set of resources” to UE-B, including which UE(s) sends it

    - How UE-A and UE-B are determined

    - How UE-A sends ”A set of resources” to UE-B, including container used for carrying it, implicitly or explicitly or both

    - How/when/whether UE-B receives “A set of resources” and takes it into account in the resource selection for its own transmission

    - How/whether to define the relationship between support/signaling of inter-UE coordination and cast type

## 2) RAN1#104-e meeting

- Conclusion:

  - RAN1 concludes that the inter-UE coordination in Mode 2 is feasible, and is beneficial (e.g., reliability, etc.) compared to Rel-16 Mode 2 RA, and thus recommends specification of the feature.

    - The detailed observations can be found in the attachment of the LS

- Draft LS in R1-2102165, along with the attachment R1-2102166, is approved (with a typo fix)

  - Final LS in R1-2102168

## 3) RAN1#104bis-e meeting

- Agreement:

  - Support the following schemes of inter-UE coordination in Mode 2:

    - Inter-UE Coordination Scheme 1:

      - The coordination information sent from UE-A to UE-B is the set of resources preferred and/or non-preferred for UE-B’s transmission

        - FFS details including a possibility of down-selection between the preferred resource set and the non-preferred resource set, whether or not to include any additional information other than indicating time/frequency of the resources within the set in the coordination information

      - FFS condition(s) in which Scheme 1 is used

    - Inter-UE Coordination Scheme 2:

      - The coordination information sent from UE-A to UE-B is the presence of expected/potential and/or detected resource conflict on the resources indicated by UE-B’s SCI

        - FFS details including a possibility of down-selection between the expected/potential conflict and the detected resource conflict

      - FFS condition(s) in which Scheme 2 is used

- Agreement:

  - Study further to determine the conditions for UEs to be UE-A(s)/UE-B(s) for inter-UE coordination:

    - Details include applicable scenario(s)/inter-UE coordination scheme(s)

    - E.g., only UE(s) among the intended receiver(s) of UE-B can be a UE-A, any UE can be a UE-A, high-layer configured, etc.

      - Including the possibility of being subject to certain conditions and/or capability

- Agreement:

  - When UE-B receives the inter-UE coordination information from UE-A, consider at least one of the following options (with details FFS including possibly down-selecting/merging one or more of the options below, applicable scenario(s)/condition(s) for each option, UE behavior) for UE-B’s to take it into account in the resource (re)-selection for its own transmission

    - For scheme 1:

      - Option 1-1: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on both UE-B’s sensing result (if available) and the received coordination information

      - Option 1-2: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based only on the received coordination information

      - Option 1-3: UE-B’s resource(s) to be re-selected based on the received coordination information

      - Option 1-4: UE-B’s resource(s) to be used for its transmission resource (re)-selection is based on the received coordination information

    - For scheme 2:

      - Option 2-1: UE-B can determine resource(s) to be re-selected based on the received coordination information

      - Option 2-2: UE-B can determine a necessity of retransmission based on the received coordination information

## 4) RAN1#106-e meeting

- Agreement:

  - For scheme 1, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B.

    - Set of resources preferred for UE-B’s transmission

    - Set of resources non-preferred for UE-B’s transmission

- Agreement:

  - For scheme 2, the following inter-UE coordination information signalling from UE-A is supported. FFS details including condition(s)/scenario(s) under which each information is enabled to be sent by UE-A and used by UE-B

    - Presence of expected/potential resource conflict on the resources indicated by UE-B’s SCI

      - FFS: UE behaviour when the presence of expected/potential resource conflict is detected by the transmitter

    - FFS: Whether to additionally support the presence of detected resource conflict on the resources indicated by UE-B’s SCI

- Agreement:

  - In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by an explicit request in Mode 2:

    - A UE that sends an explicit request for inter-UE coordination information can be UE-B

    - A UE that received an explicit request from UE-B and sends inter-UE coordination information to the UE-B can be UE-A

    - (Working assumption) At least a destination UE of a TB transmitted by UE-B can be UE A

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

  - (Working Assumption) In scheme 1, the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination information transmission triggered by a condition other than explicit request reception in Mode 2:

    - A UE that satisfies the condition mentioned in the main bullet and sends inter-UE coordination information is UE-A

    - A UE that received inter-UE coordination information from UE-A and uses it for resource (re-)selection is UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Additional details and conditions on UE-A and UE-B

- Agreement:

  - In scheme 2, at least the following is supported for UE(s) to be UE-A(s)/UE-B(s) in the inter-UE coordination transmission triggered by a detection of expected/potential resource conflict(s) in Mode 2:

    - A UE that transmitted PSCCH/PSSCH with SCI indicating reserved resource(s) to be used for its transmission, received inter-UE coordination information from UE-A indicating expected/potential resource conflict(s) for the reserved resource(s), and uses it to determine resource re-selection is UE-B

    - A UE that detects expected/potential resource conflict(s) on resource(s) indicated by UE-B’s SCI sends inter-UE coordination information to UE-B, subject to satisfy one of the following conditions, is UE-A

      - (Working assumption) At least a destination UE of one of the conflicting TBs, i.e., TBs to be transmitted in the expected/potential conflicting resource(s)

        - Whether a non-destination UE of a TB transmitted by UE-B can be UE-A is (pre-)configured

      - FFS: Additional details and condition(s) on UE-A and UE-B

    - The above feature can be enabled or disabled or controlled by (pre-)configuration

      - FFS: Details on how to support this, including (pre-)configuration signaling granularity

    - FFS: Definition of expected/potential resource conflict(s) and other details (if any)

- Agreement:

  - In scheme 2, the following UE-B’s behavior in its resource (re)selection is supported when it receives inter-UE coordination information from UE-A:

    - UE-B can determine resource(s) to be re-selected based on the received coordination information

      - UE-B can reselect resource(s) reserved for its transmission when expected/potential resource conflict on the resource(s) is indicated

        - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least following UE-B’s behavior in its resource (re-)selection is supported when it receives inter-UE coordination information from UE-A:

    - For preferred resource set, the following two options are supported:

      - Option A): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set in combination with its own sensing result

          - UE-B uses in its resource (re-)selection, resource(s) not belonging to the preferred resource set when condition(s) are met

            - FFS: Details of condition(s)

          - This option is supported when UE-B performs sensing/resource exclusion

          - FFS: Other details (if any)

      - Option B): UE-B’s resource(s) to be used for its transmission resource (re-)selection is based only on the received coordination information

        - UE-B uses in its resource (re-)selection, resource(s) belonging to the preferred resource set

          - This option is supported at least when UE-B does not support sensing/resource exclusion

            - FFS: Whether the support is conditional or UE capability

          - FFS: Other details (if any)

      - FFS: Other option(s), and other details (if any)

    - For non-preferred resource set,

      - UE-B’s resource(s) to be used for its transmission resource (re-)selection is based on both UE-B’s sensing result (if available) and the received coordination information

        - UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

          - FFS: Details including

            - Whether/how UE-B can use in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set, definition of the overlap, and other details (if any)

            - When UE-B excludes in its resource (re-)selection, resource(s) overlapping with the non-preferred resource set

        - FFS: UE-B reselects in its resource (re-)selection, resource(s) to be used for its transmission when the resource(s) are fully/partially overlapping with the non-preferred resource set

      - FFS: Other option(s), and other details (if any)

- Agreement:

  - In scheme 2, at least the following is supported to determine inter-UE coordination information:

    - Among resource(s) indicated by UE-B’s SCI, UE-A considers that expected/potential resource conflict occurs on the resource(s) satisfying at least one of the following condition(s):

      - Condition 2-A-1:

        - Other UE’s reserved resource(s) identified by UE-A are fully/partially overlapping with resource(s) indicated by UE-B’s SCI in time-and-frequency

        - FFS: Other details (if any)

        - FFS: Whether/how to specify additional criteria and other details (if any) including signaling details of conflict indication

      - (Working Assumption) Condition 2-A-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of preferred resource set:

    - UE-A considers any resource(s) satisfying all the following condition(s) as set of resource(s) preferred for UE-B’s transmission

      - Condition 1-A-1:

        - Resource(s) excluding those overlapping with reserved resource(s) of other UE identified by UE-A whose RSRP measurement is larger than a RSRP threshold

          - FFS: Other details (if any)

      - FFS: Condition 1-A-2:

        - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Condition 1-A-3:

        - Resource(s) satisfying UE-B’s traffic requirement (if available)

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

- Agreement:

  - In scheme 1, at least the following is supported to determine inter-UE coordination information of non-preferred resource set:

    - UE-A considers any resource(s) satisfying at least one of the following condition(s) as set of resource(s) non-preferred for UE-B’s transmission

      - Condition 1-B-1:

        - Reserved resource(s) of other UE identified by UE-A from other UEs’ SCI (including priority field) and RSRP measurement

          - FFS: Other details (if any)

      - FFS: Condition 1-B-2:

        - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B

          - FFS: Other details (if any)

      - FFS: Other condition(s)

    - FFS: Other details (if any)

## 5) RAN1#106bis-e meeting

- Agreement:

  - For Scheme 2, PSFCH format 0 is used to convey the presence of expected/potential resource conflict on reserved resource(s) indicated by UE-B’s SCI

- Agreement:

  - For Condition 2-A-1 of Scheme 2, down-select one or more of following additional criteria to determine resource(s) where expected/potential resource conflict occurs

    - Option 1: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

      - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations

      - Strive to reuse Rel-16 specification wherever possible

    - Option 2: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is within a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - Option 3: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) and the other UE is within a distance threshold of UE-B as determined by both UEs’ SCIs.

    - Option 4: The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

      - FFS: Whether the threshold depends on priority

    - FFS: In case of collisions of resources for two UEs having TBs with UE A as destination UE, if needed

- Working Assumption

  - For Condition 1-B-1 of Scheme 1, the following two options are supported

    - Option 1: Reserved resource(s) of other UE(s) identified by UE-A whose RSRP measurement is larger than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s)

    - Option 2: Reserved resource(s) of other UE identified by UE-A whose RSRP measurement is smaller than a (pre)configured RSRP threshold which is determined by at least priority value indicated by SCI of the UE(s) when UE-A is a destination of a TB transmitted by the UE(s)

- Working Assumption

  - For Scheme 1 with non-preferred resource set, support following condition:

    - Condition 1-B-2:

      - Resource(s) (e.g., slot(s)) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

- Agreement:

  - For Condition 1-A-1 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - When the inter-UE coordination information transmission is triggered by UE-B’s explicit request, the candidate single-slot resource(s) are determined in the same way according to Rel-16 TS 38.214 Section 8.1.4 with at least following parameters provided by signaling from UE-B. FFS whether or not to apply RSRP threshold increase in Step 7) of Rel-16 TS 38.214 Section 8.1.4.

      - Priority value to be used for PSCCH/PSSCH transmission

        - It replaces prio_TX

      - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

        - It replaces L_subCH

      - Resource reservation interval

        - It replaces P_rsvp_TX

      - FFS: Starting/ending time location of resource selection window

    - FFS : In addition to Rel-16 procedure, use inter-UE coordination information from other UEs

      - If there is no consensus in RAN1#106bis-e, no further discussions for Rel-17

- Conclusion:

  - No consensus that UE-A uses inter-UE coordination information from other UEs when it determines the preferred resource set for Condition 1-A-1 of Scheme 1.

- Working Assumption

  - For Scheme 1 with preferred resource set, support following condition:

    - Condition 1-A-2:

      - Resource(s) excluding slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation

      - This can be disabled by RRC (pre-)configuration

- Agreement:

  - For allocating PSFCH resources in Scheme 2, at least following can be (pre)configured separately from those for SL HARQ-ACK feedback.

    - Set of PRBs for PSFCH transmission/reception (sl-PSFCH-RB-Set)

- Agreement:

  - For Scheme 2,

    - Index of a PSFCH resource for inter-UE coordination information transmission is determined in the same way according to Rel-16 TS 38.213 Section 16.3 with at least following modification

      - P_ID is L1-Source ID indicated by UE-B’s SCI

      - M_ID is 0

    - FFS: How to set m_CS

    - FFS: How to set m_0

    - FFS: Whether M_ID can be (pre)configured

## 6) RAN1#107-e meeting

- Agreement:

  - A resource pool level (pre-)configuration uses either of the following options

    - Option 1: PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted

      - Reuse PSSCH-to-PSFCH timing as specified in TS 38.213 Section 16.3 to determine the PSFCH occasion for resource conflict indication

      - Time gap between the PSFCH and a slot where expected/potential resource conflict occurs is larger than or equal to T_3

    - Option 2: PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI

      - UE-A transmits the PSFCH in a latest slot that includes PSFCH resources for inter-UE coordination information and is at least T_3 slots of the resource pool before the PSSCH resource indicated by UE-B’s SCI in which expected/potential resource conflict occurs

      - FFS: How to account for processing timeline

    - Note that it is possible not to configure either option1 or option 2.

- Agreement:

  - For Condition 1-A-2 of Scheme 1, the set of resources preferred for UE-B’s transmission is a form of candidate single-slot resource as specified in Rel-16 TS 38.214 Section 8.1.4

    - UE-A excludes candidate single-slot candidate(s) belonging to “slot(s) where UE-A, when it is intended receiver of UE-B, does not expect to perform SL reception from UE-B due to half duplex operation” after Step 6) of TS 38.214 Section 8.1.4

- Agreement:

  - When PSFCH TX/RX for Scheme 2 is overlapping with LTE SL TX/RX and/or UL in a UE, reuse prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - For Scheme 2, the values of the following parameters are the same as those for SL HARQ-ACK feedback in the same resource pool

    - Period of PSFCH resources (sl-PSFCH-Period)

    - Number of cyclic shift pairs used for a PSFCH transmission that can be multiplexed in a PRB (sl-NumMuxCS-Pair)

    - Number of PSFCH resources available for multiplexing information in a PSFCH transmission (sl-PSFCH-CandidateResourceType)

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - Alt 1 (Working Assumption): MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

    - Alt 2: MAC CE is used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

    - FFS: Whether/How to use resource reservation information as coordination information

- Working Assumption:

  - A resource pool level (pre-)configuration can enable one of the following options:

    - Option 1:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for UE-B and other UE respectively

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a RSRP threshold according to the priorities included in the SCI:

            - prio_TX and prio_RX are the priorities indicated in the SCI making the overlapping reservations for other UE and UE-B respectively

    - Option 4:

      - For Condition 2-A-1 of Scheme 2, support following additional criteria to determine resource(s) where expected/potential resource conflict occurs

        - For the case when UE-A is a destination UE of a TB transmitted by UE-B

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) whose RSRP measurement is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of UE-B’s reserved resource.

        - For the case when UE-A is a destination UE of a TB transmitted by another UE

          - The resource(s) are fully/partially overlapping in time-and-frequency with other UE’s reserved resource(s) when RSRP measurement of UE-B’s reserved resource is larger than a (pre)configured RSRP threshold compared to the RSRP measurement of the resource(s).

      - Support of Option 4 is subject to UE capability

    - FFS: Whether/how RSRP threshold depends on priority, MCS, overlap

- Agreement:

  - For Scheme 1 with non-preferred resource set,

    - Physical layer at UE-B excludes in its resource (re-)selection, candidate single-slot resource(s) obtained after Step 6) of Rel-16 TS 38.214 Section 8.1.4 overlapping with the non-preferred resource set

- Agreement:

  - For Condition 1-A-1 of Scheme 1, when UE-A determines the set of resources preferred for UE-B’s transmission, apply RSRP threshold increase in the same way according to Rel-16 TS 38.214 Section 8.1.4.

    - FFS: Whether/how to introduce the maximum limit of RSRP threshold increase

- Agreement:

  - For Scheme 1, at least following parameters are provided by UE-B’s request:

    - Priority value to be used for PSCCH/PSSCH transmission

    - Number of sub-channels to be used for PSSCH/PSCCH transmission in a slot

    - Resource reservation interval

- Agreement:

  - For Scheme 2, when PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI,

    - Time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value.

      - FFS: Details of X

- Working Assumption:

  - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A, for each pair of UEs scheduling the conflicting TBs, a UE with the higher priority value is UE-B.

    - FFS whether/how to set additional condition for UE-A to send PSFCH.

    - Conclude on whether/how to handle, or differently handle, the case when at least one of UEs scheduling conflicting TBs doesn’t support Scheme 2 at the subsequent meetings

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1,

    - UE-A uses a TX resource pool used for UE-B’s request transmission to determine the set of resources and to transmit the set of resources to UE-B

- Agreement:

  - For inter-UE coordination information triggered by a condition rather than request reception in Scheme 1,

    - UE-A transmitting in a resource pool provides inter-UE coordination information associated with the same resource pool

## 7) RAN1#107bis-e meeting

- Agreement:

  - For Scheme 1, when the inter-UE coordination information transmission is triggered by UE-B’s explicit request,

    - Starting/Ending time locations of resource selection window is provided by UE-B’s explicit request

      - Starting/Ending time locations of resource selection window is a form of combination of DFN index and slot index

- Agreement:

  - When PSFCH occasion is derived by a slot where expected/potential resource conflict occurs on PSSCH resource indicated by UE-B’s SCI, time gap between the PSFCH and SCI(s) scheduling conflicting TBs is larger than or equal to X value

    - X = sl-MinTimeGapPSFCH

  - UE does not transmit the conflict indicator or receive the conflict indicator if the timeline is not satisfied

- Agreement:

  - For Scheme 1, a resource pool level (pre-)configuration can enable one of the following alternatives:

    - (Working assumption) Alt1: MAC CE and 2nd SCI are used as the container of an explicit request transmission from UE-B to UE-A

      - A single format SCI 2-C is used for inter-UE coordination information and request

        - 1 bit in format 2-C is used to indicate whether the SCI is used for request to coordination information or for conveying coordination information

      - SCI 2-C is UE RX optional

      - It is up to UE implementation to additionally use 2nd SCI (for UE-B).

    - Alt2: MAC CE is used as the container of an explicit request transmission from UE-B to UE-A

- Conclusion:

  - For Scheme 2, there is no consensus to support indication of the following

    - Condition type of a resource conflict

    - Time location of a resource conflict

- Agreement:

  - For Scheme 2,

    - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

    - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

    - FFS: Whether/How the conflict in periodic transmission is indicated by UE-A and handled by UE-B

- Agreement:

  - For PSFCH TX/RX or TX/TX prioritization in Scheme 2,

    - Priority value of PSFCH TX for a resource conflict indication is the smallest priority value of the conflicting TBs

    - Priority value of PSFCH RX for a resource conflict indication is priority value indicated by UE-B’s SCI

    - For PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s), PSFCH TX/RX for SL HARQ-ACK feedback is always prioritized over PSFCH TX/RX for a resource conflict indication

- Agreement:

  - For Scheme 1, unicast is supported for an explicit request transmission for inter-UE coordination information

    - Unicast is used for the inter-UE coordination information transmission triggered by the explicit request

- Working Assumption:

  - For Scheme 1, following cast type(s) are supported for inter-UE coordination information transmission triggered by a condition other than explicit request reception

    - Groupcast/Broadcast for non-preferred resource set, FFS for preferred resource set

      - FFS: Under which conditions groupcast/broadcast can be supported

    - Unicast

      - FFS: Under which conditions unicast can be supported

- Agreement:

  - For determining preferred resource set in Scheme 1, the value of Cresel is determined by UE-A according to Rel-16 procedure.

    - This information is not conveyed to/from UE-B

    - When inter-UE coordination information is triggered by UE-B’s request, P_rsvp_TX used for determining SL_RESOURCE_RESELECTION_COUNTER according to Rel-16 procedure is provided by resource reservation interval indicated by UE-B’s request

- Agreement:

  - For the indication of resource set in Scheme 1, the value of Sl-MaxNumPerReserve is fixed to 3.

- Agreement:

  - The following working assumption is confirmed with modification in RED.

    - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

      - For the indication of resource set, the following is supported:

        - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

          - First resource location of each TRIV is separately indicated by the inter-UE coordination information

        - If [N <= 3], MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If [N > 3], only MAC CE is used.

          - FFS: UE capability details

          - 2nd SCI is UE RX optional

          - The field size of the indication of resource set in a SCI format 2-C is determined by [N=3]

- Agreement:

  - For inter-UE coordination information transmission in Scheme 1,

    - Inter-UE coordination information can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying inter-UE coordination information is supported

  - For explicit request transmission in Scheme 1,

    - Explicit request can be multiplexed with other data only if the source/destination ID pair is the same

      - Retransmission of the TB carrying request is supported

- Agreement:

  - For inter-UE coordination triggered by an explicit request in Scheme 1, whether or not to transmit the inter-UE coordination information upon the request reception is determined by UE-A’s implementation subject to the following procedures.

    - Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control

- Agreement:

  - For inter-UE coordination triggered by a condition rather than request reception in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-A’s implementation whether or not to trigger the inter-UE coordination information generation.

      - Alt 2: the inter-UE coordination information generation can be triggered only when UE-A has data to be transmitted together with the inter-UE coordination information to UE-B

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the inter-UE coordination information triggered by a condition.

- Agreement:

  - For inter-UE coordination triggered by UE-B’s explicit request in Scheme 1,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1: it is up to UE-B’s implementation whether or not to trigger the request generation

      - Alt 2: the request generation can be triggered only when UE-B has data to be transmitted to UE-A

    - Note: Rel-16 procedure of UL/SL prioritization, LTE SL/NR SL prioritization, and congestion control is applied to the transmission of the request transmission.

- Agreement:

  - For Scheme 1 with preferred resource set Option A,

    - MAC layer selects resources using S_A and the received preferred resource set

      - MAC layer firstly selects resources for transmissions within the intersection of S_A and the preferred resource set until it becomes impossible to select a resource within the intersection under the constraint defined in Rel-16.

        - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

      - After this, if the number of selected resources is smaller than the required number of transmissions for a TB, MAC layer selects resources for the remaining transmissions outside the intersection but inside S_A under the constraint defined in Rel-16.

- Agreement:

  - For Scheme 1 with preferred resource set Option B,

    - MAC layer selects resources belonging to the received preferred resource set under the constraint defined in Rel-16

      - It is up to the UE whether to use the preferred resource set from SCI format 2-C and/or MAC CE

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as indicated by UE-B’s explicit request.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For inter-UE coordination information triggered by an explicit request in Scheme 1, the priority value of explicit request is (pre)configured priority value if it is provided by (pre)configuration. Otherwise, the priority value is the same as that of a TB to be transmitted by UE-B.

    - For the case when the explicit request is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the explicit request and data

- Agreement:

  - For inter-UE coordination information triggered by a condition other than explicit request reception in Scheme 1, the priority value of the inter-UE coordination information is (pre)configured priority value if it is provided by (pre)configuration.

    - FFS: Otherwise, the priority value is determined by UE-A’s implementation.

    - For the case when inter-UE coordination information is transmitted together with other data, the priority value of the multiplexed sidelink transmission is determined by the smallest priority value between the inter-UE coordination information and data

- Agreement:

  - For sidelink transmission carrying inter-UE coordination information in Scheme 1,

    - UE-A performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the inter-UE coordination information to UE-B.

  - For sidelink transmission carrying request in Scheme 1,

    - UE-B performs its resource (re)selection according to the same procedure in TS 38.214 Section 8.1.4 to transmit the request for the inter-UE coordination information to UE-A if UE-B performs sensing/resource exclusion. Otherwise, at least UE-B can perform random selection

  - Note: RAN1 does not pursue specific enhancement of Rel-17 resource (re)selection for the transmission of inter-UE coordination information and its request.

- Working assumption:

  - First resource location of each TRIV is a slot offset with respect to a reference slot

    - Alt 2:

      - The slot offset is the number of logical slots from the reference slot

        - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [256]

          - FFS: The detailed value range including granularity

        - Slot offset for each TRIV to indicate the set of resources is separately indicated by inter-UE coordination information

    - For the reference slot,

      - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement:

  - For determining preferred resource set in Scheme 1, when inter-UE coordination information transmission is triggered by a condition other than explicit request reception,

    - Values of following parameters are (pre)configured for a resource pool. If there is no (pre)configuration, UE-A determines by its implementation the values of the following parameters

      - prio_TX

      - L_subCH

      - P_rsvp_TX

    - UE-A determines by its implementation values of following parameters

      - n+T_1, n+T_2

    - FFS: Whether/how to support (pre)configuration of n+T_1 and n+T_2

    - Note that it is up to RAN2 decision whether/how the values of these parameters are provided by PC5-RRC signaling from UE-B to UE-A and UE-A uses the received information to determine the preferred resource set

- Agreement:

  - For inter-UE coordination information is triggered by UE-B’s request,

    - A resource pool level (pre-)configuration can enable one of the following alternatives:

      - Alt 1:

        - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

          - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

      - Alt 2:

        - Resource set type to be provided by inter-UE coordination information transmission is indicated by UE-B’s request

          - UE-B’s request indicates either preferred resource set or non-preferred resource set

    - Note that it is up to RAN2 decision whether/how UE-B provides its support of sensing/resource exclusion to UE-A via PC5-RRC signaling and UE-A uses the received information to determine the type of resource set to be transmitted to UE-B

- Agreement:

  - For inter-UE coordination information is triggered by a condition other than explicit request reception,

    - Resource set type to be provided by inter-UE coordination information transmission is determined by UE-A’s implementation and its information is indicated by UE-A’s inter-UE coordination information

      - UE-A’s inter-UE coordination information indicates either preferred resource set or non-preferred resource set

- Working assumption:

  - For Scheme 2, (pre)configuration is supported to enable or disable that 1 LSB of reserved bits of a SCI format 1-A is used to indicate of whether UE scheduling a conflict TB can be UE-B or not.

    - FFS: UE-A's behavior for the case when at least one of UEs scheduling conflicting TBs is not capable of receiving the conflict indication

## 8) RAN1#108-e meeting

- Agreement:

  - For a slot offset that is (pre)configured to indicate the first resource location of each TRIV with respect to a reference slot,

    - Granularity of the slot offset is 1 logical slot

    - (Pre)configured maximum value of the slot offset is up to 8000

      - When both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the maximum value of the slot offset is 255

      - When MAC CE only is used as the container of inter-UE coordination information, the maximum value of the slot offset is the (pre)configured maximum value

- Agreement:

  - A SCI format 2-C includes all the fields present in SCI format 2-A except cast type indicator

- Conclusion:

  - For cast type(s) of inter-UE coordination information with preferred resource set triggered by a condition other than explicit request reception, there is no consensus in RAN1 on the support of groupcast or broadcast for preferred resource set

- Agreement

  - For Scheme 2, m_CS for a resource conflict indication for the next reserved resource indicated by the corresponding UE-B’s SCI for either current TB transmission or next TB transmission is 0

- Agreement

  - For Scheme 2, when UE-B receives a conflict indicator for resource(s) indicated by its SCI, it up to UE-B’s implementation whether/how to set the reservation periodicity in the re-selected resource.

- Agreement

  - For Scheme 2,

    - m_0 for a resource conflict indication is derived in the same way as specified for HARQ-ACK information in TS 38.213 Section 16.3

    - A UE expects that different PRBs are (pre)configured between conflict indication and HARQ-ACK information

- Agreement:

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Agreement:

  - For Scheme 1, when MAC CE only is used as the container of an explicit request for inter-UE coordination information, the same bit field size for the request in a SCI format 2-C is applied to MAC CE

- Conclusion:

  - For inter-UE coordination operation in Rel-17, RAN1 understands that only UE(s) in mode 2 can be UE-A

    - Note that RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the case where UE(s) in mode 1 can be UE-A

- Agreement

  - Confirm the following working assumption with modification in RED

- Working assumption made in RAN1#107bis-e: - First resource location of each TRIV is a slot offset with respect to a reference slot - Alt 2: - The slot offset is the number of logical slots from the reference slot - The value range of slot offsets is from 0 to maximum value that is (pre)configurable up to [8000256] - FFS: The detailed value range including granularity - Slot offset for each TRIV except for first TRIV to indicate the set of resources is separately indicated by inter-UE coordination information - Slot offset for first TRIV is 0 - For the reference slot, - The reference slot is the slot indicated by the inter-UE coordination information in a form of combination of DFN index and slot index

- Agreement

  - MAC CE or 2nd SCI are used as the container of inter-UE coordination information transmission from UE A to UE B.

    - For the indication of resource set, the following is supported:

      - N combinations of TRIV, FRIV, resource reservation period as specified in Rel-16 TS 38.214 Section 8.1.5 with following modification. The value of resource reservation period is omitted at least when the transmission of preferred resource set is triggered by UE-B’s explicit request.

        - First resource location of each TRIV is separately indicated by the inter-UE coordination information

      - If N <= 2, MAC CE is used and it is up to UE implementation to additionally use 2nd SCI. When 2nd SCI and MAC CE are both used, the same resource set is indicated in the 2nd SCI and the MAC CE. If N > 2, only MAC CE is used.

        - FFS: UE capability details

        - 2nd SCI is UE RX optional

        - The field size of the indication of resource set in a SCI format 2-C is determined by N=2

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for inter-UE coordination information is given by following table:

    - Note that lowest subchannel index for the first resource location of each TRIV is separately indicated by inter-UE coordination information

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | 2*log2(N subchannel SLNsubchannel SL + 12Nsubchannel SL + 16)+9+Y Where N subchannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_period with that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResource is configured; Y=0 otherwise. |
| First resource location(s) | 8 |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | 2*log2NsubchannelSL where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel |
| (FFS) Actual number of resource combination | 1 Note: Support of this field is to be concluded by Feb 28. |

- Agreement

  - For Scheme 1, each bit field size of a SCI format 2-C for an explicit request for inter-UE coordination information is given by following table:

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Priority | 3 |
| Number of subchannels | log2(N subChannel SL) Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel |
| Resource reservation period | Y Where Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| Resource selection window location | 210+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 bit if determineResourceSetTypeScheme1 is set to ‘UE-B’s request’, otherwise, 0 bit |

  - This agreement does not imply that new field requested by RAN2 cannot be further added.

- Agreement

  - For Scheme 1, when MAC CE only is used as the container of inter-UE coordination information, each bit field size for inter-UE coordination information is given by following table from RAN1’s perspective, and RAN1 understands that the maximum value of N resource combinations to be conveyed in inter-UE coordination information is bounded so that the total payload size of inter-UE coordination information leads not to exceed the size of TB including the MAC CE

    - Details (e.g., whether/how to separately indicate the value of N in the inter-UE coordination information, how to put the following fields into MAC CE and the related field sizes in MAC CE) are up to RAN2

| Field name | Field size (in bits) |
| --- | --- |
| Providing/requesting indicator | 1 |
| Resource combination(s) | N*log2(N subChannel SLN subChannel SL + 12N subChannel SL + 16)+9+Y Where N subChannel SL is provided by the higher layer parameter sl-NumSubchannel, Y=log2Nrsv_periodwith that Nrsv_period is the number of entries in the higher layer parameter sl-ResourceReservePeriodList, if higher layer parameter sl-MultiReserveResoure is configured; Y=0 otherwise. |
| First resource location(s) | N-1*log2(X) Where X is provided by the (pre)configured maximum value of slot offset for the case when MAC CE only is used as a container of inter-UE coordination information |
| Reference slot location | 10+log2(10∙2μ) Where μ is 0, 1, 2, 3 for SCS of 15kHz, 30kHz, 60kHz, 120kHz, respectively. |
| Resource set type | 1 |
| Lowest subchannel indices for the first resource location of each TRIV | N*log2NsubchannelSL Where NsubchannelSL is provided by the higher layer parameter sl-NumSubchannel. |

- Conclusion:

  - There is no consensus in RAN1 on indicating actual number of resource combination in a SCI format 2-C for inter-UE coordination information.

    - Note: Different resource combinations can indicate the same set of resources for the case when only one resource combination is actually used

- Agreement

  - For Scheme 2,

    - The PHY layer reports S_A after Step 7) of TS 38.214 Section 8.1.4 to higher layer.

    - When UE-B receives a conflict indicator for resource(s) indicated by its SCI,

      - PHY layer at UE-B reports resources overlapping with the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

        - If (pre)configured, the PHY layer reports resources in a slot including the next reserved resource indicated by the corresponding UE-B’s SCI for current TB transmission to higher layer.

      - Higher layer at UE-B re-selects the resource(s) indicated by the conflict indicator among the S_A excluding the reported resources.

- Agreement

  - Confirm the following working assumption with modification in RED. Note that the terminology of “indicationUEB flag” means the indication of whether UE scheduling a conflict TB can be UE-B or not.

    - Working Assumption:

      - For Condition 2-A-1 in Scheme 2, when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is enabled or when “a non-destination UE of a TB transmitted by UE-B can be UE-A” is disabled and the destination UE of the conflicting TBs is UE-A,

        - for each pair of UEs scheduling the conflicting TBs whose PSFCH occasions for resource conflict indication are not yet passed and indicationUEB flag is set to 1 if the higher parameter of indicationUEBScheme2 is (pre)configured to ‘Enabled’, a UE with the higher priority value is UE-B. When the UEs in the pair have the same priority value, UE-A determines one of the UEs to be UE-B by its implementation.

          - UE-A considers the SCIs received earlier than or equal to sl-MinTimeGapPSFCH before the PSFCH occasion for conflict indication when determining UE-B.

- Agreement

  - A UE performs PSFCH TX/RX or TX/TX prioritization between SL HARQ-ACK feedback(s) and resource conflict indication(s) first, and then the UE performs prioritization between prioritized PSFCH TX(s) or RX(s) and LTE SL TX/RX or UL by reusing prioritization rule as specified in TS 38.213 Section 16.2.4.1 and 16.2.4.3.1.

- Conclusion:

  - RAN1 does not pursue specific enhancement of Rel-17 inter-UE coordination operation for handling the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication, i.e., there is no case in Rel-17 where the overlapping between UL with SL-HARQ-ACK information and PSFCH for a conflict indication occur at a UE performing inter-UE coordination operation

- Conclusion:

  - There is no consensus in RAN1 to further introduce enhancement in Rel-17 on Mode 2 resource selection procedure to ensure the timeline (i.e., minimum time gap between PSFCH and a slot where a SCI is transmitted of sl-MinTimeGapPSFCH, minimum time gap between PSFCH and a slot where expected/potential resource conflict occurs on PSSCH resource indicated by a SCI of T_3) for a conflict indication.

- Agreement

  - For Scheme 1, when both SCI format 2-C and MAC CE are used as the container of inter-UE coordination information, the same inter-UE coordination information is indicated in the SCI format 2-C and the MAC CE

    - Details (e.g., how to put the fields of SCI format 2C for inter-UE coordination information into MAC CE and the related field sizes in MAC CE) are up to RAN2

- Conclusion:

  - When PSFCH occasion is derived by a slot where UE-B’s SCI is transmitted,

    - if there is a PSFCH occasion satisfying “the minimum time gap (sl-MinTimeGapPSFCH) between the PSFCH occasion and a slot where the SCI is transmitted” but not satisfying “the minimum time gap (T_3) between the PSFCH occasion and a slot of the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion”,

      - the PSFCH occasion cannot be used by UE-A for a conflict indication for reserved PSSCH resource other than the earliest reserved PSSCH resource indicated by the corresponding SCI after the PSFCH occasion

- Agreement

  - (Pre)configuration of parameters related to n+T_1 and n+T_2 for determining the set of preferred resources in inter-UE coordination information triggered by a condition other than explicit request reception is not supported.

    - Note that T_2 is no smaller than T_2,min and 0 <= T_1 <= Tproc,1 as specified in TS 38.214 section 8.1.4.

- Agreement

  - For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

- Agreement

  - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the same UE-A

    - It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

  - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the same UE-A

    - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e

  - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A

    - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Agreement

  - For UE-B’s behavior when UE-B receives multiple preferred resource sets from the different UE-As,

    - UE-B uses each received preferred resource set for its resource selection for each TB to be transmitted to each UE-A providing the preferred resource set.

  - Conclusion: UE-B’s behavior when UE-B receives multiple non-preferred resource sets from the different UE-As.

    - No RAN1 specification change to TS38.214 is deemed necessary in RAN1#108-e (except for the processing timeline)

  - For UE-B’s behavior when UE-B receives both a single preferred resource set and a single non-preferred resource set from the different UE-As,

    - FFS: It is up to UE-B implementation to use one or multiple of them in its resource (re)selection

- Agreement

  - Notations:

    - (n+T_1) – Start slot of resource selection window for determining the set of resources

      - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_1) is provided by UE-B’s request as per the existing agreement

      - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_1) is determined by UE-A’s implementation as per the existing agreement

    - (n+T_2) – End slot of resource selection window for determining the set of resources

      - For inter-UE coordination information triggered by UE-B’s explicit request, this value of (n+T_2) is provided by UE-B’s request as per the existing agreement

      - For inter-UE coordination information triggered by a condition other than explicit request reception, this value of (n+T_2) is determined by UE-A’s implementation as per the existing agreement

    - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

    - (n’+T’_2) – End slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

    - n' is the slot where UE procedure of determining TX resources of sidelink transmission carrying inter-UE coordination information is triggered

  - For inter-UE coordination information triggered by UE-B’s explicit request

    - Alt 1-1:

      - X1 ≤ (n’+T’_1)

      - (n’+T’_2) ≤ X2

  - For inter-UE coordination information triggered by a condition other than explicit request reception,

    - Alt 2-2:

      - (n’+T’_2) < X3

  - FFS: Values for X1, X2, X3

- Agreement

  - For sensing window for determining the set of resources in Scheme 1,

    - Notations:

      - For inter-UE coordination information triggered by UE-B’s explicit request, the values of (n+T_1) and (n+T_2) are provided by the request as per the existing agreement.

      - For inter-UE coordination information triggered by a condition other than explicit request reception, the values of (n+T_1) and (n+T_2) are determined by UE-A’s implementation as per the existing agreement.

      - T’’_1 is up to UE-A’s implementation under 0 <= T’’_1 <= Tproc,1

      - (n’+T’_1) – Start slot of resource selection window used for sidelink transmission carrying inter-UE coordination information

      - n' is the slot where UE procedure of determining TX resources of inter-UE coordination information is triggered

    - Alt 1:

      - No further change is supported. Note that the sensing window for determining the set of resources is already derived based on the location (n+T_1) and (n+T_2) used for determining the set of resources in TS38.214 section 8.1.4, i.e., sensing window is defined by the range of slots [(n+T_1) – T_0 – T’’_1, (n+T_1) – T_proc,0 – T’’_1].

- Agreement

  - For the case when it is not possible that the number of candidate single-slot resources after applying the received non-preferred resource set as per the existing agreement meets the requirement of X*M_total in step 7),

    - It is up to UE-B’s implementation whether to take the received non-preferred resource set in its resource selection after step 6) to meet this requirement

## 9) RAN1#109-e meeting

- Agreement:

  - TP#1 (for TS38.213 v17.1.0, clause 16.4) in section 8.1 of R1-2203717 is endorsed.

  - TP#2 (for TS38.213 v17.1.0, clause 16.2.3) in section 8.2 of R1-2203717 is endorsed.

- Agreement:

  - When UE-B receives both a single preferred resource set and a single non-preferred resource set from the same UE-A or different UE-As, when UE-B has own sensing results

    - No RAN1 specification change to TS 38.214 is deemed necessary on how it uses the non-preferred resource set in its resource (re)selection

    - It is up to UE-B implementation to use the preferred resource set in its resource (re)selection for transmissions to the UE A providing the preferred resource set

- Agreement:

  - Remove the sentence below in Section 8.1.5A of TS 38.214

[When a preferred resource set is indicated by an SCI format 2-C, if the transmission of the set was triggered by an explicit request, the value of the resource reservation interval P_(rsvp,m) is set to 0.]

- Agreement:

  - X1, X2, and X3 are determined by UE-A’s implementation under the constraints defined in the specification (e.g., SL-LatencyBoundIUC-Report-r17, requirement of T_2min)

    - UE-B can choose to not use any resource from the preferred/non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission

      - For Tproc,2,

        - When only MAC CE is used for inter-UE coordination information transmission, it is equal to (Tproc,0+Tproc,1)

        - When MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission, it is equal to Tproc,0

          - Note: this is assuming that SCI format 2-C is received

    - Whether or not to make the time gap from the resource of inter-UE coordination information transmission to preferred/non-preferred resource in the inter-UE coordination information larger than (Tproc,0+Tproc,1+Tproc,2) is up to UE-A implementation

- Agreement:

  - Adopt the following text proposal to TS 38.214 v17.1.0:

    - Reason for change:

      - RAN1 discussed how to alleviate an ambiguity of UE-B’s behavior in terms of handling the field of resource reservation period in the inter-UE coordination information considering that there is no special mechanism that makes UE-B distinguishable whether the inter-UE coordination information is triggered by an explicit request or triggered by a condition, and agreed that this field is always valid.

    - Summary of change:

      - Delete the sentence of “[When a preferred resource set is indicated by an SCI format 2-C, if the transmission of the set was triggered by an explicit request, the value of the resource reservation interval P_(rsvp,m) is set to 0.]” in Section 8.1.5A of TS 38.214.

    - Consequences if not approved:

      - The specification is not clear regarding UE-B’s behavior on handling the field of resource reservation period in the inter-UE coordination information.

----------------------- Start of text proposal to TS 38.214 v17.1.0 ----------- 8.1.5A UE procedure for determining slots and resource blocks indicated by a preferred or non-preferred resource set The set of slots and resource blocks indicated by a set of preferred or non-preferred resource(s) is determined as described below. The set of preferred or non-preferred resources r0,r1,r2,…, is indicated by a reference slot tref and M tuples (TRIVm,FRIVm,Prsvp,m), 1≤m≤M indicated by the 'resource combination(s)' field, where for each tuple TRIVm is indicated by the 9 MSBs, followed by FRIVm and Prsvp,m (if present). The reference slot tref is indicated by the 'reference slot location' field as a combination of DFN index and slot index [5, TS 38.212], with the 10 MSBs indicating the DFN index. TRIVm and FRIVm are interpreted according to clause 8.1.5, with the following modifications: - - the value of sl-MaxNumPerReserve is fixed to 3. - - "slot where SCI format 1-A was received" is replaced by slot indicated as the first resource location of a TRIVm. - - the first resource location of each TRIVm for m>1 is indicated by a slot offset tm in logical slots with respect to the reference slot tref; the first resource location of TRIV1 is at slot offset 0 with respect to the reference slot.- "the received SCI format 1-A, except the resource in the slot where SCI format 1-A was received" is replaced by "each tuple" - - the starting sub-channel nsubCH,0start of the first resource of each tuple is separately indicated. The starting sub-channel nsubCH,0start of the first resource of each tuple is indicated by the '[lowest subchannel index for the first resource location of each TRIV]' field. The resource reservation period Prsvp,m is encoded as in SCI format 1-A. If the set is indicated by an SCI format 2-C, the number of tuples is M=2. A UE forms the union of the subsets indicated by each tuple (TRIVm,FRIVm,Prsvp,m) to obtain the set r0,r1,r2,…. Prsvp,m ------------------------ End of Text proposal to TS 38.214 v17.1.0 ----------

## 10) RAN1#110 meeting

- Agreement

  - Reason for change:

    - The RAN1 agreement related to UE-B's behaviour of determining a resource used for its resource (re-)selection among the non-preferred resource set received from UE-A is not captured in the specification.

  - Summary of change:

    - The relevant UE-B's behaviour is described in Section 8.1.4C of TS 38.214.

  - Consequences if not approved:

    - The specification text is unclear as to which resource UE-B uses for its resource (re-)selection among the non-preferred resource set received from UE-A

--------------------------- Start of text proposal to TS 38.214 v17.2.0 --------------------------- 8.1.4C UE procedure for using a received non-preferred resource set A UE configured with the higher layer parameter interUECoordinationScheme1 uses a received non-preferred resource set as follows when performing resource (re-)selection: - the UE excludes in Step 6b) of clause 8.1.4 resource(s) overlapping with the non-preferred resource set. Note: If it is not possible to meet the requirement that the number of candidate single-slot resources remaining in the set SA be at least X⋅Mtotal after excluding resource(s) overlapping with the received non-preferred resource set, it is up to UE implementation whether or not to take into account the received non-preferred resource set to meet such requirement. Note: The UE is not required to use any resource from the non-preferred resource set in its resource (re-)selection if that resource is earlier than (Tproc,0+Tproc,1+Tproc,2) after the resource of inter-UE coordination information transmission, where Tproc,2 is equal to (Tproc,0+Tproc,1) when only MAC CE is used for inter-UE coordination information transmission, or Tproc,2 is equal to Tproc,0 when MAC CE and SCI format 2-C are both used for inter-UE coordination information transmission. Note: the case when Tproc,2 is equal to Tproc,0 is assuming that SCI format 2-C is received. --------------------------- End of Text proposal to TS 38.214 v17.2.0 ---------------------------

- Agreement

  - Reason for change

    - Regarding the cast type for SCI format 2-C, the following agreement was reached. However, it seems that there is no corresponding text in the specification.

Agreement For inter-UE coordination information transmission, only when the cast type of inter-UE coordination information is unicast regardless of whether or not it is multiplexed with other data, a SCI format 2-C can be used in addition to MAC CE

  - Summary of change

    - Clarify that SCI format 2-C can be used only for unicast.

  - Consequences if not approved

    - The cast type for which SCI format 2-C was agreed is not explicitly reflected in the specification.

--------------------------- Start of text proposal to TS 38.212 v17.2.0 --------------------------- 8.4.1.3 SCI format 2-C SCI format 2-C is used for the decoding of PSSCH, and providing inter-UE coordination information or requesting inter-UE coordination information. SCI format 2-C can be used only for unicast. The following information is transmitted by means of the SCI format 2-C: <Unchanged parts omitted> --------------------------- End of Text proposal to TS 38.212 v17.2.0 ---------------------------

## 11) RAN1#110bis-e meeting

- Agreement

  - Adopt the TP below for TS 38.213

--------------------------- Start of text proposal to TS 38.213 v17.3.0 --------------------------- ### 16.3.0 UE procedure for transmitting PSFCH with control information < Unchanged parts are omitted > A first UE determines a second UE for providing the conflict information to in a PSFCH as follows - if the first UE is an intended receiver of the second UE for a reserved resource of a PSSCH transmission in a slot, - does not expect to perform reception on the sidelink due to half-duplex operation in the slot, and - the PSFCH occasion for resource conflict information of the second UE is valid, and - determines to transmit to the second UE the PSFCH with the conflict information. < Unchanged parts are omitted > --------------------------- End of Text proposal to TS 38.213 v17.3.0 ---------------------------

- Agreement

  - Draft proposal 2-10a (I) for Issue #2-10 in Section 2.1.3.2 of R1-2210336 is endorsed in principle.

- Agreement

  - The draft CR in R1-2210624 to reflect above two agreements is endorsed in principle.

  - Final CR is agreed in R1-2210732 (TS38.213, Rel-17, CR#0403, Cat F).

- Agreement

  - The TP in draft CR R1-2210039 (summarized as TP#1 for Issue #3-3 in section 4.1.1 of R1-2210335) is endorsed in principle.

- Agreement

  - The draft CR in R1-2210623 to reflect above agreement is endorsed in principle.

  - Final CR is agreed in R1-2210731 (TS38.211, Rel-17, CR#0103, Cat F).

- Agreement

  - Draft proposal 2-4 (III) for Issue #2-4 in Section 5.1.2 of R1-2210336 is endorsed in principle.

  - The corresponding draft CR is endorsed in principle in R1-2210625.

  - Final CR is agreed in R1-2210733 (TS38.213, Rel-17, CR#0404, Cat F).

- Conclusion:

  - There is no consensus in RAN1 on the support of PDCCH repetition for search space sets configured with SL DCI format 3-0 and/or 3-1 in Rel-17.

  - Send LS to RAN2 to inform that this conclusion and the current RAN1 specification does not cover the case for SL DCI format 3-0 and/or 3-1 with PDCCH repetition.

- Agreement

  - Draft LS corresponding to above conclusion is endorsed in R1-2210734. Final LS in R1-2210735.

- Agreement

  - For alignment TS38.214 CR:

    - TP#1 for Issue #2-7 in section 4.2.1 of R1-2210335 is endorsed for the editorial corrections.

    - TP#2 for Issue #2-8 in section 4.2.2 of R1-2210335 is endorsed for the editorial corrections.

  - For alignment TS38.213 CR:

    - TP#3 for Issue #2-9 in section 4.2.3 of R1-2210335 is endorsed for the editorial corrections.

    - TP#4 for Issue #2-17 in section 4.2.4 of R1-2210335 is endorsed for the editorial corrections.

    - TP#5 for Issue #2-12 in section 4.2.5 of R1-2210335 is endorsed for the editorial corrections.

  - For alignment TS38.212 CR:

    - TP#6 for Issue #2-13 in section 4.2.6 of R1-2210335 is endorsed for the editorial corrections.

- Agreement

  - For alignment TS38.214 CR:

    - Draft proposal 2-2 (III) for Issue #2-2 in Section 5.2.1 of R1-2210336 is endorsed for the editorial corrections.

- Agreement

  - TP#7 for Issue #2-11 in Section 4.2.7 of R1-2210336 is endorsed for the editorial corrections.

- Reference

- R1-2210838	Discussion on correction for inter-UE coordination Scheme 2 determination of UE-B	FUTUREWEI

- R1-2210895	Correction on power control for PSCCH/PSSCH/PSFCH/S-SSB in TS 38.213	Huawei, HiSilicon

- R1-2210978	Clarification on IUC related transmission based on latency bound	vivo

- R1-2210979	Correction for SL power control parameters	vivo

- R1-2211144	Correction on resource exclusion behavior with non-preferred resource set	CATT, GOHIGH

- R1-2211145	Discussion on resource exclusion behavior with non-preferred resource set	CATT, GOHIGH

- R1-2211262	Clarification on cast type for IUC scheme 1	LG Electronics

- R1-2211445	Draft CR on UE procedure for determining a resource conflict	OPPO

- R1-2211795	Correction on determining UE-B among UEs scheduling conflicting TBs	Apple

- R1-2211855	Clarification on determination of preferred resource set for IUC scheme 1	Sharp

- R1-2211960	Draft CR on condition to be UE-A for SL IUC scheme 2	NTT DOCOMO, INC.

- R1-2211964	Draft CR on removal of duplicated spec description on IUC scheme 2	NTT DOCOMO, INC.

- R1-2211965	Discussion on RAN2-related topics for SL maintenance	NTT DOCOMO, INC.

- R1-2212088	Draft CR on Non-preferred Resources	Qualcomm Incorporated

- R1-2212089	Draft CR on Power Control Parameters	Qualcomm Incorporated

- R1-2212204	Correction on Condition 2-A-1 of IUC scheme 1	ZTE, Sanechips

- R1-2212205	Clarification on UE procedure for receiving PSFCH with conflict information	ZTE, Sanechips

- R1-2212217	[Draft] Clarification on valid PSFCH occasions for resource conflict information	Ericsson

- R1-2212220	[Draft] Modifications on SL open loop power control formulae	Ericsson

- R1-2212447	Corrections for SL Inter-UE coordination	Nokia, Nokia Shanghai Bell

- R1-2212464	Correction on UE-A is the destination UE of a TB transmitted by UE-B in TS 38.214	Huawei, HiSilicon

- R1-2212465	Correction on description of valid PSFCH occasion for scheme 2 in TS 38.213	Huawei, HiSilicon