3GPP TSG RAN WG1 #111			R1-2212700

Toulouse, France, November 14th – 18th, 2022

Agenda item:	9.10.1

Source: 	Moderator (Qualcomm Incorporated)

Title: 	Moderator Summary #3 on XR specific power saving techniques

Document for:	Discussion and Decision

# Introduction

In RAN1 #109-e, several issues were identified for power saving enhancements for Rel-18 XR SI [1]. It was also agreed that Rel-17 evaluation methodology for XR power consumption will be used as baseline for Rel-18 XR SI [1]. Companies are expected to provide thorough comparison with existing power saving features. For the SI, evaluation results are necessary for any proposed enhancement to be captured as RAN1 input to TR 38.835 [1]. In this meeting, companies’ proposals and evaluation results will be further discussed.

Please check “Observation” and “Question 1” and “Question 2” under each “Discussion 1” section, order of discussions in Section 4 and “FL proposal” in Section 5.

# CDRX related enhancements

Semi-static periodicity alignment between CDRX and XR traffic

In the past meetings, companies evaluated semi-static periodicity alignment between CDRX and XR traffic and some agreements were made.

Ericsson proposed to configure a new DRX cycle via (i) a fixed time shift for the start of drx-onDurationTimer; and (ii) a number of DRX cycles after which the new shift should be added to align the DRX and XR periodicity. Huawei proposed to configure multiple start offsets within one C-DRX cycle to align with multiple XR data arrivals. vivo proposed to adopt semi-static solutions to align DRX cycle with XR traffic periodicity. OPPO proposed a two-cycle DRX pattern to resolve mismatch between non-integer XR traffic arrival cycle and integer DRX cycle. Intel proposed that RAN1 recommends the semi-static solution for CDRX alignment with XR traffic periodicity by configuring a periodic pattern of CDRX cycles such as consecutive DRX cycles with two DRX cycle values or start offset adjustments every N DRX cycles. ZTE compared performance for various semi-static solutions including non-uniform CDRX cycle, uniform non-integer CDRX cycle, multiple CDRX configurations. Similar performance is observed for these solutions. Qualcomm proposed semi-static configuration of enhanced DRX with non-integer rational numbers in DRX cycles and add floor operations in DRX formulas. CATT evaluated the semi-static alignment and showed that this solution has caused a significant capacity loss. MTK proposed to Introduce non-integer (rational number) DRX cycles to match typical XR traffic patterns

RAN1 #110 Agreement RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity RAN1 #110bis-e Agreement - For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1) - Prioritize semi-static solutions - FFS: Whether dynamic solutions will be also needed

Table 1: Evaluation results for semi-statically aligning CDRX with XR traffic periodicity

| Company | Evaluation results |
| --- | --- |
| Ericsson | Table 2: Results for FR1, high load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 91.7%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 8; 7; 86.9%; 4.3%; 4.9%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 8; 6; 78.2%; 10.4%; 11.0%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.2%; 10.5%; 11.4%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; H; 8; 7; 87.0%; 10.1%; 10.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.3%; 15.8%; 16.0%. Table 3: Results for FR1, low load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; L; 2; 8; 100%. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; L; 2; -; 100%; 4.8%; 4.8%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; L; 2; -; 100%; 12.4%; 12.4%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 12.8%; 12.8%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; L; 2; -; 100%; 12.5%; 12.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 22.5%; 22.5%. Table 4: Results for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 90.1%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 8; 7; 86.9%; 2.6%; 2.7%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 8; 6; 80.2%; 6.1%; 6.1%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 85.5%; 7.2%; 7.1%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; H; 8; 7; 86.4%; 7.6%; 7.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 84.7%; 7.6%; 7.6%. |
| Huawei | Table 2. Simulation results of enhanced C-DRX, Dense Urban, DL VR/AR@30Mbps Power saving scheme; CDRX cycle and/or start offsets (ms); ODT (ms); IAT (ms); Load H/L; #UE /cell; floor (Capacity); % of DL satisfied UE; Mean PSG of all UEs (%). Always On; -; -; -; H; 11; 11; 93.42%; -. Legacy C-DRX; 16; 12; 4; H; 11; 11; 83.20%; 5.57%. Enhanced C-DRX; 50 {0, 16, 33}; 12; 4; H; 11; 11; 91.43%; 7.64%. Observation 1: To handle periodicity mismatch between C-DRX cycle and XR traffic, configuring multiple start offsets within one C-DRX cycle is effective and simple. Observation 2: The proposed enhanced C-DRX, i.e., configuring multiple start offsets within one C-DRX cycle, can obtain 7.64% and 2.07% power saving gain compared with Always On baseline and legacy C-DRX scheme, respectively, with marginal loss in DL UE satisfied rate. Proposal 1: To handle periodicity mismatch between C-DRX cycle and XR traffic, - Only semi-static solutions are considered - RAN1 recommends the following semi-static solution to RAN2 - Configuring multiple start offsets within one C-DRX cycle. |
| vivo | Observation 1: DCI-based approach as solution to non-integer periodicity issue will introduce more control signalling overhead and spec impact, compared to semi-static based solution. Proposal 1: Recommend semi-static based DRX enhancements to accommodate the non-integer XR traffic periodicity for R18 XR WI. Figure 1. Three DRX configuration approaches to solve the mismatch issue Observation 2: The following semi-static solutions to align DRX cycle with XR traffic periodicity is assumed. - Approach 1: Configure DRX cycle set/pattern and each DRX cycle set contains multiple DRX cycles e.g., {16ms, 17ms, 17ms}. And apply the DRX cycle set cyclically in the time domain. - Approach 2: Multiple DRX configurations with different drx-StartOffset values. - Approach 3: Single DRX configuration and one DRX cycle can contain multiple DRX ondurations. Proposal 2: Capture the following simulation results of semi-static C-DRX enhancement for non-integer periodicity scheme in TR 38.835. Table 1. Results for FR1, MU-MIMO, Indoor Hotspot, DL video, VR (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs. Always On; -; -; -; 5; 10; 100%; -. R15/16 DRX; 16; 14; 4; 5; 10; 100%; 3.67%. 10; 8; 4; 5; 10; 100%; 5.72%. 4; 3; 1; 5; 10; 100%; 4.63%. 16; 8; 4; 5; 10; 11.67%; 19.71%. 10; 5; 2; 5; 10; 78.33%; 15.41%. 10; 4; 2; 5; 10; 52.22%; 22.17%. Enhanced DRX; 16.67; 8; 4; 5; 10; 100%; 13.05%. Always On; -; -; -; 10; 10; 92.50%; -. R15/16 DRX; 16; 14; 4; 10; 10; 91.81%; 3.46%. 10; 8; 4; 10; 10; 91.25%; 5.10%. 4; 3; 1; 10; 10; 91.68%; 4.03%. 16; 8; 4; 10; 10; 2.78%; 18.21%. 10; 5; 2; 10; 10; 45.00%; 13.10%. 10; 4; 2; 10; 10; 22.50%; 18.70%. Enhanced DRX; 16.67; 8; 4; 10; 10; 91.94%; 10.08%. |
| CATT | Proposal 8: DRX enhancement for XR service should not affect other data services. Table 4: Evaluation results of non-uniform DRX cycles of {17ms, 17ms, 16ms} compared to always-on Evaluation Schemes; #satisfied UEs per cell; % of satisfied UEs; Power Saving Gain (PSG); % of Capacity gain. Baseline: DG scheduling and UE always-on; 11.5; 95.8%; 0.0%; 0.0%. Enhanced C-DRX with (17/17/16, 8, 4); 0; 0.0%; 25.5%; -100%. Enhanced C-DRX with (17/17/16, 10, 4); 2.6; 21.7%; 14.8%; -77.4%. Enhanced C-DRX with (17/17/16, 12, 4); 10.1; 84.2%; 9.3%; -12.2%. Table 5: Evaluation results of non-uniform DRX cycles of {17ms, 17ms, 16ms} compared to C-DRX（16, 12, 4） Evaluation Schemes; #satisfied UEs per cell; % of satisfied UEs; Power Saving Gain (PSG); % of Capacity gain. Baseline: DG scheduling with C-DRX(16, 12, 4); 10.8; 90.0%; 0.0%; 0.0%. Enhanced C-DRX with (17/17/16, 8, 4); 0; 0.0%; 19.0%; -100%. Enhanced C-DRX with (17/17/16, 10, 4); 2.6; 21.7%; 7.4%; -75.9%. Enhanced C-DRX with (17/17/16, 12, 4); 10.1; 84.2%; 1.4%; -6.5%. Proposal 9：From the above evaluation results and analysis, it can be obtained the following conclusion: - The semi-static CDRX enhancement can obtain up to 25.5% PSG, but with 100% capacity loss compared with both always-on and C-DRX (16, 12, 4). The pre-configured non-uniform C-DRX does not handle the excessive delay and delay jitter caused by network transport of XR packets belong to different UE arrived in the same time. - The power saving gain achieved by semi-static C-DRX enhancement is less than 1.5%, but with 6.5% capacity loss compared with those of C-DRX (16, 12, 4). Thus, the UE power saving from semi-static C-DRX enhancement without other power saving schemes is negligible. - The dynamic XR-dedicated PDCCH monitoring scheme customized the PDCCH monitoring control matching XR traffic generation and disassociate the PDCCH monitoring control by C-DRX is the feasible solution for C-DRX enhancement and does not have impact to the delay insensitive traffic arrival compared to the pre-configured non-uniform DRX cycles. |
| Intel | Proposal 1: RAN1 deprioritizes dynamic adjustment of CDRX for alignment with XR traffic periodicity. Table 1: Power consumption and capacity performance of enhanced DRX using semi-static CDRX alignment and legacy C-DRX. Results are presented for VR and CG in Dense Urban scenario, 4 UEs, DL only with SU-MIMO scheduler. DRX (Cycle, IAT, ON). Power Saving Scheme; Jitter ON; Jitter OFF. Power consumption (PC); #satisfied UEs per cell/ #UEs per cell PDB 10ms; #satisfied UEs per cell/ #UEs per cell PDB 15ms; Power consumption (PC); #satisfied UEs per cell/ #UEs per cell PDB 10ms; #satisfied UEs per cell/ #UEs per cell PDB 15ms. Baseline; Optional; Baseline; Optional. Mean PC; PC of 5%-tile UE in PC CDF; PC of 50%-tile UE in PC CDF; PC of 95%-tile UE in PC CDF; Mean PC; PC of 5%-tile UE in PC CDF; PC of 50%-tile UE in PC CDF; PC of 95%-tile UE in PC CDF. Always On; 116.76; 108.24; 116.13; 127.31; 3.93/4; 3.95/4; 115.19; 107.43; 114.42; 126.9; 3.97/4; 3.99/4. Legacy CDRX; DRX (8,6,6); 111.36; 98.98; 109.41; 127.46; 3.72/4; 3.81/4; 109.67; 98.19; 107.4; 126.03; 3.80/4; 3.89/4. DRX (16,4,14); 112.99; 102.69; 111.63; 126.18; 3.82/4; 3.89/4; 111.58; 101.94; 109.78; 125.95; 3.84/4; 3.93/4. w/ eDRX; DRX (8,6,6); 108.36; 98.04; 107.36; 121.26; 3.83/4; 3.9/4; 103.05; 93.34; 102.26; 116.97; 3.97/4; 3.99/4. DRX (16,4,14); 111.43; 102.32; 110.63; 123.13; 3.89/4; 3.93/4; 105.55; 97.76; 104.77; 117.26; 3.97/4; 3.99/4. Proposal 2: RAN1 studies multiple active DRX configurations to support XR traffic with multiple flows. |
| OPPO | Figure 1: Mismatch between DL video @60fps and 16ms DRX cycle with 2ms on duration timer Figure 2: Two DRX cycles are applied in a semi-static pattern {17ms, 17ms, 16ms} Proposal 1: The following DRX enhancement can be further studied to solve periodicity mismatch issue: - To generate a DRX cycle pattern with two inter-DRX interval lengths, either by derivation from configured parameters or by configuration directly. Proposal 2: The two-cycle DRX pattern is configured to UE via {TP(traffic periodicity), tref (reference time in integer multiple of slot), e (traffic arrival offset in relative to referenced time)}. The m-th DRX-ON duration for m≥0 has the semi-static start at the ns-th slot in SFN-th radio frame in the hyper-frame that contains the starting instance of the m-th DRX-ON duration, where SFN and ns satisfy SFN∙10∙2μ+ns=tref+m∙TP+e∙2μ mod 10240∙2μ Table 2: Power saving results in FR1 DL Dense Urban with 60fps and 30Mbps traffic model Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Another ODT (ms); #UE /cell= floor (Capacity); satisfied UE rate; Mean PSG of all UEs (%). Always On; -; -; -; -; 5; 90.18%; /. R15/16 CDRX; 16; 14; 2; -; 5; 88.77%; 5.11%. Non-uniform CDRX cycle pattern; {17,17,16}; 10; 2; -; 5; 88.77%; 18.72%. Non-uniform CDRX cycle pattern with dynamic additional ODT; {17,17,16}; 4; 2; 4; 5; 90.18%; 30.86%. Observation 2: In FR1 Dense Urban, AR/VR 30Mbps and 60fps, the following is observed: - For R15/16 C-DRX, the power saving gain can be 5.11% with loss of satisfied UE rate less than 1%; - For non-uniform CDRX cycle pattern, the power saving gain is 18.72% with loss of satisfied UE rate less than 1%; - For Non-uniform CDRX cycle pattern with dynamic additional on duration timer, the power saving gain is 30.86% with similar satisfied UE rate. |
| ZTE | The performance evaluations of these solutions were provided in Table 1~Table 3 in [3]. The scenarios include indoor hotspot scenario, and dense urban. And results of both joint UL&DL and DL only are provided. Basically, for different cases, CDRX alignment schemes achieve power saving gain as follows. - For FR1, Indoor Hotspot, [-4,4] ms jitter range, VR30Mbps - For DL+UL, the CDRX alignment schemes have 23.85%~24% power saving gain. - For DL only, the CDRX alignment schemes have 33.8%~34% power saving gain. - For FR1, Dense Urban, [-4,4] ms jitter range, VR45Mbps, DL only, the CDRX alignment schemes have 16%~23% power saving gain. Proposal 2: Capture the evaluation results of solutions of matching periodicities between XR traffic and CDRX into TR 38.835. Table 1 Comparison among semi-static RRC configuration approaches. 1)Non-uniform CDRX cycle pattern; 2)Uniform non-integer CDRX cycle; 3)Multiple CDRX configurations. Performance; Similar power saving gain with acceptable capacity loss.. New CDRX parameters/ RRC signaling; Low/medium RRC signaling overhead, with new parameters, Form flexible DRX cycle pattern, e.g., (17,17,16), (16,16,18) etc.; a set of CDRX parameters are required with new parameters Form uniform DRX cycle pattern, e.g., (17,17,16); Multiple sets of CDRX parameters are required with new parameters. Specification impact; Complex switching among multiple sub-cycles; Modified the formula. The cycle in the formula can be non-integer, e.g., 50/3 ms; Complex management of multiple sets of timers Support multiple CDRX in one serving cell. Proposal 3: RRC-based mechanism e.g., uniform non-integer CDRX cycle, multiple CDRX configurations, is recommended due to small specification impact, small complexity. Table A1 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL + pose/control, jitter = [-4,4]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all UEs (%). Baseline; -; -; -; 11; 11; 93.18%; -. R15 CDRX; 10; 8; 4; 11; 11; 90.15%; 6.65%. 16; 10; 5; 11; 11; 81.82%; 13.9%. Non-uniform CDRX cycle; (17,17,16); 6; 4; 11; 11; 90.15%; 23.86%. Uniform non-integer CDRX cycle; (1000/60); 6; 4; 11; 11; 90.15%; 23.85%. Enhanced multiple CDRX (3 CDRX configurations); 50ms DRX cycle; 6; 4; 11; 11; 90.11%; 24%. Table A2 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL only, jitter = [-4,4]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all UEs (%). Baseline; -; -; -; 11; 11; 93.18%; -. R15 CDRX; 10; 8; 4; 11; 11; 90.15%; 7%. 16; 10; 5; 11; 11; 81.82%; 18.47%. Non-uniform CDRX cycle; (17,17,16); 6; 4; 11; 11; 90.15%; 33.9%. Uniform non-integer CDRX cycle; (1000/60); 6; 4; 11; 11; 90.18%; 33.8%. Enhanced multiple CDRX (3 CDRX configurations); 50ms DRX cycle; 6; 4; 11; 11; 90.1%; 34%. Table A3 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only) Traffic; Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all Ues (%). VR45M Jitter = [-4,4]ms; Always On; -; -; -; 7; 7; 91.16%. Aligned CDRX with XR traffic; Aligned every 50ms; 8; 5; 7; 7; 89.8%; 23%. Aligned CDRX with XR traffic; Aligned every 50ms; 10; 5; 7; 7; 90.48%; 16%. Flexible additional active time + aligned CDRX; Aligned every 50ms; 2; 5; 7; 7; 91.84%; 26.24%. Flexible additional active time + aligned CDRX + PDCCH skipping(duration = 2ms); Aligned every 50ms; 2; 5; 7; 7; 91.16%; 33.5%. WUS(start at the beginning of jitter range, WUS is monitored every 1 slot); Aligned every 50ms; 5; 5; 7; 7; 90.48%; 24.87%. PDCCH Skipping (duration = 2ms,4ms); Aligned every 50ms; 10; 5; 7; 7; 84%; 24%. VR45M Jitter = [-8,8]ms; Aligned CDRX with XR traffic; Aligned every 50ms; 14; 5; 7; 7; 87%; 4.86%. Flexible additional active time + aligned CDRX; Aligned every 50ms; 2; 5; 7; 7; 90.48%; 20.53%. WUS(start at the beginning of jitter range, WUS is monitored every 1 slot); Aligned every 50ms; 5; 5; 7; 7; 86%; 20.84%. PDCCH skipping (duration = 2ms,4ms); Aligned every 50ms; 14; 5; 7; 7; 82%; 9.2%. |
| MTK | Proposal 4: RAN1 to capture the evaluation results (including the excel sheet) of CDRX enhancement into the TR and leave remaining works to RAN2, since the spec impact of CDRX enhancement mainly falls in RAN2. Observation 11: It is not possible to align DRX on-duration occasions with XR traffic using legacy DRX cycles with integer values. This can cause a drop in UE satisfaction rate as shown in Table 1 below. Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); % of DL satisfied UE; Mean PSG of all UEs (%). AlwaysOn - baseline; 0; 0; 0; 12; 12; 94.6%; 0%. R17 CDRX; 16; 12; 8; 12; 12; 92.7%; 5.4%. eCDRX (rational DRX cycle); (50/3); 12; 8; 12; 12; 94.3%; 9.9%. Table 1 SLS evaluation results with eC-DRX, FR1, DL-only, DU, CG 30Mbps Observation 12: As shown by SLS results in Figure 9, eC-DRX using rational DRC cycle value matching CG traffic improves both power savings and UE satisfaction rate compared to Rel-17 C-DRX (PSG improved by 4.5%, UE satisfaction rate improved by 1.6%). Proposal 5: Introduce non-integer (rational number) DRX cycles to match typical XR traffic patterns. |

### 2.1.1 Discussion 1

In the past meetings, the semi-static CDRX periodicity alignment was prioritized.

RAN1 #110 Agreement RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity RAN1 #110bis-e Agreement - For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1) - Prioritize semi-static solutions - FFS: Whether dynamic solutions will be also needed

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL + UL joint evaluation, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Ericsson that

  - Semi-static alignment provides

    - mean power saving gain of 7.4% in the range of 7.2% to 7.6% for all UEs

    - mean capacity gain of -4.6% in the range of -5.1% to -4.1%%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 4.35% in the range of 2.6% to 6.1% for all UEs

    - mean capacity gain of -7.30% in the range of -11% to -3.6%

- For FR1, DL + UL joint evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - Semi-static alignment provides

    - mean power saving gain of 23.8% in the range of 24% to 23.90% for all UEs

    - mean capacity gain of -3.33% in the range of -3.33% to -3.33%

  - R15 CDRX as the performance reference provides

    - mean power saving gain of 10.28% in the range of 6.65% to 13.9% for all UEs

    - mean capacity gain of -7.75% in the range of -12.2% to -3.33%

- For FR1, DL only evaluation, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from OPPO, Huawei, Qualcomm, Ericsson and Intel that

  - semi-static alignment provides

    - mean power saving gain of 8.36% in the range of 3.92% to 18.72% for all UEs

    - mean capacity gain of -2.05% in the range of -5.10% to 0.80%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 5.68% in the range of 3.23% to 10.4% for all UEs

    - mean capacity gain of -7.61% in the range of -14.70% to -1.60%

- For FR1, DL only evaluation, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Qualcomm that

  - semi-static alignment + R17 PDCCH monitoring adaptation provides

    - mean power saving gain of 27.28% in the range of 24.5% to 30.1% for all UEs

    - mean capacity gain of -4.00% in the range of -7.60% to -2.00%

  - R15/16 CDRX + R17 PDCCH monitoring adaptation as the performance reference provides

    - mean power saving gain of 28.1% in the range of 25.7% to 30.6% for all UEs

    - mean capacity gain of -67.68% in the range of -84.80% to -49.70%

- For FR1, DL only evaluation, DU, low load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Ericsson that

  - semi-static alignment provides

    - mean power saving gain of 12.65% in the range of 12.50% to 12.80% for all UEs

    - capacity gain of 0%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 8.60% in the range of 4.80% to 12.40% for all UEs

    - capacity gain of 0%

- For FR1, DL only evaluation, DU, high load, jitter off, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Intel that

  - semi-static alignment provides

    - mean power saving gain of 9.46% in the range of 8.37% to 10.54% for all UEs

    - capacity gain of 0.0%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 3.96% in the range of 3.13% to 4.79% for all UEs

    - mean capacity gain of -3.8% in the range of -3.3% to -4.3%

- For FR1, DL only evaluation, DU, high load, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from MediaTek and Intel that

  - semi-static alignment provides

    - mean power saving gain of 7.22% in the range of 4.56% to 9.90% for all UEs

    - mean capacity gain of -0.70% in the range of -1.30% to -0.3%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 4.42% in the range of 3.23% to 5.40% for all UEs

    - mean capacity gain of -2.33% in the range of -3.5% to -1.5%

- For FR1, DL only evaluation, DU, high load, jitter off, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from Intel that

  - semi-static alignment provides

    - mean power saving gain of 9.46% in the range of 8.37% to 10.54% for all UEs

    - capacity gain of 0%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 3.96% in the range of 3.13% to 4.79% for all UEs

    - mean capacity gain of -2.0% in the range of -1.5% to -2.5%

- For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE, vivo and CATT that

  - semi-static alignment provides

    - mean power saving gain of 23.05% in the range of 9.30% to 34% for all UEs with

    - mean capacity gain of -28.54% in the range of -100% to -0.60%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 10.67% in the range of 3.46% to 18.70% for all UEs with

    - mean capacity gain of -27.63% in the range of -97.00% to -0.70%

- For FR1, DL only evaluation, InH, low load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from vivo that

  - semi-static alignment provides

    - power saving gain of 13.05% for all UEs with

    - capacity gain of 0%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 11.89% in the range of 3.67% to 22.17% for all UEs with

    - mean capacity gain of -26.30% in the range of 0% to -88.3%

- For FR2, DL only evaluation, InH, high load, jitter off, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Qualcomm that

  - semi-static alignment provides

    - mean power saving gain of 8.98% in the range of 0.30% to 18.93% for all UEs

    - capacity gain of 0%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 12.53% in the range of 0.29% to 28.60% for all UEs

    - mean capacity gain of -51.10% in the range of -100% to 0%

- For FR2, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Qualcomm that

  - semi-static alignment provides

    - mean power saving gain of 11.94% in the range of 2.43% to 25.10% for all UEs

    - mean capacity gain of -26.30% in the range of -70.00% to -2.2%

  - R15/16 CDRX as the performance reference provides

    - mean power saving gain of 14.06% in the range of 4.10% to 28.44% for all UEs

    - mean capacity gain of -57.4% in the range of -100% to -27.8%

Question 1: Do you have any further comments on the observations given the agreements above?

| Company | Views |
| --- | --- |
| ZTE, Sanechips | For various scenarios, e.g.,FR1/FR2, InH/DU, VR/CG, semi-static alignment provides clear power saving gain. Perhaps we can further discuss whether or not recommend detailed alternatives of those semi-static alignment solutions which provide power saving benefits, e.g., Uniform non-integer CDRX cycle, Multiple CDRX configurations, etc. |
| OPPO2 | We are generally fine with the above observations except the following modifications, since it is observed from our Tdoc [R1-2211490] that R15/16 CDRX provides capacity gain of -1.41%. - For FR1, DL only evaluation, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from OPPO, Huawei, Qualcomm, Ericsson and Intel that - semi-static alignment provides - mean power saving gain of 8.36% in the range of 3.92% to 18.72% for all UEs - mean capacity gain of -2.05% in the range of -5.10% to 0.80% - R15/16 CDRX as the performance reference provides - mean power saving gain of 5.68% in the range of 3.23% to 10.4% for all UEs - mean capacity gain of -7.61% in the range of -14.70% to -1.60% -1.41% Sorry for my misunderstanding for the calculation, my fault!! We are fine with the above observations😊 |
| Nokia1 | Firstly, a note that our document also contained results for the semi-static periodicity alignment (indicated by Cyclic Pattern {16,17,17} in R1-2211551). It would be good if those could accounted here as well (i.e. as included below in Section 2.2.1 by FL): - For FR1, DL only evaluation, InH, high load, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from Nokia that - semi-static CDRX periodicity alignment provides - power saving of 16.00% - capacity gain of -55.6% - R15/16 CDRX provides - mean power saving gain of 13.30% for all UEs - capacity gain of -33.3% - For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Nokia that - semi-static CDRX periodicity alignment and R15/16 CDRX provides - capacity gain of -100% In general we think that it is probably good to keep the observations generic, as the detailed solution would be under RAN2 jurisdiction. |
| Intel | We think this is valuable observation and should be captured in TR. Few comments: - PDB for CG should be corrected to 15ms - Some observations mention ‘jitter OFF’, while some do not explicitly mention anything. Jitter ON and OFF assumption can be explicitly captured in all bullets |
| Xiaomi | Generally fine with the observations. But should we add another bullet to say what the semi-static solutions are? |

Dynamic periodicity alignment between CDRX and XR traffic

Nokia proposed to propose to consider auxiliary L1/L2 signalling to dynamically adapt DRX parameters in addition to semi-static solution to better align XR traffic and C-DRX operation. ZTE proposed dynamic CDRX enhancement to adjust at least the parameter of CDRX StartOffset. Qualcomm proposed dynamic adaptation of On Duration start and PDCCH skipping over remaining DRX active time within On Duration or Inactivity Timer. Evaluation results are provided in the Table 2.

China Telecom, Sony, Lenovo, Panasonic, Rakuten Symphony, Xiaomi, CMCC, ETRI, LGE, InterDigital, OPPO, and DOCOMO also proposed dynamic adjustment of CDRX parameters (start offset, CDRX On Duration and/or CDRX cycle).

Table 2: Evaluation results for dynamically aligning CDRX with XR traffic periodicity

| Company | Evaluation results |
| --- | --- |
| Nokia | Observation 1: Semi-static solutions cannot fully solve the XR and DRX periodicity mismatch as in the beginning of the connection is not fully known in advance and in the presence of additional sources of unpredictable delay like scheduling and queuing. Observation 2: Dynamic solutions based on L1/L2 signalling can work in combination with semi-static solutions to cope with problems highlighted in Observation 1 until the frame arrival process is learned at the beginning of the connection or updated during the XR session. Observation 3: Dynamic solutions enable additional flexibility in the reconfiguration of the DRX parameters. For example, they can control the OnDuraiton in addition to the startOffset to further reduce UE power consumption with negligible impact on XR capacity. To compare static and dynamic solutions, we randomly select the beginning of the XR connection for each UE after its connection to the cell and we compare the following power saving schemes: - ON: UEs are “always ON” (i.e., any power saving scheme disabled). This is the baseline used in [2]. - CDRX(16,8,8): baseline for R16/17 power saving features. The CDRX cycle is configured using the following parameters {DRX LongCycle, OnDuration, InactivityTime} = {16, 8, 8}. - Cyclic Pattern [LongCycle={16,17,17}, Onduration=8, InactivityTimer=8]: CDRX with cyclic pattern for LongCycle={16,17,17}. LongCycle is selected in round robin, while OnDuration and Inactivity Time are both set to 8ms like the baseline CDRX. This is the baseline for static solutions. - ADRX 1 [CDRX(16,8,8) + Dynamic L1/L2 adjustment of StartOffset]: Adaptive DRX based on dynamic L1/L2 signalling on top of CDRX(16,8,8). This scheme dynamically adapts only the StartOffset until frame arrivals are within [-4;+4]ms from the learnt center-point. To this end, we set ϑ = 4ms and S = ─4ms. All other parameters remain fixed. - ADRX 2 [CDRX(LongCycle,8,8) + LongCycle={16,17,17} + Dynamic L1/L2 adjustment of StartOffset]: Adaptive DRX based on dynamic L1/L2 signalling on top of CDRX Pattern. This scheme dynamically adapts only the StartOffset until frame arrivals are within [-4;+4]ms from the learnt center-point (i.e., we set ϑ = 4ms and S = ─4ms), while LongCycle is selected in round robin in {16,17,17} . All other parameters remain fixed. - ADRX 3 [CDRX(X*16,X*8,X*8) + Dynamic L1/L2 X={1/2,1/4,1}]: Adaptive DRX based on dynamic L1/L2 signalling on top of CDRX(16,8,8). This scheme dynamically adapts all parameters of the DRX configuration using a simple multiplier to scale down the values if frame delivery time approaches a threshold set equal to 10ms. Observation 4: In the absence of knowledge about the beginning of XR traffic arrivals (i.e., the central point of the jitter range) and high traffic load, semi-static solutions cannot be optimally configured and suffers high capacity loss. Observation 5: Dynamic signalling of StartOffset working on top of semi-static solutions like Cyclic Pattern for DRX Long Cycle helps mitigating capacity loss. Observation 6: Solutions based on L1/L2 dynamic signalling shows similar power saving gain as (semi) static solutions if only StartOffset is reconfigured. Observation 7: Solutions based on L1/L2 dynamic signalling shows higher power saving gain than (semi) static solutions if multiple DRX parameters are reconfigured. Proposal 1: For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1), we propose to consider auxiliary L1/L2 signalling to dynamically adapt DRX parameters in addition to semi-static solution to better align XR traffic and C-DRX operation. Table 1 – Evaluation of enhanced ADRX enhancement for {Indoor Hotspot, CG, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs. Power Saving Scheme; Power Saving Gain (PSG) compared to ‘Always ON’; #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS; Capacity with PS [#satisfied UEs/cell with PS]; Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS. Baseline. Mean PS gain. Always ON; -; 5.4 / 6; 6; 90%. CDRX(16,8,8); 13.3%; 3.6 / 6; 4; 60%. Cyclic Pattern {16,17,17}; 16%; 2.4 / 6; 3; 40%. ADRX 1; 15%; 3.7 / 6; 4; 62%. ADRX 2; 15%; 3.7 / 6; 4; 62%. ADRX 3; 19%; 4.5 / 6; 5; 75%. Table 2 – Evaluation of enhanced ADRX enhancement for {Indoor Hotspot, AR/VR, DL Only, 30Mbps, FR1}. Option 1: PS gain computed with All UEs Power Saving Scheme; Power Saving Gain (PSG) compared to ‘Always ON’; #satisfied UEs per cell with PS / #satisfied UEs per cell w/o PS; Capacity with PS [#satisfied UEs/cell with PS]; Percentage of satisfied UEs per cell with PS at #satisfied UEs cell w/o PS. Baseline. Mean PS gain. Always ON; -; 4.7 / 5; 5; 95%. CDRX(16,8,8); -; 0 / 5; 0; 0%. Cyclic Pattern {16,17,17}; -; 0 / 5; 0; 0%. ADRX 1; 15%; 1.25 / 5; 3; 25%. ADRX 2; 15%; 1.25 / 5; 3; 25%. ADRX 3; 19%; 3.75 / 5; 4; 70%. |
| ZTE | - Proposal 1: Support dynamic CDRX enhancement to adjust at least the parameter of CDRX StartOffset. Table 3 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL + pose/control) [R1-2207061] Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all UEs (%). Dynamic indication; 50; 6; 4; 11; 11; 90.1%; 23%. Non-uniform CDRX cycle; 17-17-16; 6; 4; 11; 11; 90.15%; 23.86%. Uniform non-integer CDRX cycle; (1000/60); 6; 4; 11; 11; 90.15%; 23.85%. Multiple CDRX (3 CDRX configurations); 50ms DRX cycle; 6; 4; 11; 11; 90.11%; 24%. Table 4 FR1 power consumption results in Indoor Hotspot scenario (VR30M, fps=60, DL only) [R1-2207061] Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all UEs (%). Dynamic indication; 50; 6; 4; 11; 11; 90.1%; 33%. Non-uniform CDRX cycle; 17-17-16; 6; 4; 11; 11; 90.15%; 33.9%. Uniform non-integer CDRX cycle; (1000/60); 6; 4; 11; 11; 90.18%; 33.8%. Multiple CDRX (3 CDRX configurations); 50ms DRX cycle; 6; 4; 11; 11; 90.1%; 34%. |
| Qualcomm | Observation 9: Using fixed CDRX parameters may have negative impact on delays (PDB) and power consumption for XR traffic. Observation 10: For FR2, DL VR 30Mbps in Indoor Hotspot environment, dynamically adapting some of the (e)CDRX parameters can achieve considerable capacity gains over (e)CDRX Proposal 4: For XR, consider studying methods to dynamically adapt the CDRX parameters to the traffic bursts, specifically: - ON duration start - Inactivity timer early termination - PDCCH skipping/SSSG switching indication using a non-scheduling DCI (a) PDDCH skipping inside ON duration only; (b) PDCCH skipping inside ON duration and for IAT. Figure 11: FR2 InH Relative Power Consumption, VR (30Mbps), with Jitter, PDB =10ms Table 4: Summary of FR2 InH Power Consumption Results for 7UE per Cell, VR (30Mbps), PDB =10ms Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); % of satisfied UE; Median PSG of all UEs (%) relative to baseline; Mean PSG of all UEs (%) relative to baseline. [note 1] Baseline: eCDRX + PDCCH skipping; 16; 4; 4; 27%; 0%; 0%. 16; 8; 8; 84%; 0%; 0%. 16; 8; 16; 88%; 0%; 0%. [note 1] Enhancement: eCDRX + PDCCH skipping + Adaptive ON Start; 16; 4; 4; 27%; 43.8%; 42.4%. 16; 8; 8; 84%; 38.6%; 38.1%. 16; 8; 16; 88%; 34.6%; 34.4%. [note 2] Baseline: eCDRX + PDCCH skipping; 16; 4; 4; 27%; 0%; 0%. 16; 8; 8; 84%; 0%; 0%. 16; 8; 16; 88%; 0%; 0%. [note 2] Enhancement: eCDRX + PDCCH skipping + Adaptive ON Start; 16; 4; 4; 27%; 54.7%; 53.3%. 16; 8; 8; 84%; 56.1%; 55.3%. 16; 8; 16; 88%; 54.1%; 53.3%. Note 1: PDCCH skipping inside ON duration only Note 2: PDCCH skipping inside ON duration and for IAT (early IAT termination). Table 5: Summary of FR2 InH Power Consumption Results for 7UE per Cell, VR (30Mbps), PDB =10ms Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); % of satisfied UE; Median PSG of all UEs (%) relative to baseline (ALWAYS ON); Mean PSG of all UEs (%) relative to baseline (ALWAYS ON). Baseline: ALWAYS ON; N/A; N/A; N/A; 90%; 0%; 0%. [note 1] eCDRX + PDCCH skipping; 16; 4; 4; 27%; 29.4%; 31.5%. 16; 8; 8; 84%; 16.0%; 17.4%. 16; 8; 16; 88%; 11.7%; 12.7%. [note 1] Enhancement: eCDRX + PDCCH skipping + Adaptive ON Start; 16; 4; 4; 27%; 60.3%; 60.5%. 16; 8; 8; 84%; 48.4%; 48.9%. 16; 8; 16; 88%; 42.3%; 42.7%. [note 2] eCDRX + PDCCH skipping; 16; 4; 4; 27%; 44.3%; 45.5%. 16; 8; 8; 84%; 42.3%; 43.1%. 16; 8; 16; 88%; 43.0%; 43.7%. [note 2] Enhancement: eCDRX + PDCCH skipping + Adaptive ON Start; 16; 4; 4; 27%; 74.8%; 74.5%. 16; 8; 8; 84%; 74.7%; 74.6%. 16; 8; 16; 88%; 73.8%; 73.7%. Note 1: PDCCH skipping inside ON duration only Note 2: PDCCH skipping inside ON duration and for IAT (early IAT termination). Proposal 5: Capture evaluation results in Table 4 and Table 5 for adaptive On Duration Start in TR 38.835. |

### 2.2.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL + UL joint evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - dynamic alignment provides

    - power saving gain of 23% for all UEs

    - satisfied UE ratio of 90.1%

  - semi-static alignment as the performance reference provides

    - mean power saving gain of 23.90% in the range of 23.85% to 24% for all UEs

    - mean satisfied UE ratio of 90.14% in the range of 90.11% to 90.15%.

- For FR1, DL only evaluation, InH, high load, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from Nokia that

  - adaptive DRX provides

    - mean power saving gain of 16.33% in the range of 15.00% to 19.00% for all UEs

    - mean capacity gain of -26.3% in the range of -31.1% to -16.7%

  - semi-static CDRX periodicity alignment provides

    - power saving of 16.00%

    - capacity gain of -55.6%

  - R15/16 CDRX provides

    - mean power saving gain of 13.30% for all UEs

    - capacity gain of -33.3%

- For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Nokia that

  - adaptive DRX provides

    - mean power saving gain of 16.33% in the range of 15.00% to 19.00% for all UEs

    - mean capacity gain of -57.90% in the range of -73.7% to -26.3%

  - semi-static CDRX periodicity alignment and R15/16 CDRX provides

    - capacity gain of -100%

- For FR1, DL joint evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - dynamic alignment provides power

    - saving gain of 33% for all UEs

    - satisfied UE ratio of 90.1%.

  - semi-static alignment as the performance reference provides

    - mean power saving gain of 33.90% in the range of 33.80% to 34% for all UEs

    - mean satisfied UE ratio of 90.14% in the range of 90.1% to 90.18%.

- For FR2, DL joint evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Qualcomm that

  - adaptative On Start (on top of semi-static alignment and PDCCH skipping) provides

    - mean power saving gain of 62.48% in the range of 42.70% and 74.60% for all UEs

    - mean capacity gain of -26.30% in the range of -70.0% to -2.2%

  - semi-static alignment and PDCCH skipping as the performance reference provides

    - mean power saving gain of 32.32% in the range of 12.70% to 45.50% for all UEs

    - mean capacity gain of -26.30% in the range of -70.0% to -2.2%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |
| ZTE, Sanechips | There’s a typo in last two bullets: DL joint -> DL only In our view, we prefer to take dynamic solution as an optional feature to ensure the robustness of CDRX for XR services. |
| Intel | Few comments: - We think some of the assumptions on jitter and packet arrival taken to motivate dynamic solutions need to be cross-checked carefully, given SA2 agreed that Traffic jitter information is to be provided to the NG-RAN at PDU Session Establishment/Modification via an NGAP Message, and it is FFS how 5GC derives the above information. In this regard, 5G-RAN is expected to have some jitter information and under steady state conditions, jitter range/center point of traffic arrival is not expected to change frequently in our view. - Moreover, significant performance gain is observed when semi-static alignment is assumed wrt Always ON and legacy C-DRX and additional performance gain observed wrt semi-static alignment due to adaptive C-DRX is marginal from Nokia’s result. Also, it is not clear why CDRX alignment performs worse or comparable to legacy C-DRX in terms of capacity performance. - It is not clear from Qualcomm’s result what is additional gain wrt semi-static alignment only. - Mean satisfied UE ratio and capacity are interchangeably used? - CG PDB should be 15ms |
| Xiaomi | Generally fine with the structure of the observation. But suggest to give clear definition of the terminologies we use, such as “adaptive DRX” “dynamic alignment” |
| OPPO | Correct our position in the summary part. |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| vivo | We don’t support dynamic periodicity alignment between CDRX and XR traffic. The reasons are as follows. - Semi-static method for periodicity alignment is sufficient because of regularity of XR traffic pattern. DCI-based CDRX adjustment has no additional benefit for solving non-integer traffic periodicity. - DCI-based approach as solution to non-integer periodicity issue will introduce more control signalling overhead and spec impact, compared to semi-static based solution. - The use case where the centre point of XR traffic arrival to RAN could be drifted and whether the core network can provide with the range and frequency of the drift to RAN RAN are not justified yet. |
| MTK | Not support. We tend to think alignment between CDRX and XR traffic can be achieved by semi-static methods. Dynamic methods can be considered if the intention is to solve jitter issue, while at the same time we recall RAN1 is not sure whether NW can have instantaneous jitter value for each traffic arrival. |
| ZTE | We support the proposal of dynamic solution, because that potentially provide more robust CDRX configuration for XR services. |

Additional active time

ZTE proposed flexible (UE autonomous) additional active time if UE does not receive data within current active time. Nokia proposed the automatic Extension of Active Time (EAT) to extend DRX active time if XR frame does not arrive before the On Duration timer expires. ZTE also proposed to define XR specific RNTI to distinguish non-XR traffic with XR specific traffic. OPPO proposed to use dynamic signaling such as a DCI to trigger additional On Duration if the data packet arrives after the On Duration expires. vivo evaluated the additional On-Duration scheme and existing R17 PDCCH monitoring adaptation scheme. It shows additional On-Duration has slightly worse performance results in terms of power saving gain and capacity. Evaluation results can be found in the Table 3. CMCC, ETRI also proposed the extension of DRX On Duration.

Ericsson proposed that UE can resume PDCCH monitoring during the time that drx-RetransmissionTimerDL is running, regardless of previous PDCCH skipping indications. vivo proposed that if there is NACK feedback during the skipping duration, UE resumes PDCCH monitoring to receive corresponding retransmission scheduling within the drx-retransmissiontimer. Additionally, network can dynamically enable or disable this mechanism for the indicated PDCCH skipping, e.g., according to the remaining PDB etc. MediaTek proposed that after the UE is indicated to skip PDCCH monitoring, if any NACK is transmitted or if UL data is transmitted, the UE resumes PDCCH monitoring for a configured duration. Evaluation results can be found in Table 3. ZTE evaluated the PDCCH monitoring resuming and observed marginal gain. Huawei, OPPO, ETRI, NEC, LGE, Samsung, III also proposed to resume PDCCH monitoring after PDCCH skipping starts if UE sends NACK.

These proposals and evaluation results were discussed in following proposals in RAN1 #110bis-e, but there was no consensus to adopt any of them.

Proposal A: Support UE autonomous extension of DRX active time for Rel-18 XR-specific power saving enhancements - Define XR specific RNTI to distinguish non-XR traffic with XR specific traffic Proposal B: Support DCI triggered additional DRX active time for Rel-18 XR-specific power saving enhancements

Table 3: Evaluation results for CDRX On Duration adaptation

| Company | Evaluation results |
| --- | --- |
| Ericsson | Figure 1 Illustration of NACK sent in the same DRX Active Time as PDCCH skipping indication. In this example, with the current specifications, the retransmission cannot be sent to the UE in the same Active Time. Figure 2 Illustration of NACK sent in a DRX Active Time subsequent to the Active Time where the PDCCH skipping indication was sent. Figure 3 Illustration of NACK sent in the same DRX Active Time as PDCCH skipping indication, after enhancement. Proposal 3 UE can resume PDCCH monitoring during the time that drx-RetransmissionTimerDL is running, regardless of previous PDCCH skipping indications. Table 2: Results for FR1, high load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 91.7%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 8; 7; 86.9%; 4.3%; 4.9%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 8; 6; 78.2%; 10.4%; 11.0%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.2%; 10.5%; 11.4%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; H; 8; 7; 87.0%; 10.1%; 10.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.3%; 15.8%; 16.0%. Table 3: Results for FR1, low load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; L; 2; 8; 100%. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; L; 2; -; 100%; 4.8%; 4.8%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; L; 2; -; 100%; 12.4%; 12.4%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 12.8%; 12.8%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; L; 2; -; 100%; 12.5%; 12.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 22.5%; 22.5%. Table 4: Results for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 90.1%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 8; 7; 86.9%; 2.6%; 2.7%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 8; 6; 80.2%; 6.1%; 6.1%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 85.5%; 7.2%; 7.1%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; H; 8; 7; 86.4%; 7.6%; 7.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 84.7%; 7.6%; 7.6%. |
| vivo | Table 5. Results for FR1, MU-MIMO, Indoor Hotspot, DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Notes. Always On; -; -; -; 5; 10; 100%; -. R17 PDCCH monitoring adaptation; 16.67; 8; 4; 5; 10; 100%; 23.36%; Note1,2. Additional On-Duration; 16.67; 4; 4; 5; 10; 100%; 18.73%; Note3. Always On; -; -; -; 10; 10; 92.50%; -. R17 PDCCH monitoring adaptation; 16.67; 8; 4; 10; 10; 92.22%; 19.28%; Note1,2. Additional On-Duration; 16.67; 4; 4; 10; 10; 91.49%; 14.68%; Note3. Note1: PDCCH skipping is indicated in the DCI that schedules a dummy PDSCH after all the HARQ-ACK processes of transmissions have been completed Note2: applying R17 sparse SSSG with PDCCH monitoring every 2 slots when DRX Onduration starts and switch to dense SSSG with PDCCH monitoring every 1 slot after detecting DCI scheduling XR traffic burst Note3: additional DRX onduration length is 4ms. Observation 8: Compared with existing R17 PDCCH monitoring adaptation scheme, additional On-Duration scheme has slightly worse performance results in terms of power saving gain and capacity. Figure 4. The causing XR traffic missing by adopting autonomous additional DRX onduration scheme Observation 9: For the case of autonomous active time extension, UE may fail to extend the active time if non-XR traffic data is scheduled for the UE before XR video data arrival, such that XR video transmission may be missed. Observation 10: DCI triggered active time extension has been already achieved by R15/16 CDRX scheme i.e., an DCI scheduling initial transmission will trigger the start/re-start of drx-inactivitytimer. Proposal 7: Capture the above simulation results and observations for additional On-Duration scheme in TR 38.835. Proposal 8: The “additional On-Duration scheme” is not recommended for Rel-18 XR WI. |
| OPPO | Figure 3 jitter issue when short DRX on duration timer is configured Observation 1: There would exist early/late packet arrival issue since gNB may configure DRX on duration timer shorter than jitter range, and late packet arrival would lead to longer scheduling latency and additional power consumption. Figure 4 Additional on duration time duration DRX off Proposal 3: An additional DRX on-duration time can be dynamically indicated to UE to solve jitter issue. Table 2: Power saving results in FR1 DL Dense Urban with 60fps and 30Mbps traffic model Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Another ODT (ms); #UE /cell= floor (Capacity); satisfied UE rate; Mean PSG of all UEs (%). Always On; -; -; -; -; 5; 90.18%; /. R15/16 CDRX; 16; 14; 2; -; 5; 88.77%; 5.11%. Non-uniform CDRX cycle pattern; {17,17,16}; 10; 2; -; 5; 88.77%; 18.72%. Non-uniform CDRX cycle pattern with dynamic additional ODT; {17,17,16}; 4; 2; 4; 5; 90.18%; 30.86%. Observation 2: In FR1 Dense Urban, AR/VR 30Mbps and 60fps, the following is observed: - For R15/16 C-DRX, the power saving gain can be 5.11% with loss of satisfied UE rate less than 1%; - For non-uniform CDRX cycle pattern, the power saving gain is 18.72% with loss of satisfied UE rate less than 1%; - For Non-uniform CDRX cycle pattern with dynamic additional on duration timer, the power saving gain is 30.86% with similar satisfied UE rate. |
| ZTE | Proposal 4: Capture solutions and evaluation results for jitter handling into TR 38.835. - Proposal 5: Support flexible additional active time based on current CDRX design, with minor specification change. Table 5 FR1 power consumption results in Indoor scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all Ues (%). Aligned CDRX with XR traffic; Aligned every 50ms; 14; 5; 7; 7; 88.10%; 4.60%. Aligned CDRX with XR traffic; Aligned every 50ms; 12; 5; 7; 7; 80%; 9.46%. Flexible additional active time + aligned CDRX; Aligned every 50ms; 2; 4; 7; 7; 89.48%; 19%. PDCCH Skipping (duration = 2ms,4ms); Aligned every 50ms; 14; 5; 7; 7; 72.62%; 10.84%. PDCCH skipping with dummy grant (duration = 4ms,8ms); Aligned every 50ms; 14; 5; 7; 7; 80.95%; 13.03%. Table 6 FR1 power consumption results in Dense Urban scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all Ues (%). Aligned CDRX with XR traffic; Aligned every 50ms; 14; 5; 7; 7; 85%; 4.50%. Flexible additional active time + aligned CDRX; Aligned every 50ms; 2; 4; 7; 7; 84%; 18%. PDCCH Skipping (duration = 2ms,4ms); Aligned every 50ms; 14; 5; 7; 7; 74%; 9.6%. PDCCH skipping with dummy grant (duration = 4ms,8ms); Aligned every 50ms; 14; 5; 7; 7; 80%; 12%. - Observation 2: For FR1, Indoor, [-8,8]ms jitter range, VR45M, DL only, - Flexible additional active time has 19% PSG with 89.48% of satisfied UE - PDCCH skipping with dummy grant has 13.03% PSG with 80.95% of satisfied UE - PDCCH skipping indicated in last transmission slot has 10.84% PSG with 72.62% of satisfied UE - Observation 3: For FR1, Dense Urban, [-8,8]ms jitter range, VR45M DL only, - Flexible additional active time has 18% PSG with 84% of satisfied UE - PDCCH skipping with dummy grant has 12% PSG with 80% of satisfied UE - PDCCH skipping indicated in last transmission slot has 9.6% PSG with 74% of satisfied UE Observation 4: For evaluation of power saving techniques, the limitations of legacy PDCCH skipping should be accounted. - PDCCH skipping is indicated in a scheduling DCI - The PDCCH skipping duration may delay potential retransmission scheduling - PDCCH skipping with dummy grant costs additional power consumption due to decoding PDSCH and transmitting HARQ-ACK to the dummy grant - Proposal 6: There is no need to enhance Rel-17 PDCCH skipping for re-transmission. |
| Nokia [R1-2209535] | Figure 7 – Active time extension applied when packet arrival is after On Duration. We assumed that 1 packet is expected to arrive per DRX cycle. Observation 6: If the network can configure the UE with the number of expected frames to be received and/or transmitted during a CDRX cycle, then the UE can automatically extend the active time if the number of received and/or transmitted frames are smaller than the expected values. Observation 7: Active time extension adds the flexibility required to configure CDRX with the best trade-off between user satisfaction and power saving for XR traffic, provided that network and UE knows the number of expected frames to be received and/or transmitted during a CDRX cycle. (a) CDRX w/o Extension of Active Time; (b) CDRX w/ Extension of Active Time.. Figure 9 – Capacity of CDRX configurations w/ and w/o Extension of Active Time (EAT) for CG traffic (Video Single-Stream) in Indoor Hotspot deployment (FR1) with X=99% of frames received within PDB and cells evenly loaded. Figure 10 – Power saving gain of CDRX configurations w/ Extension of Active Time (EAT) in Indoor Hotspot deployment (FR1) with X=99% of frames received within PDB and cells evenly loaded. Power saving gain shall be ignored when there is no user that is satisfied (i.e., XR capacity is zero). Observation 8: Automatic Extension of Active Time improves the power saving gain with respect to CDRX since it allows to define a relatively short onDuration with respect to the jitter range that is extended automatically only when a frame(s) do not arrive before the onDuration timer is expired. Observation 9: Given the frame arrival distribution of XR traffic, Active time needs to be extended very seldomly beyond the (short) onDuration timer. Proposal 4: We propose to consider automatic extension of UE Active Time of CDRX based on the frame arrival to enable enhanced power saving. |

### 2.3.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made for additional DRX active time

Observation

- For FR1, DL and joint evaluation, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - on top of eCDRX, additional active time provides

    - power saving gain of 14.18% for all UEs

    - satisfied UE ratio of 90%.

  - eCDRX as the performance reference provides

    - power saving gain of 7.5% for all UEs

    - satisfied UE ratio of 88.36%.

- For FR1, DL and UL joint evaluation, InH, high load, VR 45Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - on top of eCDRX, additional active time provides

    - power saving gain of 15.16% for all UEs

    - satisfied UE ratio of 88.1%.

  - eCDRX as the performance reference provides

    - power saving gain of 7.5% for all UEs

    - satisfied UE ratio of 90%.

- For FR1, DL and joint evaluation, InH, high load, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - on top of eCDRX, additional active time provides

    - power saving gain of 21.3% for all UEs

    - satisfied UE ratio of 88.19%.

  - eCDRX as the performance reference provides

    - power saving gain of 21.2% for all UEs

    - satisfied UE ratio of 84%.

- For FR1, DL only, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - with eCDRX, additional active time provides

    - power saving gain of 30% for all UEs

    - satisfied UE ratio of 90%.

  - eCDRX as the performance reference provides power

    - saving gain of 10.4% for all UEs

    - satisfied UE ratio of 88.36%.

- For FR1, DL only, InH, low load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from vivo that

  - on top of eCDRX, additional active time provides

    - power saving gain of 18.73% for all UEs

    - capacity gain of 0%

  - eCDRX with R17 PDCCH monitoring adaptation as the performance reference provides

    - power saving gain of 23.36% for all UEs

    - capacity gain of 0%

- For FR1, DL only, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from vivo that

  - with eCDRX, additional active time provides

    - power saving gain of 14.68% for all UEs

    - capacity gain of -1.1%

  - eCDRX with R17 PDCCH monitoring adaptation as the performance reference provides

    - power saving gain of 19.28% for all UEs

    - capacity gain -0.3%

- For FR1, DL only, InH, high load, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - on top of eCDRX, additional active time provides

    - power saving gain of 32.4% for all UEs

    - satisfied UE ratio of 88.19%.

  - eCDRX as the performance reference provides

    - power saving gain of 20.6% for all UEs

    - satisfied UE ratio of 88.89%.

- For FR1, DL only, InH, high load, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from Nokia that

  - Extension of active time provides

    - mean power saving gain of 30.40% for all UEs in the range of 30.33% to 30.43%

    - mean capacity gain of -43.10% for all UEs in the range of -42.44% to -42.44%

- For FR1, DL only, InH, high load, VR 45Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - on top of eCDRX, additional active time provides mean

    - power saving gain of 19.5% in the range of 19% to 20% for all UEs

    - mean satisfied UE ratio of 88.79% in the range of 88.1% to 89.48%.

  - eCDRX performance reference provides

    - mean power saving gain of 7.51% in the range of 4.60% to 9.46% for all UEs

    - mean satisfied UE ratio of 86.03% in the range of 80% to 90%.

  - eCDRX + PDCCH skipping performance reference provides

    - mean power saving gain of 11.94% in the range of 10.84% to 13.03% for all UEs

    - mean satisfied UE ratio of 76.79% in the range of 72.62% to 80.95%.

- For FR1, DL only, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from OPPO that

  - on top of eCDRX, additional active time provides

    - power saving gain of 30.86% for all UEs

    - capacity gain of 0.0%

  - eCDRX as the performance reference provides

    - power saving gain of 18.72% for all UEs

    - capacity gain of -1.6%

- For FR1, DL only, InH, high load, VR 45Mbps traffic at 60fps and 10ms PDB, it is observed from ZTE that

  - with eCDRX, additional active time provides

    - mean power saving gain of 24.57% in the range of 18% to 33.50% for all UEs

    - mean satisfied UE ratio of 89.37% in the range of 84% to 91.84%

  - eCDRX performance reference provides

    - mean power saving gain of 12.09% in the range of 4.50% to 23% for all UEs

    - mean satisfied UE ratio of 88.07% in the range of 85% to 90.48%

  - eCDRX + PDCCH skipping performance reference provides

    - mean power saving gain of 13.70% in the range of 9.20% to 24% for all UEs

    - mean satisfied UE ratio of 80.00% in the range of 74% to 84%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |
| ZTE, Sanechips | Some results show clear power saving gain; for following cases, we have some concerns: In case of the first bullet, basically no power saving gain is observed for PDCCH skipping enhancement with eCDRX. In the fifth bullet, the results of case ‘FR1, DL only evaluation, DU, high load, DRX configured, VR 30Mbps’ shows huge capacity loss, which would affect the UE experience seriously. As FL pointed out, in some cases, the marginal power saving gain is observed for the PDCCH skipping enhancement. |
| OPPO2 | We are generally fine with the above observations except the following modifications, since it is observed from our Tdoc [R1-2211490] that eCDRX provides capacity gain of -1.4%. - For FR1, DL only, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from OPPO that - on top of eCDRX, additional active time provides - power saving gain of 30.86% for all UEs - capacity gain of 0.0% - eCDRX as the performance reference provides - power saving gain of 18.72% for all UEs - capacity gain of -1.6% -1.4% Sorry for my misunderstanding for the calculation, my fault!! We are fine with the above observations😊 |
| MTK | We think this section belongs to PDCCH monitoring adaptation and should be moved to Section 3. Schemes like retransmission-aware PDCCH skipping can work with or without CDRX. |
| Nokia1 | Like noted in last meeting, it is not fully clear to us why these two schemes are bundled together as alternatives. One group of solutions aims to facilitate the C-DRX configuration by having more ‘dynamicity’ in the OnDuration/Active time determination. One solution seems to target to separate XR-traffic and non-XR traffic via separate monitoring patterns (and don’t this this belongs to under A). Other set of solutions focuses on handling of re-transmission window on conjunction with PDCCH skipping. We don’t really see that these solutions/proposals are competing candidates. The the onDuration extension related schemes would be more associated with Section 2.6 schemes |
| LGE | We are not sure UE’s resuming PDCCH monitoring should be considered under PDCCH monitoring avoidance after XR data reception. It is more appropriate to be discussed under PDCCH monitoring adaptation related enhancements. |
| Intel | Agree with MTK that second observation should be discussed under PDCCH monitoring adaptation, not C-DRX enhancement. Satisfied UE ratio and capacity gain are used interchangeably. CG PDB should be 15ms. |
| MTK2 | At the same time, we think the observation from ZTE that “PDCCH monitoring resuming has marginal gain” is not a fair observation. Looking at ZTE’s tdoc (R1-2211905) Table 8 as copied below, it can be seen that: - “PDCCH Skipping with retransmission” can provide similar power saving gain (4%~5%) over existing mechanism (PDCCH skipping dummy grant) as “aligning CDRX with XR traffic” (4.5%), in such a harsh scenario (VR 45M, jitter=[-8,8]ms). Besides, the satisfaction rate loss of “PDCCH Skipping with retransmission” is only 1%, which is even smaller than “aligned CDRX with XR traffic” (>5%, 5=90-85) Hence, we think retransmission-aware PDCCH skipping achieves more power saving benefits than “eCDRX with XR traffic alignment” and should also be considered in R18 WI. Table 8 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-8,8]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all Ues (%). Aligned CDRX with XR traffic; Aligned every 50ms; 14; 5; 7; 7; 85%; 4.50%. PDCCH Skipping (duration = 2ms,4ms); Aligned every 50ms; 14; 5; 7; 7; 74%; 9.6%. PDCCH skipping with dummy grant (duration = 4ms,8ms); Aligned every 50ms; 14; 5; 7; 7; 80%; 12%. PDCCH Skipping with retransmission (duration = 3ms,7ms); Aligned every 50ms; 14; 5; 7; 7; 84%; 16%. |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| vivo | We don’t support additional active time. The reasons are as follows. - There is technical concern for the case of autonomous active time extension. UE may fail to extend the active time if non-XR traffic data is scheduled for the UE before XR video data arrival, such that XR video transmission may be missed. - For DCI triggered active time extension, it has been already achieved by R15/16 CDRX scheme i.e., an DCI scheduling initial transmission will trigger the start/re-start of drx-inactivitytimer. - From our simulation results, compared with existing R17 PDCCH monitoring adaptation scheme, additional On-Duration scheme has no performance gains in terms of power saving gain and capacity. |
| MTK | Not support. The reason is we tend to think 2.6 “Early stopping of On-DurationTimer” is a better solution than this one. The signaling overhead of 2.3 “Additional active time” may be larger if NW has to indicate UE to extend active time multiple times due to the late XR packet arrival. Having said this, we think the selection between 2.3 v.s. 2.6 should be determined by RAN2 since OnDuration, ActiveTime, …, are all RAN2 designs. RAN1 can just record the simulation results into the TR. |
| LGE | Support. It helps a UE to keep DRX active time as short as possible with marginal capacity loss, so results in good power saving gain. |
| ZTE, Sanechips | We support proposals of additional active time. These solutions provide a simple way to address jitter impact (Note the aligned CDRX has poor Power saving gain if not handling jitter), and in case of PDCCH skipping is not supported by UEs, the CDRX based solutions become more practical. In the meanwhile, in RAN 2, multiple companies support this kind of solution from RAN 2 expertise perspective. We may receive some conclusions regarding section 2.1, 2.2, 2.3 from RAN2 on Tuesday. |
| OPPO | Support. As we explained in our contribution, we do not think this scheme can be achieved by dummy grant triggering the start/re-start of drx-inactivitytimer since each dummy grant can only extend the DRX active time equal to the value of IAT. If gNB plans to schedule the packet long after the end of ODT, the mechanism exploiting the current network would need several DCIs, which would surely bring larger DCI overhead than the proposed solution. |

Non-uniform PMOs within CDRX On Duration

Huawei proposed to configure non-uniform PDCCH monitoring occasions within C-DRX On Duration. UE monitors PDCCH more frequently around the centre of jitter distribution (i.e., expected data arrival time) and less frequently at the tails of jitter distribution. Evaluation results can be found in Table 4.

Table 4: Evaluation results for non-uniform PMOs within CDRX On Duration

| Company | Evaluation results |
| --- | --- |
| Huawei | Figure 1 Illustration of non-uniform PDCCH monitoring during C-DRX On Duration, where UE only monitors PDCCH in the colored occasions within On Duration Table 3 Simulation results of non-uniform PDCCH monitoring occasions within C-DRX On Duration (Dense Urban, DL VR/AR@30Mbps, High Load Case, 11 UEs/cell) Case; Monitoring pattern within C-DRX On Duration (‘1’ denotes monitoring, ‘0’ denotes sleeping); Satisfied UE Ratio; Power Saving Gain compared with “Always On”. 1; Always On; 93.42%; -. 2; Legacy C-DRX; 83.20%; 5.57%. 3; [1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1, 1]; 91.43%; 7.64%. 4; [0, 0, 0, 0, 1, 1, 1, 1, 0, 0, 0, 0, 1, 1, 1, 1, 0, 0, 0, 0, 1, 1, 1, 1]; 85.89%; 15.81%. 5; [0, 0, 0, 0, 1, 0, 1, 0, 0, 1, 1, 1, 1, 1, 0, 1, 1, 0, 1, 0, 1, 0, 0, 1]; 89.70%; 16.23%. 6; [0, 0, 0, 0, 0, 0, 1, 0, 0, 0, 1, 1, 1, 0, 1, 0, 1, 0, 0, 1, 0, 0, 0, 0]; 85.71%; 20.75%. Case 1: Always On as in R17 XR Case 2: Legacy C-DRX scheme with DRX cycle 16ms, On Duration Timer 12ms, and Inactivity Timer 4ms Case 3: Enhanced C-DRX where the periodicity mismatch issue of C-DRX is solved Case 4: Rel-17 PDCCH monitoring adaptations, e.g., SSSG switching. In sparse SSSG, the search space periodicity is 8 slots and duration is 4 slots. In dense SSSG, the UE monitors PDCCH in every slot. Case 5, 6: Proposed scheme, i.e., non-uniform PDCCH monitoring occasions within C-DRX On Duration Note: Case 3-6 assumes the periodicity mismatch issue of C-DRX is already solved. For all C-DRX schemes, ODT=12ms, IAT=4ms.. Observation 3: With the similar power saving performance, non-uniform PDCCH monitoring within C-DRX On Duration can obtain much higher capacity (i.e., satisfied UE ratio) compared with Rel-17 SSSG switching. Observation 4: With the similar capacity performance (i.e., satisfied UE ratio), non-uniform PDCCH monitoring within C-DRX On Duration can obtain much higher power saving gain compared with Rel-17 SSSG switching. Observation 5: The proposed schemes, i.e., non-uniform PDCCH monitoring within C-DRX On Duration, can obtain ~20%, ~13%, ~5% power saving gain compared with Always-On scheme, and enhanced C-DRX, and R17 SSSG switching, respectively. Observation 6: To handle jitter issue, non-uniform PDCCH monitoring within C-DRX On Duration, can achieve better trade-off between power saving and capacity performance compared with Rel-17 SSSG switching. Proposal 2: To handle jitter issue, support that gNB can configure PDCCH monitoring occasions flexibly (e.g., through bitmap) within C-DRX On Duration. |

### 2.4.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL joint evaluation, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Huawei that

  - non-uniform PMOs within On Duration provides

    - mean power saving gain of 18.49% in the range of 16.23% and 20.75% for all UEs

    - mean capacity gain of -6.15% in the range of -8.3% to -4.0%

  - sparse PDCCH monitoring as the performance reference provides

    - mean power saving gain of 11.73% in the range of 7.64% to 15.81% for all UEs

    - mean capacity gain of -5.10% in the range of -8.1% to -2.1%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company |  |
| --- | --- |
| Nokia1 | A question for clarification; case 4 in afore results seems to be labelled as SSSG switching (but not in the summary observation), but seems to be modelled as static PDCCH monitoring pattern. Hence is it correct understanding that this is actually fixed PDCCH monitoring pattern, or whether there is actually some dynamic adaptation of applied SSSG based on the e.g. traffic arrival. If this is not the case, then we would agree with FL wording that this is not SSSG switching but sparce PDCCH monitoring. |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| MTK | Support. The proposal seems to achieve 5% power saving gain compared to Rel-17 SSSG switching. Considering Rel-17 SSSG switching is already quite powerful, the proposed enhancement may worth a visit. |

Two-stage CDRX On Duration

Ericsson proposed and evaluated the two-stage DRX solution to handle jitter for quasi-periodic XR traffic flows so that UE can monitor PDCCH sparsely before the data arrival for saving power. vivo compared sparse PDCCH monitoring followed by Rel-17 SSSG switching and the two-stage On-Duration. Evaluation results can be found in Table 5. Sony and Google also supported the two-stage DRX configuration.

Table 5: Evaluation results for two-stage CDRX On Duration

| Company | Evaluation results |
| --- | --- |
| Ericsson | Figure 4. Illustration of two-stage DRX. Observation 4 R17 SSSG switching may significantly degrade the percentage of satisfied UEs (down to 53%), depending on the PDCCH monitoring pattern. Observation 5 Two-stage DRX always outperforms R17 SSSG switching with similar monitoring pattern. Observation 6 Two-stage DRX achieves up to 19% PSG, while also achieving a high percentage of satisfied UEs (79.2–100%), for both high and low loads. Observation 7 Two-stage DRX combined with PDCCH skipping achives up to 27% PSG, while also achieving a high percentage of satisfied UEs (78.6–100%). Proposal 6 Adopt the two-stage DRX solution to handle jitter for quasi-periodic XR traffic flows. Proposal 7 RAN2 can address the details of the DRX specification changes for supporting two-stage DRX. Table 2: Results for FR1, high load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 91.7%; -; -. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.2%; 10.5%; 11.4%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.3%; 15.8%; 16.0%. R1-2210922; R17 SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 53.9%; 15.5%; 16.0%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 78.4%; 16.4%; 16.5%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 78.9%; 20.2%; 20.4%. R1-2210922; Two-stage DRX & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; H; 8; 79.2%; 14.8%; 15.1%. R1-2210922; Two-stage DRX & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 2; outer ODT: 10; inner ODT: 1; 4; H; 8; 80.2%; 16.2%; 16.6%. R1-2210922; Two-stage DRX & PDCCH skipping & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; H; 8; 78.6%; 19.2%; 19.4%. R1-2210922; Two-stage DRX & PDCCH skipping & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 2; outer ODT: 10; inner ODT: 1; 4; H; 8; 80.4%; 20.4%; 20.6%. Table 3: Results for FR1, low load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; L; 2; 8; 100%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 12.8%; 12.8%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 22.5%; 22.5%. R1-2210922; R17 SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 82.9%; 18.2%; 18.2%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 19.2%; 19.2%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 26.6%; 26.6%. R1-2210922; Two-stage DRX & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; L; 2; -; 100%; 16.7%; 16.7%. R1-2210922; Two-stage DRX & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 2; outer ODT: 10; inner ODT: 1; 4; L; 2; -; 100%; 19.0%; 19.0%. R1-2210922; Two-stage DRX & PDCCH skipping & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; L; 2; -; 100%; 25.9%; 25.9%. R1-2210922; Two-stage DRX & PDCCH skipping & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 2; outer ODT: 10; inner ODT: 1; 4; L; 2; -; 100%; 27.0%; 27.0%. Table 4: Results for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 90.1%; -; -. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 85.5%; 7.2%; 7.1%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 84.7%; 7.6%; 7.6%. R1-2210922; R17 SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 66.0%; 10.3%; 10.3%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 79.8%; 10.7%; 10.7%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 79.9%; 11.3%; 11.2%. R1-2210922; Two-stage DRX & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; H; 8; 80.2%; 9.1%; 9.1%. R1-2210922; Two-stage DRX & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 2; outer ODT: 10; inner ODT: 1; 4; H; 8; 81.5%; 10.7%; 10.7%. R1-2210922; Two-stage DRX & PDCCH skipping & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; H; 8; 79.8%; 9.5%; 9.5%. R1-2210922; Two-stage DRX & PDCCH skipping & matched CDRX; outer DRX: 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); inner DRX: 2; outer ODT: 10; inner ODT: 1; 4; H; 8; 80.7%; 11.5%; 11.5%. |
| vivo | Table 4. Results for FR1, MU-MIMO, Indoor Hotspot, DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Notes. Always On; -; -; -; 5; 10; 100%; -. R17 PDCCH monitoring adaptation; 16.67; 8; 4; 5; 10; 100%; 23.36%; Note1,2. 16.67; 8; 4; 5; 10; 100%; 25.39%; Note1,3. Two-stage CDRX; Outer:16.67; 10; 4; 5; 10; 100%; 21.52%; Note4. Inner:4; 2; 4. Always On; -; -; -; 10; 10; 92.50%; -. R17 PDCCH monitoring adaptation; 16.67; 8; 4; 10; 10; 92.22%; 19.28%; Note1,2. 16.67; 8; 4; 10; 10; 88.52%; 21.84%; Note1,3. Two-stage CDRX; Outer:16.67; 10; 4; 10; 10; 86.89%; 16.36%; Note4. Inner:4; 2; 4. Note1: PDCCH skipping is indicated in the DCI that schedules a dummy PDSCH after all the HARQ-ACK processes of transmissions have been completed Note2: applying R17 sparse SSSG with PDCCH monitoring every 2 slots when DRX Onduration starts and switch to dense SSSG with PDCCH monitoring every 1 slot after detecting DCI scheduling XR traffic burst Note3: applying R17 sparse SSSG with PDCCH monitoring every 4 slots when DRX Onduration starts and switch to dense SSSG with PDCCH monitoring every 1 slot after detecting DCI scheduling XR traffic burst Note4: 4ms CDRX cycle and 2ms ODT for inner CDRX. Observation 7: Compared to the existing R17 PDCCH monitoring adaptation scheme, two-stage CDRX scheme has slightly worse performance results in terms of power saving gain and capacity. Proposal 5: Capture the above simulation results and observations for Two-stage CDRX scheme in TR 38.835. Proposal 6: Two-stage CDRX scheme is not recommended for Rel-18 XR WI. |

### 2.5.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL + UL joint evaluation, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Ericsson that

  - two-stage CDRX On Duration provides

    - mean power saving gain of 10.20% in the range of 9.10% to 11.50% for all UEs

    - mean capacity gain of -10.58% in the range of -11.4% to -9.5%

  - sparse PDDCH monitoring followed by SSSG switching as the performance reference provides

    - mean power saving gain of 10.77% in the range of 10.30% to 11.30% for all UEs

    - mean capacity gain of -16.47% in the range of -26.7% to -11.3%

- For FR1, DL only, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Ericsson that

  - two-stage CDRX On Duration provides

    - mean power saving gain of 17.65% in the range of 4.80% to 20.40% for all UEs

    - mean capacity gain of -13.18% in the range of -14.30% to -12.30%

  - sparse PDDCH monitoring followed by SSSG switching as the performance reference provides

    - mean power saving gain of 17.37% in the range of 15.50% to 20.20% for all UEs

    - mean capacity gain of -23.23% in the range of -41.20% to -14.00%

- For FR1, DL only, DU, low load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Ericsson that

  - two-stage CDRX On Duration provides

    - mean power saving gain of 22.15% in the range of 6.70% to 27.00% for all UEs

    - mean capacity gain of 0%

  - sparse PDDCH monitoring followed by SSSG switching as the performance reference provides

    - mean power saving gain of 21.33% in the range of 18.20% to 26.60% for all UEs

    - mean capacity gain of -5.70% in the range of 0.00% to -17.10%

- For FR1, DL only, InH, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from vivo that

  - two-stage CDRX On Duration provides

    - power saving gain of 16.36% for all UEs

    - capacity gain of -6.1%

  - sparse PDDCH monitoring followed by SSSG switching as the performance reference provides

    - mean power saving gain of 20.56% in the range of 19.28% to 21.84% for all UEs

    - mean capacity gain of -2.3% in the range of -4.3% to -0.3%

- For FR1, DL only, InH, low load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from vivo that

  - two-stage CDRX On Duration provides

    - power saving gain of 21.52% for all UEs

    - mean capacity gain of 0.0%

  - sparse PDDCH monitoring followed by SSSG switching as the performance reference provides

    - mean power saving gain of 24.38% in the range of 23.36% to 25.39% for all UEs

    - mean capacity gain of 0.0%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| vivo | We don’t support two-stage CDRX. The reasons are as follows. - Two-stage CDRX can be deemed as similar approach of SSSG switching - According to the simulation results from Ericsson, compared to the existing R17 PDCCH monitoring adaptation scheme (SSSG switching + PDCCH skipping, two-stage CDRX scheme (+ PDCCH skipping) has almost the same performance in terms of power saving gain and capacity. |
| MTK | Not support. The reason is we tend to think 2.6 is a better solution with less spec impact and can achieve similar effect. Having said this, we tend to think 2.5 and 2.6 are both RAN2 enhancements and can be determined by RAN2. RAN1 can just document the simulation results into the TR. |
| LGE | Support. It can be used to control not only PDCCH monitoring but also CSI reporting. Furthermore, we can also discuss more than two stage CDRX. |

Early stopping of On-DurationTimer

Huawei, Xiaomi and MediaTek proposed to early stop the On Duration Timer once the InactivityTimer expires to avoid unnecessary PDCCH monitoring after the XR frame is transmitted. Evaluation results can be found in Table 6. Rakuten Symphony, DOCOMO, III and Qualcomm also proposed the early termination of DRX active time after the XR data is scheduled.

Table 6: Evaluation results for early stopping of On-DurationTimer based on expiration of InactivityTimer

| Company | Evaluation results |
| --- | --- |
| Huawei | Figure 2. Example to stop PDCCH monitoring in a DRX cycle after XR frame transmission finish Table 4 Simulation results of terminating OnDurationTimer by the expiration of InactivityTimer, Dense Urban, DL VR/AR@30Mbps Power saving scheme; CDRX cycle and/or start offsets (ms); ODT (ms); IAT (ms); Load H/L; #UE /cell; floor (Capacity); % of DL satisfied UE; Mean PSG of all UEs (%). Always On; -; -; -; H; 11; 11; 93.42%; -. Enhanced C-DRX; 50 {0, 16, 33}; 12; 4; H; 11; 11; 91.43%; 7.64%. Enhanced C-DRX with early stopping of ODT; 50 {0, 16, 33}; 12; 4; H; 11; 11; 88.23%; 10.22%. Proposal 3: To avoid unnecessary PDCCH monitoring after XR frame transmission finish, support stopping OnDurationTimer in a DRX cycle based on the expiration of InactivityTimer. |
| Xiaomi | Proposal 6: On duration can be early terminated to reduce PDCCH monitoring for XR traffic. Table 7: Relative energy of different UE power consumption states and performance of other metrics PDCCH without PDSCH; PDCCH with PDSCH; Deep sleep; Deep sleep TransitionEnergy; Light sleep Energy; Light sleep TransitionEnergy; Micro sleep Energy; Total Energy; PSG; Delay(ms); % of satisfied UEs. Baseline; 95.36; 13.93; 0; 0; 0; 0; 0; 109.29; N/A; 1.27; 100%. CDRX; 67.38; 17.39; 0; 0; 0; 0; 12.07; 96.84; 8.13%; 2.18; 98.41%. CDRX+On duration early termination; 33.73; 17.38; 0; 0; 3.31; 7.11; 0.57; 62.10; 43.18%; 2.38; 94.92%. |
| MTK | Figure 10 Stopping ODT early after data arrival Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); % of DL satisfied UE; Mean PSG of all UEs (%). AlwaysOn - baseline; 0; 0; 0; 12; 12; 94.6%; 0%. R15/16CDRX; 16; 12; 8; 12; 12; 92.7%; 5.4%. eCDRX (rational DRX cycle); (50/3); 12; 8; 12; 12; 94.3%; 9.9%. eCDRX + stop ODT early; (50/3); 12; 8; 12; 12; 92.4%; 16.6%. Table 2 SLS evaluation results with stop ODT early, FR1, DL-only, DU, CG 30Mbps Observation 13: Stopping ODT early + eC-DRX provides significant power savings with marginal impact on UE satisfaction rate compared to Rel-17 C-DRX (11.2% PSG and 0.3% reduction in UE satisfaction rate) as shown in Table 2 and Figure 11. Proposal 6: Reduce DRX on-duration after the arrival of data by stopping ODT to enable the UE to go to sleep early. |

### 2.6.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL only, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from Huawei that

  - on top of eCDRX, the early stopping of ODT provides

    - power saving gain of 10.22% for all UEs

    - capacity gain of -5.6%

  - eCDRX as the performance reference provides

    - power saving gain of 7.64% for all UEs

    - capacity gain of -2.1%

- For FR1, DL only, DU, high load, CG 30Mbps traffic at 60fps and 10ms PDB, it is observed from MediaTek that

  - with eCDRX, the early stopping of ODT provides

    - power saving gain of 16.6% for all UEs

    - capacity gain of -2.3%

  - eCDRX as the performance reference provides

    - power saving gain of 9.9% for all UEs

    - capacity gain of -0.3%

- For FR1, DL only, DU, high load, VR 45Mbps traffic at 60fps and 10ms PDB, it is observed from Xiaomi that

  - with eCDRX, the early stopping of ODT provides

    - power saving gain of 43.18% for satisfied UEs

    - capacity gain of -2.1%

  - eCDRX as the performance reference provides

    - power saving gain of 11.39% for all UEs

    - capacity gain of 1.5%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |
| ZTE, Sanechips | In fact similar function (e.g.,end the onDurationTimer) can be realized by a MAC CE signalling “go to sleep”, or legacy PDCCH skipping indication. Thus the benefit can’t be justified by only comparing with eCDRX, and above existing features should be accounted. Besides, the difference to legacy PDCCH skipping should be clarified. |
| Nokia1 | Tend to agree with ZTE comment that it would seem relevant to compare the early stopping with Rel-17 PDCCH monitoring adaptation (e.g. skipping). This might be worthwhile to capture as a part of the observation (presence or lack of comparison relevant existing schemes could be noted in observations also more generally, not just for this particular case). |
| Intel | CG PDB should be 15ms | Agree with MTK that second observation should be discussed under PDCCH monitoring adaptation, not C-DRX enhancement. Satisfied UE ratio and capacity gain are used interchangeably. CG PDB should be 15ms. |
| Xiaomi | For the following, we think there is a mistake on the capacity gain, and is corrected as follows(we take the capacity gain means the gain of satisfied UE, hope this what FL means…) - For FR1, DL only, DU, high load, VR 45Mbps traffic at 60fps and 10ms PDB, it is observed from Xiaomi that - with eCDRX, the early stopping of ODT provides - power saving gain of 43.18% for satisfied UEs - capacity gain of -2.15.08% - eCDRX as the performance reference provides - power saving gain of 11.39% for all UEs - capacity gain of 1.5-1.59% |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| vivo | We think proposed scheme 2.6 can be discussed in RAN2. We understand this is RAN2 issue. |
| MTK | Support. We think this enhancement can effectively handle the jitter issue. However, we also think we should leave the decision to RAN2 since this is more like a RAN2 mechanism. RAN1 can just document the simulation results. |

Multiple active CDRX configurations

Ericsson proposed to configure multiple simultaneous DRX configurations to serve multiple data flows with stringent PDB requirements. In the evaluation, one DRX is configured for the DL video with cycle aligned with DL video periodicity and the other DRX is configured for the DL audio. vivo showed that for multiple traffic flow (video + audio), existing scheme with single CDRX configuration for video and SPS for audio can achieve better performance than multiple active CDRX configurations. Evaluation results can be found in Table 7. TCL, Intel, ZTE, Sony, Lenovo, Xiaomi, ETRI, NEC, LGE, Apple, Rakuten Symphony, InterDigital and DOCOMO also proposed multiple active CDRX configurations.

Table 7: Evaluation results for CDRX enhancements for multiple active CDRX configurations

| Company | Evaluation results |
| --- | --- |
| Ericsson | Table 5 Results for multi-flow DRX, for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (30 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 10 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 5; 5; 94.0%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 5; 5; 90.7%; 3.0%; 3.3%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 5; 4; 85.9%; 6.9%; 7.2%. R1-2210922; Matched CDRX (with our solution); 33.3 (drx_offset=3, traffic_time_offset=1 ms, drx-LongCycle=33 ms); 10; 4; H; 5; 0; 0%; 18.4%; -. R1-2210922; Multi-flow DRX & matched CDRX; 33.3 (drx_offset=3, traffic_time_offset=1 ms, drx-LongCycle=33 ms); 10; 4; H; 5; 4; 88.4%; 13.4%; 13.6%. 10; 2; 0. R1-2210922; Multi-flow DRX & two-stage DRX & matched CDRX; outer DRX: 33.3 (drx_offset=3, traffic_time_offset=1 ms, drx-LongCycle=33 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; H; 5; 4; 81.8%; 16.9%; 17.2%. 10; 2; 4. Table 6 Results for multi-flow DRX, for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (45 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 10 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 7; 7; 90.1%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 7; 6; 86.3%; 2.7%; 2.6%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 7; 5; 78.7%; 6.4%; 6.4%. R1-2210922; Matched CDRX (with our solution); 22.2 (drx_offset=9, traffic_time_offset=2 ms, drx-LongCycle=22 ms); 10; 4; H; 7; 0; 0%; 12.3%; -. R1-2210922; Multi-flow DRX & matched CDRX; 22.2 (drx_offset=9, traffic_time_offset=2 ms, drx-LongCycle=22 ms); 10; 4; H; 7; 6; 85.9%; 10.1%; 10.0%. 10; 2; 0. R1-2210922; Multi-flow DRX & two-stage DRX & matched CDRX; outer DRX: 22.2 (drx_offset=9, traffic_time_offset=2 ms, drx-LongCycle=22 ms); inner DRX: 4; outer ODT: 10; inner ODT: 2; 4; H; 7; 71.3%; 13.1%; 12.8%. 10; 2; 0. Observation 8 A single DRX configuration matched to one traffic flow may not fulfill the PDBs of other traffic flows, resulting in a capacity of zero UEs/cell. Observation 9 Multiple simultaneous DRX configurations, each matching a traffic flow, achieves both high UE power saving gains (up to 13.6%) and a high percentage of satisfied UEs (85.9-88.4%), if a single DRX configuration matched to one flow does not satisfy the PDBs of other flows. Observation 10 If multiple DRX configurations are required, combining this with two-stage DRX is sometimes the preferred solution to achieve the highest UE power saving gains (17.2%) and a high percentage of satisfied UEs (81.8%). Proposal 8 Support multiple simultaneous DRX configurations. Proposal 9 RAN2 can address the details of the DRX specification changes for supporting multiple DRX configurations. |
| vivo | Table 6. Results for multi-flow, for FR1, MU-MIMO, Indoor Hotspot, and multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB) Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs. Always On; -; -; -; 5; 10; 100%; -. One active CDRX for video; 16.67; 8; 4; 5; 10; 100%; 11.70%. One active CDRX for video and SPS for audio; 16.67; 8; 4; 5; 10; 100%; 9.81%. Multiple CDRXs; 16.67; 8; 4; 5; 10; 100%; 9.72%. 10; 2; 0. Multiple CDRXs; 16.67; 8; 4; 5; 10; 100%; 9.52%. 10; 2; 2. Always On; -; -; -; 10; 10; 92.09%; -. One active CDRX for video; 16.67; 8; 4; 10; 10; 91.07%; 9.23%. One active CDRX for video and SPS for audio; 16.67; 8; 4; 10; 10; 91.66%; 8.14%. Multiple CDRXs; 16.67; 8; 4; 10; 10; 91.02%; 8.04%. 10; 2; 0. Multiple CDRXs; 16.67; 8; 4; 10; 10; 91.46%; 6.95%. 10; 2; 2. Table 7. Results for multi-flow, for FR1, MU-MIMO, Indoor Hotspot, and multi-stream traffic: DL video (30 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 10 ms PDB) Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs. Always On; -; -; -; 3; 6; 100%; -. One active CDRX for video; 33.33; 8; 4; 3; 6; 0.00%; 41.06%. One active CDRX for video and SPS for audio; 33.33; 8; 4; 3; 6; 100%; 20.70%. Multiple CDRXs; 33.33; 8; 4; 3; 6; 100%; 19.84%. 10; 2; 0. Multiple CDRXs; 33.33; 8; 4; 3; 6; 100%; 18.81%. 10; 2; 2. Always On; -; -; -; 6; 6; 94.91%; -. One active CDRX for video; 33.33; 8; 4; 6; 6; 0.00%; 37.32%. One active CDRX for video and SPS for audio; 33.33; 8; 4; 6; 6; 94.44%; 19.02%. Multiple CDRXs; 33.33; 8; 4; 6; 6; 93.98%; 18.34%. 10; 2; 0. Multiple CDRXs; 33.33; 8; 4; 6; 6; 94.44%; 16.85%. 10; 2; 2. Observation 11: For multiple traffic flow (video + audio), existing scheme with single CDRX configuration for video and SPS for audio can achieve better performance than multiple active CDRX configurations. Proposal 9: Capture the above simulation results and observations for multiple CDRXs for multiple flows scheme in TR 38.835. |
| InterDigital | Proposal 4: Support multiple active CDRX configurations for handling multiple flows with different traffic patterns Proposal 5: Support dynamic activation/deactivation of multiple CDRX configurations Proposal 6: Support dynamic indication from the UE to request activation of preconfigured CDRX configurations Table 1: CDRX parameters for multiple active CDRX schemes DRX cycle (ms); On duration timer value (ms); Inactivity timer value (ms). Multi-CDRX (Set 1); CDRX1 (Video); 33; 10; 6. CDRX2 (Audio); 10; 2; 6. Multi-CDRX (Set 2); CDRX1 (Video); 33; 12; 4. CDRX2 (Audio); 10; 2; 4. Table 2: Mean PSG and capacity for Cloud Gaming Video and Audio Scenario; Traffic; Capacity (%); Mean PSG (Baseline: Always-On); Mean PSG (Baseline: Single CDRX). No DRX; CG Video + Audio; 100%; N/A; N/A. Single CDRX Config (10,8,2); CG Video + Audio; 100%; 7.65%; N/A. Multi-CDRX Config Set 1; CG Video + Audio; 94.4%; 12.08%; 4.73%. Multi-CDRX Config Set 2; CG Video + Audio; 91.7%; 17.16%; 10.23%. Table 3: Mean PSG and capacity for AR Video and Audio flows Scenario; Traffic; Capacity (%); Mean PSG (Baseline: Always-ON); Mean PSG (Baseline: Single CDRX). Always-on; AR Video + Audio; 91.7%; N/A; N/A. Single CDRX Config (10,8,2); AR Video + Audio; 83.3%; 6.22%; N/A. Multi-CDRX Config Set 1; AR Video + Audio; 72.2%; 12.82%; 6.97%. Multi-CDRX Config Set 2; AR Video + Audio; 72.2%; 18.09%; 12.54%. Observation 3: Using multiple active CDRX configurations when handling multiple flows with different traffic patterns helps achieve reasonably higher PSG (from 10.2% to 12.5%) compared to using single CDRX at the tradeoff of some capacity (from 8.3 % to 11.1%) Observation 4: The less stringent PDB requirement for the cloud gaming video + audio flows allows to achieve better PSG and lower capacity loss when supporting multiple active CDRX configurations compared to AR video + audio flows Observation 5: The multiple CDRX configuration set (Set 2) which has a longer ON duration and shorter inactive timer duration achieves a higher PSG compared to an alternative multiple CDRX configuration set (Set 1), when accommodating video and audio flows |

### 2.7.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL + UL joint evaluation, DU, high load, VR 30Mbps traffic at 30fps with 10ms PDB and DL audio, it is observed from Ericsson that

  - eCDRX with a single CDRX configuration provides

    - power saving gain of 18.4% for all UEs

    - capacity gain of -100%

  - Multiple CDRX configurations + eCDRX provides

    - power saving gain of 13.4%

    - capacity gain of -6.0%

  - Multiple CDRX configurations + eCDRX + two-stage DRX provides

    - power saving gain of 17.2%

    - capacity gain of -13.0%

- For FR1, DL + UL joint evaluation, DU, high load, VR 45Mbps traffic at 30fps with 10ms PDB and DL audio, it is observed from Ericsson that

  - eCDRX with a single CDRX configuration provides

    - power saving gain of 12.3% for all UEs

    - capacity gain of -100%

  - Multiple CDRX configurations + eCDRX provides

    - power saving gain of 10.1%

    - capacity gain of -4.7%

  - Multiple CDRX configurations + eCDRX + two-stage DRX provides

    - power saving gain of 13.1%

    - capacity gain of -20.9%

- For FR1, DL only evaluation, InH, high load, AR 30Mbps traffic at 30fps with 10ms PDB and DL audio, it is observed from InterDigital that

  - Multiple CDRX configurations provides

    - mean power saving gain of 15.54% in the range of 12.82% to 18.09%

    - mean capacity gain of -21.3%

  - performance reference a single CDRX configuration provides

    - power saving gain of 6.22% for all UEs

    - capacity gain of -9.2%

- For FR1, DL only evaluation, InH, high load, CG 30Mbps traffic at 30fps with 15ms PDB and DL audio, it is observed from InterDigital that

  - single CDRX configuration provides

    - power saving gain of 7.647% for all UEs

    - capacity gain of 0%

  - Multiple CDRX configurations provides

    - mean power saving gain of 14.62% in the range of 12.08% to 17.16%

    - mean capacity gain of -6.95% in the range of -8.3% to -5.6%

- For FR1, DL-only evaluation, InH, high load, VR 30Mbps traffic at 30fps with 10ms PDB and DL audio with 10ms PDB, it is observed from vivo that

  - multiple CDRX configurations with eCDRX provides

    - mean power saving gain of 17.60% in the range of 16.85% to 18.34%

    - mean capacity gain of -0.75% in the range of -1.0% to -0.5%

  - performance reference single CDRX + SPS provides

    - power saving gain of 19.02%

    - capacity gain of -0.5%

- For FR1, DL-only evaluation, InH, low load, VR 30Mbps traffic at 30fps with 10ms PDB and DL audio with 10ms PDB, it is observed from vivo that

  - multiple CDRX configurations with eCDRX provides

    - mean power saving gain of 19.33% in the range of 18.81% to 19.84%

    - capacity gain of 0%

  - performance reference single CDRX + SPS provides

    - power saving gain of 20.70%

    - capacity gain of 0%

- For FR1, DL-only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB and DL audio, it is observed from vivo that

  - multiple CDRX configurations with eCDRX provides

    - mean power saving gain of 7.50% in the range of 6.95% to 8.04%

    - mean capacity gain of -0.95% in the range of -1.2% to -0.7%

  - single CDRX + SPS performance reference provides

    - power saving gain of 8.14%

    - capacity gain of -0.5%

- For FR1, DL-only evaluation, InH, low load, VR 30Mbps traffic at 60fps with 10ms PDB and DL audio, it is observed from vivo that

  - multiple CDRX configurations with eCDRX provides

    - mean power saving gain of 9.62% in the range of 9.52% to 9.72%

    - mean capacity gain of 0%

  - single CDRX + SPS performance reference provides

    - power saving gain of 9.81%

    - mean capacity gain of 0%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |
| InterDigital | Made some corrections to typos/values reported in the observations above |
| ZTE, Sanechips | Multiple data flows deserve more attention and further study, if we consider more traffic models. |
| Xiaomi | Generally fine with the observations. Our motivation to support multiple C-DRX configurations is to accommodate multiple traffic flows with different characteristics. but feel also OK to reuse this mechanism to cope with C-DRX periodicity alignment issue. |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| MTK | We do not object this proposal since we share the understanding that XR traffic can have multiple flows. At the same time, we tend to think this is a RAN2 solution and can be determined by RAN2. RAN1 can first document the simulation results into the TR. |

Dynamic grant enhancement with XR-specific Pre-scheduling

CATT proposed to introduce the Dynamic grant enhancement with XR-specific Pre-scheduling under CDRX enhancements for XR UE power saving. UE is pre-configured with a XR-specific PDCCH monitoring cycle and monitoring window disassociated with C-DRX. The fixed PDCCH monitoring cycle could be customized to align with the periodic packet generation cycle of XR service.  The XR-specific PDCCH monitoring cycle would allow UE to monitor PDCCH at both DRX ON and OFF. The pre-configured PDCCH monitoring can be configured by XR-dedicated search space. The search space can be configured to fully align with XR traffic arrival. In detail, the configured periodicity of the PDDCCH monitoring cycle by the XR-dedicated search space can align with the mean XR traffic inter-arrival time, e.g., 16.67ms. The time duration of PDDCCH monitoring window can be configured to cover the network jitter range. UE monitors PDCCH only for XR-specific resource allocation and/or XR-Specific RNTI, e.g., XR-RNTI. Evaluation results can be found in Table 8. III, LGE and Qualcomm also proposed the XR specific windows.

Table 8: Evaluation results for Dynamic grant enhancement with XR-specific Pre-scheduling

| Company | Evaluation results |
| --- | --- |
| CATT | Figure 3: XR-dedicated pre-scheduling DG scheme Table 6: Procedure of C-DRX enhancement schemes in single carrier Schemes; Procedure. Semi-static C-DRX enhancement scheme; UE is configured with the enhanced C-DRX with non-uniform DRX cycles of {17ms, 17ms, 16ms} to match DRX ON-duration starting points to the refresh rate of XR packet generation, e.g., 60fps. UE normal monitors PDCCH during DRX ON-duration and transform to sleep state in DRX OFF.. Dynamic grant enhancement with XR-specific pre-scheduling scheme 1; UE is configured with the dynamic grant enhancement with XR-specific pre-scheduling which has the fixed PDCCH monitoring cycle and monitoring window disassociated with DRX. The window size of PDCCH monitoring at each cycle could be dynamically adapted to the delay variation of packet arrival caused by network jitter. UE monitors only XR scheduled PDCCH during a preconfigured PDCCH monitoring window.. Dynamic grant enhancement with XR-specific pre-scheduling scheme 2 (Dynamic grant enhancement with XR-specific pre-scheduling with non-scheduling DCI for persistent PDCCH skipping indication); UE is configured with the dynamic grant enhancement with XR-specific pre-scheduling, which the PDCCH monitoring cycle and window are based on XR traffic generation cycle and network delay jitter. UE can be indicated by non-scheduling DCI to skip the indicated interval of PDCCH monitoring occasions;. Dynamic grant enhancement with XR-specific pre-scheduling scheme 3 (Dynamic grant enhancement with XR-specific pre-scheduling with go-to-sleep):; UE is configured with the Dynamic grant enhancement with XR-specific pre-scheduling, When traffic transmission is completed, UE would be indicated to go to sleep until the next pre-scheduling cycle.. Dynamic grant enhancement with XR-specific pre-scheduling scheme 4 (Dynamic grant enhancement with XR-specific pre-scheduling with enhanced PDCCH skipping and go-to-sleep); UE is configured with Dynamic grant enhancement with XR-specific pre-scheduling and would be indicated to skip the PDCCH monitoring at the beginning of the pre-scheduling window if XR packet arrives late. UE skips monitoring the PDCCH at the Monitoring Occasion (MO) until the traffic packet arrives. When traffic transmission is completed, UE would be indicated to go to sleep until the next pre-scheduling cycle.. Table 7: The evaluation result comparison between the C-DRX enhancement schemes and UE always-on ,with MU-MIMO scheduling（XR only traffic in single carrier） Evaluation Schemes; #satisfied UEs per cell; % of satisfied UEs; Power Saving Gain (PSG); % of Capacity gain. Baseline: DG scheduling and UE always-on; 11.5; 95.8%; 0.0%; 0.0%. DG scheduling with C-DRX(16, 12, 4); 10.8; 90.0%; 8.0%; -6.1%. Semi-static C-DRX enhancement scheme with (17/17/16, 8, 4); 0; 0.0%; 25.5%; -100.0%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 1: Pre-scheduling DG window (16, 12); 10.8; 90.0%; 11.7%; -6.1%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 2: Dynamic grant enhancement with XR-specific pre-scheduling with non-scheduling PDCCH skipping indication; 10.7; 89.2%; 22.4%; -7.0%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 3: Pre-scheduling DG window (16, 12) with go-to-sleep; 10.8; 90.0%; 24.0%; -6.1%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 4: Pre-scheduling DG window (16, 12) with PDCCH skipping and go-to-sleep; 10.7; 89.2%; 29.4%; -7.0%. Observation 3: Under the same system load, while the semi-static C-DRX enhancement scheme with (17/17/16, 8, 4) obtain 25.5% power saving gain with none satisfied UE, the dynamic grant enhancement with XR-specific pre-scheduling scheme could obtain the less than 10% capacity performance gap than that of the UE always-on for DG scheduling and obtain 11.7%~29.4% PSG compared to that of the UE always-on for DG scheduling. Table 8:The performance comparison between C-DRX enhancement schemes and C-DRX (16, 12, 4) with MU-MIMO scheduling（XR only traffic in single carrier） Evaluation Schemes; #satisfied UEs per cell; % of satisfied UEs; Power Saving Gain (PSG); % of Capacity gain. Baseline: DG scheduling with C-DRX(16, 12, 4) and Rel-17 PDCCH skipping scheme; 10.8; 90.0%; 0.0%; 0.0%. Semi-static C-DRX enhancement scheme with (17/17/16, 8, 4); 0; 0.0%; 19.0%; -100.0%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 1: Pre-scheduling DG window (16, 12); 10.8; 90.0%; 3.8%; 0.0%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 2: Dynamic grant enhancement with XR-specific pre-scheduling with non-scheduling PDCCH skipping indication; 10.7; 89.2%; 15.7%; -0.9%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 3: Pre-scheduling DG window (16, 12) with go-to-sleep; 10.8; 90.0%; 17.4%; 0.0%. Dynamic grant enhancement with XR-specific pre-scheduling scheme 4: Pre-scheduling DG window (16, 12) with PDCCH skipping and go-to-sleep; 10.7; 89.2%; 23.3%; -0.9%. Observation 4: Under the similar capacity performance, while the semi-static C-DRX enhancement scheme with (17/17/16, 8, 4) provide horrible capacity loss of 100%, the Dynamic grant enhancement with XR-specific pre-scheduling could obtain less than 10% capacity performance degradation than that of the UE always-on for DG scheduling and obtain 15.7%~23.3% PSG compared to that of DG scheduling with C-DRX(16, 12, 4) and Rel-17 PDCCH skipping scheme. Proposal 10: The Dynamic grant enhancement with XR-specific pre-scheduling should be supported for XR UE power saving with the following advantages: - It can obtain 11.7%~29.4% PSG and 15.7%~223.3% PSG with comparable system capacity performance gap, compared to always-on and C-DRX(16, 12, 4) and Rel-17 PDCCH skipping scheme respectively. - This scheme can be easily configured by XR-dedicated search space and/or XR-Specific RNTI, e.g., XR-RNTI disassociated with C-DRX. Thus, it has no impact in term of specification changes and other traffic. Table 9: Procedure of C-DRX enhancement schemes in multi-carriers Semi-static C-DRX enhancement scheme; - UE is configured with the enhanced C-DRX with non-uniform DRX cycles of {17ms, 17ms, 16ms} to align with the DRX cycle to the refresh rate of XR data generation cycle, e.g., 60fps. - UE normal monitors PDCCH during DRX ON-duration and transition to sleep state in DRX OFF in both carriers with XR and eMBB due to the MAC protocol constraint.. Dynamic grant enhancement with XR-specific pre-scheduling scheme; - In the carrier of XR transmission, UE is configured with Dynamic grant enhancement with XR-specific pre-scheduling PDCCH monitoring window based on XR traffic generation cycle and network delay jitter, e.g., (16,12). - In the carrier of eMBB transmission, UE can be configured with C-DRX based on mean inter-arrival time of eMBB, e.g., (320,10,80).. Table 10: The performance comparison between C-DRX enhancement schemes and C-DRX (16, 12, 4) with MU-MIMO scheduling（mixed traffic） Evaluation Schemes; % of Power Saving Gain (PSG). Baseline: DG scheduling with C-DRX(16, 12, 4) and Rel-17 PDCCH skipping scheme; XR traffic; IM Traffic. Enhanced C-DRX (16/17/17, 8, 4); 19.0%; 27.4%. Dynamic grant enhancement with XR-specific pre-scheduling window(16, 12) and C-DRX(320, 10, 80) for IM PDCCH skipping and go-to-sleep; 23.3%; 87.2%. Proposal 11: When XR and IM traffic transmitted in different carriers, the semi-static C-DRX enhancement scheme with (16/17/17, 8, 4) degrade 59.8% PSG of carrier with IM traffic compared with that of dynamic grant enhancement with XR-specific pre-scheduling scheme. The Dynamic grant enhancement with XR-specific pre-scheduling can achieve 23.3% PSG and 87.2% PSG for carrier with XR and carrier with IM traffic, respectively. Thus, the Dynamic grant enhancement with XR-specific pre-scheduling scheme disassociated with C-DRX should be supported for UE power saving optimization of different traffic. Table 11: The performance comparison between other enhanced schemes and Dynamic grant enhancement with XR-specific pre-scheduling with PDCCH skipping and go-to-sleep Scheme; Power saving; System capacity; Impact on existing service; Note. Mean PSG (%); Mean percentage of DL satisfied UE; Capacity (#UEs per cell); Dedicated to XR services. Dynamic grant enhancement with XR-specific pre-scheduling along with PDCCH skipping and go-to-sleep; 29.68%; 89.2%; 12; Yes; Note1. Traffic arrival offset staggering[9]; 2.64~9.94%; 100%; 7~9; Yes; Note2,Note 3. Enhanced PDCCH monitoring[6]; 5.08%~21.84%; 60%~84%; 5; No; Note2. Enhanced DRX with additional active time[5]; 29.43%; 86.36%; 11; No; Note1. Early C-DRX[4]; 4.9%; --; --; Yes; Note4. Two-stage DRX[10]; 9% ~16%; --; --; Yes; Note2. Adaptive PDCCH skipping[7]; 18.35%; 93.42%; 11; No; Note2. Note1: The evaluated scenario is indoor hotspot. Note2: The evaluated scenario is dense urban. Note3: With different traffic arrival offset, i.e., random or evenly spaced, system capacity is different. Note4: 60FPS, jitter [-4, 4] ms.. Proposal 12: The Dynamic grant enhancement with XR-specific pre-scheduling with the control of PDCCH monitoring window should be supported for XR UE power saving due to the advantages that it is optimized for UE power saving for different types of services independently. |

### 2.8.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that

  - semi-static C-DRX enhancement scheme with (17/17/16, 8, 4) obtains 25.5% power saving gain with none satisfied UE, the dynamic grant enhancement with XR-specific pre-scheduling scheme could obtain the less than 10% capacity performance gap than that of the UE always-on for DG scheduling and obtain 11.7%~29.4% PSG compared to that of the UE always-on for DG scheduling.

  - Under the similar capacity performance,  while the semi-static C-DRX enhancement scheme with (17/17/16, 8, 4) provides capacity loss of 100%, the Dynamic grant enhancement with XR-specific  pre-scheduling could obtain less than 10% capacity performance degradation than that of the UE always-on for DG scheduling and obtain 15.7%~23.3% PSG compared to that of DG scheduling with C-DRX(16, 12, 4) and Rel-17 PDCCH skipping scheme.

- For FR1, DL only evaluation, InH, high load, multi-carrier, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that

  - power saving gain of enhanced C-DRX (16/17/17, 8, 4) are 19.0% and 27.4% for carrier with XR transmission and carrier with IM transmission, respectively.

  - Dynamic grant enhancement with XR-specific pre-scheduling scheme for XR traffic carrier and IM traffic carrier can obtain 23.3% and 87.2% power saving gain, respectively.

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| MTK | Support. We kind of emphasize the intention to align PDCCH monitoring window with XR traffic period. |

SPS+DG with UE power saving scheme

CATT proposed SPS+DG with UE power saving scheme for XR. The enhanced SPS with additional resource allocation mechanism for XR service at both DRX ON and DRX OFF is introduced. The configured periodic SPS occasion provides wake-up timing for XR UE during DRX OFF, and subsequent dynamic grant window ensures timely transmission of XR packets. When XR packet arrives after the wake-up timing, i.e., SPS occasion, gNB could indicate UE to perform PDCCH skipping until XR packet arrival and to transform to sleep state after XR packet transmission finish. The SPS+DG with power saving schemes introduce a pre-configured PDCCH monitoring window for dynamic scheduling after the reserved SPS resources to transmit remaining packets. The pre-configured monitoring window can cover the whole jitter range which can meet 90% satisfied UE. Furthermore, when the data transmission is completed, UE can be indicated to go-to-sleep for power saving purpose until next SPS occasion(s). Evaluation results can be found in Table 9.

Table 9: Evaluation results for SPS+DG with UE power saving scheme for XR

| Company | Evaluation results |
| --- | --- |
| CATT | Figure 4: Illustration of enhanced SPS with PDCCH skipping and go-to-sleep. Proposal 13: The SPS + DG scheme with PDCCH skipping and go-to-sleep should be supported for XR UE power saving. Table 12: Evaluation results of SPS+DG with power saving schemes compared to always-on Schemes; Considered UE set; Mean PSG compared to always-on; #satisfied UEs per cell / #UEs per cell. Baseline: Always-on; -; -; 12/12. SPS+DG; All UEs; 12.5%; 10.8/12. Satisfied UEs; 12.6%. Multiple SPS configurations; All UEs; 47.4%; 0/12. Satisfied UEs; -. SPS +DG with go-to-sleep; All UEs; 39.8%; 10.6/12. Satisfied UEs; 39.9%. Observation 5: The SPS+DG with power saving schemes can obtain the up to 39.9% power saving gain, which close to that of multiple SPS configurations. Moreover, the capacity of SPS+DG schemes is near to that of DG scheduling. Table 13: Evaluation results of SPS+DG with power saving schemes compared to C-DRX Schemes; Considered UE set; Mean PSG compared to C-DRX; #satisfied UEs per cell / #UEs per cell. Baseline: C-DRX(16, 12, 4); -; -; 10.9/12. SPS+DG; All UEs; 9.8%; 10.8/12. Satisfied UEs; 9.9%. Multiple SPS configurations; All UEs; 46.1%; 0/12. Satisfied UEs; --. SPS +DG with go-to-sleep; All UEs; 38.0%; 10.6/12. Satisfied UEs; 38.1%. Observation 6: The SPS +DG with power saving schemes can obtain the capacity performance of 10.8 UEs per cell and 9.9%~38.1% power saving gain compared to that of DG scheduling with C-DRX(16, 12, 4), additionally the multiple SPS configurations hardly provide UE the XR-specific service. Proposal 14: The SPS+DG that the pre-configured PDCCH monitoring window bundled with the reserved SPS resource for PDSCH would be provide the resource to meet the QoS requirement of XR-specific traffic with obvious power saving gain. |

### 2.9.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that

  - SPS +DG with power saving schemes can obtain the capacity performance of 10.8 UEs per cell and 9.9%~38.1% power saving gain compared to that of DG scheduling with C-DRX(16, 12, 4)

  - multiple SPS configurations provide 0% of satisfied UE.

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| MTK | Not support. We do not think SPS is a suitable tool for XR DL traffic, considering the large frame size variation. |

XR-specific playoutDelayForMediaStartup for XR UE power saving enhancement

CATT proposed to introduce XR-specific playoutDelayForMediaStartup for XR UE power saving enhancement. If the size of the playout buffer is fed back to the gNB scheduler, gNB could have additional PDB for resource allocation of XR packet. Additional PDB can give gNB more time to schedule UE within the delay budget requirements of the XR service and more likely to successfully transmit packets with link adaptation gain. The XR-application awareness of UE playout buffer size at the gNB scheduler can preferentially schedule UE with packet delay close to deadline and better channel conditions. Besides, gNB also has more retransmission chance due to additional packet delay budget. Evaluation results can be found in Table 10.

Table 10: Evaluation results for XR-specific playoutDelayForMediaStartup for XR UE power saving enhancement

| Company | Evaluation results |
| --- | --- |
| CATT | Table 14: The evaluation results of the gNB scheduling awareness schemes with XR-specific playoutDelayForMediaStartup Evaluation Schemes; Capacity; Power saving. #satisfied UEs per cell; % of satisfied UEs; Capacity performance gain; PSG. DG scheduling with UE always on (Baseline); 11.5; 95.83%; -; -. XR-specific playoutDelayForMediaStartup scheme with go-to-sleep (3 frames playout delay); 20; 94.17%; 67%; 26.43%*. XR-specific playoutDelayForMediaStartup scheme with PDCCH skipping and go-to-sleep (3 frames playout delay); 20; 93.3%; 67%; 28.51%*. *Note: The power saving gain is based on the same capacity as that of the baseline scheme (DG scheduling with UE always on).. Observation 7: Large power saving gain and capacity gain can be obtained by the awareness of UE XR-specific playoutDelayForMediaStartup. Proposal 15: gNB awareness of UE playout buffer should be studied and supported for XR UE power saving. |

### 2.10.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL only, InH, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that with the awareness of UE XR-specific playoutDelayForMediaStartup, UE power saving gain of 26.43% to 28.51% can be achieved with 67% capacity gain.

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| MTK | Support. If our understanding is correct, this is like a more accurate UE buffer status report with additional timing (delay) information. At the same time, we may have to align companies’ understanding on “UE playout buffer” so this additional UE assistant information can be correctly used if introduced. |

# PDCCH monitoring adaptation related enhancements

PDCCH monitoring resume after UE transmits NACK

Ericsson proposed that UE can resume PDCCH monitoring during the time that drx-RetransmissionTimerDL is running, regardless of previous PDCCH skipping indications. vivo proposed that if there is NACK feedback during the skipping duration, UE resumes PDCCH monitoring to receive corresponding retransmission scheduling within the drx-retransmissiontimer. Additionally, network can dynamically enable or disable this mechanism for the indicated PDCCH skipping, e.g., according to the remaining PDB etc. MediaTek proposed that after the UE is indicated to skip PDCCH monitoring, if any NACK is transmitted or if UL data is transmitted, the UE resumes PDCCH monitoring for a configured duration. Evaluation results can be found in Table 11. ZTE evaluated the PDCCH monitoring resuming and observed marginal gain. Huawei, OPPO, ETRI, NEC, LGE, Samsung, III also proposed to resume PDCCH monitoring after PDCCH skipping starts if UE sends NACK.

These proposals and evaluation results were discussed in following proposals in RAN1 #110bis-e, but there was no consensus to adopt.

Proposal C: Support UE resumes PDCCH monitoring after PDCCH skipping starts if UE sends the NACK for Rel-18 XR-specific power saving enhancements

Table 11: Evaluation results for CDRX On Duration adaptation

| Company | Evaluation results |
| --- | --- |
| Ericsson | Figure 1 Illustration of NACK sent in the same DRX Active Time as PDCCH skipping indication. In this example, with the current specifications, the retransmission cannot be sent to the UE in the same Active Time. Figure 2 Illustration of NACK sent in a DRX Active Time subsequent to the Active Time where the PDCCH skipping indication was sent. Figure 3 Illustration of NACK sent in the same DRX Active Time as PDCCH skipping indication, after enhancement. Proposal 3 UE can resume PDCCH monitoring during the time that drx-RetransmissionTimerDL is running, regardless of previous PDCCH skipping indications. Table 2: Results for FR1, high load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 91.7%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 8; 7; 86.9%; 4.3%; 4.9%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 8; 6; 78.2%; 10.4%; 11.0%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.2%; 10.5%; 11.4%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; H; 8; 7; 87.0%; 10.1%; 10.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 87.3%; 15.8%; 16.0%. Table 3: Results for FR1, low load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; L; 2; 8; 100%. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; L; 2; -; 100%; 4.8%; 4.8%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; L; 2; -; 100%; 12.4%; 12.4%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 12.8%; 12.8%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; L; 2; -; 100%; 12.5%; 12.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 22.5%; 22.5%. Table 4: Results for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 90.1%; -; -. R1-2210922; R15/16 DRX (Long DRX); 10; 8; 4; H; 8; 7; 86.9%; 2.6%; 2.7%. R1-2210922; R15/16 DRX (Short DRX); 4; 2; 4; H; 8; 6; 80.2%; 6.1%; 6.1%. R1-2210922; Matched CDRX (with our solution); 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 85.5%; 7.2%; 7.1%. R1-2210922; Matched CDRX (solutions from other companies); 16.6 (17-17-16 equivalent); 10; 4; H; 8; 7; 86.4%; 7.6%; 7.5%. R1-2210922; PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 7; 84.7%; 7.6%; 7.6%. |
| vivo | Observation 13: Rel-17 UE power saving WI did not evaluated the XR type of traffic which has tight delay bound (i.e., 10ms PDB), meaning that any transmission failure will impact the system capacity. Observation 14: The existing R17 PDCCH skipping indication will cause either capacity loss or unnecessary PDCCH monitoring. Observation 15: Only using short R17 PDCCH skipping durations, e.g., 2ms or 4ms is not a reasonable R17 PDCCH skipping implementation method. Figure 7. Potential network implementation of PDCCH skipping indication using Rel-17 specification Figure 8. Example of PDCCH skipping and interaction with HARQ retransmission Figure 9. Power saving gain of VR/AR 30Mbps in InH scenario Figure 10. System capacity of VR/AR 30Mbps in InH scenario Figure 11. System capacity of VR/AR 30Mbps in InH scenario Observation 16: Compared with the existing R17 PDCCH skipping indication, the enhanced PDCCH skipping with HARQ interaction provides good balance between power saving gain and system capacity. Proposal 12: Recommend the PDCCH skipping with HARQ retransmission interaction for R18 XR WI. - Solution 1: If there is any NACK feedback for a scheduled PDSCH during the skipping duration, UE is required to resume the PDCCH monitoring for corresponding retransmission. - Solution 1-1: On top of solution 1, the network can dynamically enable or disable the HARQ interaction for the indicated PDCCH skipping, according to the remaining PDB, the importance of the transmission or the probability of decoding failure (e.g. link quality). Observation 17: By adopting the schemes of both PDCCH skipping interaction with HARQ retransmission and LP-WUS jitter handling, 41.90%~43.84% and 37.20%~38.47% total power saving gains can be obtained in low load and high load, respectively, and without capacity loss. Table 9. Results for FR1, MU-MIMO, Indoor Hotspot, DL video (30 fps, 30 Mbps, 10 ms PDB) Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Notes. Always On; -; -; -; 5; 10; 100%; -; Note1. -; -; -; 5; 10; 100%; -; Note2. R17 PDCCH monitoring adaptation; 16.67; 8; 4; 5; 10; 100%; 23.36%; Note1, 3, 4,5. 16.67; 16; 4; 5; 10; 100%; 15.79%; Note2, 3,4,5. Enhanced PDCCH skipping with HARQ interaction; 16.67; 8; 4; 5; 10; 100%; 37.13%; Note1, 3,5. 16.67; 16; 4; 5; 10; 100%; 27.77%; Note2, 3,5. LP-WUS scheme and enhanced PDCCH skipping with HARQ interaction; 16.67; 8; 4; 5; 10; 100%; 43.84%; Note1, 3,8. 16.67; 16; 4; 5; 10; 100%; 41.90%; Note2, 3,8. Always On; -; -; -; 10; 10; 92.50%; -; Note1. -; -; -; 10; 10; 97.45%; -; Note1,7. -; -; -; 10; 10; 93.12%; -; Note1,6. -; -; -; 10; 10; 97.45%; -; Note1,6,7. -; -; -; 10; 10; 91.83%; -; Note2. R17 PDCCH monitoring adaptation; 16.67; 16; 4; 10; 10; 91.05%; 12.26%; Note2,4,5. 16.67; 8; 4; 10; 10; 1.11%; 35.21%; Note1,3,5. 16.67; 8; 4; 10; 10; 2.22%; 35.21%; Note1,3,5,7. 16.67; 8; 4; 10; 10; 2.08%; 32.77%; Note1,3,5,6. 16.67; 8; 4; 10; 10; 94.91%; 32.77%; Note1,3,5,6,7. 16.67; 8; 4; 10; 10; 92.22%; 19.28%; Note1,4,5. 16.67; 8; 4; 10; 10; 97.45%; 19.28%; Note1,4,5,7. 16.67; 8; 4; 10; 10; 92.78%; 19.48%; Note1,4,5,6. 16.67; 8; 4; 10; 10; 97.45%; 19.48%; Note1,4,5,6,7. Enhanced PDCCH skipping with HARQ interaction; 16.67; 8; 4; 10; 10; 92.22%; 32.18%; Note1, 3,5. 16.67; 16; 4; 10; 10; 92.20%; 23.67%; Note2, 3,5. 16.67; 8; 4; 10; 10; 97.45%; 32.18%; Note1, 3,5,7. 16.67; 8; 4; 10; 10; 92.78%; 32.49%; Note1, 3,5,6. 16.67; 8; 4; 10; 10; 97.45%; 32.49%; Note1, 3,5,6,7. LP-WUS scheme and enhanced PDCCH skipping with HARQ interaction; 16.67; 8; 4; 10; 10; 92.22%; 38.47%; Note1, 3,8. 16.67; 16; 4; 10; 10; 91.20%; 37.20%; Note2, 3,8. Note1: jitter range = [-4, +4]ms, STD = 2ms Note2: jitter range = [-8, +8]ms, STD = 5ms Note3: the network indicates PDCCH skipping in the DCI that schedules the initial PDSCH transmission of the last DL packet of an XR traffic burst Note4: PDCCH skipping is indicated in the DCI that schedules a dummy PDSCH after all the HARQ-ACK processes of transmissions have been completed Note5: applying R17 sparse SSSG with PDCCH monitoring every 2 slots when DRX Onduration starts and switch to dense SSSG with PDCCH monitoring every 1 slot after detecting DCI scheduling XR traffic burst Note6: initial BLER is reduced from 10% to 1% Note7: satisfaction metric as 95% packet successful rate Note8: the total relative power (including the power of both LP-WUR and main radio) for LP-WUS monitoring is 45 with no wake-up latency. |
| ZTE | Proposal 4: Capture solutions and evaluation results for jitter handling into TR 38.835. - Proposal 5: Support flexible additional active time based on current CDRX design, with minor specification change. Table 5 FR1 power consumption results in Indoor scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all Ues (%). Aligned CDRX with XR traffic; Aligned every 50ms; 14; 5; 7; 7; 88.10%; 4.60%. Aligned CDRX with XR traffic; Aligned every 50ms; 12; 5; 7; 7; 80%; 9.46%. Flexible additional active time + aligned CDRX; Aligned every 50ms; 2; 4; 7; 7; 89.48%; 19%. PDCCH Skipping (duration = 2ms,4ms); Aligned every 50ms; 14; 5; 7; 7; 72.62%; 10.84%. PDCCH skipping with dummy grant (duration = 4ms,8ms); Aligned every 50ms; 14; 5; 7; 7; 80.95%; 13.03%. Table 6 FR1 power consumption results in Dense Urban scenario (VR45Mbps, fps=60, DL only, jitter=[-8,8]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all Ues (%). Aligned CDRX with XR traffic; Aligned every 50ms; 14; 5; 7; 7; 85%; 4.50%. Flexible additional active time + aligned CDRX; Aligned every 50ms; 2; 4; 7; 7; 84%; 18%. PDCCH Skipping (duration = 2ms,4ms); Aligned every 50ms; 14; 5; 7; 7; 74%; 9.6%. PDCCH skipping with dummy grant (duration = 4ms,8ms); Aligned every 50ms; 14; 5; 7; 7; 80%; 12%. - Observation 2: For FR1, Indoor, [-8,8]ms jitter range, VR45M, DL only, - Flexible additional active time has 19% PSG with 89.48% of satisfied UE - PDCCH skipping with dummy grant has 13.03% PSG with 80.95% of satisfied UE - PDCCH skipping indicated in last transmission slot has 10.84% PSG with 72.62% of satisfied UE - Observation 3: For FR1, Dense Urban, [-8,8]ms jitter range, VR45M DL only, - Flexible additional active time has 18% PSG with 84% of satisfied UE - PDCCH skipping with dummy grant has 12% PSG with 80% of satisfied UE - PDCCH skipping indicated in last transmission slot has 9.6% PSG with 74% of satisfied UE Observation 4: For evaluation of power saving techniques, the limitations of legacy PDCCH skipping should be accounted. - PDCCH skipping is indicated in a scheduling DCI - The PDCCH skipping duration may delay potential retransmission scheduling - PDCCH skipping with dummy grant costs additional power consumption due to decoding PDSCH and transmitting HARQ-ACK to the dummy grant - Proposal 6: There is no need to enhance Rel-17 PDCCH skipping for re-transmission. |
| MTK | Observation 1: In current Spec, no additional PDCCH monitoring for retransmission is allowed when PDCCH skipping is triggered. Due to XR application’s stringent latency requirement, further retransmission-aware mechanisms should be explored. We have two solutions to address the retransmission following the PDCCH skipping indication. Solution 1: If UE is indicated to skip PDCCH monitoring for a specific duration based on the Rel-17 adaptation scheme, then if any NACK is transmitted, UE resumes the monitoring and cancels the indication of Skipping. Proposal 1: (Solution 1) If UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted, UE resumes the monitoring and cancels the PDCCH skipping. Solution 2: In solution 2, when UE is indicated to skip PDCCH monitoring for a duration by the Rel-17 adaptation scheme, then if any NACK is transmitted or if the scheduled UL data is transmitted, the UE has to resume PDCCH monitoring for a configured duration. Proposal 2: (Solution 2) If UE is indicated to skip PDCCH monitoring, then if any NACK is transmitted or if UL data is transmitted, UE resumes the PDCCH monitoring for a configured duration. Observation 2: Solution 2 can be applied to both DL and UL. Also, it can get better power saving gain for an appropriate configured duration. Figure 1 Power saving gain results by SLS for retransmission aware PDCCH skipping Observation 3: In Figure 1, it can be seen that if UE does not resume PDCCH monitoring for the potential retransmission (y=0), the outage rate (ratio of unsatisfied UE) grows fast when the PDCCH skipping duration exceeds 4ms for cloud gaming (CG), and 2ms for XR. This means only short skip durations are acceptable for Rel-17 PDCCH skipping if there is no reTX handling. Observation 4: As shown in Figure 1, with Solution 2, UE performs PDCCH skipping with 12ms while resuming PDCCH monitoring for 5ms when a NACK is transmitted. With this ReTx handling method, it can achieve a significant power saving gain (20.78%~27.97%) w.r.t Rel-17 PDCCH skipping. Observation 7: During RAN1 #110bis-e discussion [5], Nokia/ZTE mentions that MTK SLS simulation result is not considering CDRX and the impact of transition of active/inactive time is not included. The reason CDRX was not included is that legacy CDRX is not suited for XR traffic and would evidently increase the outage rate. Figure 4 Power saving gain results by SLS for retransmission aware PDCCH skipping with CDRX Observation 8: To demonstrate that retransmission aware PDCCH skipping also provides power saving gain when operating with CDRX, SLS simulation is performed to evaluate power saving gain of retransmission aware PDCCH skipping with (16,10,5) CDRX cycle. From Figure 4, it can be seen that when CDRX comes into play, retransmission aware PDCCH skipping with CDRX still achieves higher power saving gain (15.29% compared to Rel-17 PDCCH skipping) with lower outage rate increment. Figure 5 Power saving gain results by SLS for DL retransmission aware PDCCH skipping with UL pose/control Observation 9: During RAN1 #110bis-e discussion [5], ZTE mentions that UL transmission should be jointly considered/optimized. To evaluate the impact of UL transmission, a joint DL/UL SLS simulation is performed for VR application. From Figure 5, it can be seen that when UL pose/control (4ms period) comes into play, DL retransmission aware PDCCH skipping still achieves 14.27% power saving gain compared to Rel-17 PDCCH skipping. Proposal 3: RAN1 to specify a DL retransmission aware PDCCH skipping in R18 XR WI; UL retransmission optimization can be jointly considered/enhanced if necessary in R18 XR WI. Observation 10: For potential enhancement of UL retransmission, the following can be considered: - For UL pose control: retransmission-less configured grant (CG) as proposed by Qualcomm in [6] - For UL AR video: UL (re-)transmission handling in C-DRX as proposed by Huawei in [7] |

### 3.3.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made for UE resume PDCCH monitoring after PDCCH skipping starts if UE sends the NACK

Observation

- For FR1, DL + UL joint evaluation, DU, high load, VR 30Mbps traffic at 60fps with 10ms PDB and DL audio, it is observed from Ericsson that

  - eCDRX as performance reference provides

    - mean power saving gain of 7.4% in the range of 7.2% to 7.6% for all UEs

    - mean capacity gain of -4.60% in the range of -5.1% to -4.1%

  - PDCCH skipping enhancement with eCDRX provides

    - power saving gain of 7.6% for all UEs

    - capacity gain of -6.0%

- For FR1, DL + UL joint evaluation, DU, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from MediaTek that

  - R17 PDCCH skipping performance reference provides

    - mean power saving gain of 5.18% in the range of 3.96% to 6.39% for all UEs

    - mean capacity gain of -19.90% in the range of -32.7% to -7.1%

  - enhanced PDCCH skipping provides

    - power saving gain of 17.66%

    - capacity gain of -3.4%

- For FR1, DL + UL joint evaluation, DU, high load, CG 30Mbps traffic at 60fps with 10ms PDB, it is observed from MediaTek that

  - Rel-17 PDCCH skipping performance reference provides

    - mean power saving gain of 11.29% in the range of 8.64% to 14.11% for all UEs

    - mean capacity gain of -15.77% in the range of -32.0% to -5.1%

  - enhanced PDCCH skipping provides

    - mean power saving gain of 25.19% in the range of 20.78% to 29.60% for all UEs

    - capacity gain of -8.8%

- For FR1, DL only evaluation, DU, high load, DRX not configured, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from MediaTek that

  - R17 PDCCH skipping as performance reference provides

    - mean power saving gain of 6.95% in the range of 5.27% to 8.62% for all UEs

    - mean capacity gain of -14.70% in the range of -25.4% to -4.0%

  - enhanced PDCCH skipping provides

    - mean power saving gain of 29.87% in the range of 27.97% to 31.77% for all UEs

    - capacity gain of -1.0%

- For FR1, DL only evaluation, DU, high load, DRX configured, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from MediaTek that

  - R17 PDCCH skipping as performance reference provides

    - mean power saving gain of 7.73% in the range of 6.04% to 9.41% for all UEs

    - mean capacity gain of -89.80% in the range of -96.6% to -83.0%

  - enhanced PDCCH skipping provides

    - power saving gain of 23.27%

    - capacity gain of -77.6%

- For FR1, DL only evaluation, DU, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from Ericsson that

  - eCDRX as performance reference provides

    - mean power saving gain of 10.3% in the range of 10.1% to 10.5% for all UEs

    - mean capacity gain of -5.0% in the range of -5.1% to -4.9%

  - PDCCH skipping enhancement with eCDRX provides

    - power saving gain of 15.8% for all UEs

    - capacity gain of -4.8%

- For FR1, DL only evaluation, DU, low load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from Ericsson that

  - eCDRX as performance reference provides

    - mean power saving gain of 12.65% in the range of 12.5% to 12.8% for all UEs

    - capacity gain of 0%

  - PDCCH skipping enhancement with eCDRX provides

    - power saving gain of 22.5% for all UEs

    - capacity gain of 0%

- For FR1, DL only evaluation, DU, high load, VR 45Mbps traffic at 60fps with 10ms PDB, it is observed from ZTE that

  - R17 PDCCH skipping + eCDRX as performance reference provides

    - mean power saving gain of 10.80% in the range of 9.6% to 12% for all UEs

    - mean UE satisfied ratio of 77% in the range of 74% to 80%

  - PDCCH skipping with retransmission enhancement + eCDRX provides

    - power saving gain of 16% for all UEs with

    - UE satisfied ratio of 84%

- For FR1, DL only evaluation, DU, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from MediaTek

  - R17 PDCCH skipping as performance reference provides

    - mean power saving gain of 6.44% in the range of 4.90% to 7.97% for all UEs

    - mean capacity gain of -18.05% in the range of -29.6% to -6.5%

  - enhanced PDCCH skipping without DRX provides

    - mean power saving gain of 27.50% in the range of 25.68% to 29.31% for all UEs

    - capacity gain of -2.70%

- For FR1, DL only evaluation, InH, high load, VR 45Mbps traffic at 60fps with 10ms PDB, it is observed from ZTE that

  - R17 PDCCH skipping + eCDRX performance reference provides

    - mean power saving gain of 11.94% in the range of 10.84% to 13.03% for all UEs

    - mean UE satisfied ratio of 76.79% in the range of 72.62% to 80.95%.

  - PDCCH skipping with retransmission enhancement + eCDRX provides

    - power saving gain of 16.31% for all UEs

    - UE satisfied ratio of 84.52%.

- For FR1, DL only evaluation, InH, low load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from vivo that

  - R17 PDCCH skipping + eCDRX performance reference provides

    - mean power saving gain of 19.58% in the range of 15.79% to 23.36% for all UEs

    - capacity gain of 0%

  - Enhanced PDCCH skipping + eCDRX provides

    - mean power saving gain of 32.45% in the range of 27.77% to 37.13% for all UEs

    - capacity gain of 0%

  - When LP-WUS is adopted, enhanced PDCCH skipping + eCDRX + LP-WUS provides

    - mean power saving gain of 42.87% in the range of 41.90% to 43.84% for all UEs

    - capacity gain of 0%

- For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from vivo that

  - R17 PDCCH skipping + eCDRX performance reference provides

    - mean power saving gain of 25.08% in the range of 12.26% to 35.21% for all UEs

    - mean UE satisfied ratio of 63.47% in the range of 1.11% to 97.45%.

  - Enhanced PDCCH skipping + eCDRX provides

    - mean power saving gain of 30.60% in the range of 23.67% to 32.49% for all UEs

    - mean UE satisfied ratio of 94.42% in the range of 92.20% to 97.45%.

  - When LP-WUS is adopted, enhanced PDCCH skipping + eCDRX provides

    - mean power saving gain of 37.84% in the range of 37.20% to 38.47% for all UEs

    - mean UE satisfied ratio of 91.71% in the range of 91.20% to 92.22%.

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |
| ZTE, Sanechips | Some results show clear power saving gain; for following cases, we have some concerns: In case of the first bullet, basically no power saving gain is observed for PDCCH skipping enhancement with eCDRX. In the fifth bullet, the results of case ‘FR1, DL only evaluation, DU, high load, DRX configured, VR 30Mbps’ shows huge capacity loss, which would affect the UE experience seriously. As FL pointed out, in some cases, the marginal power saving gain is observed for the PDCCH skipping enhancement. |
| OPPO2 | We are generally fine with the above observations except the following modifications, since it is observed from our Tdoc [R1-2211490] that eCDRX provides capacity gain of -1.4%. - For FR1, DL only, DU, high load, VR 30Mbps traffic at 60fps and 10ms PDB, it is observed from OPPO that - on top of eCDRX, additional active time provides - power saving gain of 30.86% for all UEs - capacity gain of 0.0% - eCDRX as the performance reference provides - power saving gain of 18.72% for all UEs - capacity gain of -1.6% -1.4% Sorry for my misunderstanding for the calculation, my fault!! We are fine with the above observations😊 |
| MTK | We think this section belongs to PDCCH monitoring adaptation and should be moved to Section 3. Schemes like retransmission-aware PDCCH skipping can work with or without CDRX. |
| Nokia1 | Like noted in last meeting, it is not fully clear to us why these two schemes are bundled together as alternatives. One group of solutions aims to facilitate the C-DRX configuration by having more ‘dynamicity’ in the OnDuration/Active time determination. One solution seems to target to separate XR-traffic and non-XR traffic via separate monitoring patterns (and don’t this this belongs to under A). Other set of solutions focuses on handling of re-transmission window on conjunction with PDCCH skipping. We don’t really see that these solutions/proposals are competing candidates. The the onDuration extension related schemes would be more associated with Section 2.6 schemes |
| LGE | We are not sure UE’s resuming PDCCH monitoring should be considered under PDCCH monitoring avoidance after XR data reception. It is more appropriate to be discussed under PDCCH monitoring adaptation related enhancements. |
| Intel | Agree with MTK that second observation should be discussed under PDCCH monitoring adaptation, not C-DRX enhancement. Satisfied UE ratio and capacity gain are used interchangeably. CG PDB should be 15ms. |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| vivo | We support PDCCH monitoring resume after UE transmits NACK. The reasons are as follows. - The existing R17 PDCCH skipping indication will cause either capacity loss or unnecessary PDCCH monitoring. - Only using short R17 PDCCH skipping durations, e.g., 2ms or 4ms is not a reasonable R17 PDCCH skipping implementation method. - From the simulation results, compared with the existing R17 PDCCH skipping indication, the enhanced PDCCH skipping with HARQ interaction provides good balance between power saving gain and system capacity. - Using MAC based termination of PDCCH monitoring within DRX active time would introduce delay thus cause additional power consumption. |
| MTK | Support. Retransmission is critical for XR capacity, and the proposal can obtain good capacity gain with small spec impact. At the same time, we think the observation from ZTE that “PDCCH monitoring resuming has marginal gain” is not a fair observation. Looking at ZTE’s tdoc (R1-2211905) Table 8 as copied below, it can be seen that: - “PDCCH Skipping with retransmission” can provide similar power saving gain (4%~5%) over existing mechanism (PDCCH skipping dummy grant) as “aligning CDRX with XR traffic” (4.5%), in such a harsh scenario (VR 45M, jitter=[-8,8]ms). Besides, the satisfaction rate loss of “PDCCH Skipping with retransmission” is only 1%, which is even smaller than “aligned CDRX with XR traffic” (>5%, 5=90-85) Hence, we think retransmission-aware PDCCH skipping achieves more power saving benefits than “eCDRX with XR traffic alignment” and should also be considered in R18 WI. Table 8 FR1 power consumption results in Dense Urban scenario (VR45M, fps=60, DL only, jitter=[-8,8]ms) Power saving scheme; CDRX cycle (ms); ODT (ms); IAT (ms); #UE /cell; floor (Capacity); Percentage of satisfied UE; Mean PSG of all Ues (%). Aligned CDRX with XR traffic; Aligned every 50ms; 14; 5; 7; 7; 85%; 4.50%. PDCCH Skipping (duration = 2ms,4ms); Aligned every 50ms; 14; 5; 7; 7; 74%; 9.6%. PDCCH skipping with dummy grant (duration = 4ms,8ms); Aligned every 50ms; 14; 5; 7; 7; 80%; 12%. PDCCH Skipping with retransmission (duration = 3ms,7ms); Aligned every 50ms; 14; 5; 7; 7; 84%; 16%.. |
| LGE | Support. To meet stringent latency requirement of XR, it should be addressed. |

PDCCH skipping duration enhancements

Huawei proposed the adaptive PDCCH skipping duration until the earliest possible arrival time of the next frame to maximize the seamless PDCCH skipping duration. vivo proposed the one-shot skipping to the next jitter boundary position. Xiaomi proposed to adopt four PDCCH skipping durations (e.g., 6/8/10/12ms). CATT proposed to enhance PDCCH skipping by introducing the go-to-sleep indication for fast transition to the sleep state which terminates UE DRX active time immediately. Ericsson compared two cases when CDRX is configured: i) Rel-17 PDCCH skipping with two durations only; ii) enhanced PDCCH skipping with arbitrary skipping duration covering the remaining DRX active time. Evaluation results show the arbitrary skipping duration has negligible power saving or capacity gain compared with PDCCH skipping with two durations. Evaluation results can be found in the Table 11. LGE, Google, InterDigital also proposed more or flexible PDCCH skipping duration.

Table 11: Evaluation results for PDCCH skipping duration enhancements

| Company | Evaluation results |
| --- | --- |
| Huawei | Figure 7 Illustration of the flexible duration of PDCCH skipping Table 5 Simulation results of adaptive PDCCH skipping duration, Dense Urban, DL VR/AR@30Mbps Power saving scheme; Load H/L; #UE /cell; floor (Capacity); % of DL satisfied UE; Mean PSG of all UEs (%). Always On; H; 11; 11; 93.42%; -. Legacy PDCCH Skipping (D1=5, D2=10, D3=15); H; 11; 11; 93.42%; 12.12%. Legacy PDCCH Skipping (D1=4, D2=16, D3=29); H; 11; 11; 93.42%; 11.15%. PDCCH skipping with adaptive duration; H; 11; 11; 93.42%; 18.35%. Proposal 7: To avoid unnecessary PDCCH monitoring after XR frame transmission finishes, support adaptive PDCCH skipping duration, which is determined by the gap between indication reception time and earliest possible arrival time of next frame. |
| vivo | Figure 5. Enhanced PDCCH skipping duration. Figure 6. Power saving gain of 17 PDCCH skipping and enhanced PDCCH skipping for VR/AR 30Mbps in InH scenario Observation 12: Compared with R17 PDCCH skipping indication without CDRX configuration, enhanced PDCCH skipping durations can achieve an additional {4.80%~5.18%} power saving gain with no capacity loss. Proposal 10: Enhance PDCCH skipping durations e.g., additional RRC candidate duration values or indication bits of DCI) is recommended for R18 XR WI. Table 8. Results for FR1, MU-MIMO, Indoor Hotspot, DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Notes. Always On; -; -; -; 5; 10; 100%; -. R17 PDCCH monitoring adaptation; -; -; -; 5; 10; 100%; 6.93%; Note1,3. -; -; -; 5; 10; 100%; 18.18%; Note1,2,3. Enhanced PDCCH skipping; -; -; -; 5; 10; 100%; 12.06%; Note1,4. -; -; -; 5; 10; 100%; 23.36%; Note1,2,4. Always On; -; -; -; 10; 10; 92.50%; -. R17 PDCCH monitoring adaptation; -; -; -; 10; 10; 92.28%; 5.66%; Note1,3. -; -; -; 10; 10; 92.22%; 14.25%; Note1,2,3. Enhanced PDCCH skipping; -; -; -; 10; 10; 92.30%; 10.46%; Note1,4. -; -; -; 10; 10; 92.22%; 19.28%; Note1,2,4. Note1: PDCCH skipping is indicated in the DCI that schedules a dummy PDSCH after all the HARQ-ACK processes of transmissions have been completed Note2: applying R17 sparse SSSG with PDCCH monitoring every 2 slots before XR traffic burst arrives and switch to dense SSSG with PDCCH monitoring every 1 slot after detecting DCI scheduling XR traffic burst Note3: with [6ms, 4ms, 2ms] candidate skipping durations Note4: one-shot skipping to the next jitter boundary position. |
| Xiaomi | Table 1: Relative time fraction of different UE power consumption states PDCCH without PDSCH; PDCCH with PDSCH; Deep sleep; Light sleep; Micro sleep. Baseline; 91.72%; 8.28%; 0; 0; 0. genie; 0; 8.28%; 5.35%; 66.84%; 3.80%. PDCCH skipping case 1; 32.77%; 8.28%; 0; 58.95%; 0. PDCCH skipping case 2; 24.56%; 8.26%; 0; 67.18%; 0. Table 2: Relative energy of different UE power consumption states PDCCH without PDSCH; PDCCH with PDSCH; Deep sleep; Deep sleep transition; Light sleep; Light sleep transition; Micro sleep; Total; PSG. Baseline; 91.72; 24.84; 0; 0; 0; 0; 0; 116.56; N/A. genie; 0; 24.84; 0.0535; 1.38; 13.37; 5.65; 1.71; 47.00; 59.67%. PDCCH skipping case 1; 32.77; 24.84; 0; 0; 2.84; 7.46; 0; 67.91; 41.74%. PDCCH skipping case 2; 24.56; 24.77; 0; 0; 4.67; 7.31; 0; 61.31; 47.40%. Table 3: Summary of metrics Total Energy; PSG; Delay(ms); % of satisfied UEs. Baseline; 116.56; N/A; 2.46; 96.61%. PDCCH skipping case 1; 67.91; 41.74%; 2.70; 95%. PDCCH skipping case 2; 61.31; 47.40%; 2.92; 96.56%. Observation1: Compared with PDCCH skipping with 2 candidate durations, PDCCH skipping with 4 candidate durations has 6% more power saving gain, and 1.5% more satisfied UE rate. Average delay is increased a little but still within the PDB range. Proposal 3: More candidate PDCCH skipping durations should be supported to be configured by RRC signalling. |
| CATT | Proposal 5: The XR-specific go-to-sleep indication by DCI should be supported to be jointly configured with PDCCH skipping indication in the XR-specific DCI for fast transition to the sleep state PDCCH skipping enhancement schemes with no impact to other delay-insensitive traffic. Table 1: PDCCH skipping enhancement schemes Schemes; Procedure. Baseline 1：Always-on; PDCCH monitoring is based on the configured PDCCH monitoring cycle of the search space.. Baseline 2：C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme; When the XR packet transmission is completed, UE would be indicated by scheduling DCI to skip an interval of PDCCH skipping and return to normal PDCCH monitoring afterward.. PDCCH skipping enhancement scheme 1 with C-DRX(16, 12, 4) (non-scheduling and scheduling DCI with persistent skipping indication); UE can be indicated by both scheduling and non-scheduling DCI to skip the indicated interval of PDCCH monitoring occasions persistently. The skipping duration is persistent without termination of PDCCH skipping until a new skipping indication is received.. PDCCH skipping enhancement scheme 2 with C-DRX(16, 12, 4) (go-to-sleep); When the XR packet transmission is completed, a code point of skipping duration is configured as the go-to-sleep to allow UE fast transition to the sleeping state and return to normal PDCCH monitoring afterward.. PDCCH skipping enhancement scheme 3 with C-DRX(16, 12,4) (Continuous PDCCH skipping and dynamic go-to-sleep indication); UE can be indicated by non-scheduling and scheduling DCI to skip one or more MOs continuously. When the XR packet transmission is completed, a code point of skipping duration is configured as the go-to-sleep to allow UE fast transition to the sleep state.. Table 2: Evaluation results of PDCCH skipping schemes compared to always-on with MU-MIMO scheduling PDCCH monitoring Schemes; Considered UE set; Mean PSG compared to always-on; #satisfied UEs per cell. Baseline 1: Always-on; -; -; 11.5. PDCCH skipping enhancement scheme 1 (non-scheduling and scheduling DCI with persistent skipping indication); All UEs; 22.4%; 10.7. Satisfied UEs; 22.4%. PDCCH skipping enhancement scheme 2 (go-to-sleep); All UEs; 24.0%; 10.8. Satisfied UEs; 24.4%. PDCCH skipping enhancement scheme 3 (Continuous PDCCH skipping and dynamic go-to-sleep indication); All UEs; 29.4%; 10.7. Satisfied UEs; 29.9%. Observation 1: From the results in Table 2, it could be observed that: - The non-scheduling DCI based PDCCH skipping scheme can obtain 22.4% power saving gain with compared with those of always-on. - The go-to-sleep indication scheme can obtain 24.0%~24.4% power saving gain with capacity compared with those of always-on. - The go-to-sleep indication and continuous PDCCH skipping scheme can obtain up to 29.9% power saving gain compared with those of always-on. Table 3: Evaluation results of PDCCH skipping schemes compared to C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme and MU-MIMO scheduling Schemes; Considered UE set; Mean PSG compared to C-DRX(16,12,4); #satisfied UEs per cell. Baseline: C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme; -; -; 10.8. PDCCH skipping enhancement scheme 1 with C-DRX(16, 12, 4) (non-scheduling and scheduling DCI with persistent skipping indication); All UEs; 15.7%; 10.7. Satisfied UEs; 15.7%. PDCCH skipping enhancement scheme 2 with C-DRX(16, 12, 4) (go-to-sleep); All UEs; 17.4%; 10.8. Satisfied UEs; 17.8%. PDCCH skipping enhancement scheme 3 with C-DRX(16, 12, 4) (Continuous PDCCH skipping and dynamic go-to-sleep indication); All UEs; 23.3%; 10.7. Satisfied UEs; 23.8%. Observation 2: From the results in Table 3, it could be observed that: - The non-scheduling DCI based PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain 15.7% power saving gain with negligible capacity degradation compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme. - The go-to-sleep indication scheme with C-DRX(16, 12, 4) can obtain 17.4%~17.8% power saving gain with no capacity loss compared with C-DRX(16,12,4) with Rel-17 PDCCH skipping scheme. - The go-to-sleep indication and continuous PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain up to 23.8% power saving gain without capacity loss compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme. |

### 3.2.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL + UL joint evaluation, DU, DRX configured, VR 30Mbps traffic at 60fps with 10ms PDB and DL audio, it is observed from Ericsson that

  - Rel-17 PDCCH skipping with two durations as performance reference provides

    - For high load, power saving gain of 10.4% and capacity gain of -19.9%

    - For low load, power saving gain of 11.2% and capacity gain of 7.2%

  - enhanced PDCCH skipping with arbitrary skipping duration covering the remaining DRX active time provides

    - For high load, power saving gain of 10.5% and capacity gain of -19.6%

    - For low load, power saving gain of 11.2% and capacity gain of 8.0%

- For FR1, DL-only evaluation, DU, DRX not configured, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from Huawei that

  - PDCCH skipping with adaptive duration provides

    - power saving gain of 18.35%

    - capacity gain of 0.00%

  - R17 PDCCH skipping performance reference provides

    - mean power saving gain of 11.64% in the range of 11.15% to 12.12% and

    - capacity gain of 0.00%

- For FR1, DL-only evaluation, DU, DRX not configured, high load, VR 45Mbps traffic at 60fps with 10ms PDB, it is observed from Xiaomi that

  - Rel-17 PDCCH skipping with 2 candidate durations as performance reference provides

    - power saving gain of 41.74%

    - capacity gain of -2.1%

  - PDCCH skipping enhancement with 4 candidate durations provides

    - power saving gain of 47.40% for all UEs

    - capacity gain of -1.0%

- For FR1, DL-only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that

  - go-to-sleep based on PDCCH skipping enhancement provides

    - power saving gain of 24.0% for all UEs

    - capacity gain of -6.1%

  - DG scheduling with C-DRX which provides

    - power saving gain of 8.0%

    - capacity gain of -6.1%

- For FR1, DL-only evaluation, InH, low load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from vivo that

  - enhanced PDCCH skipping provides

    - mean power saving gain of 17.71% in the range of 12.06% to 23.36%

    - mean capacity gain of 0.00%

  - R17 PDCCH monitoring provides

    - mean power saving gain of 12.56% in the range of 6.93% to 18.18%

    - mean capacity gain of 0.00%

- For FR1, DL-only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from vivo that

  - enhanced PDCCH skipping provides

    - mean power saving gain of 14.87% in the range of 10.46% to 19.28%

    - mean capacity gain of -0.23% in the range of -0.2% to -0.3%

  - R17 PDCCH monitoring provides

    - mean power saving gain of 9.96% in the range of 5.66% to 14.25%

    - mean capacity gain of -0.23% in the range of -0.2% to -0.3%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |
| Nokia1 | For evaluations that were carried without C-DRX, is it still assumed that UE would e.g. measure and report CSI during the PDCCH skipping duration? |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| vivo | We support PDCCH skipping duration enhancements. The reasons are as follows. - When PDCCH skipping indication is used without CDRX configuration, due to the existence of jitter, the interval between the end time of a previous frame transmission and the arrival time of the next frame is uncertain, it may be difficult to indicate an accurate skipping duration by the existing candidates of PDCCH skipping duration. - Additional RRC candidate duration values or indication bits of DCI can be adopted for the PDCCH skipping duration enhancements, which has very limited spec impact. |
| MTK | Support. NW instructs UE to skip to a predefined time stamp to match XR traffic period seems reasonable to us. |
| LGE | Support. Especially we think PDCCH monitoring adaptation in connected mode should be discussed to be applied to the UE not being configured with the CDRX. |
| ZTE, Sanechips | We don’t think it is valid assuming there’s no CDRX but PDCCH skipping, in current network. Besides, we agree that more PDCCH skipping durations can provide more flexibility to PDCCH skipping. |

Non-scheduling DCI based PDCCH skipping indication and Continuous PDCCH skipping

CATT proposed the non-scheduling DCI based PDCCH skipping indication to reduce unnecessary PDCCH monitoring (due to the variation of inter-arrival time caused by delay jitter from network transport) when no XR data is scheduled for transmission. Evaluation results can be found in Table 12. Intel, CMCC, NEC, Google, InterDigital, DOCOMO also proposed non-scheduling DCI based PDCCH monitoring adaptation.

CATT proposed the continuous PDCCH skipping. gNB configures a short PDCCH skipping duration and UE continuously skips the PDCCH MOs until the DCI is successfully decoded at the time of packet arrival. Evaluation results can be found in Table 12

Table 12: Evaluation results for non-scheduling DCI based PDCCH skipping indication

| Company | Evaluation results |
| --- | --- |
| CATT | Proposal 1: Rel-17 PDCCH skipping adaptation should be enhanced to customize for periodic XR traffic arrival with network delay jitter in reducing unnecessary PDCCH monitoring before XR data arrival by using the non-scheduling DCI based PDCCH skipping. Figure 1: The procedure of the PDCCH skipping with 2 bits indication in non-scheduling DCI Proposal 2: The extension of non-scheduling DCI format design could reuse the existing DCI format 1_1 in Rel-16 and dose not increase the size of DCI format with additional function in extending the PDCCH monitoring adaptation in PCell without introducing additional information field. Proposal 3: PDCCH skipping enhanced by non-scheduling DCI can reduce unnecessary PDCCH monitoring with little specification changes for the late arrival of periodic XR traffic at the beginning of PDCCH monitoring at each cycle. Table 1: PDCCH skipping enhancement schemes Schemes; Procedure. Baseline 1：Always-on; PDCCH monitoring is based on the configured PDCCH monitoring cycle of the search space.. Baseline 2：C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme; When the XR packet transmission is completed, UE would be indicated by scheduling DCI to skip an interval of PDCCH skipping and return to normal PDCCH monitoring afterward.. PDCCH skipping enhancement scheme 1 with C-DRX(16, 12, 4) (non-scheduling and scheduling DCI with persistent skipping indication); UE can be indicated by both scheduling and non-scheduling DCI to skip the indicated interval of PDCCH monitoring occasions persistently. The skipping duration is persistent without termination of PDCCH skipping until a new skipping indication is received.. Table 2: Evaluation results of PDCCH skipping schemes compared to always-on with MU-MIMO scheduling PDCCH monitoring Schemes; Considered UE set; Mean PSG compared to always-on; #satisfied UEs per cell. Baseline 1: Always-on; -; -; 11.5. PDCCH skipping enhancement scheme 1 (non-scheduling and scheduling DCI with persistent skipping indication); All UEs; 22.4%; 10.7. Satisfied UEs; 22.4%. PDCCH skipping enhancement scheme 2 (go-to-sleep); All UEs; 24.0%; 10.8. Satisfied UEs; 24.4%. PDCCH skipping enhancement scheme 3 (Continuous PDCCH skipping and dynamic go-to-sleep indication); All UEs; 29.4%; 10.7. Satisfied UEs; 29.9%. Table 3: Evaluation results of PDCCH skipping schemes compared to C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme and MU-MIMO scheduling Schemes; Considered UE set; Mean PSG compared to C-DRX(16,12,4); #satisfied UEs per cell. Baseline: C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme; -; -; 10.8. PDCCH skipping enhancement scheme 1 with C-DRX(16, 12, 4) (non-scheduling and scheduling DCI with persistent skipping indication); All UEs; 15.7%; 10.7. Satisfied UEs; 15.7%. PDCCH skipping enhancement scheme 2 with C-DRX(16, 12, 4) (go-to-sleep); All UEs; 17.4%; 10.8. Satisfied UEs; 17.8%. PDCCH skipping enhancement scheme 3 with C-DRX(16, 12, 4) (Continuous PDCCH skipping and dynamic go-to-sleep indication); All UEs; 23.3%; 10.7. Satisfied UEs; 23.8%. Observation 2: From the results in Table 3, it could be observed that: - The non-scheduling DCI based PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain 15.7% power saving gain with negligible capacity degradation compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme. - The go-to-sleep indication scheme with C-DRX(16, 12, 4) can obtain 17.4%~17.8% power saving gain with no capacity loss compared with C-DRX(16,12,4) with Rel-17 PDCCH skipping scheme. - The go-to-sleep indication and continuous PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain up to 23.8% power saving gain without capacity loss compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme. Proposal 6: The adaptation indication for PDCCH skipping should be separate between XR and other indicated to avoid different services latency requirements. Proposal 7: The Rel-17 PDCCH skipping should be enhanced with following solution: - PDCCH skipping indicated by non-scheduling DCI : - It can obtain 15.7% power saving gain with negligible capacity degradation compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme. - This solution has a small impact in terms of specification changes and can be easily enhanced to reuse the existing information field of DCI format 1_1. - The persistent PDCCH skipping scheme : - It can also achieve the same power saving and capacity performance as the non-scheduling DCI based PDCCH skipping scheme. Moreover, it can avoid frequent skipping indication signal overhead. - This solution can be easily enhanced to the PDCCH skipping scheme with small specification changes. - The XR-specific go-to-sleep indication by DCI: - It can obtain 17.4%~17.8% power saving gain with no capacity loss compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme. - This solution can be enhanced by XR-specific DCI with no impact to other traffic, which can be easily achieved by introducing a new RNTI, e.g., XR-RNTI, with small impact to specification. - The separate PDCCH skipping indication: - The separate PDCCH skipping indication scheme can dynamic adjust skipping duration to different services latency requirements. - This solution can be simply achieved by mapping different skipping durations to different intervals in scheduling DCI. |

### 3.3.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL-only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that the

  - non-scheduling DCI based PDCCH skipping scheme can obtain 22.4% power saving gain with compared with those of always-on. The go-to-sleep indication scheme can obtain 24.0%~24.4% power saving gain with capacity compared with those of always-on.

  - go-to-sleep indication and continuous PDCCH skipping scheme can obtain up to 29.9% power saving gain compared with those of always-on.

  - non-scheduling DCI based PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain 15.7% power saving gain with negligible capacity degradation compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme.

  - go-to-sleep indication scheme with C-DRX(16, 12, 4) can obtain 17.4%~17.8% power saving gain with no capacity loss compared with C-DRX(16,12,4) with Rel-17 PDCCH skipping scheme.

  - go-to-sleep indication and continuous PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain up to 23.8% power saving gain without capacity loss compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme.

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| MTK | Not support. Using a dummy scheduling DCI to indicate PDCCH skipping in current spec can achieve similar purpose as this proposal. |
| LGE | Support. Non-scheduling DCI that indicates PDCCH monitoring adaptation can be helpful for XR power saving to prevent a UE to receive PDCCH and dummy scheduled PDSCH. |

SSSG switching enhancements

Ericsson proposed to enhance SSSG switching to address jitter without increasing signaling overhead: (a) an implicit SSSG applies at the start of drx-OnDuration and another SSSG applies when a PDCCH for data traffic is received. (b) align the search space set monitoring pattern w.r.t. the DRX cycle. Evaluation results can be found in Table 13. Huawei also proposed that at the beginning of each On-duration, UE switches to a SSSG in which UE monitors PDCCH sparsely.

Table 13: Evaluation results for SSSG switching enhancements

| Company | Evaluation results |
| --- | --- |
| Ericsson | Observation 11 Enhanced SSSG switching achieves up to 19.3% PSG, while also achieving a rather high percentage of satisfied UEs (77.8–100%). Observation 12 Enhanced SSSG switching combined with PDCCH skipping achives up to 27.1% PSG, while also achieving a rather high percentage of satisfied UEs (78.1–100%). Proposal 10 Enhance SSSG switching to address jitter without increasing signaling overhead: (a) an implicit SSSG applies at the start of drx-OnDuration and another SSSG applies when a PDCCH for data traffic is received. (b) align the search space set monitoring pattern w.r.t. the DRX cycle. Table 2: Results for FR1, high load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 91.7%; -; -. R1-2210922; R17 SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 53.9%; 15.5%; 16.0%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 78.4%; 16.4%; 16.5%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 78.9%; 20.2%; 20.4%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 77.8%; 15.2%; 15.5%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 77.8%; 17.1%; 17.2%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 78.7%; 18.0%; 18.2%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 78.1%; 20.7%; 20.9%. Table 3: Results for FR1, low load, Dense Urban scenario, and VR single-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; L; 2; 8; 100%. R1-2210922; R17 SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 82.9%; 18.2%; 18.2%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 19.2%; 19.2%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 26.6%; 26.6%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 17.3%; 17.3%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 19.3%; 19.3%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 23.5%; 23.5%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; L; 2; -; 100%; 27.1%; 27.1%. Table 4: Results for FR1, high load, Dense Urban scenario, and VR multi-stream traffic: DL video (60 fps, 30 Mbps, ±4 ms jitter, 10 ms PDB), DL audio (10 ms periodicity, 30 ms PDB), UL pose (4 ms periodicity, 10 ms PDB) Tdoc #; Power Saving Scheme; CDRX cycle (ms); ODT (ms); IAT (ms); Load H/L; avg # UEs/Cell; floor (Capacity); % of satisfied UE; Mean PSG of all UEs; Mean PSG of satisfied UEs. R1-2210922; Always On; -; -; -; H; 8; 8; 90.1%; -; -. R1-2210922; R17 SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 66.0%; 10.3%; 10.3%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 79.8%; 10.7%; 10.7%. R1-2210922; R17 SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 79.9%; 11.3%; 11.2%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 2 ms on / 2 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 80.0%; 9.7%; 9.7%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 79.5%; 11.5%; 11.5%. R1-2210922; Enhanced SSSG switching (sparse SSSG: 1 ms on / 1 ms off) & PDCCH skipping & matched CDRX; 16.6 (drx_offset=3, traffic_time_offset=2 ms, drx-LongCycle=16 ms); 10; 4; H; 8; 78.7%; 12.2%; 12.1%. |

### 3.4.1 Discussion 1

Based on companies’ performance evaluations, the following observations are made.

Observation

- For FR1, DL + UL joint evaluation, DU, VR 30Mbps traffic at 60fps with 10ms PDB and DL audio, it is observed from Ericsson that

  - R17 SSSG switching performance reference provides

    - mean power saving gain of 10.50% in the range of 10.3% to 10.7%

    - mean capacity gain of -19.05% in the range of -26.7% to -11.4%

  - enhanced SSSG switching provides

    - mean power saving gain of 10.60% in the range of 9.7% to 11.5% for all UEs

    - mean capacity gain of -11.5% in the range of -11.8% to -11.2%

  - R17 SSSG switching + PDCCH skipping performance reference provides

    - power saving gain of 11.3%

    - capacity gain of -11.3%

  - enhanced SSSG switching + PDCCH skipping provides

    - mean power saving gain of 12.2%

    - capacity gain of -12.7%

- For FR1, DL only evaluation, DU, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from Ericsson that

  - enhanced SSSG switching provides

    - mean power saving gain of 16.15% in the range of 15.2% to 17.1% for all UEs

    - capacity gain of -15.2%

  - R17 SSSG switching performance reference provides

    - mean power saving gain of 15.95% in the range of 15.5% to 16.4%

    - mean capacity gain of -27.85% in the range of -41.2% to -14.5%

  - enhanced SSSG switching + PDCCH skipping provides

    - mean power saving gain of 19.35% in the range of 18.0% to 20.7%

    - mean capacity gain of -14.5% in the range of -14.8% to -14.2%

  - R17 SSSG switching + PDCCH skipping performance reference provides

    - power saving gain of 20.2%

    - capacity gain of -14.0%

- For FR1, DL only evaluation, DU, low load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from Ericsson that

  - enhanced SSSG switching provides

    - mean power saving gain of 18.30% in the range of 17.3% to 19.3% for all UEs

    - capacity gain of 0%

  - R17 SSSG switching performance reference provides

    - mean power saving gain of 18.7% in the range of 18.2% to 19.2%

    - capacity gain of 0%

  - enhanced SSSG switching + PDCCH skipping provides

    - mean power saving gain of 25.30% in the range of 23.50% to 27.10%

    - capacity gain of 0%

  - R17 SSSG switching + PDCCH skipping performance reference provides

    - power saving gain of 26.6%

    - capacity gain of 0%

Question 1: Do you have any comments on proponent companies’ proposals and observations?

| Company | Views |
| --- | --- |

Question 2: Do you support the proponent companies’ proposals or not, and why?

| Company | Views |
| --- | --- |
| MTK | Support. Using an implicit SSSG applies at the start of drx-OnDuration and another SSSG applies when a PDCCH for data traffic is received seems reasonable to match XR traffic period and handle jitter issue. |
| LGE | Support. Implicit SSSG switching can be helpful for UE’s power saving by addressing jitter issue. |

# Orders to discuss the proposals:

Please indicate your preferred order for the discussion of proposals in section 2.1, 2.2, 2.3, 2.4, 2.5, 2.6, 2.7. 2.8, 2.9, 2.10, 3.1, 3.2, 3.3, 3.4. (i.e., which section first and which section second…, e.g., 2.3, 3.1, 2.4, 2.5 …)

| Company | Views |
| --- | --- |
| vivo | We think a general principle to have the discussion among the proposed schemes is based on interested companies and contribution driven. In this regard, we suggest the following order for focusing discussion. - section 2.1, 3.1, 3.2 - section 2.3, 2.7, 2.2, 2.5 - other sections |
| MTK | We think proposals with more evident RAN1 spec impact can be discussed first. Ex. 3.1~3.4, 2.2, 2.4, 2.8, 2.9. |
| ZTE, Sanechips | I think the moderator has the judgement to identify which are high priority proposals, based on previous discussion and the evaluation results. For example, section 2.3, 2.2, 3.1, 3.2… |

# Capturing Performance Results in TR

FL proposes to capture companies’ evaluations results that comply with evaluation methodology from companies in the TP.

FL Proposal: capture all power saving gain performance results submitted companies in the Rel-18 XR SI TR

- For enhanced CDRX with semi-static periodicity alignment with XR traffic, performance results from Ericsson in R1-2210922, vivo in R1-2211024, OPPO in R1-2211490, Intel in R1-2211389, ZTE in R1-2211905, Qualcomm in R1-2212134, MediaTek in R1-2212253, CATT in R1-2211174 and Huawei in R1-2210906

- For enhanced CDRX with dynamic periodicity alignment with XR traffic, performance results from Nokia R1-2211551, ZTE in R1-2207061 and Qualcomm in R1-2212134

- For larger jitter range, performance results from vivo in R1-2211024

- For non-uniform PDCCH monitoring occasions within CDRX On Duration, performance results from Huawei in R1-2210906

- For two-stage CDRX On Duration, performance results from Ericsson in R1-2210922, vivo in R1-2211024

- For jitter handling by LP-WUS, performance results from vivo in R1-2211024, Xiaomi in R1-2211341

- For early stopping of ODT based on expiration of IAT, performance results from Xiaomi in R1-2211341, Huawei in R1-2210906, MediaTek in R1-2212253

- For additional DRX active time, performance results from ZTE in R1-2211905, Nokia in R1-2209535, OPPO in R1-2211490, vivo in R1-2211024

- For multiple active CDRX configurations, performance results from Ericsson in R1-2210922, InterDigital in R1-2211842, vivo in R1-2211024

- For Dynamic grant enhancement with XR-specific Pre-scheduling, performance results from CATT in R1-2211174

- For PDCCH skipping and interaction with HARQ retransmission, performance results from vivo in R1-2211024, MediaTek in R1-2212253, ZTE in R1-2211905

- For enhancements to PDCCH skipping duration, performance results from Huawei in R1-2210906, vivo in R1-2211024, Xiaomi in R1-2211341, CATT in R1-2211174, Ericsson in R1-2208401

- For non-scheduling DCI based PDCCH skipping, performance result from CATT in R1-2211174

- For continuous PDCCH skipping, performance result from CATT in R1-2211174

- For SSSG switching enhancements, performance result from Ericsson in R1-2210922

- For DCP indicated SSSG switching, performance results from Nokia in R1-2209535

- For retransmission-less CG for UL pose transmission, performance results from Qualcomm in R1-2212134

- For XR-application awareness of UE playout buffer, performance results from CATT in R1-2211174

- For partial UL transmission, performance results from Qualcomm in R1-2212134

# Proposals for online (Tuesday)

What was done

- FL collected evaluation results for each proposal and calculated mean and range of power saving gain and capacity gain w.r.t. AlwaysOn, for both the proposed solution and performance reference in observations.

- Companies (both proponents and opponents) provided rationales for supporting and not supporting a proposal

- Companies expressed their preference for the order of proposals for online discussions

Proposal A.B[-C] below corresponds to the proposal and discussions in Section A.B in Section 2 and 3. “Support” and “Not support” of a proposal is counted based on companies’ Tdocs and offline input to FL summary.

Proposal 3.1: support PDCCH monitoring resume if UE transmits NACK after PDCCH skipping starts

- Support: Ericsson, vivo, MTK, Huawei, OPPO, ETRI, NEC, LGE, Samsung, III

  - Capacity loss and unnecessary PDCCH monitoring are reduced by enabling long skipping duration with stringent latency requirement of XR

  - Less delay than MAC based termination of PDCCH monitoring within DRX active time

- Not support: ZTE, CATT, Nokia, Intel

  - In some cases, marginal power saving gain is observed for the PDCCH skipping enhancement

- Performance comparison:

  - The proposed solution has

    - mean power saving gain of 22.06%

    - mean capacity gain of -11.59%

  - Rel17 PDCCH skipping as performance reference has

    - mean power saving gain of 11.28%

    - mean capacity gain of -18.65%

For Proposal 3.1, the following conclusion was made in Tuesday’s online session

Conclusion No consensus on the following: Support PDCCH monitoring resume if UE transmits NACK after PDCCH skipping starts Nokia, CATT, ZTE, Intel expressed concerns on the benefit of the above proposal

Proposal 3.2: support PDCCH skipping duration enhancements by i) additional PDCCH skipping durations (>3), ii) skipping till the start of next potential data arrival

- Support: Huawei, vivo, Xiaomi, CATT, MTK, LGE, Google

  - Seamless PDCCH skipping between data arrivals with minor spec impact

- Not support: Ericsson, Nokia, [ZTE, Alt2]

  - No power saving gain when CDRX is configured

  - CDRCX should be configured if PDCCH skipping is enabled

- Performance comparison:

  - The proposed solution has

    - mean power saving gain of 20.58%

    - mean capacity gain of -2.70%

  - Rel17 PDCCH skipping as performance reference has

    - mean power saving gain of 15.07%

    - mean capacity gain of -3.02%

For Proposal 3.2, the following conclusion was made in Tuesday’s online session

Conclusion No consensus on the following: Support PDCCH skipping duration enhancements by i) additional PDCCH skipping durations (>3), ii) skipping till the start of next potential data arrival - Applies at least for the case where CDRX is not configured Ericsson, Nokia, ZTE expressed concerns on the benefit of the above proposal

Proposal 2.3: support additional active time by 1) UE extending DRX active time if UE does not receive XR data within current active time, 2) gNB using dynamic signaling such as a DCI to trigger additional On Duration if the data packet arrives after the On Duration expires

- Support: ZTE, Nokia, OPPO, CMCC, ETRI, LGE, Google (Alt2)

  - It helps a UE to keep DRX active time as short as possible

  - If PDCCH skipping is not supported by UEs, the CDRX based solutions become more practical.

  - DCI triggered extension is more flexible than dummy grant to provide extension time longer than IAT

  - Likely to be agreed in RAN2

- Not support: vivo, MTK, Ericsson, Samsung, Intel, CATT, Huawei

  - UE may fail to extend the active time if non-XR traffic data is scheduled before XR video data arrival

  - DCI triggered active time extension can be achieved by R15/16 CDRX with dummy grant

  - No performance gain compared with existing R17 PDCCH monitoring adaptation

- Performance comparison:

  - The proposed solution has

    - mean power saving gain of 22.89% in the range of 14.18% to 33.50%

    - mean capacity gain of -11% in the range of -42.44% to 0%

  - eCDRX as performance reference has

    - mean power saving gain of 14.82% in the range of 4.50% to 23.36%s

    - mean capacity gain of -1% in the range of -1.6% to 0%

For Proposal 2.3, the following conclusion was made in Tuesday’s online session

Conclusion No consensus on the following: Support additional active time by 1) UE extending DRX active time if UE does not receive XR data within current active time, 2) gNB using dynamic signaling such as a DCI to trigger additional On Duration if the data packet arrives after the On Duration expires

Proposal 2.2: support dynamic periodicity alignment between CDRX and XR traffic

- Support: ZTE, Nokia, Qualcomm, China Telecom, Sony, Lenovo, Panasonic, Rakuten Symphony, Xiaomi, CMCC, ETRI, LGE, InterDigital, OPPO, and DOCOMO

  - Potentially more robust CDRX configuration adaptive to parameter change of XR services

- Not support: vivo, MTK, Intel, Google, Ericsson, CATT, Samsung, Huawei

  - Semi-static method for periodicity alignment is sufficient. DCI-based CDRX adjustment has no additional benefit and will introduce more control signalling overhead and spec impact.

  - The use case where centre point of XR traffic arrival to RAN could be drifted and whether the core network can provide the range and frequency of the drift to RAN RAN are not justified yet.

- Performance comparison: (Intel: separately capture QC result for PDCCH skipping)

  - The proposed solution has

    - mean power saving gain of 30% in the range of 15.00% to 74.60%

    - mean capacity gain of -36.83% in the range of -73.7% to -2.2%

  - eCDRX as performance reference has

    - mean power saving gain of 21.22% in the range of 0% to 45.50%

    - mean capacity gain of -60.63% in the range of -100% to -2.2%

For Proposal 2.2, the following conclusion was made in Tuesday’s online session

Conclusion No consensus on the following: Support dynamic periodicity alignment between CDRX and XR traffic

Proposal 2.4: support non-uniform PMOs within CDRX On Duration

- Support: Huawei, MTK

  - Higher power saving gain (6.7%) can be achieved than sparse PDCCH monitoring

- Performance comparison:

  - non-uniform PMOs within On Duration provides

    - mean power saving gain of 18.49% in the range of 16.23% and 20.75% for all UEs

    - mean capacity gain of -6.15% in the range of -8.3% to -4.0%

  - sparse PDCCH monitoring as the performance reference provides

    - mean power saving gain of 11.73% in the range of 7.64% to 15.81% for all UEs

    - mean capacity gain of -5.10% in the range of -8.1% to -2.1%

Proposal 2.5: support two-stage CDRX On Duration

- Support: Ericsson, LGE, Sony, Google

  - Can control not only PDCCH monitoring but also CSI reporting

- Not support: vivo, MTK

  - Two-stage DRX can be realized by sparse PDCCH monitoring followed by SSSG switching

  - Similar performance gain to Rel-17 PDCCH monitoring adaptation

  - This has only RAN2 design, should be left to RAN2

- Performance comparison:

  - Two-stage On Duration provides

    - mean power saving gain of 17.58% in the range of 4.80% to 27.00% for all UEs

    - mean capacity gain of -5.97% in the range of -14.30% to 0%

  - sparse PDCCH monitoring as the performance reference provides

    - mean power saving gain of 18.88% in the range of 10.30% to 26.60% for all UEs

    - mean capacity gain of -9.54% in the range of -41.20% to 0%

Proposal 2.7: multiple active CDRX configurations

- Support: Ericsson, TCL, Intel, ZTE, Sony, Lenovo, Xiaomi, ETRI, NEC, LGE, Apple, Rakuten Symphony, InterDigital and DOCOMO

  - Can serve XR multiple traffic flows well, especially for 30fps DL video + DL audio

- Not support:

  - CDRX + SPS can achieve similar power saving and capacity performance

- Performance comparison 1: use single CDRX as reference

  - The proposed solution has

    - mean power saving gain of 13.42% in the range of 10.1% to 18.09%

    - mean capacity gain of -9.74% in the range of -21.3% to -4.7%

  - eCDRX as performance reference has

    - mean power saving gain of 11.14% in the range of 6.22% to 18.4%

    - mean capacity gain of -52% in the range of -100% to 0%

- Performance comparison 2: use single CDRX + SPS as reference

  - The proposed solution has

    - mean power saving gain of 13.51% in the range of 6.95% to 19.84%

    - mean capacity gain of -0.43% in the range of -1.2% to 0%

  - eCDRX + SPS as performance reference has

    - mean power saving gain of 14.42% in the range of 8.14% to 20.70%

    - mean capacity gain of -0.25% in the range of -0.5% to 0%

Proposal 2.6: support early stopping of On-Duration timer by i) inactivity timer expiration ii) after a time window after the reception of XR data

- Support: Huawei, Xiaomi and MediaTek, Rakuten Symphony, DOCOMO, III and Qualcomm

  - simple solution that can effectively handle the jitter

- Not support: ZTE, Nokia, vivo

  - MAC-CE “go-to-sleep” and PDCCH skipping can achieve similar effect

- Performance comparison:

  - The proposed solution has

    - mean power saving gain of 23.33% in the range of 10.22% to 43.18%

    - mean capacity gain of -3.33% in the range of -5.6% to -2.1%

  - eCDRX as performance reference has

    - mean power saving gain of 9.64% in the range of 7.64% to 11.39%

    - mean capacity gain of -0.30% in the range of -2.1% to 1.5%

Proposal 3.4: support an implicit SSSG at the start of drx-OnDuration and another SSSG applies when a PDCCH for data traffic is received, with search space set monitoring pattern aligned with DRX cycle

- Support: Ericsson, Huawei, MTK, LGE

  - Implicit SSSG with sparse PMOs is helpful for UE power saving

- Performance comparison 1: without PDCCH skipping

  - The proposed solution has

    - mean power saving gain of 15.02%

    - mean capacity gain of -8.90%

  - Rel17 SSSG switching as performance reference has

    - mean power saving gain of 15.05%

    - mean capacity gain of -15.63%

- Performance comparison 2: with PDCCH skipping

  - The proposed solution has

    - mean power saving gain of 18.95%

    - mean capacity gain of -9.07%

  - Rel17 SSSG switching as performance reference has

    - mean power saving gain of 19.37%

    - mean capacity gain of -8.43%

Proposal 2.8: support dynamic grant enhancement with XR-specific pre-scheduling

- Support: CATT, MTK

  - Can align PDCCH monitoring window with XR traffic period

- Performance comparison:

  - For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that

    - semi-static C-DRX enhancement scheme with (17/17/16, 8, 4) obtains 25.5% power saving gain with none satisfied UE, the dynamic grant enhancement with XR-specific pre-scheduling scheme could obtain the less than 10% capacity performance gap than that of the UE always-on for DG scheduling and obtain 11.7%~29.4% PSG compared to that of the UE always-on for DG scheduling.

    - Under the similar capacity performance,  while the semi-static C-DRX enhancement scheme with (17/17/16, 8, 4) provides capacity loss of 100%, the Dynamic grant enhancement with XR-specific  pre-scheduling could obtain less than 10% capacity performance degradation than that of the UE always-on for DG scheduling and obtain 15.7%~23.3% PSG compared to that of DG scheduling with C-DRX(16, 12, 4) and Rel-17 PDCCH skipping scheme.

  - For FR1, DL only evaluation, InH, high load, multi-carrier, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that

    - power saving gain of enhanced C-DRX (16/17/17, 8, 4) are 19.0% and 27.4% for carrier with XR transmission and carrier with IM transmission, respectively.

    - Dynamic grant enhancement with XR-specific pre-scheduling scheme for XR traffic carrier and IM traffic carrier can obtain 23.3% and 87.2% power saving gain, respectively.

Proposal 2.9: support SPS+DG with UE power saving scheme

- Support: CATT

- Not support: MTK

  - SPS is not suitable for XR DL traffic, considering the large frame size variation.

- Performance comparison:

  - For FR1, DL only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that

    - SPS +DG with power saving schemes can obtain the capacity performance of 10.8 UEs per cell and 9.9%~38.1% power saving gain compared to that of DG scheduling with C-DRX(16, 12, 4)

    - multiple SPS configurations provide 0% of satisfied UE.

Proposal 3.3-1: support non-scheduling DCI based PDCCH skipping indication

- Support: CATT, LGE

  - Can avoid UE to reive PDCCH and dummy scheduled PDSCH

- Not support: MTK

  - Using a dummy scheduling DCI to indicate PDCCH skipping in current spec can achieve similar purpose

Proposal 3.3-2: support continuous PDCCH skipping, i.e., UE continuously skips the PDCCH MOs until the DCI is successfully decoded at the time of packet arrival

- Support: CATT

Performance comparison:

- For FR1, DL-only evaluation, InH, high load, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that the

  - non-scheduling DCI based PDCCH skipping scheme can obtain 22.4% power saving gain with compared with those of always-on. The go-to-sleep indication scheme can obtain 24.0%~24.4% power saving gain with capacity compared with those of always-on.

  - go-to-sleep indication and continuous PDCCH skipping scheme can obtain up to 29.9% power saving gain compared with those of always-on.

  - non-scheduling DCI based PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain 15.7% power saving gain with negligible capacity degradation compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme.

  - go-to-sleep indication scheme with C-DRX(16, 12, 4) can obtain 17.4%~17.8% power saving gain with no capacity loss compared with C-DRX(16,12,4) with Rel-17 PDCCH skipping scheme.

  - go-to-sleep indication and continuous PDCCH skipping scheme with C-DRX(16, 12, 4) can obtain up to 23.8% power saving gain without capacity loss compared with C-DRX(16, 12, 4) with Rel-17 PDCCH skipping scheme.

Proposal 2.10: Support XR-specific plyaoutDelayForMediaStartup for XR UE power saving enhancement

- Support: CATT, MTK

  - This technique results in a more accurate UE buffer status report with additional timing (delay) information

- Performance comparison

  - For FR1, DL only, InH, VR 30Mbps traffic at 60fps with 10ms PDB, it is observed from CATT that with the awareness of UE XR-specific playoutDelayForMediaStartup, UE power saving gain of 26.43% to 28.51% can be achieved with 67% capacity gain.

# Proposals for online (Thursday)

Conclusion: There is no consensus on the following proposals in Table 1 for CDRX enhancements and Table 2 for PDCCH monitoring enhancements.

- Note: companies who support or object each proposal in RAN1 discussions during the SI are recorded in Table 1 and Table 2

Table 1: CDRX enhancements

| Proposal for CDRX enhancements | Support | Not support |
| --- | --- | --- |
| Proposal 2.2: support dynamic periodicity alignment between CDRX and XR traffic | Qualcomm, InterDigital, Dell, Nokia, Panasonic | CATT, Google, Huawei/HiSilicon, vivo, Samsung, Intel, MTK, Ericsson |
| Proposal 2.3-1: support additional active time by UE extending DRX active time if UE does not receive XR data within current active time | LGE, InterDigital, Dell, ZTE/Sanechips, TCL, Nokia | CATT, Qualcomm, Lenovo, Google, Huawei/HiSilicon,vivo, Spreadtrum, Samsung, Xiaomi , intel, Panasonic, MTK, Ericsson |
| Proposal 2.3-2: support additional active time by gNB using dynamic signaling such as a DCI to trigger additional On Duration if the data packet arrives after the On Duration expires | LGE, InterDigital, Google, Dell, ZTE/Sanechips，OPPO, TCL, CMCC, Spreadtrum | CATT, Qualcomm, Lenovo, Huawei/HiSilicon,vivo, Samsung, Xiaomi, Intel, Panasonic, MTK, Ericsson |
| Proposal 2.4: support non-uniform PMOs within CDRX On Duration | Huawei/HiSilicon, TCL, MTK | CATT, Qualcomm, InterDigital, Google, Dell, vivo, Spreadtrum, Samsung, Xiaomi, Nokia, Intel, Panasonic, Ericsson |
| Proposal 2.5: support two-stage CDRX On Duration | LGE, InterDigital, Google, Dell, Ericsson | CATT, Qualcomm, Huawei/HiSilicon, ZTE/Sanechips, TCL, vivo, Spreadtrum, Samsung, Xiaomi, Nokia, Intel, Panasonic, MTK |
| Proposal 2.6-1: support early stopping of On-Duration timer by inactivity timer expiration | Huawei/HiSilicon, Xiaomi, MTK | CATT, InterDigital, Lenovo, Google, Dell, vivo, TCL, Spreadtrum, Samsung, Nokia, Intel, Panasonic, Ericsson |
| Proposal 2.6-2: support early stopping of On-Duration timer after a time window after the reception of XR data | Qualcomm, Xiaomi, MTK | CATT, InterDigital, Lenovo, Google, Dell, Huawei/HiSilicon, vivo, TCL, Spreadtrum, Samsung, Nokia, Intel, Panasonic, Ericsson |
| Proposal 2.7: support multiple active CDRX configurations | LGE, InterDigital, Lenovo, Google, Dell, ZTE/Sanechips，OPPO, TCL, Spreadtrum, NEC, Apple, Xiaomi, Intel, Ericsson | CATT, Qualcomm, Huawei/HiSilicon, Samsung, Nokia |
| Proposal 2.8: support dynamic grant enhancement with XR-specific pre-scheduling | CATT | Huawei/HiSilicon, Samsung, Nokia, Intel, Ericsson |
| Proposal 2.9: support SPS+DG with UE power saving scheme | CATT, Dell, Panasonic | Huawei/HiSilicon, Samsung, Intel, MTK, Ericsson |
| Proposal 2.10: Support XR-specific plyaoutDelayForMediaStartup for XR UE power saving enhancement | CATT | Huawei/HiSilicon, Samsung, Nokia, Intel, Ericsson |

Table 2: PDCCH monitoring adaptation enhancements

| Proposals for PDCCH monitoring adaptation enhancements | Support | Not support |
| --- | --- | --- |
| Proposal 3.1: support PDCCH monitoring resume if UE transmits NACK after PDCCH skipping starts | LGE, Qualcomm, Google, Dell, Huawei/HiSilicon, Apple, Samsung, vivo, Panasonic, MTK, Ericsson | CATT, InterDigital, ZTE/Sanechips, TCL, Nokia , Intel |
| Proposal 3.2-1: support PDCCH skipping duration enhancements by additional PDCCH skipping durations (>3) | CATT, Qualcomm, InterDigital, Lenovol, Google, Dell, ZTE/Sanechips, TCL, Spreadtrum, Samsung, Xiaomi, vivo, Intel | Huawei/HiSilicon, Nokia, MTK, Ericsson |
| Proposal 3.2-2: support PDCCH skipping duration enhancements by PDCCH skipping till the start of next potential data arrival | Dell, Huawei/HiSilicon, TCL, NEC, vivo, Panasonic, MTK | CATT, InterDigital, ZTE/Sanechips, Samsung, Nokia, Intel, Ericsson |
| Proposal 3.3-1: support non-scheduling DCI based PDCCH skipping indication | CATT, LGE, Qualcomm, Dell, Huawei/HiSilicon, CMCC, Spreadtrum, NEC, Intel, Panasonic | InterDigital, Google, Samsung, vivo, MTK, Ericsson |
| Proposal 3.3-2: support continuous PDCCH skipping, i.e., UE continuously skips the PDCCH MOs until the DCI is successfully decoded at the time of packet arrival | CATT, Dell,NEC | Qualcomm, InterDigital, Huawei/HiSilicon, Samsung, Xiaomi, vivo, Nokia, Intel, Ericsson |
| Proposal 3.4: support an implicit SSSG at the start of drx-OnDuration and another SSSG applies when a PDCCH for data traffic is received, with search space set monitoring pattern aligned with DRX cycle | LGE, Huawei/HiSilicon, NEC, MTK, Ericsson | CATT, Qualcomm, InterDigital, Lenovo, Google, Dell, Spreadtrum, Samsung, vivo, intel |

# Proposals for online (Friday)

Conclusion: Rel-18 XR SI is completed in RAN1 for power saving and capacity enhancements.

# Conclusions

# References

- RAN1 #109-e Chairman’s Notes, May 2022

- RAN1 #110 Chairman’s Notes, August 2022

- RAN1 #111bis-e Chairman’s Notes, October 2022

- R1-2211174, UE Power saving techniques for XR, CATT

- R1-2211246, Discussion on XR specific power saving techniques, Spreadtrum Communications

- R1-2211341, Discussions on techniques for XR Power Saving, xiaomi

- R1-2211389, Discussion on XR specific power saving techniques, Intel Corporation

- R1-2211490, Discussion on XR specific power saving techniques, OPPO

- R1-2211536, Discussion on XR specific power saving enhancement for NR, China Telecom

- R1-2211539, XR specific power saving techniques, TCL Communication Ltd.

- R1-2211551, XR-specific power saving enhancements, Nokia, Nokia Shanghai Bell

- R1-2211566, Discussion on XR-specific power saving techniques, ETRI

- R1-2211624, Discussion on XR-specific power saving techniques, Sony

- R1-2211653, Discussion on XR specific power saving techniques, Panasonic

- R1-2211697, Discussion on XR-specific power saving techniques, CMCC

- R1-2211752, Discussion on XR specific power saving techniques, NEC

- R1-2211781, XR-specific power saving techniques, Lenovo

- R1-2211826, On XR specific power saving techniques, Apple

- R1-2211842, Discussion on XR specific power saving techniques, InterDigital, Inc.

- R1-2211905, Evaluation on XR specific power saving techniques, ZTE, Sanechips

- R1-2211920, Further Discussion on XR power saving	, III

- R1-2212000, Discussion on XR specific power saving techniques, NTT DOCOMO, INC.

- R1-2212062, Considerations on Power Savings for XR, Samsung

- R1-2212134, Power saving techniques for XR, Qualcomm Incorporated

- R1-2212253, On XR specific power saving techniques, MediaTek Inc.

- R1-2212305, Discussion on XR-specific power saving techniques, LG Electronics

- R1-2212387, On XR-specific power saving techniques, Google Inc.

# Appendix

Objective of SI

Objective of the Rel-18 XR enhancements SI is as follows

The study is to be based on Release 17 TR 38.838, on corresponding Release 17 work from SA4 (as per SP-210043) and on Release 18 work from SA2 (as per SP-211166). Objectives on XR-awareness in RAN (RAN2): - Study and identify the XR traffic (both UL and DL) characteristics, QoS metrics, and application layer attributes beneficial for the gNB to be aware of. - Study how the above information aids XR-specific traffic handling. Objectives on XR-specific Power Saving (RAN1, RAN2): - Study XR specific power saving techniques to accommodate XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc...). Focus is on the following techniques: - C-DRX enhancement. - PDCCH monitoring enhancement. Objectives on XR-specific capacity improvements (RAN1, RAN2): - Study mechanisms that provide more efficient resource allocation and scheduling for XR service characteristics (periodicity, multiple flows, jitter, latency, reliability, etc…). Focus is on the following mechanisms: - SPS and CG enhancements; - Dynamic scheduling/grant enhancements.

RAN1 #109-e Agreements

Agreement For power saving study of Rel-18 XR SI, CDRX enhancements to evaluate in this study item are to be selected from the following: - High priority Issue 1-1: Alignment between CDRX and XR traffic for resolving the mismatch between CDRX cycle and XR traffic periodicity for each flow - High priority Issue 1-2: C-DRX enhancements to handle jitter - Medium priority Issue 1-3: CDRX enhancements for multiple XR traffic flows [Note 2] - Low priority Issue 1-4: CDRX enhancements to adjust to variable burst sizes and frame rate - Note: Some companies think the adjustment for variable burst sizes can be realized by existing spec already - Low priority Issue 1-5: low latency handling - Low priority Issue 1-6: SFN wraparound mismatch (if handled in RAN1) FFS: how the solutions or the combination of the solutions can handle all the identified issues. Note 1: Other considerations are not precluded Note 2: It can also be adopted for addressing issue 1-1 Note 3: Companies are encouraged to clarify or provide more details of the proposed solutions, for addressing concerns from the group. Additional details can be found in R1-2205411. Agreement For power saving study of Rel-18 XR SI, PDCCH monitoring enhancements to evaluate in this study item are to be selected from the following - Low priority Issue 2-1: Alignment between PDCCH monitoring and XR traffic to resolve the mismatch between PDCCH monitoring periodicity and XR traffic periodicity. - Note: some companies think Rel-17 PDCCH monitoring adaptation can solve issue 2-1 or achieve similar intended outcome - Note: Solutions proposed for Issue 2-1 and those proposed for Issue 1-1 are motivated by the same issue, namely non-integer XR traffic periodicity. It is to be studied how they compare in in terms of power saving gain and capacity, (a) solutions proposed for Issue 1-1; (b) solutions proposed for Issue 2-1. - Low priority Issue 2-2: XR-dedicated PDCCH monitoring window to supplement CDRX for multi-flow traffic. - Note: some companies think Rel-17 PDCCH monitoring adaptation can solve issue 2-2 or achieve similar intended outcome - Note: Solutions proposed for Issue 2-2 and those proposed for Issue 1-3 are motivated by the same issue, namely multiple XR traffic flows. It is to be studied how they compare in in terms of power saving gain and capacity, (a) solutions proposed for Issue 1-3; (b) solutions proposed for Issue 2-2. - High priority Issue 2-3: Enhancements to Rel-17 PDCCH monitoring adaptation. - Note: Discussion on some enhancements may depend on the outcome of Rel-17 PDCCH monitoring adaptation maintenance - Note: The study on enhancement to R17 PDCCH monitoring adaptation should focus on the techniques that are used for addressing XR-specific issues, e.g., jitter Note 1: Other considerations are not precluded Note 2: Companies are encouraged to clarify or provide more details of the proposed solutions, for addressing concerns from the group. Agreement For Rel-18 XR power saving enhancements, RAN1 further discusses by RAN1 #110 whether the issues below are to be addressed, and if so, which solutions should be selected for evaluation in this study item. These issues are low priority. - Issue 3-1: Misaligned UE transmission and reception. - Issue 3-2: Power saving by XR-aware scheduling. - Note 1b: XR SI objective has XR-awareness in RAN listed as a specific topic of RAN2 study - Issue 3-3: Unnecessary data transmission in allocated resources. Note 1: Rel-18 XR SI objective only has CDRX enhancements and PDCCH monitoring enhancements explicitly listed as focus of RAN1 study Note 2: Other considerations are not precluded Agreement Rel-17 evaluation methodology for XR power saving captured in TR 38.838 is used as the baseline evaluation methodology for UE power evaluation of Rel-18 SI on XR enhancements Agreement Companies are encouraged to compare performance of the following Rel-15/16/17 features with the proposed enhancements for Rel-18 XR power saving evaluations. Power saving gain is calculated w.r.t. the AlwaysOn baseline. - Rel-15/16 CDRX including long DRX cycle, short DRX cycle and DRX command MAC CE and DCP - Rel-17 PDCCH adaptation including PDCCH skipping and SSSG switching Note: up to companies to report the configuration of the Rel-15/16/17 features Conclusion - If no evaluation result is provided by any company for an issue, the issue is deprioritized. The issue and proposed enhancements for the issue will not be captured by RAN1 in TR 38.835. - If no evaluation result is provided by the proponent company for a proposed enhancement, the proposed enhancement is deprioritized. The proposed enhancement will not be captured by RAN1 in TR 38.835. - If multiple enhancement techniques are proposed for the same issue, there can be down selection among them for the consideration of candidate enhancement for study item recommendation by RAN1 at least based on performance (power saving and capacity), spec impact, signaling overhead and implementation complexity. - Companies are encouraged to provide detailed information for both the proposed enhancement and the existing power saving features used as the performance reference so that the evaluation results for both can be reproduced by other companies. - When using existing power saving features as the performance reference, companies are encouraged to configure the existing power saving features to achieve the best performance. - For evaluation of a proposed enhancement and evaluation of the existing power saving features as performance reference, companies are encouraged to provide the high load case (as defined in TR 38.838, Section A.2) results. Results for low load case can also be reported optionally.

RAN1 #110 Agreements

R1-2207831 Moderator Summary #1 on XR specific power saving technique Moderator (Qualcomm Incorporated) Conclusion Conclude that “SFN wraparound mismatch” is a RAN2 issue. It can be left to RAN2 to address. RAN1 does not further study it. Agreement RAN1 recommends identifying a solution for enhancement of CDRX to align with XR traffic periodicity Conclusion RAN1 does not assume instantaneous jitter value for a frame is predictable for Rel-18 XR SI power saving study before further input is provided by SA. R1-2207832 Moderator Summary #2 on XR specific power saving techniques Moderator (Qualcomm Incorporated) Conclusion All the proposed PDCCH monitoring adaptation/reduction schemes including those for jitter handling need to be compared against the Rel-17 PDCCH monitoring adaptation which is to be used as performance reference. Conclusion UE transmission and reception alignment for Issue 3-1 is deprioritized for power saving in Rel-18 XR SI. Conclusion RAN1 does not assume dynamic switch of different XR video data rates or frame rates for Rel-18 XR power saving study before further input is provided by SA. R1-2207833 Final Moderator Summary on XR specific power saving techniques Moderator (Qualcomm Incorporated) For future meetings Companies are encouraged to account the enhancement of CDRX to align with XR traffic periodicity in their further evaluations for XR power saving enhancements. Conclusion: - Companies are requested to use the Excel sheet attached with TR 38.838 in RP-213652 for recording the simulation results that are provided in their contributions.

RAN1 #110bis-e Agreements

R1-2210337 Moderator Summary#1 on XR specific power saving techniques Moderator (Qualcomm Incorporated) Agreement - For enhancement of CDRX to align with XR traffic periodicity (i.e., Issue 1-1) - Prioritize semi-static solutions - FFS: Whether dynamic solutions will be also needed R1-2210338 Moderator Summary#2 on XR specific power saving techniques Moderator (Qualcomm Incorporated) Conclusion “Retransmission-less CG for UL pose transmission (Item 3.3-5)” is a RAN2 issue, leave the discussion to RAN2, RAN1 does not further investigate the issue. - Note: how to capture evaluation results and findings will be separately discussed Conclusion RAN1 does not further study jitter handling by LP-WUS (Item 2.2-4) in Rel-18 XR SI - Note: how to capture evaluation results and findings will be separately discussed Conclusion In addition to the values for jitter in Table 5.1-2 in TR 38.838, the following statistical parameters for jitter can also be optionally evaluated in Rel-18 XR SI. - Note: This optional assumption is not applicable to the evaluation of 90 FPS and above Parameter; unit; Optional value for evaluation. Mean; ms; 0. STD; ms; 5. Truncation range; ms; [-8, 8]. Conclusion RAN1 deprioritizes DCP indicated SSSG switching (Item 3.3-4) - Note: how to capture evaluation results and findings will be separately discussed