3GPP TSG-RAN WG1 Meeting #112	Tdoc R1-2300178

February 27th – March 3rd , 2023

Agenda Item:	9.2.1

Source:	Ericsson

Title:	Discussion on general aspects of AIML framework

Document for:	Discussion, Decision

# 1	Introduction

The study item Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved in RAN#94e [1]. The study item will explore 3GPP frameworks to enable AI/ML including, for example, AI/ML model characterization, various levels of collaboration between UE and network, data sets for training/validation/testing/inference, and life cycle management. The study should quantify the performance, robustness, complexity, and potential specification impact of AI/ML based solutions.

This contribution concerns agenda item 9.2.1, general aspects of AI/ML framework for AI on PHY. We will discuss the defining stages for ML model life cycle management (LCM).

# 2	Discussion on Various Stages of Model Operation

## 2.1 	On proprietary versus 3GPP standardized solutions

In the previous meetings there were discussions about proprietary solutions (i.e., outside 3GPP) for model transfer and data collection. In our view, the task of this RAN1-lead study item is to investigate the feasibility and benefits of standardizing the use of AI/ML for L1/L2 features. Hence, RAN1 should not spend precious time units to discuss non-3GPP solutions.

It may turn out that the SI concludes that a fully 3GPP specified solution is a too demanding task. A conclusion may possibly be that 3GPP, for some components/defining stages of the AI/ML standardization, needs to turn to other standardization organizations to fulfil some tasks. 3GPP has done this previously, for example, MPEG and 3GPP jointly developed dynamic adaptive streaming over HTTP and published dual specifications in 3GPP TS 26.247 and ISO/IEC 23009-1. However, such decisions are clearly outside the scope of RAN1.

Instead RAN1 should define the 3GPP framework and focus on the necessary requirements for Uu signalling to enable data collection, AI/ML model LCM, training procedures and AI/ML model transfer. RAN1 can specifically focus on signalling payloads and time-related requirements such as latency and reliability. RAN2 can continue to discuss the mapping of such information to, for example, L1 based, MAC, RRC, MDT-based or other solutions.

3GPP should study the impact on 3GPP specifications for introducing AI/ML for PHY and not spend any time units on non-3GPP based solutions.

## 2.2	Functionality framework and model LCM

Numerous agreements have been made by RAN1 to study AI/ML for PHY model LCM aspects related to the high-level model LCM stages of data collection, model training, model deployment, model inference, and model monitoring. Furthermore, it has been agreed to study aspects/procedures related to e.g., model selection, model activation, model deactivation, model switching, model update, and model transfer. To study model LCM aspects, RAN1 could define a functional framework as shown in Figure 1 for anchoring discussions related to NW-UE collaborations in supporting AI/ML for PHY use cases with one- or two-sided models.

The outlined framework in Figure 1 is mainly applicable for model LCM assuming offline training, and it is expected that for online training several of the functionalities can be merged and some model LCM aspects captured in the figure may not be relevant. However, it is not clear what kind of specification impact that is needed for online training for one-sided models. We expect that one-sided models can be part of an AI/ML functionality described more in subsequent sections, where such functionality should fulfill certain requirements. If such requirements are fulfilled by applying online training, it would anyway be specification transparent.

Adopt the functional framework in Figure 1 for anchoring discussions related to NW-UE collaborations.

Prioritize model LCM relying on offline model training.

Figure 1. A functional framework for discussing model LCM aspects.

In case of one-sided models, the responsibility for model LCM is clearly on the side that implements functionality for making model inference, and NW-UE collaborations then basically refer to configuring transmissions and reports to assist in data collection for model training, inference, and monitoring. Such potential model LCM assistance from the NW should be studied and determined on a per use case basis.

For use cases with one-sided model, responsibility for model LCM is clearly on the side that implements functionality for making model inference. Study per use case basis the necessity of assistance for model LCM

In case of two-sided models, model LCM needs to consider that a gNB serves multiple UEs simultaneously and that switching models in real-time, or running multiple models in parallel, at the gNB side is not realistic. Hence, the following is proposed as a requirement when studying two-sided model LCM:

Conclude that two-sided model LCM must enable a gNB to operate with a single model that works with different UE-side models.

## 2.3 	Model transfer/deployment

### 2.3.1	On the need for model transfer

In a use case with one-sided model deployed at the UE-side, transferring a model trained by the NW with data from a specific scenario can result in a highly customized and efficient model. The key idea is that when the UE enters a specific scenario in the network (e.g. a subway), then the corresponding model is transferred to the UE from the NW, to optimize the performance of the AI/ML in the UE. Since the model in the UE is trained for the specific scenario, this type of model can be of low complexity and can further be updated to reflect changes in the local environment. Another benefit can be achieved if using open-format models since these can be broadcasted, and this will reduce the DL overhead in comparison to proprietary model format transfer. The main downside excluding the large standardization effort of open-format model is that it likely needs on-UE model compilation and that models are not tailored to the specific the hardware. Proprietary model information is also disclosed across vendors when using open format.

Even though these motivations outlined above may indicate some advantages with model transfer of a NW trained model, the use cases in the study item have not provided a strong motivation why model transfer is strictly necessary to be standardized to achieve such benefits of each use case. For beam management and positioning, both use cases discuss feasible alternatives using UE-sided and NW-sided models.

Similarly, the two-sided CSI use case is discussing training that does not require model transfer. It is proposed to first conclude the performance of such alternatives without model transfer prior to discussing any model transfer solution. For the two-sided model, our view is first to explore the alternative of training the model sequentially with NW first training.

For the considered use cases in this SI, there are alternatives to model transfer with similar performance without the deployment challenges associated with model transfer

Conclude that the outlined alternatives to model transfer, shall for each use case be prioritized in further discussions in this SI

Hence, for the use cases identified so far in this SI, we don’t see a need for specification support of model transfer to be further discussed as there are alternative solutions.

### 2.3.2	Model transfer/delivery alternatives

Even though we don’t see the need of in-depth model transfer discussions in the SI, we consider it important for the completeness of the study and for future use (e.g., new use cases) to at least outline various alternatives for model transfer/delivery and initial list of pros/cons for each alternative.

Our high-level understanding of the scenarios for training and deploying a UE-sided model is summarized in the table below. Given the working assumption on terminologies, our understanding of model transfer includes a delivery over the 3GPP air-interface (spec. impact).

Table 1. Alternatives for model transfer/delivery.

| Alternative | Training entity | Model transfer/delivery | Model compilation responsibility | NW/UE-side bilateral collaboration |
| --- | --- | --- | --- | --- |
| 1 | NW | Transfer/open-format | UE | No |
| 2a | NW | Transfer/Proprietary format | UE-side | Yes |
| 2b | NW | Model delivery over-the-top | UE-side | Yes |
| 3a | UE-side | Transfer/Proprietary format | UE-side | Yes |
| 3b | UE-side | Model delivery over-the-top | UE-side | No |

Regarding option 2a and 3a, where the model is signalled using a proprietary format. This involves a bilateral collaboration with the UE vendor to be compiled fitting its hardware. This could enable faster model deployment in comparison to 2b and 3b, e.g. at UE handover or connection setup. NW are also aware of which models are deployed in each UE, with the downside that it needs to host excessive number of models in device-specific proprietary format (opt 3a). In addition to the previously listed advantages of a NW trained model, opt 2a provides simplified model monitoring when specific models are executed in all UEs. The table below lists a set of pros/cons for each scenario.

Table 2. Pros/Cons with model transfer/delivery scenarios.

| Alternative | Pros | Cons |
| --- | --- | --- |
| 1 | - Simplified LCM compared to UE-side trained models - Broadcast of models possible for efficient signalling - Fully 3GPP specified solution, no bilateral collaboration needed - NW aware of when/which models are deployed in UE, no extra procedure for model identification needed | - May require on-device compilation procedure - Challenging to design models suitable for all UE vendors - High standardization complexity involving multiple working groups - Proprietary model information is disclosed across vendors. |
| 2a | - Simplified LCM compared to UE-side trained models - Faster model deployment in comparison to model delivery (option 2b/3b) - NW aware of when/which models are deployed in UE, no extra procedure for model identification needed | - NW needs to host models in UE proprietary format - Bilateral collaboration needed for UE-side to compile a NW-trained model |
| 2b | - Simplified LCM compared to UE-side trained models - No specification impact related to model delivery | - Bilateral collaboration needed for UE-side to compile a NW-trained model - Procedure needed to identify which model that have been delivered (model registration) |
| 3a | - Model compiled and designed to the UE chipset hardware | - NW may host excessive number of models in device-specific proprietary format - Complex LCM in case NW needs to monitor a vast number of UE-sided models |
| 3b | - Model compiled and designed to the UE chipset hardware - No specification impact | - Complex LCM in case NW needs to monitor a vast number of UE-sided models |

In general, there are pros/cons for each scenario. How to trade-off among the pros/cons are assumed to be very use case dependent. It is therefore preferred to progress on the use-cases to understand how such trade-off appears in each use-case and provide a set of pros/cons for each scenario.

Use scenarios in Table 1 with pros/cons in Table 2 for the model transfer/delivery discussion

### 2.3.3 Model transfer/delivery input to RAN2

Regarding the interaction with RAN2 on the model transfer discussion: One option is for RAN1 to list functional requirements and let RAN2 study relevant mechanisms. Some examples of model transfer-related requirements include model size, number of parameters, reliability, rate of model transfer or updates, inference processing and reporting time, etc. . One example for the CSI use cases is depicted in the table below.

Table 3: Examples of RAN1 model transfer requirements used as input for RAN2

| Model size (#Parameters) | Model description | Training data description | Computational complexity | Fine-tuning update size |
| --- | --- | --- | --- | --- |
| 13k parameters | UE CSI-compression encoder-A | Urban macro scenario | 32k FLOPs | Not evaluated |

Even though use cases can provide numbers for requirement on potential model transfer. It is not clear regarding the need for model transfer as discussed above. It is proposed that we should avoid sending LSs on procedures that are not necessary.

RAN1 should provide the conclusion to RAN2 that model transfer is not needed for the current studied use cases. RAN2 may study model transfer principles for future use cases

## 2.4 Model inference

### 2.4.1	Model and functionality identification

UE capabilities

AI/ML Functionality 1

AI/ML Functionality 2

Model ID 1

Non-AI/ML Functionality 1

Model ID 2

Model ID 3

Model ID 1

Model ID 2

Model ID 3

Figure 2 Assumption on relation of functionalities and model ID

Switching

Fallback

When 3GPP ventures into the field of AI/ML, a significant challenge is to align AI/ML terminology with existing 3GPP terminology. Some terms may have similar meanings but use different wording. To address this, we provide an initial discussion on the needed definitions and specified functionality around model selection, activation, deactivation, switching, registration, and fallback operation. In the discussion below, we will exemplify this with features from the study item use cases. Given that this is a new area for 3GPP, the approach is to start with minimal specification impact solutions.

Our view on the relation of functionalities and models are depicted in Figure 2. The UE is equipped with several models for a certain functionality, where, for example, a functionality might comprise of providing beam predictions. We, therefore, see the functionality as having a wider scope than a single model (that is, the UE may have several models that can be used to provide a functionality). Next, we discuss each aspect of the LCM aspect related to model LCM stages.

Functionality identification:

We foresee that 3GPP needs to define new features that provide predictions (e.g., via AI/ML-models), which the UE can be configured with. The name of the feature would then correspond to the functionality identification. The UE will deliver predictions like the existing reports. How UE derives the predictions will not be known if 3GPP does not specify the actual AI/ML model (note that specifying an AI/ML model is out of scope of the SI). From an operational point of view, the NW needs to know that the UE supports a certain feature and this information can be obtained via UE capability signalling.

Exactly how to capture UE capabilities and features is a work item topic. Below we illustrate the details on the specific UE capability signalling, taking spatial beam prediction as an example.

Table 4. Example of feature group indicating model based on TR 38.822 (note some columns that should be in the column have been removed for easier illustration).

| Features | Index | Feature group (Functionality identification) | Components | Note (Model identification) | Mandatory/Optional |
| --- | --- | --- | --- | --- | --- |
| X. AI/ML features | X.1 | AI/ML based beam prediction in spatial domain | Support of beam prediction in spatial domain Configuration of Set A and Set B beams Model monitoring support for beam prediction | Not needed/Optional | Optional with capability signalling |

Conclude that functionality identification is used as a starting point for discussing LCM operations for one-side UE models, where the functionality is identified using the UE capability framework as defined in 38.331 and 38.306 as a starting point

Model identification:

The functionality might utilize several different models for providing certain predictions. Our view for one-sided models is that there is no need for the UE to identify the exact model it is using. Hence there is no need for the UE to provide any model ID to the NW. Nor are there any need for the NW to activate/deactivate/switch the models using the model ID. For two-sided models however, there might be a need for the UE to identify the models including the model ID, for proper selection of UE/NW model pair.

Conclude that model identification via model-ID is not needed for one-side UE models

Model activation/deactivation:

It is implicitly assumed that whenever a functionality is deactivated, the associated model is also deactivated. It is the feature that is activated and deactivated and not the AI/ML model. Taking beam prediction reporting as an example, similar to the legacy L1-RSRP report as of NR Rel-15, a UE can be configured to report beam prediction related information (e.g., L1-RSRP or/and SSBRI/CRI of the predicted beam(s)) most likely via an RRCReconfiguration message, this would then correspond to “activation” of the feature. “Deactivation” will be the opposite, i.e deconfiguring the report.

Conclude that model activation/deactivation is done transparent to the NW for one-sided UE models

Model switching

The UE can when a functionality is activated decide to switch model transparently to the NW, note that the UE still needs to comply to the performance requirements of the feature. It is up to RAN4 to study how one can ensure that the models are working properly at the UE.

Conclude that model switching is done transparent to NW for one-sided UE models (no RAN1 impact, but may have RAN4 impact)

Model fallback

For a one-sided AI/ML model on the UE side, fallback mechanisms can be achieved by the UE being configured by the non-AI/ML based features already existing in the specification. For beam predication, UE can in parallel be configured with a feature to measure instead of predicting if the NW deems it needed and can simply switch UEs into reporting mode instead. This will then be sufficient as a fallback method, switching to a non-AI/ML model-based operation will in this context mean switching back to the Rel-18 or previous release behaviour.

Conclude that for a one-sided AI/ML model on the UE side, fallback mechanisms can be achieved by the UE being configured by the non-AI/ML based features already existing in the specification for the applicable use case of the SI.

Model registration

This would comprise the functionality indication described above, no need for a separate procedure.

Conclude that model registration is not needed for one-sided UE models, it is implicit from the functionality identification

Two-sided models.

In both NW vendor joint training with model transfer to UE and NW-sided model first in sequential training, the UE would need to support at least one model per network vendor. Hence, there will be a need for the UE-side to select, or be configured with, the model that is compatible with the NW-side model. One can envision different procedures for pairing UE-side and NW-side models that are based on some model IDs, but the fundamental principles of any standardized model pairing procedure should be easy to maintain and result in minimum inter-vendor coordination efforts.

For two-sided models, standardized model pairing procedure should be easy to maintain and result in minimum inter-vendor coordination efforts

## 2.5 Model training

For one-sided models, the model training process for one side can be transparent to the other side and since without any standardization impact there are no reasons to discuss one sided model training.

For two-sided models, however, the model training process may require sharing of data from one side to the other since input and output reside within different vendor’s domain. This may thus have a standardization impact.

For two-sided model training, it can first be noticed that both joint training by a NW vendor with model transfer to UE (Type 1) and NW-sided model first in sequential training (Type 3 or Type 2 with frozen decoder) fulfil a requirement from gNB implementation that a gNB shall be able to operate with a single model in two-sided model use cases, irrespectively of the origin (vendor) of the UE side model.

However, as discussed in Section 2.3, model transfer has model deployment challenges, so NW-sided model first in sequential training shall be prioritized in further discussions on two-sided training. Note that we here preclude the less likely scenario that a NW vendor performs joint model training offline with multiple UE/chipset vendor models simultaneously, to obtain a single NW-sided model. We don’t see the need for this either as the UE can train a model for each network vendor independently and perform model switching when roaming (it is assumed that such switching of models in the UE will be very slow, based on hour timescales or much longer).

It is envisioning that initial two-sided model training with NW-sided model first in sequential training needs to be done outside 3GPP when training of the UE-sided model is done via a NW-vendor provided training API, used for exchanging forward propagations (UE-side to NW-side) and backward propagations (NW-side to UE-side). If such training API is NW-vendor specific, then there would not be any standardization impact related to the training process.

In a scenario where fine-tuning (small adjustments of model weights) of a deployed UE-sided model over-the-air could be considered, then the training API would need to be specified as part of the Uu. However, the benefits of fine-tuning a deployed UE-sided model are unclear since fine-tuning of two-sided models can alternatively and with lower complexity be done by re-training the NW-sided model only. This allows fine tuning of performance without standardization of a training API.

Deprioritize studies and discussion on over-the-air training between NW and UE.

## 2.6	Model monitoring

Model performance monitoring refers to monitoring drifts in data and model or monitoring performance metrics after the model has been deployed. This is often described as the problem of concept drift detection problem [ref “Learning under Concept Drift”].

Agreement Study at least the following metrics/methods for AI/ML model monitoring in lifecycle management per use case: - Monitoring based on inference accuracy, including metrics related to intermediate KPIs - Monitoring based on system performance, including metrics related to system peformance KPIs - Other monitoring solutions, at least following 2 options. - Monitoring based on data distribution - Input-based: e.g., Monitoring the validity of the AI/ML input, e.g., out-of-distribution detection, drift detection of input data, or something simple like checking SNR, delay spread, etc. - Output-based: e.g., drift detection of output data - Monitoring based on applicable condition Note: Model monitoring metric calculation may be done at NW or UE Agreement Study performance monitoring approaches, considering the following model monitoring KPIs as general guidance - Accuracy and relevance (i.e., how well does the given monitoring metric/methods reflect the model and system performance) - Overhead (e.g., signaling overhead associated with model monitoring) - Complexity (e.g., computation and memory cost for model monitoring) - Latency (i.e., timeliness of monitoring result, from model failure to action, given the purpose of model monitoring) - FFS: Power consumption - Other KPIs are not precluded. Note: Relevant KPIs may vary across different model monitoring approaches. FFS: Discussion of KPIs for other LCM procedures

Given model input X and model output Y, the model monitoring may detect two types of drifts:

- Input data drift, i.e., the model’s input data distribution P(X) has changed. Since P(X) change does not necessarily cause decision boundary change, this is also called “virtual drift”.

- Concept drift. The conditional probability P(Y|X) has changed compared to the training dataset. Such change causes decision boundary change, and is also called “real concept drift”.

For input drift, it corresponds to the model monitoring method of obtaining monitoring metrics based on distribution of model input. The model monitoring metrics can be obtained by using statistical methods to compare the probability distributions between a reference distribution of Xref and a monitored distribution Xt. For example, the reference distribution may come from the training dataset or a previous time window, while the monitored distribution is the current measurements Xt. Several types of monitoring metrics exist, e.g., Kolmogorov–Smirnov test, Chi-squared test, Kullback–Leibler divergence, Jensen-Shannon (or JS) divergence, etc. There are several issues with monitoring based on input data distribution only:

- Data distribution is not a reliable indicator of model drift. As discussed, P(X) shift does not necessarily mean P(Y|X) has changed. For example, for the positioning use case, the training data is collected randomly and uniformly on the entire factory floor. However, during inference, the target devices may be clustered in certain work areas, and not at all in other areas. Moreover, the areas where the target devices are clustered may change from time to time. However, the trained model can still work properly as long as the deployment environment is the same as the training environment. In this example, P(X) (also P(Y)) at inference appears very different from the P(X) (also P(Y)) at training, but P(Y|X) stayed the same and the model does not need update.

- The curse of dimensionality. The input dimension of physical layer AI/ML model is typically very high, for example, the input vector size is 256x18x2=9216 for a typical evaluated models for the positioning. Thus it is impossible to calculate a single aggregated drift metric to capture multivariate drifts (i.e., a drift metric for the joint probability of all elements of input X). Alternatively, if using univariate approach to calculate the drift metric for each feature individually, it is also impossible to figure out the correct weights for each feature according to their impact to the overall drift metric. In conclusion, it is difficult to achieve high accuracy for model monitoring.

- Tendency to cause high false alarm rate. Not all changes in the feature space of model input would cause a significant drift in model output. Different features affect the learned decision boundary differently. If an alarm is triggered whenever distribution statistics of an input feature has shifted more than a threshold, it may very well be a false alarm if the decision boundary is not affected, e.g., the Top-1 beam prediction is still correct in spite of the change in a j-th input measurement statistic. In summary, it is difficult to control the costly problem of high false alarm rate.

- High computation complexity and storage requirement. In order to calculate reliable statistical metrics for input metric, a large number of samples need to be stored and then used in the calculation of the statistics. This is particularly true when the input dimension is large, since a small sample size cannot provide useful statistics for each and every feature in the model input. Thus, the burden of computation complexity and storage requirement is expected to be high.

Based on the discussion above, model monitoring based on input data distribution should be deprioritized.

Considering model monitoring based on model output distribution, for (A), the dimension of model output can be large (e.g., CSI use case) or small (e.g., BM use case) depending on the use case. Nevertheless, the problems (A), (C) and (D) still exist if monitoring the output distribution P(Y).

In summary, model monitoring based on data distribution (both input-based and output-based) should only be considered when it is not feasible to use a performance-based monitoring metrics, where the performance can be inference accuracy (if the ground truth label can be obtained) or system performance, depending on the use case. For the three use cases of the study item, performance-based monitoring metrics are available. Thus, model monitoring based on data distribution be deprioritized for the study item.

Deprioritize model monitoring based on data distribution (both input-based and output-based).

For real concept drift, most model monitoring method is performance-based. One clear advantage of performance-based approaches is that the model monitoring metric only react to changes that actually degrade the performance (e.g., inference accuracy, system performance KPI). Thus, performance-based methods tend to have lower rate of false alarms.

For model monitoring KPIs, the existing agreement did not include two important metrics: false alarm rate (FAR) and missed detection rate (MDR). However, both metrics have large impact to the model monitoring efficiency. For example, high false alarm rate causes incorrect and expensive reaction to deactivate, modify, or retrain the model. On the other hand, high miss detection rate would cause the system performance to deteriorate beyond the designed performance level. Thus we propose the following:

Add the following as model monitoring KPIs: false alarm rate (FAR) and missed detection rate (MDR).

### 2.6.1 Model monitoring for one-sided models at the NW side

For one sided models at the NW-side, the model performance monitoring is performed at the NW side. The NW collects data from UEs or from its own radio measurements and calculates the performance metrics of the AI/ML model (e.g., inference accuracy, system performance). If an alarm is triggered based on the performance metrics, the NW node reacts to rectify the model drift problem, e.g., fallback to a default feature/model, deactivate this model, switching to another reporting configuration, etc. In general, there is no standard impact foreseen for model monitoring for one-sided models at the NW-side, except for the monitoring data collection aspects, which is discussed in section 2.7.3. The requirements in terms of signaling overhead and latency for model monitoring are use case dependent, hence, in section 2.7.3, we’ve discussed two approaches on data collection for model minoring, depending on the use case requirements.

One-sided models at the NW-side are monitored by the NW. Study data collection mechanisms to support performance-based (e.g., inference accuracy) model monitoring for NW-sided models.

### 2.6.2 Model monitoring for one-sided models at the UE side

For one-sided AI/ML model use cases, the performance of a UE-sided model is expected to be monitored the UE-side.

The UE can collect the ground truth labels or/and measurements to calculate the intermediate KPIs. For UE-sided models that require DL radio measurements (e.g., L1-RSRP) for model monitoring, the NW may assist the model monitoring by configuring DL-RS transmissions for the UE to perform measurements and then compare the measurements with the model output, the NW may also assist the model monitoring by providing ground truth or proxy ground truth. Potential standard impact on triggering a UE performing model monitoring or trigger the NW to transmit DL-RS for assisting UE-sided model monitoring can be studied. The performance metrics could comprise of a per-sample error or an aggregated statistical error over a certain model monitoring window. For example, for UE-sided beam prediction use cases, the performance metric could comprise of a statistical summary of the PDF or CDF (e.g., the 5th,10th, …, 90th percentile) of the L1-RSRP prediction error over a certain monitoring window. The performance metrics calculated at the UE can be reported to the NW if needed.

If a UE supports multiple AI/ML models for a certain feature (e.g., multiple scenario-specific UE-sided models for spatial domain beam prediction), and if the same UE reporting configuration is defined for these models, then, it is reasonable to assume that the support of multiple models at the UE-side are transparent to the NW. In this case, from 3GPP point of view, there is no need to configure/signalling different model IDs for different models that have the same UE reporting configuration, and the decision of switching between these different AI/ML models can be made by the UE. However, it should be noted that, the model performance of these models should be properly tested, and the model switching should not cause impact on normal network operation. These aspects should be studied in RAN4.

While it is possible for the NW to monitor the performance of UE-side models (e.g., for UE-sided CSI prediction use case, UE-sided spatial domain beam prediction use case, UE-sided temporal beam prediction use case, or for positioning, Case 2a: UE-assisted/LMF-based positioning with UE-side model, AI/ML assisted positioning), the burden is too high considering that the NW node has to support all the UEs connected to it. Furthermore, each UE may have different configurations, different capabilities, and different inference accuracy targets. The NW has to customize the model monitoring mechanism differently for different UEs to achieve the desired set of KPIs (drift detection accuracy, latency from drift started to drift detected, etc). Thus it is more reasonable that the task is distributed to each UE individually, i.e., each UE performs its own model monitoring for UE-side models according to its needs and capabilities.

Instead of monitoring the performance of UE-sided models, the NW may monitor the classical performance for a certain AI/ML based feature at the UE. Taking UE-sided spatial beam prediction as an example, the NW can monitor the number of beam switches, the number of beam failures and/or the UE throughput of a UE-sided AI/ML-model based spatial domain beam prediction feature. By checking the classical performance of the feature, the NW can detect if the feature at the UE is functioning or not.

Mechanisms for supporting a UE to fallback to a non-ML based scheme when detecting model/feature performance issues should be studied. The model/feature performance issues may be detected by the UE or the NW or both. However, the decision of UE-side model fallback must be made by the NW, if NW needs to configure the UE with a different UE reporting configuration.

One-sided models at the UE-side are monitored by the UE.

### 2.6.3 Model monitoring for two-sided models

Different from one-sided model use cases, a two-sided model is divided into two parts, i.e., UE-part of the two-sided model and NW-part of the two-sided model, which implies that the input and output of the two-sided model reside on different ends of the Uu interfaces. For two-sided model use cases (e.g., CSI compression), as stated in Section 2.2, to enable a gNB serving multiple UEs simultaneously and to keep implementation efficiency, cost, and complexity feasible at the NW side, it is required that a single model is operated at the gNB side, regardless of the models operated in different UEs with different vendor versions and/or chipset versions. Any new models to be deployed at the UE-side must be trained, validated, and properly tested together with the single model at the NW-side before being deployed at the UEs. These testing aspects should be studied in RAN4.

The performance monitoring of a two-sided model is expected to be performed at the NW side. The label data (e.g., target CSI for the CSI compression use case) and the output of the UE part of the two-sided model (e.g., the encoder output of the CSI compression use case) can be occasionally reported to the NW and the NW side can then asses the performance using the same loss function used during training (which is unknown to the UE side). Using these data together with the associated output of the NW part of the two-sided model, the NW can obtain the intermediate KPI(s) of the two-sided model. The model performance metric together with the feature performance KPIs can be used by the NW to monitor the two-sided model performance.

The performance monitoring of two-sided models is performed at the NW side. Study data collection mechanisms to support performance-based (e.g., inference accuracy) model monitoring for two-sided models.

## Data Collection

Conclusion Data collection may be performed for different purposes in LCM, e.g., model training, model inference, model monitoring, model selection, model update, etc. each may be done with different requirements and potential specification impact. FFS: Model selection refers to the selection of an AI/ML model among models for the same functionality. (Exact terminology to be discussed/defined) Agreement Study potential specification impact needed to enable the development of a set of specific models, e.g., scenario-/configuration-specific and site-specific models, as compared to unified models. Note: User data privacy needs to be preserved. The provision of assistance information may need to consider feasibility of disclosing proprietary information to the other side.

Data Collection is a stage that collects and provides input data (raw data or pre-processed data) to the Model training stage (e.g., for model training) and the Model inference in deployment stage (e.g., for model inference and model performance monitoring). AI/ML algorithm specific data preparation (e.g., data ingestion and data refinement) is not carried out in the Data Collection stage. For example, for the CSI use case, raw data is the output from the channel estimation/filtering algorithm implemented at the data collection functionality and the algorithm is kept proprietary. As the requirements on data collection can be different for different purposes in LCM, in the following, we discuss the data collection for model training, model inference and model monitoring separately. And we focus on the cases where the AI/ML models are trained at the NW side.

### 2.7.1 Data collection for NW side model training

The period of performing model training or retraining is in general long, hence, data collection for model training does not need to be performed frequently relative to L1 timescales. The latency requirement for NW to obtain training data can be relaxed as compared to model inference.

If an AI/ML model only requires legacy uplink gNB radio measurements (e.g., UL-SRS-based measurements), then the NW-side can collect and label data via existing 3GPP procedures or using proprietary non-3GPP methods. In such settings, the training data collection problem is a proprietary NW-side issue, which is outside the scope of 3GPP.

For cases where the AI/ML model on the NW side needs to be trained using downlink L1 measurements performed by UEs, it is important to standardize appropriate data collection mechanisms for the UE’s measurements to be signaled to the NW over the air interface. This is to ensure that the NW side can collect unified data format from different UE/chipset vendors, which is essential for reducing the complexity and data processing cost of the AI/ML model design. In addition, the training data quality can be determined by 3GPP standard.

In addition, in case of “localized” data collection to enable training of site specific or area specific NW side AI/ML models, there is a need to trigger collection of data from a specific site of group of sites, or at a certain point of the day or week. In this case, the NW side needs to be in control of triggering data collection procedure in the UE. Hence, a standardized data collection procedure in the UE for NW side training purpose is beneficial.

As the time of measurements and the time of reporting the collected measurements back to the NW are decoupled (these can be two different events separated by e.g. hours), a UE can log/store its radio/non-radio measurements together with the meta information (e.g., time stamps, cell ID, and/or UE location) for multiple measurement occasions. The meta information can be seen as “tagging” the measurements with information on where/when the data was collected. After performing the radio measurements and logging the data, the UE can be triggered to report the accumulated data to the NW over the 3GPP air-interface for subsequent off-line proprietary AI/ML model training at the NW side.

This UE reporting over the 3GPP air-interface based training data collection has standard impact and it should be studied in this SI.

Study 3GPP data collection mechanisms to support UE performing data logging/collection and reporting the collected data to NW over the air-interface for model training.

To aim for designing a general framework to enable NW collecting training data for different AI on PHY use cases, it is important that the following aspects are considered.

The data collection framework design consists of:

- Definition of Data types

The data types to be measurement and logged by the UE are use case dependent. However, in general, they can be divided into two types: radio measurement data and non-radio-measurement data. For example

  - Beam management use cases: Radio measurement data can consist of UE L1-RSRP measurements of all beams in set A (prediction set of beams) and set B (measurement set of beams), at one- or multiple measurement occasions. Non-radio-measurement data can consist of cell-IDs, area IDs, UE RX beam IDs, time stamps, UE locations, and possibly also measurement accuracy.

  - CSI feedback enhancement use cases: Radio measurement data can consist of UE CSI-RS measurements to be used as target CSI (e.g., measured channel H or eigenvectors per layer). Non-radio-measurement data can include cell-IDs, area IDs, UE antenna configurations, time stamps, UE locations, and possibly also measurement accuracy.

  - Positioning use cases: DL-PRS-based L1-measurements (e.g., CIR measured at UE and reported to the LMF for UE assisted LMF-based positioning with LMF-sided model, Case 2b in 3GPP AI-positioning discussions). Non-radio-measurement data contains at least UE location to be used as ground truth label for model training.

- Study of Data sizes

The collection of training data is expected to be distributed over many UEs from one or multiple cells. Hence, data collection may be localized to a specific area (or time span) or more general, non-localized spread across cells in the entire network.

A UE can log/store its radio measurements performed on these multiple time instances together with other types of data (e.g., meta data or assistance information). The UE is configured or triggered to report accumulated data to the NW-side at a suitable point in time, e.g. when network load is low. Hence, the overhead of training data collection might not impose significant UL overhead. The data sample size for AI model training is use case dependent as further discussed below:

  - Beam management: The number of bits needed for a UE to report its L1-RSRP measurements depends on how many beams the UE is configured to measure and report. Based on the current standard, 7 or 4 bits are used for reporting a L1-RSRP value. Therefore, the payload size per UE report for the beam management use cases is not expected to be too large (e.g., around 259 bits for L1-RSRP measurements of 64 beams). Non-radio-measurement data may or may not need to be reported, depending on the type of information and whether UE mobility is supported in combination with data collection or not.

  - CSI feedback enhancement: The number of bits needed for reporting a single CSI-RS measurement depends on how the target CSI is represented (the data format and pre-processing/quantization method). For example, if raw channel measurements are reported, then, a UE may need to feedback multiple complex values in the order of (number of Rx virtual antenna ports * number of CSI-RS ports* number of subcarriers, e.g., 4*32*52) for one channel measurement per time instance. Depending on the required quantization bits used to quantize each complex value (phase and amplitude), the data size per UE report can be very large for this CSI use case. Non-radio-measurement data may or may not need to be reported, depending on the type of information and whether UE mobility is supported in combination with data collection or not.

  - Positioning: Among the considered positioning use cases, collecting UE measurements is only needed for UE assisted LMF-based positioning with LMF-sided model (i.e., Case 2b in 3GPP discussions). The number of bits needed for reporting a single UE measurement report depends on the type of measurements the UE should report, how the measurement (e.g., CIR) is represented, and how many TRPs are involved. For example, the CIR data size for a single UE location may have a size of 18 (#TRPs)*2 (#antenna ports)*256 (#sample in time domain) = 9216 complex values. Depending on the number of quantization bits used to quantize each complex value, the data size per UE report can be very large. For this use case, the NW-sided model is at the LMF node and the L1 measurements need to be reported from UE to LMF over LPP. In addition, the UE locations (non-radio-measurement type of data used as ground truth labels) need to be either reported from UE to LMF over LPP or obtained at LMF via other solutions.

- Latency and periodicity

The period of performing AI model training/retraining can be relatively long (e.g., months) and the data collection for model training is not required to be performed very frequently. Therefore, the latency requirement for collecting training data is much relaxed as compared to that for model inference.

- Configuration-related requirements

The configuration-related requirements for training data collection are also use case dependent. However, in general, the following aspects should be considered:

  - Study need to assess whether it requires periodic or semi-persistent data collection or event-triggered (aperiodic) data collection or possibly support for multiple of these?

  - It should be possible to configure a UE to store measurements on multiple measurement occasions and then report the accumulated data to the NW.

    - Depending on the use cases, a measurement occasion may consist of a single RS resource set (e.g., for spatial beam prediction or CSI compression use case) or a burst of multiple RS resource sets at different time instances (e.g., for temporal beam prediction use case).

      - For the spatial beam prediction use case, a measurement occasion can consist of a set of CSI-RS/SSB resources that are corresponding to the union of Set A and Set B beams.

    - For the temporal beam prediction use case, a measurement occasion can consist of multiple sets of CSI-RS/SSB resources, with each resource set corresponding to the union of set A and set B beams transmitted at a given time instance. For this type of use cases, it is also important that the time instances configured in a measurement occasion should cover both the observation time instances (related to model input) and the prediction time instances (related to model output).Measurement occasion interval or logging interval

      - The time duration between two neighboring measurement occasions. UE performs measurements on each measurement occasion and stores/logs the data. In this case, the data logging interval is equal to the measurement occasion interval, and it is the periodicity for a UE to store measurement results and associated non-radio-measurement data.

  - Typical data collection duration or logging duration in case of periodic or semi-persistent data collection.

  - Conditions for a UE to log/store its L1-measurements (e.g., under RRC IDLE, RRC INACTIVE or/and RRC CONNECTED states)

  - Conditions for a UE to report its L1-measurements to the NW (e.g., at off-peak times, at configured time occasions, etc.)

  - Conditions for a UE to discard the measurements.

  - Measurement data pre-processing or data format configuration (e.g., the format of target CSI for the CSI-compression use case, absolute or differential L1-RSRP values for beam prediction use cases, or only report L1-RSRP for the top-K beams, etc.).

  - Data logging triggers

  - The events and conditions that triggers a UE to start performing radio measurements and data logging. For instance, a UE can start performing radio measurements and data logging after receiving a data collection related measurement configuration message from the NW. The data collection can also be event-based triggered. The events and conditions for triggering radio measurements and data logging at UE should be studied per use case.

  - Data reporting triggers

  - Whether a UE reporting the logged data is event triggered or the logged data is periodically reported from the UE to the NW. The events and conditions that triggers a UE to perform data reporting should also be studied per use case.

Different methods can be considered for supporting UE performing data logging/collection and reporting the accumulated data to NW for model training. Comparing to the layer-1 CSI reporting method (UCI on PUSCH), the RRC message-based data collection solution can enable the NW to collect large amount of measurement data from a UE with reduced signaling overhead and radio resource consumption.

Recommend RAN2 to study specific details of an RRC-message based data collection framework to support UE performing data logging/collection and reporting the collected data to NW for model training

### 2.7.2 Data collection for NW side model inference

For AI on PHY use cases, the data collected for NW side model inference is mainly about collecting model input, which will likely require fast L1 feedback from the UE, hence, it is expected that the L1 CSI reporting framework or SRS framework will be reused/enhanced to support data collection (at least for collecting radio measurement related data) for model inference.

For use cases where NW-side model inference takes UL-RS radio measurements as model input, there is no need of UE reporting L1-measurement data for NW-sided model inference. For example, for the NG-RAN node assisted positioning use case with gNB-sided model (i.e., Case 3a in 3GPP AI positioning use case discussions), it is expected that the SRS framework will be reused/enhanced to support UE transmitting SRS so that the gNB can perform radio measurements for collecting its model input.

If model inference at the NW-side requires DL-RS based UE L1-measurements or UCI type of data, then, collecting data for NW-sided model inference will require fast L1 UE reporting. The data collected for model inference are use case dependent: Depending on the use cases, the following aspects should be studied

- Methods to configure UE reporting measurement-based and non-measurement-based data for network-sided model inference.

- Tighter UE radio measurement accuracy requirements for AI/ML-based features.

- Methods to signalling DL/UL-RS resource configurations for UE performing radio measurement or UL-RS transmissions.

It should be noted that whether there is a need for standard enhancements needs to be studied per use case in RAN1. For example, for the NW-sided spatial domain beam prediction sub use case, the data collection for model inference should support a UE to report measurements related to the set B of beams. For the NW-sided temporary beam prediction sub use case, the data collection for model inference should support a UE to report measurement related to set B of beams over multiple measurement time instances (e.g., within observation time window T1). Tighter L1-RSRP measurement accuracy requirement and methods to reduce the CSI reporting overhead can be studied for both sub use cases. For the two-sided CSI compression use case, the data collection for model inference should support a UE to transmit the output of the model at the UE side, e.g., as a new type of UCI on PUSCH/PUCCH. For NG-RAN node assisted positioning with gNB-side model, AI/ML assisted positioning sub use case, the data collection for model inference should support a UE to transmit SRS so that the gNB can perform radio measurements for its model input.

Study in RAN1 the requirements of data collection for model inference per AI on PHY use case.

### 2.7.3 Data collection for monitoring of NW-sided models

For AI for PHY use cases, different performance metrics can be considered for AI/ML model performance monitoring. Examples of performance metrics include inference accuracy (also called as intermediate KPIs, like L1-RSRP accuracy for beam prediction use cases, SGCS for CSI feedback compression use cases, and positioning accuracy for direct AI-positioning use cases), system performance KPIs (e.g., throughput, BLER, NACK-ACK, etc.), input data distribution. Hence, depending on the model monitoring methods/performance-metrics, monitoring data collection can be needed for

•	obtaining the ground truth labels or/and measurements to calculate the inference accuracy (intermediate KPIs), and/or

•	obtaining the measurements to calculate the input data distributions/statistics to detect data drifts, and/or

•	obtaining the system performance KPI related information

Depending on the use case requirements, e.g., whether it is time critical to identify an AI/ML model performance issue or not, the latency requirement for collecting data for model monitoring can be different. This can also impact the data types/content, and the frequency/periodicity of data collection for model monitoring purpose.

For NW-sided models, it is expected that the model monitoring is performed at the NW-side. Based on the use case requirements, the NW configures the frequency of model monitoring (e.g., periodically, or event-triggered), collects monitoring data from UEs or based on its own radio measurements, and calculates the performance metrics (e.g., inference accuracy, system performance KPIs, input data distribution, etc.). Based on the performance metrics, the NW performs actions, e.g., fallback to a default feature/model or switch to another ML model.

For the AI on PHY use cases where the L1-measurements required for model monitoring can be obtained based on legacy radio measurements performed at gNBs (e.g., UL-SRS-based measurements) and the NW-side can collect label data either via existing 3GPP signalling or via non-3GPP method, it is expected that the data collecting for model monitoring can be performed at the NW-side proprietarily.

For the use cases where at least L1 UE measurements need to be collected at the NW-side for model monitoring, two options can be considered for data collection for model monitoring of NW-side models:

- Option 1: RRC-message based data collection for model monitoring

- Option 2: L1 fast CSI reporting-based data collection for model monitoring

For option 1, the methods and requirements of data collection for model monitoring can be similar to the ones that have be discussed in Section 2.7.1 on the data collection for model training. Consider NW-sided beam prediction as an example, the NW can monitor its model performance by comparing predicted values with the corresponding measurement values reported from UE. To collect sufficient data for obtaining the performance metrics for model monitoring, e.g., the statistic info about the input data distribution and the distribution of prediction error, the RRC-message based data collection framework discussed in Section 2.7.1 can be reused here.

If the NW requires a fast check of its model performance, then, Option 2 can be used. For option 2, the methods and requirements of data collection for model monitoring can be similar to the ones that have been discussed in Section 2.7.2 on the data collection for model inference, with the difference that, for some use cases, the UE should report not only the data related to model input but also the data related to model output or ground truth. For example, for spatial beam prediction use case, the data collection for model monitoring should support the UE to report measurements for beams in both set A (prediction set of beams) and set B (measurement set of beams). For CSI compression use case, the data collection for model monitoring should support the UE to report both the model output of the UE-sided model but also the target CSI (e.g., high-resolution channel H or eigenvectors of the channel transmit covariance).

Recommend both an RRC-message based and L1 fast (i.e., similar to aperiodic CSI reporting) based data collection methods for NW side model monitoring.

RAN1 to study specific requirements on RRC-message based data collection method for NW side model monitoring. When conclusions are reached on the requirements, RAN1 would indicate that to RAN2 so that RAN2 can study the corresponding RAN2 aspects.

RAN1 to study specific requirements and specification impacts of an L1 fast CSI reporting based data collection method for NW side model monitoring.

# 3 Model and computational complexity reporting

During the RAN1 #111 AI 9.2.4.1 discussion, it was identified that the computational complexity values reported by different companies may not be mutually comparable since some of them are deviating from general observed trend by two orders of magnitude or more. This is illustrated in Figure 3.

- Since each parameter of a model should be used at least once, we can use the number of parameters as a lower bound for the computational complexity (shown as lower bound 1 in the figure).

  - For both dense and convolutional layers, the number of multiplicative parameters dominates the total number of parameters. The number of parameters can be used as an approximate lower bound on the number of MACs (multiply-accumulate). Hence, a second approximate lower bound on the computational complexity FLOPs can be obtained as two times the number of parameters (shown as lower bound 2 in the figure).

    - Due to its nature of weight sharing, convolutional neural networks tend to exhibit much higher FLOPs-to-parameters ratios than these lower bounds.

- Two regression lines are fitted to the reported model vs computational complexity values. The first regression excludes one source below the lower bounds. The second regression excludes five sources near or below the lower bounds.

- It can be observed that most reported model and computational complexity values are near or at least within an order of magnitude of the regression lines. However, there are some outliers that deviate substantially from these general observed model vs computational complexity trend.

Figure 3: Model complexity vs computational complexity for direct positioning models as reported in RAN1 #111 except for three updated Ericsson’s reported values from [10]

To assist this discussion, we consider a simple example where the computational complexity can be exactly calculated by hand.

Example:

Given a  BHiWiCi =132323 input tensor to a convolutional layer with

- Batch size B = 1,

- Kernel size: HkWk =33

- Stride: 11

- Padding: “VALID”, i.e., without padding

- Number of input channels Ci=3

- Number of output channels: Co =16

The shape of the output tensor becomes BHoWoCo =1x303016. That is, Co =16 output channels each with size HoWo =3030.

The convolutional layer will need

- HkWkCiCo =33316 = 432 parameters for the kernel

- Co =16 parameters for the bias

TensorFlow/PyTorch tools report the total number of parameters as 448 as expected.

In this simple example, one can calculate the nominal computational complexity by hand:

- The number of MACs (multiply-accumulate) using the convolutional kernel can be calculated as (HkWkCi) (HoWoCo) =(333) (303016)=388,800.

- The number of ADDs (addition) using the bias can be calculated as HoWoCo =303016 = 14,400.

- The total number of FLOPs is given by 2MACs + ADDs = 792,000.

On a CPU platform, TensorFlow/PyTorch tools report the same number of FLOPs as hand calculated above.

On a GPU or TPU platform, some TensorFlow/PyTorch tools may report a number of FLOPs that can be one, two or even more orders of magnitude lower. It is assumed these lower FLOP numbers are accelerator-optimized computational complexity for the underlying GPU/TPU.

It can be observed from the above example that

- Nominal computational complexity is a general characterization of a model’s algorithmic complexity that can be compared across different sources. To a first degree, this complexity can even by calculated by hand.

- The accelerator-optimized complexity is specific to a particular device and its computational capability.

  - Since it’s unlikely to deploy a full-blown data-center class GPU/TPU in a UE or even a gNB, these GPU/TPU optimized complexity values do not provide useful information to the discussion in 3GPP.

  - No explanation on what these accelerator-optimized complexity values mean was given by these software tools, either. Taking one of our large centralized model with 11.2 M presented in our positioning use case paper [10] as an example. While the nominal complexity is 410 M FLOPs, the reported accelerator-optimized complexity for a GPU is given as 4.3 M FLOPs, which is well below the lower bounds. Since all those MACs cannot just disappear simply because the model is executed on a GPU, these accelerator-optimized complexity values seem more related to clock cycles of the GPU than the algorithmic complexity of the model.

  - Furthermore, given different sources will have different ML-optimized implementations, such type of computational complexity numbers is not comparable across sources.

The discussion above echoes the Conclusions that RAN1 has made:

Conclusion (RAN1#110bis) This RAN1 study considers ML TOP/FLOP/MACs as KPIs for computational complexity for inference. However, there may be a disconnection between actual complexity and the complexity evaluated using these KPIs due to the platform- dependency and implementation (hardware and software) optimization solutions, which are out of the scope of 3GPP. Conclusion (RAN1#111) Companies describe how their computational complexity values are obtained. - It is out of 3GPP scope to consider computational complexity values that have platform-dependency and/or use implementation (hardware and software) optimization solutions.

Hence, we propose the following for fair comparison between model complexities as reported by different companies.

For 3GPP AI/ML for PHY SI discussion, companies shall report nominal computational complexity values (and not accelerator-optimized computational complexity values).

To further 3GPP discussion and preparation of observations/conclusions for the technical report, we propose three model size classes based on the reported model and computational complexity values. The model size classes can be defined per use case. Table 5 shows an example of proposed model size classes for the positioning use cases.

Table 5 Proposed model size classes for positioning use cases.

| Model class | Model complexity in millions [# of parameters] | Computational complexity in millions [FLOPs] | Number of sources as reported in RAN1 #111 for direct positioning |
| --- | --- | --- | --- |
| Small models | < 1 M | < [60] M | 4 |
| Medium-size models | 1 – 8 M | [60 – 300] M | 8 |
| Large models | > 8 M | > [300] M | 5 |

To further 3GPP discussion and preparation of observations/conclusions for the technical report, three model size classes are defined per use case, as follows:
- Small models (e.g., < 1 M model parameters for positioning use cases)
- Medium-size models (e.g., 1 – 8 M model parameters for positioning use cases)
- Large models (e.g., > 8 M model parameters for positioning use cases)

Another aspect to align among companies is the unit for the “number of model parameters”. Some companies use real number arithmetic, while other companies use complex number arithmetic. If using complex value arithmetic, N complex value parameters is equivalent to 2×N real value parameters. To support model complexity comparison among companies, it is simpler to align unit for the “number of model parameters” to “number of (real-value) model parameters”.

For 3GPP AI/ML for PHY SI discussion, companies shall report model complexity in terms of “number of real-value model parameters” regardless of underlying model arithmetic.

# 4 On UE processing/capability, requirement setting, and testing

Several LCM scenarios considered in the SI (e.g. at least two-sided models and one-sided models including assistance signalling) would most likely impact how performance requirement setting and compliance testing of UEs can be carried out. The ability to ensure that UEs achieve a predetermined minimum performance and behave consistently according to RAN4 requirements throughout their lifetime should in no case be compromised. Numerous issues are foreseen that may motivate RAN4 framework oversight and modifications. For example, compliance needs to be guaranteed for a functionality that might include model update/switching. Compliance for two-sided models does not fit the current frameworks, considering that compliance cannot be a joint responsibility. Further, it is not clear how to achieve compatibility of fixed test scenarios and data-driven (and in some case, scenario-specific) models. Requirement-setting and testing aspects in relevant LCM stages for UE-sided models are needed, particularly model monitoring, model switching/fallback, and data collection. There may be a need for tightening measurement requirements where measurements are used as input for training/inference of a NW-sided model.

While the above challenges are expected to be considerable in general, UE performance conformance may be particularly difficult to ensure when proprietary/OTT approaches are used for e.g. model updates, data collection and model monitoring. Piecemeal RAN4 solutions for different use-cases should be avoided, as it would be detrimental to reaching a long-term, overarching solution for handling requirements and conformance for data driven algorithms.

It could be that some LCM scenarios or feature combinations may not be feasible if the ability to define relevant requirements and test cases is to be maintained. This is not within the RAN1 area to judge but should be decided in RAN4 discussions. Final decisions about LCM scenarios and procedure details should not be taken in RAN1 before at least high-level RAN4 input is available.

RAN4 considerations about requirement setting and testability implications of should be taken into account when down-selecting use cases and options of LCM configurations and procedures.

It should be a starting assumption that introducing new ML-based functions will not compromise executing conventional NR functionalities due to processing resource conflicts. If necessary, capability reporting solutions may be studied where UEs indicate ML model support depending on other functionality, conventional or ML-based, executed in parallel.

Enabling of ML capabilities/operations should not impede operations and behaviors per existing NR specs.

In terms of UE and gNB processing, the discussion within the use case has not yet come long enough to be able to make a conclusion on what UE or gNB processing is required. The results presented currently are initial results and specific sub-uses cases that are targeted are neither selected. Hence discussing these aspects before knowing what is targeted is difficult. Going forward to ensure efficient and technically relevant discussions, timely confirmation, or feedback on feasibility of other UE processing constraints should also be ensured, where RAN1 may obtain additional input from RAN4. RAN4 may be able to confirm that RAN1 assumptions or estimates are reasonable for UE and gNB implementations, or to provide improved estimates or corrected assumptions.

# 5 Conclusions

In the previous sections we made the following observations:

Observation 1	3GPP should study the impact on 3GPP specifications for introducing AI/ML for PHY and not spend any time units on non-3GPP based solutions.

Observation 2	For the considered use cases in this SI, there are alternatives to model transfer with similar performance without the deployment challenges associated with model transfer

Observation 3	For two-sided models, standardized model pairing procedure should be easy to maintain and result in minimum inter-vendor coordination efforts

Observation 4	RAN4 considerations about requirement setting and testability implications of should be taken into account when down-selecting use cases and options of LCM configurations and procedures.

Observation 5	Enabling of ML capabilities/operations should not impede operations and behaviors per existing NR specs.

Based on the discussion in the previous sections we propose the following:

Proposal 1	Adopt the functional framework in Figure 1 for anchoring discussions related to NW-UE collaborations.

Proposal 2	Prioritize model LCM relying on offline model training.

Proposal 3	For use cases with one-sided model, responsibility for model LCM is clearly on the side that implements functionality for making model inference. Study per use case basis the necessity of assistance for model LCM

Proposal 4	Conclude that two-sided model LCM must enable a gNB to operate with a single model that works with different UE-side models.

Proposal 5	Conclude that the outlined alternatives to model transfer, shall for each use case be prioritized in further discussions in this SI

Proposal 6	Use scenarios in Table 1 with pros/cons in Table 2 for the model transfer/delivery discussion

Proposal 7	RAN1 should provide the conclusion to RAN2 that model transfer is not needed for the current studied use cases. RAN2 may study model transfer principles for future use cases

Proposal 8	Conclude that functionality identification is used as a starting point for discussing LCM operations for one-side UE models, where the functionality is identified using the UE capability framework as defined in 38.331 and 38.306 as a starting point

Proposal 9	Conclude that model identification via model-ID is not needed for one-side UE models

Proposal 10	Conclude that model activation/deactivation is done transparent to the NW for one-sided UE models

Proposal 11	Conclude that model switching is done transparent to NW for one-sided UE models (no RAN1 impact, but may have RAN4 impact)

Proposal 12	Conclude that for a one-sided AI/ML model on the UE side, fallback mechanisms can be achieved by the UE being configured by the non-AI/ML based features already existing in the specification for the applicable use case of the SI.

Proposal 13	Conclude that model registration is not needed for one-sided UE models, it is implicitly from the functionality identification

Proposal 14	Deprioritize studies and discussion on over-the-air training between NW and UE.

Proposal 15	Deprioritize model monitoring based on data distribution (both input-based and output-based).

Proposal 16	Add the following as model monitoring KPIs: false alarm rate (FAR) and missed detection rate (MDR).

Proposal 17	One-sided models at the NW-side are monitored by the NW. Study data collection mechanisms to support performance-based (e.g., inference accuracy) model monitoring for NW-sided models.

Proposal 18	One-sided models at the UE-side are monitored by the UE.

Proposal 19	The performance monitoring of two-sided models is performed at the NW side. Study data collection mechanisms to support performance-based (e.g., inference accuracy) model monitoring for two-sided models.

Proposal 20	Study 3GPP data collection mechanisms to support UE performing data logging/collection and reporting the collected data to NW over the air-interface for model training.

Proposal 21	Recommend RAN2 to study specific details of an RRC-message based data collection framework to support UE performing data logging/collection and reporting the collected data to NW for model training

Proposal 22	Study in RAN1 the requirements of data collection for model inference per AI on PHY use case.

Proposal 23	Recommend both an RRC-message based and L1 fast (i.e., similar to aperiodic CSI reporting) based data collection methods for NW side model monitoring.

Proposal 24	RAN1 to study specific requirements on RRC-message based data collection method for NW side model monitoring. When conclusions are reached on the requirements, RAN1 would indicate that to RAN2 so that RAN2 can study the corresponding RAN2 aspects.

Proposal 25	RAN1 to study specific requirements and specification impacts of an L1 fast CSI reporting based data collection method for NW side model monitoring.

Proposal 26	For 3GPP AI/ML for PHY SI discussion, companies shall report nominal computational complexity values (and not accelerator-optimized computational complexity values).

Proposal 27	To further 3GPP discussion and preparation of observations/conclusions for the technical report, three model size classes are defined per use case, as follows: - Small models (e.g., < 1 M model parameters for positioning use cases) - Medium-size models (e.g., 1 – 8 M model parameters for positioning use cases) - Large models (e.g., > 8 M model parameters for positioning use cases)

Proposal 28	For 3GPP AI/ML for PHY SI discussion, companies shall report model complexity in terms of “number of real-value model parameters” regardless of underlying model arithmetic.

# References

- RP-213599, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, Qualcomm, 3GPP TSG RAN Meeting #94e, Electronic Meeting, Dec. 6 - 17, 2021.

- 3GPP TR 37.817, Study on enhancement for Data Collection for NR and EN-DC (Release 17), RAN3, V17.0.0, 2022-04.

- R1-2300154 Evaluation of AI-CSI, Ericsson, February 2023

- RP-180524, Summary of calibration results for IMT-2020 self evaluation, Huawei, 3GPP TSG RAN Meeting #79, Mar. 19-22, 2018.

- TR 38.843, Study on Artificial Intelligence (AI) / Machine Learning (ML) for NR Air Interface.

- TR 38.901, Study on channel model for frequencies from 0.5 to 100 GHz.

- TR 38.857, Study on NR positioning enhancements

- R1-2300179, Evaluations of AIML for beam management, Ericsson, 3GPP TSG-RAN WG1 Meeting 112, February 27th - March 3rd, 2023.

- R1-2206885 Discussion on general aspects of AI-ML framework, Ericsson, 3GPP TSG-RAN WG1 Meeting 110, August, 2022.

- R1-2300141	Evaluation of AI/ML for Positioning Accuracy Enhancement,  Ericsson, 3GPP TSG-RAN WG1 Meeting 112, February 27th - March 3rd, 2023.

# Appendix A

Table: Working list of terminologies

| Terminology | Description |
| --- | --- |
| Data collection | A process of collecting data by the network nodes, management entity, or UE for the purpose of AI/ML model training, data analytics and inference |
| AI/ML Model | A data driven algorithm that applies AI/ML techniques to generate a set of outputs based on a set of inputs. |
| AI/ML model training | A process to train an AI/ML Model [by learning the input/output relationship] in a data driven manner and obtain the trained AI/ML Model for inference |
| AI/ML model Inference | A process of using a trained AI/ML model to produce a set of outputs based on a set of inputs |
| AI/ML model validation | A subprocess of training, to evaluate the quality of an AI/ML model using a dataset different from one used for model training, that helps selecting model parameters that generalize beyond the dataset used for model training. |
| AI/ML model testing | A subprocess of training, to evaluate the performance of a final AI/ML model using a dataset different from one used for model training and validation. Differently from AI/ML model validation, testing does not assume subsequent tuning of the model. |
| UE-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the UE |
| Network-side (AI/ML) model | An AI/ML Model whose inference is performed entirely at the network |
| One-sided (AI/ML) model | A UE-side (AI/ML) model or a Network-side (AI/ML) model |
| Two-sided (AI/ML) model | A paired AI/ML Model(s) over which joint inference is performed, where joint inference comprises AI/ML Inference whose inference is performed jointly across the UE and the network, i.e, the first part of inference is firstly performed by UE and then the remaining part is performed by gNB, or vice versa. |
| AI/ML model transfer | Delivery of an AI/ML model over the air interface, either parameters of a model structure known at the receiving end or a new model with parameters. Delivery may contain a full model or a partial model. |
| Model download | Model transfer from the network to UE |
| Model upload | Model transfer from UE to the network |
| Federated learning / federated training | A machine learning technique that trains an AI/ML model across multiple decentralized edge nodes (e.g., UEs, gNBs) each performing local model training using local data samples. The technique requires multiple interactions of the model, but no exchange of local data samples. |
| Offline field data | The data collected from field and used for offline training of the AI/ML model |
| Online field data | The data collected from field and used for online training of the AI/ML model |
| Model monitoring | A procedure that monitors the inference performance of the AI/ML model |
| Supervised learning | A process of training a model from input and its corresponding labels. |
| Unsupervised learning | A process of training a model without labelled data. |
| Semi-supervised learning | A process of training a model with a mix of labelled data and unlabelled data |
| Reinforcement Learning (RL) | A process of training an AI/ML model from input (a.k.a. state) and a feedback signal (a.k.a. reward) resulting from the model’s output (a.k.a. action) in an environment the model is interacting with. |
| Model activation | enable an AI/ML model for a specific function |
| Model deactivation | disable an AI/ML model for a specific function |
| Model switching | Deactivating a currently active AI/ML model and activating a different AI/ML model for a specific function |