3GPP TSG RAN WG1 #112		R1-2300302

Athens, Greece, February 27th – March 3rd, 2023

Source:	OPPO

Title:	Discussion on measurements and reporting for SL positioning

Agenda Item:	9.5.1.2

Document for:	Discussion and Decision

Introduction

In RAN#98-e, the WI on expanded and improved NR positioning was approved with following objectives on SL positioning [1]:

Objective: - Specify solutions for support of sidelink positioning (including ranging) in NR systems, including the following [RAN1, RAN2, RAN3, RAN4]: - Specify SL PRS for support of sidelink positioning such that the SL PRS uses a comb-based (full RE mapping pattern is not precluded) frequency domain structure and a pseudorandom-based sequence where the existing sequence of DL-PRS is used as a starting point [RAN1]. - Specify support for SL PRS bandwidths of up to 100 MHz in FR1 spectrum. - NOTE: SL PRS transmission in FR2 is not precluded but no FR2 specific aspects will be specified. - Specify measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA [RAN1, RAN2]. - Specify support of resource allocation for SL PRS: - Including resource allocation Scheme 1 and Scheme 2, where Scheme 1 corresponds to a network-centric SL PRS resource allocation and Scheme 2 corresponds to UE autonomous SL PRS resource allocation [RAN1]. - For resource allocation mechanism for SL PRS in Scheme 2: - Study and specify support of sensing-based resource allocation, and/or a random resource selection [RAN1]. - Study and specify solutions for congestion control for SL PRS and/or inter-UE coordination for SL-PRS [RAN1]. - Support resource allocation for shared resource pool with Rel-16/17/18 sidelink communication and dedicated resource pool for SL PRS [RAN1]. - NOTE: For SL positioning resource (pre-)configuration in a shared resource pool with Rel-16/17/18 sidelink communication, backward compatibility with legacy Rel-16/17 UEs should be ensured. - Specify procedures for transmit power control for SL PRS transmissions at least based on open loop power control (OLPC) [RAN1]. - Specify signalling and associated UE behavior for support of unicast, groupcast (not including many to one) and broadcast of SL PRS transmissions [RAN1, RAN2]. - Specify reporting signalling and procedures to facilitate support of SL positioning in all coverage scenarios and for PC5-only and joint PC5-Uu scenarios [RAN2, RAN3]: - Specify the protocol and procedures for SL positioning between UEs (Protocol for Sidelink positioning procedures (SLPP)). - Specify the protocol and procedures for SL positioning between UEs and LMF. - Specify signalling to NG-RAN for sidelink positioning and ranging service authorizations as needed. [RAN3, RAN2] - Specify corresponding new core requirements, as well as identifying and specify the impact on the existing RAN4 specification, including RRM measurements and procedures [RAN4].

According to the discussion and agreements during SI, the following measurements are introduced [2]

With regards to the Positioning methods supported using SL PRS measurements at least the following measurements are agreed to be introduced: - SL Rx-Tx time difference measurement - SL RSTD measurement - SL RSRP measurement - SL RSRPP measurement - SL RTOA measurement - SL Azimuth angle of Arrival (AoA) and SL Zenith angle of Arrival (ZoA) measurement.

This contribution discusses the measurements and reporting for Sidelink Positioning.

Discussion

  - Sidelink Rx-Tx time difference

In NR Uu positioning, the UE Rx-Tx time difference is defined as follows.

| Definition | The UE Rx – Tx time difference is defined as TUE-RX – TUE-TX Where: TUE-RX is the UE received timing of downlink subframe #i from a Transmission Point (TP) [18], defined by the first detected path in time. TUE-TX is the UE transmit timing of uplink subframe #j that is closest in time to the subframe #i received from the TP. Multiple DL PRS or CSI-RS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the TP. For frequency range 1, the reference point for TUE-RX measurement shall be the Rx antenna connector of the UE and the reference point for TUE-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TUE-RX measurement shall be the Rx antenna of the UE and the reference point for TUE-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

For SL positioning, the above principle can be reused by SL RTT-type solutions for both target UE and anchor UE.

- The definition of SL Rx-Tx time difference can be modified based on UE Rx-Tx time difference definition as follows:

| Definition | The SL Rx – Tx time difference is defined as TSL-RX – TSL-TX Where: TSL-RX is the UE received timing of sidelink subframe #i containing SL PRS from another UE, defined by the first detected path in time. TSL-TX is the UE transmit timing of sidelink subframe #j that is closest in time to the subframe #i received from another UE. Multiple SL PRS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the anchor UE or target UE. For frequency range 1, the reference point for TSL-RX measurement shall be the Rx antenna connector of the UE and the reference point for TSL-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TSL-RX measurement shall be the Rx antenna of the UE and the reference point for TSL-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

  - Sidelink reference signal time difference (SL RSTD)

The DL RSTD in NR positioning is defined in [3] as follows.

| Definition | DL reference signal time difference (DL RSTD) is the DL relative timing difference between the Transmission Point (TP) [18] j and the reference TP i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the UE receives the start of one subframe from TP j. TSubframeRxi is the time when the UE receives the corresponding start of one subframe from TP i that is closest in time to the subframe received from TP j. Multiple DL PRS resources can be used to determine the start of one subframe from a TP. For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the target UE. For frequency range 2, the reference point for the SL RSTD shall be the antenna of the target UE. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

According to [2], for SL-only positioning, at least for the purpose of absolute positioning estimation of a target UE, SL-TDOA corresponds to a method wherein SL PRS are transmitted from multiple anchor UEs to a target UE (i.e., DL-TDOA-like operation), and/or from a target UE to multiple anchor UEs (i.e., UL-TDOA-like operation).

From the perspective of target UE, the SL reference signal time difference (SL RSTD) can be obtained by measurement on SL PRSs from multiple anchor UEs to the target UE. The similar principle of defining DL RSTD can be reused for SL RSTD with modification.

- The definition of SL RSTD can be modified based on DL RSTD definition which is as follows:

| Definition | Sidelink reference signal time difference (SL RSTD) is the SL relative timing difference between an anchor UE j and another anchor UE i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the target UE receives the start of one subframe from anchor UE j. TSubframeRxi is the time when the target UE receives the corresponding start of one subframe from anchor UE i that is closest in time to the subframe received from anchor UE j. Multiple SL PRS resources can be used to determine the start of one subframe from an anchor UE. For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the SL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

  - Sidelink Relative Time of Arrival (SL RTOA)

The UL RTOA in NR positioning is defined in [3] as follows.

| Definition | The UL Relative Time of Arrival (TUL-RTOA) is the beginning of subframe i containing SRS received in Reception Point (RP) [18] j, relative to the RTOA Reference Time [16]. The UL RTOA reference time is defined as T0+tSRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time [15, TS 38.455] - tSRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SRS, respectively. Multiple SRS resources can be used to determine the beginning of one subframe containing SRS received at a RP. The reference point for TUL-RTOA shall be: - for type 1-C base station TS 38.104 [9]: the Rx antenna connector, - for type 1-O or 2-O base station TS 38.104 [9]: the Rx antenna (i.e. the centre location of the radiating region of the Rx antenna), - for type 1-H base station TS 38.104 [9]: the Rx Transceiver Array Boundary connector. |
| --- | --- |

From the perspective of anchor UE, the SL relative time of arrival (SL RTOA) can be obtained by measurement on SL PRS from a target UE to the anchor UE. The similar principle of defining UL RTOA can be reused for SL RTOA with modification.

Regarding SL RTOA Reference Time, although target UE and/or anchor UEs may be out-of-coverage, thanks to SL synchronization mechanism, the SFN among target UE and anchor UEs should be aligned, hence the reference time can be defined as the start of SL PRS subframe.

- The definition of SL RTOA can be modified based on UL RTOA definition which is as follows:

| Definition | The SL Relative Time of Arrival (SL RTOA) is the beginning of subframe i containing SL PRS received by anchor UE j, relative to the SL RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time - tSL-PRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SL-PRS, respectively. Multiple SL PRS resources can be used to determine the beginning of one subframe containing SL PRS received at an anchor UE. |
| --- | --- |

  - Sidelink PRS Reference signal received power (SL PRS-RSRP)

The DL PRS-RSRP in NR positioning is defined in [3] as follows.

| Definition | DL PRS reference signal received power (DL PRS-RSRP), is defined as the linear average over the power contributions (in [W]) of the resource elements that carry DL PRS reference signals configured for RSRP measurements within the considered measurement frequency bandwidth. For frequency range 1, the reference point for the DL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported DL PRS-RSRP value shall not be lower than the corresponding DL PRS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

SL PRS transmission(s) can be associated with PSCCH, and the SL PRS-RSRP can be defined by considering PSCCH.

- The definition of SL PRS-RSRP can be modified based on DL PRS-RSRP definition which is as follows:

| Definition | SL PRS reference signal received power (SL PRS-RSRP), is defined as the linear average over the power contributions of the resource elements that carry SL PRS reference signals. For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

  - Sidelink PRS Reference signal received path power (SL PRS-RSRPP)

The DL PRS-RSRPP in NR positioning is defined in [3] as follows.

| Definition | DL PRS reference signal received path power (DL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry DL PRS signal configured for the measurement, where DL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the DL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, DL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_CONNECTED, RRC_INACTIVE |

Regarding SL PRS-RSRPP, the similar principle of defining DL PRS-RSRPP can be reused with modification

- The definition of SL PRS-RSRPP can be modified based on DL PRS-RSRPP definition which is as follows:

| Definition | SL PRS reference signal received path power (SL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

  - Sidelink Angle of Arrival (SL AoA)

The UL AoA in NR positioning is defined in [3] as follows.

| Definition | UL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901 [15]. The UL AoA is determined at the gNB antenna for an UL channel corresponding to this UE. |
| --- | --- |

SL AoA includes both Azimuth of Arrival (AoA) and Zenith of Arrival (ZoA). The above principle can be reused to define SL AoA.

- The definition of SL AoA can be modified based on UL AoA definition which is as follows:

| Definition | SL Angle of Arrival (SL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901. The SL AoA is determined at a UE antenna for an SL channel corresponding to a target UE or anchor UE. |
| --- | --- |

Conclusion

In this contribution, the measurements to support RTT-type solutions using SL, SL-AoA, and SL-TDOA are discussed, and the corresponding definitions on the measurements are proposed as follows:

- The definition of SL Rx-Tx time difference can be modified based on UE Rx-Tx time difference definition as follows:

| Definition | The SL Rx – Tx time difference is defined as TSL-RX – TSL-TX Where: TSL-RX is the UE received timing of sidelink subframe #i containing SL PRS from another UE, defined by the first detected path in time. TSL-TX is the UE transmit timing of sidelink subframe #j that is closest in time to the subframe #i received from another UE. Multiple SL PRS for tracking resources, as instructed by higher layers, can be used to determine the start of one subframe of the first arrival path of the anchor UE or target UE. For frequency range 1, the reference point for TSL-RX measurement shall be the Rx antenna connector of the UE and the reference point for TSL-TX measurement shall be the Tx antenna connector of the UE. For frequency range 2, the reference point for TSL-RX measurement shall be the Rx antenna of the UE and the reference point for TSL-TX measurement shall be the Tx antenna of the UE. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

- The definition of SL RSTD can be modified based on DL RSTD definition which is as follows:

| Definition | Sidelink reference signal time difference (SL RSTD) is the SL relative timing difference between an anchor UE j and another anchor UE i, defined as TSubframeRxj – TSubframeRxi, Where: TSubframeRxj is the time when the target UE receives the start of one subframe from anchor UE j. TSubframeRxi is the time when the target UE receives the corresponding start of one subframe from anchor UE i that is closest in time to the subframe received from anchor UE j. Multiple SL PRS resources can be used to determine the start of one subframe from an anchor UE. For frequency range 1, the reference point for the SL RSTD shall be the antenna connector of the UE. For frequency range 2, the reference point for the SL RSTD shall be the antenna of the UE. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

- The definition of SL RTOA can be modified based on UL RTOA definition which is as follows:

| Definition | The SL Relative Time of Arrival (SL RTOA) is the beginning of subframe i containing SL PRS received by anchor UE j, relative to the SL RTOA Reference Time. The SL RTOA reference time is defined as T0+tSL-PRS, where - T0 is the nominal beginning time of SFN 0 provided by SFN Initialization Time - tSL-PRS=10nf+nsf×10-3, where nf and nsf are the system frame number and the subframe number of the SL-PRS, respectively. Multiple SL PRS resources can be used to determine the beginning of one subframe containing SL PRS received at an anchor UE. |
| --- | --- |

- The definition of SL PRS-RSRP can be modified based on DL PRS-RSRP definition which is as follows:

| Definition | SL PRS reference signal received power (SL PRS-RSRP), is defined as the linear average over the power contributions of the resource elements that carry SL PRS reference signals. For frequency range 1, the reference point for the SL PRS-RSRP shall be the antenna connector of the UE. For frequency range 2, SL PRS-RSRP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. For frequency range 1 and 2, if receiver diversity is in use by the UE, the reported SL PRS-RSRP value shall not be lower than the corresponding SL PRS-RSRP of any of the individual receiver branches. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

- The definition of SL PRS-RSRPP can be modified based on DL PRS-RSRPP definition which is as follows:

| Definition | SL PRS reference signal received path power (SL PRS-RSRPP), is defined as the power of the linear average of the channel response at the i-th path delay of the resource elements that carry SL PRS signal configured for the measurement, where SL PRS-RSRPP for the 1st path delay is the power contribution corresponding to the first detected path in time. For frequency range 1, the reference point for the SL PRS-RSRPP shall be the antenna connector of the UE. For frequency range 2, SL PRS-RSRPP shall be measured based on the combined signal from antenna elements corresponding to a given receiver branch. |
| --- | --- |
| Applicable for | RRC_IDLE intra-frequency, RRC_IDLE inter-frequency, RRC_CONNECTED intra-frequency, RRC_CONNECTED inter-frequency |

- The definition of SL AoA can be modified based on UL AoA definition which is as follows:

| Definition | SL Angle of Arrival (UL AoA) is defined as the estimated azimuth angle and vertical angle of a UE with respect to a reference direction, wherein the reference direction is defined: - In the global coordinate system (GCS), wherein estimated azimuth angle is measured relative to geographical North and is positive in a counter-clockwise direction and estimated vertical angle is measured relative to zenith and positive to horizontal direction - In the local coordinate system (LCS), wherein estimated azimuth angle is measured relative to x-axis of LCS and positive in a counter-clockwise direction and estimated vertical angle is measured relatize to z-axis of LCS and positive to x-y plane direction. The bearing, downtilt and slant angles of LCS are defined according to TS 38.901. The SL AoA is determined at a UE antenna for an SL channel corresponding to a target UE or anchor UE. |
| --- | --- |

Reference

- RP-223549, New WID on Expanded and Improved NR Positioning, RAN#98-e, e-Meeting, December 12th – 16th, 2022.

- 3GPP TR 38.859, v18.0.0, Study on expanded and improved NR positioning, (Release 18), December, 2022.

- 3GPP TP 38.215, v17.1.0, Physical layer measurements, (Release 17), March, 2022.