3GPP TSG-RAN WG1 #112	R1-2300337

Athens, Greece, February 27th – March 3rd, 2023

Title: 	Remaining issues for UE features topics 1

Source: 	ZTE

Agenda item:	8.16.1

Document for:	Discussion/Decision

Introduction

In RAN1#111 meeting, the UE feature list is updated and most of the issues have been finalized [1]. In this document, some remaining issues are discussed.

UE feature for MBS

The “Consequence if the feature is not supported by the UE” for FG33-3-33a and FG33-3-3b is still FFS. Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast is included in both FG 33-3-3a and 33-3-3b. It is kind of “default UE behaviour”. The “Consequence if the feature is not supported by the UE” for FG 33-3-3a and FG 33-3-3b can be updated as following: If UE doesn’t indicate support of FG33-3-3a nor FG33-3-3b, UE doesn’t support multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast.

Proposal 1: Add the following for “Consequence if the feature is not supported by the UE” for FG 33-3-3a and FG 33-3-3b

- If UE doesn’t indicate support of FG33-3-3a nor FG33-3-3b, UE doesn’t support multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast.

| 33. NR_MBS | 33-3-3a | FDM-ed Type-1 and Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of FDM-ed Type-1 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-3-2, at least one of {33-2a, 33-4, 33-5-1a, 33-5-1f} | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: FDM-ed Type-1 HARQ-ACK codebook is generated by concatenating the Type-1 sub-codebook for unicast and the Type-1 sub-codebook for multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-3-3b | Mode 2 TDM-ed Type-1 and Type-2 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast | 1. Support of Mode 2 TDM-ed Type-1 HARQ-ACK codebook for multiplexing HARQ-ACK for unicast and ACK/NACK-based HARQ-ACK for multicast on PUCCH or PUSCH 2. Support of Type-2 HARQ-ACK codebooks for multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast on PUCCH or PUSCH with max number X of G-RNTIs | 33-2a or 33-4 or 33-5-1a or 33-5-1f | Yes |  | FFS | Per BC | N/A | N/A |  | Note1: Mode 2 TDM-ed Type-1 HARQ-ACK codebook is generated based on the union TDRA tables from unicast and multicast and the union of k1 sets from unicast and multicast. Note2: The Type-2 HARQ-ACK codebook is generated by concatenating the Type-2 sub-codebook for unicast and the Type-2 sub-codebook for multicast. Candidate values of X is {1, 2, 3, 4} with X no larger than max number of G-RNTIs of FG33-2e Note: the value of X should be common across FG33-2a, 33-3-3a and 33-3-3b if reported | Optional with capability signalling |

The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is still FFS. From our perspective, all of them can be reported as per UE without FR1/FR2 separation and without TDD/FDD separation.

Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation.

| 33. NR_MBS | 33-5-1e | Dynamic Slot-level repetition for SPS group-common PDSCH for multicast | Support up to X times dynamic slot-level repetition for SPS group-common PDSCH for multicast. | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate values for X is: {8, 16} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 33. NR_MBS | 33-5-1f | NACK-only based HARQ-ACK feedback for multicast RRC-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | 1) Support NACK-only based HARQ-ACK feedback, and support of enabling/disabling NACK-only based HARQ-ACK feedback configured by RRC signalling for SPS group-common PDSCH without PDCCH scheduling 2) Support of PTM retransmission associated with G-CS-RNTI for SPS multicast | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| 33. NR_MBS | 33-5-1g | DCI-based enabling/disabling NACK-only based feedback for SPS group-common PDSCH for multicast | Support of DCI-based enabling/disabling NACK-only based HARQ-ACK feedback configured per G-CS-RNTI for multicast by RRC signaling via DCI format 4_2 | 33-5-1f, 33-5-1i | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |

| 33. NR_MBS | 33-5-1i | Multicast SPS scheduling with DCI format 4_2 | 1.Support of DCI format 4_2 with CRC scrambled with G-CS-RNTI for multicast SPS scheduling 2. Retransmission scheduled by DCI format 4_2 with CRC scrambled with G-CS-RNTI | 33-5-1 | Yes |  |  | FFS | FFS | FFS |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 33. NR_MBS | 33-5-2 | Multiple SPS group-common PDSCH configuration on PCell | 1. Support up to 8 SPS group-common PDSCH configuration per CFR for multicast 2. Support M>=1 activated SPS group-common PDSCH configurations per CFR for multicast 3. The total number of SPS configurations for both multicast and unicast is no larger than 8 in a BWP of a serving cell, and activated SPS group-common PDSCH configurations is no larger than M. 4. The total number of SPS configurations for both multicast and unicast in a cell group is no larger than 32 | 33-5-1 | Yes |  |  | [Per UE] | [No] | [No] |  | Candidate value set for M is {1, 2, …, 8} | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 33. NR_MBS | 33-8-1 | PUCCH resource configuration for multicast feedback for dynamically scheduled multicast | Support of a PUCCH-Config for multicast HARQ-ACK feedback, separate from that of unicast configurations | 33-2a | Yes |  |  | [Per band or per FSPC] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

| 33. NR_MBS | 33-9 | Supporting unicast PDCCH to release SPS group-common PDSCH | Supports unicast PDCCH scrambled with CS-RNTI to release SPS group-common PDSCH | 33-5-1, 12-2 | Yes |  |  | [Per UE] | [No] | [No] |  |  | Optional with capability signalling |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |

Conclusion

In this contribution, we provide proposals for remaining issues for UE features topics 1.

MBS

Proposal 1: Add the following for “Consequence if the feature is not supported by the UE” for FG 33-3-3a and FG 33-3-3b

- If UE doesn’t indicate support of FG33-3-3a nor FG33-3-3b, UE doesn’t support multiplexing HARQ-ACK for unicast and HARQ-ACK for multicast.

Proposal 2: The reporting type for FG33-5-1e, 33-5-1f, 33-5-1g, 33-5-1i, 33-5-2, 33-8-1, 33-9 is per UE without FR1/FR2 separation and without TDD/FDD separation.

Reference

- R1-2212895, Updated RAN1 UE features list for Rel-17 NR after RAN1#111, Moderators (AT&T, NTT DOCOMO, INC.), RAN1#111.